3GPP RAN WG2 Meeting #125bis	R2-2402690

# Changsha, China, April 15th –19th, 2024

Agenda Item:	8.4.2

Source:	HONOR

Title:	Overall procedure of LP-WUS in RRC_IDLE/INACTIVE

Document for:	Discussion and Decision

1. Introduction

In RAN#103 meeting, the following objectives of Rel-19 LP-WUS work item have been agreed [1]:

- To specify an LP-WUS design commonly applicable to both IDLE/INACTIVE and CONNECTED modes (RAN1, RAN4) - Specify OOK (OOK-1 and/or OOK-4) based LP-WUS with overlaid OFDM sequence(s) over OOK symbol - The LP-WUS design shall ensure that for IDLE/INACTIVE operation, the same information is delivered irrespective of LP-WUR type. The same information can be delivered by the OOK and OFDM signal constructing the overlaid sequences. - At least duty-cycled monitoring of LP-WUS is supported - For IDLE/INACTIVE modes (RAN1, RAN2, RAN3,RAN4) - Specify procedure and configuration of LP-WUS indicating paging triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring - Specify LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell. - LP-SS is based on OOK-1 and/or OOK-4 waveform with or without overlaid OFDM sequences. Further down selection between with and without overlaid OFDM sequences is to be done within WI. - Note: For LP-WUR that can receive existing PSS/SSS, existing PSS/SSS can be used for synchronization and RRM instead of LP-SS. - Y will be decided within WI. 320ms is the start point. - Specify further RRM relaxation of UE MR for both serving and neighbour cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2)

In this contribution, we focus on the discussion of overall procedure for LP-WUS in RRC_IDLE/INACTIVE.

2. Discussion

In our understanding, LP-WUS procedure in RRC_IDLE/INACTIVE state is as follows:

- LP-WUS configuration needs to be provided by the network.

- The UE supporting LP-WUS should continuously evaluate the monitoring condition of LP-WUS based on corresponding configuration. If the monitoring condition is fulfilled, the UE can open LR to monitor LP-WUS. Otherwise, LR needs to be closed and MR should be used to monitor PO as in legacy.

- For the UE, if the monitoring condition is fulfilled, it monitors LP-WUS according to configuration, e.g., LP-WUS subgroup, time/frequency resource of LP-WUS, etc. For the gNB, LP-WUS can be used to wake up the MR of the UE to monitor paging according to UE capability.

2.1. LP-WUS configuration

At least the following configurations need to be provided to the UE for LP-WUS monitoring:

- Monitoring condition. According to evaluation in section 8.2 of TR38.869 [2], there could be partial LP-WUS coverage in the cell. The UE needs to continuously evaluate whether the monitoring condition of LP-WUS is fulfilled to decide when to start/stop LP-WUS monitoring.

- Resource for LP-WUS. If monitoring condition is fulfilled, UE needs to know how to monitor LP-WUS in time-frequency domain.

- LP-WUS subgroup. If subgroup is supported, the UE needs to know the number of subgroups and which subgroup needs to be monitored.

- Other configuration may also need to be provided depending on further discussion, e.g., LP-SS configuration, RRM relaxation configuration, etc.

Generally, SIB can be used to provide cell specific LP-WUS configuration for UEs in both RRC_IDLE and RRC_INACTIVE. Furthermore, for UEs in RRC_INACTIVE, RRC release message can also be used to provide UE specific configuration. For the time being, we do not see benefit to have UE specific configuration for LP-WUS. Thus, we think LP-WUS configuration via SIB is sufficient for both RRC_IDLE and RRC_INACTIVE.

Proposal 1: SIB is used to provide LP-WUS configuration (including monitoring condition, LP-WUS resource and subgroup, etc.), for both RRC_IDLE and RRC_INACTIVE.

2.2. Monitoring condition of LP-WUS

UE should continuously evaluate the monitoring condition of LP-WUS if configured by the network:

- Before the monitoring condition of LP-WUS is not fulfilled, the UE needs to keep MR open and monitor PO as in legacy. MR measurement on legacy SSB is used be compared with pre-configured monitoring condition to decide when to start monitor LP-WUS.

- When the monitoring condition is fulfilled, the UE should turn on LR to monitor LP-WUS and turn off MR to enter ultra-deep-sleep state for power saving. LR is also used to perform measurement to decide whether to stop monitoring LP-WUS and turn on MR to monitor PO as in legacy. According to the WID, both LP-SS and existing PSS/SSS are possible for LR to perform measurement:

  - For LR that can only receive LP-SS, LP-SS should be used as the target reference signal for RRM measurement.

  - For LR that can receive existing PSS/SSS, existing PSS/SSS can also be used for RRM measurement instead of LP-SS.

Proposal 2: UE should perform MR RRM measurement based on legacy SSB of the serving cell before the monitoring condition is fulfilled.

Proposal 3: UE should perform LR RRM measurement when the monitoring condition is fulfilled, and the target reference signal could be different according to the capability of LR.

2.3. LP-WUS monitoring

Subgrouping for LP-WUS is beneficial to reduce the negative impact from false alarm thus needs to be supported according to the WID.

The maximum total number of subgroups associated with certain PO/PF is determined by the number of bits that can be carried in payload of LP-WUS and the payload format, e.g., bitmap indication or group ID. The number of subgroups of LP-WUS was discussed in the last RAN1 meeting but no agreement was achieved. It has been concluded in RAN1 that RAN2 needs to decide how to map a UE to a subgroup for LP-WUS.

Conclusion For idle/inactive mode, how to map a UE to a subgroup ID for LP-WUS is left to RAN2 to decide.

Thus, it is proposed to discuss how to map a UE to a subgroup based on further RAN1 input on subgroup number.

Proposal 4: RAN2 to discuss how to map a UE to a subgroup based on further RAN1 input on subgroup number.

From gNB perspective, UE capability related to LP-WUS is needed to decide whether to use LP-WUS to wake up the UE to monitor paging. For CN paging, LP-WUS related capability can be provided by AMF in UERadioPagingInformation. For RAN paging, corresponding capability can be reported by the UE in UECapabilityInformation.

Proposal 5: The gNB needs to be aware of UE capability related to LP-WUS via UERadioPagingInformation or UECapabilityInformation.

3. Conclusions

In this contribution, we discussed overall procedure of LP-WUS and the proposals are summarised as below.

Proposal 1: SIB is used to provide LP-WUS configuration (including monitoring condition, LP-WUS resource and subgroup, etc.), for both RRC_IDLE and RRC_INACTIVE.

Proposal 2: UE should perform MR RRM measurement based on legacy SSB of the serving cell before the monitoring condition is fulfilled.

Proposal 3: UE should perform LR RRM measurement when the monitoring condition is fulfilled, and the target reference signal could be different according to the capability of LR.

Proposal 4: RAN2 to discuss how to map a UE to a subgroup based on further RAN1 input on subgroup number.

Proposal 5: The gNB needs to be aware of UE capability related to LP-WUS via UERadioPagingInformation or UECapabilityInformation.

4. References

[1]	RP-240801 Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)

[2]	TR 38.869 V2.0.0 Study on low-power wake up signal and receiver for NR