# 3GPP TSG RAN WG2 Meeting #125b      	                           R2-2402854
Changsha, China, 15th – 19th April, 2024

Agenda item:	7.3.3

Source:	Apple

Title:	Remaining control plane open issues on NES

WID/SID:	Netw_Energy_NR-Core– Release 18

Document for:	Discussion and Decision

# 1 Introduction

The Network Energy saving (NES) WID RP-223540 was agreed in RAN#98-e [1]. By RAN2#125 [2], the latest RRC CR has been endorsed in [3] and latest TS 38.306 has been endorsed in [4].

In this contribution, we share our view on below remaining RRC open issue:

- Issue 1: CSI-RS measurement capability signaling of inter-band SSB-less CA

- Issue 2: Capability of A-TRS based inter-band SSB-less SCell activation

# 2 Discussion

## 2.1 Issue 1: CSI-RS meas. capability of inter-band SSB-less CA

In LS R2-2401808 [5], RAN4 included the agreed UE capability signalling of inter-band SSB-less CA:

As can be seen, it was agreed as per FS per reference band per band combination. And current endorsed TS 38.306 has captured it accordingly [4].

| scellWithoutSSB Defines whether the UE supports configuration of SCell that does not transmit SS/PBCH block. This is conditionally mandatory with capability signalling for intra-band CA but not supported for inter-band CA. | FS | CY | N/A | N/A |
| --- | --- | --- | --- | --- |
| scellWithoutSSB-InterBandCA-r18 Indicates whether the UE supports SCell without SS/PBCH block for inter-band CA. For each band within the BC, UE indicates if it supports the SSB-less operation when this band is the reference band and other band(s) in the BC as the SSB-less band(s). | FS | No | N/A | FR1 only |

Observation 1: LS R2-2401808, RAN4 agreed the UE capability signalling of inter-band SSB-less CA is per FS per reference band per band combination. And current endorsed TS 38.306 has captured it accordingly.

However, we think one capability related to CSI-RS measurements in inter-band SSB-less CA was missed. Please note that below capability signalling of CSI-RS measurement in intra-band SSB-less CA was introduced in Rel-15 [4]:

| csi-RS-MeasSCellWithoutSSB Defines whether the UE can perform CSI-RSRP and CSI-RSRQ measurement as specified in TS 38.215 [13], where CSI-RS resource is configured for a cell that does not transmit SS/PBCH block. A UE that supports this feature shall also support scellWithoutSSB. | FS | No | N/A | N/A |
| --- | --- | --- | --- | --- |

Note that in RAN4#108b [6], the following agreement on CSI-RS based measurement for inter-band SSB-less SCell was made:

Issue 1-4-1: Whether to have L1 measurement on SSB-less SCell Agreement: - When CSI-RS based L1 measurement is configured, legacy requirements apply. - No RRM spec impact.

Observation 2: Capability signalling of CSI-RS measurement in intra-band SSB-less CA (csi-RS-MeasSCellWithoutSSB was introduced in Rel-15. The capability of CSI-RS measurements in inter-band SSB-less CA was missed.

Thus, we think a similar capability for CSI-RS based measurement for inter-band SSB-less SCell needs to be introduced.

Proposal 1: Introduce a separate capability of CSI-RSRP and CSI-RSRQ measurements in inter-band SSB-less CA.

We also provide a TP on top of endorsed TS 38.306 spec [4] in Appendix 1.

Proposal 2: RAN2 agree the TP in Appendix 1 to add missed capability of CSI-RSRP and CSI-RSRQ measurements in inter-band SSB-less CA.

## 2.2 Issue 2: A-TRS based inter-band SSB-less SCell activation

| aperiodicCSI-RS-FastScellActivation-r17 Indicates whether the UE supports aperiodic CSI-RS for tracking for fast SCell activation, i.e., 1) Aperiodic CSI-RS for tracking for fast SCell activation is triggered by enhanced SCell activation/deactivation MAC CE; 2) Aperiodic CSI-RS for tracking for fast SCell activation is triggered within the BWP indicated by firstActiveDownlinkBWP-Id for the SCell. d This field includes the following parameters: - maxNumberAperiodicCSI-RS-PerCC-r17 indicates the maximum number of aperiodic CSI-RS resource set configurations for tracking for fast SCell activation that can be configured to UE per CC in a reported band. Value n8 corresponds to 8, n16 corresponds to 16, and so on. - maxNumberAperiodicCSI-RS-AcrossCCs-r17 indicates the maximum number of aperiodic CSI-RS resource set configurations for tracking for fast SCell activation that can be configured to UE across CCs in a reported band. Value n8 corresponds to 8, n16 corresponds to 16, and so on. NOTE: - maxNumberAperiodicCSI-RS-PerCC-r17 and maxNumberAperiodicCSI-RS-AcrossCCs-r17 values refer to the number of RS configurations for fast SCell activation that can be indicated by the MAC CE. - The NZP-CSI-RS configured as RS for tracking for fast SCell activation are not considered when counting the maximum NZP-CSI-RS configurations of CSI-RS and CSI-IM reception for CSI feedback. | Band | No | N/A | N/A |
| --- | --- | --- | --- | --- |

Above capability signalling of A-TRS based intra-band SSB-less SCell activation was introduced in Rel-17 [4]. In RAN4#110 [7], it was agreed to reuse this capability for inter-band SSB-less SCell activation:

Issue 1-2-2: UE capability indication for A-TRS based inter-band SSB-less SCell activation Agreement: Reuse AperiodicCSI-RS-FastScellActivation-r17

Thus, we can clarify that it is applied to both intra-band SSB-less SCell activation and inter-band SSB-less SCell activation in the field description.

Proposal 3: In the field description of capability aperiodicCSI-RS-FastScellActivation-r17, clarify that it is applied to both intra-band SSB-less SCell activation and inter-band SSB-less SCell activation.

We also provide a TP on top of endorsed TS 38.306 spec [4] in Appendix 2.

Proposal 4: RAN2 agree the TP in Appendix 2 on aperiodicCSI-RS-FastScellActivation-r17.

# 3 Conclusion

In this contribution, we discuss remaining control plane open issue. Our observations can be found below.

Observation 1: LS R2-2401808, RAN4 agreed the UE capability signalling of inter-band SSB-less CA is per FS per reference band per band combination. And current endorsed TS 38.306 has captured it accordingly.

Observation 2: Capability signalling of CSI-RS measurement in intra-band SSB-less CA (csi-RS-MeasSCellWithoutSSB was introduced in Rel-15. The capability of CSI-RS measurements in inter-band SSB-less CA was missed.

Based on the observation, the proposals are:

Proposal 1: Introduce a separate capability of CSI-RSRP and CSI-RSRQ measurements in inter-band SSB-less CA.

Proposal 2: RAN2 agree the TP in Appendix 1 to add missed capability of CSI-RSRP and CSI-RSRQ measurements in inter-band SSB-less CA.

Proposal 3: In the field description of capability aperiodicCSI-RS-FastScellActivation-r17, clarify that it is applied to both intra-band SSB-less SCell activation and inter-band SSB-less SCell activation.

Proposal 4: RAN2 agree the TP in Appendix 2 on aperiodicCSI-RS-FastScellActivation-r17.

# 4 References

[1] RP-223540, New WID: Network energy savings for NR, Huawei

[2] RAN2#125, Chair Notes

[3] R2-2401877, Network energy savings for NR miscellaneous RRC CR, Huawei.

[4] R2-2401690, Corrections and Updates to UE capabilities for Rel-18 WIs, Intel Corporation.

[5] R4-2403842, Rel-18 RAN4 UE feature list for NR (version 3), CMCC.

[6] R4-2317306, WF on RRM requirements for NR network energy saving, Huawei, HiSilicon

[7] R4-2403526, WF on RRM requirements for NR network energy saving, Huawei, HiSilicon

# Appendix 1

| csi-RS-MeasSCellWithoutSSB Defines whether the UE can perform CSI-RSRP and CSI-RSRQ measurement as specified in TS 38.215 [13], where CSI-RS resource is configured for a cell that does not transmit SS/PBCH block in intra-band CA. A UE that supports this feature shall also support scellWithoutSSB. | FS | No | N/A | N/A |
| --- | --- | --- | --- | --- |
| csi-RS-MeasSCellWithoutSSB-InterBandCA-r18 Defines whether the UE can perform CSI-RSRP and CSI-RSRQ measurement as specified in TS 38.215 [13], where CSI-RS resource is configured for a cell that does not transmit SS/PBCH block in inter-band CA. A UE that supports this feature shall also support scellWithoutSSB-InterBandCA-r18. | FS | No | N/A | N/A |

# Appendix 2

| aperiodicCSI-RS-FastScellActivation-r17 Indicates whether the UE supports aperiodic CSI-RS for tracking for fast SCell activation, i.e., 1) Aperiodic CSI-RS for tracking for fast SCell activation is triggered by enhanced SCell activation/deactivation MAC CE; 2) Aperiodic CSI-RS for tracking for fast SCell activation is triggered within the BWP indicated by firstActiveDownlinkBWP-Id for the SCell. d This field includes the following parameters: - maxNumberAperiodicCSI-RS-PerCC-r17 indicates the maximum number of aperiodic CSI-RS resource set configurations for tracking for fast SCell activation that can be configured to UE per CC in a reported band. Value n8 corresponds to 8, n16 corresponds to 16, and so on. - maxNumberAperiodicCSI-RS-AcrossCCs-r17 indicates the maximum number of aperiodic CSI-RS resource set configurations for tracking for fast SCell activation that can be configured to UE across CCs in a reported band. Value n8 corresponds to 8, n16 corresponds to 16, and so on. This field applies to both intra-band SSB-less SCell activation and inter-band SSB less SCell activation. NOTE: - maxNumberAperiodicCSI-RS-PerCC-r17 and maxNumberAperiodicCSI-RS-AcrossCCs-r17 values refer to the number of RS configurations for fast SCell activation that can be indicated by the MAC CE. - The NZP-CSI-RS configured as RS for tracking for fast SCell activation are not considered when counting the maximum NZP-CSI-RS configurations of CSI-RS and CSI-IM reception for CSI feedback. | Band | No | N/A | N/A |
| --- | --- | --- | --- | --- |