3GPP TSG-RAN WG2 Meeting #125bis	R2-2402984

Changsha, China,  April 15th – 19th, 2024

Agenda item:	7.4.4

Source:	Samsung

Title:	Support of Activating the Flexible Number of TCI States using Candidate Cell TCI States Activation/Deactivation MAC CE

WID/SID:	NR_Mob_enh2-Core

Document for:	Discussion and Decision

Introduction

In the current TS 38.321 [1], the Candidate Cell TCI States Activation/Deactivation MAC CE has introduced to support to activate/deactivate TCI states associated to a subset of codepoints in DCI for LTM. However, the current MAC CE format makes difficult to know the actual numbers of activated TCI codepoint(s) because there is no explicit parameter for the activated TCI codepoint in the MAC CE.

In this contribution, we suggest to revise the Candidate Cell TCI States Activation/Deactivation MAC CE to add the new field to indicate the actual numbers of activated TCI codepoint(s) in this MAC CE.

Discussion

Introduction

Rel-18 LTM UE performs L1 measurement and reports CSI measurement results to NW, based on which NW decides the target cell for cell switch. NW activates/deactivates a list of TCI states before cell switch and indicates one activated TCI state in LTM cell switch MAC CE. The signal in MAC CE is needed to activate/deactivate TCI states associated to a subset of codepoints in DCI. Here, the number of activated TCI states codpoint(s) should be flexible from one to eight. However, the current MAC CE format makes difficult to know the actual numbers of activated TCI codepoint(s) because there is no explicit parameter for the activated TCI codepoint in the MAC CE, see Appendix which captures the MAC CE related text from TS 38.321 [1].

Observation 1: The current MAC CE format for Candidate Cell TCI States Activation/Deactivation MAC CE makes difficult to know the actual numbers of activated TCI codepoint(s) because there is no explicit parameter for the activated TCI codepoint in the MAC CE.

Support the Flexible Number of TCI States using MAC CE

From our understanding, this MAC CE format referred the Unified TCI States Activation/Deactivation MAC CE which was introduced in Rel-17 feMIMO WI. We think this MAC CE format is not optimized so need to be enhanced. As we mentioned above, the required feature of the MAC CE signaling to indicate at most 8 TCI codepoints and this MAC CE has variable length. So, if UE decodes all fields and the subheader after receiving this MAC CE, UE could know the number of activated TCI codepoints and the TCI states using the combination of MAC SDU length, the Pi field and TCI state ID fields.

However, it is not good MAC CE design i.e. it cause the complex UE implementation.

We think the Rel-18 Candidate Cell TCI States Activation/Deactivation MAC CE could be enhanced to add the explicit field to indicate the activated TCI codepoints in the MAC CE. It gives a simple UE implementation to support the activating at most 8 TCi codepoints using this MAC CE.

Proposal: Add the new explicit indication of the number of activated TCI codepoints in the Candidate Cell TCI States Activation/Deactivation MAC CE.

Proposed MAC CE format

In this contribution, we provides the two options to support indicating the activated TCI codepoints in Candidate Cell TCI States Activation/Deactivation MAC CE. Both options could work from our understanding and we have slight prefer the option 1 because of simplicity.

- Option 1: Adding the new Number of codepoints field

The Candidate Cell TCI States Activation/Deactivation MAC CE is identified by a MAC subheader with eLCID as specified in Table 6.2.1-1b [1]. It has a variable size consisting of following fields: - Candidate Cell ID: This field indicates the identity of an LTM candidate cell for which the MAC CE applies, corresponding to the ltm-CandidateId minus 1 as specified in TS 38.331 [5]. The length of the field is 3 bits; - Number of codepoints: This field indicates the number of codepoints of the DCI Transmission Configuration Indication field to which TCI states presented/activated in this MAC CE are associated. The length of the field is 3 bits. The value of this field is from 1 to 8. The field set to value i (i=1, 2, .., 8) indicates the TCI states presented/activated in this MAC CE are associated with first i codepoints (i.e., codepoints with value 1 to i), the first Pi fields (i.e., P1, P2, .., Pi) are present, and R bits are present instead of Pi+1, .., P8. - Pi: This field indicates whether each TCI codepoint has multiple TCI states or a single TCI state. If the Pi field is set to 1, the ith TCI codepoint includes the DL TCI state and the UL TCI state. If the Pi field is set to 0, the ith TCI codepoint includes only the DL/joint TCI state or the UL TCI state. The codepoint to which a TCI state is mapped is determined by its ordinal position among all the TCI state ID fields; - D/U: This field indicates whether the TCI state ID in the same octet is for a joint/downlink or an uplink TCI state. If this field is set to 1, the TCI state ID in the same octet is for joint/downlink TCI state. If this field is set to 0, the TCI state ID in the same octet is for uplink TCI state; - TCI state ID: This field indicates the TCI state identified by TCI-StateId in ltm-DL-OrJointTCI-StateToAddModList or TCI-UL-StateId in ltm-UL-TCI-StatesToAddModList as specified in TS 38.331 [5]. If D/U is set to 1, 7-bits length TCI state ID i.e. TCI-StateId as specified in TS 38.331 [5] is used. If D/U is set to 0, the most significant bit of TCI state ID is considered as the reserved bit and remaining 6 bits indicate the TCI-UL-StateId as specified in TS 38.331 [5]. The maximum number of activated TCI states is 16; A TCI state is activated if its TCI state ID is present. Otherwise, it is deactivated. - R: Reserved bit, set to 0. Figure 4: Candidate Cell TCI state activation/deactivation MAC CE Example-1

- Option 2: Adding the new Ci field (bitmap)

The Candidate Cell TCI States Activation/Deactivation MAC CE is identified by a MAC subheader with eLCID as specified in Table 6.2.1-1b [1]. It has a variable size consisting of following fields: - Candidate Cell ID: This field indicates the identity of an LTM candidate cell for which the MAC CE applies, corresponding to the ltm-CandidateId minus 1 as specified in TS 38.331 [5]. The length of the field is 3 bits; - Ci: This field indicates whether TCI state(s) associated to the i-th codepoint of the DCI Transmission Configuration Indication field are presented/activated in this MAC CE. This field set to value 1 indicates Pi field is present and the TCI state(s) associated to the i-th codepoint are present/activated. This field set to 0 indicates R bit is present instead of Pi and the TCI state(s) associated to the i-th codepoint are absent. - Pi: This field indicates whether each TCI codepoint has multiple TCI states or a single TCI state. If the Pi field is set to 1, the ith TCI codepoint includes the DL TCI state and the UL TCI state. If the Pi field is set to 0, the ith TCI codepoint includes only the DL/joint TCI state or the UL TCI state. The codepoint to which a TCI state is mapped is determined by its ordinal position among all the TCI state ID fields; - D/U: This field indicates whether the TCI state ID in the same octet is for a joint/downlink or an uplink TCI state. If this field is set to 1, the TCI state ID in the same octet is for joint/downlink TCI state. If this field is set to 0, the TCI state ID in the same octet is for uplink TCI state; - TCI state ID: This field indicates the TCI state identified by TCI-StateId in ltm-DL-OrJointTCI-StateToAddModList or TCI-UL-StateId in ltm-UL-TCI-StatesToAddModList as specified in TS 38.331 [5]. If D/U is set to 1, 7-bits length TCI state ID i.e. TCI-StateId as specified in TS 38.331 [5] is used. If D/U is set to 0, the most significant bit of TCI state ID is considered as the reserved bit and remaining 6 bits indicate the TCI-UL-StateId as specified in TS 38.331 [5]. The maximum number of activated TCI states is 16; A TCI state is activated if its TCI state ID is present. Otherwise, it is deactivated. - R: Reserved bit, set to 0. Figure 6: Candidate Cell TCI state activation/deactivation MAC CE Example-2

Proposal 2: RAN2 determine either Option 1 or Option 2 for the revised Candidate Cell TCI States Activation/Deactivation MAC CE format.

Conclusion

Based on the discussion, we have the following proposals.

Observation 1: The current MAC CE format for Candidate Cell TCI States Activation/Deactivation MAC CE makes difficult to know the actual numbers of activated TCI codepoint(s) because there is no explicit parameter for the activated TCI codepoint in the MAC CE.

Proposal 1: Add the new explicit indication of the number of activated TCI codepoints in the Candidate Cell TCI States Activation/Deactivation MAC CE.

Proposal 2: RAN2 determine either Option 1 or Option 2 for the revised Candidate Cell TCI States Activation/Deactivation MAC CE format.

References

[1] TS 38.321	Medium Access Control (MAC) protocol specification (Release 18).

Appendix

6.1.3.76	Candidate Cell TCI States Activation/Deactivation MAC CE

The Candidate Cell TCI States Activation/Deactivation MAC CE is identified by a MAC subheader with eLCID as specified in Table 6.2.1-1b. It has a variable size consisting of following fields:

-	Candidate Cell ID: This field indicates the identity of an LTM candidate cell for which the MAC CE applies, corresponding to the ltm-CandidateId minus 1 as specified in TS 38.331 [5]. The length of the field is 3 bits;

-	Pi: This field indicates whether each TCI codepoint has multiple TCI states or a single TCI state. If the Pi field is set to 1, the ith TCI codepoint includes the DL TCI state and the UL TCI state. If the Pi field is set to 0, the ith TCI codepoint includes only the DL/joint TCI state or the UL TCI state. The codepoint to which a TCI state is mapped is determined by its ordinal position among all the TCI state ID fields;

-	D/U: This field indicates whether the TCI state ID in the same octet is for a joint/downlink or an uplink TCI state. If this field is set to 1, the TCI state ID in the same octet is for joint/downlink TCI state. If this field is set to 0, the TCI state ID in the same octet is for uplink TCI state;

-	TCI state ID: This field indicates the TCI state identified by TCI-StateId in ltm-DL-OrJointTCI-StateToAddModList or TCI-UL-StateId in ltm-UL-TCI-StatesToAddModList as specified in TS 38.331 [5]. If D/U is set to 1, 7-bits length TCI state ID i.e. TCI-StateId as specified in TS 38.331 [5] is used. If D/U is set to 0, the most significant bit of TCI state ID is considered as the reserved bit and remaining 6 bits indicate the TCI-UL-StateId as specified in TS 38.331 [5]. The maximum number of activated TCI states is 16;

-	R: Reserved bit, set to 0.

Figure 6.1.3.76-1: Candidate Cell TCI state activation/deactivation MAC CE

- 6.2	Formats and parameters

- 6.2.1	MAC subheader for DL-SCH and UL-SCH

-- skipped

Table 6.2.1-1b: Values of one-octet eLCID for DL-SCH

| Codepoint | Index | LCID values |
| --- | --- | --- |
| 0 to 216 | 64 to 280 | Reserved |
| 217 | 281 | Enhanced SP CSI reporting on PUCCH Activation/Deactivation MAC CE |
| 218 | 282 | Cross-RRH TCI State Indication for UE-specific PDCCH MAC CE |
| 219 | 283 | LTM Cell Switch Command |
| 220 | 284 | Candidate Cell TCI States Activation/Deactivation |
| 221 | 285 | PSI-Based SDU Discard Activation/Deactivation MAC CE |
| 222 | 286 | Enhanced Unified TCI state Activation/Deactivation MAC CE for Joint TCI States |
| 223 | 287 | Enhanced Unified TCI state Activation/Deactivation MAC CE for Separate TCI States |
| 224 | 288 | NCR Access Link Beam Indication MAC CE |
| 225 | 289 | NCR Downlink Backhaul Link Beam Indication MAC CE |
| 226 | 290 | NCR Uplink Backhaul Link Beam Indication MAC CE |
| 227 | 291 | Serving Cell Set based SRS TCI State Indication MAC CE |
| 228 | 292 | SP/AP SRS TCI State Indication MAC CE |
| 229 | 293 | BFD-RS Indication MAC CE |
| 230 | 294 | Differential Koffset |
| 231 | 295 | Enhanced SCell Activation/Deactivation MAC CE with one octet Ci field |
| 232 | 296 | Enhanced SCell Activation/Deactivation MAC CE with four octet Ci field |
| 233 | 297 | Unified TCI States Activation/Deactivation MAC CE |
| 234 | 298 | PUCCH Power Control Set Update for multiple TRP PUCCH repetition MAC CE |
| 235 | 299 | PUCCH spatial relation Activation/Deactivation for multiple TRP PUCCH repetition MAC CE |
| 236 | 300 | Enhanced TCI States Indication for UE-specific PDCCH |
| 237 | 301 | Positioning Measurement Gap Activation/Deactivation Command |
| 238 | 302 | PPW Activation/Deactivation Command |
| 239 | 303 | DL Tx Power Adjustment |
| 240 | 304 | Timing Case Indication |
| 241 | 305 | Child IAB-DU Restricted Beam Indication |
| 242 | 306 | Case-7 Timing advance offset |
| 243 | 307 | Provided Guard Symbols for Case-6 timing |
| 244 | 308 | Provided Guard Symbols for Case-7 timing |
| 245 | 309 | Serving Cell Set based SRS Spatial Relation Indication |
| 246 | 310 | PUSCH Pathloss Reference RS Update |
| 247 | 311 | SRS Pathloss Reference RS Update |
| 248 | 312 | Enhanced SP/AP SRS Spatial Relation Indication |
| 249 | 313 | Enhanced PUCCH Spatial Relation Activation/Deactivation |
| 250 | 314 | Enhanced TCI States Activation/Deactivation for UE-specific PDSCH |
| 251 | 315 | Duplication RLC Activation/Deactivation |
| 252 | 316 | Absolute Timing Advance Command |
| 253 | 317 | SP Positioning SRS Activation/Deactivation |
| 254 | 318 | Provided Guard Symbols |
| 255 | 319 | Timing Delta |