3GPP TSG-RAN WG2 Meeting #125bis        	                       R2-2403024

Changsha, China, 15 - 19 April, 2024

Agenda item:	8.3.4

Source:	CMCC

Title:	Discussion on HOF and RLF prediction

WID/SID:	FS_NR_AIML_Mob

Document for:	Discussion

- Introduction

The study objectives for Rel-19 NR AI/ML for Mobility were approved in [1].

Study and evaluate potential benefits and gains of AI/ML aided mobility for network triggered L3-based handover, considering the following aspects: - AI/ML based RRM measurement and event prediction, - Cell-level measurement prediction including intra and inter-frequency (UE sided and NW sided model) [RAN2] - Inter-cell Beam-level measurement prediction for L3 Mobility (UE sided and NW sided model) [RAN2] - HO failure/RLF prediction (UE sided model) [RAN2] - Measurement events prediction (UE sided model) [RAN2] - Study the need/benefits of any other UE assistance information for the network side model [RAN2] - The evaluation of the AI/ML aided mobility benefits should consider HO performance KPIs (e.g., Ping-pong HO, HOF/RLF, Time of stay, Handover interruption, prediction accuracy, and measurement reduction) etc.) and complexity tradeoffs [RAN2] - NOTE: Simulation assumption and methodology can leverage TR 38.901, 38.843 and 36.839. And leave the detail discussion to RAN2 - Potential AI mobility specific enhancement should be based on the Rel19 AI/ML-air interface WID general framework (e.g. LCM, performance monitoring etc) [RAN2] - NOTE: This would only be treated after sufficient progress is made in the Rel-19 AI/ML air interface WID - Potential specification impacts of AI/ML aided mobility [RAN2] - Evaluate testability, interoperability, and impacts on RRM requirements and performance [RAN4] - NOTE 1: RAN1/3 work can be triggered via LS - NOTE 2: RAN4 scope/work can be defined and confirmed by RAN#105 after some RAN2 discussions (within the RAN4 pre-allocated TUs) NOTE 3: To avoid duplicate study with “AI/ML for NG-RAN” led by RAN3 NOTE 4: Two-sided model is not included

In this contribution, we will discuss on HOF and RLF specific sub use cases and relevant performance metrics/KPIs to evaluate.

- Discussion

- 2.1 Potential benefits for HOF/RLF prediction

- In legacy mobility procedure, Handover failure can include the following events as specified in TS 38.300 [2]. It can be observed that the HO failure is related to RLF and RLF can occur in the source cell or target cell.

-	Too Late Handover: an RLF occurs after the UE has stayed for a long period of time in the cell; the UE attempts to re-establish the radio link connection in a different cell.

-	Too Early Handover: an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in the source cell.

-	Handover to Wrong Cell: an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts to re-establish the radio link connection in a cell other than the source cell and the target cell.

- Observation 1: RLF can occur in the source cell or target cell during the handover procedure.

- Based on SID, handover decision is always made in network side. Therefore, in our understanding, the predicted HOF/RLF can be used for NW to decide the optimal time and best target cell for handover to avoid the unintended events mentioned above. For example,

-	If the UE predicts that the RLF will occur at a certain time in the source cell, the UE can report the information to gNB, and gNB can trigger the handover procedure before the RLF, which will reduce the occurrence of “Too Late Handover”.

-  Similarly, if the UE predicts that the RLF will occur at a certain time in the target cell, the UE can report the information to gNB, so that gNB can trigger the handover procedure earlier or handover the UE to other target cell, which will reduce the occurrence of “Too Early Handover” and “Handover to Wrong Cell”.

- Figure 1. Basic flow for HOF/RLF prediction

- Proposal 1: RAN2 study the potential benefits of HOF/RLF prediction, such as used for NW to decide the optimal time and suitable target cell for handover to avoid the unintended events (e.g. too early/late HO, HO to wrong cell).

- 2.2 Potential sub-use cases for HOF/RLF prediction

- As specified in TS 38.300[2], the RLF is declared by the UE when one of the following criteria are met:

-	Expiry of a radio problem timer started after indication of radio problems from the physical layer (if radio problems are recovered before the timer is expired, the UE stops the timer);

-	Expiry of a timer started upon triggering a measurement report for a measurement identity for which the timer has been configured while another radio problem timer is running;

-	Random access procedure failure;

-	RLC failure;

- Detection of consistent uplink LBT failures for operation with shared spectrum channel access as described in 5.6.1;

- In general, we think the main cause of RLF is the radio link quality becomes poor. And the RRM measurement is usually used to reflect the radio link quality. Therefore, the RLF can be derived by RRM measurement. Furthermore, the HOF/RLF prediction can be derived based on predicted RRM measurement.

- On the other hand, the occurrence of HOF/RLF could be predicted by measurements directly.

- Based on analysis above, we think there are two potential sub-use cases for HOF/RLF prediction with UE-sided model.

- - Sub-use case 1 (Intermediate way): The UE predict RRM measurement firstly, and derive HOF/RLF based on predicted RRM measurement

- - Sub-use case 2 (Direct way): The UE predict HOF/RLF directly based on input

- Case 1

- Case 2

- Figure 2. Potential sub-use cases for HOF/RLF prediction

- For input and output of case 1, the studies for RRM measurement prediction can be reused. How to derive HOF/RLF is up to UE implementation.

- For input and output of case 2, RAN2 should further study. For example, the input can be L1/L3 measurement, historic HOF/RLF, etc. The output can be whether the RLF will occur and if so, the time info of occurrence, etc.

- Proposal 2: RAN2 further study the potential sub-use cases for HOF/RLF prediction.

- - Sub-use case 1 (Intermediate way): The UE predicts RRM measurement, and derive HOF/RLF based on predicted RRM measurement

- - Sub-use case 2 (Direct way): The UE predicts HOF/RLF directly based on input

- 2.2 Performance metrics/KPIs

- As defined in TR 36.839 [3], the handover procedure is divided into 3 states as shown in Figure 5.2.1.3.1.

- State 1: Before the event A3 entering condition is satisfied;

- State 2: After the event A3 entering condition is satisfied but before the handover command is successfully received by the UE; and

- State 3: After the handover command is received by the UE, but before the handover complete is successfully sent by the UE

Figure 5.2.1.3.1: A handover failure is declared when the criterion 1) is met in state 2.

Figure 5.2.1.3.2: A handover failure is declared when the criterion 2) is met in state 2.

- Furthermore, the definition of RLF and RLF performance metric, HO failure and HO failure rate and RLF are also provided in TR 36.839 [3].

- Definition 1: The occurrence of RLF can be categorized into two distinctive states: state 1 and state 2 of the handover process.

- Definition 2: The RLF performance metric is defined as: the average number of RLF occurrences per UE per second. RLF performance in states 1 and 2 are logged separately.

- Definition 3: A handover failure is counted if a RLF occurs in state 2, or a PDCCH failure is detected in state 2 or state 3.

- Definition 4: The handover failure rate is defined as: Handover failure rate = (number of handover failures) / (Total number of handover attempts).

- We think the definitions can be reused as performance metrics for study on AI/ML aided mobility.

- Proposal 3: The definitions of RLF performance metric and HO failure rate in TR 36.839 can be reused as performance metrics for HOF/RLF prediction.

- For sub-use case 1, the performance metrics/KPIs defined in RRM measurement prediction can be reused.

- For sub-use case 2, as we analyzed above, AI/ML may predict the occurrence of HOF/RLF and timing info. Therefore, the prediction accuracy for HOF/RLF and timing info (if any) could be further considered. For the prediction accuracy of the timing info, the timing difference (e.g. RMSE) between real time and predicted time can be further studied.

Proposal 4: The following performance metrics/KPIs could be considered for HOF/RLF prediction:

- The performance metrics/KPIs defined for RRM measurement prediction can be reused for sub-use case 1

- The prediction accuracy for the occurrence of HOF/RLF

- Timing difference between timing of actual HOF/RLF and predicted HOF/RLF

# 3	Conclusion

Here are the observations and proposals for HOF/RLF prediction.

Observation 1: RLF can occur in the source cell or target cell during the handover procedure.

- Proposal 1: RAN2 study the potential benefits of HOF/RLF prediction, such as used for NW to decide the optimal time and suitable target cell for handover to avoid the unintended events (e.g. too early/late HO, HO to wrong cell).

- Proposal 2: RAN2 further study the potential sub-use cases for HOF/RLF prediction.

- - Sub-use case 1 (Intermediate way): The UE predicts RRM measurement, and derive HOF/RLF based on predicted RRM measurement

- Sub-use case 2 (Direct way): The UE predicts HOF/RLF directly based on input

- Proposal 3: The definitions of RLF performance metric and HO failure rate in TR 36.839 can be reused as performance metrics for HOF/RLF prediction.

Proposal 4: The following performance metrics/KPIs could be considered for HOF/RLF prediction:

- The performance metrics/KPIs defined for RRM measurement prediction can be reused for sub-use case 1

- The prediction accuracy for the occurrence of HOF/RLF

- Timing difference between timing of actual HOF/RLF and predicted HOF/RLF

# 4	Reference

- RP-234039 New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- TS 38.300 NR; NR and NG-RAN Overall Description; Stage 2

- TR 36.839 Evolved Universal Terrestrial Radio Access (E-UTRA); Mobility enhancements in heterogeneous networks