3GPP TSG-RAN WG2 #125bis	R2-2403179

# Changsha, China, 15th – 19th April 2024

Agenda Item:	7.4.4

Source:	Ericsson

Title:	Adding SR resources within the LTM cell switch MAC CE

Document for:	Discussion

# 1	Introduction

In this contribution we propose to add SR resources within the LTM cell switch MAC CE. This is because when an LTM cell switch is done, the UE may rely on a configured grant or dynamic grant to send the RRCReconfigurationComplete message. However, if a dynamic grant is used, how fast the UE can get a grant from the network is also linked how fast the UE can send an SR to the network.

# 2	Discussion

According to what has been agreed during Rel-18 for LTM, upon performing an LTM cell switch, a UE needs to send an RRCReconfigurationComplete message. This message can be sent either using a configured grant which is part of the LTM candidate cell configuration applied, or by using a dynamic grant. For the latter case, the UE would send an SR to the network which in turn replies with a dynamic grant over PDCCH.

Upon performing an LTM cell switch, a UE sends the RRCReconfigurationComplete message either using a configured grant, which is part of the LTM candidate cell configuration applied, or by using a dynamic grant (which implies sending an SR first).

Since LTM is about performance, ideally one would like the UE to send the SR request as fast as possible so to not delay further the LTM cell switch procedure. However, providing an SR periodicity within an LTM candidate cell configuration which is quite short it may not be always feasible for the network, as the resources which can be pre-reserved are limited.

In order to not delay the LTM cell switch procedure, the SR periodicity for sending the SR request should be as short as possible.

Providing a short SR periodicity within an LTM candidate cell configuration may not be always feasible for the network as resources which can be reserved for LTM (or in general) are limited.

According to TS 38.331, the SR periodicity is configured within the SchedulingRequestResourceConfig IE:

-- ASN1START

-- TAG-SCHEDULINGREQUESTRESOURCECONFIG-START

SchedulingRequestResourceConfig ::=     SEQUENCE {

schedulingRequestResourceId             SchedulingRequestResourceId,

schedulingRequestID                     SchedulingRequestId,

periodicityAndOffset                    CHOICE {

sym2                                    NULL,

sym6or7                                 NULL,

sl1                                     NULL,                       -- Recurs in every slot

sl2                                     INTEGER (0..1),

sl4                                     INTEGER (0..3),

sl5                                     INTEGER (0..4),

sl8                                     INTEGER (0..7),

sl10                                    INTEGER (0..9),

sl16                                    INTEGER (0..15),

sl20                                    INTEGER (0..19),

sl40                                    INTEGER (0..39),

sl80                                    INTEGER (0..79),

sl160                                   INTEGER (0..159),

sl320                                   INTEGER (0..319),

sl640                                   INTEGER (0..639)

}                                                                                                       OPTIONAL,   -- Need M

resource                                PUCCH-ResourceId                                                OPTIONAL    -- Need M

}

SchedulingRequestResourceConfigExt-v1610 ::=   SEQUENCE {

phy-PriorityIndex-r16                       ENUMERATED {p0, p1}                                         OPTIONAL,   -- Need M

...

}

SchedulingRequestResourceConfigExt-v1700 ::=    SEQUENCE {

periodicityAndOffset-r17                        CHOICE {

sl1280                                          INTEGER (0..1279),

sl2560                                          INTEGER (0..2559),

sl5120                                          INTEGER (0..5119)

}                                                                                                       OPTIONAL   -- Need M

}

-- TAG-SCHEDULINGREQUESTRESOURCECONFIG-STOP

-- ASN1STOP

Further, it is possible to identify a configuration which include a certain periodicity and slot offset according to the field schedulingRequestResourceId. Since the network has the possibility to provide multiple scheduling request configurations (and resources) within an RRC reconfiguration, a possible way for the network to indicate to the UE which SR resources to use upon an LTM cell switch procedure is to indicate those directly in the LTM cell switch command.

With current RRC signalling, the network may provide a list of scheduling configurations and scheduling resources but cannot indicate to the UE which ones to use upon an LTM cell switch procedure.

Therefore, considering that it is always challenging for the network to do a resource reservation, and considering that this resource reservation problem is more severe when the number of configured candidate cell increases (not only LTM, but any sort of candidate cell, such as CHO), the easiest way would be to indicate the schedulingRequestResourceId in the LTM cell switch MAC CE. In this way, the network may always provide a very short SR periodicity to the UE so to send the SR request as fast as possible without delaying further the LTM cell switch procedure. Therefore, we propose:

The network indicates the schedulingRequestResourceId within the LTM cell switch MAC CE.

# Conclusion

In the previous sections we made the following observations:

Observation 1	Upon performing an LTM cell switch, a UE sends the RRCReconfigurationComplete message either using a configured grant, which is part of the LTM candidate cell configuration applied, or by using a dynamic grant (which implies sending an SR first).

Observation 2	In order to not delay the LTM cell switch procedure, the SR periodicity for sending the SR request should be as short as possible.

Observation 3	Providing a short SR periodicity within an LTM candidate cell configuration may not be always feasible for the network as resources which can be reserved for LTM (or in general) are limited.

Observation 4	With current RRC signalling, the network may provide a list of scheduling configurations and scheduling resources but cannot indicate to the UE which ones to use upon an LTM cell switch procedure.

Based on the discussion in the previous sections we propose the following:

Proposal 1	The network indicates the schedulingRequestResourceId within the LTM cell switch MAC CE.

# References