3GPP TSG-RAN WG2 Meeting #125bis	R2-2403389

# Changsha, China, 15-19 April 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 4734 | rev | - | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification to SIB2 processing when SUL is not configured on serving cell |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2024-04-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Consider the case when the serving cell doesn’t have SUL configured or UE doesn’t meet the rest of requirements in Section 5.2.2.4.2 to consider SUL as configured in the serving cell. However, it is possible that a neighbour cell has SUL configured and UE meets the requirements to support transmission in one of the configured SUL band on that cell. In future, UE could reselect to this cell when it satisfies the S criteria based on “q-RxLevMinSUL” advertised in SIB2. - But the above is not possible, because as per current procedural text in Section 5.2.2.4.3, upon receiving SIB2, UE would skip the SUL related sub-clause because it requires that UE selected a SUL band in serving cell. - As a result, UE would not be able reselect to a neighbour cell with SUL configured that UE can support, if UE fails to satisfy the S criteria for NUL but satisfies the S criteria for SUL. This increases the probability of RLF being triggered at cell edge due to UE not being able to do cell reselection to neighbouring cells with SUL configured. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the SUL related procedural description in Section 5.2.2.4.3 with else clause to handle the case where UE did not select a SUL frequency band in serving cell. Impact analysis When UE did not select SUL in serving cell, the proposal allows UE to still be able to reselect to neighbour cell with SUL configured when S criteria based on “q-RxLevMinSUL” is satisfied but S criteria based on “q-RxLevMin” is not satisfied. Impacted 5G architecture options: NR SA, NR-DC Impacted functionality: Cell reselection Inter-operability analysis: If the network is implemented according to this CR while the UE is not, the UE may not reselect to a neighbor cell with SUL when its current serving cell is NUL. If the UE is implemented according to this CR while the network is not, there is no inter-operability issues. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE would not be able reselect to a neighbour cell with SUL configured that UE can support, if UE fails to satisfy the S criteria for NUL but satisfies the S criteria for SUL. This increases the probability of RLF being triggered at cell edge due to UE not being able to do cell reselection to neighbouring cells with SUL configured. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR …CR … |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

##### 5.2.2.4.3	Actions upon reception of SIB2

Upon receiving SIB2, the UE shall:

1>	if in RRC_IDLE or in RRC_INACTIVE or in RRC_CONNECTED while T311 is running:

2>	if, for the entry in frequencyBandList with the same index as the frequency band selected in clause 5.2.2.4.2, the UE supports at least one additionalSpectrumEmission in the NR-NS-PmaxList within the frequencyBandList:

3>	apply the first listed additionalSpectrumEmission which it supports among the values included in NR-NS-PmaxList within frequencyBandList;

3>	if the additionalPmax is present in the same entry of the selected additionalSpectrumEmission within NR-NS-PmaxList:

4>	apply the additionalPmax;

3>	else:

4>	apply the p-Max;

3>	if the UE selects a frequency band (from the procedure in clause 5.2.2.4.2) for the supplementary uplink:

4>	if, for the entry in frequencyBandListSUL with the same index as the frequency band selected in clause 5.2.2.4.2, the UE supports at least one additionalSpectrumEmission in the NR-NS-PmaxList within the frequencyBandListSUL:

5>	apply the first listed additionalSpectrumEmission which it supports among the values included in NR-NS-PmaxList within frequencyBandListSUL;

5>	if the additionalPmax is present in the same entry of the selected additionalSpectrumEmission within NR-NS-PmaxList:

6>	apply the additionalPmax;

5>	else:

6>	apply the p-Max;

4>	else:

5>	apply the p-Max.

3> else:

4> if the UE supports one or more frequency bands indicated in the frequencyBandListSUL; and

4> if the UE supports at least one additionalSpectrumEmission in the nr-NS-PmaxList for a supported band in frequencyBandListSUL; and

4> if the UE is not a RedCap UE, or if the UE is a RedCap UE and halfDuplexRedCapAllowed is present, or if the UE is a RedCap UE and the RedCap UE supports full-duplex FDD operation on the frequency bands indicated in the frequencyBandListSUL; and

5> select the first frequency band in the frequencyBandListSUL which the UE supports and for which the UE supports at least one of the       additionalSpectrumEmission values in nr-NS-PmaxList, if present, and for RedCap UEs if the halfDuplexRedCapAllowed is not present, for which the UE supports full-duplex FDD operation;

5> apply the first listed additionalSpectrumEmission which it supports among the values included in nr-NS-PmaxList for the selected frequency band in frequencyBandListSUL;

5> if the additionalPmax is present in the same entry of the selected additionalSpectrumEmission within nr-NS-PmaxList for the selected frequency band in frequencyBandListSUL:

6> apply the additionalPmax;

5> else:

6> apply the p-Max.

4> else:

5> apply the p-Max.

2>	else:

3>	apply the p-Max;

End of CHANGE