3GPP TSG-RAN WG2 Meeting #125bis	R2-2403427

Changsha, China,  April 15th – 19th, 2024

Agenda Item:	    8.5.4

Source:	III

Title:	Discussion on adaptation of common signal/channel transmissions

Document for:	Discussion

Introduction

In RAN1#116[1] meeting, the agreements of adaption of common signal/channel for paging.

Agreement For adaptation of paging, - Study further from RAN1 perspective, techniques for adaptation of paging occasions in time-domain and achievable network energy savings - Note: Specification details for PO/PF determination and paging-related configuration/procedures to be handled by RAN2

In this contribution, adaptation of the Paging Frame (PF) is discussed.

Discussion

Adaptation of paging periodicity

In TR 38.864[2], it is shown that adaptation of PF/PO can achieve BS energy savings by 0.2%~42.3%. In RAN1#116 email discussion [3], compact PF periodicity were mentioned [4] [5].We propose a mechanism for compact PF adaptation.

In TS 38.304[6], PF formula as follows:

The UE may use Discontinuous Reception (DRX) in RRC_IDLE and RRC_INACTIVE state in order to reduce power consumption. The UE monitors one paging occasion (PO) per DRX cycle. One Paging Frame (PF) is one Radio Frame and may contain one or multiple PO(s) or starting point of a PO. The PF and PO for paging are determined by the following formulae: SFN for the PF is determined by: (SFN + PF_offset+) mod T = (T div N)*(UE_ID mod N) N: number of total paging frames in T Ns: number of paging occasions for a PF PF_offset: offset used for PF determination UE_ID: If an eDRX cycle is configured by RRC or upper layers and eDRX-Allowed is signalled in SIB1: - 5G-S-TMSI mod 4096 else: - 5G-S-TMSI mod 1024

An I-DRX cycle can be divided into M segments. All PFs can be compacted into one segment and the NES cell can sleep in other M-1 segments, as figure 1 (N=4, M=2).

(SFN + PF_offset/M) mod T = [((T/M) div N)*(UE_ID mod N)]

Figure 1: Compact PF in paging cycle

Observation 1: Compact PF is feasible for cell energy saving.

Legacy UEs are difficult to receive paging massage properly in compact PF. Therefore, legacy UEs should not be allowed to camp on the NES cell. That is, legacy idle mode UEs should be barred by NES cell following Rel-18 NES barring procedures.

Observation 2: Legacy idle mode UEs can’t support compact PF.

Proposal 1: Legacy idle mode UEs should be barred by NES cell following Rel-18 NES barring procedures.

In RAN2#121bis-e agreement [7], the UE does not transmission SPS/SR/CG during Cell DTX/DRX non-active period.

There is no agreement about common channel behavior during Cell DTX/DRX non-active period, e.g., paging channel for idle mode UE. The compact PF is one feasible method for NES UE receiving paging massage during cell DTX on-duration. Further clarification is needed for this issue:

- In R18 Cell DTX/DRX discussions, only connected mode UEs were addressed, How to extend the cell DTX/DRX mechanism for idle mode UE.

- How to align the compact PF with cell DTX on-duration for idle mode UE.

Proposal 2: Compact PF should be further studied at least in the scenario of cell DTX/DRX mechanism for idle mode UE.

Conclusions

In this contribution, the following proposals are made:

Observation 1: Compact PF is feasible for cell energy saving.

Observation 2: Legacy idle mode UEs can’t support compact PF.

Proposal 1: Legacy idle mode UEs should be barred by NES cell following Rel-18 NES barring procedures.

Proposal 2: Compact PF should be further studied at least in the scenario of cell DTX/DRX mechanism for idle mode UE.

- References

- RAN1 Chair’s Notes, RAN WG1 #116, Feb 2024.

- TR 38.864,” Study on network energy savings for NR”, Mar 2023.

- R1-2401870,” Final summary of AI 9.5.3 for R19 NES”, Ericsson, Feb 2024.

- R1-2400371,” Adaptation of common signal/channel transmissions for Network Energy Saving”, Intel, Feb 2024.

- R1-2401145,” Adaptation of common signal/channel transmissions for NES”, Ericsson, Feb 2024.

- TS 38.304 V17.2.0,” User Equipment (UE) procedures in Idle mode and RRC”, Sep 2022.

- R2-2304203,” Report from Session on NES, UAV, Rel-15-17 UP, Rel-17 Small Data, IIoT/URLLC, and RACH partitioning”, InterDigital , April 2023.