3GPP TSG-RAN2 Meeting #125bis	R2-2403474

# Changsha, China, 15th – 19th April, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 4747 | rev | - | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections for SL relay |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_relay-Core |  |  |  |  |  | Date: |  |  | 2024-4-3 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - In 5.8.3.3, the IE name “sl-RLC-ModeIndication” is not aligned with ASN.1. Actually, it should be “sl-RLC-ModeIndicationList”. Since the list may include multiple RLC modes of the associated sidelink DRBs, change the sidelink DRB with plural. - Similarly, the IE name in the field description is not right. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In 5.8.3.3, change the IE name to “sl-RLC-ModeIndication” to “sl-RLC-ModeIndicationList”. And change the sidelink DRB with plural. - In the fields description of SidelinkUEInformation, use the right IE name. Impact analysis Impacted functionality: The supporting of SL relay. Inter-operability: - If Sidelink UE implements this change and gNB does not, there is no inter-operability issue between UE and gNB. - If Sidelink TX UE implements this change, but the Sidelink RX UE is not implemented this change, there is no inter-operability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Mistakes are remaining in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.8.3.3, 6.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

5.8.3.3	Actions related to transmission of SidelinkUEInformationNR message

The UE shall set the contents of the SidelinkUEInformationNR message as follows:

1>	if the UE initiates the procedure to indicate it is (no more) interested to receive NR sidelink communication; or

1>	if the UE initiates the procedure to request (configuration/ release) of NR sidelink communication transmission resources or to report to the network that a sidelink radio link failure or sidelink RRC reconfiguration failure has been declared; or

1>	if the UE initiates the procedure to report to the network the sidelink DRX configuration for NR sidelink unicast reception; or

1>	if the UE initiates the procedure to report to the network the sidelink DRX assistance information or the sidelink DRX configuration reject information for NR sidelink unicast transmission; or

1>	if the UE initiates the procedure to report to the network the Destination Layer-2 ID and QoS profile(s) associated with its interested service(s) that sidelink DRX is applied for NR sidelink groupcast or broadcast reception; or

1>	if the UE initiates the procedure to report to the network the Destination Layer-2 ID and the sidelink DRX on/off indication for the corresponding destination for NR sidelink groupcast transmission; or

1>	if the UE initiates the procedure to indicate it is (no more) interested to receive NR sidelink discovery messages; or

1>	if the UE initiates the procedure to request (configuration/ release) of NR sidelink discovery messages transmission resources; or

1>	if the UE initiates the procedure to request (configuration/ release) of NR sidelink U2N relay communication transmission resources or report other parameters related to U2N relay operation (i.e. UE includes all concerned information, irrespective of what triggered the procedure):

2>	if SIB12 including sl-ConfigCommonNR is provided by the PCell:

3>	if configured by upper layers to receive NR sidelink communication:

4>	include sl-RxInterestedFreqList and set it to the frequency for NR sidelink communication reception;

3>	if configured by upper layers to transmit non-relay NR sidelink communication and/or to transmit NR sidelink relay communication:

4>	include sl-TxResourceReqList and set its fields (if needed) as follows for each destination for which it requests network to assign NR sidelink communication resource:

5>	set sl-DestinationIdentity to the destination identity configured by upper layer for NR sidelink communication transmission;

5>	set sl-CastType to the cast type of the associated destination identity configured by the upper layer for the NR sidelink communication transmission;

5>	set sl-RLC-ModeIndicationList to include the RLC mode(s) and optionally QoS profile(s) of the sidelink QoS flow(s) of the associated RLC mode(s), if the associated bi-directional sidelink DRB(s) have been established due to the configuration by RRCReconfigurationSidelink;

5>	set sl-QoS-InfoList to include QoS profile(s) of the sidelink QoS flow(s) of the associated destination configured by the upper layer for the NR sidelink communication transmission;

5>	set sl-TxInterestedFreqList to indicate the frequency of the associated destination for NR sidelink communication transmission;

5>	set sl-TypeTxSyncList to the current synchronization reference type used on the associated sl-TxInterestedFreqList for NR sidelink communication transmission;

5>	set sl-CapabilityInformationSidelink to include UECapabilityInformationSidelink message, if any, received from the associated peer UE;

4>	if a sidelink radio link failure or a sidelink RRC reconfiguration failure has been declared, according to clauses 5.8.9.3 and 5.8.9.1.8, respectively;

5>	include sl-FailureList and set its fields as follows for each destination for which it reports the NR sidelink communication failure:

6>	set sl-DestinationIdentity to the destination identity configured by upper layer for NR sidelink communication transmission;

6>	if the sidelink RLF is detected as specified in clause 5.8.9.3:

7>	set sl-Failure as rlf for the associated destination for the NR sidelink communication transmission;

6>	else if RRCReconfigurationFailureSidelink is received:

7>	set sl-Failure as configFailure for the associated destination for the NR sidelink communication transmission;

3>	if SIB12 includes sl-NonRelayDiscovery and if configured by upper layers to receive NR sidelink non-relay discovery messages, or if SIB12 includes sl-L2U2N-Relay and if configured by upper layers to receive NR sidelink L2 U2N relay discovery messages, or if SIB12 includes sl-L3U2N-RelayDiscovery and if configured by upper layers to receive NR sidelink L3 U2N relay discovery messages:

4>	include sl-RxInterestedFreqListDisc and set it to the frequency for NR sidelink discovery messages reception;

3>	if SIB12 includes sl-L2U2N-Relay and the UE is capable of L2 U2N remote UE:

4>	include sl-SourceIdentityRemoteUE and set it to the source identity configured by upper layer for NR sidelink L2 U2N relay communication transmission;

3>	if SIB12 includes sl-NonRelayDiscovery and if configured by upper layers to transmit NR sidelink non-relay discovery messages, or if SIB12 includes sl-L2U2N-Relay and if configured by upper layers to transmit NR sidelink L2 U2N relay discovery messages, or if SIB12 includes sl-L3U2N-RelayDiscovery and if configured by upper layers to transmit NR sidelink L3 U2N relay discovery messages:

4>	include sl-TxResourceReqListDisc and set its fields (if needed) as follows for each destination for which it requests network to assign NR sidelink discovery messages resource:

5>	set sl-DestinationIdentityDisc to the destination identity configured by upper layer for NR sidelink discovery messages transmission;

5>	if the UE is acting as L2 U2N Relay UE:

6>	set sl-SourceIdentityRelayUE to the source identity configured by upper layer for NR sidelink L2 U2N relay discovery messages transmission;

5>	set sl-CastTypeDisc to the cast type of the associated destination identity for the NR sidelink discovery messages transmission;

5>	set sl-TxInterestedFreqListDisc to indicate the frequency of the associated destination for NR sidelink discovery messages transmission;

5>	set sl-TypeTxSyncListDisc to the current synchronization reference type used on the associated sl-TxInterestedFreqListDisc for NR sidelink discovery messages transmission;

5>	set sl-DiscoveryType to the current discovery type of the associated destination identity configured by the upper layer for NR sidelink discovery messages transmission;

3>	if SIB12 includes sl-L2U2N-Relay and if configured by upper layers to transmit NR sidelink L2 U2N relay communication and the UE is acting as L2 U2N Relay UE:

4>	include sl-TxResourceReqL2U2N-Relay in sl-TxResourceReqListCommRelay and set its fields (if needed) as follows for each destination for which it requests network to assign NR sidelink L2 U2N relay communication resource:

5>	set sl-DestinationIdentityL2U2N to the destination identity configured by upper layer for NR sidelink L2 U2N relay communication transmission;

5>	set sl-TxInterestedFreqListL2U2N to indicate the frequency of the associated destination for NR sidelink L2 U2N relay communication transmission;

5>	set sl-TypeTxSyncListL2U2N to the current synchronization reference type used on the associated sl-TxInterestedFreqListL2U2N for NR sidelink L2 U2N relay communication transmission;

5>	set sl-LocalID-Request to request local ID for L2 U2N Remote UE transiting to RRC_CONNECTED or in RRC_CONNECTED state;

5>	set sl-PagingIdentityRemoteUE to the paging UE ID received from peer L2 U2N Remote UE, if it is not released as in 5.8.9.8.3;

5>	set sl-CapabilityInformationSidelink to include UECapabilityInformationSidelink message, if any, received from peer UE;

4>	include ue-Type and set it to relayUE;

3>	if SIB12 includes sl-L2U2N-Relay and if configured by upper layers to transmit NR sidelink L2 U2N relay communication and the UE has a selected L2 U2N Relay UE:

4>	include sl-TxResourceReqL2U2N-Relay in sl-TxResourceReqListCommRelay and set its fields (if needed) as follows to request network to assign NR sidelink L2 U2N relay communication resource:

5>	set sl-TxInterestedFreqListL2U2N to indicate the frequency of the associated destination for NR sidelink L2 U2N relay communication transmission;

5>	set sl-TypeTxSyncListL2U2N to the current synchronization reference type used on the associated sl-TxInterestedFreqListL2U2N for NR sidelink L2 U2N relay communication transmission;

5>	set sl-CapabilityInformationSidelink to include UECapabilityInformationSidelink message, if any, received from peer UE;

4>	include ue-Type and set it to remoteUE;

3>	if SIB12 includes sl-L3U2N-RelayDiscovery and if configured by upper layers to transmit NR sidelink L3 U2N relay communication:

4>	include sl-TxResourceReqL3U2N-Relay in sl-TxResourceReqListCommRelay and set its fields (if needed) as follows for each destination for which it requests network to assign NR sidelink L3 U2N relay communication resource:

5>	set sl-DestinationIdentity to the destination identity configured by upper layer for NR sidelink L3 U2N relay communication transmission;

5>	set sl-CastType to the cast type of the associated destination identity configured by the upper layer for the NR sidelink L3 U2N relay communication transmission;

5>	set sl-RLC-ModeIndicationList to include the RLC mode(s) and optionally QoS profile(s) of the sidelink QoS flow(s) of the associated RLC mode(s), if the associated bi-directional sidelink DRB(s) have been established due to the configuration by RRCReconfigurationSidelink;

5>	set sl-QoS-InfoList to include QoS profile(s) of the sidelink QoS flow(s) of the associated destination configured by the upper layer for the NR sidelink L3 U2N relay communication transmission;

5>	set sl-TxInterestedFreqList to indicate the frequency of the associated destination for NR sidelink L3 U2N relay communication transmission;

5>	set sl-TypeTxSyncList to the current synchronization reference type used on the associated sl-TxInterestedFreqList for NR sidelink L3 U2N relay communication transmission;

5>	set sl-CapabilityInformationSidelink to include UECapabilityInformationSidelink message, if any, received from peer UE;

4>	include ue-Type and set it to relayUE if the UE is acting as NR sidelink L3 U2N Relay UE or to remoteUE otherwise;

3>	if sl-DRX-ConfigCommonGC-BC is included in SIB12-IEs:

4>	if configured by upper layers to perform NR sidelink reception:

5>	include sl-RxDRX-ReportList and set its fields (if needed) as follows for each destination for which it reports to network:

6>	set sl-DRX-ConfigFromTx to include the accepted sidelink DRX configuration of the associated destination for NR sidelink unicast communication, if received from the associated peer UE;

5>	include sl-RxInterestedGC-BC-DestList and set its fields (if needed) as follows for each Destination Layer-2 ID for which it reports to network:

6>	set sl-RxInterestedQoS-InfoList to include the QoS profile of its interested service(s) that sidelink DRX is applied for the associated destination for NR sidelink groupcast or broadcast reception;

NOTE 1:	It is up to UE implementation to set the QoS profile in sl-RxInterestedQoS-InfoList for reception of NR sidelink discovery message or ProSe Direct Link Establishment Request message as described in TS 24.554 [72], or for reception of Direct Link Establishment Request message as described in TS 24.587 [57].

6>	set sl-DestinationIdentity to the associated destination identity configured by upper layer for NR sidelink groupcast or broadcast reception;

4>	if configured by upper layers to perform NR sidelink transmission and configured with sl-ScheduledConfig:

5>	include sl-TxResourceReqList and/or sl-TxResourceReqListCommRelay and/or sl-FailureList and set its fields (if needed) as follows for each destination for which it reports to network:

6>	set sl-DRX-InfoFromRxList to include the sidelink DRX assistance information of the associated destination, if any, received from the associated peer UE;

6>	if the RRCReconfigurationCompleteSidelink message includes the sl-DRX-ConfigReject:

7>	set sl-Failure as drxReject-v1710 for the associated destination for the NR sidelink communication transmission;

6>	set sl-DRX-Indication to include the sidelink DRX on/off indication for the associated destination for NR sidelink groupcast transmission;

1>	if the UE initiates the procedure while connected to an E-UTRA PCell:

2>	submit the SidelinkUEInformationNR to lower layers via SRB1, embedded in E-UTRA RRC message ULInformationTransferIRAT as specified in TS 36.331 [10], clause 5.6.28;

1>	else:

2>	submit the SidelinkUEInformationNR message to lower layers for transmission.

NOTE 2:	When multiple lists are reported in SidelinkUEInformationNR, a UE can report up to maxNrofSL-Dest-r16 SL destinations in sl-TxResourceReqList, sl-TxResourceReqListDisc and sl-TxResourceReqListCommRelay in total.

NEXT CHANGE

–	SidelinkUEInformationNR

The SidelinkUEinformationNR message is used for the indication of NR sidelink UE information to the network.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to Network

SidelinkUEInformationNR message

-- ASN1START

-- TAG-SIDELINKUEINFORMATIONNR-START

SidelinkUEInformationNR-r16::=         SEQUENCE {

criticalExtensions                  CHOICE {

sidelinkUEInformationNR-r16         SidelinkUEInformationNR-r16-IEs,

criticalExtensionsFuture            SEQUENCE {}

}

}

SidelinkUEInformationNR-r16-IEs ::=    SEQUENCE {

sl-RxInterestedFreqList-r16            SL-InterestedFreqList-r16           OPTIONAL,

sl-TxResourceReqList-r16               SL-TxResourceReqList-r16            OPTIONAL,

sl-FailureList-r16                     SL-FailureList-r16                  OPTIONAL,

lateNonCriticalExtension               OCTET STRING                        OPTIONAL,

nonCriticalExtension                   SidelinkUEInformationNR-v1700-IEs   OPTIONAL

}

SidelinkUEInformationNR-v1700-IEs ::=  SEQUENCE {

sl-TxResourceReqList-v1700             SL-TxResourceReqList-v1700                                                 OPTIONAL,

sl-RxDRX-ReportList-v1700              SL-RxDRX-ReportList-v1700                                                  OPTIONAL,

sl-RxInterestedGC-BC-DestList-r17      SL-RxInterestedGC-BC-DestList-r17                                          OPTIONAL,

sl-RxInterestedFreqListDisc-r17        SL-InterestedFreqList-r16                                                  OPTIONAL,

sl-TxResourceReqListDisc-r17           SL-TxResourceReqListDisc-r17                                               OPTIONAL,

sl-TxResourceReqListCommRelay-r17      SL-TxResourceReqListCommRelay-r17                                          OPTIONAL,

ue-Type-r17                            ENUMERATED {relayUE, remoteUE}                                             OPTIONAL,

sl-SourceIdentityRemoteUE-r17          SL-SourceIdentity-r17                                                      OPTIONAL,

nonCriticalExtension                   SEQUENCE {}                                                                OPTIONAL

}

SL-InterestedFreqList-r16 ::=          SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF INTEGER (1..maxNrofFreqSL-r16)

SL-TxResourceReqList-r16 ::=           SEQUENCE (SIZE (1..maxNrofSL-Dest-r16)) OF SL-TxResourceReq-r16

SL-TxResourceReq-r16 ::=               SEQUENCE {

sl-DestinationIdentity-r16             SL-DestinationIdentity-r16,

sl-CastType-r16                        ENUMERATED {broadcast, groupcast, unicast, spare1},

sl-RLC-ModeIndicationList-r16          SEQUENCE (SIZE (1.. maxNrofSLRB-r16)) OF SL-RLC-ModeIndication-r16         OPTIONAL,

sl-QoS-InfoList-r16                    SEQUENCE (SIZE (1..maxNrofSL-QFIsPerDest-r16)) OF SL-QoS-Info-r16          OPTIONAL,

sl-TypeTxSyncList-r16                  SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF SL-TypeTxSync-r16                OPTIONAL,

sl-TxInterestedFreqList-r16            SL-TxInterestedFreqList-r16                                                OPTIONAL,

sl-CapabilityInformationSidelink-r16   OCTET STRING                                                               OPTIONAL

}

SL-TxResourceReqList-v1700 ::=         SEQUENCE (SIZE (1..maxNrofSL-Dest-r16)) OF SL-TxResourceReq-v1700

SL-RxDRX-ReportList-v1700 ::=          SEQUENCE (SIZE (1..maxNrofSL-Dest-r16)) OF SL-RxDRX-Report-v1700

SL-TxResourceReq-v1700 ::=             SEQUENCE {

sl-DRX-InfoFromRxList-r17              SEQUENCE (SIZE (1..maxNrofSL-RxInfoSet-r17)) OF SL-DRX-ConfigUC-SemiStatic-r17   OPTIONAL,

sl-DRX-Indication-r17                  ENUMERATED {on, off}                                                             OPTIONAL,

...

}

SL-RxDRX-Report-v1700 ::=              SEQUENCE {

sl-DRX-ConfigFromTx-r17                SL-DRX-ConfigUC-SemiStatic-r17,

...}

SL-RxInterestedGC-BC-DestList-r17 ::=  SEQUENCE (SIZE (1..maxNrofSL-Dest-r16)) OF SL-RxInterestedGC-BC-Dest-r17

SL-RxInterestedGC-BC-Dest-r17 ::=      SEQUENCE {

sl-RxInterestedQoS-InfoList-r17        SEQUENCE (SIZE (1..maxNrofSL-QFIsPerDest-r16)) OF SL-QoS-Info-r16,

sl-DestinationIdentity-r16             SL-DestinationIdentity-r16

}

SL-TxResourceReqListDisc-r17 ::=       SEQUENCE (SIZE (1..maxNrofSL-Dest-r16)) OF SL-TxResourceReqDisc-r17

SL-TxResourceReqDisc-r17 ::=           SEQUENCE {

sl-DestinationIdentityDisc-r17         SL-DestinationIdentity-r16,

sl-SourceIdentityRelayUE-r17           SL-SourceIdentity-r17                                                      OPTIONAL,

sl-CastTypeDisc-r17                    ENUMERATED {broadcast, groupcast, unicast, spare1},

sl-TxInterestedFreqListDisc-r17        SL-TxInterestedFreqList-r16,

sl-TypeTxSyncListDisc-r17              SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF SL-TypeTxSync-r16,

sl-DiscoveryType-r17                   ENUMERATED {relay, non-Relay},

...

}

SL-TxResourceReqListCommRelay-r17 ::=  SEQUENCE (SIZE (1..maxNrofSL-Dest-r16)) OF SL-TxResourceReqCommRelayInfo-r17

SL-TxResourceReqCommRelayInfo-r17 ::=  SEQUENCE {

sl-RelayDRXConfig-r17                 SL-TxResourceReq-v1700                                                       OPTIONAL,

sl-TxResourceReqCommRelay-r17         SL-TxResourceReqCommRelay-r17

}

SL-TxResourceReqCommRelay-r17 ::=      CHOICE {

sl-TxResourceReqL2U2N-Relay-r17        SL-TxResourceReqL2U2N-Relay-r17,

sl-TxResourceReqL3U2N-Relay-r17        SL-TxResourceReq-r16

}

SL-TxResourceReqL2U2N-Relay-r17 ::=    SEQUENCE {

sl-DestinationIdentityL2U2N-r17        SL-DestinationIdentity-r16                                                 OPTIONAL,

sl-TxInterestedFreqListL2U2N-r17       SL-TxInterestedFreqList-r16,

sl-TypeTxSyncListL2U2N-r17             SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF SL-TypeTxSync-r16,

sl-LocalID-Request-r17                 ENUMERATED {true}                                                          OPTIONAL,

sl-PagingIdentityRemoteUE-r17          SL-PagingIdentityRemoteUE-r17                                              OPTIONAL,

sl-CapabilityInformationSidelink-r17   OCTET STRING                                                               OPTIONAL,

...

}

SL-TxInterestedFreqList-r16 ::=        SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF INTEGER (1..maxNrofFreqSL-r16)

SL-QoS-Info-r16 ::=                    SEQUENCE {

sl-QoS-FlowIdentity-r16               SL-QoS-FlowIdentity-r16,

sl-QoS-Profile-r16                    SL-QoS-Profile-r16                                                          OPTIONAL

}

SL-RLC-ModeIndication-r16 ::=          SEQUENCE {

sl-Mode-r16                            CHOICE  {

sl-AM-Mode-r16                         NULL,

sl-UM-Mode-r16                         NULL

},

sl-QoS-InfoList-r16                SEQUENCE (SIZE (1..maxNrofSL-QFIsPerDest-r16)) OF SL-QoS-Info-r16

}

SL-FailureList-r16 ::=                 SEQUENCE (SIZE (1..maxNrofSL-Dest-r16)) OF SL-Failure-r16

SL-Failure-r16 ::=                     SEQUENCE {

sl-DestinationIdentity-r16             SL-DestinationIdentity-r16,

sl-Failure-r16                         ENUMERATED {rlf,configFailure, drxReject-v1710, spare5, spare4, spare3, spare2, spare1}

}

-- TAG-SIDELINKUEINFORMATIONNR-STOP

-- ASN1STOP

SidelinkUEinformationNR field descriptions

sl-RxDRX-ReportList Indicates the accepted DRX configuration that is received from the peer UE and reported to the network for NR sidelink unicast communication.

sl-RxInterestedFreqList Indicates the index of frequency on which the UE is interested to receive NR sidelink communication. The value 1 corresponds to the frequency of first entry in sl-FreqInfoList broadcast in SIB12, the value 2 corresponds to the frequency of second entry in sl-FreqInfoList broadcast in SIB12 and so on. In this release, only value 1 can be included in the interested frequency list.

sl-RxInterestedGC-BC-DestList Indicates the reported QoS profile and associated destination for which UE is interested in reception to the network for NR sidelink groupcast and broadcast communication, or for NR sidelink discovery or ProSe Direct Link Establishment Request as described in TS 24.554 [72], or for Direct Link Establishment Request (TS 24.587 [57]).

sl-SourceIdentityRemoteUE This field is used to indicate the Source Layer-2 ID to be used to establish PC5 link with the target L2 U2N Relay UE for path switch.

sl-TxResourceReq Parameters to request the transmission resources for NR sidelink communication to the network in the Sidelink UE Information report.

sl-TxResourceReqList List of parameters to request the transmission resources for NR sidelink communication for the associated destination. If sl-TxResourceReqList-v1700 is present, it shall contain the same number of entries, listed in the same order as in sl-TxResourceReqList-r16.

ue-Type Indicates the UE is acting as U2N Relay UE or U2N Remote UE.

SL-TxResourceReq field descriptions

sl-CapabilityInformationSidelink Includes the UECapabilityInformationSidelink message (which can be also included in ueCapabilityInformationSidelink-r16 in UECapabilityEnquirySidelink from peer UE) received from the peer UE.

sl-CastType Indicates the cast type for the corresponding destination for which to request the resource.

sl-DestinationIdentity Indicates the destination for which the TX resource request and allocation from the network are concerned.

sl-DRX-Indication Indicates the sidelink DRX is applied (value on) or not applied (value off) for the associated destination. This field is only valid for NR sidelink groupcast communication.

sl-DRX-InfoFromRxList Indicates list of the sidelink DRX configurations as assistance information received from the peer UE for NR sidelink unicast communication.

sl-QoS-InfoList Includes the QoS profile of the sidelink QoS flow as specified in TS 23.287 [55].

sl-QoS-FlowIdentity This identity uniquely identifies one sidelink QoS flow between the UE and the network in the scope of UE, which is unique for different destination and cast type.

sl-RLC-ModeIndicationList Each entry of this field indicates the RLC mode and optionally the related QoS profiles for the sidelink radio bearer, which has not been configured by the network and is initiated by another UE in unicast. The RLC mode for one sidelink radio bearer is aligned between UE and NW by the sl-QoS-FlowIdentity.

sl-TxInterestedFreqList Each entry of this field indicates the index of frequency on which the UE is interested to transmit NR sidelink communication. The value 1 corresponds to the frequency of first entry in sl-FreqInfoList broadcast in SIB12, the value 2 corresponds to the frequency of second entry in sl-FreqInfoList broadcast in SIB12 and so on. In this release, only value 1 can be included in the interested frequency list. In this release, only one entry can be included in the list.

sl-TypeTxSyncList A list of synchronization reference used by the UE. The UE shall include the same number of entries, listed in the same order, as in sl-TxInterestedFreqList, i.e. one for each carrier frequency included in sl-TxInterestedFreqList.

SL-Failure field descriptions

sl-DestinationIdentity Indicates the destination for which the SL failure is reporting for unicast.

sl-Failure Indicates the sidelink cause for the sidelink RLF (value rlf), sidelink AS configuration failure (value configFailure) and the rejection of sidelink DRX configuration (value drxReject-v1710) for the associated destination for unicast.

SL-RxDRX-Report field descriptions

sl-DRX-ConfigFromTx Indicates the sidelink DRX configuration received from the peer UE for NR sidelink unicast communication.

SL-RxInterestedGC-BC-Dest field descriptions

sl-RxInterestedQoS-InfoList Indicates the QoS profile for which UE reports its interested service to which SL DRX is applied to the network, for NR sidelink groupcast or broadcast reception.

SL-TxResourceReqDisc field descriptions

sl-CastTypeDisc Indicates the cast type for the NR sidelink discovery messages. Only value broadcast can be set in this release.

sl-DestinationIdentityDisc This field is used to indicate the destination L2 ID for which the TX resource request and allocation from the network are concerned for relay discovery and non-relay discovery.

sl-SourceIdentityRelayUE This field is used to indicate the source L2 ID of relay-related discovery transmission by L2 U2N Relay UE.

sl-TxInterestedFreqListDisc Each entry of this field indicates the index of frequency on which the UE is interested to transmit NR sidelink discovery. The value 1 corresponds to the frequency of first entry in sl-FreqInfoList broadcast in SIB12, the value 2 corresponds to the frequency of second entry in sl-FreqInfoList broadcast in SIB12 and so on. In this release, only value 1 can be included in the interested frequency list. In this release, only one entry can be included in the list.

SL-TxResourceReqCommRelayInfo field descriptions

sl-RelayDRXConfig This field is used to indicate the applied sidelink DRX configuration for the relay related communication.

sl-DestinationIdentityL2U2N This field is used to indicate the destination L2 ID for which the TX resource request and allocation from the network are concerned for the established PC5 link for relay by L2 U2N Relay UE.

sl-LocalID-Request This field is used to request local UE ID for the corresponding destination by the L2 U2N Relay UE.

sl-TxInterestedFreqListL2U2N Each entry of this field indicates the index of frequency on which the UE is interested to transmit NR sidelink communication for established PC5 link for relay. The value 1 corresponds to the frequency of first entry in sl-FreqInfoList broadcast in SIB12, the value 2 corresponds to the frequency of second entry in sl-FreqInfoList broadcast in SIB12 and so on. In this release, only value 1 can be included in the interested frequency list. In this release, only one entry can be included in the list.

sl-PagingIdentityRemoteUE This field is used to indicate the paging UE ID(s) for the corresponding destination(s) by the L2 U2N Relay UE.

END OF CHNAGE