3GPP TSG-RAN WG2 Meeting #125-bis	R2-2403535

Changsha, China, April 15th – 19th, 2024

Agenda Item:	7.2.2

Source:	Ericsson

Title:	Text Proposal for Stage2 TS 38.305

Document for:	Discussion, Decision

# 1	Introduction

This paper discusses RILE101 and provides the correction needed in TS 38.305.

# 2	Discussion

## 2.1 RIL E101

During RAN2#125 RIL E101 was discussed which was to introduce conditionality when configuring the time window for measurements. That is the time window is only applicable when simultaneous measurements are configured between a PRU and target UE as per the RAN1 agreement. It was decided that the behaviour can be captured in stage2.

Agreement:

Replace the IE description for NR-DL-PRS-MeasurementTimeWindowsConfig with “The IE NR-DL-PRS-MeasurementTimeWindowsConfig provides a set of indicated time window(s) which is configured for the target device to perform measurements on indicated DL PRS resource set(s) occurring within indicated time window(s).”

Below we present a text proposal in the annex.

## 2.2 Carrier phase measurement report

Further, for carier phase measurement, it should be clearly stated that UE does not send the carrier phase measurement only; that is when UE sends carrier phase measurement (RSCP, RSCPD measurement), it also includes UE Rx-Tx, RSTD measurements.

#### 8.10.2.2	Information that may be transferred from the UE to LMF

The information that may be signalled from UE to the LMF is listed in Table 8.10.2.2-1. The individual UE measurements are defined in TS 38.215 [37].

Table 8.10.2.2-1: Measurement results that may be transferred from UE to the LMF

Information

PCI, GCI, and PRS ID, ARFCN, PRS resource ID, PRS resource set ID for each measurement

DL-PRS-RSRP measurement

UE Rx-Tx time difference measurement

DL-RSCP measurementNOTE1, NOTE2

Time stamp of the measurement

Quality for each measurement

TA offset used by UE

UE Rx TEG IDs, UE Tx TEG IDs, and UE RxTx TEG IDs associated with UE Rx-Tx time difference measurements

LOS/NLOS information for UE measurements

DL-PRS-RSRPP measurement

The association of UE Tx TEG ID and SRS

Indication that DL-PRS bandwidth aggregation has been used for UE Rx-Tx time difference measurement

Indication that the reported measurements are based on receiving single or multiple hops of DL-PRS

UE Rx – Tx time difference subframe offset

DL timing drift

NOTE 1: The DL-RSCP measurement is reported along with the UE Rx-Tx time difference measurement. NOTE 2: The DL-RSCP is measured from a single DL PRS positioning frequency layer.

Table 8.12.2.2.0-1: Measurement results that may be transferred from UE to the LMF

| Information | UE-assisted | UE-based |
| --- | --- | --- |
| Latitude/Longitude/Altitude, together with uncertainty shape | No | Yes |
| PCI, GCI, ARFCN, PRS resource ID, PRS resource set ID and PRS ID for each measurement | Yes | No |
| DL RSTD measurement | Yes | No |
| DL-PRS-RSRP measurement | Yes | No |
| DL-RSCPD measurementNOTE 1, NOTE 2 | Yes | No |
| Time stamp of the measurements | Yes | No |
| Time stamp of location estimate | No | Yes |
| Quality for each measurement | Yes | No |
| UE Rx TEG IDs for DL RSTD measurements | Yes | No |
| DL-PRS-RSRPP measurement | Yes | No |
| LOS/NLOS information for UE measurements | Yes | No |
| Indication that DL-PRS bandwidth aggregation has been used for DL RSTD measurement | Yes | No |
| Indication that the reported measurements are based on receiving single or multiple hops of DL-PRS. | Yes | No |
| Protection Level, optionally together with achievable Target Integrity Risk | No | Yes |
| NOTE 1: The DL-RSCPD measurement is reported along with the DL RSTD measurement. NOTE 2: The DL-RSCPD is measured from a single DL PRS positioning frequency layer. |  |  |

Capture the simultaneous measurement between target UE and PRU in TS 38.305 as shown in TP and correct the Note description in Table 8.10.2.2-1 and Table 8.12.2.2.0-1 from “may be” to “is”.

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	Capture the simultaneous measurement between target UE and PRU in TS 38.305 as shown in TP and correct the Note description in Table 8.10.2.2-1 and Table 8.12.2.2.0-1 from “may be” to “is”.

# 4	Text Proposal/Draft CR

Beginning of Changes

### 5.4.5	Positioning Reference Unit (PRU)

A Positioning Reference Unit (PRU) at a known location can perform DL positioning measurements (e.g., RSTD, RSRP, UE Rx-Tx Time Difference measurements, DL-RSCPD, DL-RSCP, etc.) and report these DL measurements to a location server. In addition, the PRU can transmit SRS to enable TRPs to measure and report UL positioning measurements (e.g., RTOA, UL-AoA, gNB Rx-Tx Time Difference, UL-RSCP, etc.) from PRU at a known location. The PRU measurements can be compared by a location server with the measurements expected at the known PRU location to determine correction terms for other nearby target devices. The DL- and/or UL location measurements for other target devices can then be corrected based on the previously determined correction terms. LMF may configure one or multiple measurement time window for a PRU and another target device such that the DL measurements are performed during the same time. Similarly, serving gNB may configure one or multiple time window for UL-SRS transmission during the same time for PRU and another target device.

PRU measurements may also be provided to the target device in the assistance data as described in clause 8.12.

From a location server perspective, the PRU functionality is realized by a UE with known location.

End of changes