3GPP TSG-RAN WG2 Meeting #125-bis	R2-2403538

Changsa, China,  15th – 19th April 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.305 | CR | 0159 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of LCS User Plane |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Intel Corporation, Huawei, HiSilicon, ZTE Corporation, vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2024-04-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | CT1 and SA2 group has reached to consensus and defined a new LCS User plane protocol. CT1 has decided to specify a new protocol, LCS-UPP, in 3GPP TS 24.572. The LCS-UPP protocol provides connectivity on a secure user plane connection and supports both LPP and supplementary services. The archietcure diagram needs to be updated to capture the user plane impacts. Since stage2 captures the overall architecture, it is essential to update the diagram. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add changes corresponding to the impacts of addition of LCS-UPP in RAN2 stage2 specification. Update the diagram, add the reference and describe in brief the impacts. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The user plane specification work will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.2, 5, 6.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Beginning of Changes

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501 "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 22.071: "Location Services (LCS); Service description, Stage 1".

[4]	3GPP TS 23.032: "Universal Geographical Area Description (GAD)".

[5]	IS-GPS-200, Revision D, Navstar GPS Space Segment/Navigation User Interfaces, March 7th, 2006.

[6]	IS-GPS-705, Navstar GPS Space Segment/User Segment L5 Interfaces, September 22, 2005.

[7]	IS-GPS-800, Navstar GPS Space Segment/User Segment L1C Interfaces, September 4, 2008.

[8]	Galileo OS Signal in Space ICD (OS SIS ICD), Draft 0, Galileo Joint Undertaking, May 23rd, 2006.

[9]	Global Navigation Satellite System GLONASS Interface Control Document, Version 5, 2002.

[10]	IS-QZSS, Quasi Zenith Satellite System Navigation Service Interface Specifications for QZSS, Ver.1.0, June 17, 2008.

[11]	Specification for the Wide Area Augmentation System (WAAS), US Department of Transportation, Federal Aviation Administration, DTFA01-96-C-00025, 2001.

[12]	RTCM 10402.3, RTCM Recommended Standards for Differential GNSS Service (v.2.3), August 20, 2001.

[13]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification".

[14]	3GPP TS 38.331: "NR Radio Resource Control (RRC) protocol specification".

[15]	OMA-AD-SUPL-V2_0: "Secure User Plane Location Architecture Approved Version 2.0".

[16]	OMA-TS-ULP-V2_0_6: "UserPlane Location Protocol Approved Version 2.0.6".

[17]	3GPP TS 36.214: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer – Measurements".

[18]	3GPP TS 36.302: "Evolved Universal Terrestrial Radio Access (E-UTRA); Services provided by the physical layer".

[19]	3GPP TS 36.355: "Evolved Universal Terrestrial Radio Access (E-UTRA); LTE Positioning Protocol (LPP)".

[20]	BDS-SIS-ICD-B1I-3.0: "BeiDou Navigation Satellite System Signal In Space Interface Control Document Open Service Signal B1I (Version 3.0)", February, 2019.

[21]	IEEE 802.11: "Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications"

[22]	Bluetooth Special Interest Group: "Bluetooth Core Specification v4.2", December 2014.

[23]	ATIS-0500027: "Recommendations for Establishing Wide Scale Indoor Location Performance", May 2015.

[24]	3GPP TS 36.211: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical channels and modulation".

[25]	3GPP TS 36.305: "Stage 2 functional specification of User Equipment (UE) positioning in E-UTRA".

[26]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[27]	3GPP TS 38.455: "NG-RAN; NR Positioning Protocol A (NRPPa)".

[28]	3GPP TS 29.518: "5G System; Access and Mobility Management Services; Stage 3".

[29]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[30]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[31]	RTCM 10403.3, "RTCM Recommended Standards for Differential GNSS Services (v.3.3)", October 7, 2016.

[32]	3GPP TS 38.133: "NR; Requirements for support of radio resource management".

[33]	3GPP TS 29.572: "Location Management Services; Stage 3".

[34]	BDS-SIS-ICD-B1C-1.0: "BeiDou Navigation Satellite System Signal In Space Interface Control Document Open Service Signal B1C (Version 1.0)", December, 2017

[35]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[36]	IS-QZSS-L6-001, Quasi-Zenith Satellite System Interface Specification – Centimetre Level Augmentation Service, Cabinet Office, November 5, 2018.

[37]	3GPP TS 38.215: "NR; Physical layer – Measurements".

[38]	3GPP TS 38.401: "3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NG-RAN; Architecture description".

[39]	3GPP TS 38.321: "NR; Medium Access Control (MAC) protocol specification".

[40]	3GPP TS 38.212: "NR; Multiplexing and channel coding".

[41]	3GPP TS 24.571: "Control plane Location Services (LCS) procedures".

[42]	3GPP TS 37.355: "Technical Specification Group Radio Access Network; LTE Positioning Protocol (LPP)".

[43]	IRNSS Signal-In-Space (SPS) Interface Control Document (ICD) for standard positioning service version 1.1, August 2017.

[44]	BDS-SIS-ICD-B2a-1.0: "BeiDou Navigation Satellite System Signal In Space Interface Control Document Open Service Signal B2a (Version 1.0)", December, 2017.

[45]	BDS-SIS-ICD-B3I-1.0: "BeiDou Navigation Satellite System Signal In Space Interface Control Document Open Service Signal B3I (Version 1.0)", February, 2018.

[46]	3GPP TS 23.586: "Architectural Enhancements to support Ranging based services and Sidelink Positioning".

[47]	3GPP TS 38.355: "NR; Sidelink Positioning Protocol (SLPP); Protocol specification".

[48]	3GPP TS 23.287: "Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services".

[49]	3GPP TS 23.304: "Proximity based Services (ProSe) in the 5G System (5GS)".

[50]	3GPP TS 38.211: "NR; Physical channels and modulation".

[51]	Bluetooth Special Interest Group: "Bluetooth Core Specification v5.4", February 2023.

[52]	3GPP TS 38.300: "NR; NR and NR-RAN Overall Description; Stage 2".

[xx]	3GPP TS 24.572: "User Plane Location Services (LCS) Protocols And Procedures".

Next change

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

5GC	5G Core Network

5GS	5G System

A-AoA	Azimuth-Angle of Arrival

ADR	Accumulated Delta Range

AL	Alert Limit

AoA	Angle of Arrival

AP	Access Point

APC	Antenna Phase Center

ARP	Antenna Reference Point

BDS	BeiDou Navigation Satellite System

BSSID	Basic Service Set Identifier

CID	Cell-ID (positioning method)

CLAS	Centimetre Level Augmentation Service

DL-AoD	Downlink Angle-of-Departure

DL-PRS	Downlink Positioning Reference Signal

DL-RSCP	Downlink Reference Signal Carrier Phase

DL-RSCPD	Downlink Reference Signal Carrier Phase Difference

DL-TDOA	Downlink Time Difference Of Arrival

DNU	Do Not Use

DRX	Discontinuous Reception

E-SMLC	Enhanced Serving Mobile Location Centre

E-CID	Enhanced Cell-ID (positioning method)

ECEF	Earth-Centered, Earth-Fixed

ECI	Earth-Centered-Inertial

eDRX	Extended Discontinuous Reception

EGNOS	European Geostationary Navigation Overlay Service

E-UTRAN	Evolved Universal Terrestrial Radio Access Network

FDMA	Frequency Division Multiple Access

FKP	Flächenkorrekturparameter (Engl: Area Correction Parameters)

GAGAN	GPS Aided Geo Augmented Navigation

GLONASS	GLObal'naya NAvigatsionnaya Sputnikovaya Sistema (Engl.: Global Navigation Satellite System)

GMLC	Gateway Mobile Location Centre

GNSS	Global Navigation Satellite System

GPS	Global Positioning System

GRS80	Geodetic Reference System 1980

HESSID	Homogeneous Extended Service Set Identifier

IOD	Issue of Data

LCS	LoCation Services

LCS-UPP	Location Services User Plane Protocol

LMF	Location Management Function

LPP	LTE Positioning Protocol

MAC	Master Auxiliary Concept

MBS	Metropolitan Beacon System

MO-LR	Mobile Originated Location Request

MT-LR	Mobile Terminated Location Request

Multi-RTT	Multi-Round Trip Time

NavIC	NAVigation with Indian Constellation

NG-C	NG Control plane

NG-AP	NG Application Protocol

NI-LR	Network Induced Location Request

N-RTK	Network – Real-Time Kinematic

NRPPa	NR Positioning Protocol A

NTN	Non-Terrestrial Network

OTDOA	Observed Time Difference Of Arrival

PCO	Phase Center Offset

PCV	Phase Center Variation

PDU	Protocol Data Unit

posSI	Positioning System Information

posSIB	Positioning SIB

PPP	Precise Point Positioning

PPP-RTK	Precise Point Positioning – Real-Time Kinematic

PRS	Positioning Reference Signal (for E-UTRA)

PRU	Positioning Reference Unit

QZSS	Quasi-Zenith Satellite System

RP	Reception Point

RRM	Radio Resource Management

RSPP	Ranging/SL Positioning Protocol

RSRP	Reference Signal Received Power

RSRPP	Reference Signal Received Path Power

RSRQ	Reference Signal Received Quality

RSSI	Received Signal Strength Indicator

RSTD	Reference Signal Time Difference

RTK	Real-Time Kinematic

SBAS	Space Based Augmentation System

SDT	Small Data Transmission

SET	SUPL Enabled Terminal

SIB	System Information Block

SL	Sidelink

SL-PRS	Sidelink Positioning Reference Signal

SL-PRS-RSRP	Sidelink PRS Reference Signal Received Power

SL-PRS-RSRPP	Sidelink PRS Reference Signal Received Path Power

SL-RSTD	Sidelink Reference Signal Time Difference

SL-RTOA	Sidelink Relative Time of Arrival

SLP	SUPL Location Platform

SLPP	Sidelink Positioning Protocol

SP	Semi-Persistent

SRS	Sounding Reference Signal

SSB	Synchronization Signal Block

SSID	Service Set Identifier

SSR	State Space Representation

STEC	Slant TEC

SUPL	Secure User Plane Location

TADV	Timing Advance

TBS	Terrestrial Beacon System

TEC	Total Electron Content

TEG	Timing Error Group

TP	Transmission Point

TRP	Transmission-Reception Point

TTA	Time To Alert

TxTEG	Tx Timing Error Group

UE	User Equipment

UL-AoA	Uplink Angle of Arrival

UL-RSCP	Uplink Reference Signal Carrier Phase

UL-RTOA	Uplink Relative Time of Arrival

UL-SRS	Uplink Sounding Reference Signal

UL-TDOA	Uplink Time Difference of Arrival

UPF	User Plane Function

URA	User Range Accuracy

WAAS	Wide Area Augmentation System

WGS-84	World Geodetic System 1984

WLAN	Wireless Local Area Network

Z-AoA	Zenith Angles of Arrival

Next change

# 5	NG-RAN UE Positioning Architecture

## 5.1	Architecture

Figure 5.1-1 shows the architecture in 5GS applicable to positioning of a UE with NR or E-UTRA access. The positioning architecture also supports the NR PC5 interface as illustrated in Figure 5.1-1. Sidelink positioning can be supported when the UE is inside NG-RAN coverage (UE A and UE B in Figure 5.1-1) and when the UE is outside NG-RAN coverage (UE C and UE D in Figure 5.1-1).

NOTE:	Sidelink positioning for UEs in partial coverage is not supported in this Release of the specification.

The AMF receives a request for some location service associated with a particular target UE from another entity (e.g., GMLC or UE) or the AMF itself decides to initiate some location service on behalf of a particular target UE (e.g., for an IMS emergency call from the UE) as described in TS 23.502 [26] and TS 23.273 [35]. The AMF then sends a location services request to an LMF. The LMF processes the location services request which may include transferring assistance data to the target UE to assist with UE-based and/or UE-assisted positioning and/or may include positioning of the target UE. The LMF then returns the result of the location service back to the AMF (e.g., a position estimate for the UE. In the case of a location service requested by an entity other than the AMF (e.g., a GMLC or UE), the AMF returns the location service result to this entity.

An NG-RAN node may control several TRPs/TPs, such as remote radio heads, or DL-PRS-only TPs for support of PRS-based TBS.

An LMF may establish a secure user plane connection with UE via UPF to transfer supplementary services messages and LPP messages. LCS-UPP to support supplementary services messages and LPP messages transport between the UE and the LMF is defined in TS 24.572 [xx].

An LMF may have a proprietary signalling connection to an E-SMLC which may enable an LMF to access information from E-UTRAN (e.g. to support the OTDOA for E-UTRA positioning method using downlink measurements obtained by a target UE of signals from eNBs and/or PRS-only TPs in E-UTRAN). Details of the signalling interaction between an LMF and E-SMLC are outside the scope of this specification.

An LMF may have a proprietary signalling connection to an SLP. The SLP is the SUPL entity responsible for positioning over the user plane. Further details of user-plane positioning are provided in [15][16]. Details of the signalling interaction between an LMF and SLP are outside the scope of this specification.

Figure 5.1-1: UE Positioning Overall Architecture applicable to NG-RAN

NOTE 1:	Void

NOTE 2:	Void

NOTE 3:	Void

In case of split gNB architecture, a gNB-DU may include TRP functionality where the TRP functionality may support functions for a TP, RP or both TP and RP. A gNB-DU which includes TRP functionality does not need to offer cell services.

Next change

### 6.2.1	LTE Positioning Protocol (LPP)

The LTE Positioning Protocol (LPP) is terminated between a target device (the UE in the control-plane case or the UE in the 3GPP 5GS user-plane case or SET in the OMA user-plane case) and a positioning server (the LMF in the control-plane case or the LMF in the 3GPP 5GS user-plane case or SLP in the user-plane case). It may use either the control- or user-plane protocols as underlying transport. In this specification, only control plane use of LPP is defined. OMA based User plane support of LPP is defined in [15] and [16]. 3GPP 5GS based User Plane support of LPP is defined in TS 23.273 [35] and TS 24.572 [xx].

LPP messages are carried as transparent PDUs across intermediate network interfaces using the appropriate protocols (e.g., NGAP over the NG-C interface, NAS/RRC over the LTE-Uu and NR-Uu interfaces). The LPP protocol is intended to enable positioning for NR and LTE using a multiplicity of different position methods, while isolating the details of any particular positioning method and the specifics of the underlying transport from one another.

The protocol operates on a transaction basis between a target device and a server, with each transaction taking place as an independent procedure. More than one such procedure may be in progress at any given moment. An LPP procedure may involve a request/response pairing of messages or one or more "unsolicited" messages. Each procedure has a single objective (e.g., transfer of assistance data, exchange of LPP related capabilities, or positioning of a target device according to some QoS and use of one or more positioning methods). Multiple procedures, in series and/or in parallel, can be used to achieve more complex objectives (e.g., positioning of a target device in association with transfer of assistance data and exchange of LPP related capabilities). Multiple procedures also enable more than one positioning attempt to be ongoing at the same time (e.g., to obtain a coarse location estimate with low delay while a more accurate location estimate is being obtained with higher delay).

An LPP session is defined between a positioning server and the target device, the details of its relation with transactions are described in clause 4.1.2 of TS 36.355 [19].

For the 3GPP 5GS Control Plane solution defined in TS 23.501 [2], TS 23.502 [26] and TS 23.273 [35] and for 3GPP 5GS User Plane solution, the UE is the target device and the LMF is the server. For 3GPP 5GS User Plane solution, the LCS-UPP as defined in 3GPP TS 24.572 [xx] provides connectivity on a secure user plane connection and supports both LPP and supplementary services. The LMF and UE establish a PDU session used for user plane positioning via User Plane Function (UPF) as defined in TS 23.273 [35] using the NR Uu, N3 and N6 interfaces. For SUPL 2.0 support, the SUPL Enabled Terminal (SET) is the target device and the SUPL Location Platform (SLP) is the server. The operations controlled through LPP are described further in clause 7.1.

LPP defined data structures for assistance data information are reused for supporting RRC broadcast of assistance data information which are embedded in positioning SIBs. This enables broadcast assistance data using the same data structures which are used for point to point location.

End of Changes