3GPP TSG RAN WG2 #125-bis		R2-2403658

Changsha, China, April 15th – April 19th, 2024

Agenda Item:	8.1.4

Source:	Indian Institute of Technology Madras (IITM), IIT Kanpur

Title:	Discussion on UE side data collection

Document for:	Discussion

# Introduction

RAN #102 finalized the new WID for AI/ML for NR air interface [1]. RAN #103 further revised the WID [2]. The WID describes the objectives of general framework for one-sided AI/ML models as follows:

Provide specification support for the following aspects: - AI/ML general framework for one-sided AI/ML models within the realm of what has been studied in the FS_NR_AIML_Air project [RAN2]: - Signaling and protocol aspects of Life Cycle Management (LCM) enabling functionality and model (if justified) selection, activation, deactivation, switching, fallback - Identification related signaling is part of the above objective - Necessary signaling/mechanism(s) for LCM to facilitate model training, inference, performance monitoring, data collection (except for the purpose of CN/OAM/OTT collection of UE-sided model training data) for both UE-sided and NW-sided models - Signaling mechanism of applicable functionalities/models

This document includes our discussions, observations and proposals on the UE-side data collection for AI/ML-based beam management and AI/ML-based positioning accuracy enhancement. We expand upon agreements and observations captured in [3]

# Discussion on UE-side data collection

  - Training data collection for AI/ML based beam management

- UE sends a request to the NW for data collection. The request contains:

  - AI/ML use case – BM-case1, BM-case 2

  - Data request category – Training, Monitoring. This information may help the NW to configure RS transmission (measurement configuration) within the appropriate time constraints. Training data may have a more relaxed time constraint compared to monitoring data.

  - Reference signal type – Synchronization Signal Block (SSB) (This implies that Set A and Set B are different), Channel State Information Reference Signals (CSI-RS) (This implies that Set B is a subset of Set A)

- NW responds with the measurement configuration containing:

  - Measurement duration indicating the time during which the UE can collect necessary training data (periodicity, number of slots/frames)

  - NW can assign an ID to identify the current instance of data being collected. This ID could be referenced, if necessary, to achieve alignment of NW-side additional conditions at a later stage.

  - Inference data collection for AI/ML based beam management

  - UE reports selected model

  - NW configures measurements at the UE based on the UE-side model input characteristics (Set B). These measurements include: Beams within Set B.

  - UE performs inference and reports the model-specific output characterises to the NW.

  - BM-Case1 – beam related information of top K beams (beam index or L1-RSRP) in Set A

  - BM-Case2 – beam related information of top K beams (beam index or L1-RSRP) in Set A at future time instance(s)

  - Monitoring data collection for AI/ML based beam management

- UE reports selected model

- NW configures measurements at the UE based on the UE-side model input characteristics. These measurements include: (1) Beams within Set A if Set B is a subset of Set A, (2) Beams within both Set A and Set B if Set B is different from Set A.

- UE performs inference using Set B and computes KPIs with model outputs and Set A measurements.

  - Training data collection for AI/ML based positioning

- Case 1: UE-based positioning with UE-side model, direct AI/ML or AI/ML assisted positioning

  - Direct AI/ML

    - UE requests the NW for data collection. The request contains:

      - Data request category – Training, Monitoring. This information may help the NW to configure RS transmission (measurement configuration) within the appropriate time constraints. Training data may have a more relaxed time constraint compared to monitoring data.

      - Input dimension of the model to be trained (Number of TRPs etc)

    - NW responds with a measurement configuration containing:

      - Information on the upcoming DL reference signal transmission

      - Measurement duration, indicating the time during which the UE can collect necessary data (periodicity, number of slots/frames)

      - NW assigns an ID to identify the current instance of data being collected. This ID could be referenced, if necessary, to achieve alignment of NW-side additional conditions at a later stage

    - NW (one or more TRPs) sends DL reference signals

    - UE measures parameters such as CIR, RSRP, PDP, DP etc. The UE assigns time stamps to its measurements.

    - Ground truth position measurements are made either by a UE with known position or a PRU, based on non-NR and or NR RAT-dependent or NR RAT-independent methods. Alternatively, the LMF may provide ground truth information, if available. A time stamp and label quality indicator are assigned to the ground truth labels.

  - AI/ML-Assisted

    - UE requests the NW for data collection. The request contains:

      - Data request category – Training, Monitoring. This information may help the NW to configure RS transmission (measurement configuration) within the appropriate time constraints. Training data may have a more relaxed time constraint compared to monitoring data.

      - Input dimension of the model to be trained (Number of TRPs etc)

    - NW responds with a measurement configuration containing:

      - Information on the upcoming DL reference signal transmission

      - Measurement duration, indicating the time during which the UE can collect necessary data (periodicity, number of slots/frames)

      - NW assigns an ID to identify the current instance of data being collected. This ID could be referenced, if necessary, to achieve alignment of NW-side additional conditions at a later stage

    - NW (one or more TRPs) sends DL reference signals

    - UE measures parameters such as CIR, RSRP, PDP, DP etc. The UE assigns time stamps to its measurements.

    - Ground truth measurements of ToA, RSTD, LoS/NLoS etc are made either by a UE with known position or a PRU, based on non-NR and or NR RAT-dependent or NR RAT-independent methods. Alternatively, the LMF may provide ground truth information, if available. A time stamp and label quality indicator are assigned to the ground truth labels.

- Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning

  - AI/ML-Assisted

    - UE requests the NW for data collection. The request contains:

      - Data request category – Training, Monitoring

      - Input dimension of the model to be trained (Number of TRPs etc)

    - NW responds with a measurement configuration containing:

      - Information on the upcoming DL reference signal transmission

      - Measurement duration, indicating the time during which the UE can collect necessary data (periodicity, number of slots/frames)

      - NW assigns an ID to identify the current instance of data being collected. This ID could be referenced, if necessary, to achieve alignment of NW-side additional conditions at a later stage

    - NW (one or more TRPs) sends DL reference signals

    - UE measures parameters such as CIR, RSRP, PDP, DP etc. The UE assigns time stamps to its measurements.

    - Ground truth measurements of ToA, RSTD, LoS/NLoS etc are made either by a UE with known position or a PRU, based on non-NR and or NR RAT-dependent or NR RAT-independent methods. Alternatively, the LMF may provide ground truth information, if available. A time stamp and label quality indicator are assigned to the ground truth labels.

  - Inference data collection for AI/ML based positioning

Inference refers to the scenario in which an AI/ML functionality has already been configured and specific models have been selected and activated.

- Case 1: UE-based positioning with UE-side model, direct AI/ML or AI/ML assisted positioning

  - Direct AI/ML

    - UE reports selected model to the NW

    - NW sends DL reference signals

    - UE measures parameters such as CIR, RSRP, PDP, DP etc

    - UE uses the above measurements as inputs to an AI/ML model whose output is the position estimate

  - AI/ML Assisted

    - UE reports selected model to the NW

    - NW sends DL reference signals

    - UE measures parameter(s) such as CIR, RSRP, PDP, DP etc

    - UE uses the above measurements as inputs to an AI/ML model whose output is an estimate of intermediate parameter(s) such as ToA, RSTD, LoS/NLoS etc

    - UE then uses the model output as input to another algorithm that estimates the position

- Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning

  - AI/ML Assisted

    - UE reports selected model to the NW

    - NW sends DL reference signals

    - UE measures parameter(s) such as CIR, RSRP, PDP, DP etc

    - UE uses the above measurements as inputs to an AI/ML model whose output is an estimate of intermediate parameter(s) such as ToA, RSTD, LoS/NLoS etc

    - UE sends the estimated intermediate parameter(s) to the LMF via the NW

    - LMF estimates the position using the UE’s estimates

# Conclusion

In this contribution, we have provided a discussion on UE-side data collection for AI/ML-based beam management and AI/ML-based positioning accuracy enhancement.

Proposal 1: For UE-side beam management model training data collection, UE sends a request to the NW and the NW responds with the measurement configurations. UE makes the measurements and trains the model. During configuration, the NW could assign an ID to identify the current instance of data being collected. This ID could be referenced, if necessary, to achieve alignment of additional conditions at a later stage.

Proposal 2: For UE-side beam management inference data collection, the UE reports the selected model. The NW then configures measurements at the UE, based on the UE-side model input characteristics. Lastly the UE performs inference and reports the model-specific output characterises to the NW.

Proposal 3: For UE-side beam management monitoring data collection, the NW configures measurements. The UE then collects the monitoring data. Decision regarding model activation/deactivation/switching/fallback can either be taken at the UE or NW based either on KPIs or direct measurements of model output.

Proposal 4: Specification of signaling for UE-side positioning training data collection should be based on whether the AI/ML method being used is Direct AI/ML or AI/ML assisted.

Proposal 5: Specification of signaling for UE-side positioning inference data collection should be based on whether the AI/ML method being used is Direct AI/ML or AI/ML assisted.

# References

RP-234039 New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, 3GPP TSG RAN Meeting #102, Edinburgh, Scotland, December 11-15, 2023

RP-240774 Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, 3GPP TSG RAN Meeting #103, Maastricht, Netherlands, March 18-21, 2024

3GPP TR 38.843 Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18), V2.0.0 (2023-12)