3GPP TSG RAN WG2#126	R2-2404105

Fukuoka, Japan, 20th - 24th May 2024

3GPP TSG-RAN WG1 Meeting #116bis	R1-2403539

Changsha, China, April 15 - 19, 2024

Title:	Reply LS on SRS BW aggregation impact on other channels/signals

Response to:	R1-2401956 (R4-2403489) LS on SRS BW aggregation impact on other channels/signals

Release:	Rel-18

Work Item:	NR_pos_enh2-Core

Source:	RAN1

To:	RAN4

Cc:	RAN2, RAN3

Contact Person:

#### Name:	Jinhuan Xia

Tel. Number:

E-mail Address:	jinhuan.xia@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

RAN1 thanks RAN4 for LS on SRS BW aggregation impact on other channels/signals.

Regarding the following question raised by RAN4

RAN4 respectfully asks RAN1 whether RAN1 has defined or will define any solution to handle the impact of SRS transmission for BW aggregation on other channels/signals.

RAN1 confirms RAN1 has defined the solution to handle the impact of SRS transmission for BW aggregation on other channels/signals.

The relevant agreements are as follows:

Agreement For positioning SRS aggregation transmission in RRC_INACTIVE state, reuse Rel-17 prioritization rule of SRS outside initial BWP, i.e. SRS is dropped in the symbol(s) of all aggregated carriers where collision occurs. Agreement When an SRS resource configured within a CC without PUSCH/PUCCH is linked for aggregation with an SRS resource configured within an UL active BWP of a UL communication CC, a guard period is needed before and after the aggregated SRS transmissions. - Send an LS to RAN4 with the above information and a request to provide the retuning time values needed. Agreement In RRC_CONNECTED state, for positioning SRS aggregation across CCs, if SRS in one of aggregated carriers is dropped in a symbol, stop SRS transmission in all aggregated carriers in the same symbol

In addition, RAN1 also has defined a FG41-4-9 indicating which other bands in the band combination are affected due to the need of a guard period.

2. Actions:

To RAN4:

Action: RAN1 confirms that RAN1 has defined the solution to handle the impact of SRS transmission for BW aggregation on other channels/signals and respectfully requests RAN4 to take above information into account in their future work.

3. Date of Next RAN WG1 Meetings:

RAN WG1 Meeting#117	May 20th – 24th 2024	Fukuoka, Japan

RAN WG1 Meeting#118	August 19th – 23rd 2024	Maastricht, Netherlands