3GPP TSG RAN WG2#126	R2-2404109

Fukuoka, Japan, 20th - 24th May 2024

3GPP TSG RAN WG1 Meeting #116bis	R1-2403588

Changsha, China, April 15th – 19th, 2024

Title:	LS on updating RAN1 agreement about minimum time gap Z

Response to:

Release:	Rel-18

Work Item:	NR_SL_enh2-Core

Source:	RAN1

To:	RAN2

CC:

Contact Person:

#### Name:	Xiang Mi

E-mail Address:	shawn.mixiang@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none.

1. Overall Description:

RAN1#116bis made following agreement.

Agreement

Endorse the red updates in the following RAN1#114bis agreement, and inform RAN2 by LS to update their specification considering this updated agreement.

Agreement In “one PSCCH/PSSCH transmission has N associated candidate PSFCH occasion(s)”, regarding “the minimum time gap Z=a+b between any two selected resources of a TB in case PSFCH is configured for this resource pool”: - Alt 2: Z = a’ + b, where - For unlicensed operation, a’ is defined as: a’ is the time gap between the end of the last symbol of a PSSCH transmission of the first resource and the start of the first symbol of the last corresponding PSFCH reception occasion determined by sl-MinTimeGapPSFCH, and sl-PSFCH-Period, and sl-candidatePSFCH-Occasions for the pool of resources; and - b remains the same as legacy NR SL - Note: the meaning of a and b in legacy NR SL is - a is the time gap between the end of the last symbol of a PSSCH transmission of the first resource and the start of the first symbol of the corresponding PSFCH reception determined by sl-MinTimeGapPSFCH and sl-PSFCH-Period for the pool of resources - b is the time required for PSFCH reception and processing plus sidelink retransmission preparation including multiplexing of necessary physical channels and any TX-RX/RX-TX switching time.

2. Actions:

To RAN2

ACTION: 	RAN1 respectfully requests RAN2 to take above updated agreement into account and update their specification accordingly.

3. Date of Next RAN WG1 Meetings:

RAN WG1 Meeting#117	May 20th – 24th 2024	Fukuoka, Japan

RAN WG1 Meeting#118	August 19th – 23rd 2024	Maastricht, Netherlands