3GPP TSG-RAN WG2 meeting #127	R2-2406202

Agenda Item:	2.2

Source: 	ETSI MCC

Title:		Report of 3GPP TSG RAN WG2 meeting #126, Fukuoka, Japan

Report of 3GPP TSG RAN WG2 meeting #126

Fukuoka, Japan

20 - 24 May, 2024

Document for: Approval

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

© 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

# Contents

Organisation of the meeting	8

Statistics/Executive Summary	8

General	9

1	Opening of the meeting	9

1.1	Call for IPR	9

1.2	Network usage conditions	9

1.3	Other	9

2	General	10

2.1	Approval of the agenda	10

2.2	Approval of the report of the previous meeting	10

2.3	Reporting from other meetings	10

2.4	Instructions	10

2.5	Others	12

3	Incoming liaisons	13

4	EUTRA Rel-17 and earlier	14

4.1	EUTRA corrections Rel-17 and earlier	14

4.1.0	In-principle agreed CRs	14

4.1.1	Other	14

4.2	Void	17

4.3	V2X and Sidelink corrections Rel-15 and earlier	17

4.4	Positioning corrections Rel-16 and earlier	17

5	NR Rel-15 and Rel-16	18

5.1	Common	18

5.1.1	Stage 2 and Organisational	19

5.1.1.0	In-principle agreed CRs	19

5.1.1.1	Other	20

5.1.2	User Plane corrections	20

5.1.2.0	In Principle Agreed CRs	20

5.1.2.1	MAC	20

5.1.2.2	RLC PDCP SDAP BAP	20

5.1.2.3	Other	20

5.1.3	Control Plane corrections	20

5.1.3.0	In-principle agreed CRs	20

5.1.3.1	NR RRC	21

5.1.3.2	UE capabilities	23

5.1.3.3	Other	26

5.2	NR V2X	26

5.3	NR Positioning Support	26

6	NR Rel-17	29

6.1	Common	29

6.1.1	Stage 2 and Organisational	30

6.1.1.0	In-principle agreed CRs	30

6.1.1.1	Other	30

6.1.2	User Plane corrections	31

6.1.2.0	In Principle Agreed CRs	31

6.1.2.1	Other	31

6.1.3	Control Plane corrections	36

6.1.3.0	In Principle Agreed CRs	36

6.1.3.1	NR RRC	38

6.1.3.2	UE capabilities	44

6.1.3.3	Other	48

6.2	NR Sidelink relay	48

6.3	Void	49

6.4	NR positioning enhancements	49

6.6	NR Sidelink enhancements	52

7	Rel-18	53

7.0	Common	53

7.0.1	UE Capabilities	53

7.0.2	CCCH LCID extension	54

7.0.3	ASN.1 Review	54

7.0.4	RACH-less HO	57

7.0.5	Other	62

7.1	NR network-controlled repeaters	63

7.1.1	Organizational	63

7.1.2	Others	63

7.2	Expanded and improved NR positioning	64

7.2.1	Organizational	64

7.2.2	Stage 2	71

7.2.3	SLPP corrections	72

7.2.4	LPP corrections	77

7.2.5	RRC corrections	81

7.2.6	MAC corrections	86

7.2.7	UE capabilities	91

7.2.8	Corrections to other specifications	92

7.3	Network energy savings for NR	92

7.3.1	Organizational	92

7.3.2	User Plane	94

7.3.3	Control Plane corrections	96

7.4	Further NR mobility enhancements	97

7.4.1	Organizational	97

7.4.2	Stage-2 Corrections	98

7.4.3	RRC Corrections	99

7.4.3.1	L1L2 Triggered Mobility	99

7.4.3.2	Conditional Mobility	103

7.4.3.3	Reporting of Idle Inactive and reselection measurements	105

7.4.4	MAC Corrections	106

7.4.5	UE capabilities	108

7.5	XR Enhancements for NR	110

7.5.1	Organizational	110

7.5.2	RRC corrections	112

7.5.3	User plane corrections	112

7.5.3.1	MAC corrections	112

7.5.3.2	PDCP and discard operation	115

7.6	IoT NTN enhancements	116

7.6.1	Organizational	117

7.6.2	Stage 2 corrections	118

7.6.3	RRC Corrections	119

7.6.4	Other Stage 3 corrections	123

7.7	NR NTN enhancements	126

7.7.1	Organizational	126

7.7.2	Stage 2 corrections	128

7.7.3	RRC corrections	129

7.7.4	Other Stage 3 corrections	132

7.8	NR support for UAV	133

7.8.1	Organizational	133

7.8.2	RRC RIL	135

7.8.3	Other	135

7.9	Enhanced NR Sidelink Relay	135

7.9.1	Organizational	135

7.9.2	Stage 2 corrections	137

7.9.3	RRC corrections	138

7.9.4	SRAP corrections	143

7.9.5	MAC corrections	144

7.9.6	RLC and PDCP corrections	144

7.9.7	UE capabilities	145

7.9.8	Idle mode corrections	146

7.10	IDC enhancements for NR and MR-DC	146

7.11	Enhancements of NR Multicast and Broadcast Services	146

7.11.1	Organizational	146

7.11.2	RRC corrections	147

7.11.3	Other corrections	148

7.12	Mobile IAB (Integrated Access and Backhaul) for NR	149

7.12.1	Organizational and Stage-2	149

7.12.2	Stage-3	150

7.12.2.1	BAP	150

7.12.2.2	Control plane corrections	150

7.12.2.3	User plane corrections	150

7.13	Further enhancement of data collection for SON MDT in NR and EN-DC	150

7.13.1	Organizational	150

7.13.2	Papers related to RILs	151

7.13.3	Other	153

7.14	Enhancement on NR QoE management and optimizations for diverse services	154

7.14.1	Organizational	154

7.14.2	RRC corrections	154

7.14.3	Other corrections	157

7.15	NR Sidelink evolution	157

7.15.1	Organizational	157

7.15.2	Control plane corrections	157

7.15.3	User plane corrections	160

7.16	Void	166

7.17	Dual Transmission/Reception (Tx/Rx) Multi-SIM for NR	166

7.17.1	Organizational	166

7.17.2	RRC	166

7.17.3	Other	169

7.18	Mobile Terminated Small Data Transmission	169

7.18.1	Organizational	169

7.18.2	Others	170

7.19	Enhanced support of reduced capability NR devices	172

7.19.1	Organizational	172

7.19.2	Papers related to RILs	173

7.19.3	Other	173

7.20	NR MIMO evolution	175

7.20.1	Organizational	175

7.20.2	MAC	176

7.20.3	RRC	179

7.21	Further NR coverage enhancements	179

7.21.1	Organizational	180

7.21.2	Other Essential corrections	181

7.22	Void	185

7.23	Timing Resiliency and URLLC Enh	185

7.23.1	Organizational	185

7.23.2	General	185

7.24	TEI18	186

7.24.1	TEI proposals by Other Groups	186

7.24.2	TEI proposals by RAN2	186

7.24.2.0	In Principle agreed CRs	186

7.24.2.1	2Rx XR	187

7.24.2.2	Other RAN2 TEI-18	188

7.25	R18 Other	197

7.25.1	RAN4 led items	197

7.25.1.1	Lower MSD capability	197

7.25.1.2	Intra-band non-collocated NR-CA. EN-DC	197

7.25.1.3	TCI State Switch indication for HST	197

7.25.1.4	FR2 Multi Rx operation	197

7.25.1.5	FR2 SCell Enhancements	197

7.25.1.6	ATG	197

7.25.1.7	Other	197

7.25.2	RAN1 led items	201

7.25.3	Other	202

8	Rel-19	202

8.0	General	202

8.1	AI/ML for NR air interface	204

8.1.1	Organizational	204

8.1.2	Functionality based LCM	204

8.1.2.1	LCM for NW-sided model for Beam Management use case	205

8.1.2.2	LCM for UE-sided model  for Beam Management use case	207

8.1.2.3	LCM for Positioning use case	212

8.1.3	NW side data collection	216

8.1.4	UE side data collection	219

8.2	Ambient IoT	225

8.2.1	Organizational	225

8.2.2	Stage 2 General aspects	225

8.2.3	Functionality aspects	229

8.2.4	A-IoT Paging	233

8.2.5	A-IoT Random Access	237

8.3	AI/ML for Mobility	242

8.3.1	Organizational	242

8.3.2	RRM measurement prediction	242

8.3.2.1	Simulation assumptions and evaluation methodology for RRM measurement prediction	242

8.3.2.2	Other aspects related to RRM measurement prediction	250

8.3.3	Measurement event predictions	251

8.3.4	RLF/HO failure prediction	251

8.3.4.1	Simulation assumptions and evaluation methodology for RLF failure prediction	251

8.3.4.2	Other aspects related to RLF/HO failure prediction	253

8.4	Low-power wake-up signal and receiver for NR (LP-WUS/WUR)	257

8.4.1	Organizational	257

8.4.2	Procedure and configuration of LP-WUS in RRC_IDLE/INACTIVE	258

8.4.3	RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE	261

8.4.4	Procedures for LP-WUS in RRC_CONNECTED	263

8.5	Network Energy Saving Enh.	266

8.5.1	Organizational	266

8.5.2	On-demand SSB SCell operation	266

8.5.3	On-demand SIB1	267

8.5.4	Adaptation of common signal/channel transmissions	272

8.6	Mobility Enhancement Ph4	273

8.6.1	Organizational	273

8.6.2	Inter-CU LTM	273

8.6.3	Measurement enhancements for LTM	278

8.7	XR Enhancements Ph3	282

8.7.1	Organizational	282

8.7.1.1	Discussion on LSs from SA2	283

8.7.2	Multi-modality support	287

8.7.3	RRM measurement gaps/restrictions related enhancements	290

8.7.4	Scheduling enhancements	290

8.7.5	RLC enhancements	294

8.8	NTN for NR Ph3	297

8.8.1	Organizational	297

8.8.2	Downlink coverage enhancements	298

8.8.3	Uplink Capacity/Throughput Enhancement	301

8.8.4	Support of Broadcast service	301

8.8.5	Support of regenerative payload	304

8.8.6	LTE to NR NTN mobility	307

8.9	IoT NTN Ph3	310

8.9.1	Organizational	310

8.9.2	Support of Store & Forward	310

8.9.3	Uplink Capacity Enhancement	313

8.10	SON MDT Ph4	317

8.10.1	Organizational	317

8.10.2	MRO enhancements for Rel-18 mobility features	317

8.10.3	SON/MDT for Slicing	319

8.10.4	SON/MDT for NTN	320

8.10.5	Leftovers from Rel-18	320

9	Breakout session reports	322

9.1	Session on V2X/SL, R19 NES and MOB	322

9.2	Session on R18 MIMOevo, R18 MUSIM, and R19 LP-WUS	322

9.3	Session on NR NTN and IoT NTN	322

9.4	Session on positioning and sidelink relay	322

9.5	Session on Mobility Enh and Mobile IAB	322

9.6	Session on R18 MBS, R18 QoE and R19 XR	322

9.7	Session on maintenance, SON/MDT and eRedCap	322

9.8	Session on further NR coverage enhancements	322

Closing of the meeting	322

Annex A:	List of participants	323

Annex B:	List of Tdocs	323

Annex C:	Incoming liaison statements	323

Annex D:	Outgoing liaison statements	327

Annex E:	List of agreed in principle CRs	328

Annex F:	List of email discussions during the meeting	341

Annex G:	Post-meeting email discussions	342

Guidelines for email discussions:	342

Inactive periods and other planning comments	343

Short email discussions, Deadline June 6th, 10:00 UTC (unless otherwise stated)	343

Long email discussions, Deadline July 26th, 10:00 UTC (unless otherwise stated)	350

Annex H:	History	351

# Organisation of the meeting

Meeting:	3GPP TSG RAN2#126

Meeting location:	Fukuoka, Japan

Duration:	20 - 24.05.2024

Host:	Japanese Friends of 3GPP

TSG RAN WG2 Chair	Diana Pani (InterDigital) (diana.pani@interdigital.com)

TSG RAN WG2 Vice chair:	Kyeongin Jeong (Samsung) (kyeongin.j@samsung.com)

TSG RAN WG2 Vice chair:	Erlin Zeng (CATT) (erlin.zeng@catt.cn)

TSG RAN WG2 MCC Support:	Juha Korhonen (ETSI MCC) (juha.korhonen@etsi.org)

Email reflector:	3GPP_TSG_RAN_WG2@LIST.ETSI.ORG

Technical documents:	ftp://ftp.3gpp.org/tsg_ran/WG2_RL2/TSGR2_126/Docs

Next meetings:	TSG RAN2#127	19 - 23.08.2024, Maastricht , NL

TSG RAN2#128	14 - 18.10.2024, China

# Statistics/Executive Summary

TSG RAN2#126 was a normal face-to-face meeting, with a possibility for one-way(listen only) remote access.

There were 63 numbered email discussions during this meeting.

The topics discussed were:

-	NR Rel-15, Rel-16 and Rel-17 User Plane corrections, Rel-18 Common, ASN.1 review, NR network-controlled repeaters, Network energy savings for NR, XR Enhancements for NR, NR support for UAV, Mobile Terminated Small Data Transmission, Timing Resiliency and URLLC Enh, TEI18, Rel-18 Other, Rel-19 General, AI/ML for NR air interface, Ambient IoT, AI/ML for Mobility - Diana Pani (Chair)

-	NR V2X, NR Sidelink enhancements, NR Sidelink evolution, Network Energy Saving Enh, Mobility Enhancement Ph4  - Kyeongin Jeong (VC)

-	Dual Transmission/Reception (Tx/Rx) Multi-SIM for NR, NR MIMO evolution, Low-power wake-up signal and receiver for NR - Erlin Zeng (VC)

-	Further NR mobility enhancements, Mobile IAB for NR - Johan Johansson

-	IoT NTN enhancements, NR NTN enhancements, NTN for NR Ph3, IoT NTN Ph3 - Sergio Parolari

-	Rel-15 and Rel-16 NR Positioning Support, NR sidelink relay, NR positioning enhancements, Expanded and improved NR positioning, Enhanced NR Sidelink Relay, NR TEI18 (Positioning, Relay) - Nathan Tenny

-	NR NCR corrections - Sasha Sirotkin

-	Enhancements of NR Multicast and Broadcast Services, Enhancement on NR QoE management and optimizations for diverse services, TEI18: eDRX/MICO, MBS, XR Enhancements Ph3 - Dawid Koziol

-	IDC enhancements for NR and MR-DC - Yi Guo

-	EUTRA corrections Rel-17 and earlier, NR Rel-15, Rel-16 and Rel-17 Common (except user plane), Further enhancement of data collection for SON MDT in NR and EN-DC, Enhanced support of reduced capability NR devices, SON MDT Ph4 - Mattias Bergström

-	Further NR coverage enhancements - Eswar Vutukuri

The statistics from this meeting are:

-	447 participants

-	1996 Tdoc numbers allocated with 1975 available contributions. (See the attached tdoc list)

-	51 incoming liaison statements, out of which 48 were noted. The remaining non-treated liaison will be treated in RAN2#127 meeting.

-	17 outgoing liaison statements

-	63 at-meeting email discussions

-	63 email approvals/discussions scheduled after the RAN2#126 meeting (57 short and 6 long), see Annex G for details.

Number of CRs submitted: 626. Out of these, 270 were agreed. See Annex E for details.

# General

This meeting was an ordinary meeting and had full decision power, i.e. full decision power to make agreements and approvals according to RAN WG2 terms of reference, without any need to ratify decisions at a later RAN2 or other meeting.

# 1	Opening of the meeting

## 1.1	Call for IPR

The attention of the delegates of this Working Group is drawn to the fact that 3GPP Individual Members have the obligation under the IPR Policies of their respective Organizational Partners to inform their respective Organizational Partners of Essential IPRs they become aware of. The delegates were asked to take note that they were hereby invited: - to investigate whether their organization or any other organization owns IPRs which were, or were likely to become Essential in respect of the work of 3GPP. - to notify their respective Organizational Partners of all potential IPRs, e.g., for ETSI, by means of the IPR Statement and the Licensing declaration forms (https://www.etsi.org/images/files/IPR/etsi-ipr-form.doc)

NOTE:	IPRs may be declared to the Director-General or Chairman of the SDO, but not to the RAN WG2 Chairman.

## 1.2	Network usage conditions

1/ 	To avoid email system overload, please don’t attach files and documents to emails e.g. for offline email discussions, but instead use files placed on the meeting server instead. Inbox/Drafts folder is used for meeting offline discussions.

## 1.3	Other

In accordance with the Working Procedures it is reaffirmed that: (i) compliance with all applicable antitrust and competition laws is required; (ii) timely submissions of work items in advance of TSG or WG meetings are important to allow for full and fair consideration of such matters; and (iii) the chair will conduct the meeting with strict impartiality and in the interests of 3GPP

Note on (i): In case of question please contact your legal counsel.

Note on (ii): WIDs don’t need to be submitted to the RAN2 meeting and will typically not be discussed here either.

# 2	General

## 2.1	Approval of the agenda

R2-2404101	Agenda for RAN2#126	Chairlady	agenda

=> Revised in R2-2405855

R2-2405855	Agenda for RAN2#126	Chairlady	agenda

=>	The agenda is approved

## 2.2	Approval of the report of the previous meeting

R2-2404102	RAN2#125bis Meeting Report	MCC	report

=>	The report is approved

## 2.3	Reporting from other meetings

## 2.4	Instructions

CRs

- Use latest CR template version 12.3 for all CRs submitted to RAN2 meeting

Rel-17 maintenance CRs

- Only essential/critical corrections are expected

- Editorial and clarification corrections should be sent to be reviewed and approved by spec rapporteurs prior to submission.

- Editorials corrections should be collected and submitted by spec rapporteurs.

Rel-18 CR Handling

-	CR editors / Rapporteurs continue to support maintenance related to their respective CR / WI and are required to follow drafting rules

-	Single correction CR per spec coordinated by CR editor/rapporteurs will be agreed per feature for RAN#104

-	CR editors / Rapporteurs are to gather miscellaneous and non-controversial issues, if any, for their respective specification prior to submission deadline.  Other companies are expected to give inputs to these CRs and not have contributions on such issues.

-	Companies should give inputs on editorials and clarifications to the CR editors/rapporteurs and not have individual CRs/contributions on such issues. Emails to CR editors/rapporteurs should follow the following naming convention when sending emails to rapporteurs:

[Pre_RAN2#126][CR xx.yyy] Clarification CRs

-	The organizational AIs for each WIs are reserved for rapporteurs only.  CR rapporteurs are expected to submit only 1 CR per spec.

-	For RRC corrections, only selected RIL can be submitted in the agenda (i.e. only if RRC editor suggests to discuss the RIL under this agenda)

-	Companies are expected to submit Tdocs with TP (not CRs). More specifically, the Tdoc should contain description of open issues/proposal and the proposed corrections/TP in the contribution itself. Small issues can be included in the tdoc with just short justification, same level of detail as in cover sheet.

-	RRC ASN.1 changes can be drafted in a NBC way until ASN.1 is frozen, to avoid unnecessary RRC overhead. The focus should be on drafting the changes in the best possible way.

-	Inter-op analysis on Rel-18 CR coverpages in NOT needed

Remaining/updated Rel-18 RRC parameters and MAC CEs

-	RRC parameters updates/corrections, including those requested by other groups, e.g. RAN1, are covered by WI-specific RRC CRs.

-	MAC CE parameters updates/corrections, including those requested by other groups, e.g. RAN1, are covered by WI-specific MAC CRs

Rel-18 UE capabilities

-	EUTRA UE capabilities corrections are covered by separate CRs

-	NR UE capabilities (new) and corrections are covered in Rel-18 common MegaCRs (38306 and 38331) covering all rel-18 WIs (end outcome).

-	UE capabilities in LPP 37355 and SLPP 38355 are covered in the main CRs for the Positioning WI.

During the work on NR UE caps:

-	In a Common Rel-18 Agenda Item (AI): RAN1 and RAN4 feature corrections are handled jointly under a common AI, with some explicit exceptions. Running UE cap MegaCRs are maintained for the parts handled in the common AI.

-	In WI-specific Rel-18 Agenda Items: RAN2 features/corrections are handled per WI and only a draft CR per WI is expected and will be merged with the running mega CR

ASN.1 Review

-	Please follow the instructions provided in ASN.1 review rapporteur and read section “Review execution” on what to expect for paper submission.

https://www.3gpp.org/ftp/Email_Discussions/RAN2/%5BMisc%5D/ASN1%20review/Rel-18%202024-03

- Contributions on WI specific RILs should be submitted under the corresponding WI specific AI and NOT in the general ASN.1 review AI (7.0.3).  That AI is reserved for common/cross-WI specific identified RILs

- Title of contribution should start with [RIL number] Title, or "[RIL number1][RIL number N] Title” if there are more than one RIL in a Tdoc.

- Proposals related to RIL resolution should include RIL number in the proposal

ASN.1 Review deadlines

- ANS.1 review final deadline: May 3rd

- RIL resolution proposals from CR rapporteurs: May 7th

- Tdoc reservation and submission:  May 10th same as normal Tdoc deadline

- IMPORTANT: Please follow the guidelines in the “review execution” for resolving RIL issues. Companies are expected to coordinate with other companies and WI CR rapporteurs on ASN.1 issues identified, either postponed from last meeting or newly identified.

https://www.3gpp.org/ftp/Email_Discussions/RAN2/%5BMisc%5D/ASN1%20review/Rel-18%202024-03

Tdoc limitations

Tdoc limitations doesn’t apply to Rapporteur Input, i.e.

-	Assigned summary rapporteur input of the summary.

-	Email / offline discussions outcomes by discussion rapporteur,

-	WI rapporteurs input for WI planning etc,

-	TS rapporteur input for TS maintenance.

-	Contact Company of a LSin that triggers RAN2 action may submit one tdoc to facilitate the LS reply. This only applies to one of the contact companies in case there are several (default the first).

-	Spec rapporteur list of open issues for Rel-18 items

Tdoc limitations doesn’t apply to Input created at the meeting, revisions, assigned documents etc.

Tdoc limitations doesn’t apply to shadow / mirror CRs (Cat A), or In-Principle Agreed CRs.

Tdoc limitations doesn’t apply to Tdocs related to RILs which has been assigned during ASN.1 review. Single Tdoc containing 1 or more RIL resolutions per WI is expected.

Tdoc limitations applies to all other submitted tdocs (e.g. discussion tdoc and CR tdoc are counted as two).

Tdoc request/submission for RAN2#126 deadlines:

- Tdoc Submission deadline: May 10th, 2024 1000 UTC

## 2.5	Others

R2-2404103	RAN2 Handbook	MCC	discussion

RAN3 endorsed stage-2 CRs for RAN2 agreement:

R2-2406048	Correction on the MT communication handling procedure	R3 (Huawei, ZTE, Ericsson, Nokia, CATT, Qualcomm Incorporated, China Telecom)	CR	Rel-18	38.300	18.1.0	0871	-	F	NR_redcap_enh-Core

=> Agreed

R2-2406049	Correction on SDT RRC Release with resume indication [SDT_ReleaseEnh]	R3 (ZTE, Huawei, China Telecom, Qualcomm, LG Electronics, China Unicom, Samsung, CATT)	CR	Rel-18	38.300	18.1.0	0872	-	B	TEI18

=> Agreed

R2-2406050	Clarification on non-homogeneous deployment	R3 (Nokia, Nokia Shanghai Bell, ZTE, Huawei, CATT, Ericsson, Qualcomm Inc, CMCC, Xiaomi, Samsung, Lenovo, CATT)	CR	Rel-18	38.300	18.1.0	0873	-	F	NR_XR_enh-Core

=> Agreed

R2-2406051	Correction on DL PSI based discard	R3 (Huawei, CATT, CMCC, Qualcomm, Ericsson, Nokia, Nokia Shanghai Bell, ZTE, China Telecom)	CR	Rel-18	38.300	18.1.0	0874	-	F	NR_XR_enh-Core

=> Agreed

R2-2406052	Introduction of Measurement Quality and Time Stamp Information to E-CID [ECIDQualTimeStamp]	R3 (Ericsson, Comtech, Verizon Wireless, Polaris Wireless, Nokia, ZTE)	CR	Rel-18	38.305	18.1.0	0170	-	B	TEI18

=> Agreed

R2-2406053	Correction on burst arrival time in handover	R3 (Lenovo, Huawei, CATT, CMCC, ZTE, Xiaomi, Samsung, Qualcomm, Nokia, Nokia Shanghai Bell, Ericsson)	CR	Rel-18	38.300	18.1.0	0875	-	F	NR_XR_enh-Core

=> Agreed

R2-2406054	Clarification of the bearer menagement in case of CHO with SCG	R3 (Nokia, LG Electronics)	CR	Rel-18	37.340	18.1.0	0394	-	F	NR_Mob_enh2-Core

=> Agreed

R2-2406055	Stage2 correction on direct data forwarding	R3 (Samsung, CATT, Huawei, Cybercore, ZTE, Nokia, LG Electronics, Ericsson)	CR	Rel-18	37.340	18.1.0	0395	-	F	NR_Mob_enh2-Core

=> Agreed

R2-2406056	Correction of Slice Availability and Support	R3 (Nokia, Deutsche Telekom, Ericsson, Huawei, CATT, ZTE)	CR	Rel-18	38.300	18.1.0	0876	-	F	eNS_Ph3-NR-Core

=> Agreed

R2-2406057	Addition of CPAC detection in stage 2	R3 (Samsung, Lenovo, CATT, Cybercore)	CR	Rel-18	37.340	18.1.0	0396	-	F	NR_ENDC_SON_MDT_enh2-Core

=> Agreed

R2-2406058	Correction to OAM requirement for UE location verification	R3 (CATT, Huawei, ZTE, Samsung, Nokia)	CR	Rel-18	38.305	18.1.0	0171	-	F	NR_NTN_enh-Core

=> Agreed

R2-2406059	Addition of SON for fast MCG recovery in stage 2	R3 (Samsung, Lenovo, CATT, Nokia, ZTE)	CR	Rel-18	38.300	18.1.0	0877	-	F	NR_ENDC_SON_MDT_enh2-Core

=> Agreed

R2-2406060	Stage 2 update on AI/ML for NG-RAN	R3 (CATT, Nokia, ZTE, Ericsson, Huawei, Samsung)	CR	Rel-18	38.300	18.1.0	0878	-	F	NR_AIML_NGRAN-Core

=> Agreed

R2-2406061	Correction on Handover Cancel in CHO with SN for EN-DC	R3 (Samsung, ZTE, Ericsson, Huawei)	CR	Rel-18	37.340	18.1.0	0397	-	F	LTE_NR_DC_enh2-Core, TEI18

=> Agreed

R2-2406062	Correction on MDT for PNI-NPN	R3 (ZTE, Ericsson, Huawei, CATT)	CR	Rel-18	37.320	18.1.0	0133	-	F	NR_ENDC_SON_MDT_enh2-Core

=> Agreed

R2-2406063	Correction on Inter-system MRO	R3 (Samsung, ZTE, Nokia, CMCC)	CR	Rel-18	38.300	18.1.0	0879	-	F	NR_SON_MDT-Core, TEI18

=> Agreed

R2-2406064	Correction on Inter-system MRO	R3 (Samsung, Nokia, ZTE, CMCC)	CR	Rel-18	36.300	18.1.0	1404	-	F	NR_SON_MDT-Core, TEI18

=> Agreed

R2-2406065	Stage 2 corrections for AI/ML for NG-RAN	R3 (Huawei, Nokia, Ericsson, InterDigital, NEC, Deutsche Telekom, Telecom Italia, CATT, Lenovo, ZTE, Samsung)	CR	Rel-18	38.300	18.1.0	0880	-	F	NR_AIML_NGRAN-Core

=> Agreed

R2-2406066	Indicator on MDT configuration in MR-DC	R3 (ZTE)	CR	Rel-18	37.320	18.1.0	0134	-	F	NR_ENDC_SON_MDT_enh2-Core

=> Agreed

R2-2406067	Correction on user consent for trace reporting	R3 (Ericsson, AT&T, Deutsche Telekom, ZTE, CMCC, Nokia, Huawei)	CR	Rel-18	37.320	18.1.0	0135	-	F	NR_SON_MDT-Core, TEI18

=> Agreed

R2-2406068	Correction on of PSCell List of RA report	R3 (ZTE, CATT, China Telecom, Nokia, Huawei, Ericsson, CMCC, China Unicom, Samsung)	CR	Rel-18	37.340	18.1.0	0398	-	F	NR_ENDC_SON_MDT_enh2-Core

=> Revised in R2-2406101

R2-2406101	Correction on of PSCell List of RA report	R3 (ZTE, CATT, China Telecom, Nokia, Huawei, Ericsson, CMCC, China Unicom, Samsung)	CR	Rel-18	37.340	18.1.0	0398	1	F	NR_ENDC_SON_MDT_enh2-Core

=> Agreed

# 3	Incoming liaisons

Note: LSs are moved to the respective agenda items if any.

# 4	EUTRA Rel-17 and earlier

Only essential corrections. No documents should be submitted to 4. Please submit to 4.x

## 4.1	EUTRA corrections Rel-17 and earlier

(NB_IOTenh4_LTE_eMTC6-Core; leading WG: RAN1; REL-17; WID: RP-211340)

(UPIP_EN-DC_UE; leading WG: RAN3; REL-17; WID: RP-213669)

(LTE TEI17)

Essential corrections to LTE Rel-17 topics not covered by other agenda items.

(NB_IOTenh3-Core; leading WG: RAN1; REL-16; started: Jun 18; Completed: June 20; WID: RP-200293); REL-15 and Earlier NB-IoT WIs are in scope but not listed explicitly (long list).

(LTE_eMTC5-Core; LTE_eMTC5-Core; leading WG: RAN1; REL-16; started: Jun 18; Completed:  June 20; WID: RP-192875), REL-15 and Earlier eMTC WIs are in scope but not listed explicitly (long list).

(LTE_feMob-Core; leading WG: RAN2; REL-16; started: Jun 18; Completed: June 20; WID: RP-190921);

(LTE_terr_bcast-Core, LTE_DL_MIMO_EE-Core, LTE_high_speed_enh2-Core; LTE TEI16 Non-positioning);

(LTE_NBIOT_eMTC_NTN; leading WG: RAN1; REL-17; WID: RP-211601)

REL-15 and Earlier EUTRA WIs are in scope but not listed explicitly (long list), Except V2X and Sidelink WIs and Positioning WIs, which are addressed by AIs below.

NOTE that LTE corrections related to NR WIs or Joint NR LTE WIs should be submitted to NR AIs below.

NOTE that LTE corrections which are the same as an NR correction should be submitted to the respective NR AI (so the NR CR and LTE CR can be treated together).

This Agenda Item is treated in the Maintenance Breakout session (Corrections for LTE_NBIOT_eMTC_NTN might be treated in the NTN breakout session)

### 4.1.0	In-principle agreed CRs

### 4.1.1	Other

Resume

R2-2404516	Further corrections to RRCConnectionResume in LTE RRC	Lenovo	discussion	Rel-14	TEI14

Proposal 1: From Rel-14 onwards replace field name "rrcConnectionResume-v1430-IEs" by "nonCriticalExtension" in RRCConnectionResume-r13-IEs.

Proposal 2: From Rel-15 onwards replace field name "rrcConnectionResume-v1510-IEs" by "nonCriticalExtension" in RRCConnectionResume-v1430-IEs.

Proposal 3: From Rel-17 onwards correct the need code for the field scg-State-r17 to “Need ON”.

Disc:

-	CATT are OK with P1 and P2 but think that for P3, it should not be ON. Huawei agrees with CATT. Lenovo thinks that there are rules when using OP, we need to specify what the UE shall do when the field is absent. QC agrees with Lenovo. Huawei instead wants to see if we can clarify in the procedural text instead, and want to come back to this in a later meeting.

From Rel-14 onwards replace field name "rrcConnectionResume-v1430-IEs" by "nonCriticalExtension" in RRCConnectionResume-r13-IEs.

From Rel-15 onwards replace field name "rrcConnectionResume-v1510-IEs" by "nonCriticalExtension" in RRCConnectionResume-v1430-IEs.

NTN

R2-2405120	Adding definition of earth-moving cell for IoT NTN	Huawei, HiSilicon, ZTE Corporation, Sanechips, Nokia, Nokia Shanghai Bell, CATT, Intel Corporation	CR	Rel-17	36.331	17.8.0	5019	-	F	LTE_NBIOT_eMTC_NTN

R2-2405121	Adding definition of earth-moving cell for IoT NTN	Huawei, HiSilicon, ZTE Corporation, Sanechips, Nokia, Nokia Shanghai Bell, CATT, Intel Corporation	CR	Rel-18	36.331	18.1.0	5020	-	A	LTE_NBIOT_eMTC_NTN

Merged by spec rapporteur in his rapporteur’s CR

R2-2405452	IoT NTN Kmac correction Ericsson	CR	Rel-17	36.300	17.7.0	1402	-	F	LTE_NBIOT_eMTC_NTN-Core

Moved from 7.6.1

-	NTN-session chair (ZTE) explains that there was a R18 CR agreed and now Ericsson found out this change should be backported to R17. So to make things neat, we should update both CRs to a R17 cat F which explains neutrally what needs to be done and then have a R18 Cat A CR.

[AT126][751][Maint] IoT NTN Kmac correction (Ericsson)

Scope:

    - Produce agreeable CRs

Intended outcome:

    - Agreed CRs in R2-2405801 (Rel17) and R2-2405802 (Rel18) (Ericsson)

Deadline:

    - Thursday lunch. Intention is to agree the CRs over email.

R2-2405801	IoT NTN Kmac correction Ericsson	CR	Rel-17	36.300	17.7.0	1402	1	F	LTE_NBIOT_eMTC_NTN-Core	R2-2405452

R2-2405802	IoT NTN Kmac correction Ericsson	CR	Rel-18	36.300	18.1.0	1403	-	A	LTE_NBIOT_eMTC_NTN-Core

Both agreed

R2-2405752	IoT NTN measurement corrections	Ericsson	CR	Rel-18	36.300	18.1.0	1401	2	F	LTE_NBIOT_eMTC_NTN-Core

Agreed

Misc

R2-2405397	Miscellaneous Corrections for TS 36.331	Samsung, Qualcomm	CR	Rel-14	36.331	14.16.0	5023	-	F	TEI14, TEI10

=> Revised in R2-2405803

R2-2405803	Miscellaneous Corrections for TS 36.331	Samsung, Qualcomm	CR	Rel-14	36.331	14.16.0	5023	1	F	TEI14, TEI10

=> Revised in R2-2406106

R2-2405398	Miscellaneous Corrections for TS 36.331	Samsung, Qualcomm	CR	Rel-15	36.331	15.21.0	5024	-	F	TEI14, TEI10, LTE_NBIOT_eMTC_NTN

=> Revised in R2-2405804

R2-2405804	Miscellaneous Corrections for TS 36.331	Samsung, Qualcomm	CR	Rel-15	36.331	15.21.0	5024	1	F	TEI14, TEI10, LTE_NBIOT_eMTC_NTN

=> Revised in R2-2406107

R2-2405399	Miscellaneous Corrections for TS 36.331	Samsung, Qualcomm	CR	Rel-16	36.331	16.15.0	5025	-	A	TEI14, TEI10, LTE_NBIOT_eMTC_NTN

=> Revised in R2-2405805

R2-2405805	Miscellaneous Corrections for TS 36.331	Samsung, Qualcomm	CR	Rel-16	36.331	16.15.0	5025	1	A	TEI14, TEI10, LTE_NBIOT_eMTC_NTN

=> Revised in R2-2406108

R2-2405400	Miscellaneous Corrections for TS 36.331	Samsung, Qualcomm	CR	Rel-17	36.331	17.8.0	5026	-	F	TEI14, TEI10, LTE_NBIOT_eMTC_NTN, TEI17, NR_ext_to_71GHz-Core

=> Revised in R2-2405806

R2-2405806	Miscellaneous Corrections for TS 36.331	Samsung, Qualcomm	CR	Rel-17	36.331	17.8.0	5026	1	F	TEI14, TEI10, LTE_NBIOT_eMTC_NTN, TEI17, NR_ext_to_71GHz-Core

=> Revised in R2-2406109

R2-2405401	Miscellaneous Corrections for TS 36.331	Samsung, Qualcomm	CR	Rel-18	36.331	18.1.0	5027	-	F	TEI14, LTE_NBIOT_eMTC_NTN, TEI10, TEI17, NR_ext_to_71GHz-Core, NR_mobile_IAB-Core

=> Revised in R2-2405807

R2-2405807	Miscellaneous Corrections for TS 36.331	Samsung, Qualcomm	CR	Rel-18	36.331	18.1.0	5027	1	F	TEI14, LTE_NBIOT_eMTC_NTN, TEI10, TEI17, NR_ext_to_71GHz-Core, NR_mobile_IAB-Core

=> Revised in R2-2406110

[Post126][751][Maint] Miscellaneous Corrections for TS 36.331 (Samsung)

Scope:

    - Review 36.331 rapporteurs misc CRs.

Intended outcome:

    - Agreed CRs in R2-2405803, R2-2405804, R2-2405805, R2-2405806, R2-2405807 (Samsung)

Deadline:

    - Short

R2-2406106	Miscellaneous Corrections for TS 36.331	Samsung, Qualcomm	CR	Rel-14	36.331	14.16.0	5023	2	F	TEI14, TEI10

=> Agreed

R2-2406107	Miscellaneous Corrections for TS 36.331	Samsung, Qualcomm	CR	Rel-15	36.331	15.21.0	5024	2	F	TEI14, TEI10, NB_IOTenh2, TEI15

=> Agreed

R2-2406108	Miscellaneous Corrections for TS 36.331	Samsung, Qualcomm	CR	Rel-16	36.331	16.15.0	5025	2	A	TEI14, TEI10, NB_IOTenh2, TEI15

=> Agreed

R2-2406109	Miscellaneous Corrections for TS 36.331	Samsung, Qualcomm	CR	Rel-17	36.331	17.8.0	5026	2	F	TEI14, TEI10, NB_IOTenh2, TEI17, NR_ext_to_71GHz-Core, TEI15, LTE_NBIOT_eMTC_NTN

=> Agreed

R2-2406110	Miscellaneous Corrections for TS 36.331	Samsung, Qualcomm	CR	Rel-18	36.331	18.1.0	5027	2	F	TEI14, TEI10, NB_IOTenh2, TEI17, NR_ext_to_71GHz-Core, TEI15, LTE_NBIOT_eMTC_NTN

=> Agreed

=> Coversheet update by MCC: a long list of WIs, but not a single one for Rel-18. But this is a Rel-18 CR.

R2-2406146	Miscellaneous Corrections for TS 36.331	Samsung, Qualcomm	CR	Rel-18	36.331	18.1.0	5027	3	F	TEI14, LTE_NBIOT_eMTC_NTN, TEI10, TEI17, NR_ext_to_71GHz-Core, TEI15, NB_IOTenh2-Core, TEI18

=> Agreed

## 4.2	Void

## 4.3	V2X and Sidelink corrections Rel-15 and earlier

REL-15 and Earlier WIs related to V2x and Sidelink are in scope but not listed explicitly (long list).

This Agenda Item is treated in the V2X and Sidelink Breakout session

Tdoc Limitation: 1 tdocs

R2-2405433	Correction on transmission of SidelinkUEInformation	Philips International B.V.	CR	Rel-13	36.331	13.17.0	5028	-	F	LTE_eD2D_Prox-Core

R2-2405434	Correction on transmission of SidelinkUEInformation	Philips International B.V.	CR	Rel-14	36.331	14.16.0	5029	-	A	LTE_eD2D_Prox-Core

R2-2405435	Correction on transmission of SidelinkUEInformation	Philips International B.V.	CR	Rel-15	36.331	15.21.0	5030	-	A	LTE_eD2D_Prox-Core

R2-2405436	Correction on transmission of SidelinkUEInformation	Philips International B.V.	CR	Rel-16	36.331	16.15.0	5031	-	A	LTE_eD2D_Prox-Core

R2-2405437	Correction on transmission of SidelinkUEInformation	Philips International B.V.	CR	Rel-17	36.331	17.8.0	5032	-	A	LTE_eD2D_Prox-Core

R2-2405438	Correction on transmission of SidelinkUEInformation	Philips International B.V.	CR	Rel-18	36.331	18.1.0	5033	-	A	LTE_eD2D_Prox-Core

- Not pursued.

[NEC]: Did we have relay in LTE D2D? [Philips]: Yes, for L3 relay. [Ericsson]: Remember there was IE name which already implies relay. [Philips]: Think it is not a real editorial correction. [Qualcomm]: If it is not real editorial correction, we need to make sure whether it is needed or not. [Philips]: Based on the offline discussion during the break-time, we don’t need comeback since relay is distinguished by the IE name.

## 4.4	Positioning corrections Rel-16 and earlier

(LTE_NavIC-Core, LTE TEI16 Positioning), REL-15 and Earlier WIs related to positioning are in scope but not listed explicitly (long list).

This Agenda Item will be handled by email.

Tdoc Limitation: 1 tdoc

R2-2404749	Correction to mandatory 80ms scheduling offset for positioning SI acquisition	Huawei, HiSilicon, Ericsson	CR	Rel-15	36.306	15.11.0	1885	-	F	LCS_LTE_acc_enh-Core

- Revised in R2-2406013 (email discussion [AT126][400])

R2-2404750	Correction to mandatory 80ms scheduling offset for positioning SI acquisition	Huawei, HiSilicon, Ericsson	CR	Rel-16	36.306	16.12.0	1886	-	A	LCS_LTE_acc_enh-Core

- Revised in R2-2406014 (email discussion [AT126][400])

R2-2404751	Correction to mandatory 80ms scheduling offset for positioning SI acquisition	Huawei, HiSilicon, Ericsson	CR	Rel-17	36.306	17.6.0	1887	-	A	LCS_LTE_acc_enh-Core

- Revised in R2-2406015 (email discussion [AT126][400])

R2-2404752	Correction to mandatory 80ms scheduling offset for positioning SI acquisition	Huawei, HiSilicon, Ericsson	CR	Rel-18	36.306	18.1.0	1888	-	A	LCS_LTE_acc_enh-Core

- Revised in R2-2406016 (email discussion [AT126][400])

[AT126][401][POS] Rel-15 LTE positioning CR (Huawei)

Scope: Check the CR in R2-2404749 (and mirrors in R2-2404750 / R2-2404751 / R2-2404752).

Intended outcome: Agreed CRs (without CB if possible)

Deadline:  Thursday 2024-05-23 1000 JST

R2-2406012	Summary for [AT126][401][POS] Rel-15 LTE positioning	Huawei, HiSilicon, Ericsson	discussion	Rel-18	LCS_LTE_acc_enh-Core

- Noted (email discussion [AT126][401])

R2-2406013	Correction to mandatory 80ms scheduling offset for positioning SI acquisition	Huawei, HiSilicon, Ericsson	CR	Rel-15	36.306	15.11.0	1885	1	F	LCS_LTE_acc_enh-Core

- Agreed (email discussion [AT126][401])

R2-2406014	Correction to mandatory 80ms scheduling offset for positioning SI acquisition	Huawei, HiSilicon, Ericsson	CR	Rel-16	36.306	16.12.0	1886	1	A	LCS_LTE_acc_enh-Core

- Agreed (email discussion [AT126][401])

R2-2406015	Correction to mandatory 80ms scheduling offset for positioning SI acquisition	Huawei, HiSilicon, Ericsson	CR	Rel-17	36.306	17.6.0	1887	1	A	LCS_LTE_acc_enh-Core

- Agreed (email discussion [AT126][401])

R2-2406016	Correction to mandatory 80ms scheduling offset for positioning SI acquisition	Huawei, HiSilicon, Ericsson	CR	Rel-18	36.306	18.1.0	1888	1	A	LCS_LTE_acc_enh-Core

- Agreed (email discussion [AT126][401])

# 5	NR Rel-15 and Rel-16

Essential corrections only.

Tdoc Limitation: 2 tdocs in total for all sub agenda items NOTE: some agenda items have additional Tdoc limits.

In case a correction need to be reflected in both NR TS and LTE TS, the corrections should be submitted under one single AI (so the NR and LTE correction can be treatee together), the sub-Ais below this

## 5.1	Common

Includes the following WIs and input that doesn’t fit elsewhere.

(NR_newRAT-Core; leading WG: RAN1; REL-15; started: Mar. 17; closed: Jun. 19: WID: RP-191971)

(NR_IAB-Core; leading WG: RAN2; REL-16; started: Dec 18; target Aug 20; WID: RP-200840)

(NR_unlic-Core; leading WG: RAN1; REL-16; started: Dec 18; Closed June 20; WID: RP-192926).

(NR_IIOT-Core; leading WG: RAN2; REL-16; started: Mar 19; Completed: Jun 20; WID: RP-200797)

(NR_UE_pow_sav-Core; leading WG: RAN1; REL-16; started: Mar 19; Completed Jun 20; WID: RP-200494).

(NR_2step_RACH-Core; leading WG: RAN1; REL-16; started: Dec 18; Completed: June 20; WID: RP-200085).

(SRVCC_NR_to_UMTS-Core; leading WG: RAN2; REL-16; started: Dec 18; Completed; Mar 20; WID: RP-190713)

(RACS-RAN-Core, leading WG: RAN2; REL-16; started: Mar 19; completed: Jun 20; WID: RP-191088)

(NG_RAN_PRN-Core; leading WG: RAN3; REL-16; started: Mar 19; completed: June 20; WID: RP-200122)

(NR_eMIMO-Core, leading WG: RAN1; REL-16; started: Jun 18; target; Aug 20; WID: RP-200474)

(NR_CLI_RIM; leading WG: RAN1; REL-16; started: Dec 18; Completed: Jun 20; WID: RP-191997)

(NR_L1enh_URLLC-Core, leading WG: RAN1; REL-16; Completed: June 20; WID: RP-191584)

(LTE_NR_DC_CA_enh-Core; leading WG: RAN2; REL-16; started: Jun 18; Target Aug 20; WI RP-200791)

(NR_Mob_enh-Core; leading WG: RAN2; REL-16; started: Jun 18; Completed June 20; WID: RP-192277).

(NR_SON_MDT-Core; leading WG: RAN3; REL-16; started: Jun 19; Completed June 20; WID: RP-191776)

(NR_HST, NR_RRM_enh-Core, NR_RF_FR1, NR_RF_FR2_req_enh, NR_n66_BW, LTE_NR_B41_Bn41_PC29dBm-Core, NR_CSIRS_L3meas,)

(NR TEI16)

LTE mob enh corrections that are common with NR mobility enhancements should be submitted to this AI.

### 5.1.1	Stage 2 and Organisational

Incoming LSs, etc. You should discuss your stage 2 CRs with the specification rapporteurs before submission. Includes impact to 38.300, 36.300, 37.340

R2-2404124	Reply LS on combination of HST and RRM relaxation (R4-2403532; contact: Apple)	RAN4	LS in	Rel-16	NR_HST, NR_UE_pow_sav-Core	To:RAN2

Noted

R2-2404133	LS on 5GS missing CBC support for shared networks (S2-2405210; contact: Ericsson)	SA2	LS in	Rel-15	5GS_Ph1	To:CT1	Cc:RAN2, RAN3, SA1

Noted

#### 5.1.1.0	In-principle agreed CRs

R2-2405727	Correction to UE capability description for fallback BC behavior	Ericsson, Nokia (Rapporteur)	CR	Rel-15	38.300	15.16.0	0843	2	F	NR_newRAT-Core	R2-2404360

R2-2405728	Correction to UE capability description for fallback BC behavior	Ericsson, Nokia (Rapporteur)	CR	Rel-16	38.300	16.15.0	0844	2	A	NR_newRAT-Core	R2-2404361

R2-2405729	Correction to UE capability description for fallback BC behavior	Ericsson, Nokia (Rapporteur)	CR	Rel-17	38.300	17.8.0	0845	2	A	NR_newRAT-Core	R2-2404362

R2-2405730	Correction to UE capability description for fallback BC behavior	Ericsson, Nokia (Rapporteur)	CR	Rel-18	38.300	18.1.0	0846	2	A	NR_newRAT-Core	R2-2404363

R2-2404670	Clarification on the combination of HST and RRM measurement relaxation	Apple, Ericsson, Nokia (Rapporteur)	CR	Rel-16	38.300	16.15.0	0839	1	F	NR_HST, NR_UE_pow_sav-Core	R2-2402869

R2-2405624	Clarification on the combination of HST and RRM measurement relaxation	Apple, Ericsson, Nokia (Rapporteur)	CR	Rel-17	38.300	17.8.0	0840	2	A	NR_UE_pow_sav-Core, NR_HST	R2-2404671

R2-2405625	Clarification on the combination of HST and RRM measurement relaxation	Apple, Ericsson, Nokia (Rapporteur)	CR	Rel-18	38.300	18.1.0	0841	2	A	NR_UE_pow_sav-Core, NR_HST	R2-2404672

All above are agreed

Old revisions

R2-2404671	Clarification on the combination of HST and RRM measurement relaxation	Apple, Ericsson, Nokia (Rapporteur)	CR	Rel-17	38.300	17.8.0	0840	1	A	NR_HST, NR_UE_pow_sav-Core	R2-2402870	Revised

R2-2404672	Clarification on the combination of HST and RRM measurement relaxation	Apple, Ericsson, Nokia (Rapporteur)	CR	Rel-18	38.300	18.1.0	0841	1	A	NR_HST, NR_UE_pow_sav-Core	R2-2402871	Revised

R2-2404360	Correction to UE capability description for fallback BC behavior	Ericsson, Nokia (Rapporteur)	CR	Rel-15	38.300	15.16.0	0843	1	F	NR_newRAT-Core	R2-2403004

R2-2404361	Correction to UE capability description for fallback BC behavior	Ericsson, Nokia (Rapporteur)	CR	Rel-16	38.300	16.15.0	0844	1	A	NR_newRAT-Core	R2-2403005

R2-2404362	Correction to UE capability description for fallback BC behavior	Ericsson, Nokia (Rapporteur)	CR	Rel-17	38.300	17.8.0	0845	1	A	NR_newRAT-Core	R2-2403006

R2-2404363	Correction to UE capability description for fallback BC behavior	Ericsson, Nokia (Rapporteur)	CR	Rel-18	38.300	18.1.0	0846	1	A	NR_newRAT-Core	R2-2403007

#### 5.1.1.1	Other

### 5.1.2	User Plane corrections

#### 5.1.2.0	In Principle Agreed CRs

#### 5.1.2.1	MAC

User Plane corrections will be handled in the User Plane break out session

R2-2405414	Clarification on CG re-initialization upon SCell activation	ASUSTeK	CR	Rel-16	38.321	16.15.0	1862	-	F	LTE_NR_DC_CA_enh-Core, TEI16

-	LG thinks that there is no need for CR. Qualcomm thinks intention is right but the CR is not needed

=>	The CR is not pursued

R2-2405415	Clarification on CG re-initialization upon SCell activation	ASUSTeK	CR	Rel-17	38.321	17.8.0	1863	-	A	LTE_NR_DC_CA_enh-Core, TEI16

R2-2405416	Clarification on CG re-initialization upon SCell activation	ASUSTeK	CR	Rel-18	38.321	18.1.0	1864	-	A	LTE_NR_DC_CA_enh-Core, TEI16

R2-2405417	DRX timer handling regarding dormant BWP and SCell deactivation	ASUSTeK	CR	Rel-16	38.321	16.15.0	1865	-	F	LTE_NR_DC_CA_enh-Core, TEI16

-	LG doesn’t think this is needed.  Huawei, Oppo, Nokia and Samsung doesn’t support this change. Samsung thinks that the only case is the carrier aggregation scenario and this is changing the legacy. Nokia thinks that this is a NBC change.

=>	the CR is not pursued

R2-2405418	DRX timer handling regarding dormant BWP and SCell deactivation	ASUSTeK	CR	Rel-17	38.321	17.8.0	1866	-	A	LTE_NR_DC_CA_enh-Core, TEI16

R2-2405419	DRX timer handling regarding dormant BWP and SCell deactivation	ASUSTeK	CR	Rel-18	38.321	18.1.0	1867	-	A	LTE_NR_DC_CA_enh-Core, TEI16

#### 5.1.2.2	RLC PDCP SDAP BAP

#### 5.1.2.3	Other

User plane related corrections that should be handled in User plane break out session.

### 5.1.3	Control Plane corrections

#### 5.1.3.0	In-principle agreed CRs

R2-2404962	Dummy the rrc-TransactionIdentifier field from IABOtherInformation	Ericsson	CR	Rel-16	38.331	16.16.0	4702	1	F	NR_IAB-Core	R2-2403171

R2-2404963	Dummy the rrc-TransactionIdentifier field from IABOtherInformation	Ericsson	CR	Rel-1F5850

7	38.331	17.8.0	4703	1	A	NR_IAB-Core	R2-2403172

R2-2404964	Dummy the rrc-TransactionIdentifier field from IABOtherInformation	Ericsson	CR	Rel-18	38.331	18.1.0	4704	1	A	NR_IAB-Core	R2-2403173

All above are agreed

#### 5.1.3.1	NR RRC

Corrections to 38331, and related change to other TS if applicable, e.g. 36331, Stage-2 etc.

Configured grant

R2-2404465	Clarification of configured grant in shared spectrum	Xiaomi	CR	Rel-16	38.331	16.16.0	4778	-	F	NR_unlic-Core

Agreed

R2-2404466	Clarification of configured grant in shared spectrum	Xiaomi, OPPO	CR	Rel-17	38.331	17.8.0	4779	-	A	NR_unlic-Core

Category should be F instead of A, with this the CR is Agreed unseen in R2-2405808

R2-2405808	Clarification of configured grant in shared spectrum	Xiaomi, OPPO	CR	Rel-17	38.331	17.8.0	4779	1	F	NR_unlic-Core

=> Agreed

=> Coversheet update by MCC: Rel-17 CR with a Rel-16 WI needs TEI17 in addition.

R2-2406137	Clarification of configured grant in shared spectrum	Xiaomi, OPPO	CR	Rel-17	38.331	17.8.0	4779	2	F	NR_unlic-Core, TEI17

=> Agreed

R2-2404467	Clarification of configured grant in shared spectrum	Xiaomi, OPPO	CR	Rel-18	38.331	18.1.0	4780	-	A	NR_unlic-Core

Agreed

=> Coversheet update by MCC: a Cat A CR must have the same WIs as the Cat F CR.

R2-2406141	Clarification of configured grant in shared spectrum	Xiaomi, OPPO	CR	Rel-18	38.331	18.1.0	4780	1	A	NR_unlic-Core, TEI17

=> Agreed

-	Qualcomm agrees to those, but think that the R17 CR is not a Cat A since it has different changes.

SRVCC

R2-2405724	Clarification on UE capability reporting for UTRA-FDD	MediaTek Inc.	CR	Rel-16	38.331	16.16.0	4804	1	F	SRVCC_NR_to_UMTS-Core	R2-2405009

R2-2405725	Clarification on UE capability reporting for UTRA-FDD	MediaTek Inc.	CR	Rel-17	38.331	17.8.0	4805	1	A	SRVCC_NR_to_UMTS-Core	R2-2405010

R2-2405726	Clarification on UE capability reporting for UTRA-FDD	MediaTek Inc.	CR	Rel-18	38.331	18.1.0	4806	1	A	SRVCC_NR_to_UMTS-Core	R2-2405011

-	CATT agrees with this CR and think that this was the original intention and think that there is no point in including UTRA-FDD capas if the UE doesn’t support SRVCC. Lenovo thinks that the NW would simply not ask the UTRA-FDD capas if the UE doesn’t support SRVCC, and if the NW does the UE can ignore the request, Lenovo thinks we can clarify this in chair notes. Ericsson thinks the NW doesn’t know the UE’s SRVCC capas at that point in time, but agrees with Lenovo that the UE can ignore sending the capas. Apple wants to add a note saying that NW should not ask for the capas. Ericsson is not OK with such a note. Nokia understand the issue but think the spec is fine as it is.

RAN2 understands that the UE can ignore sending UTRA-FDD capabilities to the NR gNB if it does not support SRVCC. No spec change needed.

ETWS/CMAS

R2-2405175	Clarification on SIB1 reception for ETWS/CMAS	Samsung, Ericsson	CR	Rel-15	38.331	15.25.0	4817	-	F	NR_newRAT-Core

R2-2405176	Clarification on SIB1 reception for ETWS/CMAS	Samsung, Ericsson	CR	Rel-16	38.331	16.16.0	4818	-	A	NR_newRAT-Core

R2-2405177	Clarification on SIB1 reception for ETWS/CMAS	Samsung, Ericsson	CR	Rel-17	38.331	17.8.0	4819	-	A	NR_newRAT-Core

R2-2405178	Clarification on SIB1 reception for ETWS/CMAS	Samsung, Ericsson	CR	Rel-18	38.331	18.1.0	4820	-	A	NR_newRAT-Core

-	Nokia thinks this clarification is not needed since SIB6, SIB7 and SIB8 in the current note is sufficient and implies SIB1. Samsung thinks that it was discussed earlier and common understanding that the behaviour applies also to SIB1 so it would be good to capture in the note. Ericsson also want to capture this explicitly since this has been a long and tedious discussion, as a compromise we can consider capturing it from R18. Qualcomm agrees with the CRs. Nokia could accept the CRs if there is a majority wanting them, but thinks this is too much details anyway.

All 4 agreed

Security algorithms

R2-2405393	Discussion on Applying Security Algorithm in EN-DC	Samsung	discussion	Rel-15

•	Solution 1

Upon both HO and the configuration of RBs configured with NR PDCP, if UE has received RRCConnectionReconfiguration including NR security algorithms only, for RBs configured with LTE PDCP, UE shall apply the received NR security algorithms (or LTE security algorithms identical to the received NR security algorithm), and release LTE security algorithms the currently configured.

•	Solution 2

Upon both HO and the configuration of RBs configured with NR PDCP, if UE has received RRCConnectionReconfiguration including NR security algorithms only, for RBs configured with LTE PDCP, UE shall ignore the received NR security algorithms, and apply LTE security algorithms the currently configured.

•	Solution 3

Upon both HO and the configuration of RBs configured with NR PDCP, for RBs configured with LTE PDCP, the network always configures LTE security algorithms in the IE SecurityConfigHO

-	Ericsson think this is not a reasonable scenario that the NW only changes the MN security, not at the SN, so it never happen. Qualcomm think that its up to NW. CATT think that NW can handle it. Huawei agrees NW should handle this for MN-terminated bearers but it is a different case for SN-terminated bearers.

RAN2 understands that for MN-terminated bearers, upon both HO and configuration of RBs, if the NW changes security algorithms, the network aligns the security algorithms for NR and LTE PDCP.

Transaction identifier

R2-2405675	Clarification on when to include the rrc-TransactionIdentifier	Ericsson	CR	Rel-15	38.331	15.25.0	4852	-	F	NR_newRAT-Core

R2-2405676	Clarification on when to include the rrc-TransactionIdentifier	Ericsson	CR	Rel-16	38.331	16.16.0	4853	-	A	NR_newRAT-Core

R2-2405677	Clarification on when to include the rrc-TransactionIdentifier	Ericsson	CR	Rel-17	38.331	17.8.0	4854	-	A	NR_newRAT-Core

R2-2405678	Clarification on when to include the rrc-TransactionIdentifier	Ericsson	CR	Rel-18	38.331	18.1.0	4855	-	A	NR_newRAT-Core

Not pursued

Old revisions

R2-2405009	Clarification on UE capability reporting for UTRA-FDD	MediaTek Inc.	CR	Rel-16	38.331	16.16.0	4804	-	F	SRVCC_NR_to_UMTS-Core

R2-2405010	Clarification on UE capability reporting for UTRA-FDD	MediaTek Inc.	CR	Rel-17	38.331	17.8.0	4805	-	A	SRVCC_NR_to_UMTS-Core

R2-2405011	Clarification on UE capability reporting for UTRA-FDD	MediaTek Inc.	CR	Rel-18	38.331	18.1.0	4806	-	A	SRVCC_NR_to_UMTS-Core

#### 5.1.3.2	UE capabilities

UE cap corrections 38306, 38331

Bandwidth support

R2-2404450	Correction to mandatory supported capability of channel bandwidth	CAICT	CR	Rel-15	38.306	15.24.0	1092	-	F	NR_newRAT-Core

R2-2404451	Correction to mandatory supported capability of channel bandwidth	CAICT	CR	Rel-16	38.306	16.16.0	1093	-	A	NR_newRAT-Core	Late

R2-2404452	Correction to mandatory supported capability of channel bandwidth	CAICT	CR	Rel-17	38.306	17.8.0	1094	-	A	NR_newRAT-Core	Late

R2-2404453	Correction to mandatory supported capability of channel bandwidth	CAICT	CR	Rel-18	38.306	18.1.0	1095	-	A	NR_newRAT-Core	Late

-	Huawei thinks this is NBC. Ericsson thinks that we should not touch this now, the current spec works. Qualcomm acknowledges that this part of the spec is not beautify but it works and better not to change it. Can discuss more offline if a change is needed.

Not pursued, unless critical issues are found

Two PUCCH groups

R2-2404701	Discussion on UE capabilities for two PUCCH groups	Qualcomm Incorporated, Ericsson, ZTE Corporation, Sanechips	discussion	Rel-15	NR_newRAT-Core

RAN2 will not specify that twoPUCCH-Group (FG6-7) is a prerequisite of diffNumerologyAcrossPUCCH-Group (FG6-8).

R2-2404702	Correction on prerequisite feature for csi-ReportingCrossPUCCH-Grp-r16	Qualcomm Incorporated	CR	Rel-16	38.306	16.16.0	1018	2	F	TEI16	R2-2402956

R2-2404703	Correction on prerequisite feature for csi-ReportingCrossPUCCH-Grp-r16	Qualcomm Incorporated	CR	Rel-17	38.306	17.8.0	1019	2	A	TEI16	R2-2402957

R2-2404704	Correction on prerequisite feature for csi-ReportingCrossPUCCH-Grp-r16	Qualcomm Incorporated	CR	Rel-18	38.306	18.1.0	1020	2	A	TEI16	R2-2402958

All 3 agreed

PUSCH-less SCell

R2-2404724	Clarification on the SRS Carrier Switching for the PUSCH-less Cell (r15)	ZTE Corporation, Sanechips, Ericsson, Samsung	CR	Rel-15	38.306	15.24.0	1100	-	F	NR_newRAT-Core

R2-2404725	Clarification on the SRS Carrier Switching for the PUSCH-less Cell (r16)	ZTE Corporation, Sanechips, Ericsson, Samsung	CR	Rel-16	38.306	16.16.0	1101	-	A	NR_newRAT-Core

R2-2404726	Clarification on the SRS Carrier Switching for the PUSCH-less Cell (r17)	ZTE Corporation, Sanechips, Ericsson, Samsung	CR	Rel-17	38.306	17.8.0	1102	-	A	NR_newRAT-Core

R2-2404727	Clarification on the SRS Carrier Switching for the PUSCH-less Cell (r18)	ZTE Corporation, Sanechips, Ericsson, Samsung	CR	Rel-18	38.306	18.1.0	1103	-	A	NR_newRAT-Core

-	ZTE explains that they received some comments offline and the CRs needs to be revised. Also there are cover page and styling issues.

[AT126][752][Maint] Clarification on the SRS Carrier Switching for the PUSCH-less Cell (ZTE)

Scope:

    - Polish CRs, including fixing styling issues, etc.

Intended outcome:

    - Agreeable CRs in R2-2405809, R2-2405810, R2-2405811 and R2-2405812 (ZTE)

Deadline:

    - Thursday lunch. Intention is to agree the CRs over email.

R2-2405809	Clarification on the SRS Carrier Switching for the PUSCH-less Cell (r15)	ZTE Corporation, Sanechips, Ericsson, Samsung	CR	Rel-15	38.306	15.24.0	1100	1	F	NR_newRAT-Core	R2-2404724

R2-2405810	Clarification on the SRS Carrier Switching for the PUSCH-less Cell (r16)	ZTE Corporation, Sanechips, Ericsson, Samsung	CR	Rel-16	38.306	16.16.0	1101	1	A	NR_newRAT-Core	R2-2404725

R2-2405811	Clarification on the SRS Carrier Switching for the PUSCH-less Cell (r17)	ZTE Corporation, Sanechips, Ericsson, Samsung	CR	Rel-17	38.306	17.8.0	1102	1	A	NR_newRAT-Core	R2-2404726

R2-2405812	Clarification on the SRS Carrier Switching for the PUSCH-less Cell (r18)	ZTE Corporation, Sanechips, Ericsson, Samsung	CR	Rel-18	38.306	18.1.0	1103	1	A	NR_newRAT-Core	R2-2404727

All 4 agreed

Parallel Tx

R2-2404728	Clarification on the parallel Tx Capability (r15)	ZTE Corporation, Sanechips, Ericsson	CR	Rel-15	38.306	15.24.0	1104	-	F	NR_newRAT-Core

R2-2404729	Clarification on the parallel Tx Capability (R16)	ZTE Corporation, Sanechips, Ericsson	CR	Rel-16	38.306	16.16.0	1105	-	A	NR_newRAT-Core, NR_2step_RACH

R2-2404730	Clarification on the parallel Tx Capability (R17)	ZTE Corporation, Sanechips, Ericsson	CR	Rel-17	38.306	17.8.0	1106	-	A	TEI17, NR_newRAT-Core, NR_2step_RACH, NR_IIOT_URLLC_enh

R2-2404731	Clarification on the parallel Tx Capability (R18)	ZTE Corporation, Sanechips, Ericsson	CR	Rel-18	38.306	18.1.0	1107	-	A	TEI17, NR_newRAT-Core, NR_2step_RACH, NR_IIOT_URLLC_enh

Fix styling issues. Agreed unseen in R2-2405813, R2-2405814, R2-2405815 and R2-2405816.

=> Coversheet update by MCC: wrong rev value in R2-2405814, R2-2405815 and R2-2405816. Wrong meeting dates in 2405814 and R2-2405816.

=> Revised in R2-2406116, R2-2406117 and R2-2406118

R2-2406116	Clarification on the parallel Tx Capability (R16)	ZTE Corporation, Sanechips, Ericsson	CR	Rel-16	38.306	16.16.0	1105	3	A	NR_newRAT-Core, NR_2step_RACH

=> Agreed

R2-2406117	Clarification on the parallel Tx Capability (R17)	ZTE Corporation, Sanechips, Ericsson	CR	Rel-17	38.306	17.8.0	1106	3	A	TEI17, NR_newRAT-Core, NR_2step_RACH, NR_IIOT_URLLC_enh

=> Agreed

R2-2406118	Clarification on the parallel Tx Capability (R18)	ZTE Corporation, Sanechips, Ericsson	CR	Rel-18	38.306	18.1.0	1107	3	A	TEI17, NR_newRAT-Core, NR_2step_RACH, NR_IIOT_URLLC_enh

=> Agreed

SDL

R2-2405505	Clarification on xDD differentiation for SDL bands	Huawei, HiSilicon	CR	Rel-15	38.306	15.24.0	1116	-	F	NR_newRAT-Core

R2-2405506	Clarification on xDD differentiation for SDL bands	Huawei, HiSilicon	CR	Rel-16	38.306	16.16.0	1117	-	A	NR_newRAT-Core

R2-2405507	Clarification on xDD differentiation for SDL bands	Huawei, HiSilicon	CR	Rel-17	38.306	17.8.0	1118	-	A	NR_newRAT-Core

R2-2405508	Clarification on xDD differentiation for SDL bands	Huawei, HiSilicon	CR	Rel-18	38.306	18.1.0	1119	-	A	NR_newRAT-Core

-	Nokia has concerns with wording and want to polish it. Qualcomm wants to add SDL in the abbreviation section, or at least clarify somehow somewhere what SDL is.

[AT126][753][Maint] Clarification on xDD differentiation for SDL bands (Huawei)

Scope:

    - Polish CRs to sort out and/or issues and see if clarification of SDL is needed.

Intended outcome:

    - Agreeable CRs in R2-2405817, R2-2405818, R2-2405819 and R2-2405820, (Company)

Deadline:

    - Thursday lunch. Intention is to agree the CRs over email.

R2-2405817	Clarification on xDD differentiation for SDL bands	Huawei, HiSilicon	CR	Rel-15	38.306	15.24.0	1116	1	F	NR_newRAT-Core	R2-2405505

R2-2405818	Clarification on xDD differentiation for SDL bands	Huawei, HiSilicon	CR	Rel-16	38.306	16.16.0	1117	1	A	NR_newRAT-Core	R2-2405506

R2-2405819	Clarification on xDD differentiation for SDL bands	Huawei, HiSilicon	CR	Rel-17	38.306	17.8.0	1118	1	A	NR_newRAT-Core	R2-2405507

R2-2405820	Clarification on xDD differentiation for SDL bands	Huawei, HiSilicon	CR	Rel-18	38.306	18.1.0	1119	1	A	NR_newRAT-Core	R2-2405508

All 4 agreed

Misc

R2-2404529	Miscellaneous non-controversial rapporteur corrections	Intel Corporation	CR	Rel-16	38.306	16.16.0	1096	-	F	NR_newRAT-Core, NR_eMIMO-Core, NR_unlic-Perf, 5G_V2X_NRSL-Core

Agreed

#### 5.1.3.3	Other

This agenda item addresses the idle and inactive behaviour specified in 38.304 or 36.304, LTE-specific changes for the applicable WIs, Other parts not covered elsewhere.

## 5.2	NR V2X

(5G_V2X_NRSL-Core; leading WG: RAN1; REL-16; started: Mar 19; target; Aug 20; WID: RP-200129).

CR rapporteurs will take care of miscellaneous CRs to collect small changes. Please contact / coordinate with CR rapporteur company first for small changes (e.g. non-controversial clarification/correction, editorial correction, etc.).

Tdoc Limitation: 1 tdocs

R2-2404491	Correction to RRC for SL configured grant	Ericsson	CR	Rel-16	38.331	16.16.0	4782	-	F	5G_V2X_NRSL-Core

R2-2404492	Correction to RRC for SL configured grant	Ericsson	CR	Rel-17	38.331	17.8.0	4783	-	A	5G_V2X_NRSL-Core

R2-2404493	Correction to RRC for SL configured grant	Ericsson	CR	Rel-18	38.331	18.1.0	4784	-	A	5G_V2X_NRSL-Core

- Withdrawn

R2-2405346	Corrections for SL RLC mode indication	ZTE, Sanechips	CR	Rel-16	38.331	16.16.0	4825	-	F	5G_V2X_NRSL-Core

R2-2405347	Corrections for SL RLC mode indication	ZTE, Sanechips	CR	Rel-17	38.331	17.8.0	4826	-	A	5G_V2X_NRSL-Core

R2-2405348	Corrections for SL RLC mode indication	ZTE, Sanechips	CR	Rel-18	38.331	18.1.0	4827	-	A	5G_V2X_NRSL-Core

- Agreed.

## 5.3	NR Positioning Support

(NR_newRAT-Core; leading WG: RAN1; REL-15; started: Mar. 17; closed: Jun. 19: WID: RP-191971)

(NR_pos-Core; leading WG: RAN1; REL-16; started: Mar 19; target; Jun 20; WID: RP-200218).

(NR TEI16 Positioning)

Stage 2 corrections shall be discussed with the specification rapporteur (Sven Fischer sfischer@qti.qualcomm.com) before submission. Stage 2 CRs not discussed with the specification rapporteur will not be treated.

Tdoc Limitation: 1 tdoc

AIP CRs

R2-2405252	Missing Conditionally mandatory features without UE radio access capability parameters for 80ms scheduling offset for positioning SI acquisition	Ericsson	CR	Rel-16	38.306	16.16.0	1087	2	F	NR_pos-Core	R2-2403797

- Revised in R2-2405854

R2-2405854	Missing Conditionally mandatory features without UE radio access capability parameters for 80ms scheduling offset for positioning SI acquisition	Ericsson	CR	Rel-16	38.306	16.16.0	1087	3	F	NR_pos-Core	R2-2403797

- Agreed

R2-2405251	Missing Conditionally mandatory features without UE radio access capability parameters for 80ms scheduling offset for positioning SI acquisition	Ericsson	CR	Rel-17	38.306	17.8.0	1088	2	A	NR_pos-Core	R2-2403798

- Revised in R2-2405853

R2-2405853	Missing Conditionally mandatory features without UE radio access capability parameters for 80ms scheduling offset for positioning SI acquisition	Ericsson	CR	Rel-17	38.306	17.8.0	1088	3	A	NR_pos-Core	R2-2403798

- Agreed

R2-2405250	Missing Conditionally mandatory features without UE radio access capability parameters for 80ms scheduling offset for positioning SI acquisition	Ericsson	CR	Rel-18	38.306	18.1.0	1086	2	A	NR_pos-Core	R2-2403799

- Revised in R2-2405852

R2-2405852	Missing Conditionally mandatory features without UE radio access capability parameters for 80ms scheduling offset for positioning SI acquisition	Ericsson	CR	Rel-18	38.306	18.1.0	1086	3	A	NR_pos-Core	R2-2403799

- Agreed

Other documents

R2-2404753	Correction to Positioning SRS Configuration	Huawei, HiSilicon	CR	Rel-16	38.331	16.16.0	4791	-	F	NR_pos-Core

- Revised in R2-2405867

R2-2404754	Correction to Positioning SRS configuration	Huawei, HiSilicon	CR	Rel-17	38.331	17.8.0	4792	-	A	NR_pos-Core

- Revised in R2-2405868

R2-2404755	Correction to Positioning SRS configuration	Huawei, HiSilicon	CR	Rel-18	38.331	18.1.0	4793	-	A	NR_pos-Core

- Revised in R2-2405869

Discussion:

Lenovo agree with the intention but think it would be better to introduce new field descriptions instead of adding the behaviour in the resource type field description.

vivo think it should be clarified that the SRS configuration is for positioning.  Ericsson think it should also apply for legacy; vivo understand that this case is specific to positioning.

Samsung agree with Lenovo.

Huawei indicate that the field descriptions are separate for legacy (resource set level) and positioning (resource level).

Intel also agree with Lenovo and Samsung and clarify that the intention is to introduce the field description for slotOffset-r16.  Huawei think this is a bit divergent from MIMO SRS but possible to do.

Nokia felt the differing granularity between MIMO and positioning caused some confusion.  They would like to see a revised CR.

[AT126][402][POS] Slot offset for positioning SRS configuration (Huawei)

Scope: Update the CR in R2-2404753 (and its mirror CRs) to introduce the change in a field description for the field slotOffset-r16.

Intended outcome: Agreeable CR (with CB) in R2-2405867 (Rel-16), mirror CRs in R2-2405868 (Rel-17) and R2-2405869 (Rel-18)

Deadline:  Thursday 2024-05-23 1000 JST

R2-2405867	Correction to Positioning SRS Configuration	Huawei, HiSilicon	CR	Rel-16	38.331	16.16.0	4791	1	F	NR_pos-Core

- Styles to be corrected

- Revision history to be added in coversheet

- Agreed with this change as R2-2405983

Discussion:

Nokia think there are some formatting issues pointed out late in the discussion.

R2-2405868	Correction to Positioning SRS configuration	Huawei, HiSilicon	CR	Rel-17	38.331	17.8.0	4792	1	A	NR_pos-Core

- Styles to be corrected

- Revision history to be added in coversheet

- Agreed with this change as R2-2405984

R2-2405869	Correction to Positioning SRS configuration	Huawei, HiSilicon	CR	Rel-18	38.331	18.1.0	4793	1	A	NR_pos-Core

- Styles to be corrected

- Revision history to be added in coversheet

- Agreed with this change as R2-2405985

R2-2405104	Corrections on the prerequisite feature groups of NR-Multi-RTT-MeasurementCapability	Xiaomi	CR	Rel-16	37.355	16.13.0	0505	-	F	NR_pos-Core

- Not pursued

R2-2405105	Corrections on the prerequisite feature groups of NR-Multi-RTT-MeasurementCapability	Xiaomi	CR	Rel-17	37.355	17.8.0	0506	-	A	NR_pos-Core

- Not pursued

R2-2405106	Corrections on the prerequisite feature groups of NR-Multi-RTT-MeasurementCapability	Xiaomi	CR	Rel-18	37.355	18.1.0	0507	-	A	NR_pos-Core

- Not pursued

Discussion:

Lenovo agree with the intention but note that the IE has been extended in Rel-17 and -18 with contents that are not conditional, so it seems not right to have the condition at IE level.  They think instead it should be in the field description.

CATT think there is no need for the new wording in light of the last two fields in the IE.

Qualcomm think the CR is not needed; the UE needs SRS for multi-RTT, of course, but they think this is obvious without stating it here.

Nokia also wonder why this change is in the LPP spec instead of being clarified in RRC.

CATT understand that the prerequisite capability is already captured by the last two fields.

Intel have some sympathy with the intention; they think normally we would capture it in the field description.

Huawei agree with Qualcomm that nothing is needed; the UL-SRSp capability is mandatory present in the multi-RTT capabilities, so this already implies that SRSp has to be supported and the current sentence is not wrong.  Intel understand after checking that with the current structure, if the UE indicates support of multi-RTT, it always has to indicate support of the SRS capability.

Xiaomi think there is no technical impact if the CR is not agreed, but the precondition features should generally be aligned with the feature list.  Intel think it is OK as long as there is no UE/NW misalignment.

R2-2405574	OAM solution for the provisioning PRS Assistance Data to LMF	Ericsson	CR	Rel-16	38.305	16.10.0	0169	-	F	NR_pos-Core

- Not pursued

R2-2405573	OAM solution for the provisioning PRS Assistance Data to LMF	Ericsson	CR	Rel-17	38.305	17.7.0	0168	-	A	NR_pos-Core

- Not pursued

R2-2405572	OAM solution for the provisioning PRS Assistance Data to LMF	Ericsson	CR	Rel-18	38.305	18.1.0	0167	-	A	NR_pos-Core

- Not pursued

Discussion:

ZTE wonder what “DL-PRS related assistance data” means in this context; they think the mobile TRP information cannot be set by OAM.

vivo think the contribution is based on a RAN3 agreement and RAN3 could do the stage 2 change.

Nokia would also prefer for RAN3 to handle it, and they think that both OAM and implementation options are possible in any case and not precluded from RAN2 spec perspective.

Qualcomm think nothing is broken in Rel-16 with or without the NOTE.

Ericsson intended the NOTE mainly for the beam-related configuration.  They also think there is some connection to the NTN text added in Rel-18.  Qualcomm think this is a different situation and we already have everything specified in NRPPa, but not excluding the OAM solution, whereas NTN uses OAM for things that are not in NRPPa.

# 6	NR Rel-17

Essential corrections only.  Editorial/clarifications should be sent to be reviewed and approved by spec rapporteurs prior to submission.  Editorials should only be submitted by spec rapporteurs.

## 6.1	Common

(NR_MG_enh-Core; leading WG: RAN4; REL-17; WID: RP-211591)

(NR_UDC_enh-Core; leading WG: RAN2; REL-17; WID: RP-211203)

(NG_RAN_PRN_enh-Core; leading WG: RAN3; REL-17; WID: RP-202363)

(NR_IAB_enh-Core; leading WG: RAN2; REL-17; WID: RP-211548)

(NR_UE_pow_sav_enh-Core; leading WG: RAN2; REL-17; WID: RP-212630)

(LTE_NR_DC_enh2-Core; leading WG: RAN2; REL-17; WID: RP-201040)

(LTE_NR_MUSIM-Core; leading WG: RAN2; REL-17; WID: RP-212610)

(NR_Slice -Core; leading WG: RAN2; REL-17; WID: RP-212534)

(NR_QoE-Core; leading WG: RAN3; REL-17; WID: RP-211406)

(NR_ext_to_71GHz-Core; leading WG: RAN1; REL-17; WID: RP-212637)

(NR_cov_enh-Core; leading WG: RAN1; REL-17; WID: RP-211566): non-RACH-indication parts

(NR_redcap-Core; leading WG: RAN1; REL-17; WID: RP-211574)

(NR_feMIMO-Core; leading WG: RAN1; REL-17; WID: RP-212535)

(NR_SmallData_INACTIVE-Core, leading WG: RAN2; REL-17; WID: RP-212594)

(NR_IIOT_URLLC_enh-Core; leading WG: RAN2; REL-17; WID: RP-210854)

(NR_MBS-Core; leading WG: RAN2; REL-17; WID: RP-201038)

(NR_ENDC_SON_MDT_enh-Core; leading WG: RAN3; REL-17; WID: RP-201281)

(NR_NTN_solutions-Core; leading WG: RAN2; REL-17; WID: RP-211557)

PRACH partitioning items

NR TEI17: Corrections are accepted. New TEI17 tech proposal requirements: a) authored by an operator (and preferably co-signed by more), AND: b) resolves a concrete problem in the market for this operator (no new vendor initiated enhancements).

Includes Rel-17 Work Items without specific R2 Agenda Item, e.g. RAN1 and RAN4 led items, SA2 and CT1 led items (was previously “Rel-17 Other”)

Includes aspects that does not fit under the more specific AIs, e.g. multi-WI aspects.

Corrections for NR_NTN_solutions-Core might be treated in the NTN breakout session.

Tdoc limitation: 4 Tdocs

### 6.1.1	Stage 2 and Organisational

Incoming LSs, etc. You should discuss your stage 2 CRs with the specification rapporteurs before submission. Includes impact to 38.300, 37.340, (36.300 if applicable)

#### 6.1.1.0	In-principle agreed CRs

R2-2404720	Clarification on the srs-AntennaSwitchingBeyond4RX-r17	ZTE Corporation, Sanechips	CR	Rel-17	38.306	17.8.0	1075	1	F	NR_FeMIMO-Core	R2-2403433

R2-2404721	Clarification on the srs-AntennaSwitchingBeyond4RX-r17	ZTE Corporation, Sanechips	CR	Rel-18	38.306	18.1.0	1076	1	A	NR_FeMIMO-Core	R2-2403434

R2-2404722	Correction on the supportedBandwidthDL/UL-v1780 for the NR-DC (r17)	ZTE Corporation, Sanechips	CR	Rel-17	38.306	17.8.0	1077	1	F	NR_BCS4-Core	R2-2403438

R2-2404723	Correction on the supportedBandwidthDL/UL-v1780 for the NR-DC (r18)	ZTE Corporation, Sanechips	CR	Rel-18	38.306	18.1.0	1078	1	A	NR_BCS4-Core	R2-2403439

All above agreed.

=> R2-2404723 is coversheet revised by MCC: wrong tdoc number in the header, wrong meeting dates.

=> Revised in R2-2406114

R2-2406114	Correction on the supportedBandwidthDL/UL-v1780 for the NR-DC (r18)	ZTE Corporation, Sanechips	CR	Rel-18	38.306	18.1.0	1078	2	A	NR_BCS4-Core

=> Agreed

#### 6.1.1.1	Other

NTN

R2-2404404	Terminology alignment on the types of NR NTN deployment	CATT, Nokia, Nokia Shanghai Bell, Ericsson, Huawei, HiSilicon, ZTE Corporation, Sanechips	CR	Rel-17	38.300	17.8.0	0861	-	F	NR_NTN_solutions-Core

R2-2404405	Terminology alignment on the types of NR NTN deployment	CATT, Nokia, Nokia Shanghai Bell, Ericsson, Huawei, HiSilicon, ZTE Corporation, Sanechips	CR	Rel-18	38.300	18.1.0	0862	-	A	NR_NTN_solutions-Core

Both agreed

MBS

R2-2405278	Reference for User Service Description	Nokia (Rapporteur), Ericsson, Xiaomi, Qualcomm Incorporated	CR	Rel-17	38.300	17.8.0	0865	-	F	NR_MBS-Core

R2-2405279	Reference for User Service Description	Nokia (Rapporteur), Ericsson, Xiaomi, Qualcomm Incorporated	CR	Rel-18	38.300	18.1.0	0866	-	A	NR_MBS-Core

Both agreed

### 6.1.2	User Plane corrections

User Plane Related aspects will be handled in the User Plane break out session. (exception: TEI new proposals if any).

#### 6.1.2.0	In Principle Agreed CRs

R2-2404447	Correction on RACH resource set availability check	vivo, Guangdong Genius	CR	Rel-17	38.321	17.8.0	1827	2	F	NR_redcap-Core, NR_cov_enh-Core, NR_SmallData_INACTIVE-Core	R2-2403986

=>	The CR is agreed

R2-2404448	Correction on RACH resource set availability check	vivo, Guangdong Genius	CR	Rel-18	38.321	18.1.0	1828	2	A	NR_redcap-Core, NR_cov_enh-Core, NR_SmallData_INACTIVE-Core	R2-2403686

=>	The CR is revised in R2-2405924 and agreed to make the change exactly like Rel-17

R2-2405924	Correction on RACH resource set availability check	vivo, Guangdong Genius	CR	Rel-18	38.321	18.1.0	1828	3	A	NR_redcap-Core, NR_cov_enh-Core, NR_SmallData_INACTIVE-Core

=> Agreed

#### 6.1.2.1	Other

RACH Resource set selection - redcap

R2-2404442	RACH resource set selection for PDCCH-ordered CFRA for RedCap	vivo, LG Electronics, Ericsson, Guangdong Genius	discussion	Rel-17	NR_redcap-Core

Proposal 1: RAN2 to confirm which is the correct understanding when PDCCH order based CFRA resource is provided for RedCap UEs:

−	Understanding #1: MAC entity selects the set of RACH resource only configured with RedCap indication, i.e. fall into the green highlighted part;

−	Understanding #2: MAC entity selects the set of RACH resource that are not associated with any feature indication, i.e. fall into the yellow highlighted part.

The corresponding TP for understanding #1 and understanding #2 are provided in Annex A and B, respectively.

-	Qualcomm thinks understanding 1 is correct and we need to update RRC spec field description.  Samsungs, ZTE, Huawei and LG agrees with understanding 1. Nokia thought it was understanding 2, so sometime need to discuss.

=>	RAN2 confirms that the correct understanding is that MAC entity selects the set of RACH resource only configured with RedCap indication, i.e. fall into the green highlighted part

=>	The CR from ZTE will be discussed and agreed in CP session

=>	Noted

R2-2405554	Correction on the RACH resource selection for Msg1 based SI request on the RedCap-specific initial BWP	Xiaomi	CR	Rel-17	38.321	17.8.0	1870	-	F	NR_redcap-Core

-	Samsung doesn’t agree and the field description put some restrictions on some configuration. The spec is clear and there is nothing broken. Huawei agrees with Samsung and this is changing Rel-17 behavior and it is NBC. Ericsson agrees with the motivation but not what is proposed in the CR. There is another case SI request and we have to do something for that too. Vivo also agrees with Samsung.

-	LG supports the intention of the CR

-	ZTE explains that the CR makes sense but it is NBC.  The concequence that for redcap 4 preambles will be wasted.

=>	 The CR is not pursued

MBS and NTN

R2-2404250	Discussion on DRX for MBS Multicast in NR NTN	CATT	discussion

Proposal 1: RAN2 confirms configuration of MBS Multicast in NR NTN can be supported in Rel-17/18 Spec.

Proposal 2: If the support of MBS Multicast in NR NTN is confirmed in P1, RAN2 agrees to introduce an NTN specific HARQ RTT timer for PTM in 5.7b, e.g. HARQ-RTT-TimerDL-PTM-NTN.

-	Vivo asks how the network knows the UE supports this. CATT Thinks we should add a capability.

Proposal 2a: When DRX for MBS Multicast is performed in NR NTN, the MAC entity shall:

•	Extend drx-HARQ-RTT-TimerDL-PTM by “UE-gNB” RTT, and set the extended values to HARQ-RTT-TimerDL-PTM-NTN;

•	Operate HARQ-RTT-TimerDL-PTM-NTN and HARQ-RTT-TimerDL-NTN following the existing procedure for HARQ RTT timer handling.

Proposal 3: Adopt the TP in Appendix 2, if Proposals 2 and 2a are agreeable.

=>	Noted

R2-2404897	Discussion on the support of MBS in NTN	Huawei, HiSilicon	discussion	Rel-17	NR_MBS-Core, NR_NTN_solutions-Core

Proposal 2: UE only determines the HARQ feedback as enabled when both HARQ feedback for NTN and HARQ feedback for MBS multicast are indicated as enabled. Otherwise, UE determines the HARQ feedback as disabled.

-	Qualcomm thinks it should be simple, MBS configuration should take precedence.

=>	Noted

Agreements:

- RAN2 confirms configuration of MBS Multicast in NR NTN can be supported for RRC connected mode in Rel-17/18 Spec.  FFS for inactive in R18

- RAN2 agrees to introduce an NTN specific HARQ RTT timer for PTM in 5.7b, e.g. HARQ-RTT-TimerDL-PTM-NTN. No new configuration in RRC will be added. We will introduce a capability.

- drx-HARQ-RTT-TimerDL-PTM is extended by “UE-gNB” RTT and set the extended values to HARQ-RTT-TimerDL-PTM-NTN.

- MBS Multicast HARQ configuration takes precedence.

[POST126][003][R17 MBS-NTN] Agree to CRs (LG)

Intended outcome: Agree to CRs implementing agreements from this meeting

Deadline:  short

R2-2405521	Correction on multicast DRX to support NTN	LG eletronics Inc., Samsung, Xiaomi, Ericsson	CR	Rel-17	38.321	17.8.0	1820	1	F	NR_MBS_enh-Core, NR_NTN_enh-Core	R2-2403343

=> Revised in R2-2406001

R2-2406001	Correction on multicast DRX to support NTN	LG eletronics Inc., Samsung, Xiaomi, Ericsson	CR	Rel-17	38.321	17.8.0	1820	2	F	NR_MBS_enh-Core, NR_NTN_enh-Core

=> Revised in R2-2406077

R2-2406077	Correction on multicast DRX to support NTN	LG eletronics Inc., Samsung, Xiaomi, Ericsson, vivo	CR	Rel-17	38.321	17.8.0	1820	3	F	NR_MBS_enh-Core, NR_NTN_enh-Core

=> Agreed

=> Coversheet update by MCC: Rel-18 WIs used instead of Rel-17 WIs.

R2-2406131	Correction on multicast DRX to support NTN	LG eletronics Inc., Samsung, Xiaomi, Ericsson, vivo	CR	Rel-17	38.321	17.8.0	1820	4	F	NR_MBS-Core, NR_NTN_solutions-Core

=> Agreed

R2-2405524	Correction on multicast DRX to support NTN	LG eletronics Inc., Samsung, Xiaomi, Ericsson	CR	Rel-18	38.321	18.1.0	1821	1	A	NR_MBS_enh-Core, NR_NTN_enh-Core	R2-2403344

=> Revised in R2-2406002

R2-2406002	Correction on multicast DRX to support NTN	LG eletronics Inc., Samsung, Xiaomi, Ericsson	CR	Rel-18	38.321	18.1.0	1821	2	A	NR_MBS_enh-Core, NR_NTN_enh-Core

=> Revised in R2-2406078

R2-2406078	Correction on multicast DRX to support NTN	LG eletronics Inc., Samsung, Xiaomi, Ericsson, vivo	CR	Rel-18	38.321	18.1.0	1821	3	A	NR_MBS_enh-Core, NR_NTN_enh-Core

=> Agreed

=> Coversheet update by MCC: Rel-18 WIs used instead of Rel-17 WIs.

R2-2406132	Correction on multicast DRX to support NTN	LG eletronics Inc., Samsung, Xiaomi, Ericsson, vivo	CR	Rel-18	38.321	18.1.0	1821	4	A	NR_MBS-Core, NR_NTN_solutions-Core

=> Agreed

R2-2406003	Correction on multicast DRX to support NTN	LG Electronics Inc., Samsung, Xiaomi, Ericsson, vivo	CR	Rel-17	38.306	17.8.0	1129	-	F	NR_NTN_enh-Core, NR_MBS_enh-Core

=> Revised in R2-2406079

R2-2406079	Correction on multicast DRX to support NTN	LG Electronics Inc., Samsung, Xiaomi, Ericsson, vivo	CR	Rel-17	38.306	17.8.0	1129	1	F	NR_NTN_enh-Core, NR_MBS_enh-Core

=> Agreed

=> Coversheet update by MCC: Rel-18 WIs used instead of Rel-17 WIs.

R2-2406133	Correction on multicast DRX to support NTN	LG Electronics Inc., Samsung, Xiaomi, Ericsson, vivo	CR	Rel-17	38.306	17.8.0	1129	2	F	NR_MBS-Core, NR_NTN_solutions-Core

=> Agreed

R2-2406004	Correction on multicast DRX to support NTN	LG Electronics Inc., Samsung, Xiaomi, Ericsson, vivo	CR	Rel-18	38.306	18.1.0	1130	-	A	NR_NTN_enh-Core, NR_MBS_enh-Core

=> Revised in R2-2406080

R2-2406080	Correction on multicast DRX to support NTN	LG Electronics Inc., Samsung, Xiaomi, Ericsson, vivo	CR	Rel-18	38.306	18.1.0	1130	1	A	NR_NTN_enh-Core, NR_MBS_enh-Core

=> Agreed

=> Coversheet update by MCC: Rel-18 WIs used instead of Rel-17 WIs.

R2-2406134	Correction on multicast DRX to support NTN	LG Electronics Inc., Samsung, Xiaomi, Ericsson, vivo	CR	Rel-18	38.306	18.1.0	1130	2	A	NR_MBS-Core, NR_NTN_solutions-Core

=> Agreed

R2-2406005	Correction on multicast DRX to support NTN	LG eletronics Inc., Samsung, Xiaomi, Ericsson	CR	Rel-17	38.331	17.8.0	4859	-	F	NR_NTN_enh-Core, NR_MBS_enh-Core

=> Revised in R2-2406081

R2-2406081	Correction on multicast DRX to support NTN	LG eletronics Inc., Samsung, Xiaomi, Ericsson, vivo	CR	Rel-17	38.331	17.8.0	4859	1	F	NR_NTN_enh-Core, NR_MBS_enh-Core

=> Agreed

=> Coversheet update by MCC: Rel-18 WIs used instead of Rel-17 WIs.

R2-2406135	Correction on multicast DRX to support NTN	LG eletronics Inc., Samsung, Xiaomi, Ericsson, vivo	CR	Rel-17	38.331	17.8.0	4859	2	F	NR_MBS-Core, NR_NTN_solutions-Core

=> Agreed

R2-2406006	Correction on multicast DRX to support NTN	LG eletronics Inc., Samsung, Xiaomi, Ericsson	CR	Rel-18	38.331	18.1.0	4860	-	A	NR_NTN_enh-Core, NR_MBS_enh-Core

=> Revised in R2-2406082

R2-2406082	Correction on multicast DRX to support NTN	LG eletronics Inc., Samsung, Xiaomi, Ericsson, vivo	CR	Rel-18	38.331	18.1.0	4860	1	A	NR_NTN_enh-Core, NR_MBS_enh-Core

=> Agreed

=> Coversheet update by MCC: Rel-18 WIs used instead of Rel-17 WIs.

R2-2406136	Correction on multicast DRX to support NTN	LG eletronics Inc., Samsung, Xiaomi, Ericsson, vivo	CR	Rel-18	38.331	18.1.0	4860	2	A	NR_MBS-Core, NR_NTN_solutions-Core

=> Agreed

R2-2406083	Discussion on corrections for multicast DRX to support NTN	LG eletronics Inc.	discussion	Rel-18	NR_NTN_enh-Core, NR_MBS_enh-Core

=> Noted

Not treated

R2-2404898	MAC correction on supporting MBS in NTN	Huawei, HiSilicon	CR	Rel-17	38.321	17.8.0	1847	-	F	NR_MBS-Core, NR_NTN_solutions-Core

R2-2404899	MAC correction on supporting MBS in NTN	Huawei, HiSilicon	CR	Rel-18	38.321	18.1.0	1848	-	A	NR_MBS-Core, NR_NTN_solutions-Core

MIMO -  Flexible TCI state activation

R2-2404406	On the postponed issue for flexible number TCI state activation	CATT	discussion

Proposal 1: RAN2 confirms that current unified TCI state activation/deactivation MAC CE can include less than 8 TCI codepoints (which can be figured out by the UE via L field in the subheader and Pi field).

Proposal 2: No change to the current Spec is needed to clarify P1 (if agreed).

=>	Noted

R2-2405395	Support of flexible number of TCI state activation	Samsung	CR	Rel-17	38.321	17.8.0	1860	-	F	NR_FeMIMO-Core

=>	The CR is postponed

R2-2405396	Support of flexible number of TCI state activation	Samsung	CR	Rel-18	38.321	18.1.0	1861	-	A	NR_FeMIMO-Core

=>	Not treated

Agreements

1	RAN2 confirms that current unified TCI state activation/deactivation MAC CE can include less than 8 TCI codepoints (which can be figured out by the UE via L field in the subheader and Pi field).

2	No change to the current Spec is needed to clarify P1 (if agreed).

3	FFS Whether Pi field modifications are critical

MIMO – BFD-SR set specific BFR

R2-2405188	Clarification to 38.321 On BFD-RS set specific BFR	ZTE Corporation	CR	Rel-17	38.321	17.8.0	1856	-	F	NR_FeMIMO-Core

-	Vivo, oppo, not needed.  LG supports the intention

=>	The CR is postponed

R2-2405189	Clarification to 38.321 On BFD-RS set specific BFR	ZTE Corporation	CR	Rel-18	38.321	18.1.0	1857	-	A	NR_FeMIMO-Core

HARQ RTT Timer

R2-2404669	Clarification on HARQ RTT Timer operation	Apple, Nokia, Nokia Shanghai Bell, MediaTek Inc., ZTE	CR	Rel-18	38.321	18.1.0	1843	-	F	TEI18

=> Revised in R2-2406017

R2-2406017	Clarification on HARQ RTT Timer operation	Apple, Nokia, Nokia Shanghai Bell, MediaTek Inc., ZTE	CR	Rel-18	38.321	18.1.0	1843	1	F	TEI18

=> Revised in R2-2406020

R2-2406020	Clarification on HARQ RTT Timer operation	Apple, Nokia, Nokia Shanghai Bell, MediaTek Inc., ZTE, Huawei, HiSilicon, Samsung	CR	Rel-18	38.321	18.1.0	1843	2	F	TEI18

=> Agreed

R2-2405115	Correction on the start of HARQ RTT timer	Huawei, HiSilicon	CR	Rel-17	38.321	17.8.0	1852	-	F	TEI17

R2-2405116	Correction on the start of HARQ RTT timer	Huawei, HiSilicon	CR	Rel-18	38.321	18.1.0	1853	-	A	TEI17

R2-2405475	Clarification on HARQ RTT Timer operation	LG Electronics, Samsung	CR	Rel-17	38.321	17.8.0	1868	-	F	TEI17

R2-2405476	Clarification on HARQ RTT Timer operation	LG Electronics, Samsung	CR	Rel-18	38.321	18.1.0	1869	-	A	TEI17

R2-2404249	Impact on HARQ RTT timer handling due to UL and SL prioritization	CATT	discussion

[AT126][004][R17 UP] HARQ RTT offline (Apple)

Intended outcome: discussion on HARQ RTT

Deadline:  05-24-24

R2-2405996	Report of [AT126][004][R17 UP] HARQ RTT offline (Apple)	Apple	discussion	Rel-17	TEI17

Proposal 1: Reflect RAN2#125 agreement on HARQ RTT Timer in spec from R18, and it’s up to UE implementation in R17. (Option 1)

Proposal 2: To capture RAN2 agreements in MAC spec as indicated in R2-2404669.

-	LG and Samsung, CATT think that we shouldn’t have different behaviour between R17 and R18. Samsung indicates that Option 1 is not acceptable. Ericsson also thinks it should be done from R17.

-	Qualcomm explains that we can’t change R17 as there are actual UEs in the field. Prefer to have same behaviour and that’s why they propose to add a UE capability. Samsung thinks that the capability is not very useful if we only start in R18.

-	Samsung and CATT think that it should be clear that it is based on actual transmition.

-	Ericsson thinks that there are network implementations as well in the field.

-	Apple indicates that there are different implementations in the field.

-	Samsung indicates that there will interoperability problem. Qualcomm thinks that the network has to handle R17 UEs, so there is no problem. Nokia thinks that there is the option to only configure this feature from R18 if R17 UEs are a problem.

-	Samsung asks if we have to add a table in MAC spec. Nokia indicates that we have had many early implementable MAC CRs and we didn’t add a table.

=>	Noted

Agreements

-	No specification update for R17.

-	From R18, we will specify that:

When drx-LastTransmissionUL is configured, drx-HARQ-RTT-TimerUL is started after the last PUSCH transmission occasion of a bundle regardless of whether that last PUSCH transmission occasion is used for a PUSCH transmission for that bundle or not.

-	add magic sentence indicating a R17 UE may implement this CR (if it wishes to).

SDT

R2-2404918	Correction on cg-SDT-RetransmissionTimer stop	NEC	CR	Rel-17	38.321	17.8.0	1849	-	F	NR_SmallData_INACTIVE-Core

-	Vivo, ZTE, Huawei think it’s already clear this timer is for initial trnamsision

=>	The CR is not pursued

R2-2404919	Correction on cg-SDT-RetransmissionTimer stop	NEC	CR	Rel-18	38.321	18.1.0	1850	-	A	NR_SmallData_INACTIVE-Core

=>	Not treated

### 6.1.3	Control Plane corrections

#### 6.1.3.0	In Principle Agreed CRs

R2-2405718	CEF and RLF reporting for RedCap UEs	MediaTek Inc., Qualcomm Incorporated, Nordic Semiconductor ASA, Ericsson	CR	Rel-17	38.306	17.8.0	1060	2	F	NR_SON_MDT-Core, NR_redcap-Core	R2-2404178

R2-2405719	CEF and RLF reporting for (e)RedCap UEs	MediaTek Inc., Qualcomm Incorporated, Nordic Semiconductor ASA, Ericsson	CR	Rel-18	38.306	18.1.0	1061	2	A	NR_SON_MDT-Core, NR_redcap-Core, NR_redcap_enh-Core	R2-2404179

=> Coversheet update by MCC: a Cat A CR must have the same WIs as the cat F CR.

R2-2406139	CEF and RLF reporting for (e)RedCap UEs	MediaTek Inc., Qualcomm Incorporated, Nordic Semiconductor ASA, Ericsson	CR	Rel-18	38.306	18.1.0	1061	3	A	NR_redcap-Core, NR_SON_MDT-Core

=> Agreed

R2-2405720	CEF and RLF reporting for RedCap UEs	MediaTek Inc., Qualcomm Incorporated, Nordic Semiconductor ASA, Ericsson	CR	Rel-17	38.331	17.8.0	4647	2	F	NR_SON_MDT-Core, NR_redcap-Core	R2-2404180

R2-2405721	CEF and RLF reporting for (e)RedCap UEs	MediaTek Inc., Qualcomm Incorporated, Nordic Semiconductor ASA, Ericsson	CR	Rel-18	38.331	18.1.0	4648	2	A	NR_SON_MDT-Core, NR_redcap-Core, NR_redcap_enh-Core	R2-2404181

=> Coversheet update by MCC: a Cat A CR must have the same WIs as the cat F CR.

R2-2406140	CEF and RLF reporting for (e)RedCap UEs	MediaTek Inc., Qualcomm Incorporated, Nordic Semiconductor ASA, Ericsson	CR	Rel-18	38.331	18.1.0	4648	3	A	NR_SON_MDT-Core, NR_redcap-Core

=> Agreed

R2-2404698	Correction on TRS for idle and inactive UEs	CATT, Ericsson	CR	Rel-18	38.300	18.1.0	0836	2	F	NR_UE_pow_sav_enh-Core	R2-2403847

=> Coversheet update by MCC: NR_UE_pow_sav_enh-Core needs WI TEI18 in a Rel-18

R2-2406143	Correction on TRS for idle and inactive UEs	CATT, Ericsson	CR	Rel-18	38.300	18.1.0	0836	3	F	NR_UE_pow_sav_enh-Core, TEI18

=> Agreed

R2-2404845	Clarification on usage of LEO or NGSO	MediaTek Inc.	CR	Rel-17	38.331	17.8.0	4745	1	F	NR_NTN_solutions-Core	R2-2403466

R2-2404846	Clarification on usage of LEO or NGSO	MediaTek Inc.	CR	Rel-18	38.331	18.1.0	4746	1	A	NR_NTN_solutions-Core	R2-2403467

R2-2404847	Clarification on usage of LEO or NGSO	MediaTek Inc.	CR	Rel-17	38.306	17.8.0	1082	1	F	NR_NTN_solutions-Core	R2-2403468

R2-2404848	Clarification on usage of LEO or NGSO	MediaTek Inc.	CR	Rel-18	38.306	18.1.0	1083	1	A	NR_NTN_solutions-Core	R2-2403470

R2-2404988	Clarification on TRS in idle and inactive	Ericsson, MediaTek, ZTE, Nokia, Huawei, HiSilicon, Apple, CATT	CR	Rel-17	38.331	17.8.0	4754	2	F	NR_UE_pow_sav_enh-Core	R2-2403848

Moved from 6.1.3.1

R2-2404989	Correction on TRS in idle and inactive	Ericsson, MediaTek, ZTE, Nokia, Huawei, HiSilicon, Apple, CATT	CR	Rel-18	38.331	18.1.0	4755	2	F	NR_UE_pow_sav_enh-Core	R2-2403849

Moved from 6.1.3.1

=> Coversheet update by MCC: NR_UE_pow_sav_enh-Core needs WI TEI18 in a Rel-18 CR.

R2-2406144	Correction on TRS in idle and inactive	Ericsson, MediaTek, ZTE, Nokia, Huawei, HiSilicon, Apple, CATT	CR	Rel-18	38.331	18.1.0	4755	3	F	NR_UE_pow_sav_enh-Core, TEI18

=> Agreed

R2-2405717	Clarification RLM/BFD relaxation and short DRX	Ericsson, Nokia, Qualcomm, Huawei, HiSilicon	CR	Rel-17	38.331	17.8.0	4770	2	F	NR_UE_pow_sav_enh-Core	R2-2404990

R2-2404991	Clarification RLM/BFD relaxation and short DRX	Ericsson, Nokia, Qualcomm, Huawei, HiSilicon	CR	Rel-18	38.331	18.1.0	4771	1	F	NR_UE_pow_sav_enh-Core	R2-2403863

Moved from 6.1.3.1

=> Coversheet update by MCC: NR_UE_pow_sav_enh-Core needs WI TEI18 in a Rel-18 CR.

R2-2406145	Clarification RLM/BFD relaxation and short DRX	Ericsson, Nokia, Qualcomm, Huawei, HiSilicon	CR	Rel-18	38.331	18.1.0	4771	2	F	NR_UE_pow_sav_enh-Core, TEI18

=> Agreed

R2-2404522	Correction CR for QoE measurements and conditional handover	Ericsson, China Unicom	CR	Rel-18	38.331	18.1.0	4713	1	F	NR_QoE-Core	R2-2403251

All above agreed, though R2-2404522 was coversheet revised by Ericsson in R2-2406029

R2-2406029	Correction CR for QoE measurements and conditional handover	Ericsson, China Unicom	CR	Rel-18	38.331	18.1.0	4713	2	F	NR_QoE-Core

=> Coversheet revised by MCC in R2-2406113: NR_QoE-Core is a Rel-17 WI, thus the addition of TEI18.

R2-2406113	Correction CR for QoE measurements and conditional handover	Ericsson, China Unicom	CR	Rel-18	38.331	18.1.0	4713	3	F	NR_QoE-Core, TEI18

=> Agreed

R2-2404237	Correction to PDCP configuration for multicast MRB	MediaTek inc.	CR	Rel-17	38.331	17.8.0	4652	1	F	NR_MBS-Core	R2-2402294

R2-2404239	Correction to PDCP configuration for multicast MRB	MediaTek Inc.	CR	Rel-18	38.331	18.1.0	4651	1	A	NR_MBS-Core	R2-2402293

-	QC says that to align with other parts of the spec, we should update the wording and say only “The first/leftmost bit of the bit string contains the most significant bit.”

Update to say only “The first/leftmost bit of the bit string contains the most significant bit”. Agreed unseen in R2-2405829 and R2-2405830

Old revisions

R2-2404990	Clarification RLM/BFD relaxation and short DRX	Ericsson, Nokia, Qualcomm, Huawei, HiSilicon	CR	Rel-17	38.331	17.8.0	4770	1	F	NR_UE_pow_sav_enh-Core	R2-2403862

Moved from 6.1.3.1

R2-2404178	CEF and RLF reporting for RedCap UEs	MediaTek Inc., Qualcomm Incorporated, Nordic Semiconductor ASA, Ericsson	CR	Rel-17	38.306	17.8.0	1060	1	F	NR_SON_MDT-Core, NR_redcap-Core	R2-2402238

R2-2404179	CEF and RLF reporting for (e)RedCap UEs	MediaTek Inc., Qualcomm Incorporated, Nordic Semiconductor ASA, Ericsson	CR	Rel-18	38.306	18.1.0	1061	1	A	NR_SON_MDT-Core, NR_redcap-Core, NR_redcap_enh-Core	R2-2402239

R2-2404180	CEF and RLF reporting for RedCap UEs	MediaTek Inc., Qualcomm Incorporated, Nordic Semiconductor ASA, Ericsson	CR	Rel-17	38.331	17.8.0	4647	1	F	NR_SON_MDT-Core, NR_redcap-Core	R2-2402240

#### 6.1.3.1	NR RRC

Corrections to 38331, and related change to other TS if applicable, except UE caps.

QoE

R2-2405658	Alternative correction CR for QoE measurements and conditional handover	Ericsson	discussion	Rel-17	NR_QoE-Core	R2-2404482

noted

SHR

R2-2405087	Adding PCI and ARFCN of target cell for intra-RAT SHR	ZTE Corporation, Sanechips, Ericsson, Huawei, CATT, Samsung	CR	Rel-17	38.331	17.8.0	4811	-	F	NR_ENDC_SON_MDT_enh-Core

R2-2405700	Adding PCI and ARFCN of target cell for intra-RAT SHR	ZTE Corporation, Sanechips, Ericsson, Huawei	CR	Rel-18	38.331	18.1.0	4812	1	A	NR_ENDC_SON_MDT_enh2-Core

Change from “targetCell-PCI-ARFCN-v17xx” to “targetCell-PCI-ARFCN-r17”.

Remove “on” in the note.

Agreed unseen in R2-2405821 and R2-2405822.

=> R2-2405822 is coversheet revised by MCC in R2-2406119: Wrong WI code - should use the same WI code as the cat A CR

R2-2406119	Adding PCI and ARFCN of target cell for intra-RAT SHR	ZTE Corporation, Sanechips, Ericsson, Huawei, CATT, Samsung	CR	Rel-18	38.331	18.1.0	4812	3	A	NR_ENDC_SON_MDT_enh-Core

=> Agreed

R2-2405587	Correction on successful handover report configuration	Samsung	CR	Rel-17	38.331	17.8.0	4845	-	F	NR_ENDC_SON_MDT_enh-Core

R2-2405606	Correction on successful handover report configuration	Samsung	CR	Rel-18	38.331	18.1.0	4848	-	A	NR_ENDC_SON_MDT_enh-Core

-	Huawei is not OK with the last change since it is not needed. ZTE and CATT agrees with Huawei. Qualcomm think that for the setup-release change can be handled by another CR, Google has one good candidate for that.

Potential changes like the first change are plenty, the spec rapporteur can solve this if/when he finds suitable.

Second change is not pursued.

Need codes

R2-2405324	Correction on the field descriptions of some Need M fields	Google	CR	Rel-17	38.331	17.8.0	4823	-	F	NR_unlic-Core, 5G_V2X_NRSL-Core, NR_IIOT-Core, NR_SON_MDT-Core, NR_ENDC_SON_MDT_enh-Core

R2-2405327	Correction on the field descriptions of some Need M fields	Google Inc.	CR	Rel-18	38.331	18.1.0	4824	-	A	NR_unlic-Core, 5G_V2X_NRSL-Core, NR_IIOT-Core, NR_SON_MDT-Core, NR_ENDC_SON_MDT_enh-Core

Spec rapporteur has some other way of addressing this and he will handle this in his rapporteur CR.

RLF report

R2-2405089	Misclassification of RLF reports as Too Early HO failure	Ericsson	CR	Rel-17	38.331	17.8.0	4813	-	F	NR_ENDC_SON_MDT_enh-Core

R2-2405090	Misclassification of RLF reports as Too Early HO failure	Ericsson	CR	Rel-18	38.331	18.1.0	4814	-	A	NR_ENDC_SON_MDT_enh2-Core

-	QC want to clarify somehow that UEs may not implement this as the CR is late. Huawei would like to add this as a capability without indication.

[AT126][754][Maint] Misclassification of RLF reports as Too Early HO failure (Ericsson)

Scope:

    - Create a 38.306 CR where this functional change is made optional without capability indications. Also refer to that 38.306 CR on the RRC CRs.

Intended outcome:

    - Agreeable CRs in R2-2405823, R2-2405824, R2-2405825 and R2-2405826 (Ericsson)

Deadline:

    - Thursday lunch. Intention is to agree the CRs over email.

R2-2405823	Misclassification of RLF reports as Too Early HO failure	Ericsson	CR	Rel-17	38.331	17.8.0	4813	1	F	NR_ENDC_SON_MDT_enh-Core

R2-2405973	Misclassification of RLF reports as Too Early HO failure	Ericsson	CR	Rel-18	38.331	18.1.0	4814	2	A	NR_ENDC_SON_MDT_enh2-Core

R2-2405825	UE capability for misclassification of RLF reports as Too Early HO failure	Ericsson	CR	Rel-17	38.306	17.8.0	1127	-	F	NR_ENDC_SON_MDT_enh-Core

R2-2405826	UE capability for misclassification of RLF reports as Too Early HO failure	Ericsson	CR	Rel-18	38.306	18.1.0	1128	-	A	NR_ENDC_SON_MDT_enh2-Core

Above 4 agreed

=> R2-2405826 and R2-2405973 are coversheet updated by MCC in R2-2406120 and R2-2406126: wrong WI code (should use the same WI as the cat F CR).

R2-2406120	UE capability for misclassification of RLF reports as Too Early HO failure	Ericsson	CR	Rel-18	38.306	18.1.0	1128	1	A	NR_ENDC_SON_MDT_enh-Core

=> Agreed

R2-2406126	Misclassification of RLF reports as Too Early HO failure 	Ericsson	CR	Rel-18	38.331	18.1.0	4814	3	A	NR_ENDC_SON_MDT_enh-Core

=> Agreed

SDT – Pathloss reference RSRP

R2-2405485	Clarification on the RSRP measurement for SDT	Xiaomi	CR	Rel-17	38.331	17.8.0	4839	-	F	NR_SmallData_INACTIVE-Core

R2-2405486	Clarification on the RSRP measurement for SDT	Xiaomi	CR	Rel-18	38.331	18.1.0	4840	-	A	NR_SmallData_INACTIVE-Core

-	Ericsson highlights that its unclear if the note as proposed affects positioning.

Change the note to “NOTE 1: If the UE is configured with multiple SSB configurations, the downlink pathloss reference RSRP for TA validation is derived from the SSB configured by SIB1.”. With this it is agreed unseen in R2-2405827 and R2-2405828.

SDT and MBS

R2-2405503	Prioritization of SDT unicast over MBS broadcast	Samsung, CATT, Nokia, LG Electronics Inc.	CR	Rel-17	38.331	17.8.0	4842	-	F	NR_MBS-Core, NR_SmallData_INACTIVE-Core

-	QC thinks this CR is not needed because UE behaviour is transparent to the NW. vivo thinks the current change is NBC.

Postponed

NTN terminology

R2-2405714	Terminology alignment for NR NTN	Ericsson, ZTE Corporation, Sanechips, Intel Corporation, CATT, Huawei, HiSilicon, Nokia, Nokia Shanghai Bell	CR	Rel-17	38.331	17.8.0	4836	1	F	NR_NTN_solutions-Core

Moved from 7.7.1

R2-2405751	Terminology alignment for NR NTN	Ericsson, ZTE Corporation, Sanechips, Intel Corporation, CATT, Huawei, HiSilicon, Nokia, Nokia Shanghai Bell	CR	Rel-18	38.331	18.1.0	4837	2	A	NR_NTN_enh-Core

Both merged in rapp CR.

ATG

R2-2405921	Discussion on correction of the range of DL-DataToUL-ACK	CMCC

R2-2405922	CR to TS 38.331 Correction of the range of DL-DataToUL-ACK-v1700 	CMCC	CR	Rel-17	38.331	17.8.0	4856	-	A	NR_NTN_solutions-Core	Late

R2-2405923	CR to TS 38.331 Correction of the range of DL-DataToUL-ACK-v1700(R18)	CMCC	CR	Rel-18	38.331	18.1.0	4857	-	A	NR_NTN_solutions-Core	Late

Category is not correct (both A). CR-title needs polishing

-	QC thinks this is NBC, there may be other ways to address this. CMCC suggests to discuss offline.

[AT126][755][Maint] Correction of the range of DL-DataToUL-ACK-v1700 (CMCC)

Scope:

    - Discuss the issue offline and produce CRs if needed.

Intended outcome:

    - Agreeable CRs in R2-2405831 and R2-2405832 (CMCC)

Deadline:

    - Friday

R2-2405832	Correction of the range of DL-DataToUL-ACK for ATG	CMCC	CR	Rel-18	38.331	18.1.0	4857	1	F	NR_ATG

The new fields to be added in the same [[]] as the other existing r18 fields. Do syntax checking. With this it is agreed unseen in R2-2405831

R2-2405831	Correction of the range of DL-DataToUL-ACK for ATG	CMCC	CR	Rel-18	38.331	18.1.0	4857	2	F	NR_ATG

=> Coversheet update by MCC: wrong tdoc number, wrong CR number, wrong rev number.

R2-2406121	Correction of the range of DL-DataToUL-ACK for ATG	CMCC	CR	Rel-18	38.331	18.1.0	4857	3	F	NR_ATG

=> Agreed

RA paritioning – Empty feature combination

R2-2404965	Correction on when featureCombination is empty	Ericsson	CR	Rel-17	38.331	17.8.0	4801	-	F	NR_redcap-Core, NR_SmallData_INACTIVE-Core, NR_cov_enh-Core, NR_slice-Core

R2-2404966	Correction on when featureCombination is empty	Ericsson	CR	Rel-18	38.331	18.1.0	4802	-	A	NR_redcap-Core, NR_SmallData_INACTIVE-Core, NR_cov_enh-Core, NR_slice-Core

-	Samsung thinks the NW should not have an empty combo. QC think that we may want to allow empty combos in the future. Nokia agrees with Samsung and think we can come back later if we should allow empty combos. ZTE think we have the SI-request issue to consider.

Postponed

RA paritioning – RACH-ConfigCommon

R2-2405052	Clarification on RACH-ConfigCommon for PDCCH order based CFRA and SI request	ZTE Corporation, Samsung	CR	Rel-17	38.331	17.8.0	4807	-	F	NR_redcap-Core

R2-2405053	Correction on RACH-ConfigCommon for PDCCH order based CFRA and SI request	ZTE Corporation, Samsung	CR	Rel-18	38.331	18.1.0	4808	-	F	NR_redcap-Core, NR_cov_enh2, NR_redcap_enh

[AT126][756][Maint] PDCCH order based CFRA and SI request (ZTE)

Scope:

    - Discuss how to address the issue that this CR is handling. Produce agreeable CRs, for both MAC and RRC (whichever are needed).

Intended outcome:

    - Agreed CRs in R2-2405833, R2-2405834, R2-2405835, and R2-2405836 (ZTE)

Deadline:

    - Friday

R2-2405833	Clarification on RACH-ConfigCommon for PDCCH order based CFRA and SI request	ZTE	CR

R2-2405834	Clarification on RACH-ConfigCommon for PDCCH order based CFRA and SI request	ZTE	CR

Both above agreed

R2-2405835	Report of [AT126][756][Maint] PDCCH order based CFRA and SI request (ZTE)

Confirm for SI-request without Msg1 repetition in RedCap-specific initial BWP, the UE selects the RACH-ConfigCommon that not associated with any feature (no change to R17 MAC spec).

FFS if possible and if spec change needed to allow that, for a RedCap specific initial uplink BWP, whether the same CB preambles in “RA partition not associated with any feature” can be configured in another RA partition sharing the ROs. (can be discussed in next meeting)

RA partitioning - RedCap

R2-2405559	Correction on the RACH resource selection for Msg1 based SI request	Xiaomi	CR	Rel-17	38.331	17.8.0	4844	-	F	NR_redcap-Core

-	Samsung thinks this is NBC and disagrees that there is an issue. Huawei also does not think there is an issue. ZTE thinks it relates to the topic above and can be discussed in the offline above.

Can be discussed in the offline [AT126][756].

Misc

R2-2404784	Miscellaneous non-controversial corrections Set XXI	Ericsson	CR	Rel-15	38.331	15.25.0	4797	-	F	NR_newRAT-Core	Late

R2-2404785	Miscellaneous non-controversial corrections Set XXI	Ericsson	CR	Rel-16	38.331	16.16.0	4798	-	F	NR_newRAT-Core	Late

R2-2404786	Miscellaneous non-controversial corrections Set XXI	Ericsson	CR	Rel-17	38.331	17.8.0	4718	1	F	NR_newRAT-Core	R2-2403331

R2-2404787	Miscellaneous non-controversial corrections Set XXI	Ericsson	CR	Rel-18	38.331	18.1.0	4799	-	F	NR_newRAT-Core

Endorsed and to be updated in post meeting email disc.

[Post126][752][Maint] Miscellaneous non-controversial corrections for NR RRC (Ericsson)

Scope:

    - Produce RRC rapporteurs misc CRs

Intended outcome:

    - Agreed CRs in R2-2405847, R2-2405848, R2-2405849, R2-2405850 (Ericsson)

Deadline:

    - Short

R2-2404784	Miscellaneous non-controversial corrections Set XXI	Ericsson	CR	Rel-15	38.331	15.25.0	4797	-	F	NR_newRAT-Core

=> Agreed

R2-2404785	Miscellaneous non-controversial corrections Set XXI	Ericsson	CR	Rel-16	38.331	16.16.0	4798	-	F	NR_newRAT-Core

=> Agreed

R2-2405849	Miscellaneous non-controversial corrections Set XXI	Ericsson	CR	Rel-17	38.331	17.8.0	4718	2	F	NR_newRAT-Core

=> Agreed

R2-2405850	Miscellaneous non-controversial corrections Set XXI	Ericsson	CR	Rel-18	38.331	18.1.0	4799	1	A	NR_newRAT-Core, TEI18

=> Agreed

=> Coversheet update by MCC: This is a cat F CR, not a cat A.

=> Revised in R2-2406138

R2-2406138	Miscellaneous non-controversial corrections Set XXI	Ericsson	CR	Rel-18	38.331	18.1.0	4799	2	F	NR_newRAT-Core, TEI18

=> Agreed

Withdrawn and old revisions

R2-2404737	Correction of gNB identity derivation in NPN-only cells	Nokia	CR	Rel-17	38.331	17.8.0	4789	-	F	TEI17	Late

R2-2404738	Correction of gNB identity derivation in NPN-only cells	Nokia	CR	Rel-18	38.331	18.1.0	4790	-	A	TEI17	Late

R2-2405599	Correction on successful handover report configuration 	Samsung	CR	Rel-18	38.331	18.1.0	4846	-	A	NR_ENDC_SON_MDT_enh-Core	Withdrawn

R2-2404482	Alternative correction CR for QoE measurements and conditional handover	Ericsson	discussion	Rel-17	NR_QoE-Core	Revised

R2-2405088	Adding PCI and ARFCN of target cell for intra-RAT SHR	ZTE Corporation, Sanechips, Ericsson, Huawei	CR	Rel-18	38.331	18.1.0	4812	-	A	NR_ENDC_SON_MDT_enh2-Core

R2-2405824	Misclassification of RLF reports as Too Early HO failure	Ericsson	CR

#### 6.1.3.2	UE capabilities

UE cap corrections 38306, 38331.

Parallel Tx capability

R2-2404110	Reply LS on Parallel Tx Capability (R1-2403619; contact: ZTE)	RAN1	LS in	Rel-17	TEI17, NR_newRAT-Core, NR_2step_RACH, NR_IIOT_URLLC_enh	To:RAN2

Noted

NTN terminology

R2-2404531	Terminology alignment for NR NTN	Intel Corporation, Ericsson, ZTE Corporation, Sanechips, Huawei, HiSilicon	CR	Rel-17	38.306	17.8.0	1098	-	F	NR_NTN_solutions-Core

Endorsed and merged in rapp CR.

R2-2404532	Terminology alignment for NR NTN	Intel Corporation, Ericsson, ZTE Corporation, Sanechips, Huawei, HiSilicon	CR	Rel-18	38.306	18.1.0	1099	-	A	NR_NTN_solutions-Core, NR_NTN_enh-Core

Endorsed and merged in mega CR.

BSC5 clarification

R2-2404553	Correction to BCS5 bandwidth capabilities	Nokia, Qualcomm Incorporated, ZTE Corporation, Sanechips, Huawei, HiSilicon, Ericsson	CR	Rel-17	38.306	17.8.0	1080	1	F	NR_BCS4-Core	R2-2403450

R2-2404554	Correction to BCS5 bandwidth capabilities	Nokia, Qualcomm Incorporated, ZTE Corporation, Sanechips, Huawei, HiSilicon, Ericsson	CR	Rel-18	38.306	18.1.0	1081	1	A	NR_BCS4-Core	R2-2403451

Both agreed

Internode signalling for max agg BW

R2-2404747	Introduction of Inter-node Coordination on the Aggregated Bandwidth for the NR-DC (r17)	ZTE Corporation, Sanechips,Ericsson,Nokia,Huawei, HiSilicon	CR	Rel-17	38.331	17.8.0	4735	2	F	NR_BCS4-Core	R2-2403983

R2-2404748	Introduction of Inter-node Coordination on the Aggregated Bandwidth for the NR-DC (r18)	ZTE Corporation, Sanechips,Ericsson,Nokia,Huawei, HiSilicon	CR	Rel-18	38.331	18.1.0	4736	2	A	NR_BCS4-Core	R2-2403984

-	There are styling issues to be fixed, everything is “Normal” style

Fix the styling issues. Agreed unseen in R2-2405966 and R2-2405967

mux-HARQ-ACK-withoutPUCCH-onPUSCH

R2-2404120	LS on the capability mux-HARQ-ACK-withoutPUCCH-onPUSCH-r16 (R1-2403760; contact: MediaTek)	RAN1	LS in	Rel-17	TEI17, NR_newRAT-Core	To:RAN2

Moved from 6.1.1

R2-2405722	Mandating the capability mux-HARQ-ACK-withoutPUCCH-onPUSCH-r16	MediaTek Inc., Ericsson	CR	Rel-17	38.306	17.8.0	1112	1	F	TEI17, NR_newRAT-Core	R2-2405006

R2-2405723	Mandating the capability mux-HARQ-ACK-withoutPUCCH-onPUSCH-r16	MediaTek Inc., Ericsson	CR	Rel-18	38.306	18.1.0	1113	1	F	TEI18, NR_newRAT-Core	R2-2405007

-	Huawei thinks that that this should be mandatory with signalling.

Both agreed

R2-2405008	[DRAFT] Reply LS on the capability mux-HARQ-ACK-withoutPUCCH-onPUSCH-r16	MediaTek Inc.	LS out	Rel-17	TEI17, NR_newRAT-Core	To:RAN1

EN-DC band combos

R2-2405379	Introduction of new intra-band EN-DC capabilities for inter-band EN-DC	Google Inc., CATT	CR	Rel-17	38.331	17.8.0	4750	2	F	NR_newRAT-Core	R2-2403842

R2-2405381	Introduction of new intra-band EN-DC capabilities for inter-band EN-DC	Google Inc., CATT	CR	Rel-17	38.306	17.8.0	1084	2	F	NR_newRAT-Core	R2-2403843

R2-2405471	Introduction of new intra-band EN-DC capabilities for inter-band EN-DC	Google Inc., CATT	CR	Rel-18	38.331	18.1.0	4751	2	A	NR_newRAT-Core	R2-2403518

R2-2405474	Introduction of new intra-band EN-DC capabilities for inter-band EN-DC	Google Inc., CATT	CR	Rel-18	38.306	18.1.0	1085	2	A	NR_newRAT-Core	R2-2403523

R2-2405479	Draft Reply LS on IE supportedBandwidthCombinationSetIntraENDC and IE intraBandENDC-Support	Google Inc.	LS out	TEI17	To:RAN4

-	Google explains they need more time offline.

-	QC think that if we send the LS to RAN4 we should explain what we have done.

=> Revised in R2-2406090

[AT126][757][Maint] Intra-band EN-DC capabilities for inter-band EN-DC (Google)

Scope:

    - Update the CRs until agreeable. Produce approvable draft LS

Intended outcome:

    - Agreeable CRs in R2-2405961, R2-2405962, R2-2405963, R2-2405964

    - Approvable draft LS in R2-2405965

Deadline:

    - Friday

R2-2405961	Introduction of new intra-band EN-DC capabilities for inter-band EN-DC	Google Inc., CATT	CR	Rel-17	38.331	17.8.0	4750	3	F	NR_newRAT-Core, TEI17

=> Revised in R2-2405975

R2-2405975	Introduction of new intra-band EN-DC capabilities for inter-band EN-DC	Google Inc., CATT	CR	Rel-17	38.331	17.8.0	4750	4	F	NR_newRAT-Core, TEI17

=> Agreed

R2-2405962	Introduction of new intra-band EN-DC capabilities for inter-band EN-DC	Google Inc., CATT	CR	Rel-17	38.306	17.8.0	1084	3	F	NR_newRAT-Core, TEI17

=> Revised in R2-2405976

R2-2405976	Introduction of new intra-band EN-DC capabilities for inter-band EN-DC	Google Inc., CATT	CR	Rel-17	38.306	17.8.0	1084	4	F	NR_newRAT-Core, TEI17

=> Agreed

R2-2405963	Introduction of new intra-band EN-DC capabilities for inter-band EN-DC	Google Inc., CATT	CR	Rel-18	38.331	18.1.0	4751	3	A	NR_newRAT-Core, TEI17

=> Revised in R2-2405977

R2-2405977	Introduction of new intra-band EN-DC capabilities for inter-band EN-DC	Google Inc., CATT	CR	Rel-18	38.331	18.1.0	4751	4	A	NR_newRAT-Core, TEI17

=> Agreed

R2-2405964	Introduction of new intra-band EN-DC capabilities for inter-band EN-DC	Google Inc., CATT	CR	Rel-18	38.306	18.1.0	1085	3	A	NR_newRAT-Core, TEI17

=> Revised in R2-2405978

R2-2405978	Introduction of new intra-band EN-DC capabilities for inter-band EN-DC	Google Inc., CATT	CR	Rel-18	38.306	18.1.0	1085	4	A	NR_newRAT-Core, TEI17

=> Agreed

-	Companies had detailed comments on the cover page and the changes, pushed to post meeting email discussion.

[Post126][754][Maint] Intra-band EN-DC capabilities for inter-band EN-DC (Google)

Scope:

    - Produce agreeable CRs. Produce approvable LS.

Intended outcome:

    - Agreed CRs in R2-2405975, R2-2405976, R2-2405977, and R2-2405978 (Google)

    - Approved LS in R2-2406090

Deadline:

    - Short

R2-2406090	Draft Reply LS on IE supportedBandwidthCombinationSetIntraENDC and IE intraBandENDC-Support	Google Inc.	LS out	TEI17	To:RAN4

=> Revised in R2-2406111

R2-2406111	Reply LS on IE supportedBandwidthCombinationSetIntraENDC and IE intraBandENDC-Support	RAN2	LS out	Rel-17	TEI17	To:RAN4

=> Approved

SRS carrier switching

R2-2405509	Clarification on srs-SwitchingAffectedBandsListNR	Huawei, HiSilicon	CR	Rel-17	38.306	17.8.0	1120	-	F	TEI17

R2-2405510	Clarification on srs-SwitchingAffectedBandsListNR	Huawei, HiSilicon	CR	Rel-18	38.306	18.1.0	1121	-	A	TEI17

-	Huawei explains that some update of the CR is needed and can be done offline.

[AT126][758][Maint] Clarification on srs-SwitchingAffectedBandsListNR (Huawei)

Scope:

    - Produce agreeable CRs

Intended outcome:

    - Agreed CRs in R2-2405837 and R2-2405838 (Huawei)

Deadline:

    - Thursday lunch. Intention is to agree the CRs over email.

R2-2405837	Clarification on srs-SwitchingAffectedBandsListNR	Huawei, HiSilicon	CR	Rel-17	38.306	17.8.0	1120	1	F	TEI17	R2-2405509

R2-2405838	Clarification on srs-SwitchingAffectedBandsListNR	Huawei, HiSilicon	CR	Rel-18	38.306	18.1.0	1121	1	A	TEI17	R2-2405510

Both agreed

Channel Raster

R2-2404794	Support of Enhanced channel raster by (e)RedCap UE	Ericsson	discussion	Rel-17	NR_redcap-Core

Postponed to if/when we hear from RAN4.

Misc

R2-2404530	Miscellaneous non-controversial rapporteur corrections	Intel Corporation	CR	Rel-17	38.306	17.8.0	1097	-	F	NR_newRAT-Core, NR_eMIMO-Core, NR_unlic-Perf, 5G_V2X_NRSL-Core, NR_IIOT_URLLC_enh-Core, NR_pos_enh-Core, NR_cov_enh-Core, NR_FeMIMO-Core, NR_ext_to_71GHz-Core, NR_MBS-Core, NR_demod_enh2-Perf, NR_SL_enh-Core

[AT126][759][Maint] Miscellaneous non-controversial rapporteur corrections (Intel)

Scope:

    - Produce agreeable Miscellaneous non-controversial rapporteur correction CRs for 38.306

Intended outcome:

    - Agreed CR in R2-2405839 (Intel)

Deadline:

    - Friday

R2-2405839	Miscellaneous non-controversial rapporteur corrections	Intel Corporation	CR	Rel-17	38.306	17.8.0	1097	1	F	NR_newRAT-Core, NR_eMIMO-Core, NR_unlic-Perf, 5G_V2X_NRSL-Core, NR_IIOT_URLLC_enh-Core, NR_pos_enh-Core, NR_cov_enh-Core, NR_FeMIMO-Core, NR_ext_to_71GHz-Core, NR_MBS-Core, NR_demod_enh2-Perf, NR_SL_enh-Core

Agreed

=> Coversheet update by MCC: "Source to TSG" field is empty, wrong spec number in the coversheet.

R2-2406122	Miscellaneous non-controversial rapporteur corrections	Intel Corporation	CR	Rel-17	38.306	17.8.0	1097	2	F	NR_newRAT-Core, NR_eMIMO-Core, NR_unlic-Perf, 5G_V2X_NRSL-Core, NR_IIOT_URLLC_enh-Core, NR_pos_enh-Core, NR_cov_enh-Core, NR_FeMIMO-Core, NR_ext_to_7, 1GHz-Core, NR_MBS-Core, NR_demod_enh2-Perf, NR_SL_enh-Core, NR_NTN_solutions-Core

=> Agreed

Old revisions

R2-2405006	Mandating the capability mux-HARQ-ACK-withoutPUCCH-onPUSCH-r16	MediaTek Inc., Ericsson	CR	Rel-17	38.306	17.8.0	1112	-	F	TEI17, NR_newRAT-Core

R2-2405007	Mandating the capability mux-HARQ-ACK-withoutPUCCH-onPUSCH-r16	MediaTek Inc., Ericsson	CR	Rel-18	38.306	18.1.0	1113	-	F	TEI18, NR_newRAT-Core

#### 6.1.3.3	Other

Including idle and inactive behaviour specified in 38.304 or 36.304.

MBS

R2-2405280	Reference for User Service Description	Nokia, Ericsson, Xiaomi, Qualcomm Incorporated	CR	Rel-17	38.304	17.8.0	0404	-	F	NR_MBS-Core

R2-2405281	Reference for User Service Description	Nokia, Ericsson, Xiaomi, Qualcomm Incorporated	CR	Rel-18	38.304	18.1.0	0405	-	A	NR_MBS-Core

Both agreed

Misc

R2-2405457	Miscellaneous Corrections (Rapporteur)	Qualcomm Incorporated, Nokia, CATT	CR	Rel-17	38.304	17.8.0	0406	-	F	NR_UE_pow_sav_enh-Core, NR_redcap-Core

R2-2405459	Miscellaneous Corrections (Rapporteur)	Qualcomm Incorporated, Ericsson, Nokia, CATT	CR	Rel-18	38.304	18.1.0	0407	-	F	NR_UE_pow_sav_enh-Core, NR_redcap-Core, NR_UAV-Core

Both agreed

## 6.2	NR Sidelink relay

(NR_SL_Relay-Core; leading WG: RAN2; REL-17; WID: RP-212601)

Tdoc Limitation: 1 tdoc

AIP CRs

R2-2405365	Miscellaneous RRC corrections for  SL relay	Huawei, HiSilicon	CR	Rel-17	38.331	17.8.0	4682	2	F	NR_SL_relay-Core	R2-2403800

- Agreed

R2-2405366	Miscellaneous RRC corrections for SL relay	Huawei, HiSilicon	CR	Rel-18	38.331	18.1.0	4683	2	A	NR_SL_relay-Core	R2-2403801

- Agreed

R2-2405408	Correction on SidelinkUEInformationNR	Philips International B.V.	CR	Rel-17	38.331	17.8.0	4731	1	F	NR_SL_relay-Core	R2-2403398

- Agreed

R2-2405412	Correction on SidelinkUEInformationNR	Philips International B.V.	CR	Rel-18	38.331	18.1.0	4732	1	A	NR_SL_relay-Core	R2-2403400

- Agreed

Other documents

R2-2405349	Corrections for sidelink UE Information	ZTE, Sanechips	CR	Rel-17	38.331	17.8.0	4828	-	F	NR_SL_relay-Core

- Not pursued

R2-2405350	Corrections for sidelink UE Information	ZTE, Sanechips	CR	Rel-18	38.331	18.1.0	4829	-	A	NR_SL_relay-Core

- Not pursued

Discussion:

Apple understand that the Tx resource request is a general case, and they are not sure we need to exclude all of this functionality from the relay case.  They find the current text more flexible.

Ericsson think nothing is broken.

Nokia have some concern because the affected IE is also needed for the relay case.

ZTE indicate that if the relay and non-relay cases are not distinguished, sidelink communication behaviours may also cover the relay case.  On Nokia’s concern, they understand that the first level 4 bullet only applies to the non-relay case and the second level 4 bullet applies to both.

Huawei understand the change is that for the relay case, the UE will not include the Tx resource request, which is covered under other branches for relays.  They have some sympathy for the change but also agree with Ericsson that nothing is broken, because there is an “if needed” condition already.  ZTE think the “if needed” applies only to the subfields.

CATT agree with Ericsson and Huawei.

R2-2405363	Clarification to s-MeasConfig for L2 U2N relay case	Huawei, HiSilicon, Sharp, CATT, LG	CR	Rel-17	38.331	17.8.0	4830	-	F	NR_SL_relay-Core

- Agreed

R2-2405364	Clarification to s-MeasConfig for L2 U2N relay case	Huawei, HiSilicon, Sharp, CATT, LG	CR	Rel-18	38.331	18.1.0	4831	-	A	NR_SL_relay-Core

- Agreed

## 6.3	Void

## 6.4	NR positioning enhancements

(NR_pos_enh-Core; leading WG: RAN1; REL-17; WID: RP-210903)

Tdoc Limitation: 1 tdoc

AIP CR

R2-2405255	Introduction of NR UE Rx-Tx time difference measurement in NR UL E-CID	Ericsson, Polaris Wireless, China Telecom, NTT Docomo, AT&T, FirstNet, Intel, Comtech, Nokia, Nokia Shanghai Bell, Verizon Wireless, Huawei, ZTE	CR	Rel-18	38.305	18.1.0	0164	2	F	NR_pos_enh-Core	R2-2403740

- Agreed

Other documents

R2-2404625	Correction on SP SRS activation/deactivation MAC CE	ZTE Corporation	CR	Rel-17	38.321	17.8.0	1840	-	F	NR_pos_enh-Core

- Postponed

R2-2404626	Correction on SP SRS activation deactivation MAC CE(R18)	ZTE Corporation	CR	Rel-18	38.321	18.1.0	1841	-	F	NR_pos_enh-Core

- Postponed

Discussion:

Huawei understand the idea but think the correction does not completely resolve the problem, because there is also a cell ID.  ZTE understand that the network would use the last serving cell ID.  Huawei think SP-SRS in RRC_INACTIVE in Rel-17 is not a big problem, because the RRCRelease can just configure periodic SRS.

Qualcomm note that there is subsequent DL data transfer; the UE can send MO-LR via SDT and SP-SRS can be activated with subsequent DL data.  They also wonder if the cell ID by itself is enough.

ZTE intended that the cell ID would use the last serving cell, but they could revise the CR to make a similar change to the cell ID.

Intel think there is nothing wrong with the cell ID, just the BWP ID, and they think changing the behaviour for the cell ID might create a problem in Rel-18.  Huawei wonder if we will reuse this MAC CE in Rel-18; Intel understand that we will not.

ZTE suggest that we change BWP ID now and see how the cell ID should be handled in light of the MAC CE decisions for aggregated SRS.

R2-2404756	Correction on the UL TEG report	Huawei, HiSilicon, Ericsson	CR	Rel-17	38.331	17.8.0	4794	-	F	NR_pos_enh-Core

- Multi-RTT to be removed from the coversheet

- “a ue-TxTEG-ID” to be changed to “the ue-TxTEG-ID” in the last field description

- Remove duplicated field description table

- Add 6.2.2 to affected clauses

- Agreed with these changes as R2-2405862

R2-2404757	Correction on the UL TEG report	Huawei, HiSilicon, Ericsson	CR	Rel-18	38.331	18.1.0	4795	-	A	NR_pos_enh-Core

- Multi-RTT to be removed from the coversheet

- “a ue-TxTEG-ID” to be changed to “the ue-TxTEG-ID” in the last field description

- Remove duplicated field description table

- Add 6.2.2 to affected clauses

- Agreed with these changes as R2-2405863

R2-2405863	Correction on the UL TEG report	Huawei, HiSilicon, Ericsson	CR	Rel-18	38.331	18.1.0	4795	1	A	NR_pos_enh-Core

=> Revised in R2-2406099

* [Post126][417][POS] Update of mirror CR in R2-2405863 (Huawei)

Scope: Update the category A CR R2-2405863 to correct coversheet formalities and bring the changes in line with the agreed Rel-17 category F CR in R2-2405862.

Intended outcome: Agreed CR

Deadline: Short (for RP)

=> Agreed in R2-2406099

R2-2406099	Correction on the UL TEG report	Huawei, HiSilicon, Ericsson	CR	Rel-18	38.331	18.1.0	4795	2	A	NR_pos_enh-Core

=> Agreed

Discussion:

Qualcomm think the impacted functionality on the coversheet should not mention multi-RTT.

CATT think “a ue-TxTEG-ID” should be “the ue-TxTEG-ID” in the last field description.

Lenovo note there is a duplicated field description table, and 6.2.2 is missing from the affected clauses.

R2-2405282	Corrections to NR-On-Demand-DL-PRS-Information IE and ten-ms-unit-ResponseTime capability	Nokia	CR	Rel-17	37.355	17.8.0	0508	-	F	NR_pos_enh-Core

- Postponed

R2-2405283	Corrections to NR-On-Demand-DL-PRS-Information IE and ten-ms-unit-ResponseTime capability	Nokia	CR	Rel-18	37.355	18.1.0	0509	-	A	NR_pos_enh-Core

- Postponed

Discussion:

CATT think the fourth change is not accurate because OD-PRS is just a request for the configuration, not the transmission of PRS, so all the changes of the “transmission” wording are wrong.

Ericsson would also be fine with the removal of “transmission” in the fourth set of changes.  They think the purpose does not need to be extensively described here since we already have it in the stage 2.

ZTE think there is nothing functionally wrong and it is somewhat editorial.  They think the procedure is already clear from the stage 2 and stage 3 specs.

Nokia note that there are requests for start time/duration and PRS information, and the “transmission” changes were to disambiguate what the first one means.  They agree there are no functional changes, but they think at least the “recommended” aspect is important for clarity of the spec.

Intel think there is nothing wrong, but they agree the changes clarify.  ZTE understand that it is clear in the stage 2.

Qualcomm think the description of the QCL information is on the borderline of functional impact and should be in Rel-17.

ZTE think the QCL part is also clear without the CR.

R2-2405405	Correction on posSIB(s) acquisition [SI-SCHEDULING]	Philips International B.V., Ericsson	CR	Rel-17	38.331	17.8.0	4467	2	F	NR_pos_enh-Core	R2-2403387

- Work item to change to TEI17

- First change to be removed

- NE-DC and NR-DC should be impacted

- Filename format to be corrected (start with tdoc number)

- Agreed with these changes as R2-2405864

R2-2405406	Correction on posSIB(s) acquisition [SI-SCHEDULING]	Philips International B.V., Ericsson	CR	Rel-18	38.331	18.1.0	4725	1	A	NR_pos_enh-Core	R2-2403388

- Work item to change to TEI17

- First change to be removed

- NE-DC and NR-DC should be impacted

- Filename format to be corrected (start with tdoc number)

- Agreed with these changes as R2-2405865

Discussion:

Chair notes the WI code should be TEI17.

Lenovo think most of the changes are OK, but for change 1, the case is already covered because the type 2 SIB is discussed elsewhere in the existing text.  They also think some coversheet changes are needed (NE-DC and NR-DC should be impacted).

Huawei note the filename should start with the tdoc number.

R2-2405570	Correction of when to stop the triggered SR for positioning measurement gap activation/deactivation	Ericsson, vivo	CR	Rel-17	38.321	17.8.0	1871	-	F	NR_pos_enh-Core

- Postponed

R2-2405571	Correction of when to stop the triggered SR for positioning measurement gap activation/deactivation	Ericsson, vivo	CR	Rel-18	38.321	18.1.0	1872	-	A	NR_pos_enh-Core

- Postponed

Discussion:

Ericsson indicate that some more offline time is needed.

Huawei think this has been discussed previously and they do not see that anything is wrong.

vivo think Huawei’s comment makes sense for the current spec, but they see that the MAC CE may not be cancelled, and they think the CR can be lightly modified to make sense.  Ericsson indicate the CR has changed relative to vivo’s proposal.

Withdrawn/Not available

R2-2404617	Correction on SP SRS activation deactivation MAC CE(R17)	ZTE Corporation	draftCR	Rel-17	38.321	17.8.0	F	NR_pos_enh-Core	Withdrawn

R2-2404618	Correction on SP SRS activation deactivation MAC CE(R18)	ZTE Corporation	draftCR	Rel-18	38.321	18.1.0	F	NR_pos_enh-Core	Withdrawn

R2-2405608	Correction on internode message for posSRS in RRC_INACTIVE	Huawei, HiSilicon	CR	Rel-17	38.331	17.8.0	4849	-	F	NR_pos_enh-Core	Withdrawn

R2-2405609	Correction on internode message for posSRS in RRC_INACTIVE	Huawei, HiSilicon	CR	Rel-18	38.331	18.1.0	4850	-	A	NR_pos_enh-Core	Withdrawn

## 6.6	NR Sidelink enhancements

(NR_SL_enh-Core; leading WG: RAN1; REL-17; WID: RP-202846)

Tdoc Limitation: 1 tdoc

Note for RRC and MAC CRs, CR rapporteur’s summary and suggestion may be provided. CR rapporteurs will take care of miscellaneous CRs to collect small changes. Please contact / coordinate with CR rapporteur company first for small changes (e.g. non-controversial clarification/correction, editorial correction, etc.). This AI also includes in-principle agreed CRs (in-principle agreed CRs are not counted in tdoc limitation).

R2-2404494	Correction to MAC on cast type	Ericsson	CR	Rel-16	38.321	16.15.0	1836	-	F	NR_SL_enh-Core	Withdrawn

R2-2404495	Correction to MAC on cast type	Ericsson	CR	Rel-17	38.321	17.8.0	1837	-	F	NR_SL_enh-Core

R2-2404523	Correction to MAC on cast type	Ericsson	CR	Rel-18	38.321	18.1.0	1838	-	A	NR_SL_enh-Core

- “carryies” is changed to “includes”

- “eneity” is changed to “entity”

- Agreed in R2-2405891 and R2-2405892 with the changes above.

[Huawei]: Understand the intention. Alternatively, we can consider removing “as indicated by upper layer”, which may be simpler. [LG]: Support Huawei’s alternative option. [Ericsson]: If we remove it, it may bring some unclearness, e.g. whether MAC entity can set all cast type by itself, etc. [Apple]: Agree with Ericsson. [Toyota]: There is some typo, “carryies” should be changed to “carries”. And in the cover sheet, “eneity” should be changed to “entity”. [Nokia]: Prefer “includes” instead of “carries”.

R2-2405234	Correction on tx profile for SL DRX	ZTE Corporation, Sanechips	CR	Rel-17	38.331	17.8.0	4757	2	F	NR_SL_enh-Core	R2-2403921

R2-2405235	Correction on tx profile for SL DRX	ZTE Corporation, Sanechips	CR	Rel-18	38.331	18.1.0	4758	2	A	NR_SL_enh-Core	R2-2403922

- Latest CR form is used.

- CRs in R2-2405893 and R2-2405894 are agreed.

[Samsung]: Latest CR form is v12.3 [OPPO]: Have some more comments for the cover page. [Session chair]: Let’s comeback with the updated CRs in the afternoon (R2-2405893, R2-2405894)

R2-2405413	Correction on NR SL discovery transmission	Philips International B.V.	CR	Rel-17	38.331	17.8.0	4834	-	F	NR_SL_enh-Core

R2-2405432	Correction on NR SL discovery transmission	Philips International B.V.	CR	Rel-18	38.331	18.1.0	4835	-	A	NR_SL_enh-Core

[Nokia]: CR category is “D”.

- Agreed in R2-2405895 and R2-2405896 with the CR category change to “D”.

# 7	Rel-18

## 7.0	Common

Multi-WI Rel-18 items, e.g. cross-WI-issues not handled under another WI. UE capabilities.

### 7.0.1	UE Capabilities

Multi-WI handling of Rel-18 feature lists and UE capability Mega CRs.

RAN1/RAN4 Feature list LSs

R2-2404116	LS on Rel-18 RAN1 UE features list for NR after RAN1#116bis (R1-2403705; contact: NTT DOCOMO, AT&T)	RAN1	LS in	Rel-18	NR_MIMO_evo_DL_UL, NR_pos_enh2, Netw_Energy_NR, NR_netcon_repeater, NR_NTN_enh, NR_Mob_enh2, NR_SL_enh2, NR_redcap_enh, NR_MC_enh, NR_XR_enh, NR_FR1_lessthan_5MHz_BW, NR_DSS_enh, NR_BWP_wor, NR_cov_enh2, TEI18	To:RAN2, RAN4

=>	Noted

R2-2404130	LS on RAN4 UE feature list for Rel-18 (version 4) (R4-2406679; contact: CMCC)	RAN4	LS in	Rel-18	NR_ENDC_RF_FR1_enh2, NR_channel_raster_enh, NR_FR2_multiRX_DL, NR_RRM_enh3, NR_MG_enh2, NonCol_intraB_ENDC_NR_CA, NR_HST_FR2_enh, NR_ATG, NR_demod_enh3, NR_pos_enh2, NR_MC_enh, NR_Mob_enh2, NR_NTN_enh, NR_cov_enh2, Netw_Energy_NR, 4Rx_low_NR_band_handheld_3Tx_NR_CA_ENDC, NR_SL_enh2	To:RAN2	Cc:RAN1

=>	Noted

Rapporteur CRs

R2-2404527	Corrections and Updates to UE capabilities for Rel-18 WIs, including TEI18 [RAN1]	Intel Corporation	draftCR	Rel-18	38.306	18.1.0	B	NR_MIMO_evo_DL_UL-Core, NR_cov_enh2-Core, NR_MC_enh-Core, NR_Mob_enh2-Core, NR_FR2_multiRX_DL-Core, NR_DSS_enh-Core, NR_NTN_enh-Core, Netw_Energy_NR-Core, NR_FR1_lessthan_5MHz_BW-Core, NR_MG_enh2-Core, NR_SL_enh2-Core, NR_UAV-Core, TEI18

=>	The CR is endorsed as baseline and will be further updated post meeting

=> Endorsed and to be merged in the mega CR.

R2-2404528	Corrections and Updates to UE capabilities for Rel-18 WIs, including TEI18 [RAN1]	Intel Corporation	draftCR	Rel-18	38.331	18.1.0	B	NR_MIMO_evo_DL_UL-Core, NR_cov_enh2-Core, NR_MC_enh-Core, NR_Mob_enh2-Core, NR_FR2_multiRX_DL-Core, Netw_Energy_NR-Core, NR_FR1_lessthan_5MHz_BW-Core, NR_MG_enh2-Core, NR_SL_enh2-Core, NR_ATG-Core, TEI18

=>	The CR is endorsed as baseline and will be further updated post meeting

=> Endorsed and to be merged in the mega CR.

Mobility – to be treated in mobility breakout session first (maybe CB)

R2-2404705	Band-pair signalling for Early TA acquisition UE capabilities	Qualcomm Incorporated	discussion	Rel-18	NR_Mob_enh2-Core

Proposal:	Define dynamic UE capability reporting mechanism for RAN4’s FG39-4/4a/5, in which:

	In RRCReconfiguration/RRCResume, the network configures, a list of NR bands that the UE is requested to report as the target bands for RACH transmission.

	In RRCReconfigurationComplete/RRCResumeComplete, the UE reports, for each requested target band, the interruption time / preparation time required for the serving cells.

=>	Noted

R2-2405245	Mobility UE capabilities with Per band pair per band combination granularity	Huawei, HiSilicon	discussion	Rel-18	NR_Mob_enh2-Core

Proposal: Implement the new per band pair per band combination Rel-18 mobility capabilities with per FS granularity.

=>	Noted

### 7.0.2	CCCH LCID extension

Tdoc limitation: 1

Corrections only

### 7.0.3	ASN.1 Review

Contributions on common ASN.1 identified issues and other general issues (Tdoc limitation of 1 addressing all RILs applies to this AI as well)

Agreement

=>	ASN.1 is considered ready to be frozen after approval of CRs discussed over short email discussion

ASN.1 Rapporteur Input

NR Informational (to be Noted)

R2-2404788	NR ASN.1 Q2 Class 0 Issues	Ericsson	discussion	Rel-18	TEI18

=>	Noted

R2-2404789	NR ASN.1 Q2 Review file	Ericsson	discussion	Rel-18	TEI18

=>	Noted

R2-2404790	NR RIL List Q2	Ericsson	discussion	Rel-18	TEI18

=>	Noted

LTE Informational (to be Noted)

R2-2405402	LTE ASN.1 Review file	Samsung	discussion	Rel-18

=>	Noted

R2-2405403	LTE RIL List	Samsung	discussion	Rel-18

=>	Noted

Miscellaneous Rapporteur corrections

R2-2404791	Miscellaneous corrections from ASN.1 review Q2	Ericsson	CR	Rel-18	38.331	18.1.0	4717	1	F	NR_MT_SDT-Core, NR_MBS-Core	R2-2403322

=>	The CR is endorsed and will be further updated post meeting

=> Revised in R2-2406074

[POST126][004][ASN.1 common] 38.331 CR (Ericsson)

Intended outcome: Agree to CR capturing ASN.1 common issues – revision of R2-2404791

Deadline:  short

=> Agreed in R2-2406074

R2-2406074	Miscellaneous corrections from ASN.1 review Q2	Ericsson	CR	Rel-18	38.331	18.1.0	4717	2	F	NR_MT_SDT-Core, NR_MBS-Core

=> Agreed

Multi/General ASN.1 RIL

Rapporteur Proposed RIL Status

R2-2404865	RIL List for Multi/Gen issues	Ericsson	discussion	Rel-18	TEI18

=>	The following RILs are PropAgree: H071, E105, E159, E158, O310, E169, O318, O311

=>	The following RILs are PropReject: S865, E242, E167, E172

=>	Noted

[O319/O320]: SL feature co-configuration [Proposed status: Agree, Spec impact FFS] – [Impacted Features: Multi]

R2-2404718	[O319][O320] TP for SL features co-configuration 	OPPO	discussion	Rel-18	NR_SL_enh2-Core, NR_SL_relay_enh-Core, NR_pos_enh2	Late

Proposal 1: R2 adopt the TP in the Annex to reflect the agreement on SL feature co-configuration.

-	ZTE thinks that is is too complicate to clarify all these co-existance in ASN.1 and stage 2 would be sufficient. Qualcomm agrees with ZTE and the clarifications can be confusing. Ericsson agrees, these can be up to network implementation.

-	Interdigital thinks that we can extend the existing co-existance text in stage 2 to cover this text. Qualcommm thinks that we can just do it in 300, but not in positioning stage 2.

=>	Try to capture something simple in stage 2 in 38.300.

=>	Noted

R2-2404303	[O319][S865]Discussion on issues related to multi-features	vivo	discussion

SL-U co-existence with SL-PRS transmission

Proposal 3: RAN2 to decide the criteria for Tx UE to (re-)select a carrier for SL-PRS transmission at one time:

-	by UE implementation;

-	following the Tx carrier (re-)selection criteria for SL data transmission.

-	Vivo thinks that we should capture something in MAC CR. ZTE doesn’t think this is needed. SL PRS can only be transmitted in one carrier. Vivo would at least like to add a note to clarify which carrier is used for PRS transmission. ZTE thinks this is already clarified in SIB23 in field description there is only one entry.

=>	Noted

[S020]: SS-RSRPRef after RACH-less [Proposed status: ToDo] – [Impacted Features: MULTI]

R2-2405167	[S020] Setting SS-RSRPref for relaxed measurements after RACHless mobilty	Samsung	discussion

On whether RACHless is supported for ReDcap

-	ZTE thinks that the TP is incomplete and similar text should be added to 5.7.4.4.  Ericsson thinks that we should discuss this in RedCap, whether RACHless is supported for RedCap. Huawei thinks it would be good to clarify whether this feature can be supported for RedCap. ZTE thinks that we shouldn’t add a restriction and it should be linked to capability signaling. Oppo doesn’t think we should exclude as it is optional capability for the UE. Ericsson would like to avoid needed other changes and we should evaluate before we agree. Interdigital explains that when we agreed to generalize we also agreed to not optimize for features.

=>	 SS-RSRPRef is set upon indication from the lower layers that the LTM cell switch or RACH-less handover is successfully completed. This also applies to RedCap.

=>	S020 status is updated to PropAgree and the change/TP will be merged with RACHless CR.

=>	Noted

[S865]: MUSIM temporary capability restriction [Proposed status: Prop Reject] – [Impacted Features: MUSIM/Mob]

R2-2404303	[O319][S865]Discussion on issues related to multi-features	vivo	discussion

Proposal 7: [S865] is rejected.

=>	Noted

Other ASN.1 Issues

R2-2404517	Further discussion on use of non-critical extensions in LTE SIBs	Lenovo	discussion	Rel-18	LTE_UAV_enh-Core, IoT_NTN_enh-Core

Proposal 1: Move the Rel-18 aerial-related extensions from CarrierFreqNR-r15 to CarrierFreqListNR-v1810. This correction can be added into the endorsed LTE UAV CR [2].

-	Qualcomm thinks that make sense

Proposal 2: Define a new parallel list CarrierFreqListNR-v17xy on SIB24 level and add the Rel-17 ns-PmaxListNR-related extensions therein. This correction requires separate Rel-17/18 CRs .

-	Qualcomm explains that this means we have to dummify R17 fields.

-	Huawei, Ericsson, Nokia thinks we need to doublecheck

Proposal 3: Define a non-critical extension ServingSatelliteInfo-v18xy on SIB31 level, and add the Rel-18 IoT NTN-related extensions therein. This correction can be added into the IPA IoT NTN CR [3].

-	Qualcomm doesn’t see the overhead but if we want to be consistent on how we do it then this could be ok.  Lenovo agrees for Rel-18 this is not to improve overhead.

Proposal 4: Define a parallel list InterFreqCarrierFreqList-NB-v18xy on SIB5-NB level, and add the field satelliteAssistanceInfo-r18 into this new list element. This correction can be added into the IPA IoT NTN CR [3].

=>	Noted

Agreements:

- Move the Rel-18 aerial-related extensions from CarrierFreqNR-r15 to CarrierFreqListNR-v1810. This correction can be added into the endorsed LTE UAV CR.

- Define a non-critical extension ServingSatelliteInfo-v18xy on SIB31 level, and add the Rel-18 IoT NTN-related extensions therein. This correction can be added into the IPA IoT NTN CR.

- Define a parallel list InterFreqCarrierFreqList-NB-v18xy on SIB5-NB level, and add the field satelliteAssistanceInfo-r18 into this new list element. This correction can be added into the IPA IoT NTN CR.

### 7.0.4	RACH-less HO

Corrections to generalized RACH-less HO procedure, including NTN, mIAB, and overlapping sections of the LTM cell switch procedure

Tdoc limitation 1

Rapporteur CRs

R2-2404776	Rapporteur correction for RACH-less HO [RACH-lessHO]	Huawei, HiSilicon	CR	Rel-18	38.321	18.1.0	1845	-	F	TEI18

=>	The CR is revised and reviewed over email

=> Revised in R2-2406097

[POST126][037][RACHless] MAC CR (Huawei)

Intended outcome: agree to CR

Deadline:  short

R2-2406097	Rapporteur correction for RACH-less HO [RACH-lessHO]	Huawei, HiSilicon	CR	Rel-18	38.321	18.1.0	1845	1	F	TEI18, NR_Mob_enh2-Core, NR_NTN_enh-Core, NR_mobile_IAB-Core

=> Agreed

R2-2404971	Rapporteur corrections on RRC for the generalization of RACH-less [RACH-lessHO]	Ericsson	CR	Rel-18	38.331	18.1.0	4706	1	F	NR_mobile_IAB-Core, NR_Mob_enh2-Core, NR_NTN_enh-Core, TEI18	R2-2403182

=>	The CR is revised and reviewed over email discussion

=> Revised in R2-2406087

[POST126][036][RACHless] RRC CR  (Ericsson)

Intended outcome: Agree to RRC CR

Deadline:  short

R2-2406087	Rapporteur corrections on RRC for the generalization of RACH-less [RACH-lessHO]	Ericsson	CR	Rel-18	38.331	18.1.0	4706	2	F	NR_mobile_IAB-Core, NR_Mob_enh2-Core, NR_NTN_enh-Core, TEI18

=> Agreed

R2-2405044	RACH-less handover (38.306)	Samsung	CR	Rel-18	38.306	18.1.0	1114	-	F	TEI18, NR_Mob_enh2-Core, NR_NTN_enh-Core, NR_mobile_IAB-Core

=>	The CR is revised in R2-2406041

R2-2405069	RACH-less handover (38.331)	Samsung	CR	Rel-18	38.331	18.1.0	4810	-	F	TEI18, NR_Mob_enh2-Core, NR_NTN_enh-Core, NR_mobile_IAB-Core

=>	The CR is revised in R2-2406042

[Post126][038][RACHless] UE capability (Samsung)

Intended outcome: Endorse CRs

Deadline:  short

R2-2406041	RACH-less handover (38.306)	Samsung	CR	Rel-18	38.306	18.1.0	1114	1	F	TEI18, NR_Mob_enh2-Core, NR_NTN_enh-Core, NR_mobile_IAB-Core

=> Endorsed and merged to the mega CR

R2-2406042	RACH-less handover (38.331)	Samsung	CR	Rel-18	38.331	18.1.0	4810	1	F	TEI18, NR_Mob_enh2-Core, NR_NTN_enh-Core, NR_mobile_IAB-Core

=> Endorsed and merged to the mega CR

R2-2405555	RACH-less support generalization [RACH-lessHO]	Nokia	CR	Rel-18	38.300	18.1.0	0799	3	B	TEI18	R2-2403588

-	ZTE thinks that weed to clarify the early TA is only for LTM.  Ericsson points out that for LTM it is already captured in another section in RACH.

=>	check for early TA and whether the NR_Mob_enh2-Core is needed

=>	The CR is endorsed as a baseline

[AT126][012][RACHless] Stage 2 CR  (Nokia)

Intended outcome: Agree to CR by email

Deadline:  05-24-24

R2-2406018	RACH-less support generalization [RACH-lessHO]	Nokia	CR	Rel-18	38.300	18.1.0	0799	4	B	TEI18

=>	The CR is agreed

R2-2405997	Reply LS on parameters used for CG RACH-less Handover

-	CATT asks if we need to add some restrictions in the field description.

-	Ericsson confirms and that changes for SDT should be in a separate CR.

=>	ZTE will prepare a CR for REl-17 SDT for next meeting

=>	Ericsson will include the corresponding changes in the RACHless CR

=>	Noted

DRX and measurement gap behaviour during RACH-less

Configuration application during RACH-less HO

R2-2404666	Remaining issues on RACH-less HO	Apple	discussion	Rel-18	NR_mobile_IAB-Core, NR_Mob_enh2-Core, NR_NTN_enh-Core, TEI18

Proposal 1: For CG RACH-less HO, UE is required to acquire the timing information which is needed for the CG transmission before initiating the initial UL transmission in target cell.

Proposal 2: Agree the following RRC TP to reflect proposal 1.

Proposal 3: When UE initiates NTN RACH-less HO, UE is required to acquire the SFN timing information of target PCell and apply the DRX configuration.  [Can continue this in NTN Session]

Proposal 4: Agree the following RRC TP to reflect proposal 3.

-	LG thinks this is already clear.  Apple wonders if the UE should acquire the SFN to apply the DRX.

=>	Noted

R2-2405074	Handling of measurement gaps during RACH-less handover	Samsung, Lenovo	discussion

=>	Noted

R2-2405373	DRX during RACH-less handover	InterDigital	discussion	Rel-18	NR_NTN_enh-Core, NR_mobile_IAB-Core, TEI18

Proposal 1: As in LTM, RAN2 confirms that the DRX configuration and measurement gap configuration may be applied during on-going RACH-less handover. But, as in the current MAC specification, during on-going terrestrial RACH-less handover, UE considers DRX in Active Time and monitors PDCCH during measurement gap. No specification impact.

=>	Noted

Discussion

-	Asustek agrees with Samsung and Lenovo, but we need to remove “if there is an ongoing RACH-less LTM cell switch”.  Qualcomm agrees

-	Huawei think that the idea is to monitor PDCCH during RACHless so agree with Samsung’s proposal.

-	NEC agrees with Interdigital as it is already clear. Apple indicates that their paper aligns the behavior between LTM. Huawei inidcates that if we go with Apple’s TP on issue three then this fixes the issue and we don’t need to specify when the UE applies measurement gaps.

PDCCH monitoring during DRX/measurement gap

R2-2404666	Remaining issues on RACH-less HO	Apple	discussion	Rel-18	NR_mobile_IAB-Core, NR_Mob_enh2-Core, NR_NTN_enh-Core, TEI18

Proposal 5: During TN RACH-less HO and RACH-less LTM, in measurement gap if applied, UE monitors the PDCCH as specified in clause 5.7.

Proposal 6: During NTN RACH-less HO, in measurement gap if applied, UE monitors the PDCCH as specified in clause 5.7.

Proposal 7: Agree the following MAC TP to reflect proposal 5 and proposal 6.

=>	Noted

R2-2404688	Open issues on RACH-less HO	Qualcomm Incorporated	discussion	Rel-18	NR_mobile_IAB-Core, NR_Mob_enh2-Core, NR_NTN_enh-Core, TEI18

Proposal 2: Clarify the section 5.33 in TS 38.321 that “monitor the PDCCH as specified in TS 38.213” applies only when DG-based RACH-less HO is configured.

=>	Noted

R2-2404407	Remaining issues in RACH-less HO procedure	CATT	discussion

Proposal 6: There is no spec impact needed to support the agreements reached in RAN2#125bis for DRX handling during RACH-less handover (i.e. already supported by the current Spec).

=>	Noted

Agreements

1	During TN RACH-less HO and RACH-less LTM, in measurement gap if applied, UE monitors the PDCCH as specified in clause 5.7.

2	 During NTN RACH-less HO, in measurement gap if applied, UE monitors the PDCCH as specified in clause 5.7.

3	 Agree the MAC TP issue 3 in R2-2404666 to reflect above proposals.  No need for RRC change for measurement gaps

4	Agree to RRC TP NOTE 2:	The UE may omit reading the MIB if the UE already has the required timing information, or the timing information is not needed for random access or not needed for RACH-less initial UL transmission

Corrections to RACH-less HO

R2-2405997	Reply LS on parameters used for CG RACH-less Handover (R1-2405563; contact: Samsung)	RAN1	LS in	Rel-18	NR_NTN_enh-Core, NR_mobile_IAB-Core, TEI18	To:RAN2

=>	Noted

R2-2406009	Reply LS on parameters used for CG RACH-less Handover from LTM perspective (R1-2405671; contact: Fujitsu)	RAN1	LS in	Rel-18	-NR_mob_enh2-Core	To:RAN2

=>	Noted

R2-2404407	Remaining issues in RACH-less HO procedure	CATT	discussion

Proposal 1: Remove the condition "PDCCH addressed to the MAC entity's C-RNTI has been received" for the new transmission case for the configured grant in RACH-less handover/LTM cell switch.

Proposal 2: Define the following conditions for RACH-less handover successful completion:

-	If DL assignment is for a new transmission or;

-	If UL grant is for a new transmission on the same HARQ process.

Proposal 3: Replace the condition "PDCCH addressed to the MAC entity's C-RNTI has not been received for configured grant retransmission for RACH-less handover" with "the RACH-less handover procedure has not been completed successfully" in section 5.4.1.

Proposal 4: Remove the condition "PDCCH addressed to the MAC entity's C-RNTI has not been received" in section 5.8.2.

Proposal 5: Adopt the TP in the Appendix.

=>	Noted

Agreements: - Agree to TP: if the configured UL Grant is for the first PUSCH transmission during an on-going RACH-less handover procedure - Agree to TP: if there is an on-going RACH-less handover procedure: consider the RACH-less handover to be successfully completed and indicate to upper layers the successful completion of RACH-less handover. 3 Agree to the intention to: Replace the condition "PDCCH addressed to the MAC entity's C-RNTI has not been received for configured grant retransmission for RACH-less handover" with "the RACH-less handover procedure has not been completed successfully" in section 5.4.1.

R2-2404777	Discussion on the remaining issues RACH-less HO [RACH-lessHO]	Huawei, HiSilicon	discussion	Rel-18	TEI18

Proposal1: Remove the UE procedure for RACH-less HO/LTM cell switch from the paragraph for subsequent initial UL transmission.

=>	Noted

R2-2405330	Remaining issue on RACH-less handover generalization in MAC	LG Electronics Inc.	discussion	Rel-18	NR_Mob_enh2-Core, TEI18	R2-2403463

Proposal 1. SDT condition for retransmission of initial transmission using a configured grant is not merged with LTM and RACH-less HO, i.e.:

-	For SDT, “if PDCCH addressed to the MAC entity's C-RNTI has not been received” is applied;

-	For LTM and RACH-less HO, “if PDCCH addressed to the MAC entity's C-RNTI has not been received on the same HARQ process used for the first PUSCH transmission to the Serving Cell” is applied.

Proposal 2. RAN2 agree the TP in the Annex.

=>	Noted

targetNTA

R2-2405168	RIL S268	Samsung	discussion	Rel-18

Proposal 1: targetNTA-r18 is mandatory present when RACH-less HO configuration is provided. Remove the optional code for targetNTA-r18 in ASN.1.

Proposal 2: For generalized RACH-less HO, the N_TA to be applied for target PTAG is either 0 or the N_TA of a TAG indicated by a MCG TAG ID or a SCG TAG ID.

=>	Noted

R2-2404688	Open issues on RACH-less HO	Qualcomm Incorporated	discussion	Rel-18	NR_mobile_IAB-Core, NR_Mob_enh2-Core, NR_NTN_enh-Core, TEI18

Proposal 1: In the field description of targetNTA, clarify that the NTA value of the source corresponds to the source PTAG indicated by the tag-Id.

-	Ericsson clarifies that the TCI state already links the PTAG. Qualcomm would need to check for NTN as we don’t provide SSB index.

=>	Noted

R2-2404972	[S268][Q369] Avoid to optimize RACH-less for other Rel-18 features	Ericsson	discussion	Rel-18	NR_mobile_IAB-Core, NR_Mob_enh2-Core, NR_NTN_enh-Core, TEI18

Proposal 1	Mark RIL S268 as “Rejected”, and the current text is kept and RAN2 does not optimize to handle specific cases such as mTRP.

Proposal 2	Mark RIL Q639 as “Rejected”, and RAN2 does not further optimize RACH-less for any other Rel-18 feature which is not mobile IAB or NTN

=>	Noted

Agreements

- Mark RIL S268 as “Rejected”, and the current text is kept and RAN2 does not optimize to handle specific cases such as mTRP

- RIL Q639 will be address by rapporteur and changed to PropAgree

Miscellaneous corrections (if time allows)

R2-2404777	Discussion on the remaining issues RACH-less HO [RACH-lessHO]	Huawei, HiSilicon	discussion	Rel-18	TEI18

=>	Specify in the MAC - Only configured grant type 1 can be configured for RACH-less handover.

=>	Noted

R2-2405157	Further considerations on capabilities related to generalization of RACH-less	Samsung	discussion	Rel-18	TEI18

Proposal 1: RAN2 to discuss whether inter-frequency, FR1-FR2 and FDD-TDD handovers should be possible for RACH-less.

Proposal 2: Rel-18 generalized RACH-less is limited to intra- and inter-frequency handovers. FR1-FR2 and FDD-TDD handovers are not supported.

-	Nokia is asking what this means FR1-FR2 and why we can’t support it in the case of 0 TA.  Apple thinks it is not feasible to support FR1 and FR2 HO. Lenovo thinks that this is an optimization over an optimization.

Proposal 3: A capability for inter-frequency RACH-less is introduced. FFS whether the capability applies to both CG and DG RACH-less.

-	CMCC thinks that if the UE supports intra frequency there should be no problem for UE to supporte interfrequency. Samsung thinks that for RACHless you would have to synchronize over another frequency. Ericsson thinks that there may some more testing but from the UE side there shouldn’t be a need. Nokia agrees with Ericsson.

-	Vivo doesn’t think we need a separate capability as we can combine the different capabilities.

-	ZTE wonders what does this 1 bit mean as the RACHless capability is per band. Vivo explains that it will indicate this bit if it support it in all the UE supported intra-frequency RACHless bands.

=>	Noted

Agreements:

1	Rel-18 generalized RACH-less is limited to intra- and inter-frequency handovers. FR1-FR2 and FDD-TDD handovers are not supported.

2	A capability for inter-frequency RACH-less is introduced.  If the UE indicates this capability it means it supports inter-frequency RACH-less on all the UE supported intra-frequency RACHless bands.

### 7.0.5	Other

Including outcome of [POST125bis][003][RRC parameters] LS to RAN1 on RRC parameters (Ericsson)

Rapporteur Misc. Corrections CRs

R2-2404284	Correcting Figures	Nokia (Rapporteur)	CR	Rel-18	38.300	18.1.0	0859	-	F	TEI18, NR_newRAT-Core

=>	Need to update TEI code, updated bullets in summary of change, clauses impacted not completed, category should be D, add date, remove mark for ME and RAN (check).

=>	The CR is agreed in R2-2405859 with changes above

R2-2405859	Correcting Figures	Nokia (Rapporteur)	CR	Rel-18	38.300	18.1.0	0859	1	F	NR_newRAT-Core, TEI18

=> Agreed

R2-2405359	Miscellaneous corrections	Samsung (Rapporteur)	CR	Rel-16	38.321	16.15.0	1810	1	F	5G_V2X_NRSL-Core	R2-2403038

=>	The CR is agreed

R2-2405360	Miscellaneous corrections	Samsung (Rapporteur)	CR	Rel-17	38.321	17.8.0	1811	1	F	5G_V2X_NRSL-Core, NR_SL_enh-Core, NR_FeMIMO-Core	R2-2403039

=>	The CR is agreed

R2-2405361	Miscellaneous corrections	Samsung (Rapporteur)	CR	Rel-18	38.321	18.1.0	1812	1	F	5G_V2X_NRSL-Core, NR_SL_enh-Core, NR_FeMIMO-Core, NR_HST_FR2_enh-Core, NR_MT_SDT-Core	R2-2403040

=>	The CR is agreed

LS to RAN1 on RRC parameters

R2-2404782	Draft Reply LS on Rel-18 higher-layers parameter list	Ericsson	LS out	To:RAN1	Cc:RAN3, RAN4

-	Lenovo thinks that we should refer to the latest LS 2710 that RAN1 sent

-	Need to update R2-2405999 in the attachment section

=>	Review over email discussion and approve by email by end of week

[AT126][023][RRC parameters] LS to RAN1 (Ericsson)

Intended outcome: Approve LS to RAN1 by email

Deadline:  05-24-24

R2-2405999	Reply LS on Rel-18 higher-layers parameter list	RAN2	LS out	Rel-18	NR_MC_enh-Core, NR_MIMO_evo_DL_UL-Core, NR_pos_enh2-Core, Netw_Energy_NR, NR_cov_enh2, NR_XR_enh-Core, NR_Mob_enh2, NR_BWP_wor-Core, NR_NTN_enh, IoT_NTN_enh-Core, NR_SL_enh2-Core, TEI18	To:RAN1	Cc:RAN3, RAN4

=> Revised in R2-2406147

R2-2404783	ASN.1 names in RAN1 parameter list	Ericsson	discussion	NR_newRAT-Core

=>	add a tab as a cover page to include tdoc number, meeting number, and a quick explanation what the sheet is about.

=>	Revise in R2-2405860 with the changes above and agree to the content of the CR

R2-2406147	Reply LS on Rel-18 higher-layers parameter list	RAN2	LS out	Rel-18	NR_MC_enh-Core, NR_MIMO_evo_DL_UL-Core, NR_pos_enh2-Core, Netw_Energy_NR, NR_cov_enh2, NR_XR_enh-Core, NR_Mob_enh2, NR_BWP_wor-Core, NR_NTN_enh, IoT_NTN_enh-Core, NR_SL_enh2-Core, NR_netcon_repeater-Core, NR_DSS_enh, NR_redcap_enh-Core, TEI18	To:RAN1	Cc:RAN3, RAN4

=> Attachments are missing

=> Revised in R2-2406148

R2-2406148	Reply LS on Rel-18 higher-layers parameter list	RAN2	LS out	Rel-18	NR_MC_enh-Core, NR_MIMO_evo_DL_UL-Core, NR_pos_enh2-Core, Netw_Energy_NR, NR_cov_enh2, NR_XR_enh-Core, NR_Mob_enh2, NR_BWP_wor-Core, NR_NTN_enh, IoT_NTN_enh-Core, NR_SL_enh2-Core, NR_netcon_repeater-Core, NR_DSS_enh, NR_redcap_enh-Core, TEI18	To:RAN1	Cc:RAN3, RAN4

=> Approved

R2-2405860	ASN.1 names in RAN1 parameter list	Ericsson	discussion		NR_newRAT-Core

Other issues

R2-2405470	ASN.1 Review Overhaul	Nokia Corporation	discussion	Rel-19

Proposal 1: Discuss and consider a new ASN.1 and procedural text review procedure using Git version control.

=>	The document will be treated in August

=>	Noted

## 7.1	NR network-controlled repeaters

(NR_NetConRepeater; leading WG: RAN1; REL-18; WID: RP-230175)

Time budget: 0 TU

Essential corrections only. For smaller corrections please contact CR editor / Rapporteur directly.

### 7.1.1	Organizational

Including incoming LSs and rapporteur inputs.

R2-2405054	RILs conclusion for NCR	ZTE Corporation (Rapporteur)	report	Rel-18	NR_netcon_repeater

=>	N141 and N142 are confirmed as agreed

=>	Noted

R2-2405055	Miscellaneous RRC corrections for NCR	ZTE Corporation (Rapporteur), Nokia	CR	Rel-18	38.331	18.1.0	4809	-	F	NR_netcon_repeater

=>	The CR is agreed

R2-2405263	Clarification to Network-Controlled Repeaters Stage-2 description	Ericsson, Nokia	CR	Rel-18	38.300	18.1.0	0808	3	F	NR_netcon_repeater	R2-2403970

=>	The CR is agreed

### 7.1.2	Others

R2-2405679	38.306 correction on reference for NCR	Samsung	CR	Rel-18	38.306	18.1.0	1126	-	F	NR_netcon_repeater

=>	The CR is endorsed and will be merged in the mega CR

[AT126][015][NCR] Miscellaneous corrections (Apple)

Intended outcome: Discuss corrections submitted to AI 7.1 and agree to final CRs (if needed)

Deadline:  05-24-24

## 7.2	Expanded and improved NR positioning

(NR_pos_enh2; leading WG: RAN1; REL-18; WID: RP-232670)

Time budget: 0 TU

Tdoc Limitation: 3 tdocs

### 7.2.1	Organizational

Including incoming LSs and rapporteur inputs. CR rapporteurs are asked to continue maintaining an open issues list reflecting known issues to be handled during the maintenance phase.

Incoming LSs with RAN2 in Cc:

R2-2404105	Reply LS on SRS BW aggregation impact on other channels/signals (R1-2403539; contact: Huawei)	RAN1	LS in	Rel-18	NR_pos_enh2-Core	To:RAN4	Cc:RAN2, RAN3

- Noted

R2-2404140	LS reply on LCS user plane connection binding to the UE (S2-2405797; contact: CATT)	SA2	LS in	Rel-18	5G_eLCS_Ph3	To:CT1	Cc:CT4, RAN2

- Noted

Incoming LSs with “take into account” action and no related document

R2-2404107	LS on UE’s reporting SL PRS CBR measurement to gNB (R1-2403577; contact: Qualcomm)	RAN1	LS in	Rel-18	NR_pos_enh2-Core	To:RAN2

- Noted

R2-2404118	LS on the dci-FormatsSL and DCI format 3_2 (R1-2403732; contact: Qualcomm)	RAN1	LS in	Rel-18	NR_pos_enh2-Core	To:RAN2

- Noted

Other incoming LSs and related documents

R2-2404104	Reply LS on positioning MAC agreements (R1-2403536; contact: Huawel)	RAN1	LS in	Rel-18	NR_pos_enh2-Core	To:RAN2

- Noted

Discussion:

Huawei understand that the spec is in line with RAN1 understanding.

R2-2404111	Reply LS on decisions on SLPP (R1-2403622; contact: vivo)	RAN1	LS in	Rel-18	NR_pos_enh2-Core	To:RAN2

R2-2404304	Discussion on RAN1 Reply LS regarding SLPP parameters	vivo	discussion	Rel-18	FS_NR_pos_enh2

Proposal 1: Endorse TP in Annex1, to move sl-POS-ARP-ID-Tx into CommonSL-PRS-MethodsIEsProvideAssistanceData. Inform RAN1 about the decision.

Discussion:

Qualcomm think this does not make sense as assistance data.  They think it works fine in the RequestLocationInformation and is only valid for a single UE, and they do not see from the contributions a strong motivation for making this change.

Intel think if we use the location request procedure, the Tx UE has to signal a requested positioning method, and this is a misuse of the procedure.  Qualcomm still think the alternative does not work because the server talks only to a single UE (“UE1”), and there is no way for the server to know when the SL-PRS have been transmitted/measured.

Intel understand that the server can request assistance information from each anchor via CT1’s upper layer procedure.

vivo agree with Intel, and they think this information is subject to change with time and can also be provided as assistance data.

Qualcomm understand the procedures from SA2 are the same for the server UE and the LMF; it talks to the target UE, including SLPP messages for the anchor UEs, and the anchor UEs do not respond to the server directly.  They also observe that the assistance data have no timestamp, and the measurement reports include the time when the signal was measured.  They see no difference in procedures since the server (UE or LMF) only talks to UE1.

Intel understand we already agreed the server needs to get the sequence ID from each anchor UE.

ZTE understand Qualcomm’s argument is that the assistance data procedure should come first, but in their view there is no strict requirement on the order of the procedures.

Huawei wonder if the information might be needed in both RequestLocationInformation and ProvideAssistanceData.  If the UE is acting like a gNB (DL-like), it looks more like ProvideAssistanceData, and in the UL-like case it may be more like assistance information.

vivo indicate that other WGs view this information as assistance data, and they see no technical issue about where to put it.  They also understand that SLPP does not specify which role each UE is in; they are just endpoints.

Intel agree with vivo.

Qualcomm see it as similar to reporting TEGs as part of the measurement; this is information that will be used for position calculation, not to “assist” the measurement procedure.  They understand that the ProvideAssistanceData is for a list of UEs.

Intel understand that the server will receive several SLPP messages via UE1, each for one anchor UE.

[AT126][403][POS] Tx UE configuration as assistance data or location information (vivo)

Scope: F2F offline to further discuss P1 of R2-2404304 and attempt to converge.

Intended outcome: Report to CB session in R2-2405870 and approvable LS to RAN1 (to include responses to other SLPP issues where RAN1 need to be notified) in R2-2405871

Schedule: Tuesday 2024-05-21 0930-1030 in Brk3

Deadline:  Thursday 2024-05-23 1000 JST

Proposal 2: Endorse TP in Annex2, to introduce a list of measurement elements, to support multiple Rx ARPs reporting in single measurement report. Inform RAN1 about the decision.

Discussion:

ZTE think the list size should be larger than 4 based on the RAN1 LS.  Qualcomm have a different understanding and think we just need to repeat the measurement list multiple times if there are additional measurements; however, they think the request and capabilities also need to be modified.

vivo indicate there should be only one ARP ID corresponding to one SL-PRS resource in Rel-18, so they understand 4 is enough.

Agreement:

Introduce a list of 4 measurement elements, to support multiple Rx ARPs reporting in single measurement report.  Implementation to be worked out in SLPP rapporteur CR (including request and potentially capability, to be discussed).

Proposal 3: Adopt Draft Reply LS to RAN1 in Annex3.

R2-2405870	Report of [AT126][403][POS] Tx UE configuration as assistance data or location information	vivo	discussion	Rel-18	FS_NR_pos_enh2

Proposal 1: Endorse TP in Annex1, to add sl-POS-ARP-ID-Tx into CommonSL-PRS-MethodsIEsProvideAssistanceData. Inform RAN1 about the decision.

Agreement:

Add sl-POS-ARP-ID-Tx into CommonSL-PRS-MethodsIEsProvideAssistanceData. Inform RAN1 about the decision.  TP in Annex 1 of R2-2405870 can be used as implementation baseline.

R2-2405871	Reply LS on SLPP parameters provision	vivo	LS out	Rel-18	NR_pos_enh2-Core	To:RAN1

- Approved as R2-2405986

Discussion:

ZTE note that it is still formatted as a draft (source should be RAN2).

R2-2404112	Reply LS on questions on RAN1 parameter list (R1-2403636; contact: CATT)	RAN1	LS in	Rel-18	NR_pos_enh2-Core	To:RAN2	Cc:RAN3, RAN4

- Noted

R2-2404433	TPs for the Reply LS on questions on RAN1 parameter list	CATT	discussion	Rel-18	NR_pos_enh2

Proposal 1: RAN2 to agree the TP 1 above to support Rx hopping in DL-AoD.

Proposal 2: RAN2 to agree the TP 2 above for the aggregated report.

Discussion:

ZTE understand the LS indicates two cases, one where a single DL-PRS-ID is enough, another where multiple IDs are needed, and they think the TP does not address the first one, but the legacy DL-PRS-ID can be used for this case.  They think this can be reflected in the field description.

CATT understand the TP from ZTE changes the DL-PRS-ID from mandatory to OPTIONAL.

Qualcomm think the DL-PRS-ID should still be mandatory, because the resource can never be identified uniquely with only the resource set ID.  They understand we always need set ID / resource ID / DL-PRS-ID.

Huawei think it is OK to keep it optional, but if there is only one DL-PRS-ID it will be duplicated in multiple linkages.

Qualcomm think the point is that the IE should be included in the first measurement and the additional measurements, and as RAN1 indicated the subsequent measurements come from the same TRP and the ID is not needed there, so they think it makes sense to change it to OPTIONAL as proposed by CATT.

Ericsson think we could specify in the field description that it is always included for the first measurement.

ZTE think we can say “if the field is present and the DL-PRS-ID is present, the legacy DL-PRS-ID shall be ignored”.  Qualcomm think this is wrong because the legacy DL-PRS-ID should always be there in the first measurement and would apply to the additional measurements.

Proposal 3: RAN2 to agree set the RIL H006 “Rejected”.

[Chair’s note: Issue N013 is also related to the TPs in this document.]

Agreements:

Rx hopping in DL-AoD is supported as indicated by RAN1, with TP 1 from R2-2404433 as baseline.

Aggregated measurement reports are supported as indicated by RAN1, with TP 2 from R2-2404433 as baseline (DL-PRS-ID changed to OPTIONAL, with an indication that it is always provided for the first measurement).  TP to be checked as part of the LPP CR update.

H006 is rejected.

N013 is agreed.

R2-2404117	LS on PRS resource ID for bandwidth aggregation (R1-2403728; contact: ZTE)	RAN1	LS in	Rel-18	NR_pos_enh2-Core	To:RAN2

- Noted

R2-2404611	Discussion on reporting PRS resource ID in PRS BW aggregation	ZTE Corporation	discussion	Rel-18	NR_pos_enh2

Proposal 1: To achieve using the existing PRS resource ID for PRS aggregation, RAN2 to adopt the following LPP changes:

	Change nr-AggregatedDL-PRS-ResourceSetID-List-r18 from (SIZE (2.. 3)) to (SIZE (1.. 2));

	Change field description in nr-AggregatedDL-PRS-ResourceSetID-List-r18 to ‘if the field is present, the nr-DL-PRS-ResourceSetID-r16 should be present, and the nr-DL-PRS-ResourceID-r16 is taken as one of aggregated PRS resource ID’.

Adopt the corresponding TP.

Discussion:

Huawei would prefer to directly add the PRS resource ID into the linkage; they think this would be simpler.  Intel agree.  Samsung also agree.

ZTE clarify that this is intended to follow RAN1’s wording, but they agree that the exact structure from the LS is not mandatory; they can accept adding the resource ID to the linkage.

CATT understand that each measurement report includes the resource ID, but the TP from ZTE has only one resource ID.  They understand that if there are several aggregated measurement reports there should be several resource ID.  ZTE indicate that whether the PRS resources are aggregated depends on the configuration, and the LMF only needs to know one PRS resource ID; they understand this is why RAN1 said reuse is also workable.

Nokia are not sure how you can use one resource set to aggregate.  ZTE clarify that it is aggregated with the legacy value, so the number of aggregated resource set IDs is still 2..3.  However, companies seem to prefer putting the ID directly in the linkage.

Samsung understand that the field should be OPTIONAL since the LMF can infer the other aggregate resource sets.

Agreement:

Address PRS BW aggregation by adding the PRS resource ID to the linkage as an OPTIONAL field.

R2-2404125	LS on SL positioning measurements (R4-2406386; contact: Huawei)	RAN4	LS in	Rel-18	NR_pos_enh2-Core	To:RAN1, RAN2

R2-2404770	Discussion on the reply LS for SL positoning measurement	Huawei, HiSilicon	discussion	Rel-18	NR_pos_enh2

Proposal1: Specify in the field description of SL-AOA measurement that either one of Azimuth and Zenith results or both shall be present in SL-AOA-MeasElement. Adopt the TP in Annex A

Proposal2: Reply to RAN4 that it is not possible for SL-AOA measurement to report only SL PRS-RSRP and/or SL PRS-RSRPP measurement (without SL AzimuthAoA or SL ZenithAoA measurement) for SL-AoA positioning. Adopt the rely LS in Annex B.

Discussion:

Intel think this is in line with company views and Qualcomm have a related TP.

Ericsson think the power measurements could make sense by themselves for SL “E-CID” if we had such a thing.

Agreements:

At least one angular measurement (AoA/ZoA) is mandatory in the measurement report for SL-AoA positioning.

R2-2404769	LS on SL positioning measurements	Huawei, HiSilicon	LS out	Rel-18	NR_pos_enh2	To:RAN4

- Revised in R2-2405866

[AT126][404][POS] LS to RAN4 on SL positioning measurements (Huawei)

Scope: Revise the draft LS in R2-2404769 to include our agreement on AoA/ZoA measurements.

Intended outcome: Approved LS (without CB if possible) in R2-2405872

Deadline:  Thursday 2024-05-23 1000 JST

R2-2405872	LS on SL positioning measurements	Huawei, HiSilicon	LS out	Rel-18	NR_pos_enh2	To:RAN4

- Approved (email discussion [AT126][404])

Additional LSs received during meeting, to be treated on a time-available basis

R2-2405928	LS on higher layer parameters for Rel-18 Positioning (R1-2405403; contact: Intel)	RAN1	LS in	Rel-18	NR_pos_enh2-Core	To:RAN2

- Noted

R2-2405929	Reply LS on SRS bandwidth aggregation (R1-2405456; contact: ZTE)	RAN1	LS in	Rel-18	NR_pos_enh2-Core	To:RAN2	Cc:RAN3

- Noted

RIL and open issue lists

Note: RRC open issues document proposes resolutions for O800, H907, H908

R2-2404432	LPP RIL list for Rel-18 Positioning	CATT	discussion	Rel-18	NR_pos_enh2

- Noted

R2-2405256	Open issues list For RRC Positioning	Ericsson	discussion	Rel-18

Proposal 1	Current text which follows legacy SL-Commuication “if the UE is configured with sl-RxPool and/or sl-PRS-RxPool included in RRCReconfiguration message with reconfigurationWithSync (i.e. handover)” is kept as it is; i.e O800 is set to PropDisagree.

Proposal 2	H905 is agreed.

Proposal 3	H907 and H908 are PropDisagree and O320 is removed from Positioning RIL.

Agreements:

O800 is set to Rejected.

H905 is set to Agreed.

H907/H908 are set to Rejected.

R2-2405258	RIL For RRC Positioning	Ericsson	discussion	Rel-18

- Noted

Agreement:

PropAgree and PropReject/PropDisagree RILs from R2-2405258 are confirmed as Agreed/Rejected respectively.

=>Revised in R2-2406033

Rapporteur CRs (endorsed after RAN2#125bis)

R2-2404434	Corrections to TS 37.355	CATT	CR	Rel-18	37.355	18.1.0	0500	2	F	NR_pos_enh2	R2-2403818

=> Revised in R2-2405883

[Post126][406][POS] Rel-18 positioning LPP CR (CATT)

Scope: Update the CR in R2-2404434 in line with decisions of this meeting.  Late-arriving parameter updates from RAN1 can be taken into account if possible.

Intended outcome: Agreed CR in R2-2405883

Deadline:  Short (for RP)

R2-2405883	Corrections to TS 37.355	CATT	CR	Rel-18	37.355	18.1.0	0500	3	F	NR_pos_enh2-Core

=> Agreed

R2-2406100	[Post126][406][POS] Rel-18 positioning LPP CR (CATT)	CATT	discussion	Rel-18	NR_pos_enh2-Core

=> Noted

[Post126][406][POS] Rel-18 positioning LPP CR (CATT)

Scope: Update the CR in R2-2404434 in line with decisions of this meeting.  Late-arriving parameter updates from RAN1 can be taken into account if possible.

Intended outcome: Agreed CR in R2-2405883

Deadline:  Short (for RP)

R2-2405257	Miscellaneous and RRC Positioning RILs based Corrections	Ericsson	CR	Rel-18	38.331	18.1.0	4759	2	F	NR_pos_enh2	R2-2403819

=> Revised in R2-2405884

[Post126][407][POS] Rel-18 positioning RRC CR (Ericsson)

Scope: Update the CR in R2-2405257 in line with decisions of this meeting, including implementation of constraints on configuration of SL-PRS carrier in SIB23/preconfiguration.  Late-arriving parameter updates from RAN1 can be taken into account if possible.

Intended outcome: Agreed CR in R2-2405884

Deadline:  Short (for RP)

R2-2405884	Miscellaneous and RRC Positioning RILs based Corrections	Ericsson	CR	Rel-18	38.331	18.1.0	4759	3	F	NR_pos_enh2-Core

=> Agreed

R2-2406033	RIL For RRC Positioning	Ericsson	discussion	Rel-18	NR_pos_enh2-Core

=> Noted

R2-2406112	[Post126][407][POS] Rel-18 positioning RRC CR (Ericsson)	Ericsson	report	Rel-18	NR_pos_enh2-Core

R2-2405257	Miscellaneous and RRC Positioning RILs based Corrections	Ericsson	CR	Rel-18	38.331	18.1.0	4759	2	F	NR_pos_enh2	R2-2403819

[Post126][409][POS] Rel-18 positioning stage 2 (300) CR (vivo)

Scope: Develop a positioning CR to 38.300 in line with decisions of this meeting.

Intended outcome: Agreed CR in R2-2405886

Deadline:  Short (for RP)

=> Agreed in R2-2405886

[Post126][410][POS] Rel-18 positioning SLPP CR (Intel)

Scope: Update the CR in R2-2404191 in line with decisions of this meeting.  Late-arriving parameter updates from RAN1 can be taken into account if possible.

Intended outcome: Agreed CR in R2-2405887

Deadline:  Short (for RP)

R2-2405887	Miscellaneous corrections to SLPP specification	Intel Corporation	CR	Rel-18	38.355	18.1.0	0003	3	F	NR_pos_enh2-Core

=> Agreed

[Post126][412][POS] Rel-18 positioning RRC capability CRs (Xiaomi)

Scope: Update the CRs in R2-2404623 and R2-2404624 in line with decisions of this meeting.  Late-arriving parameter updates from RAN1 can be taken into account if possible.

Intended outcome: Endorsed CRs in R2-2405889 (38.331) and R2-2405890 (38.306)

Deadline:  Very short (for merge into mega CRs)

R2-2405889	Miscellaneous corrections for Rel-18 positioning UE capabilities	Xiaomi	CR	Rel-18	38.331	18.1.0	4772	2	F	NR_pos_enh2-Core

=> Endorsed and to be merged in the mega CR.

R2-2405890	Miscellaneous corrections for Rel-18 positioning UE capabilities	Xiaomi	CR	Rel-18	38.306	18.1.0	1090	2	F	NR_pos_enh2-Core

=> Endorsed and to be merged in the mega CR.

[Post126][414][POS] Rel-18 positioning SLPP capability CR (Xiaomi)

Scope: Update the CR in R2-2404760 in line with decisions of this meeting.  Late-arriving parameter updates from RAN1 can be taken into account if possible.

Intended outcome: Agreed CR in R2-2405982

Deadline:  Short (for RP)

R2-2405982	CR 38.355 for SLPP capability	Xiaomi	CR	Rel-18	38.355	18.1.0	0004	2	B	NR_pos_enh2-Core	R2-2403977

=> Agreed

Agreements:

The Rel-18 positioning RRC ASN.1 is considered in condition to freeze upon conclusion of the email discussion on the rapporteur CR.

The Rel-18 LPP ASN.1 is considered in condition to freeze upon conclusion of the email discussion on the rapporteur CR.

The Rel-18 SLPP ASN.1 is considered in condition to freeze upon conclusion of the email discussion on the rapporteur CR.

### 7.2.2	Stage 2

Impact to 38.300, 37.340, and 38.305. For each specification, a single CR with miscellaneous corrections is requested from the CR rapporteur; minor and editorial issues should be coordinated with the rapporteur and merged into the miscellaneous CR. Larger issues can be discussed based on contributions.

This agenda item may be handled at lower priority.

Rapporteur CR

R2-2405247	Miscellaneous Stage 2 Corrections	Qualcomm Incorporated	CR	Rel-18	38.305	18.1.0	0163	1	F	NR_pos_enh2	R2-2403188

=> Revised in R2-2405885

[Post126][408][POS] Rel-18 positioning stage 2 (305) CR (Qualcomm)

Scope: Update the CR in R2-2405247 in line with decisions of this meeting.

Intended outcome: Agreed CR in R2-2405885

Deadline:  Short (for RP)

R2-2405885	Miscellaneous Stage 2 Corrections	Qualcomm Incorporated	CR	Rel-18	38.305	18.1.0	0163	2	F	NR_pos_enh2

=> Agreed

Impact to 38.300

R2-2404765	Remaining issues R18 POS stage2 for TS 38.300	Huawei, HiSilicon, VIVO	discussion	Rel-18	NR_pos_enh2

Proposal1: Add the following descriptions for RAN stage2 spec for SL positioning, Adopt the TP in Annex A

	Resource allocation scehems for SL positionig, included scheme1 and scheme2

	Identities for scheduling in dedicated SL-PRS resource pool

Agreement:

Add the following descriptions for RAN stage2 spec for SL positioning:

Resource allocation schemes for SL positioning, included scheme1 and scheme2

Identities for scheduling in dedicated SL-PRS resource pool

Impact to 38.305

R2-2404435	Corrections on TS 38.305 for CPP	CATT, Nokia, NSB, Ericsson	CR	Rel-18	38.305	18.1.0	0165	-	F	NR_pos_enh2

- Postponed

Discussion:

Qualcomm wonder if the note is correct; it says “carrier phase positioning”, but the general requirement for the UE and PRU measurements is already there in the section.  CATT clarify that the note is not just for the measurement window but for the specific case of configuring a measurement window to the UE and the PRU.

Qualcomm think the mention of CPP should be removed.

Nokia think there was some confusion about which measurements the window applies to, and they can accept removing the mention of CPP, although they understand that the intention of the feature is specifically for CPP.

CATT do not see a RAN1 requirement that the window would be associated for the two devices for legacy measurements; this is why they identified CPP.  Qualcomm agree but also think there is no such requirement for CPP, only the behaviour of a reasonable implementation.

Ericsson understood that CPP would not work without alignment of the window.  Qualcomm do not see that this is a spec requirement.

Ericsson think we could check offline if there are RAN1 requirements to drive this.

Qualcomm note that we have not mentioned CPP in stage 2 before.

R2-2404766	Remaining issues R18 POS stage2 for TS 38.305	Huawei, HiSilicon	discussion	Rel-18	NR_pos_enh2

Proposal1: Add the following descriptions for RAN stage2 spec for LPHAP with pre-configured SRS when the UE sends RRC resume request. Adopt the TP in Annex A

	Event triggered when posSRS is pre-configured in RRC_INACTIVE

	When positioning SRS tarnsmission is on-going in RRC_INACTIVE:

	The UE moves from one validity area to another validity area

	The UE moves out of validity area

R2-2405259	DRX and PRS alignment for positioning	Ericsson	CR	Rel-18	38.305	18.1.0	0166	-	F	NR_pos_enh2

R2-2405284	Further clarifications for Positioning in RRC_INACTIVE state	Nokia	discussion	Rel-18	38.305	NR_pos_enh2-Core	R2-2403500

### 7.2.3	SLPP corrections

Impact to 38.355. A single CR with miscellaneous corrections is requested from the spec rapporteur; minor and editorial issues should be coordinated with the rapporteur and merged into the miscellaneous CR. Larger issues can be discussed based on contributions.

Open issues list

Open RILs: Rapp022, Rapp023, Rapp024, Rapp025, Rapp026

R2-2404189	[Post125bis][406][POS] 38.355 update Open Issue list	Intel Corporation	discussion	Rel-18	NR_pos_enh2-Core

- Noted

Rapporteur CRs (endorsed after RAN2#125bis)

R2-2404191	Miscellaneous corrections to SLPP specification	Intel Corporation	CR	Rel-18	38.355	18.1.0	0003	2	F	NR_pos_enh2-Core	R2-2403817

R2-2404760	CR 38.355 for SLPP capability	Xiaomi	CR	Rel-18	38.355	18.1.0	0004	1	B	NR_pos_enh2-Core	R2-2403977

Rapp022, Rapp023, Rapp026

R2-2404190	Rapp022, Rapp023, Rapp026, Further Considerations on SLPP related open issues	Intel Corporation	discussion	Rel-18	NR_pos_enh2-Core

Proposal 1: For SL-AoA-MeasElement, SL-RTT-MeasElement and SL-TDOA-MeasElement, change maxNrOfUE to maxNrOfARP (max number 256).

Proposal 2: reverse RAN2 agreements, the association information between ARP-ID and the already transmitted SL PRS resource(s) is still placed inside CommonSL-PRS-MethodsIEsRequestAssistanceData and SL-PRS-AssistanceData of CommonSL-PRS-MethodsIEsProvideAssistanceData;

Proposal 3: Server needs to inform the Rx UE of upcoming assistance data post SL PRS reception, i.e., the Rx UE should expect to receive this information subsequent to a SL PRS reception. Server should trigger the second round of assistance data transfer procedure to configure the association information for ARP-ID and already transmitted SL PRS to the Rx UE.

Proposal 4: introduce a new field to indicate the Tx UE to transmit SL-PRS immediately once resource is available. If this field is absent, the UE can store the SL-PRS-TxInfo for future SL-PRS transmission (e.g., triggered by SCI from a peer UE).

Discussion:

ZTE think one UE should not indicate in SLPP to another UE when to start transmitting SL-PRS; they understand that this is only used for the SL-RTT scenario, in which case the anchor UE should wait for the SCI, so they do not see the need to signal this explicitly.  Intel clarify that the proposal is for the server to indicate to the Tx UE.

Huawei think from the resource allocation pov, this is not entirely feasible: For RA scheme 1, the UE requests resources from the network, and for scheme 2 it is uncertain when the UE can secure the resources.  They also want to clarify that the peer UE should have started transmission before the ProvideAssistanceData.

Samsung think the intention of the proposal is to introduce a flag to indicate whether the SLPP message is for triggering transmission “right now” or providing Tx information for a future transmission triggered by SCI; they can accept the proposal.

vivo think the current Tx info can be used to trigger the UE to send SL-PRS and there is no need to indicate explicitly.

Qualcomm think the flag is needed for the TDOA scenario with anchor UEs that do not support Rx PRS, because such UEs will never receive an SCI, but we still need also the Tx parameters in SLPP in both cases.

ZTE think if the UE receives SCI after transmitting, it can always transmit it again; they see no restriction that if the UE sends according to SLPP it cannot send it according to a later SCI, so they think the current spec is OK.  They agree with Qualcomm’s scenario, but they think it can be done without an explicit flag.

Samsung think without the flag, the UE would always transmit the SL-PRS at least once, even though it might not be needed.  They think the 1-bit indication can just be used to indicate “transmit now” or not, with no restriction on future transmission.

vivo think we already have an agreement that the SL-PRS transmission parameters are provided by upper layer by implementation, so they do not see that the provision of transmission parameters is relevant to this scenario.

Intel understand that the server should provide bandwidth, delay budget, etc. to allow the UE to select resources properly, and the anchor does not know what positioning method is in use, so the Tx UE should be informed of when to transmit.  ZTE think the anchor does know based on which IEs were used to configure it.  Intel indicate we did not structure the assistance data in this way with different IEs per positioning method.

ZTE checked the assistance data and understand that the Tx UE can distinguish which method the assistance data are being configured for.  Intel indicate that the assistance data found by ZTE are location calculation assistance for the Rx UE.

Agreement:

Introduce a new field in the ProvideAssistanceData to indicate to the Tx UE to transmit SL-PRS once resource is available. If this field is absent, the UE can store the SL-PRS-TxInfo for future SL-PRS transmission (e.g., triggered by SCI from a peer UE).

Rapp025

R2-2404518	Text proposal on error messaging in SLPP	Lenovo	discussion	Rel-18	NR_pos_enh2

Proposal 1: Agree on the initial set of SL-specific error cause values as listed in Table 1.

[Chair’s note: Error causes as follows:

General::undefined

Measurements::notAllRequestedMeasurementsPossible

Assistance data::assistanceDataMissing

Assistance data::assistanceDataNotSupportedByServer

Assistance data::assistanceDataSupportedButCurrentlyNotAvailableByServer

Assistance data::notAllRequestedAssistanceDataAvailableByServer

]

Discussion:

Qualcomm think it is not clear why we need an explicit error for “not all measurements possible”, but they acknowledge we have it in LPP and we need a baseline set of values.

OPPO wonder if we need to capture something in stage 2.  Lenovo think we could do it in the stage 3 field descriptions as in LPP.

Intel understand we only have field descriptions for fields where they are needed, and if the errors are already clear we don’t need them.

Nokia generally agree with Qualcomm and think we need a motivation for the codes and clear behaviour for what will happen when we receive them.  Lenovo think if we do not agree an initial set we will just have a generic undefined error.

OPPO agree with Nokia.

Intel think some of the causes are not different from the device perspective, e.g., AD “not available” vs. “not supported”.  They think we should have different causes when the UE behaviour is different.

Proposal 2: Discuss and decide on the option for introducing SL-specific error cause values in SLPP.  [Common vs. method-specific IEs]

Discussion:

Qualcomm would somewhat prefer method-specific, in case we have method-specific error causes later.

[AT126][405][POS] SLPP error causes (Lenovo)

Scope: F2F offline to converge on a list of SLPP error causes.  If possible, determine whether to capture them in a common or method-specific way.

Intended outcome: Report to CB session in R2-2405873

Schedule: Wednesday 2024-05-22 0930-1030 in Brk3

Deadline:  Thursday 2024-05-23 1000 JST

R2-2405873	Report from offline discussion [AT126][405][POS] SLPP error causes (Lenovo)	Lenovo	discussion	Rel-18	NR_pos_enh2

Proposal 1: Define type of error information similar to LPP (“Option 1”). That means, an error cause value in SLPP will indicate that an error occurred at an endpoint without exposing any further information.

Proposal 2: Introduce the following error cause values: “undefined”, “notAllRequestedMeasurementsPossible”, “assistanceDataMissing” and “assistanceDataNotAvailable”.

Proposal 3: Add the new error cause values in the common SL-PRS methods ProvideLocationInformation IE and ProvideAssistanceData IE, method-specific ProvideLocationInformation IE and ProvideAssistanceData IE. TP to SLPP can be found in Annex B.

Discussion:

Lenovo clarify that a question was received about whether to extend the SL-RTT-ProvideAssistanceData (currently an empty IE) with an error cause.  They understand that we should have an error container.

Intel understand that there is no reason to have an error code where there is nothing that could cause an error, and any error on SL-RTT can be reported under the common IEs.

Qualcomm agree that we do not need an error code in an empty IE (and note that the request is also empty).  Lenovo are OK with not adding it in these cases.  Intel confirm that the empty IEs are extensible.

Agreements:

Define type of error information similar to LPP (“Option 1”). That means, an error cause value in SLPP will indicate that an error occurred at an endpoint without exposing any further information.

Introduce the following error cause values: “undefined”, “notAllRequestedMeasurementsPossible”, “assistanceDataMissing” and “assistanceDataNotAvailable”.

Add the new error cause values in the common SL-PRS methods ProvideLocationInformation IE and ProvideAssistanceData IE, method-specific ProvideLocationInformation IE and ProvideAssistanceData IE (except for SL-RTT-ProvideAssistanceData). TP to SLPP from Annex B of R2-2405873 can be used as implementation baseline.

Rapp024

R2-2405268	Open SLPP issues	Nokia	discussion	Rel-18

Proposal 2: RAN2 to define relative velocity with uncertainty as defined in TS 23.032 based on

-	absolute value of the radial speed component,

-	vector of the traversal speed component, consisting of its absolute value, azimuth and elevation

while specifying

-	independent uncertainty and confidence values for each radial / traversal component parameter.

Proposal 3: Application layer ID is used to define the reference point employed for relative velocity measurements.

Proposal 4: RAN2 to adopt the text proposal associated with Proposals 2 and 3 as section 2.2b below.

Proposal 5: RAN2 to decide if to send an LS to SA2 with the request to define the traverse velocity component as a vector specified by its absolute value, azimuth and elevation.

Discussion:

Huawei think this is OK as a baseline.  They wonder why/how SA2 settled on defining relative to the horizontal plane of device A.

Qualcomm think there is some confusion about the term “velocity” since the unit is degrees/second rather than m/s.  They think P5 does not define transverse velocity properly in this sense.

Nokia understand that the change of direction is a 3-D vector, irrespective of specific units.

Qualcomm think we do not need the application layer ID; the reference point is always the requesting device.

Agreement:

Define relative velocity with uncertainty as defined in TS 23.032 based on

-	radial velocity component,

-	angular velocity components (exact representation to be discussed in CR implementation)

while specifying

-	independent uncertainty and confidence values for each radial / traversal component parameter.

Details to be further discussed in SLPP CR implementation.

[AT126][406][POS] Remaining SLPP issues (Intel)

Scope: F2F offline to discuss remaining SLPP issues with ASN.1 impact.

Intended outcome: Report to CB session in R2-2405874

Schedule: Wednesday 2024-05-22 0900-0930 in Brk3

Deadline:  Thursday 2024-05-23 1000 JST

R2-2405874	[AT126][406][POS] Remaining SLPP issues (Intel)	Intel Corporation	discussion	Rel-18	NR_pos_enh2-Core

[Proposals:]

	Rapp022, update the TP (P1 in R2-2405248) to treat sl-LCS-GCS-Translation in the same way as applicationLayerID. Capture the updated changes in Rapporteur CR. The field name of SL-AoA-MeasElementPerARP-ID-Rx can be reconsidered in next meeting.

	introduce “sl-AoA-Request                        ENUMERATED { aoa, zoa, both },” in SL-AoA-RequestLocationInformation

	Delete the two-level structure and the applicationLayerID in SL-TOA-SignalMeasurementInformation., i.e. P3 from R2-2404612

	Rapp024 Introduce relative velocity, capture the TP P3 from R2-2405248 into Rapporteur CR for relative velocity.

	Send LS to SA2, indicate the differences between RAN2 TP on relative velocity and SA2 definition (Nokia).

Discussion:

Nokia want to clarify exactly what we are asking SA2.  Intel understand that we should just describe our implementation and its differences from SA2 design and ask them to take it into account.

Qualcomm think we should tell them how we interpreted their requirements and ask if they have any comments.

	P1 multiple UE capabilities in ProvideUECapabilties of R2-2404763 is not pursued.

	Send LS to CT4 and CC SA2, indicate the maximum devices supported in SLPP is 256. Ask CT4 to take it into account in their future work (Huawei).

Discussion:

Huawei wonder exactly what we indicate.  Intel think we can just say that in our specs there can be up to 256 devices.

vivo wonder if we should clarify that there may be multiple messages for a UE.  Huawei think this is already supported in the CT1/CT4 specs.

	P2 of R2-2404763 on synchronization is not pursued

	positioning method for server in Metadata (R2-2404869) is not pursued.

Agreements:

Rapp022 moves to Agreed.  Update the TP (P1 in R2-2405248) to treat sl-LCS-GCS-Translation in the same way as applicationLayerID. Capture the updated changes in Rapporteur CR. The field name of SL-AoA-MeasElementPerARP-ID-Rx can be reconsidered in next meeting.

Introduce “sl-AoA-Request                        ENUMERATED { aoa, zoa, both },” in SL-AoA-RequestLocationInformation.

Delete the two-level structure and the applicationLayerID in SL-TOA-SignalMeasurementInformation., i.e. P3 from R2-2404612.

Rapp024 moves to Agreed.  Introduce relative velocity, capture the TP P3 from R2-2405248 into Rapporteur CR for relative velocity. Send LS to SA2 to indicate the agreed RAN2 TP on relative velocity, and invite SA2 to comment (Nokia).

P1 of R2-2404763 on multiple UE capabilities in ProvideUECapabilties is not pursued.

Send LS to CT4 and CC SA2, indicate the maximum devices supported in SLPP is 256. Ask CT4 to take it into account in their future work (Huawei).

P2 of R2-2404763 on synchronization is not pursued.

Positioning method for server in Metadata (R2-2404869) is not pursued.

[Post126][415][POS] LS to SA2 on relative velocity (Nokia)

Scope: Draft an LS to SA2 in line with the conclusion of RIL Rapp024, indicating what RAN2 have agreed on relative velocity and inviting SA2 to comment.

Intended outcome: Approved LS in R2-2405987

Deadline:  Short (not for RP)

R2-2405987	LS on relative velocity	RAN2	LS out	Rel-18	NR_pos_enh2-Core	To:SA2

=> Approved

[Post126][416][POS] LS to CT1/CT4 on maximum devices supported in SLPP (Huawei)

Scope: Draft an LS to CT1/CT4, Cc: SA2, indicating our requirement in SLPP to support up to 256 devices and requesting them to take it into account.

Intended outcome: Approved LS in R2-2405988

Deadline:  Short (not for RP)

R2-2405988	LS on the maximum number of devices supported in SLPP	RAN2	LS out	Rel-18	NR_pos_enh2-Core	To:CT1, CT4; Cc:SA2

=> Approved

Other contributions

R2-2404305	Discussion on open issue of SLPP specification	vivo	discussion	Rel-18	FS_NR_pos_enh2

R2-2404612	Discussion on remaining corrections in SLPP	ZTE Corporation	discussion	Rel-18	NR_pos_enh2

R2-2404742	Discussion on SLPP relative velocity	Xiaomi	discussion	Rel-18	NR_pos_enh2

R2-2404763	Discussion on the remaining issues for R18 SLPP	Huawei, HiSilicon	discussion	Rel-18	NR_pos_enh2

R2-2404869	Discussion on the necessity of including the server UE positioning method in the discovery message	OPPO	discussion	Rel-18	NR_pos_enh2

R2-2405248	Remaining issues for SLPP	Qualcomm Incorporated	discussion

R2-2405390	Discussion on SL-PRS Tx triggering by SLPP	Samsung	discussion	Rel-19	NR_pos_enh2	Late

Withdrawn/Not available

R2-2404710	Open SLPP issues 	Nokia France	discussion	Withdrawn

### 7.2.4	LPP corrections

Impact to 37.355. A single CR with miscellaneous corrections is requested from the CR rapporteur; minor and editorial issues should be coordinated with the rapporteur and merged into the miscellaneous CR. Larger issues can be discussed based on contributions.

Rapporteur CR on capabilities (endorsed after RAN2#125bis)

R2-2404595	Miscellaneous corrections on LPP for Rel-18 positioning UE capabilities	Xiaomi	CR	Rel-18	37.355	18.1.0	0503	1	F	NR_pos_enh2-Core	R2-2403978

[Post126][413][POS] Rel-18 positioning LPP capability CR (Xiaomi)

Scope: Update the CR in R2-2404595 in line with decisions of this meeting.  Late-arriving parameter updates from RAN1 can be taken into account if possible.

Intended outcome: Agreed CR in R2-2405981

Deadline:  Short (for RP)

R2-2405981	Miscellaneous corrections on LPP for Rel-18 positioning UE capabilities	Xiaomi	CR	Rel-18	37.355	18.1.0	0503	2	F	NR_pos_enh2-Core

=> Agreed

ToDo RILs: M001, H006, N013

Note: H006 and N013 are addressed in R2-2404433

R2-2404510	[M001] Inclusion of PRU as an LPP endpoint	MediaTek Inc., Ericsson	draftCR	Rel-18	37.355	18.1.0	F	NR_pos_enh2-Core

Discussion:

Samsung agree with the principle but wonder if it should go from Rel-17, since we already have the PRU concept in stage 2 there.  Qualcomm think this is true but we have no stage 3 impact from PRUs in Rel-17.

Huawei think it depends on how we interpret “target device”.

Nokia think the coversheet is a bit unclear about the PRU not being positioned; it needs to determine its location somehow.

Agreement:

Paragraph on PRUs from R2-2404510 to be captured in the LPP rapporteur CR.

M001 is Agreed.

ASN.1 impact

Measurement time window (P1,P2)

Failure cause for RSCP/RSCPD measurements (P3)

Failure cause for location information (P4 first, then detailed proposals in P5/P6/P7 if agreeable; also related to R2-2404235 P1)

R2-2405261	Removing Legacy terminology for Correction on Time Window Config and other CPP corrections and PRU Error cause	Ericsson	discussion	Rel-18

Proposal 1	Replace supportOfLegacyMeasurementInTimeWindow by supportOfMeasurementInTimeWindowForDL-TDOA and similarly for other methods.

Proposal 2	Adopt the text proposal from section 2.2 signal which measurements are to be performed in the configured time window.

Discussion:

Nokia wonder why an explicit measurement needs to be part of the time window configuration instead of relying on the requested measurements.  Ericsson wonder if it is clear that the UE is always intended to perform all requested measurements in the time window.  Qualcomm understand that the Ericsson proposal would decouple the measurements in the time window from the UE capability to perform legacy measurements in the time window, and they are not sure if there is a use case for it; however, they find Ericsson’s proposal a bit safer in case a use case is found.

Qualcomm think some details are missing from the TP.

Xiaomi think the terminology can be changed, but there is no need for an explicit list.

Proposal 3	Allow UE to report failure (error cause) for RSCP, RSCPD measurements when timing estimation are unrerliable.

Discussion:

Qualcomm think this is not needed and we have no definition of “unreliable” timing.  The UE will always report the measurements it is able to make, and if it cannot measure, it is clear from the report that the measurement is absent.  They see a new UE requirement if we would expect it to detect the reliability of the timing, and this would be RAN4 scope.

Intel think we should not introduce a new failure cause for each measurement result; they have the same understanding as Qualcomm about the missing measurement.

Ericsson think the UE will have to take timing measurements, and it will inherently determine what positioning error it has; they think there is no point if the UE’s timing is not good enough to do phase estimation.  They understand that existing DL-TDOA requirements already force the UE to do this timing evaluation.

Qualcomm understand that the UE is just reporting measurements, not resolving the integer ambiguity, and the UE should report what it can report, with whatever quality it achieves.

Nokia wonder what the LMF would do with this information.

Lenovo think the premise is that feedback may be needed when the measurement is not possible, and it would be reasonable to provide something so the LMF can declare failure of the positioning operation.

OPPO think the TDOA method also requires synchronization and the issue is not specific to CPP.

Huawei wonder if there is a clear way for the UE to make the judgement or if it would be left to UE implementation.  Ericsson understand that the UE always derives timing quality, and from there it could decide whether the phase measurement is feasible to do; they are not sure if there is a corresponding RAN4 requirement.

MediaTek think we would need a definition of “unreliable” from RAN4.

Proposal 4	Allow the UE configured as a PRU and configured with the locationInformatioType locationEstimateAndMeasurementsRequired-r18 to indicate a specific error when it is able to provide location measurements but not a location.

Discussion:

Qualcomm think if the PRU omits the location in the report, it is clear that something happened and it cannot provide its location.  They also think the common IEs should not be used for this error case.

Ericsson wonder if absence of the location means the UE will not report anything or report with the location absent.  They can accept moving this indication outside of the common IEs.

CATT wonder if we should specify an error cause for the location estimate, and if we need to specify how the PRU provides the location.  Qualcomm think there is no other choice since we use the location information for this report; the location estimate cannot be separated from the measurements.

CATT understand that the current requirement is that the PRU should provide its location, but this does not mean it should calculate its location according to the indication from the LMF.  They are not sure what the LMF will use this error for.  Ericsson indicate that the LMF could filter its choice of PRUs.  Chair wonders if this is different from omitting the location.

CATT also think the PRU should report an error, but they think it should not be specific to the positioning method.

Ericsson think we could clarify in the field description that absence of the location indicates that the PRU could not provide it.  Qualcomm think we have a statement in other places saying that information not provided means the information was not available; they could accept it here but wonder if we should start adding such comments everywhere.

Proposal 5	Agree to add a specific error to represent indications of failed location estimates from a PRU when configured with the locationInformatioType locationEstimateAndMeasurementsRequired-r18

Proposal 6	Add a specific location source for the location estimate part for the UE configured as a PRU and configured with the locationInformatioType locationEstimateAndMeasurementsRequired-r18.

Proposal 7	Agree to the text proposal in Appendix.

Agreements:

Replace the field name supportOfLegacyMeasurementInTimeWindow by supportOfMeasurementInTimeWindowForDL-TDOA and similarly for other methods.

Adopt the principle of the text proposal from section 2.2 to signal optionally which measurements are to be performed in the configured time window.  Details to be checked in CR implementation.

Capture in field description that the absence of a PRU location indicates that the PRU was unable to provide it.  Exact wording to be determined in CR implementation.

PFL indication for CPP

R2-2404872	Discussion on DL RSCPD RSCP measurement on single DL PFL	OPPO	discussion	Rel-18	NR_pos_enh2

Proposal : RAN2 to discuss which of following option should be adopted:

1.	LMF could configure on which PFL the UE shall perform DL RSCPD/RSCP measurement in the NR-Multi-RTT- RequestlocationInformation/NR-DL-TDOA-RequestLocationInformation message

2.	UE reports the information of the PFL on which it performs the DL RSCPD/RSCP measurement in the NR-Multi-RTT- ProvideLocationInformation/NR-DL-TDOA-ProvideLocationInformation message

Discussion:

ZTE think this should be a RAN1 discussion, and they think it is not necessary because the measurement will be taken on the same PFL as the other measurements.

CATT indicate that the existing protocol already allows the LMF to tell the UE which PFL to measure, and RAN1 agreed that RSCPD and RSTD should be measured on the same PFL.  Huawei agree.

Xiaomi think it is needed for PRS aggregation.  CATT understand CPP cannot work with carrier aggregation, and if the LMF wants the UE to perform aggregation, the LMF will not request CPP measurements.

OPPO understand that nothing stops the LMF from configuring more than one PFL and requesting RSCPD.

Nokia understand RAN1 indicated that there is no impact on reporting if the UE is supposed to use one PFL, so they think it can be left to UE implementation unless RAN1 give us a requirement.

Huawei think for CA, there are cases where it is not feasible for different carriers to be phase-synchronized, and in this case CPP would be incompatible with CA.

Ericsson think if option 1 is supported, we do not need to discuss that point further, but the broader discussion of CPP with CA is different; they understand that the view of most companies is that they are incompatible.  They think the intention of the proposal is a bit unclear.

Qualcomm think deciding the compatibility is RAN1 business.  Nokia have the same view and think we cannot decide anything here.

DL-AoD capability issue

R2-2404870	Discussion on no support of bandwidth aggregation by the DL-AOD	OPPO	discussion	Rel-18	NR_pos_enh2

Proposal 1: RAN2 to agree to delete nr-DL-AoD-OnDemandPRS-ForBWA-Support-r18 IE from NR-DL-AoD-ProvideCapabilities.

Discussion:

Xiaomi think it is OK to remove.

Qualcomm wonder about the case that RSRP/RSRPP measurements would be made in aggregated form; they do not see why this is not compatible with DL-AoD.

CATT understand RAN1 do not support bandwidth aggregation for DL-AoD, so there is no need for the capability; also there is no such capability in the feature list.

ZTE agree with CATT about the state of RAN1 agreements; aggregation is supported only for timing measurements.  They also wonder why this capability is connected with OD-PRS support.

Xiaomi clarify that this capability was introduced by RAN2, not from the RAN1 feature list.  Huawei wonder if OD-PRS is per positioning method in any case; they think it is not specified this way.

Agreement:

Delete nr-DL-AoD-OnDemandPRS-ForBWA-Support-r18 IE from NR-DL-AoD-ProvideCapabilities.

ASN.1 impact, but may be covered by other discussions

R2-2404235	Discussion on the remaining LPP issues	CATT	discussion	Rel-18	NR_pos_enh2

R2-2404761	Discussion on the remaining issues for R18 LPP	Huawei, HiSilicon	discussion	Rel-18	NR_pos_enh2

Other documents

R2-2404613	Discussion on PRS bandwidth aggregation in LPP	ZTE Corporation	discussion	Rel-18	NR_pos_enh2

R2-2404871	Discussion on UE capability on bandwith aggregation positioning	OPPO	discussion	Rel-18	NR_pos_enh2

Proposal 1: RAN2 to agree to delete the words “within a MG” from both of the IE names and the field descriptions for the IEs prs-BWA-TwoContiguousIntrabandInMG-RRC-IdleAndInactive and prs-BWA-ThreeContiguousIntrabandInMG-RRC-IdleAndInactive.

Discussion:

Lenovo think it is OK for the field names but not the IE names, because they are also used for common capabilities.  Xiaomi have the same understanding.

Qualcomm wonder if the reporting really is intended to be the same in connected and idle/inactive.  Xiaomi confirm this is the intention; Lenovo have the same understanding but clarify that the values can be set differently for different RRC states.

Xiaomi think we could restructure the IEs to have separate versions for the different states.

Agreement:

Delete the words “within a MG” from the field descriptions for the IEs prs-BWA-TwoContiguousIntrabandInMG-RRC-IdleAndInactive and prs-BWA-ThreeContiguousIntrabandInMG-RRC-IdleAndInactive.

### 7.2.5	RRC corrections

Impact to 38.331, except for UE capabilities. A single CR with miscellaneous corrections is requested from the CR rapporteur; minor and editorial issues should be coordinated with the rapporteur and merged into the miscellaneous CR. Larger issues can be discussed based on contributions.

G117 (status unclear)

R2-2405323	Correction on srs-PosRRC-Inactive handling	Google	CR	Rel-18	38.331	18.1.0	4822	-	F	NR_pos_enh-Core

Discussion:

Ericsson think this is a bit a matter of taste, and since it has no ASN.1 impact we could come back and follow what is decided for other WIs.

Google understand that the conclusion was that we only need to include explicit handling for specific important cases, and they think we could evaluate whether this is one of the important cases.

Huawei think there is value in the handling of the legacy vs. Rel-18 fields, but they think it could be left for the rapporteur to decide.

Intel understand that we have no autonomous release for Rel-18, so the changes as drafted are not correct.  They understand that Rel-17 is a different behaviour.

Agreements:

G117 is marked as Rejected.

Rapporteur to consider in CR implementation how much explicit handling of release branches is useful to have.

Rel-17 behaviour can be discussed separately based on contributions if needed.

Additional RILs (closed but with contributions)

R2-2404306	[V800] Correction on UE not supporting NR sidelink positioning in limited service state	vivo	draftCR	Rel-18	38.331	18.1.0	F	FS_NR_pos_enh2

- Endorsed (to be merged into rapporteur CR)

Discussion:

vivo clarify that other instances of “positioning” should be removed in the same bullet.

R2-2404764	Discussion on the remaining issues for R18 RRC [H905][H920-921]	Huawei, HiSilicon	discussion	Rel-18	NR_pos_enh2

Proposal2: The application of the pre-configured positioning SRS is not dependent on the pre-configuration set to "setup". [H920] Adopt the TP below:

Proposal3: Clarify in the cell selection/reselection that the srs-PosConfigValidityArea corresponds to the SRS configuration being applied for the on-going positioning SRS transmission. [H921] Adopt the TP below

Discussion:

ZTE think for P2, in Rel-17 we have a NOTE saying that the network always provides full SRS configuration to the UE in RRCRelease, and they think the same principle applies here: Only when the network configures SRS can the UE apply the preconfiguration.  So they think the current form of the spec is good.

Samsung understand Huawei’s concern and proposed a wording change previously; they are OK with the change if the current wording is still considered confusing.

Huawei clarify that this is not about delta configuration but just when the UE should send the activation request.

Intel think there are two issues being discussed, when the configuration can be applied and when the SRS can be activated.  They understand that Huawei want to avoid linking the two, so they see the proposal as valid.

Ericsson think the UE should store the configuration when provided, and for the other part, the UE should only start transmitting when an event occurs; they are not sure how best to capture this, e.g., “if the UE has a stored configuration and an event from the upper layer arrives”.

ZTE, Intel, and Huawei think the new bullet can be at level 1, so it does not depend on including the suspendConfig in this RRCRelease.

Proposal4: Add the RRC fields for R18 enhancements for positioning SRS transmission in RRC_INACTIVE in R18 into a container. Adopt the TP in Annex A.

Discussion:

Huawei understand that this aligns with other features in RRC_INACTIVE.

Ericsson do not see that any new configuration is needed in Rel-18 over Rel-17 in this respect, but maybe some offline time is needed.

Huawei understand that the new field in the NCE for Rel-18 is not in a container.

Intel agree with the proposal.

Proposal5: Add the RRC fields for R18 enhancements for positioning SRS transmission in RRC_INACITVE in R18 in the internode message HandoverPreparationInformation

Discussion:

Ericsson think RAN3 are also discussing this and have a well-defined procedure for the context transfer; they do not see why there should be RAN2 impact.  They also note that the handover preparation is not used in RRC_INACTIVE.

Huawei think it is analogous to other contents of the INMs: RAN3 does not have visibility to our enhancements.  However, they agree that there is related RAN3 discussion.

ZTE think we left this to RAN3 last meeting and we should not add something in RAN2 at this stage.

Intel think we could take the issue as homework.  Huawei think it would be safest to add the field now and we can dummify later if needed.  Ericsson think the extension marker will be available.

Samsung think we could come back at the end of the meeting.

Agreements:

The application of the pre-configured positioning SRS is not dependent on the pre-configuration set to "setup".  Details of implementation to be discussed in rapporteur CR (no ASN.1 impact).

H920 is set to Agreed [rapporteur to confirm if a change in the RIL is needed].

Add the RRC fields for R18 enhancements for positioning SRS transmission in RRC_INACTIVE in R18 into a container.  [Chair’s note: This agreement is superseded by the agreements under R2-2405875.]

[AT126][407][POS] Rel-18 SRSp enhancement fields in INMs (Huawei)

Scope: Monitor RAN3 progress related to P5 of R2-2404764 and converge company views where possible.

Intended outcome: Report to CB session in R2-2405875

Deadline:  Thursday 2024-05-23 1000 JST

R2-2405875	Summary for [AT126][407][POS] Rel-18 SRSp enhancement fields in INMs (Huawei)	Huawei, HiSilicon	discussion	Rel-18	NR_pos_enh2

Proposal1: Confirm that RAN2 discusses the issue of whether to include R18 LPHAP SRS configuration in INM for RRC_INACTIVE in RAN2.

Porposal2: SRS configurations for LPHAP introduced in RRCRelease message in R18 is added to the internode message HandoverPreparationInformation.

Discussion:

Huawei indicate that RAN3 had an offline that concluded to leave the issue to RAN2, but it is not an official decision yet.

Nokia understand that the RAN3 offline produced a proposed WF, and they think it is a bit strange that RAN3 pass it back to RAN2.

Huawei think it is a RAN2 feature and should be transparent to RAN3; they understand that the functionality is not controversial and we just need to decide where to include it.

ZTE agree that the issue is in RAN2, but they see the spec impact in RAN3 because it is gNB-gNB signalling.

Ericsson think the container in handover is target to serving, and this goes in the opposite direction.  So they agree with ZTE that Xn may be better.

CATT agree that the issue should be fixed and there are RAN2 and RAN3 potential solutions.  They understand that there was a majority in the email discussion for the RAN2 solution, and RAN3 can move ahead once we agree.

Samsung also think that preconfiguration forwarding from the last to serving gNB is essential for completion; we left it to RAN3 last meeting, and RAN3 are now sending it back to us.  Samsung support using the handover preparation.

Nokia wonder if this is not exchanged between gNBs, if it would be OAM to ensure preconfiguration is consistent across gNBs.  They also wonder if RAN3 will accept RAN2 conclusion if we ask for an Xn-based solution.

Intel understand that the target gNB needs to learn the configuration from the source gNB, so an OAM solution would not fully address the issue; on the solution itself, they think similar behaviours historically have used the INMs.

Huawei indicate that the INM already exists, and since this is a Uu configuration they think it makes sense in the container.

Agreements:

Confirm that RAN2 discusses the issue of whether to include R18 LPHAP SRS configuration in INM for RRC_INACTIVE in RAN2.

SRS preconfigurations for LPHAP introduced in RRCRelease message in R18 is added to the internode message HandoverPreparationInformation.

ASN.1 impact

R2-2404614	Discussion on remaining corrections in RRC	ZTE Corporation	discussion	Rel-18	NR_pos_enh2

Proposal 1: RAN2 to rename srs-PosRRC-Inactive-v1800 to avoid conflict with srs-PosRRC-Inactive-r17.

Discussion:

Huawei understand that the Rel-18 field is an NCE.  Intel point out that the Rel-17 and Rel-18 behaviours are different, so they think we should change the name.

Proposal 2: Add ‘sl-TxPoolSelectedNormal, sl-TxPoolScheduling, sl-TxPoolExceptional’ in section 5.3.5.14 to cover the case that UE can use shared pool to transmit SL-PRS. Adopt the corresponding TP.

Proposal 3: Add in the field description in sl-PRS-ResourcesSharedSL-PRS-RP-r18 that ‘The UE can use the resource pool to transmit or receive SL-PRS only if this field is present’, in order to distinguish legacy SL data pool and shared pool. Adopt the corresponding TP.

Proposal 4: Deleting sl-PRS-starting-symbol from SL-ResourcePool. Adopt the corresponding TP.

Discussion:

Ericsson wonder if this is related to the parameter list.  ZTE indicate it is not.

Huawei think the change is reasonable.

Proposal 5: Add a new IE to include the RAN1 requested DCI formats. Take the example TP.

Proposal 6: Add a condition tag for SRS-PosRRC-AggBW-InactiveConfigList-r18 that ’This field is optionally present if SRS-PosRRC-InactiveConfig-r17 is included; otherwise it is not present’. Adopt the corresponding TP.

Proposal 7: Change the size of SRS-PosResourceSetLinkedForAggBWList-r18 from ‘SIZE(1..maxNrOfLinkedSRS-PosResourceSet-r18)’ to ‘SIZE(2..maxNrOfLinkedSRS-PosResourceSet-r18)’.

Agreements:

Delete sl-PRS-starting-symbol from SL-ResourcePool.

Add in the field description for SRS-PosRRC-AggBW-InactiveConfigList-r18 that the field is included only if SRS-PosRRC-InactiveConfig-r17 is configured.  Wording to be polished in CR implementation.

Change the size of SRS-PosResourceSetLinkedForAggBWList-r18 from ‘SIZE(1..maxNrOfLinkedSRS-PosResourceSet-r18)’ to ‘SIZE(2..maxNrOfLinkedSRS-PosResourceSet-r18)’.

Rename srs-PosRRC-Inactive-v1800 to srs-PosRRC-InactiveBW-AggFH-r18 avoid conflict with srs-PosRRC-Inactive-r17.

Add ‘sl-TxPoolSelectedNormal, sl-TxPoolScheduling, sl-TxPoolExceptional’ in section 5.3.5.14 to cover the case that UE can use shared pool to transmit SL-PRS.

Add in the field description in sl-PRS-ResourcesSharedSL-PRS-RP-r18 that ‘The UE can use the resource pool to transmit or receive SL-PRS only if this field is present’, in order to distinguish legacy SL data pool and shared pool.

R2-2405249	Addition of SL DCI Formats and SL PRS CBR measurement Results	Qualcomm Incorporated	discussion

- Noted

Other documents

R2-2404236	Activation of SRS when configured with validity area	CATT	discussion	Rel-18	NR_pos_enh2

Proposal 1: The activation of srs-PosRRC-InactiveValidityAreaPreConfigList should be triggered by RRC layer rather than upper layer, and the access category should also be determined by RRC layer because upper layer does not know whether the UE has valid SRS and whether the SRS type is srs-PosRRC-InactiveValidityAreaPreConfigList or srs-PosRRC-InactiveValidityAreaNonPreConfig or R17 SRS configuration. Adopt the TP in Annex with name P1.

Discussion:

Huawei think this is an internal UE behaviour issue.  Intel agree; they understand that the need for SRS is known by upper layers, but the upper layers do not know if there is an SRS configuration, but the inter-layer interaction should be able to do something without specification impact.

CATT think we should specify something that is possible for the activating layer to do.

Intel see that we only need to specify something if it is an interaction between nodes.

MediaTek agree with Intel that this is internal.  Nokia also agree and think we should not get too detailed.

Proposal 2: For both srs-PosRRC-InactiveValidityAreaPreConfigList and srs-PosRRC-InactiveValidityAreaNonPreConfig, the transmission of SP SRS should be activated by MAC CE as legacy. There is no protocol impact.

Discussion:

Qualcomm understand that RAN3 are not dependent on this decision and they are discussing only the aggregated activation, which we will discuss under the MAC topic.

Huawei have the same view as Qualcomm and think this discussion can be triggered from the MAC email discussion.  ZTE understand that the MAC CEs are different and this discussion is about activation of the legacy SP-SRS.

Huawei see a connection to aggregation and think the issue is the same for single and multiple carriers.  Qualcomm also see it as the same issue.

Xiaomi think we could confirm that Rel-18 SP-SRS (aggregated or not) is activated by the MAC CE.

Ericsson understand the point is whether the validity area SP-SRS can be activated by the legacy MAC CE.

Working assumption:

For both srs-PosRRC-InactiveValidityAreaPreConfigList and srs-PosRRC-InactiveValidityAreaNonPreConfig (non-aggregated cases), the transmission of SP SRS is activated by MAC CE as legacy.  Aggregated cases to be discussed under the MAC AI.

Proposal 3: For mechanisms of non-preconfigure SRS, if SP SRS is configured, the resume cause srs-PosConfigOrActivationReq can be used for RRC request activating the SP SRS. Adopt the TP in Annex with name P3.

Discussion:

ZTE think it would introduce a new purpose for sending Msg3 and it is not needed.

CATT understand that we have not discussed how to do the activation in this case.

Qualcomm wonder what the spec impact would be.  CATT think the RRC procedure needs to specify that the resume cause is set in the resume request for this case.

Intel think we do not need to specify this; we already have a way to notify the gNB that the UE has moved to a new area, and the gNB may then decide to activate.  CATT understand that for the SP case the UE needs to get network confirmation that it can use the SRS.

Qualcomm indicate that the RRC procedure does not distinguish the SRS type.

Huawei agree with others that there is no difference between the periodic and SP cases for this purpose; for SP, the gNB can send the MAC CE along with Msg4.

R2-2405103	Discussion on the activation of the semi-persistent SRS of preconfigured SRS	Xiaomi	discussion

### 7.2.6	MAC corrections

Impact to 38.321. A single CR with miscellaneous corrections is requested from the CR rapporteur; minor and editorial issues should be coordinated with the rapporteur and merged into the miscellaneous CR. Larger issues can be discussed based on contributions.

Including outcome of [Post125bis][401][POS] Aggregated SP-SRS activation/deactivation MAC CE (ZTE)

Rapporteur CR (endorsed after RAN2#125bis)

R2-2404762	Rapporteur MAC CR for R18 positioning	Huawei, HiSilicon	CR	Rel-18	38.321	18.1.0	1844	-	F	NR_pos_enh2

[Post126][411][POS] Rel-18 positioning MAC CR (Huawei)

Scope: Update the CR in R2-2404762 in line with decisions of this meeting.

Intended outcome: Agreed CR in R2-2405888

Deadline:  Short (for RP)

R2-2405888	Rapporteur MAC CR for R18 positioning	Huawei, HiSilicon	CR	Rel-18	38.321	18.1.0	1844	1	F	NR_pos_enh2-Core

=> Agreed

R2-2406098	Summary for [Post126][411][POS] Rel-18 positioning MAC CR (Huawei)	Huawei, HiSilicon	discussion	Rel-18	NR_pos_enh2-Core

=> Noted

Email discussion report

R2-2404615	Report of [Post125bis][401][POS] Aggregated SP-SRS activation deactivation MAC CE	ZTE Corporation	discussion	Rel-18	NR_pos_enh2

Discussion:

ZTE think we should simplify the discussion by focussing only on the Rel-18 BW aggregation feature.

Easy proposal:

Proposal 1: One MAC CE can activate/deactivate only one aggregated combination within 32 aggregated combinations. (5/5)

proposal 2: Introduce a 5 bit field for aggregated combination in the new MAC CE. (5/5)

Proposal 3: Include the aggregation indication in the new MAC CE, where the aggregation indication is to activate/deactivate specific carriers within each aggregated combination.(5/5)

Proposal 8: The current designed SRS BW aggregation MAC CE can be used for RRC_CONNECTED. Wait for RAN1’s reply LS to decide whether there should be a change for RRC_INACTIVE. (5/5)

Discussion:

ZTE indicate that the RAN1 LS calls for 16 combinations for RRC_INACTIVE, so they think the same MAC CE can be used.

Agreements:

One MAC CE can activate/deactivate only one aggregated combination within 32 aggregated combinations.

Introduce a 5 bit field for aggregated combination in the new MAC CE.

Include the aggregation indication in the new MAC CE, where the aggregation indication is to activate/deactivate specific carriers within each aggregated combination.

The currently designed SRS BW aggregation MAC CE can be used for RRC_CONNECTED and RRC_INACTIVE.

Need discussion:

Proposal 4: The aggregation indication in the MAC CE is 3 bits, in order to indicate the following conditions:

	1 carrier out of a 2-carrier-RRC-linkage;

	2 carriers out of a 2-carrier-RRC-linkage;

	1 carrier out of a 3-carrier-RRC-linkage;

	2 carriers out of a 3-carrier-RRC-linkage;

	3 carriers out of a 3-carrier-RRC-linkage.

Discussion:

Qualcomm agree with the proposal but wonder from the RRC configuration how the UE will know which configuration indication corresponds to which carrier.  ZTE understand that in RRC_CONNECTED, each combination contains the serving cell ID, and in RRC_INACTIVE it contains a carrier parameter. Qualcomm think this means the UE needs to sort the list of combinations to know which entry in the list corresponds to which carrier.

Huawei wonder how to downselect one carrier out of three carriers with 3 bits.  ZTE think it is just a bitmap.  Qualcomm think this is fine but wonder what defines which carrier in the combination is bit 1/bit 2/bit 3.

Qualcomm think the structure can indicate a carrier unambiguously if there is a nested sequence in RRC.  ZTE think there is an order already; the first 5 bits indicate the combination and the 3 bits correspond to the carriers that were indicated for that combination.

Ericsson indicate that there is a sequence of up to three carriers in the RRC signalling.  ZTE think the sequence of bits in the MAC CE can align with the order of this sequence in the RRC.  Ericsson think there is some alignment work needed in RRC.

Agreement:

The aggregation indication in the MAC CE is 3 bits, in order to indicate the following conditions:

1 carrier out of a 2-carrier-RRC-linkage;

2 carriers out of a 2-carrier-RRC-linkage;

1 carrier out of a 3-carrier-RRC-linkage;

2 carriers out of a 3-carrier-RRC-linkage;

3 carriers out of a 3-carrier-RRC-linkage.

Aggregation structure will be reviewed in the RRC CR to make sure the three bits can map to a sequence of carriers in the combination as signalled in the RRC.

Proposal 5: The new MAC CE does not contain a A/D field, a 3 bit bitmap can indicate the activation or deactivation of the corresponding carriers, i.e., using ‘1’ as activation or ‘0’ as deactivation.

Discussion:

Qualcomm wonder if it works, in terms of a bit set to 0 meaning that the UE has to deactivate a carrier that may not have been previously activated.  ZTE think this should be OK, but we may need to check the language.

Agreement:

The new MAC CE does not contain a A/D field separate from the 3-bit activation indication; the bitmap indicates the activation or deactivation of the corresponding carriers, i.e., using ‘1’ as activation or ‘0’ as deactivation.  Deactivation is idempotent, i.e., deactivating an already deactivated carrier leaves it still in deactivated state.

Proposal 6: RAN2 to adopt up to 16 Spatial Relation for Resource IDi fields in the new MAC CE.

Discussion:

Huawei think the proposal is fine, but the more important issue is the related RRC configuration; there can be 16 resources within a resource set, and it might be easier to have the RRC handle the aggregation.

ZTE indicate this is analogous to PRS aggregation.  They think the issue raised by Huawei is network implementation; the network will always put the aggregated resources in an order, and they do not see any spec impact to manage it.

Huawei think that there is a condition in the RAN1 spec on which resources can be aggregated, and there should be guidance in the RRC spec that the network does not aggregate resources that violate this condition.  Huawei and Qualcomm understand that the UE should not need to check the condition.

ZTE think even in aggregated resource sets there can be non-aggregated resources, and there is no resource-level linkage indicated in RRC, so the UE needs to determine which resources can be aggregated.  Huawei find this model unnatural and it is not clear how the spatial relation should be handled.

Ericsson want to understand if Huawei are proposing a new spatial relation for the aggregated carriers, or if it is field description impact only.  Huawei think we can reuse the Rel-16 definition of the spatial relation, but the mapping from the spatial relation to the resource is not clear.

ZTE understand we could constrain the network to aggregate the resources in order, or we could reuse the Rel-17 MAC CE exactly and the UE knows the aggregated resource ID naturally.  Qualcomm understand the UE should not be required to check this, and they think this is why the aggregation is on resource set level.

Huawei think the UE should not need to determine anything when it transmits SRS; it should not need to check if the aggregated resources satisfy the RAN1 condition but just follow the network configuration.  ZTE agree with this as stated.

Ericsson would like to see a TP from Huawei for the field description, and they think we can continue fine-tuning the field description after the freeze since there is no ASN.1 impact.

CATT think in the legacy MAC CE, the gNB can change the spatial relation, but here the RRC configures the spatial relation to the UE on resource set level.  They think if the gNB can change the spatial relation for a resource, the UE should be able to interpret it and change the aggregation status.  ZTE think for this reason, the UE will have to determine which resources are aggregated based on the current configuration, and they think this was RAN1 intention.

Proposal 7: In the IE description of Spatial Relation for Resource IDi field, clarify that the SRS resource ID of the spatial relation comes from the first linked SRS resource set in this MAC CE. Other Spatial Relation for Resource IDi field design should remain as legacy.

Discussion:

ZTE intend to indicate the resource ID in the spatial relation field.

Qualcomm checked the RAN1 spec and think it is consistent with ZTE’s view.  Huawei think the network configuration should ensure that the requirement cannot be violated.

Ericsson think this does not affect the design of the MAC CE.

Huawei interpret the RAN1 requirement as a constraint on the network.

Agreements:

Adopt up to 16 Spatial Relation for Resource IDi fields in the new MAC CE.

In the IE description of Spatial Relation for Resource IDi field, clarify that the SRS resource ID of the spatial relation comes from the first linked SRS resource set in this MAC CE. Other Spatial Relation for Resource IDi field design should remain as legacy.

The UE is expected to determine which resources can be aggregated based on the RRC configuration and the criteria defined by RAN1.  This decision can be revisited based on contributions to determine if the expected UE behaviour is possible to simplify.

Other documents

R2-2404741	Discussion on SL positioning MAC open issues	Xiaomi	discussion	Rel-18	NR_pos_enh2

Proposal 1	RAN2 to adopt the TP for LCP for SL-PRS (TP is provided in the document).

Proposal 2	To determine whether the SL grant is applicable for the transmission of the pending SL-PRS, the following condition is considered(TP is provided in the document):

- whether the bandwidth meets the SL-PRS requirement;

- whether the remaining delay budget of the SL-PRS can be met.

Proposal 3	For mode 2 resource allocation, the selected resource pool should meet the bandwidth requirement of SL-PRS(TP is provided in the document).

R2-2404767	Discussion on the remaining isues for MAC for R18 POS	Huawei, HiSilicon	discussion	Rel-18	NR_pos_enh2

Proposal1: Configure the same number of SRS resources for the aggregated SRS resource sets from different carriers and indicate the same number of spatial relations in the SP posSRS activation/deactivation MAC CE.

Proposal2: Confirm that the current spec already supports R18 RSRP-based TA validation for SRS positioning validity area and SRS bandwidth aggregation working together in RRC_INACTIVE state. No spec impacts are needed

Discussion:

Huawei think P2 can be confirmed.

Agreement:

Confirm that the current spec already supports R18 RSRP-based TA validation for SRS positioning validity area and SRS bandwidth aggregation working together in RRC_INACTIVE state. No spec impacts are needed\.

R2-2405260	Addressing MAC open issues	Ericsson	discussion	Rel-18

Proposal 1	5-bits are used for bandwidth request for SL-PRS aperidoc transmission.

Proposal 2	Allow peer UE to provide the priority, but if peer UE does not provide then only UE upper layers may select one.

Proposal 3	The priority and SL-PRS delay is determined based upon positioning QoS and is provided by NW node LMF for LMF involved cases.

Proposal 4	Add one octet with 3 Reserved bits and 5 bits to point out which out of 32 combinations for bandwidth aggregation is to be activated in the DL MAC CE for SRS carrier aggregation.

R2-2405267	Miscellaneous MAC issues 	Nokia	discussion	Rel-18

Proposal 1: If SL PRS transmission priority is not determined by own upper layers or not provided to them by explicit SLPP signaling, the lowest priority is selected by default to prevent accidental / malicious priority inflation and ensure worst-case compliance with priorities of other session members.

Proposal 2: RAN2 to consider the adoption of higher default priority in case SL PRS transmissions with narrower bandwidth, eg less than 50% of the resource pool bandwidth.

R2-2405420	Corrections for SL-PRS transmission and reception	ASUSTeK	discussion	Rel-18	NR_pos_enh2

- Noted

Proposal 1:  Confirm that SL-PRS in shared resource pool should be always multiplexed with SL MAC PDU (PSSCH).

Proposal 2:  (Option 1) In shared resource pool, Tx UE continues to transmit SL-PRS with the ACKed SL MAC PDU using the provided multiple resources until the maximum number of SL-PRS transmissions.

Discussion:

Huawei indicate that we agreed on SL-PRS transmission without MAC PDU for the case that the UE would otherwise have to wait for data forever.

ZTE understand that it should be possible to transmit SL-PRS without data bits and think P2 is correct.

ASUSTeK indicate that their understanding is that in shared pool the SL-PRS should always be multiplexed with data bits/padding bits.

vivo think this discussion is about P1.

OPPO think P2 is an optimization, and in the case that a MAC PDU has been acked, they think the retransmission should be avoided.

Huawei are not sure of the intention for P2, but they think the HARQ buffer could be flushed when the PSSCH is acked; however, they do not see that this requires a change from the current spec.  They understand that the 2-stage SCI would still be transmitted but without the data.

vivo understand the intention of P2 is that if the peer UE of the Tx UE requested SL-PRS multiple times, and if there is no SL MAC PDU pending, it provides a way to transmit the SL-PRS.

Proposal 3:  In shared resource pool, when Tx UE transmits SL-PRS, Rx UE should keep active for receiving the SL-PRS.

Proposal 4:  If the higher layer triggers SL-PRS transmission of the peer UE, UE should set the SL-PRS request to request.

Proposal 5:  Following RAN1 design, UE shall set the SL-PRS request to “request” in any PSSCH / SL-PRS transmission, if requested from the higher layer, regardless of initial transmission or re-transmission.

Discussion:

Huawei think the proposal confuses the triggers from higher layer and from SCI.

Agreement:

“to the peer UE” to be changed to “of the peer UE” in accordance with the TP for P4 from R2-2405420.  To be implemented in rapporteur CR.

Proposal 6:  (Option 1) Add a NOTE to clarify that the selected number of HARQ retransmissions is also applied for SL-PRS retransmissions, if available.

MAC CE design (overlap with email discussion)

R2-2405504	MAC CE for activation/deactivation of aggregated SP SRS for positioning	Samsung	discussion	Rel-18	NR_pos_enh2	Late

Withdrawn/Not available

R2-2404714	Miscellaneous MAC issues	Nokia France	discussion	Rel-18	Withdrawn

### 7.2.7	UE capabilities

Impact to 38.306 and capability-related impact to 38.331. A single CR with miscellaneous corrections is requested from the CR rapporteur; minor and editorial issues should be coordinated with the rapporteur and merged into the miscellaneous CR. Larger issues can be discussed based on contributions.

Rapporteur CRs (endorsed after RAN2#125bis)

R2-2404623	Miscellaneous corrections for Rel-18 positioning UE capabilities	Xiaomi	CR	Rel-18	38.331	18.1.0	4772	1	F	NR_pos_enh2-Core	R2-2403971

=> Revised in R2-2405889

R2-2404624	Miscellaneous corrections for Rel-18 positioning UE capabilities	Xiaomi	CR	Rel-18	38.306	18.1.0	1090	1	F	NR_pos_enh2-Core	R2-2403972

=> Revised in R2-2405890

Other documents

R2-2404768	Discussion on the remaining issues for UE capability	Huawei, HiSilicon	discussion	Rel-18	NR_pos_enh2

Proposal 1: Delete the NOTE3 in the feature posSRS-BWA-IndependentCA-RRC-Connected-r18 and the NOTE4 in the feature posSRS-BWA-RRC-Connected-r18 in TS 38.306.

Proposal 2: Delete the signalling for the component in a FG without candidate values in TS 38.331.

Proposal 3: Delete the “optional” for the component in a FG with candidate values in TS 38.331.

Discussion:

Huawei think P2/P3 may need more discussion from RAN1 side.

Lenovo think the NOTEs in P1 are good to have.  Intel think the NOTEs are from the feature list and should be kept.  On P2, Intel understand RAN1 will update and ask us to delete this signalling.

Huawei indicate that on P1, the legacy NOTE says the UE only reports a certain number of bands, but this was for Rel-16 SRS, and it is not applicable here but should be in LPP.

Lenovo think P3 is not right because of the per-band signalling structure; FR2 should be able to omit an FR1-only capability.  Huawei indicate updates are expected.

Agreement:

Delete the NOTE3 in the feature posSRS-BWA-IndependentCA-RRC-Connected-r18 and the NOTE4 in the feature posSRS-BWA-RRC-Connected-r18 in TS 38.306.

R2-2404519	Corrections on NR positioning capability signaling	Lenovo	discussion	Rel-18	NR_pos_enh2

Proposal 1: In the CRs to RRC and SLPP replace the SEQUENCE type by CHOICE type for the components 1, 2, 3 of FG 41-1-1. Furthermore, in RRC CR remove OPTIONAL for component 4.

Proposal 2: In LPP CR revise the field descriptions of reducedNumOfSampleInMeasurementWithPRS-BWA-RRC-Connected and reducedNumOfSampleInMeasurementWithPRS-BWA-RRC-IdleAndInactive to make it condition to the support of component 1 of R1 FG 41-4-1.

Proposal 3: In LPP CR fix the editorial issues in the field descriptions of posSRS-Preconfigured-RRC-InactiveInitialUL-BWP, posSRS-Preconfigured-RRC-InactiveOutsideInitialUL-BWP, posSRS-ValidityAreaRRC-InactiveInitialUL-BWP, posSRS-ValidityAreaRRC-InactiveOutsideInitialUL-BWP and posSRS-TxFH-RRC-Inactive.

Discussion:

Xiaomi think the proposals can be taken into account in the rapporteur CRs.

Agreements:

In the CRs to RRC and SLPP replace the SEQUENCE type by CHOICE type for the components 1, 2, 3 of FG 41-1-1. Furthermore, in RRC CR remove OPTIONAL for component 4.

In LPP CR revise the field descriptions of reducedNumOfSampleInMeasurementWithPRS-BWA-RRC-Connected and reducedNumOfSampleInMeasurementWithPRS-BWA-RRC-IdleAndInactive to make it condition to the support of component 1 of R1 FG 41-4-1.

In LPP CR fix the editorial issues in the field descriptions of posSRS-Preconfigured-RRC-InactiveInitialUL-BWP, posSRS-Preconfigured-RRC-InactiveOutsideInitialUL-BWP, posSRS-ValidityAreaRRC-InactiveInitialUL-BWP, posSRS-ValidityAreaRRC-InactiveOutsideInitialUL-BWP and posSRS-TxFH-RRC-Inactive.

### 7.2.8	Corrections to other specifications

Impact to any specifications not identified above.

## 7.3	Network energy savings for NR

(Netw_Energy_NR -Core; leading WG: RAN1; REL-18; WID: RP-223540)

Time budget: 0 TU

Tdoc Limitation: 2 tdocs

### 7.3.1	Organizational

LS, workplan, email discussion etc

Spec rapporteurs are expected to submit additional contribution on open issues to conclude WI by December

R2-2405135	Miscellaneous MAC corrections for network energy savings	InterDigital	CR	Rel-18	38.321	18.1.0	1855	-	F	Netw_Energy_NR-Core

=>	The CR is agreed

R2-2405617	NES WI RIL list	Huawei, HiSilicon	report	Rel-18	Netw_Energy_NR-Core

[E305] There is currently no way to release referenceCell-r18. This could usually be solved by just changing the need code to “Need R”, but absence of this field implies in the default behaviour defined by RAN4. Hence, the easier could be to wrap this field on a SetupRelease structure.

=>	The RIL305 is rejected

=>	Noted

R2-2405618	Network energy savings for NR miscellaneous RRC CR	Huawei, HiSilicon	CR	Rel-18	38.331	18.1.0	4692	2	F	Netw_Energy_NR-Core	R2-2403998

=>	The CR is endorsed and used as a baseline.

=> Agreed

[POST126][009][NES] RRC CR (Huawei)

Intended outcome: Agree to RRC CR with any additional changes needed and updated RIL List

Deadline:  Short

=> Agreed in R2-2405618

R2-2406088	NES WI RIL list	Huawei, HiSilicon	discussion	Rel-18	Netw_Energy_NR-Core

=> Noted

R2-2405659	Discussion on UE capabilities for inter-band SSB-less SCell operation	vivo, Ericsson, Huawei, HiSilicon, Apple, Xiaomi, CATT, ZTE Corporation, Sanechips	discussion	Rel-18

Observation 1. Due to the UE capability guideline on not to use a signaled band combination to derive supported features for higher order band combinations, the NW cannot configure CA of a higher-order BC while only configuring inter-band SSB-less operation of a lower-order BC when the UE only reports ‘supported’ for this lower-order BC .

Agreements

1. Instead of reporting ‘supported’ for a band within the BC to indicate this band can be configured as the reference band for all other bands within the BC, the UE optionally reports ‘referenceBand’ or ‘scellWithoutSSB’ for a band to indicate the support of inter-band SSB-less SCell operation.

2. The band indicated as ‘referenceBand’ can be configured as the reference band for all other band(s) indicated as ‘scellWithoutSSB’.

3. If the field scellWithoutSSB-InterBandCA-r18 is absent for a band, this band is not involved in the inter-band SSB-less SCell operation.

4. If inter-band SSB-less SCell operation is supported between two bands, it is understood that there is no direction between the two bands, which means that the network can configure either band as the reference band and the other band as the SSB-less band.

R2-2405304	UE Capability for Inter-band SSB-less CA	Qualcomm Incorporated	discussion	Rel-18

Observation 1: RAN4 wording allows indicating multiple bands as “reference” within the same interband SSB-less CA band grouping, i.e., no restriction that one band must be indicated as “reference” with other bands indicated as SSB-less.

Observation 2: Restricting “reference” value indication to a single band within inter-band CA grouping makes capability signalling impractical since UE must report up to N feature sets for N Band Combinations to indicate whether each band can be used as reference for other SSB-less bands.

Proposal 1:  If RAN2 agrees on using the two values {reference, SSB-less} to indicate capability within Interband SSB-less CA band grouping, there are no restrictions on how many bands the UE can indicate as ‘reference’.

Proposal 2: RAN2 capability framework need not support multiple independent SSB-less band CA groupings within the same BC. This can be confirmed with RAN4 via an LS.

-	Huawei doesn’t think this is a new concept and it is not that complicated.

-	CMCC supports Vivo’s proposal

-	MEdiatek doesn’t see a requirement for groups.  Mediatek is also concerned about complexity.

=>	Noted

After CB

Compromise solution.  CR to be agreed by email

If scheme1 (to be named) is indicated, the band indicated as ‘scheme1’ can be configured as either the reference band or SSB-less band.

If scheme2 (to be named) is indicated, the band indicated as ‘referenceBand1’ can be configured as the reference band for all other band(s) indicated as ‘scellWithoutSSB1’, and  the band indicated as ‘referenceBand2’ can be configured as the reference band for all other band(s) indicated as ‘scellWithoutSSB2’.

If the field scellWithoutSSB-InterBandCA-r18 is absent for a band, this band is not involved in the inter-band SSB-less SCell operation.

In a band combination, only scheme1 or scheme2 is indicated.

R2-2405949	Summary of offline discussion for UE capabilities on supporting multiple inter-band SSB-less SCell groups and Proposed Way-forward	vivo	discussion	Rel-18	Netw_Energy_NR-Core

=>	Not treated

[POST126][010][NES] UE capabilities (Vivo)

Intended outcome: endorse CRs

Deadline:  one week email discussion

R2-2406039	Correction on NES UE capabilities to 38306	vivo	draftCR	Rel-18	38.306	18.1.0	-	Netw_Energy_NR-Core

=> Endorsed and to be merged in the mega CR.

R2-2406040	Correction on NES UE capabilities to 38331	vivo	draftCR	Rel-18	38.331	18.1.0	-	Netw_Energy_NR-Core

=> Endorsed and to be merged in the mega CR.

[AT126][011][NES] 38.300  (Ericsson)

Intended outcome: agree to 38.300 by email

Deadline:  05-24-24

R2-2405959	Miscellaneous stage-2 corrections for network energy savings	Ericsson	CR	Rel-18	38.300	18.1.0	0870	-	F	Netw_Energy_NR-Core

=>	the CR is agreed

### 7.3.2	User Plane

Coexistence of Cell DTX/DRX and LTM

R2-2404946	Coexistence of Cell DTXDRX and RACH-less LTM	CATT	discussion	Rel-18	Netw_Energy_NR-Core

Proposal 1: The following two options should be considered, and for simplicity, option 1 should be preferred:

=>	Noted

R2-2405616	Discussion on the coexistence of cell DTX/DRX and RACH-less LTM	Huawei, HiSilicon, InterDigital	discussion	Rel-18	Netw_Energy_NR-Core

Proposal 1: The coexistence of cell DTX/DRX and LTM can be handled by the current specifications without spec changes. No optimisation of cell DTX/DRX and RACH-less LTM is needed.

=>	Noted

R2-2404632	Remaining issue on coexistence between Cell DTX/DRX and LTM	Apple	discussion	Rel-18	Netw_Energy_NR-Core

Proposal 1: RAN2 confirm that the UE may apply Cell DTX/DRX during on-going RACH-less LTM cell switch.

Proposal 2: Same as conclusion of RACH-less LTM and RACH-less HO for TN network, the UE regards the on-going LTM cell switch procedure as active time of Cell DTX/DRX.

Proposal 3: RAN2 agree the TP in Appendix to capture that active time of Cell DTX/DRX includes the on-going RACH-less LTM cell switch.

=>	Noted

R2-2404364	Co-existence of LTM and NES	Ericsson	discussion

Proposal 1	In order to support the co-existence of LTM and cell DTX/DRX, the active time of a UE for Serving cells in a DTX group includes the time while there is an ongoing RACH-less LTM cell switch.

Proposal 2	If co-existence of LTM and cell DTX/DRX is not supported, this restriction should be captured in 38.300.

=>	Noted

R2-2405586	Coexistence of Cell DTXDRX and RACH-less LTM	Sharp	discussion

Observation 1: There is NO “active time” definition for Cell DTX/DRX, and cell DTX/DRX “active period” doesn’t include exceptional cases.

Proposal 1: Support coexistence of Cell DTX/DRX and RACH-less LTM cell switch.

Proposal 2: If there is an ongoing RACH-less LTM cell switch or RACH-less handover, UE monitors PDCCH regardless of Cell DTX.

Proposal 3: If there is an ongoing RACH-less LTM cell switch or RACH-less handover, UE can deliver configured uplink grant regardless of Cell DRX.

=>	Noted

Agreements

Understanding is that an LTM target is not a serving cell and hence it will not apply CELL DTX/DRX until the switch is completed. Nothing is optimized or specified for RACHless

Clarifications

R2-2404945	Discussion on the unaligned SFN for Cell DTXDRX in MAC	CATT	discussion	Rel-18	Netw_Energy_NR-Core

Proposal: In case of unaligned SFN across carriers in a cell group, the SFN of the SpCell is used to calculate the Cell DTX/DRX duration.

-	LG doesn’t think we need that change as each serving cell would look for it’s own time, as DTX is executed per serving cell.  Apple doesn’t think this is even in scope.

-	Nokia wonders if the UE knows the SFN of the SCells. LG thinks that the UE knows the SFN of the serving cell. Nokia explains that the SFN is in the MIB.   LG and Lenovo explains that the UE has to read the MIB for synchornization purposes.  Samsung thinks that there is no requirement to read the SFN in the MIB

-	Xiaomi aks if we can do CELL DTX/DRX with SSBless

=>	Noted

Stage-2

Not treated

R2-2405590	CellDTRX-RNTI description in TS 38.300	Samsung	discussion	Rel-18	Netw_Energy_NR-Core

Proposal. A description of CellDTRX-RNTI is added in clause 8 of TS 38.300. The corresponding TP is added in the Annex.

R2-2404830	Coexistence of Cell DTXDRX and RACH-less LTM	Rakuten Mobile, Inc	discussion	Rel-18

R2-2405045	Coexistence of Cell DTX/DRX and RACH-less LTM	OPPO	discussion	Rel-18	Netw_Energy_NR

R2-2405288	Discussion on coexistence of Cell DTX/DRX and RACH-less LTM	Fujitsu	discussion	Rel-18	Netw_Energy_NR_enh-Core

R2-2405292	Coexistence of Cell DTXDRX mechanism and RACH-less LTM	LG Electronics Inc.	discussion	Rel-18	Netw_Energy_NR-Core

R2-2405488	Coexistence issue for NES and RACH-less	Xiaomi	discussion	Rel-18	Netw_Energy_NR-Core	R2-2402846

### 7.3.3	Control Plane corrections

TRS Meas Capability for SSB-less SCell Activation:

R2-2404664	Discussion on remaining issue on UE capability of aperiodicCSI-RS-FastScellActivation	Apple	discussion	Rel-18	Netw_Energy_NR-Core

Proposal 1: On UE capability aperiodicCSI-RS-FastScellActivation-r17, it is applied to intra-band contiguous SSB-less SCell activation. RAN2 wait RAN4 conclusion on whether it is also applied to intra-band non-contiguous SSB-less SCell activation.

=>	Noted

R2-2405046	Discussion on the TRS meas capability for intra-band SSB-less CA 	OPPO	discussion	Rel-18	Netw_Energy_NR

Proposal 1	Not consider the case of TRS-based intra-band non-contiguous SSB-less SCell activation in Rel-18 NES.

=>	Noted

Agreement

=>	RAN2 will not consider the case of TRS-based intra-band contiguous and non-contiguous SSB-less SCell activation in Rel-18 NES

Clarification on SSB-less Configuration:

R2-2404827	Clarification on SSB-less configuration for R18 UE	Xiaomi, China Unicom, CMCC, CATT, Lenovo, Ericsson, Sharp, Spreadtrum, Fujitsu, Nokia, Nokia Shanghai Bell, Samsung, NEC, China Telecom, LG electronics Inc	discussion	Rel-18

=>	Noted

Understanding 1.1: The UE will try to search for intra-band activated serving cell as reference cell as R15 first, if no intra-band serving cell found, then UE will try to search for inter-band default cell as reference cell.

Understanding 1.2: The UE will try to search for inter-band default cell as reference cell first, if no default cell found, then UE will try to search for intra-band activated serving cell as reference cell as R15.

Understanding 2: The UE will try to search for intra-band activated serving cell as reference cell as R15.

Understanding 3: The UE will try to search for inter-band default cell as reference cell

Understanding 4: It is up to UE implementation if there is no performance difference between option 1.1 and option 1.2.

Understanding 5: One indication is configured in FrequencyInfoDL to indicate the UE will follow R15 behavior or R18 default cell behavior (e.g., one explicit indication or wrap referenceCell field in a setupRelease structure, as suggested in E305).

Understanding 6: It is up to network to ensure, there is no such configuration confusion for UE.

-	Vivo and Qualcomm think that RAN4 should discuss this.  Xiaomi thinks that this is a configuration issue and not RAN4

-	Vivo and Samsung think understanding 2 is correct.

-	Fujitsu thinks that this can be handled by network implementation, understanding 6.  LG agrees.

=>	Noted

Stage-2

R2-2405670	Clarification on public safety related service handling in 38.300	ZTE Corporation, Sanechips	discussion	Rel-18	Netw_Energy_NR-Core

=>	Agree to remove “(e.g. MPS or MCS)” in 38.300 for the public safety related service handling in NES.

=>	Noted

R2-2404365	Clarification on cellDTRX-RNTI	Ericsson	discussion

=>	Agree to intention, but use Samsungs wording at the end and can clarify further

=>	noted

## 7.4	Further NR mobility enhancements

(NR_Mob_enh2-Core; leading WG: RAN2; REL-18; WID:RP-233970)

Time budget: 0 TU)

Tdoc Limitation: 4 tdocs (if you want to input beyond the tdoc limitation, please cooperate with CR Rapporteurs).

### 7.4.1	Organizational

Including LSs.

Including outcome of [Post125bis][519][R18 Mob] Power Control Parameters after LTM cell switch (Fujitsu), with Scope: Collect RAN2 input in order to determine impacts and make decision as requested in R1 LS R1-2403683.

R2-2404126	Reply LS on n-TimingAdvanceOffset for PDCCH order RACH (R4-2406444; contact: Apple)	RAN4	LS in	Rel-18	NR_Mob_enh2-Core	To:RAN2	Cc:RAN1

-	already take into acct (in UE cap)

-	Ericsson think we need a piece of text to handle the absence in RRC.

Noted, consider also RRC impact acc to comment.

R2-2404115	LS on the identification of the power control parameters after LTM cell switch (R1-2403683; contact: Fujitsu)	RAN1	LS in	Rel-18	NR_Mob_enh2-Core	To:RAN

Noted

R2-2404619	Summary of [Post125bis][519][R18 Mob] Power Control Parameters after LTM cell switch (Fujitsu)	Fujitsu	discussion	Rel-18	NR_Mob_enh2-Core	Late

-	MTK think this is related to TCI state config so discuss together with that.

-	HW think the UE doesn’t need to read this before the cell switch, so why is this needed. CATT agrees. OPPO agrees as well think there is no additional delay.

-	Ericsson and ZTE think we should go with Approach 2

-	Ericsson think that the UE will not apply UE power control parameters related to TCI state in the candidate configuration.

[AT126][501][R18MobE] Power Control Parameters after LTM cell switch (Fujitsu)

Scope: Sort out the usage of outer config vs candidate config, establish R2 view.

Intended outcome: Report

Deadline: CB, see schedule

R2-2405948	Report [AT126][501][R18MobE] Power Control Parameters after LTM cell switch	Fujitsu

DISCUSSION

-	Huawei would like to confirm the R1 agreement also and capture it: that TCI configurations under LTM TCI Info is a subset of configurations in candidate configuration.

-	Ericsson think this is not the case.

RAN2 assumes that TCI configuration under LTM TCI info could be different to TCI configurations in candidate configuration.

RAN2 to adopt Approach 2, i.e., introduce new RRC parameters under LTM-TCI-Info.

R2-2404620	[Draft] Reply LS on the identification of the power control parameters after LTM cell switch	Fujitsu	LS out	Rel-18	NR_Mob_enh2-Core	To:RAN1	Late

-	Fujitsu report that RAN1 doesn’t need a reply LS

noted

### 7.4.2	Stage-2 Corrections

Corrections to 38300 (MTK) and 37340 (ZTE) and stage-2 centric issues (including tdocs on stage-2 centric issue that also impact other TS). Preferably work with CR Rapporteurs for Stage-2 corrections instead of separate CRs.

Agreed in-principle

R2-2404607	Stage-2 corrections on LTM	Mediatek Inc.	CR	Rel-18	38.300	18.1.0	0842	2	F	NR_Mob_enh2-Core	R2-2404009

agreed (but may be revised, see [511] below)

=> Revised in R2-2406084

R2-2405058	Miscellaneous corrections for mobility enhancements	ZTE Corporation	CR	Rel-18	37.340	18.1.0	0392	-	F	NR_Mob_enh2-Core

Same as CR 0391 which was agreed in-principle?

Endorsed, further update expected to capture impact from current meeting, short email discussion.

=> Revised in R2-2406102

[Post126][510][R18MobE] 37340 CR (ZTE)

Scope: Based on endorsed CR in R2-2405058, determine and add agreeable impact of / capture of agreements from current meeting.

Intended outcome: Agreed CR

Deadline: Short

R2-2406102	Miscellaneous corrections for mobility enhancements	ZTE Corporation	CR	Rel-18	37.340	18.1.0	0392	1	F	NR_Mob_enh2-Core

=> Agreed

PDCP impact

R2-2404781	Discussion on PDCP SN gap at LTM fast recovery	NTT DOCOMO, INC.	discussion	Rel-18

Moved from 7.4.3.1

-	Intel think the network knows this and can then handle this gap without impacts. Ericsson agrees. MTK too. OPPO too.

-	Docomo think we should have some note, e.g. in 331 T304 expiry.

-	HW and Nokia think that network usually handles cases ..

-	Nokia think that if we have a note it could be in stage-2.

R2 assumes that network implementation can handle this.

Will have a note, where and what discussed in CR discussions after meeting.

[Post126][511][R18MobE] PDCP SN gap at LTM fast recovery (MediaTek)

Scope: Converge on the note and where to put it. If it is agreed to have the note in 38300, revise the agreed 38300 CR, if it is agreed to have the note in 38331 then add to RRC discussion, if other TS without ongoing CR then make new CR.

Intended outcome: Agreed CR (if applicable)

Deadline: Short

R2-2406084	Stage-2 corrections on LTM	Mediatek Inc.	CR	Rel-18	38.300	18.1.0	0842	3	F	NR_Mob_enh2-Core

=> Agreed

### 7.4.3	RRC Corrections

RRC corrections and Control Plane Centric Issues (including tdocs on control plane centric issue that also impact other TS). Including ASN.1 review issues and their resolutions. For RRC issues, please input to ASN.1 review rather than just providing a tdoc. Including outcome of [Post125bis][510][R18Mob] RRC CR (Ericsson),

R2-2404967	Miscellaneous corrections on further mobility enhancements in NR	Ericsson	CR	Rel-18	38.331	18.1.0	4705	1	F	NR_Mob_enh2-Core	R2-240317

-	Outcome of long email discussion + solution for L063

-	Huawei observe also some changes in 5.3.5.3, which we discussed before and this should not be done. Ericsson confirms that this is L063.

-	LGE explains that the intention is to prevent CHO recovery after failed LTM recovery. Lenovo think that the opposite case is also an issue.

-	ZTE think nothing is needed

Endorsed, except for the newly added part in 5.3.5.3 to cover L063

=> Revised in R2-2406086

[Post126][512][R18MobE] 38331 (Ericsson)

Scope: Reflect agreements and progress in the CR, resolve remaining points as discussed at the meeting.

Intended outcome: Agreed CR

Deadline: Short

R2-2406086	Miscellaneous corrections on further mobility enhancements in NR	Ericsson	CR	Rel-18	38.331	18.1.0	4705	2	F	NR_Mob_enh2-Core

=> Agreed

R2-2404970	RILs conclusions for feMob	Ericsson	discussion	Rel-18	NR_Mob_enh2-Core

New propAgree L063

noted

#### 7.4.3.1	L1L2 Triggered Mobility

TCI state related configurations

R2-2405216	[H094][H095] Issues on LTM-TCI-Info	Huawei, HiSilicon	discussion	Rel-18	NR_Mob_enh2-Core

P1

-	LG wonder if this is used for TRS resource tracking.

-	Can consider offline whether additional explanation is needed.

P5

-	Ericsson think that the IEs are in different places, think we don’t need to optimize. HW think there are inconsistencies that doesn’t make sense.

-	Can consider offline whether it is worthwhile to go this way.

Capture that pathlossReferenceRS-Id and csi-RS-Index in candidate TCI states refer to items configured in LTM-TCI-Info.

R2-2404298	Candidate TCI states issues in LTM and LS from RAN1	MediaTek inc.	discussion	Rel-18	NR_Mob_enh2-Core

Noted

R2-2404771	Linkage between candidate TCI states and TCI states inside candidate cell configuration	Panasonic	discussion

Moved from 7.4.4

Noted

R2-2404300	LTM with 2TA co-existance	MediaTek inc.	discussion	Rel-18	NR_Mob_enh2-Core

-	HW think all info so already there in the outer config.

-	ZTE think indeed if all TCI states and all SSBs are included then this is possible.

-	Apple think we can skip this CATT agrees

UE-based TA measurement should not be configured if candidate cell is configured with Rel-18 MIMO 2TA

Other corrections

R2-2405595	Control plane centric issues for LTM	Langbo	discussion	Rel-18	38.331	NR_Mob_enh2-Core

-	Lenovo think T316 doesn’t need update.

-	Nokia, Ericsson think LTM is already covered by recofinguration cases ..

Consider offline whether clarifications are needed for timers table, T310, T312, for LTM Cell switch

R2-2404804	[B123] Coexistence of LTM recovery and CHO recovery	Lenovo	discussion	Rel-18

DISCUSSION B123 L063

-	Nokia think P2O1 can be considered only if very simple.

-	Samsung agrees with LG, think TP by Lenovo is small.

-	Apple also think we should consider what is logically correct to do, it is not so complex.

CHO recovery is not allowed after LTM recovery failure (TP considered offline)

R2-2405527	Discussion on measurement gaps for LTM in NR-DC	Samsung	discussion

Moved from 7.4.2

DISCUSSION

- 	ZTE think the L3 measurent gaps are sufficient.

- 	Ericsson think same samples are used for L1 measurements as for L3 measurements

-	Samsung think that no of cells can be different between L1 and L3 meas.

-	Xiaomi think 3 in the table may need disc, vivo think this is then a R4 discussion.

-	Apple think there could be an issue, and MN cannot fully understand SN situation.

-	Samsung expect that if we address this, it will impact Inter-node-signalling

-	Session Chair: can have time to check, until next meeting.

Postponed

R2-2405467	RLC entity handling for IAB/mIAB during LTM execution	Samsung	discussion	Rel-18	NR_Mob_enh2-Core

Add handling of BH RLC channels in IAB/mIAB during LTM cell switch execution.

R2-2404437	Remaining issues on s-Measure and recovery after RLF due to RLC retransmission	vivo	discussion	Rel-18	NR_Mob_enh2-Core

DISCUSSION

P1

-	HW not current behaviour. LG agree with HW, already agreed that L1 meas doesn’t apply S-meas. Ericsson think this might not be captured in the TS.

P3

-	HW agrees and wonder if this should not always be done.

-	OPPO agrees with this proposal

-	LG think this applies only at LTM recovery.

-	Ericsson think also at LTM cell switch. Huawei think that before cell switch you may be at max retx-1 so also at cell switch. QC agrees.

-	Samsung wonder if this only affects RRC or also RLC.

Capture that s-Measure criterion is not applicable to L1 measurements, if not already clear (disc in CR update discussion)

UE initialize RETX_COUNT when performing LTM fast recovery and LTM cell switch (for all RLC SDUs), when not triggered by other condition, discuss how to capture in CR discussion.

R2-2404968	[E400] Issues on INM signalling and power control parameters for LTM	Ericsson	discussion	Rel-18	NR_Mob_enh2-Core

R2-2405059	Discussion on remaining issues for LTM	ZTE Corporation	discussion	Rel-18	NR_Mob_enh2-Core

Both noted

DISCUSSION on the two tdocs above

-	HW and Ericsson think the ZTE and Ericsson solutions are the same.

-	OPPO wonder if this also applies to cond mobility. Ericsson think this is only for LTM.

In order to keep supporting subsequent LTM at the SN, a new indication is introduced in CG-Config message so SN can indicate to the MN if there are available LTM candidate cell configurations, if more LTM candidate cell configurations are needed, or if the LTM candidate cell configuration indicated by the MN are accepted/rejected (or equivalent signalling solution), TP details in the CR discussion.

R2-2406089	Clarification on RETX_COUNT during an LTM cell switch	Ericsson	CR	Rel-18	38.322	18.0.0	0056	-	B	NR_Mob_enh2-Core

=> Agreed

R2-2404828	[G125] Discussion on LTM cell switch execution during fast MCG recovery procedure	Google Inc.	discussion	Rel-18	38.331	NR_Mob_enh2-Core	R2-2403454

DISCUSSION

-	Ericsson think the timer t304 is there and the UE knows about it so the UE can just wait and if the MCG doesn’t become available the timer would trigger further recovery. Xiaomi agrees. Vivo also agrees, and we can confirm this understanding, no need to update TS. Apple agrees.

-	MTK think if there are split bearer or SRB3 it can be used. HW agrees, think this case is no real problem. OPPO agrees.

-	ZTE think the handling is up to UE acc to current TS so we don’t need additional handling.

-	Google think that the TS says the RRC reconfig complete shall be submitted by MCG.

-	Samsung think that the MCG condition, that UE shall attempt to transmit while MCG is not suspended, is correct and may be worth capturing.

-	Possible Preliminary conclusion: R2 understands that the UE has time to reattempt transmission of this message (until t304 expiry) and could wait until MCG is available, or until t304 expires which would then trigger recovery, according to current TS.

-	Ericsson think we can clarify that the UE uses SRB1 (which can be split also), which is more correct than saying by MCG.

-	Nokia think this is a corner case. Should be avoidable by network impl in most cases. Vivo  agrees.

-	QC also think this is a corner case. Think split SRB1 is not common.

-	HW think the current TS is not clear, was assuming that we could clarify.

the UE uses SRB1 for this case, details to be confirmed in CR discussion

R2-2404438	[E231][E240]RRC issues on LTM	vivo	discussion	Rel-18	NR_Mob_enh2-Core

DISCUSSION

P1

-	Ericsson think this is obvious, note is not needed.

P3

-	Ericsson think that RA-RNTI is not useful, but is already in signalling, can leave as is

-	Session chair: nothing needed

it’s up to network implementation to ensure the correct security key is applied during and after UE access to the target cell if LTM and CHO are configured for the target cell with different security keys. No TS change needed

R2-2405482	[X131] Discussion on the SFN acquisition for LTM	Xiaomi	discussion	Rel-18	NR_Mob_enh2-Core

DISCUSSION

-	QC think we don’t need a note. Xiaomi think we have notes for other cases.

R2 assumes that the UE is required to have valid SFN (read MIB at some point) of the candidate cell for early RACH if the timing information is required for the early RACH procedure of LTM, not much support to explicitly capture.

R2-2404805	[B120][B121]Coexistence of LTM and conditional reconfiguration	Lenovo	discussion	Rel-18

DISCUSSION

-	P2 P5 OPPO wonder how the network can avoid this. Think P3 P6 are already captured in stage-2 TS.

-	P2 HW think it is not possible that the network can avoid. Think nothing R3 can/should do also for the other cases.

-	Ericsson agrees with HW, Nokia as well.

Noted, no support to further capture behaviours.

R2-2405144	On Early decoding, power control for UL transmission after LTM switch and coexistence with NES	Nokia	discussion	Rel-18	NR_Mob_enh2-Core

DISCUSSION (only P1)

P1

-	LGE think this shall be up to UE impl, this is not needed

-	HW wonder what is the difference. Nokia think the latency will be different. Xiaomi think R4 defines max delay, so some UEs will be a lot faster.

-	Ericsson likes p1. NEC too.

-	HW think that is a UE indicates 4 then network can use 4, this is one way to use this.

-	Apple think that the network will anyway not be able to guess the delay very accurately based on a configuration, there are other delays.

-	session chair: there seems to be some support, and some opposition

P1 postponed

Withdrawn

R2-2405159	On the selection of RA preambles for LTM recovery [N161]	Nokia	discussion	Rel-18	NR_Mob_enh2-Core	Withdrawn

#### 7.4.3.2	Conditional Mobility

Includes both Subsequent CPAC and CHO including target MCG and candidate SCGs for CPC CPA in NR-DC.

R2-2404439	[V138] Discussion on simultaneous evaluation for both condExecutionCond and condExecutionCondSCG	vivo, CATT, OPPO, LG Electronics	discussion	Rel-18	NR_Mob_enh2-Core

-	Ericsson think the term applicable measID is not defined. Vivo explain that other wordings can be possible.

-	Huawei think indeed there is an unclarity.

-	Nokia think the NR-DC check is not needed. vivo think this is C147.

Intention agreeable but TP need updates

R2-2405060	[Z062][Z063][Z064] Remaining issues for subsequent CPAC	ZTE Corporation	discussion	Rel-18	NR_Mob_enh2-Core

DISCUSSION

-	Huawei think both TPs are ok.

-	OPPO think no change is needed, think for P1 the legacy text is applicable. ZTE think there is a mismatch between stage-2 and stage-3.

-	NEC also agree with intention, think TP for P1 is ok, for P2 need some modification.

-	Ericsson agrees with intentions of P1 P2 think TP can be simplified. Think P4 is not needed

- 	vivo support.

Direction of P1P2 is agreeable, offline disc TP and whether clarification discussed for P4 is needed/desired.

R2-2404483	Open issues for subsequent CPAC		Ericsson	discussion	Rel-18	NR_Mob_enh2-Core

DISCUSSION

P123

-	Ericsson clarifies that the compliance check need to cover that there is an skcounter when needed. Nokia think that the network shall ensure this doesn’t happen. QC agrees.

-	HW think we could have a clarification that the network is supposed to handle this. Not by a note in compliance check.

-	LG agree with the intention, think the assumption for UE behaviour is reasonable, but think this should be captured elsewhere.

-	Ericsson think anyway that the note is misleading and should be updated.

P4

-	OPPO agrees, but think it need to be complemented.

-	Nokia think this is already covered

P6

-	hw think this could be raised directly in SA3. HW think that the UE TS is crystal clear.

-	QC agrees that UE beh is clear, not sure why we need to send LS to SA3.

-	Ericsson think R2 stage-3 is inconsistent with SA3 stage-2.

-	Session chair: no support right now to send LS, (and no willingness to change in R2), can allow to check.

P7

-	Nokia wonder why we need exec conditions after measurement config has been released. HW also think the current CR text is ok, and removing this is strange.

-	Ericsson think this is SCG config and will anyway be released, HW think this is MCG variable and will not be auto-released by SCG release if not explicit .

-	Session Chair: can check offline

Confirm that network shall ensure that UE has sk-counter value. Not much support for additional clarifications.

Check offline whether clarifications related to P4 are needed [504]

R2-2404605	[E220] Issue on the presence of sk-counter in SCPAC	CATT	discussion	Rel-18	NR_Mob_enh2-Core

R2-2404606	Remaining SCPAC issues	CATT	discussion	Rel-18	NR_Mob_enh2-Core

R2-2404415	Discussion on remaining issues for SCPAC execution	OPPO	discussion	Rel-18	NR_Mob_enh2-Core

R2-2404412	Discussion on L2 reset for SCPAC execution	OPPO, NEC	discussion	Rel-18	NR_Mob_enh2-Core

R2-2405190	On MCG Reset handling for SCPAC in MN-Format	Nokia	discussion

R2-2405217	[C147] MAC behaviours in SCPAC	Huawei, HiSilicon	discussion	Rel-18	NR_Mob_enh2-Core

R2-2405386	[C147] MCG MAC reset upon SCPAC execution	CATT	discussion	NR_Mob_enh2-Core

7 tdocs noted

[AT126][504][R18MobE] Conditional Mobility (OPPO)

Scope: Cover and Converge on MCG reset to the extent reasonable, at least pave the way for decision at online CB. Can include some remaining part of other untreated tdocs if needed. Progress indicated parts and TP aspects.

Intended outcome: Report w TP

Deadline: CB, see schedule

R2-2405944	Report [AT126][504][R18MobE] Conditional Mobility (OPPO)	OPPO

P6:

-	Ericsson think we don’t need to specify what the network configures.

-	HW think that “initial configuration .. “ is not so clear.

-	Xiaomi think that companies understand the intention, can maybe capture in chair notes.

MCG MAC is not reset at SCPAC execution and the MN needs to use different LCID values for the MCG RLC bearer serving a radio bearer when the radio bearer is terminated in the MN and in the SN. Remove “reset MCG MAC” in section 5.3.5.13.8 for Subsequent CPAC execution.

The sk-Counter is absent in the RRCReconfiguration message contained in condRRCReconfig which is for a SCPAC candidate. Add” This field is absent if the RRCReconfiguration message is contained in condRRCReconfig for subsequent CPAC” in the field description of sk-Counter and remove the restriction in the field description of sk-counterConfiguration.

SCG RLC entity for radio bearers without termination point change or security key update is always re-established and the PDCP data recovery is performed for AM DRB if the associated RLC entity is released/re-established. Adopt the updated TP (Annex 4.3), as base.

The servingSecurityCellSetId is not present when there is no SCG

Adopt the updated TP(Annex 4.1) for RIL [V138], as base

Adopt the updated TP(Annex4.2) for RIL [Z062][Z063], as bas

RAN2 clarifies that the candidateCellInfoListSubsequentCPC is only included in the CG-Config message contained in the CG-CandidateList message, to imply the association between the candidate cell and the list of execution conditions for the following execution of subsequent CPAC. ZTE TP use as base,

Regarding: the other conditional reconfiguration (with condReconfigId) is a legacy CPC/CPA configuration should be included in the procedure for evaluation of CPA/CPC/CPAC configurations in 5.3.5.13.4. Assume Base TP from Ericsson and further review as part of CR discussion.

P1P2 in R2-2404606 and P1-P4 in R2-2404415 are discussed during post email discussion for RRC CR implementation.

- The current RAN2 TS behaviour is confirmed:

The legacy sk-Counter can be configured for Rel-15 inter-SN PSCell change/addition if subsequent CPAC configuration is configured or included in the same RRC message. No RAN2 spec change is needed.

The legacy sk-Counter can be configured for Rel-17 CPAC if subsequent CPAC configuration is configured or included in the same RRC message. No RAN2 spec change is needed.

#### 7.4.3.3	Reporting of Idle Inactive and reselection measurements

Draft CRs UE cap

R2-2405561	UE capability for EMR and reselection measurements	Nokia	draftCR	Rel-18	38.331	18.1.0	NR_Mob_enh2-Core

R2-2405562	UE capability for EMR and reselection measurements	Nokia	draftCR	Rel-18	38.306	18.1.0	NR_Mob_enh2-Core

-	No comments Tuesday

Both endorsed (for merge in the mega CR)

Corrections

R2-2405563	EMR and reselection measurements details	Nokia	discussion	Rel-18	NR_Mob_enh2-Core

Noted

R2-2404484	Discussion on early measurements enhancements	Ericsson	discussion	Rel-18	NR_Mob_enh2-Core

DISCUSSION

P1 etc

-	vivo support, think this is helpful.

-	ZTE think that the problem is that the target cannot obtain the configuration, 2 approaches. Think only one bit is not sufficient. Prefer to send the configured timer X to the network. QC wonder what the network will do if the timer is reported, support reporting of validity status. Nokia thinking along the same lines.

-	HW think we should provide the information by the network. ZTE think it doesn’t work for Idle UEs.

- 	ZTE think that different configured X can be common, think that the RRC release method is the issue.

-	Options: a) report validity status (with some remaining ambiguity), b) remove config of X in RRC release, c) report X as an indication of validity status. d) no change

-	Nokia wonder if a) means that we also remove the request in MSG4. Huawei think we cannot do that unless we also add one more bit in SIB1. HW think the network has the UE caps and there is no ambiguity whether the UE reports validated measurements or not.

Tentative way forward:

- ? UE reports value of configured X as an indication that measurement is valid, when the network request valid measurements.

Continue disc OFFLINE [503]

R2-2404379	Discussion on eEMR SCell setup delay	vivo	discussion

R2-2405061	Remaining issues on eEMR and IMR	ZTE Corporation	discussion	Rel-18	NR_Mob_enh2-Core

R2-2405218	[H146][H147][H166][H167][H168] Issues on EMR and reporting of cell reselection results	Huawei, HiSilicon	discussion	Rel-18	NR_Mob_enh2-Core

3 above tdocs Noted (taken into account in [503])

[AT126][503][R18MobE] Early Measurements (Nokia)

Scope: Need to have a functioning procedure, can disc the procedure options and impacts, with the on-line discussion on validity status etc as starting point. 
Include in this discussion: bugfixes from the submitted tdocs.

Intended outcome: Report, TP if applicable

Deadline: CB, see schedule

R2-2405995	[AT126][503][R18MobE] Early Measurements (Nokia)	Nokia

DISCUSSION

P4

-	Ericsson support 2.

P1: The UE uses the SIB11 value of validity timer if no validity timer was configured in RRCRelease (i.e. not depending on if frequencies were configured or not).

P4: Go with approach 2, as proposed. Attached TP as starting point. Review details in the CR discussion.

### 7.4.4	MAC Corrections

MAC corrections and User Plane Centric Issues (including tdocs on user plane centric issue that also impact other TS)

In-principle agreed CR

R2-2405219	Miscellaneous corrections for further mobility enhancements	Huawei, HiSilicon	CR	Rel-18	38.321	18.1.0	1817	2	F	NR_Mob_enh2-Core	R2-2404023

-	Baseline for further updates

Revised in post email discussion

=> Revised in R2-2406030

[Post126][515][R18MobE] 38321 (Huawei)

Scope: Reflect agreements and progress in the CR, resolve remaining points as discussed at the meeting.

Intended outcome: Agreed CR

Deadline: Short

R2-2406030	Miscellaneous corrections for further mobility enhancements	Huawei, HiSilicon	CR	Rel-18	38.321	18.1.0	1817	3	F	NR_Mob_enh2-Core

=> Agreed

Corrections

R2-2404229	MAC corrections for LTM	Samsung	discussion	Rel-18	NR_Mob_enh2-Core

- 	Samsung rports that there is another change in main session that overlaps with and covers this

noted

R2-2405331	UL grant handling during RACH-less LTM cell switch	LG Electronics Inc, Lenovo, ZTE Corporation	discussion	Rel-18	NR_Mob_enh2-Core

-	Nokia wonder why it would be beneficial to skip grants. Network can know the situation anyway. Think that the full rrc reconfig complete may not fit CG. Lenovo agrees.

-	ZTE would like to prevent data loss for the RRC reconfig complete. Not sure there is an issue. ZTE think indeed Dyn retransmission for CG shall be able to be used.

-	Nokia think the network can send a new grant for the CG new transmission. Nokia think this worsens the performance of the handover.

-	Session chair: not much support, consider this issue closed.

Noted

R2-2405220	MAC remaining issues for LTM	Huawei, HiSilicon	discussion	Rel-18	NR_Mob_enh2-Core

DISCUSSION

P1

-	Nokia still think we should go the other way, not much impact

2b

-	Nokia wonder why we need this. Chair think we received an LS with this.

P6

-	Samsung think the current TS works and think RACH config dedicated has other helpful configurations that is not possible with RACH config common.

-	CATT support HW, and think this is the intention.

-	ZTE support HW.

-	Nokia think RACH config dedicated allows RACH resources not used for common at all which is intended for CFRA. Nokia think that already resources from RACH config common can be used if no resource in RACH config dedicated.

-	Session Chair: no consensus

P7

-	Nokia think this allows the gNB to use the CFRA resource for other purposes.

Keep the current MAC procedure: when UE-based TA measurement is configured, the UE determines RACH-based or RACH-less cell switch by checking whether it has successfully measured a TA for the target cell.

In MAC: capture that the unifiedTCI-StateType for the TCI states in the LTM cell switch MAC CE refers to the field in LTM-TCI-Info.

In RRC: capture that the unifiedTCI-StateType in LTM-TCI-Info should be the same with the unifiedTCI-StateType in candidate cell configuration (ltm-CandidateConfig).

For recovery of RLF / reconfiguration with sync failure via LTM, the UE performs CBRA to the selected PCell.

R2-2405181	Consideration On Remaining Issues For LTM	ZTE Corporation	discussion	Rel-18	NR_Mob_enh2-Core

DISCUSSION

P2P3

-	OPPO think this is discussed in R1 and have already agreed this direction.

-	HW think P2 is already captured in current R1 procedures, no need to capture more. Not clear that we need to send LS to R3.

-	Samsung think 2TA is form a single PCI so support P3 from ZTE.

-	Ericsson think there is the same proposal in RAN3. Can wait.

-	LG wonder why P3 is needed if P2 is the assumption.

R2 assumes that In the case of the candidate cell configured with inter-cell mTRP with 2TA, RAN2 confirm the early RACH configuration (e.g. EarlyUL-SyncConfig) and SSB configuration (e.g. ltm-SSB-Config-r18 ) should be related to the TRP associated to the candidate cell not the TRP associated to the additional cells of the candidate cell.

In case of early RACH across DUs with intra-cell mTRP with 2TA, R2 assumes the target DU should forward the TAG information (e.g. tag-Id-ptr) to the source DU.

R2-2405663	Remaining MAC issues for LTM	Nokia	discussion	Rel-18

R2-2404440	Discussion on MAC open issue for LTM	vivo	discussion	Rel-18	NR_Mob_enh2-Core

R2-2405421	Discussion on fallback RACH for LTM	ASUSTeK	discussion	Rel-18	38.331	NR_Mob_enh2-Core	R2-2402579

R2-2404413	Discussion on remaining MAC issues for LTM	OPPO	discussion	Rel-18	NR_Mob_enh2-Core

R2-2404414	Issues on supporting MIMO 2TA for LTM	OPPO	discussion	Rel-18	NR_Mob_enh2-Core

R2-2404920	Discussion on TCI state in LTM command MAC CE	NEC	discussion	Rel-18	NR_Mob_enh2-Core

R2-2404969	Remaining MAC issues for LTM	Ericsson	discussion	Rel-18	NR_Mob_enh2-Core

R2-2405422	Discussion on LTM candidate configuration for different CGs	ASUSTeK	discussion	Rel-18	38.321	NR_Mob_enh2-Core	R2-2402580

R2-2405661	On the selection of RA preambles for LTM recovery [N161]	Nokia	discussion	Rel-18

Moved from 7.4.3.1

withdrawn

R2-2405160	Remaining MAC issues for LTM	Nokia	discussion	Rel-18	NR_Mob_enh2-Core	Withdrawn

### 7.4.5	UE capabilities

Including outcome of [Post125bis][516][R18Mob] UE cap CRs (Intel)

Email disc

R2-2404272	Draft 306 CR for UE capability for feMob	Intel Corporation	draftCR	Rel-18	38.306	18.1.0	NR_Mob_enh2-Core

R2-2404273	Draft 331 CR for UE capability for feMob	Intel Corporation	draftCR	Rel-18	38.331	18.1.0	NR_Mob_enh2-Core

Both endorsed

Remove LTM cap, see below

Email approval (super short email disc)

=> Revised in R2-2406035 (38.306) and R2-2406036 (38.331)

[Post126][513][R18MobE] UE capabilities (Intel)

Scope: Reflect agreements and progress in the CRs, resolve points as discussed at the meeting.

Intended outcome: endorsed CRs 38306 38331 (for merge)

Deadline: Very Short (for merge)

R2-2406035	Draft 306 CR for UE capability for feMob	Intel Corporation	draftCR	Rel-18	38.306	18.1.0	NR_Mob_enh2-Core

=> Endorsed to be merged to R2-2406069

R2-2406036	Draft 331 CR for UE capability for feMob	Intel Corporation	draftCR	Rel-18	38.331	18.1.0	NR_Mob_enh2-Core

=> Endorsed to be merged to R2-2406070

Corrections

R2-2404705	Band-pair signalling for Early TA acquisition UE capabilities	Qualcomm Incorporated	discussion	Rel-18	NR_Mob_enh2-Core

Moved from 7.0.

Noted

R2-2405245	Mobility UE capabilities with Per band pair per band combination granularity	Huawei, HiSilicon	discussion	Rel-18	NR_Mob_enh2-Core

Moved from 7.0.1

Noted

DISCUSSION, the two tdocs above.

-	Ericsson think there is also another solution, MSD approach, think offline is good.

-	Intel don’t agree with O1, but ok to go either way (QC or HW).

-	Nokia confirm O1, think this is related to R4, and cannot only resolve in R2. R4 is discussing

-	MTK think that O1 has been confirmed in RAN4 already and prefer HW CR.

-	ZTE want to go offline, think O1 is not supported by HW proposal. HW agrees with O1.

-	Intel agrees w O1 and think this is supported in the current CR. QC has assumed differently, maybe need to check.

[AT126][502][R18MobE] UE cap Early TA acquisition (Qualcomm)

Scope: Continue offline, i.e. based on R2-2405245 and R2-2404705 and the related discussion, determine if change is needed / desired and converge as far as reasonable, determine way forward, or alternatives / discussion points, If applicable, check impact on this discussion of R4 involvement, if any.

Intended outcome: Report

Deadline: CB, see schedule

R2-2405954	[AT126][502][R18MobE] UE cap Early TA acquisition (Qualcomm)	Qualcomm	discussion

-	Apple wonder if applied freq bands will be used by network vendors for this purpose.

RAN2 assumes that the target band for RACH transmission is any supported band within or outside the band combination. This can be revisited if RAN1 or RAN4 indicates otherwise in the future

RAN2 pursues signalling solution where the target bands for RACH transmission are signalled per feature set, and further discuss how the target bands are indicated, by pointing to appliedFreqBandList.

R2-2404299	LTM UE capabilities	MediaTek inc.	discussion	Rel-18	NR_Mob_enh2-Cor

DISCUSSION

-	Intel think other groups have difficulty converging.

-	Ericsson think R4 has made agreements at least for FR1.

P1

-	Huawei think for point 2, need to think about whether this granularity need to be considered.

-	ZTE think this si now per CG, need to think about how to do this.

P2

-	Intel Think we don’t need to ask. They should progress their features anyway.

The LTM L1 measurement capability and LTM capabilities are decoupled

As the LTM L1 measurement capability and LTM capabilities are decoupled:

- 1. A UE which reports LTM capability without 45-1 may not perform L1 measurement reporting, and it is up to network implementation how to trigger the LTM execution.

- 2. The granularity is FFS, e.g. whether LTM capability is divided into two separate capabilities for inter-frequency and intra-frequency.

Remove LTM capability from current TS

Long email discussion on the FFS and possibly other points (can take into account late LSes from other groups).

[Post126][514][R18MobE] UE capabilities Open Issues (intel)

Scope: progress open issues

Intended outcome: Report, TP/draftCRs if applicable

Deadline: Long

## 7.5	XR Enhancements for NR

(NR_XR_enh-Core; leading WG: RAN2; REL-18; WID: RP-230786)

Time budget: 0 TU

Tdoc Limitation: 2 Tdocs

### 7.5.1	Organizational

Including LSs, any rapporteur inputs (e.g. work plan, SA2/SA4 progress reports) and running CRs (currently endorsed CRs exist fo Stage-2 (Nokia), MAC (Qualcomm), PDCP (LGE), RRC (Huawei) and RLC (vivo))

Stage 2 corrections

R2-2404286	Miscellaneous XR corrections	Nokia (Rapporteur)	CR	Rel-18	38.300	18.1.0	0860	-	F	NR_XR_enh-Core

-	Fujitsu asks where the SN gap reporting part

=>	The CR is revised

[AT126][016][XR] Stage 2 CR  (Nokia)

Intended outcome: Agree to CR by email

Deadline:  05-24-24

R2-2405934	Miscellaneous XR corrections	Nokia (Rapporteur)	CR	Rel-18	38.300	18.1.0	0860	1	F	NR_XR_enh-Core

=>	The CR is agreed

RAN3 overview

R2-2404285	RAN3 Overview	Nokia, Qualcomm (Rapporteurs)	discussion	Rel-18	NR_XR_enh-Core

=>	Noted

MAC corrections

R2-2404172	Corrections for XR services	Qualcomm Incorporated	CR	Rel-18	38.321	18.1.0	1833	-	F	NR_XR_enh-Core

=>	The CR will be used as a baseline and will be revised with agreements from this meeting

=> Revised in R2-2406092

[POST126][017][XR] CR to 38.321  (Qualcomm)

Intended outcome: Agree to CR

Deadline:  Short

R2-2406092	Corrections for XR services	Qualcomm Incorporated	CR	Rel-18	38.321	18.1.0	1833	1	F	NR_XR_enh-Core

=> Agreed

RRC corrections

R2-2405643	RRC corrections for XR	Huawei, HiSilicon	CR	Rel-18	38.331	18.1.0	4700	2	F	NR_XR_enh-Core	R2-2404008

-	The CR will be used as a baseline and will be revised with agreements from this meeting

[AT126][018][XR] CR to 38.331 (Huawei)

Intended outcome: Agree to CR by email

Deadline:  05-24-24

R2-2405785	RRC corrections for XR	Huawei, HiSilicon	CR	Rel-18	38.331	18.1.0	4700	3	F	NR_XR_enh-Core

=>	The CR is agreed

RIL resolutions

R2-2405644	RIL resolutions for XR	Huawei, HiSilicon	discussion	Rel-18	NR_XR_enh-Core	R2-2404036

=>	B020 is agreed

=>	Noted

Capability spec corrections

R2-2404524	UE Capability on PDCP SN Gap Reporting	Intel Corporation	draftCR	Rel-18	38.306	18.1.0	NR_XR_enh-Core

-	Futurewei thinks that we should change the IE name as it is confusing and similar to PCPD one.  Huawei thinks we can keep the name here but update it in RRC.

=>	The CR is endorsed and will be merged in the Mega CR

R2-2404525	UE Capability on PDCP SN Gap Reporting	Intel Corporation	draftCR	Rel-18	38.331	18.1.0	NR_XR_enh-Core

=>	The CR is endorsed and will be merged in the Mega CR

### 7.5.2	RRC corrections

Including RIL and UE capabilties

TP correction from LCG to LCH for DSR trigger

R2-2404324	Correction on DSR Trigger Modelling	CATT	discussion	Rel-18	NR_XR_enh-Core

Proposal 1: Modify the DSR trigger from LCG to LCH in TS38.300.

Proposal 2: Modify the DSR trigger from LCG to LCH in TS38.331.

-	Nokia reminds company to provide editorial directly to rapporteur

-	Xiaomi and LG think the correction is ok in 38.300 but in 38.331 it is not needed.  Huawei suggests that in the field description we mention it is the logical channel belong to that group.

=>	The change will be done by rapporteurs in 38.300 and 38.331 will in the field description mention it is the logical channel belong to that group

=>	Noted

### 7.5.3	User plane corrections

No documents should be submitted to 7.5.3. Please submit to 7.5.3.x

#### 7.5.3.1	MAC corrections

DRX_SFN_COUNTER

R2-2404547	Open issues for DRX_SFN_COUNTER for XR	ZTE Corporation, Sanechips	discussion	Rel-18	NR_XR_enh-Core

Proposal 1: The DRX_SFN_COUNTER is initialized in the first symbol of the slot immediately after the first PUSCH transmission which contains the response message of the RRC (re-) configuration message (i.e. point of initialization is same as the one captured in the current MAC spec)

Proposal 2: If, the first symbol of the slot immediately after the first PUSCH transmission which contains the response message of the RRC (re-) configuration message falls in the first part of the HFN, the DRX_SFN_COUNTER shall be initialized to 0. Otherwise, it shall be initialized to 1.

=>	Noted

R2-2404173	Discussion on DRX_SFN_COUNTER	Qualcomm Incorporated	discussion	Rel-18	NR_XR_enh-Core

Proposal 1. 	UE determines the initial value of DRX_SFN_COUNTER based on the time when a HARQ ACK for the RRC message containing a DRX re-/configuration is sent. UE applies the new DRX configuration when it decodes the RRC message (as in legacy).

=>	Noted

R2-2404513	DRX SFN counter initialization	Ericsson	discussion	Rel-18	NR_XR_enh-Core

Proposal 1	Adopt the TP above for initializing the DRX_SFN_COUNTER.

=>	Noted

R2-2405047	Discussion on the DRX_SFN_COUNTER initialization	OPPO	discussion	Rel-18	NR_XR_enh-Core

Proposal 1	Update the MAC spec to initialize DRX_SFN_COUNTER upon the corresponding RRC (re-)configuration message for DRX being received.

=>	Noted

R2-2404772	Remaining issues for DRX operations for XR	Huawei, HiSilicon	discussion	Rel-18	NR_XR_enh-Core

Proposal 1: Remove the sentence “in the first symbol of the slot immediately after the first PUSCH transmission which contains the RRCReconfigurationComplete message of the RRC (re-) configuration as specified in TS 38.331 [5]” from the procedure text. Adopt the TP for TS 38.321 for DRX_SFN_COUNTER initialization.

=>	Noted

Discussion

-	Nokia thinks that we don’t need a specific timing and thinks we should go with Huawei’s proposal.  Oppo, Xiaomi, LG agree.

-	Samsung thinks that ZTE and Ericsson’s make sense as it should be done after synchronization with target cell.

-	ZTE thinks that the problem is that the processing time is unknown.  We are leaving it to UE implementation that may cause problem so it would be good to make the point very clear.

-	Qualcomm thinks that there is still ambiguity when the RRC message was received and whether the UE and the NW are aware.

-	LG thinks that the UE applies the configuration when the RRC configuration message is received and HW proposal addresses this.   Huawei thinks that when the network would send the message and then increment it by one and so would the UE.

-	Samsung is concerned with the HO case.  ZTE is not concerned with the HO case.   Xiaomi explains that we agree that the UE would initialize when it acquires a new SFN in target cell.

-	Lenovo thinks that we never specify small timing issues and ambiguities.

-	ZTE explains that the network won’t know that the UE applied the configuration until it receives the RRC response.

-	Huawei explains that one problem is that the UE would apply part of the configuration in one point and another one at another point.   ZTE thinks that the configuration would apply at the same time, it is just about the counter managing and we initialize counter at different points.

-	Intel suggests to add a clarification for the HO scenario.

=>	Remove the sentence “in the first symbol of the slot immediately after the first PUSCH transmission which contains the RRCReconfigurationComplete message of the RRC (re-) configuration as specified in TS 38.331 [5]” from the procedure text. Adopt the TP for TS 38.321 for DRX_SFN_COUNTER initialization.   FFS HO case

DRX formula for non-integer

R2-2404889	Change the formula of non-integer short DRX cycle	NEC, Xiaomi, Ericsson, Lenovo, CATT, LGE, vivo, Nokia, Nokia Shanghai Bell, CMCC, MediaTek Inc	discussion	Rel-18	NR_XR_enh-Core

Proposal 1: Change the formula of short DRX cycle to floor([(DRX_SFN_COUNTER × 10240) + (SFN × 10) + subframe number - drx-StartOffset)] modulo (drx-NonIntegerShortCycle)) = 0

-	Samsung supports

=>	Change the formula of short DRX cycle to floor([(DRX_SFN_COUNTER × 10240) + (SFN × 10) + subframe number - drx-StartOffset)] modulo (drx-NonIntegerShortCycle)) = 0

=>	Noted

DSR cancelation

R2-2404709	Remaining issues on DSR and proposed TP to MAC	Xiaomi Communications, Nokia, Nokia Shanghai Bell, Qualcomm Incorporated	discussion	Rel-18	NR_XR_enh-Core

=>	A PDCP SDU is considered to be associated with a DSR if it has not been transmitted in any MAC PDU and is a delay-critical PDCP SDU (as defined in TS38.323) associated with the LCH which triggered the DSR.

=>	Noted

Refined long BSR

R2-2404707	Refined Long BSR determination	Nokia, Nokia Shanghai Bell, Samsung	discussion	Rel-18	NR_XR_enh-Core

Proposal 1: Long BSR or Refined Long BSR determination should be done based on the buffer status after the MAC PDU is built.

Proposal 2: it is applicable to both regular BSR and padding BSR.

-	Qualcomm, Xiaomi, Lenovo, Huawei and LG is concerned as this impacts the MAC PDU assembly time.  LG thinks that the MAC PDU may need to be assemble twice and not desirable from UE perspective.

-	Samsung understand the complexity but it is beneficial for the UE to follow this proposal.

-	Nokia suggests to instead add the existing text “when the MAC PDU containing the BSR is to be built” to the other two bullets.  Rapporteur will check offline.

=>	Not supported

=>	Noted

Initial state of PSI-based SDU discard after RRC resume

R2-2404325	MAC Corrections	CATT	discussion	Rel-18	NR_XR_enh-Core

Proposal 1: Clarify in TS 38.321 that the initial state of PSI-based SDU discard is deactivated when the UE restores configuration during resume procedure.

-	Samsung and Xiaomi thinks that this is already covered by currect spec

=>	Not needed

=>	Noted

UTO-UCI

R2-2404773	Discussion on MAC procedure for UTO-UCI	Huawei, HiSilicon	discussion	Rel-18	NR_XR_enh-Core

Proposal1: The MAC entity shall determine UTO-UCI at MAC PDU assembly.

-	Samsung, LG, Qualcomm, Apple, Lenovo, thinks that this should be up to UE implementation.  QC explains that RAN1 discussed and agreed not to.

-	Ericsson thinks the UEs are too free.

-	Huawei would like to at least specify something lose.  Qualcomm thinks there is no point as it is not testable.

=>	Not supported

=>	Noted

R2-2405447	UTO-UCI and cases where UE is allowed to not generate MAC PDUs	Ericsson, ZTE Corporation	discussion	Rel-17	NR_XR_enh-Core

Proposal 1	RAN2 to resolve the case where the UE, for a configured grant with UTO-UCI configured, do not follow the conditions for not generating a MAC PDU in violation of RAN1 agreements.

Proposal 2	When determining whether a configured uplink grant shall be indicated as unused for PUSCH transmission, the UE shall consider if the configured uplink grants do not satisfy the conditions for not generating a MAC PDU according to 5.4.3.1.3.

-	LG thinks that we discussed this before and we don’t want to specify everything and all the cases.    Oppo also thinks that there is no issue.   Ericsson thinks that we break a RAN1 agreement, the gNB doesn’t know what the UE decides.

-	Samsung, Apple and Qualcomm doesn’t think this is an issue. UTO-UCI can be sent in advance and there is no decoding ambiguity.

=>	Not supported

=>	Noted

Not treated

R2-2404238	Discussion on redundancy of BSR in the presence of DSR MAC CE	IIT, Kharagpur	discussion	Rel-18

R2-2404441	Remaining time report in DSR MAC CE	vivo	discussion	Rel-18	NR_XR_enh-Core

R2-2404548	Correction for initialization of DRX_SFN_COUNTER for XR	ZTE Corporation, Sanechips	CR	Rel-18	38.321	18.1.0	1839	-	F	NR_XR_enh-Core

R2-2404837	Initialization of DRX_SFN_COUNTER	Xiaomi	discussion	Rel-18	NR_XR_enh-Core

R2-2405407	Clarification on SR triggering for DSR	DENSO CORPORATION	discussion	Rel-18	NR_XR_enh-Core

R2-2405494	Remaining issues on BSR and DSR	Samsung	discussion	Rel-18	NR_XR_enh-Core

R2-2405605	Correction on DSR MAC CE	Langbo	discussion	Rel-18	38.321	NR_XR_enh-Core

#### 7.5.3.2	PDCP and discard operation

Including outcome of [POST125bis][016][XR] PDCP SN gap reporting  (Ericsson)

PDCP SN gap reporting

R2-2404468	Report of [POST125bis][016][XR] PDCP SN gap reporting  (Ericsson)	Ericsson	discussion	Rel-18

=>	PDCP SN gap report is not triggered when the discard timer expires for the most recent PDCP SDU after it has been submitted to the lower layers but is not ACKed.

=>	The TPs will be incorporated in the rapporteurs specs

=>	Noted

TP update: RX_NEXT update for the gap reports

R2-2404287	PDCP SN gap report and updating RX_NEX	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_XR_enh-Core

Proposal 1: RX_NEXT is incremented (upon reception of a SN gap report and upon being updated by a received SDU) only if, without the incrementing, the SDU with associated COUNT equal to RX_NEXT would be considered as discarded (i.e. indicated as discarded in some received SN gap report).

Proposal 2: RAN2 agree the text proposal in Annex for Proposal 1.

-	Samsung shares the concerns.   LG doesn’t think there is a problem as the reporting is done later than stored SDUs.  Nokia thinks that it assumes no pre-processing and the scenario can occur so we can even prevent the scenario to occur or we have a fix.

-	Ericsson has some sympathy for the proposal, the prioritization in the DL is possible but there may be difference in solution.  Intel also acknowledges the existence of this problem.

-	Vivo thinks that this is a big impact to UE and it can be handle by network implementation.  Nokia thinks that we have to compare it to the TP we have, and we are already changing a behavior.

-	Huawei thinks that this can be solved by NW implementation by setting T-ordering time properly.   Ericsson doesn’t think we can solve it with configuration.

=>	Noted

TP update: PDCP SDUs that are to be discarded directly exempted from transmit operations

R2-2404648	Clarifications of Operations for Directly Discarded PDCP SDUs	Apple	discussion	Rel-18	NR_XR_enh-Core

Proposal 1: RAN2 confirms that, for the PDCP SDUs received from the upper layers that are to be discarded directly, the transmit operations specified in the clause 5.2.1 of TS 38.323 are not applicable.

Proposal 2: Adopt the text proposal in the Annex to clarify the behavior described in Proposal 1.

-	LG, Intel, Ericson, Samsung think it is clear.  Intel points out that this was discussed at length before.  Vivo has a similar view.

=>	Not supported

=>	Noted

Miscellaneous TP updates

R2-2404665	Corrections for PDCP SN Gap Reporting	Samsung	discussion	Rel-18

Proposal 1: Include another required condition for triggering of a PDCP SN gap report that “the discarded PDCP SDU(s) have not been reported earlier in PDCP SN gap report”. Adopt the text proposal provided.

-	LG doesn’t understand the problem and the current TP avoids the trigger already.  Samsung thinks that it is left to UE to avoid triggering too much report.   LG thinks we agreed to not trigger for same discared SDU.   Ericsson agrees with LG.

Proposal 2: PDCP SN gap report is composed with considering the first discarded PDCP SDU and the last discarded PDCP SDU in the bitmap based approach that have not been reported earlier in PDCP SN gap report. Adopt text proposal provided.

-	Ericsson and LG thinks it is not a problem and it doesn’t need to be explicitly captured.

-	Vivo thinks the problem is valid but the correction is not.

Proposal 3: In accordance to RAN2#125bis agreement, for transmit operation of the PDCP SN gap reporting replace “transmitted by lower layers” by “have not been submitted by RLC to lower layers”. Adopt text proposal provided.

=>	Noted

=>	In accordance to RAN2#125bis agreement, for transmit operation of the PDCP SN gap reporting replace “transmitted by lower layers” by “have not been submitted by RLC to lower layers”. Adopt text proposal provided

[AT126][019][XR] CR to 38.323 (LG)

Intended outcome: agree to CR by email

Deadline:  05-24-24

R2-2405932	PDCP SN gap reporting	LG Electronics Inc. (Rapporteur)	CR	Rel-18	38.323	18.1.0	0139	-	F	NR_XR_enh-Core

=>	The CR is agreed

Not treated

R2-2405048	Discussion on spec impact on SDAP PDU set remapping	OPPO	discussion	Rel-18	NR_XR_enh-Core

## 7.6	IoT NTN enhancements

(IoT_NTN_enh-Core; leading WG: RAN1; REL-18; WID: RP-223519)

Time budget: 0 TU

Tdoc Limitation: 2 tdocs

### 7.6.1	Organizational

LSs, rapporteur inputs and other organizational documents.

Editorials/clarifications should not be included in any tdoc but sent to the WI spec rapporteurs, who can submit a rapporteur CR as part of this AI.

Rapporteur inputs and other pre-assigned documents in this AI do not count towards the tdoc limitation.

Rapporteur CRs

R2-2405452	IoT NTN Kmac correction	Ericsson	CR	Rel-17	36.300	17.7.0	1402	-	F	LTE_NBIOT_eMTC_NTN-Core

To be handled in AI 4.1.1 (if anything is agreed for R17, then a R18 Cat A mirror CR needs to be agreed as well, while only R18-specific changes should be kept in a separate R18 Cat F CR)

R2-2405453	IoT NTN Kmac and measurment corrections	Ericsson, Huawei	CR	Rel-18	36.300	18.1.0	1401	1	F	IoT_NTN_enh-Core	R2-2403776

(R18 IPA CR subject to changes if a R17 Cat F CR with a mirror R18 Cat A CR will be agreed)

Revised in R2-2405752 to only keep the R18-specific changes (as a R17 CR will be agreed, with a corresponding R18 Cat A mirror CR, for changes that are applicable from R17)

R2-2405752	IoT NTN Kmac and measurment corrections	Ericsson, Huawei	CR	Rel-18	36.300	18.1.0	1401	2	F	IoT_NTN_enh-Core	R2-2405453

Agreed (in the NR maintenance session)

R2-2405128	Corrections to IOT NTN	Huawei, HiSilicon	CR	Rel-18	36.331	18.1.0	5021	-	F	IoT_NTN_enh-Core

Revised in R2-2405758 to take into account further agreements this week

[Post126][305][IoT-NTN Enh] 36.331 CR (Huawei)

Scope: update the RRC CR with meeting agreements

Intended outcome: Agreed CR

Deadline for agreed CR (in R2-2405758): short

R2-2405758	Corrections to IOT NTN	Huawei, HiSilicon	CR	Rel-18	36.331	18.1.0	5021	1	F	IoT_NTN_enh-Core

=> Agreed

R2-2406105	Report of [Post126][305][IoT-NTN Enh] 36.331 CR	Huawei	report	Rel-18	IoT_NTN_enh-Core

=> Noted

R2-2405129	IOT NTN ASN1 RIL List	Huawei, HiSilicon	report	Rel-18	IoT_NTN_enh-Core

H005, S068, E805 and E806 are still ToDo and need to be finalized at this meeting

R2-2404954	Corrections on UE behaviour on DRX for IoT NTN	MediaTek	CR	Rel-18	36.321	18.1.0	1585	2	F	IoT_NTN_enh-Core	R2-2404007

- HW thinks the change “when downlinkHARQ-FeedbackDisabledDCI is configured” is not needed

Remove the change “when downlinkHARQ-FeedbackDisabledDCI is configured”

Revised in R2-2405759 for editorial changes and to take into account further agreements this week

[Post126][306][IoT-NTN Enh] 36.321 CR (Mediatek)

Scope: update the MAC CR with meeting agreements

Intended outcome: Agreed CR

Deadline for agreed CR (in R2-2405759): short

R2-2405759	Corrections on UE behaviour on DRX for IoT NTN	MediaTek	CR	Rel-18	36.321	18.1.0	1585	3	F	IoT_NTN_enh-Core

=> Agreed

Moved here from 7.6.4

R2-2405534	Miscellaneous correction for IoT-NTN	Nokia	CR	Rel-18	36.304	18.1.0	0873	2	F	IoT_NTN_enh-Core	R2-2403768

Agreed

R2-2405302	Corrections on terminology fixed cell	Qualcomm Incorporated	CR	Rel-18	36.306	18.1.0	1889	-	F	IoT_NTN_enh-Core

Revised in R2-2406094 to reflect meeting agreements

[Post126][307][IoT-NTN Enh] 36.306 CR (Qualcomm)

Scope: update the 36.306 CR with meeting agreements

Intended outcome: Agreed CR

Deadline for agreed CR (in R2-2406094): short

R2-2406094	Miscellaneous corrections for IoT NTN capabilities	Qualcomm Incorporated	CR	Rel-18	36.306	18.1.0	1889	1	F	IoT_NTN_enh-Core

=> Agreed

Withdrawn

R2-2404687	Corrections on terminology fixed cell	Qualcomm Incorporated	CR	Rel-18	36.331	18.1.0	5018	-	F	IoT_NTN_enh-Core	Withdrawn

### 7.6.2	Stage 2 corrections

R2-2404593	Discussion on GNSS operation for IoT NTN	OPPO	discussion	Rel-18	IoT_NTN_enh-Core

Proposal 1	The Differential Koffset is cleared when UE is performing GNSS measurement.

- Google thinks this is not needed. QC and Nokia agree with Google

- Oppo thinks the issues is the same as during HO. Nokia thinks this is different from HO as in this case the network can predict when this happens

Proposal 2	Adopt the above TP in stage-2 spec if proposal 1 is agreed.

Proposal 3	UE capability of GNSS measurements during C-DRX inactive time is not reported to the network.

Already agreed in the last meeting

R2-2405117	Remaining issues on GNSS operation	Huawei, HiSilicon	discussion	Rel-18	IoT_NTN_enh-Core

- Whether UE goes to IDLE

Proposal 1: Update the wording in Stage 2 to cover the autonomous GNSS measurement cases as follows “Upon outdated GNSS position the UE shall move to idle mode, unless GNSS acquisition was triggered or being performed or uplink transmission extension is active.”

-	ZTE thinks this is not necessary as existing text (was triggered) already covers also UE autonomous trigger

Proposal 2: Upon GNSS validity expiry, UE doesn’t move to idle mode if UE is performing autonomous GNSS measurement during C-DRX inactive time.

-	Samsung does not agree with this

- Whether UE triggers RACH

Proposal 3: RACH is not triggered for the GNSS Validity Duration report if there is no UL resource available for the case of autonomous GNSS measurement in C-DRX inactive time.

-	Nokia thinks this could be left to UE implementation

-	QC is fine with p3. Google agrees.

-	Ericsson thinks the UE shall trigger RACH but it’s up to the UE when to do it

-	Apple thinks we could have a note

Continue the discussion in [303]

R2-2405152	Stage 2 corrections related to NTN measurements	Samsung	discussion	Rel-18	IoT_NTN_enh-Core

Proposal 1: Agree Stage 2 text proposal in Section 5.1 on measuring NTN cells in a terrestrial network.

Proposal 2: If none of the satellite IDs for a specific frequency in SIB3/SIB5 are present in SIB33, it is up to UE implementation to measure the frequency.

Proposal 3: Agree Stage 2 correction text.

### 7.6.3	RRC Corrections

T390

- [S068] (T390 behaviour for HO/CHO)

R2-2405151	[S068] Handling GNSS uplink transmission extension during handover	Samsung, ZTE, Google	discussion	Rel-18	IoT_NTN_enh-Core

Observation 1: Release 17 IoT NTN never specified requirements for UE to perform GNSS measurement during a handover.

Proposal 1: Any solution shall ensure that a handover execution to target cell does not occur if UE does not have a valid GNSS position fix.

Proposal 2: RAN2 considers the following solutions:

-	A) Network does not trigger handover while T390 is running

-	B) A GNSS position fix is performed before the handover execution if T390 is running.

Proposal 3: If B) in P2 is agreed, agree the text proposal in Section 5.

[AT126][302][IoT NTN Enh] T390 issues (Samsung)

Scope: discuss T390 issues (based on the proposals in R2-2405151, R2-2405499, R2-2405440, R2-2404157, R2-2404653, R2-2405526, R2-2404594)

Intended outcome: report of offline discussion

Deadline for companies' feedback:  Wednesday 2024-05-22 20:00 (but F2F discussion is invited)

Deadline for rapporteur's summary (in R2-2405760):  Thursday 2024-05-23 08:00

R2-2405760	Summary of [AT116][302][IoT NTN Enh] T390 issues	Samsung	discussion	Rel-18	IoT_NTN_enh-Core

Proposal 1: RAN2 confirms understanding that connected mode random access to Pcell (including intra-cell handover case) is possible. No RAN2 spec impact.

-	Apple thinks that RAN4 did not discuss the case of RACH during T390

-	MTK thinks RAN4 should be in CC at most

RAN2 confirms the understanding that connected mode random access to Pcell (including intra-cell handover case) is possible while T390 is running. No RAN2 spec impact.

Proposal 2: RAN2 sends LS to RAN1 only to inform RAN1 about the understanding.

-	Oppo suggests to add a sentence saying that if they see any issues they can come back on this

RAN2 sends LS to RAN1 (cc RAN4) to inform RAN1 about the understanding with a sentence saying that if they see any issues they can come back on this

-	Apple and Nokia think we could also ask about the HO (target cell) case

Proposal 3: Network ensures that neither inter-cell handover or conditional handover is triggered to target cell while T390 is running via network implementation. No stage 3 impact.

-	QC thinks intra-cell HO should be similar to RACH in connected and then should be allowed

-	ZTE thinks the real trigger of CHO is UE based

Network ensures that neither inter-cell handover or conditional handover is triggered to target cell while T390 is running via network implementation. No stage 3 impact.

Include also this RAN2 decision in the LS to RAN1

Proposal 4: T390 is stopped after successful GNSS position fix during C-DRX inactive time.

-	Google could be fine but thinks we should have a single criterion for stopping T390.

-	QC agrees and thinks we can remove “during C-DRX inactive time”

-	Oppo is fine with p4 and thinks this is a special case to be treated separately. Nokia agree

Agreed (can further discuss how to capture this in the CR review)

Proposal 5: Discuss in CR review phase how to capture T390 stopping condition for the following cases:

- Network-triggered GNSS position fix

- Autonomous GNSS position fix

- GNSS position fix during C-DRX inactive time

-	ZTE thinks the first 2 points have already been covered and we just need to focus on the last point. Apple agrees

In the CR review we focus on how to capture T390 stopping condition for GNSS position fix during C-DRX inactive time

[AT126][307][IoT NTN Enh] LS to RAN1 on T390 (Samsung)

Scope: Draft an LS to RAN1 on T390 based on meeting agreements

Intended outcome: approved LS

Deadline for rapporteur's summary (in R2-2405764):  Friday 2024-05-23 08:00

R2-2405764	LS on T390 	Samsung	LS out		LS out	Rel-18	IoT_NTN_enh-Core	To:RAN1, Cc:RAN4

Revised in R2-2406019

R2-2406019	LS on T390 	Samsung	LS out		LS out	Rel-18	IoT_NTN_enh-Core	To:RAN1, Cc:RAN4

Add “impacts”

Revised in R2-2405766

R2-2405766	LS on T390 	Samsung	LS out		LS out	Rel-18	IoT_NTN_enh-Core	To:RAN1, Cc:RAN4

Approved

R2-2405499	[S068] UE behaviours on T390 upon handover or CHO	Huawei, HiSilicon	discussion	Rel-18	IoT_NTN_enh-Core

Observation 1: In handover and CHO, target cell can know whether UE has started T390 based on the absence of gnss-ValidityDuration in the RRC connection reconfiguration complete message.

Proposal 1: Clarify in the spec that UE only includes gnss-ValidityDuration in the RRC connection reconfiguration complete message when it has a valid GNSS. There is no need to specify the stop of T390 upon handover or CHO.

-	Apple wonders if the UE can actually connect to the target cell in this case and this

Proposal 2: Clarify in the spec that T390 is stopped if UE autonomously starts GNSS measurements during C-DRX non-active periods.

Proposal 3: Adopt the TP in the Annex.

R2-2405440	On left open issues for T390 handling	Nokia, Nokia Shanghai Bell	discussion	Rel-18	IoT_NTN_enh-Core

- T390 behavior for HO/CHO

Observation 1: If the UE stops T390 when handing over to the target cell the UE will be regarded as uplink un-synchronized.

Observation 2: The target cell is not aware whether T390 is running in a UE performing handover to the cell.

Observation 3: For normal handover, the source cell can inform the UE’s T390 status to the target cell via inter-node message.

Observation 4: For conditional handover, the UE can report the UE’s T390 status to the target cell via Uu interface after CHO execution.

Proposal 1: For HO/CHO, the UE shall report the UE’s T390 status (e.g., the remaining value of the T390) to the target cell in the RRCConnectionReconfigurationComplete message.

Proposal 2: If Proposal1 is agreed, RAN2 to adopt the below text proposal for TS 36.331.

- T390 behavior for for C-DRX non-active periods

Observation 5: The UE and cell must have a common understanding of the T390 status to avoid the cell assumes an autonomous GNSS measurement has started or UE has moved to RRC Idle.

Proposal 3: The UE stops the T390 when it receives an acknowledgement of the cell receiving the new remaining GNSS validity duration, when the UE performed a successful GNSS measurement during a C-DRX non-active period.

Proposal 4: If Proposal3 is agreed, RAN2 to adopt the below text proposal for TS 36.331.

R2-2404157	Further Discussion on T390	vivo	discussion	Rel-18	IoT_NTN_enh-Core

Observation 1: There is no requirement that UE may need to acquire the valid GNSS position before HO or CHO, which is different than the RRC re-establishment case.

Observation 2: For HO or CHO, if T390 is stopped upon starting handover, the out-of-date GNSS position will be used to synchronize to the target cell, and UE does not have the opportunity to acquire a new valid GNSS position.

Observation 3: For HO or CHO, if T390 is not stopped upon starting handover, although the out-of-date GNSS position is used to synchronize to the target cell before T390 expiration, UE would have the opportunity to acquire a new valid GNSS position after T390 expiration.

Observation 4: Network implementation can ensure T390 not running and UE has a valid GNSS position during the handover procedure.

Proposal 1: T390 is not stopped upon starting HO or CHO (no spec changed).

R2-2404653	T390 handling during mobility [S067][S068]	Apple	discussion	Rel-18	IoT_NTN_enh-Core

Observation 1: If T390 expires when UE is performing GNSS acquisition, UE would go to RRC idle before RRC re-establishment is initiated.

Proposal 1: For RRC re-establishment procedure, T390 is stopped once GNSS acquisition is initiated.

Observation 2: In handover procedure, target cell cannot inherit network UL timing correction from source cell during T390.

Observation 3: When UE performs initial access to target cell, UE location is required for UE to perform pre-compensation on timing and frequency.

Proposal 2: RAN2 to down-select one of the two options on T390 handling during handover and CHO.

- Option 1: Network initiates GNSS measurement before handover and CHO to guarantee UE has a valid GNSS upon handover and CHO initiation.

- Option 2: UE should acquire GNSS location upon initiating handover and CHO if T390 is running.

Proposal 3: If Option 2 above is selected, T390 is stopped once GNSS acquisition is initiated during handover and CHO.

R2-2405526	Corrections on the conditions for stopping T390	Google Inc.	discussion	Rel-18	36.331

Proposal 1	UE stops T390 (if it is running) upon performing the GNSS measurement, no matter whether the GNSS measurement is triggered by the network, performed by the UE using an autonomous gap, or performed by the UE using available idle periods.

Proposal 2	Replace the changes relevant to the conditions for stopping T390 in R2-2403774 with the text proposal in Section 3.

Proposal 3	RAN2 to discuss the exceptions (e.g., a very short GNSS position fix duration) that allow UE to remain in RRC_CONNECTED when UE’s GNSS position becomes outdated, even if the UE is neither configured with autonomous GNSS measurement nor provided with an aperiodic GNSS measurement gap.

- RACH triggering during T390

R2-2404594	Discussion on RACH triggering during T390 running	OPPO	discussion	Rel-18	IoT_NTN_enh-Core

Neighbour Satellite information in MO

R2-2404896	Redundant ephemeris info and other issues in MO for IoT NTN	ZTE Corporation, Sanechips, Apple, Samsung	discussion	Rel-18	IoT_NTN_enh-Core

Proposal 1: To remove IE ephemerisInfo-r18 from MeasObjectEUTRA and keep the IE satelliteId-r18 as it is.

-	Ericsson does not agree on this, updated ephemeris are needed to allow the UE to continuously track the satellite. QC agrees. Nokia also agrees

-	HW supports this, and think Ericsson concerns would be applicable to other cases

-	CATT thinks we don’t need either ephemeris nor satellite ID

Make IE ephemerisInfo-r18 optional (without condition) in MeasObjectEUTRA (check the details in the CR review phase)

-	HW thinks we should apply this to NR as well

-	Ericsson thikns the paprameters should be optional for NR (rater than mandatory as today)

Check in the NR RRC CR review whether a similar change can be considered as well

Proposal 2: To clarify in the field description of IE epochTime-r18 in MeasObjectEUTRA that this IE is only assiocaited with IE referenceLocation-r18, not applicable to ephemeris information. And the previous description “configured in the associated ReportConfigEUTRA” for reference location is no longer correct and can be removed.

Continue in the CR review

Proposal 3: To clarify that the condition “OPTIONAL	-- Cond Moving” for the IE epochTime-r18 and referenceLocation-r18 in MeasObjectEUTRA means the corresponding field is only mandatory present if EventD2 or condEventD2 is configured. Otherwise the field is not present.

Continue in the CR review

Proposal 4: To clarify the description of Event D2 and CondEvent D2 that the referenceLocation's corresponding satellite ephemeris information is the one indicated by the satellite ID. Also to editorially align the description of Ml2 and Thresh2 with that of Event D2 and CondEvent D2.

Continue in the CR review

R2-2404592	Removing ephemeris from dedicated signaling for EMC neighbor cells	OPPO	CR	Rel-18	36.331	18.1.0	5017	-	F	IoT_NTN_enh-Core

Summary of change: Remove the ephemerisInfo-r18 from measObjectEUTRA.

R2-2404209	On the necessity of satellite assistance information for measurement in IoT NTN	CATT	discussion

Observation 1: Different from NR NTN, IoT NTN does not support SMTC based mechanism for RRM measurement; thus a UE does not need to adjust the measurement timing window with the variation of PDD. This makes it questionable on whether/how the satellite assistance information of neighbour cell is useful/necessary for RRM measurements in RRC_IDLE/INACTIVE or in RRC_CONNECTED.

Proposal 1: RAN2 clarifies how/why the satellite assistance information is necessary/useful for RRM measurement in IoT NTN. If no necessity is identified, remove the satellite ID list in SIB3, SIB5 and MeasObjectEUTRA.

[H005]

R2-2405498	[H005] Start of autonomous GNSS measurements	Huawei, HiSilicon	discussion	Rel-18	IoT_NTN_enh-Core

Proposal 1: Update the start condition of autonomous GNSS gap in clause 5.3.3.21 and 5.5.9: UE may re-acquire GNSS (when configured by the network) in the GNSS measurement timer, if eNB does not trigger UE to make GNSS measurement within duration T, where T is latest reported remaining GNSS validity duration plus UL transmission extension duration X (if any).

Proposal 2: Adopt the TP in the Annex.

-	Nokia thinks the current text is already sufficiently clear. Samsung agrees. ZTE also agrees

[H005] is rejected

Misc

Moved here from 7.6.4

R2-2404408	Remaining issues on GNSS operation in IoT NTN	CATT	discussion

Proposal 1: No need to reset the value of N_TA before resuming UL operation after GNSS is fixed.

Proposal 2: Add the start condition "when indication from lower layers to extend UL transmission is received" for T390 in Clause 7.3.1.

Proposal 3: Add the stop condition "when the UE starts GNSS measurement" for T318 in Clause 7.3.1.

Proposal 4: Adopt the TP in Appendix.

R2-2405528	Further RRC corrections for IoT NTN	Samsung	discussion	Rel-18	IoT_NTN_enh-Core

Observation 1: With current implementation UE cannot signal feature for both GSO and NGSO.

Proposal 1: Agree H005.

Proposal 2: Agree E805 and E806.

### 7.6.4	Other Stage 3 corrections

Corrections related to other specs, e.g. 36.321, 36.304, 36.306.

MAC issues

Moved here from 7.6.3

R2-2404686	Remaining issues on out-of-date GNSS fix	Qualcomm Incorporated	discussion	Rel-18	IoT_NTN_enh-Core

Proposal 1	UE resets the value of N_TA before resuming UL operation after GNSS is fixed. Text proposal is provided above.

Proposal 2	Add stop criteria for timer T390 as “upon receiving indication that new GNSS is valid”.

Proposal 3	If the UE receives GNSS measurement trigger too early (i.e., remaining GNSS validity is still x minutes), the UE is allowed not to trigger the GNSS measurement but trigger the GNSS Validity Duration Report. Text proposal is provided above.

R2-2404900	Remaining issues of MAC spec for IoT NTN	ZTE Corporation, Sanechips	discussion	Rel-18	IoT_NTN_enh-Core

Proposal 1: RAN2 assume the exiting way for maintaining N_TA after GNSS is fixed is enough, no enhancement is needed.

Proposal 2: It’s no need to enhance the process for the UE to report remaining GNSS Validity Duration after successful GNSS measurement during C-DRX.

Moved here from 7.6.3

R2-2405198	NTA handling after GNSS fix	NEC	discussion	Rel-18	IoT_NTN_enh-Core

Proposal: reset NTA to 0 after GNSS fix

R2-2405441	Further discussion on UE behaviour after successful GNSS acquisition	Nokia, Nokia Shanghai Bell	discussion	Rel-18	IoT_NTN_enh-Core

Observation 1: The value of N_TA is not clear after a UE has completed the GNSS measurement successfully.

Observation 2: If the UE transmits on PUSCH resources to the network after a GNSS measurement, the following PUSCH transmissions may cause interference to other UEs until the NW corrects any potential TA errors after sending multiple Timing Advance Commands MAC CE.

Observation 3: The network can correct large TA errors via the Random Access Response message.

Proposal 1: After a successful GNSS measurement /autonomous GNSS measurement, the UE shall  perform the Random Access procedure for UL synchronization using N_TA = 0.

Proposal 2: Up to UE implementation to decide when to report the GNSS Validity Duration Report MAC CE after a successful GNSS measurement during C-DRX inactive time.

Proposal 3: Add a Note to indicate when the UE should report the new remaining GNSS validity duration to NW after a successful GNSS measurement in C-DRX inactive time.

Proposal 4: If Proposal 3 is agreed, adopt the Note to TS 36.331.

R2-2405451	R18 IoT NTN GNSS extension	Ericsson	discussion	Rel-17	IoT_NTN_enh-Core

Observation 1	The UE shall trigger GNSS measurement reporting every time upon completing GNSS fix operation.

Observation 2	The GNSS Duration Report MAC CE shall not trigger SR; instead CBRA shall be used.

Observation 3	If the UE has a grant for new transmission, it will send the GNSS validity duration report MAC CE on that grant.

Observation 4	The current MAC spec does not always trigger CBRA to report the GNSS validity duration.

Proposal 1	Correct the MAC procedure to make it clear that a triggered GNSS validity duration report always trigger CBRA in line with the RAN2 agreements.

Continue in offline [303]

Proposal 2	Consider the MAC text proposal in section 3.1

Observation 5	The network use NTA to compensate for errors due to estimation of UE and satellite’s position/common TA.

Observation 6	Setting NTA=0 after GNSS reacquisition is not optimal as the NTA value, configured by the network, may compensated for more than the UE position error.

Observation 7	Alt A, C and D do not provide optimal performance.

Observation 8	The UE can calculate the timing error due to inaccurate UE position by comparing the values of N"TA,adjUE"  based on its previous GNSS position and its new GNSS position after GNSS reacquisition, i.e., Terror_UE_position=N"TA,adjUE"-N"TA,adj[OLD]UE" .

Proposal 3	After a new GNSS position fix, set NTA=NTA_old-Terror_UE_position where Terror_UE_position=N"TA,adj UE"-N"TA,adj[OLD]UE"  is the timing error due to inaccurate UE position and is calculated from N"TA,adj [OLD]UE"  based on the previous GNSS position and N"TA,adjUE"  based on the new GNSS position.

Proposal 4	Consider the MAC text proposal in section 3.2.

[AT126][303][IoT NTN Enh] MAC issues (Ericsson)

Scope: Discuss the N_TA issue and p1 from R2-2405451(and other similar proposals)

Intended outcome: report of offline discussion

Deadline for companies' feedback:  Wednesday 2024-05-22 20:00 (but F2F discussion is invited)

Deadline for rapporteur's summary (in R2-2405761):  Thursday 2024-05-23 08:00

R2-2405761	Summary of [AT116][303][IoT NTN Enh] MAC issues	Ericsson	discussion	Rel-18	IoT_NTN_enh-Core

Observation 1	The first proposal implies the UE do not need to drop configured UL and DL resources, and the UE must initiate CBRA to reacquire uplink time alignment.

Observation 2	In RAN1 spec, UE always use N_TA equal to zero when doing CBRA.

Observation 3	When timeAlignmentTimer is not running, the UE will always apply the Timing Advance Command received in RAR.

Observation 4	Besides the initiation of Random Access, this method to trigger the inclusion of a MAC CE is exactly how PHR MAC CEs and TAR MAC CEs are included for transmission.

Proposal 1	When the MAC entity receives a trigger to send the GNSS validity duration report, the UE shall stop the timeAlignmentTimer of the pTAG

-	ZTE wonders if we need a new trigger/MAC CE? Ericsson thinks there is no new trigger/MAC CE.

Agreed

Proposal 2	In MAC 5.4.10, add a NOTE that during inactive time of C-DRX, it is up to UE implementation when to stop timeAlignmentTimer and initiate CBRA

In MAC 5.4.10, add a NOTE that during inactive time of C-DRX, it is up to UE implementation whether to stop timeAlignmentTimer and initiate CBRA

Proposal 3	Consider this MAC text proposal for 5.4.10:

-	HW wonders if we need the remaining text in the subclause. Ericsson we need this. Oppo/Vivo agree

Take TP in R2-2405761 as a basis, moving the Note up (and reflecting updated p2)

R2-2405454	IoT NTN MAC corrections	Ericsson	CR	Rel-18	36.321	18.1.0	1586	-	F	IoT_NTN_enh-Core

[E805][E806]

R2-2404843	[E805][E806] Revert implementation of UE capability for HARQ and GNSS	Ericsson	discussion	Rel-18	NR_NTN_enh-Core

Observation 1	RIL [H002] proposed to modify the capability for HARQ and GNSS to distinguish between GSO and NGSO cases.

Observation 2	In RAN2#125, RAN2 swiftly agreed to implement the change with the only rationale to align with Rel-17 design and latest agreements reflected in the “UE Feature list”,, without due consideration.

Observation 3	RAN2’s decision to modify the UE capability for HARQ and GNSS contradicts RAN1 consensus without technical justification nor appropriate communication.

Proposal 1	Revert the implementation of [H002] and send an LS to RAN1 to communicate this correction.

-	QC thinks we should stick to what we have. HW agrees, thinks that we already made a decision. Nordic also agrees. Oppo also agree

-	Nokia shares Ericsson view

-	CATT thinks there is currently a misalignment and if we don’t align we need to inform RAN1

-	Ericsson thinks that RAN1 already decided not to change the implementation

-	QC thinks we should send the LS

-	MTK prefers not to change.

-	QC thinks a possible solution is to indicate that if the feature is supported, support for GSO is mandatory with capability signalling (IODT bit)

-	MTK and HW prefers not to change

We modify the field description in 36.306 to indicate that if the feature (HARQ / GNSS enhancements) is supported, support for GSO is mandatory with capability signalling (IODT bit)

[AT126][308][IoT NTN Enh] UE capability for HARQ and GNSS (Ericsson)

Scope: discuss offline and, if needed, draft an LS to RAN1

Intended outcome: draft LS (if needed)

Deadline for rapporteur's summary (in R2-2405765):  Friday 2024-05-24 08:00

R2-2405765	LS on UE capability for HARQ and GNSS	Ericsson	LS out		LS out	Rel-18	IoT_NTN_enh-Core	To:RAN1

Withdrawn

Withdrawn

R2-2405530	Miscellaneous correction for IoT-NTN	Nokia, Nokia Shanghai Bell	CR	Rel-18	36.304	18.1.0	0875	-	F	IoT_NTN_enh-Core	Withdrawn

## 7.7	NR NTN enhancements

(NR_NTN_enh -Core; leading WG: RAN1; REL-18; WID: RP-232669)

Time budget: 0 TU

Tdoc Limitation: 2 tdocs

### 7.7.1	Organizational

LSs, rapporteur inputs and other organizational documents.

Editorials/clarifications should not be included in any tdoc but sent to the WI spec rapporteurs, who can submit a rapporteur CR as part of this AI.

Rapporteur inputs and other pre-assigned documents in this AI do not count towards the tdoc limitation.

Incoming LSs

R2-2404127	LS on UE capability for NW verified location (R4-2406496; contact: Huawei)	RAN4	LS in	Rel-18	NR_NTN_enh-Core	To:RAN1, RAN2

-	ZTE thinks that RAN1 is still discussing based on this LS and then we could wait before introducing changes

Noted

R2-2404248	Correction on NR NTN	CATT (Rapporteur)	CR	Rel-18	37.355	18.1.0	0504	-	F	NR_NTN_enh-Core

Intention for change 1 and 3 is confirmed. Continue the discussion on the CR implementation in the [Post126] CR review

Revised in R2-2405768

[Post126][303][NR-NTN Enh] 37.355 CR (CATT)

Scope: update the 37.355 CR with meeting agreements

Intended outcome: Agreed CR

Deadline for agreed CR (in R2-2405768): short

R2-2405768	Correction on NR NTN	CATT (Rapporteur)	CR	Rel-18	37.355	18.1.0	0504	1	F	NR_NTN_enh-Core

=> Agreed

R2-2404146	LS reply on OAM requirements for UE location verification (S5-242198; contact: CATT)	SA5	LS in	Rel-18	OAM_NTN	To:RAN3	Cc:SA2, RAN1, RAN2

Noted

Rapporteur CRs

Moved here from 7.7.4

R2-2404839	Proposed conclusions for NR NTN RILs Rel-18	Ericsson	report	Rel-18	NR_NTN_enh-Core

E302, H010 and H115 are still ToDo and need to be finalized at this meeting

S268 is Rejected

S487 is Agreed

R2-2404855	Miscellaneous corrections to Rel-18 NR NTN	Ericsson	CR	Rel-18	38.331	18.1.0	4761	2	F	NR_NTN_enh-Core	R2-2403772	Revised

This IPA CR is revised in R2-2404863 to include further changes

R2-2404863	Miscellaneous corrections to Rel-18 NR NTN	Ericsson	CR	Rel-18	38.331	18.1.0	4761	3	F	NR_NTN_enh-Core	R2-2404855

Revised in R2-2405753 to include other possible agreements this week (and to update the coversheet). Also include R18-specific changes for terminology alignment from R2-2405715

[Post126][301][NR-NTN Enh] 38.331 CR (Ericsson)

Scope: update the RRC CR with meeting agreements

Intended outcome: Agreed CR

Deadline for agreed CR (in R2-2405753): short

R2-2405753	Miscellaneous corrections to Rel-18 NR NTN	Ericsson	CR	Rel-18	38.331	18.1.0	4761	4	F	NR_NTN_enh-Core	R2-2404855

=> Agreed

R2-2405374	Corrections for Non-terrestrial Networks	InterDigital	CR	Rel-18	38.321	18.1.0	1858	-	F	NR_NTN_enh-Core

-	IDC thinks that a clarification or restructuring of the change would be helpful

Revised in R2-2405754

R2-2405754	Corrections for Non-terrestrial Networks	InterDigital	CR	Rel-18	38.321	18.1.0	1858	1	F	NR_NTN_enh-Core

-	QC thinks we need to update section 5.33 to clarify the PDCCH monitoring according to DRX

Revised in R2-2406103

[Post126][302][NR-NTN Enh] 38.321 CR (Interdigital)

Scope: update the MAC CR with meeting agreements

Intended outcome: Agreed CR

Deadline for agreed CR (in R2-2406103): short

R2-2406103	Corrections for Non-terrestrial Networks	InterDigital	CR	Rel-18	38.321	18.1.0	1858	1	F	NR_NTN_enh-Core

=> Agreed

R2-2404856	Terminology alignment in 38.304 for NR-NTN	ZTE Corporation, Sanechips, Ericsson, CATT, Nokia, Intel, Huawei, HiSilicon	CR	Rel-18	38.304	18.1.0	0403	-	F	NR_NTN_enh-Core	Revised

Revised in R2-2405623

R2-2405623	Terminology alignment in 38.304 for NR-NTN	ZTE Corporation, Sanechips, Ericsson, CATT, Nokia, Intel, Huawei, HiSilicon	CR	Rel-18	38.304	18.1.0	0403	1	F	NR_NTN_enh-Core	R2-2404856

-	Samsung thinks there are a few other places where “system” needs to be changed to “cell”

Revised in R2-2405755

R2-2405755	Terminology alignment in 38.304 for NR-NTN	ZTE Corporation, Sanechips, Ericsson, CATT, Nokia, Intel, Huawei, HiSilicon	CR	Rel-18	38.304	18.1.0	0403	2	F	NR_NTN_enh-Core

Agreed

R2-2404526	Add FR2 band reference to NTN related UE Capabilities	Intel Corporation	draftCR	Rel-18	38.306	18.1.0	NR_NTN_Ph3-Core

-	CATT thinks that further changes might be added to reflect the latest decision in RAN1 (related to FG 44-1. ZTE thinks no update is needed for this

Change is endorsed

Also include changes related to UE capabilities agreements in this meeting

Revised in R2-2406037

[Post126][304][NR-NTN Enh] UE Caps CRs (Intel)

Scope: update CRs with meeting agreements

Intended outcome: Endorsed CRs

Deadline for CRs (in R2-2406037 / R2-2406038): very short

R2-2406037	Corrections on FR2 band support for NR NTN related UE Capabilities	Intel Corporation	draftCR	Rel-18	38.306	18.1.0	NR_NTN_Ph3-Core

=> Endorsed and to be merged in the mega CR.

R2-2406038	Corrections on FR2 band support for NR NTN related UE Capabilities	Intel Corporation	draftCR	Rel-18	38.331	18.1.0	NR_NTN_Ph3-Core

=> Endorsed and to be merged in the mega CR.

R2-2405455	Terminology alignment for NR NTN	Ericsson, ZTE Corporation, Sanechips, Intel Corporation, CATT	CR	Rel-17	38.331	17.8.0	4836	-	F	NR_NTN_solutions-Core

Revised in R2-2405714

R2-2405714	Terminology alignment for NR NTN	Ericsson, ZTE Corporation, Sanechips, Intel Corporation, CATT, Huawei, HiSilicon, Nokia, Nokia Shanghai Bell	CR	Rel-17	38.331	17.8.0	4836	1	F	NR_NTN_solutions-Core

To be handled in AI 6.1.3.1

R2-2405456	Terminology alignment for NR NTN	Ericsson, ZTE Corporation, Sanechips, Intel Corporation, CATT	CR	Rel-18	38.331	18.1.0	4837	-	F	NR_NTN_enh-Core

Revised in R2-2405715

R2-2405715	Terminology alignment for NR NTN	Ericsson, ZTE Corporation, Sanechips, Intel Corporation, CATT, Huawei, HiSilicon, Nokia, Nokia Shanghai Bell	CR	Rel-18	38.331	18.1.0	4837	1	F	NR_NTN_enh-Core

Revised in R2-2405751 to make it a Cat A mirror CR for R2-2405714 (R18-specific changes to be moved to the R18 RRC Rapporteur CR)

R2-2405751	Terminology alignment for NR NTN	Ericsson, ZTE Corporation, Sanechips, Intel Corporation, CATT, Huawei, HiSilicon, Nokia, Nokia Shanghai Bell	CR	Rel-18	38.331	18.1.0	4837	2	A	NR_NTN_enh-Core

To be handled in AI 6.1.3.1

### 7.7.2	Stage 2 corrections

R2-2405243	Stage-2 corrections on NR NTN	THALES	CR	Rel-18	38.300	18.1.0	0858	1	F	NR_NTN_enh-Core	R2-2403773

-	Nokia thinks there is a comment that should be removed and the coversheet should be updated

-	Ericsson thinks that kmac changes should be done in Rel-17

Revised in R2-2405756

R2-2405756	Stage-2 corrections on NR NTN	THALES	CR	Rel-18	38.300	18.1.0	0858	2	F	NR_NTN_enh-Core	R2-2403773

Agreed

=> Coversheet update by MCC: wrong WI code.

=> Revised in R2-2406115

R2-2406115	Stage-2 corrections on NR NTN	THALES	CR	Rel-18	38.300	18.1.0	0858	3	F	NR_NTN_enh-Core

=> Agreed

R2-2404890	Correction on Location-based CHO	OPPO	CR	Rel-18	38.300	18.1.0	0863	-	F	NR_NTN_enh-Core

-	QC and HW support the change and think there would be no need to update Stage 3

-	Ericsson does not agree

Come back in the next meeting

### 7.7.3	RRC corrections

Rapporteur’s suggestions

R2-2404844	Comments on remaining NR NTN RILs	Ericsson	discussion	Rel-18	NR_NTN_enh-Core

Proposal 1	Accept E302 to align NR NTN and IoT NTN specification.

Proposal 2	Reject [H115]. UE re-acquires system information upon satellite switch with resynchronization.

Proposal 3	If P2 is not agreed, it is up to network implementation to secure that value tags are set consistently across cells upon satellite switch with resynchronization.

RAN2 understands that upon satellite switch with resynchronization, legacy behaviour applies for SIB update (apart from SIB19), i.e. it is up to network implementation to secure that value tags are properly set (no spec change)

Proposal 4	Reject H010

H010 is Rejected

Satellite switch with resync

- [H115]

R2-2405501	[H115] SIB19 acquisition after satellite switching	Huawei, HiSilicon	discussion	Rel-18	NR_NTN_enh-Core

Proposal 1: Clarify in clause 5.2.2.2.1 that only SIB19 is re-acquired after satellite switching with re-sync. Add the UE behaviour of obtaining SIB19 in clause 5.2.2.4.21.

-	Ericsson wonders whether we need to clarify this in the spec or leave it to implementation

-	Ericsson suggests to make changes in 5.7.19

-	vivo wonders about MIB

Clarify that the UE shall re-acquire SIB19 after satellite switching with re-sync, leaving the exact time up to UE implementation (without referring to the reacquisition of other SIBs). Also remove current text in 5.2.2.2.1 mandating the UE to reacquire all SIBs. (can check further details in the CR review)

Proposal 2: Adopt the TP for [H115] in the Annex.

-	Apple could be ok with the change in 5.2.2.2.1 only. HW is also fine

R2-2404685	SIB acquisition after Satellite switch with re-sync	Qualcomm Incorporated	discussion	Rel-18	NR_NTN_enh-Core

Proposal 1	Unless the validity duration expires or UE receives paging notification of SI change, there is no need for UE to re-acquire the MIB, SIB1, SIB19 and other SIBs after satellite switch with resync.

-	HW thinks SIB19 reacquisition is for neighbour satellite ephemeris

-	QC thinks we don’t have to say that the UE has to acquire it immediately

-	Ericsson wonders about SMTC updates. QC thinks that for UEs in idle mode is up to UE and for UEs in connected they can be reconfigured after switch

R2-2405169	Discussion on E302, H115	Samsung	discussion	Rel-18	NR_NTN_enh-Core

- [H010] related

R2-2405672	[H010][H115] and skipping MIB acquisition	Sequans Communications	discussion	Rel-18	NR_NTN_enh-Core

Proposal 1: If time offset is not clear enough, consider adding “with the same index”, or using “SSB half-frame” or “SSB burst”

-	CATT thinks that attempts to clarify this were already rejected and should not be further considered

We don’t attempt to clarify this further

Observation 1: During satellite switch with resync, cell is unchanged hence SI acquisition should not be required

Observation 2: If broadcasted SMTCs needs to be updated at satellite switch, it means configured SMTCs shall also be updated, killing the benefit of the feature

Proposal 2: RAN2 to discuss if any fields require updating, and why (particularly SMTCs)

Continue the discussion in [301]

Observation 3: Omitting PBCH acquisition can reduce interruption time

Observation 4: It is beneficial for the UE to know if cell/SSB timing is unchanged (at gNB)

Proposal 3: Capture that in hard switch (t-serviceStart absent), cell timing/SSB location is unchanged (at gNB)

RAN2 understands that in hard switch (t-serviceStart absent), cell timing/SSB location is unchanged (at gNB)

R2-2405680	Soft satellite switch SSB time offset and SMTC impact	Sequans Communications	discussion	Rel-18	NR_NTN_enh-Core

Observation 1: SMTC broadcasted/configured in neighbor cells for inbound mobility to an “unchanged PCI cell” may become erroneous at each satellite switch

Observation 2: SMTC broadcasted/configured in serving cells for outbound mobility from an “unchanged PCI cell” may become erroneous at each satellite switch

Observation 3: Synchronous “Shifted cell timing” implementation might alleviate the issue with additional changes, but may be only applicable to same satellite orbit cells

Observation 4: Synchronous “Shifted SSB location” implementation might alleviate the issue with additional changes, but may be only applicable to same satellite orbit cells

Proposal 1: Consider soft switch where SSB-timeOffset is applied only between t-serviceStart and t-service

-	QC thinks there is no issue and we should not make further optimizations now

Continue the discussion in [301]

[AT126][301][NR NTN Enh] SMTC impacts for soft satellite switch (Sequans)

Scope: discuss the impact on SMTC adjustment for soft satellite switch and possible impact on decision for H115

Intended outcome: report of offline discussion

Deadline for companies' feedback:  Wednesday 2024-05-22 20:00 (but F2F discussion is invited)

Deadline for rapporteur's summary (in R2-2405757):  Thursday 2024-05-23 08:00

R2-2405757	Summary of [AT116][301][NR NTN Enh] SMTC impacts for soft satellite switch	Sequans Communications	discussion	Rel-18	NR_NTN_enh-Core

Noted

- Other

R2-2404158	Further Discussion on Satellite Switch with Resync	vivo	discussion	Rel-18	NR_NTN_enh-Core

Proposal 1: Change the granularity of capabilities softSatelliteSwitchResyncNTN-r18 and hardSatelliteSwitchResyncNTN-r18 as per band level.

Can come back in the next meeting

Proposal 2: RAN2 confirms that parallelMeasurementWithoutRestriction-r17 and/or simultaneousRxDataSSB-DiffNumerology is not the pre-requisite capability of softSatelliteSwitchResyncNTN-r18.

R2-2405143	On Reducing the NW Uncertainty in Satellite Switching with Resynchronization	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_NTN_enh-Core

R2-2405273	Discussion on beam management for satellite switch with re-sync	ETRI	discussion	Rel-18	NR_NTN_enh-Core

R2-2405423	Discussion on satellite switch with resync in RRC states other than RRC connected	ASUSTeK	discussion	Rel-18	NR_NTN_enh-Core

(Cond)Event D2

R2-2405079	Clarification on (cond)event D2 configuration	ZTE Corporation, Sanechips	discussion	Rel-18	NR_NTN_enh-Core

Proposal 1: It is proposed to reject second changes ( i.e., optionally present part) proposed in RIL 302.

-	QC agrees with the analysis of how this was implemented so far but sees the benefit of leaving this optional

-	Nokia supports Ericsson and QC views

-	Samsung is ok with E302 but not as an alignement to IoT NTN

E302 is agreed

R2-2404208	Corrections on Event D2 measurement reporting	CATT	discussion

Observation 1: Current Spec does not specific how to derive the applicable cells for Event D2. This leads to the issue that when UE initiates measurement reporting procedure for Event D2, it can report nothing (including RRM measurement and PCI) about the neighbour cell that fulfils the reporting conditions.

Observation 2: According to the current ASN.1 structure of MeasObjectNR, one measID can be associated with multiple neighbour cells each of which is further associated with one reference location. When receiving measurement reporting from UE, the NW cannot know which cell fulfils the reporting condition due to Observation 1.

Observation 3: For Event D2, UE fulfils either the entering condition or leaving condition will initiate measurement reporting procedure. According to the current measurement reporting mechanism, the NW could not know whether the UE meets entering condition or leaving condition due to Observation 1.

Proposal 1: The measurement results reporting for event D2 should include the physCellId of the neighbour cells that fulfils the reporting conditions.

-	CATT thinks we should solve the problem without reverting previous decisions

-	CATT thinks that for D1 we don’t have the same problem

-	Ericsson doesn’t see a problem with this and we could treat it as D1. QC agrees

-	CATT thinks the issue is related to which cell is reported to the NW

-	Samsung thinks this is common problem for D1 and D2

P1 is not pursued

Proposal 2: One cellsTriggeredList should be kept for the measID which is associated with event D2:

-	When the entering condition is fulfilled for one neighbour cell of the event D2, the UE includes this cell to the cellsTriggeredList for this measID and initiates measurement reporting procedure.

-	When the leaving condition is fulfilled for one cell included in the cellsTriggeredList which is associated with event D2, the UE removes this cell from the cellsTriggeredList for this measID and initiates measurement report procedure.

Proposal 3: If Proposal 1 and Proposal 2 are agreed, adopt the TP in Annex.

Proposal 4: if Proposal 1 and Proposal 2 are agreed, RAN2 confirms the similar issues exists also in TS 36.331 and agrees to apply a similar way of change as in P1/2 in TS 36.331 as well.

FR2 capability

R2-2405500	Correction on NR NTN FR2 capabilities	Huawei, HiSilicon	CR	Rel-18	38.331	18.1.0	4841	-	F	NR_NTN_enh-Core

Changes are endorsed and included in the rapporteur CR

Other

R2-2404673	Open issues on NR NTN enhancements	Apple	discussion	Rel-18	NR_NTN_enh-Core

R2-2404883	Discussion on the leftover issues for NR-NTN	Google Inc.	discussion	Rel-18

### 7.7.4	Other Stage 3 corrections

Corrections related to other specs, e.g. 38.321, 38.304, 38.306.

R2-2405080	Consideration on remaining FR2 UE capability issues	ZTE Corporation, Sanechips, Eutelsat Group	discussion	Rel-18	NR_NTN_enh-Core

Proposal 1: Update TS 38306 to allow indicate below UE capabilities for NTN FDD-FR2:

- SDT related: cg-SDT-r17, mt-CG-SDT-r18

- MBS related: maxDynamicSlotRepetitionForSPS-Multicast-r17, maxNumberG-CS-RNTI-r17, maxNumberG-RNTI-r17, multiPUCCH-HARQ-ACK-ForMulticastUnicast-r17, priorityIndicatorInDCI-Multicast-r17, priorityIndicatorInDCI-SPS-Multicast-r17, releaseSPS-MulticastWithCS-RNTI-r17, sps-MulticastMultiConfig-r17, re-LevelRateMatchingForMulticast-r17, twoHARQ-ACK-CodebookForUnicastAndMulticast-r17

-	vivo thinks something is needed for some capabilities, like RACH-less. ZTE thinks this is handled in the main session. CATT thinks RACH-less is per band so nothing else is needed

-	HW wonders about CHO.

Agreed (can further check the need to update other capabilities)

Proposal 2: If P1 is agreed, RAN2 consider the corresponding TP in annex for CR implementation.

R2-2405170	Discussion on TAT handling in RACH-less CHO	Samsung	discussion	Rel-18	NR_NTN_enh-Core

Proposal 1: For the start of TAT in RACH-less CHO, UE follows the procedure for RACH-less HO (i.e., no specification change).

Moved here from 7.0.4

R2-2404666	Remaining issues on RACH-less HO	Apple	discussion	Rel-18	NR_mobile_IAB-Core, NR_Mob_enh2-Core, NR_NTN_enh-Core, TEI18

Proposal 3: When UE initiates NTN RACH-less HO, UE is required to acquire the SFN timing information of target PCell and apply the DRX configuration.

-	vivo agrees with the intention but thinks we could keep the behaviour aligned for all the cases, so no change

-	IDC also supports the principle of this CR

-	QC thinks there is no need to change anything

-	HW is ok to keep the current description but have a different understanding on the behaviour

-	Nokia thinks there is no need to change

RAN2 confirms that during an NTN RACH-less HO DRX applies. Can check in the NR RRC CR review whether any clarification is needed

R2-2405118	Correction on HARQ buffer flush during satellite switch with re-synchronization	Huawei, HiSilicon	CR	Rel-18	38.321	18.1.0	1854	-	F	NR_NTN_enh-Core

R2-2405142	On Scheduling Restrictions in Satellite Soft Switching with Resynchronization – further view	Nokia	discussion	Rel-18	NR_NTN_enh-Core	R2-2403300

Proposal 1: In case the UE supports softSatelliteSwitchResyncNTN-r18, the UE shall also support parallelMeasurementWithoutRestriction.

Proposal 2: RAN2 is asked to reconsider the decision made at RAN2#125 regarding PDD during satellite switching.

Proposal 3: In case the UE supports softSatelliteSwitchResyncNTN-r18, the UE shall also support serviceLinkPropDelayDiffReporting-r17.

Proposal 4: If the decision regarding PDD is revised, confirm with RAN4 that service link propagation delay difference (PDD) helps in reducing the scheduling restriction duration during satellite soft-switching with resynchronization.

Proposal 5: If the LS to RAN4 is sent, RAN2 asks how early such measurements, calculations and reporting should be done.

Proposal 6: Adopt the 38.306 and 38.331 Text Proposals in the Annex A and B.

R2-2405424	Discussion on TA timer for satellite switch	ASUSTeK	discussion	Rel-18	38.321	NR_NTN_enh-Core

Proposal 1: RAN2 to discuss whether the TA timer is started or restarted upon satellite switch with resynchronization.

## 7.8	NR support for UAV

(NR_UAV -Core; leading WG: RAN2; REL-18; WID: RP-230782 and LTE WID: RP-230783 )

Time budget: 0 TU

Tdoc Limitation: 1

### 7.8.1	Organizational

Editorials/clarifications should not be included in any tdoc but sent to the WI spec rapporteurs, who can submit a rapporteur CR as part of this AI.

CR rapporteurs are asked to continue maintaining an open issues list reflecting known issues to be handled during the maintenance phase

Rapporteur CRs - NR

R2-2404280	Corrections for NR Support for UAV (Uncrewed Aerial Vehicles)	Qualcomm Incorporated	CR	Rel-18	38.331	18.1.0	4693	2	F	NR_UAV-Core	R2-2403956

=>	The CR is endorsed will be used as a baseline for any further updates

=> Revised in R2-2406043

[POST126][006][UAV] CR to 38.331 and 36.331 (Qualcomm)

Intended outcome: Agree to CRs and RIL list

Deadline:  short

R2-2406043	Corrections for NR Support for UAV (Uncrewed Aerial Vehicles)	Qualcomm Incorporated	CR	Rel-18	38.331	18.1.0	4693	3	F	NR_UAV-Core

=> Agreed

R2-2406044	Corrections to Enhanced LTE Support for UAV (Uncrewed Aerial Vehicles)	Qualcomm Incorporated, Nokia	CR	Rel-18	36.331	18.1.0	5004	3	F	LTE_UAV_enh-Core

=> Agreed

R2-2406045	LTE UAV: Status of ASN.1 review issues after RAN2#126	Qualcomm Incorporated	discussion	Rel-18	NR_UAV-Core

=> Noted

R2-2406046	NR UAV: Status of ASN.1 review issues after RAN2#126	Qualcomm Incorporated	discussion	Rel-18	LTE_UAV_enh-Core

=> Noted

R2-2405139	Clarifications for NR UAV capabilities	Huawei, HiSilicon, CATT	draftCR	Rel-18	38.306	18.1.0	F	NR_UAV-Core

=>	Need to include the change for TransmissionMode1 similar to openloop

=>	the CR is endorsed in R2-2405925 with the change above and will be merged with mega CR

R2-2405925	Clarifications for NR UAV capabilities	Huawei, HiSilicon, CATT	draftCR	Rel-18	38.306	18.1.0	F	NR_UAV-Core

=> Merged to the mega CR

R2-2405145	Miscellaneous UAV-related Stage-2 Corrections	Nokia, Ericsson, Qualcomm	CR	Rel-18	38.300	18.1.0	0867	-	F	NR_UAV-Core

=>	the CR is agreed

R2-2405531	Correction on resource pool selection for A2X communication	Samsung, Sharp	CR	Rel-18	38.321	18.1.0	1813	2	F	NR_UAV-Core	R2-2403957

=>	the CR will be revised with the agreement from this meeting and approved by email by Friday this week

[AT126][005][UAV] CR to 321 (Samsung)

Intended outcome: agree to CR

Deadline:  05-24-24

R2-2405998	Correction on resource pool selection for A2X communication	Samsung, Sharp	CR	Rel-18	38.321	18.1.0	1813	3	F	NR_UAV-Core

=>	The CR is agreed

Rapporteur CRs - LTE

R2-2404281	Corrections to Enhanced LTE Support for UAV (Uncrewed Aerial Vehicles)	Qualcomm Incorporated, Nokia	CR	Rel-18	36.331	18.1.0	5004	2	F	LTE_UAV_enh-Core	R2-2404033

=>	The CR is revised with changes agreed in main ASN.1 session and will be reviewed/agreed by email

=> Revised in R2-2406044

R2-2405131	Capabilities for Rel-18 Enhanced LTE Support for UAV WI	Huawei, HiSilicon, CATT	CR	Rel-18	36.306	18.1.0	1884	1	F	LTE_UAV_enh-Core	R2-2402495

=>	The CR is revised in R2-2405972

R2-2405972	Capabilities for Rel-18 Enhanced LTE Support for UAV WI	Huawei, HiSilicon, CATT	CR	Rel-18	36.306	18.1.0	1884	2	F	LTE_UAV_enh-Core

=>	The CR is agreed

Other CRs

R2-2405297	A2X services that leverage V2X mechanisms	Qualcomm Incorporated	draftCR	Rel-18	37.985	18.0.0	F	NR_UAV-Core, LTE_UAV_enh-Core

-	Nokia is not sure this is needed as we didn’t do any enhancements to SL.  Ericsson agrees with Nokia, it is not necessary and it is not leveraging A2X mechanism as it has it’s own layer.  Samsung also thinks we didn’t do any enhancements to SL. CATT is ok with the TR but points out that this TR was not in the list of impacted spec.

=>	Not pursued

R2-2405971	Capability for UAV NS values	Ericsson, Qualcomm Inc.	draftCR	Rel-18	38.306	18.1.0	F	NR_UAV-Core

=>	The CR is endorsed and will be merged in mega CR

### 7.8.2	RRC RIL

### 7.8.3	Other

Other critical corrections

R2-2405134	Implicit altitude-based NumberOfTriggeringCells	Huawei, HiSilicon	discussion	Rel-18	NR_UAV-Core

Proposal 1: RAN2 to confirm that the altitude-based NumberOfTriggeringCells can be implicitly implemented by configuring NumberOfTriggeringCells and different altitude thresholds in event AxHy.

-	Nokia explains that this true but there is nothing to agree.

Proposal 2: Altitude-based NumberOfTriggeringCells should be captured in stage 2 spec, i.e., TS 38.300 because it was a new feature introduced for NR UAV.

-	LG and Nokia thinks that this is clear in 38.331. Vodafone thinks that it should be clear in stage2 what can be found in stage 3. Ericsson also thinks that it is fair to describe in stage 2 how to make this feature work. Nokia thinks it is clear and it can be inferred from the description.  Ericsson thinks that this was a specific agreement. Samsung also thinks that this is clear.

=>	Noted

R2-2405461	discussion on SL LCP for A2X	ZTE Corporation, Sanechips	discussion	Rel-18	NR_UAV-Core

=>	 exclude the Destination associated to A2X communication if the new transmission is not associated to a sidelink grant in A2X dedicated resource pool when the UE is configured with A2X dedicated resource pool in SL LCP procedure, and adopt the following TP for TS38.321.

=>	Noted

## 7.9	Enhanced NR Sidelink Relay

(NR_SL_relay_enh-Core; leading WG: RAN2; REL-18; WID: RP-223501)

Time budget: 0TU

Tdoc Limitation: 3 tdocs

### 7.9.1	Organizational

Including incoming LSs and rapporteur inputs. CR rapporteurs are asked to continue maintaining an open issues list reflecting known issues to be handled during the maintenance phase.

Incoming LS with “take into account” action and no related document

R2-2404134	Reply LS on L2ID and user info for L2 based U2U (S2-2405379; contact: OPPO)	SA2	LS in	Rel-18	NR_SL_relay_enh-Core	To:RAN2, CT1

- Noted

Incoming LS with “take into account” action and related document

R2-2404136	Reply LS on U2U relay selection (S2-2405531; contact: OPPO)	SA2	LS in	Rel-18	NR_SL_relay_enh-Core	To:RAN2	Cc:CT1

R2-2404251	Discussion on LS S2-2405531	OPPO	discussion	Rel-18	NR_SL_relay_enh-Core

Proposal 1	RAN2 not pursue specification effort regarding the reply LS from SA2 in S2-2405531.

Discussion:

Nokia agree with the proposal and think the LS does not require us to change anything.

Agreement:

RAN2 will not pursue specification effort regarding the reply LS from SA2 in S2-2405531.

Rapporteur CRs (endorsed after RAN2#125bis)

R2-2405367	RRC corrections for Rel-18 SL relay enhancements	Huawei, HiSilicon, Ericsson, ZTE, CATT, Sharp, Lenovo, OPPO, Nokia, Apple, MediaTek, Xiaomi, Samsung, ASUSTeK	CR	Rel-18	38.331	18.1.0	4684	2	F	NR_SL_relay_enh-Core	R2-2403813

=> Revised in R2-2405877

R2-2405532	Correction for SL Relay UE capability	Samsung	CR	Rel-18	38.331	18.1.0	4774	1	F	NR_SL_relay_enh-Core	R2-2403975

=> Revised in R2-2405878

R2-2405533	Correction for SL Relay UE capability	Samsung	CR	Rel-18	38.306	18.1.0	1091	1	F	NR_SL_relay_enh-Core	R2-2403976

=> Revised in R2-2405879

[Post126][401][Relay] Rel-18 relay RRC CR (Huawei)

Scope: Update the CR in R2-2405367 in line with decisions of this meeting.

Intended outcome: Agreed CR in R2-2405877

Deadline:  Short (for RP)

=> Agreed

R2-2405877	RRC corrections for Rel-18 SL relay enhancements	Huawei, HiSilicon, Ericsson, ZTE, CATT, Sharp, Lenovo, OPPO, Nokia, Apple, MediaTek, Xiaomi, Samsung, ASUSTeK	CR	Rel-18	38.331	18.1.0	4684	3	F	NR_SL_relay_enh-Core

=> Agreed

[Post126][402][Relay] Rel-18 relay capability CRs (Samsung)

Scope: Update the CRs in R2-2405532 and R2-2405533 in line with decisions of this meeting.

Intended outcome: Endorsed CRs in R2-2405878 (38.331) and R2-2405879 (38.306)

Deadline:  Very short (for merge into mega CRs)

R2-2405878	Correction for SL Relay UE capability	Samsung	CR	Rel-18	38.331	18.1.0	4774	1	F	NR_SL_relay_enh-Core	R2-2403975

=> Endorsed and to be merged to the mega CR

R2-2405879	Correction for SL Relay UE capability	Samsung	CR	Rel-18	38.306	18.1.0	1091	1	F	NR_SL_relay_enh-Core	R2-2403976

=> Endorsed and to be merged to the mega CR

[Post126][403][Relay] Rel-18 relay stage 2 CR (LG)

Scope: Develop and check a relay CR to 38.300 in line with decisions of this meeting, taking the endorsed CR from RAN2#125bis in R2-2403974 as baseline.

Intended outcome: Agreed CR in R2-2405880

Deadline:  Short (for RP)

R2-2405880	Corrections on NR sidelink relay enhancements	LG Electronics (Rapporteur)	CR	Rel-18	38.300	18.1.0	0857	1	F	NR_SL_relay_enh-Core

=> Revised in R2-2406104

R2-2406104	Corrections on NR sidelink relay enhancements	LG Electronics (Rapporteur)	CR	Rel-18	38.300	18.1.0	0857	2	F	NR_SL_relay_enh-Core

=> Agreed

Agreement:

The Rel-18 ASN.1 for the relay enhancements feature is considered ready to freeze (upon conclusion of the CR implementation discussion).

### 7.9.2	Stage 2 corrections

Impact to 38.300. A single CR with miscellaneous corrections is requested from the CR rapporteur. Minor and editorial issues should be coordinated with the rapporteur and merged into the miscellaneous CR. Larger issues can be discussed based on contributions.

R2-2404254	Discussion on stage-2 corrections	OPPO	discussion	Rel-18	NR_SL_relay_enh-Core

Proposal 1	In clause 16.21.1, clarify that “For the indirect path, both L2 and L3 MP Relay architectures are supported for MP Relay using PC5 interface, and only L2 MP Relay architecture is supported for MP Relay using N3C interface.”.

Proposal 2	In clause 16.21.3.1, rewording the RRCRconfiguration transmission procedure in step-4 to make it clear that DL non-split SRB1 on indirect path only is not supported.

Discussion:

Huawei think P2 is no longer needed after the post-meeting discussion after RAN2#125bis.  ZTE agree with Huawei.

Agreement:

In clause 16.21.1, clarify that “For the indirect path, both L2 and L3 MP Relay architectures are supported for MP Relay using PC5 interface, and only L2 MP Relay architecture is supported for MP Relay using N3C interface.”

R2-2404326	Candidate MP Relay UEs Reporting for N3C Remote UE	CATT	discussion	Rel-18	NR_SL_relay_enh-Core

Proposal 1: In 38.300, clarify the remote UE reports C-RNTI(s) of candidate relay UE(s) to gNB via the UEAssistanceInformation message for Multi-Path Scenario 2 in case of indirect path addition on top of direct path.

Discussion:

LG think the change is aligned with the current RRC CR.

Agreement:

In 38.300, clarify the remote UE reports C-RNTI(s) of candidate relay UE(s) to gNB via the UEAssistanceInformation message for Multi-Path Scenario 2 in case of indirect path addition on top of direct path.

### 7.9.3	RRC corrections

Impact to 38.331, except for capability-related issues (see agenda item 7.9.7). A single CR with miscellaneous corrections is requested from the CR rapporteur. Minor and editorial issues should be coordinated with the rapporteur and merged into the miscellaneous CR. Larger issues where no clear conclusion was reached in [Post125][417] can be discussed based on contributions.

RIL list

[Chair’s note: Per offline discussion during the meeting, K008 should be ToDo]

R2-2405369	RIL list for SL relay	Huawei, HiSilicon	report	Rel-18	NR_SL_relay_enh-Core

=> Revised in R2-2406085

R2-2406085	RIL list for SL relay	Huawei, HiSilicon	report	Rel-18	NR_SL_relay_enh-Core

=> Noted

Agreements:

K008 is ToDo (subject to later decisions this meeting). [Chair’s note: This agreement is superseded by the agreements under R2-2405425.]

Other PropAgree/PropReject RILs from R2-2405369 are confirmed as Agreed/Rejected respectively.

ToDoRILs: N122, B109, O437, K008

[Chair’s note: O437 is expected to be handled under the capability agenda item]

R2-2404803	[B109]Sidelink RRC reconfiguration failure for U2U	Lenovo	discussion	Rel-18

Proposal 1: A U2U Relay UE transmits NotificationMessageSidelink message including an indication type to source remote UE upon reception of RRCReconfigurationFailureSidelink from target remote UE.

Discussion:

OPPO think we have discussed the issue before, and they do not think the reconfiguration failure needs to trigger relay reselection; they see it as different from the case in Rel-16 with SUI because the network is not involved here and the reconfiguration did not come from the source remote UE.

LG think the source remote UE knows the state of the second link, and to OPPO’s comment, they think if the previous configuration is valid there could be a fallback, but if it happens in the first reconfiguration they are not sure what happens.

Qualcomm understand that if there is no valid configuration in the second hop, the relay UE could reconfigure the RLC channel on the first hop, but they are not sure if the spec currently allows this.

Lenovo think the relay might be able to reconfigure the channel, but they doubt if the performance is good enough.  Qualcomm think the relay can just release the first-hop configuration.

Huawei agree with OPPO and Qualcomm; they think if the reconfiguration fails on the second hop, the relay can try to reconfigure the second hop, then give up and release the link if it fails repeatedly, and the existing reselection trigger will be initiated.

ZTE think the source remote UE should be able to learn the status of the second hop and decide whether to reselect to another relay, so they support the proposal.

Lenovo think if Huawei’s comment is correct, we would not need the Rel-16 handling for reconfiguration failure.  Huawei mean that the relay UE should not skip the existing reporting, but in the Rel-16 case, if the network is involved it should know about the failure, and in idle/inactive the relay can retry by itself.

Ericsson understood the intention was to let the source remote UE know that something is wrong with the second leg so it can trigger reselection, but they think it is already possible to do that by detecting the problem in upper layers, either with connection failure or with QoS failure.  So they do not see the need for a new mechanism.

Agreement:

B109 moves to Rejected.

R2-2405458	[N122] and capability issue	Nokia	discussion	NR_SL_relay_enh-Core

Proposal 1: No capability is introduced to indicate that the UE supports only one path in UL while two paths in DL for MP split RB.

Discussion:

Qualcomm think with the SRB, the intention was to enable DL-only duplication for a UE that does not support UL duplication, and this is why we have the capability as it is.  So they think the existing capability makes sense.

Nokia think RAN3 have not discussed the UL split, only DL split, and without RAN3 input they see no need for the capability.  From the network perspective the simplification of not having it is a benefit.  Qualcomm clarify that RAN3 discussed it in the context of DC.

Huawei thought the capability would be helpful for the network, but they do not have a strong view.

Ericsson understand there is no need for the capability, and it would create a situation of supporting a subset of relay UEs.  Qualcomm think if the UE does not support duplication with split SRB, the network cannot configure DL-only duplication.

Ericsson want to understand if this means the duplication capability would link UL and DL for SRB.  Huawei think we would have to revert a previous agreement.  Nokia clarify that if there is only one capability, it has to indicate both.

Proposal 2: Allow configuring split SRB1 without duplication and direct SRB1 even when either the remote UE or the relay UE does not support PC5-RRC trigger.

Proposal 3: Introduce an indication whether the remote UE sends RemoteUEInformationSidelink including connectionForMP or not (option 1). Alternatively, RAN2 considers allowing at least the direct SRB1 based on gNB implementation even when PC5-RRC trigger is not supported by either the remote UE or the relay UE (option 2).

Discussion:

Nokia suggest that when the gNB knows that the relay UE is in RRC_CONNECTED, it should be possible to configure split SRB1 without duplication and direct SRB1.

Huawei think we have had a lot of discussion on the PC5-RRC trigger, and we should not change the UE behaviour for split SRB.  Nokia do not think this is a new UE behaviour and rather intend that the UE is not required to duplicate when not necessary; they wonder if there is any harm from the proposal.

Qualcomm agree that there is no harm, but they also think nothing is broken without it.

OPPO wonder what the underlying issue is; they understand option 2 in P3 is the current specification.  Nokia consider that in the current spec, if the UE does not support PC5-RRC trigger, the gNB always has to configure duplication.

Huawei understand that in the current spec, we did not capture such a network restriction about configuring duplication.  Nokia agree that there is no explicit restriction, but the trigger for MP connection is based on SRB1, so for it to work, SRB1 would need to be configured with duplication.

Ericsson think this is not necessarily the case, because the gNB could infer when the relay UE is in connected and not do duplication; they do not see that the current spec restricts this, i.e., the current spec is consistent with P2.

InterDigital think we should not specify network behaviour.

Agreements:

No capability is introduced to indicate that the UE supports only one path in UL while two paths in DL for MP split RB.

If the UE does not support duplication with split SRB, the network cannot configure DL-only duplication for this UE.

Confirm that split SRB1 without duplication or direct SRB1 can be configured even when the remote or relay UE does not support PC5-RRC trigger (e.g., because the gNB knows the relay UE is in connected mode).  No normative spec impact expected.

N122 moves to Agreed.

R2-2405425	[K008] How to set the contents of sidelink UE capability messages for L2 U2U Relay	ASUSTeK	discussion	Rel-18	38.331	NR_SL_relay_enh-Core

Proposal 1: NOTE 2 in section 5.8.9.2.3 is limited to unicast communication.

Proposal 2: Add a new NOTE in section 5.8.9.2.4 to leave it to UE implementation whether to include fields other than pdcp-ParametersSidelink in the UECapabilityInformationSidelink message for U2U Relay.

Discussion:

Apple think it would be more clear to talk about “direct” unicast communication, to distinguish from the relay case.

Agreements:

NOTE 2 in section 5.8.9.2.3 is limited to direct unicast communication.

Add a new NOTE in section 5.8.9.2.4 to leave it to UE implementation whether to include fields other than pdcp-ParametersSidelink in the UECapabilityInformationSidelink message for U2U Relay.

K008 moves to Agreed.

R2-2404327	Discussion on N122, B109 and SA2 LS Reply	CATT	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2405286	On RIL B109	Ericsson	discussion	Rel-18

Rapporteur/multi-company corrections CR

R2-2405601	RRC corrections for Rel-18 SL relay enhancements	Huawei, HiSilicon, ASUSTeK, Nokia, OPPO	CR	Rel-18	38.331	18.1.0	4847	-	F	NR_SL_relay_enh-Core

Discussion:

Huawei clarify this aligns with agreements already taken and is intended for merge into the rapporteur CR.

ASN.1 impact

R2-2404732	SL relay RRC correction proposals	Nokia	discussion	Rel-18	NR_SL_relay_enh-Core

Proposal 4.1: RAN2 to agree that the indication of support of N3C MP is not essential for cell access, and thus it should not be in SIB1.

Proposal 4.2: Move the indication of support of N3C MP (n3c-Support-r18) from SIB1 to SIB12.

Discussion:

LG are fine with the proposals and think the information in SIB12 is all optional, but it would be good to add some clarifying description.

Huawei think other information besides cell access information is in SIB1, e.g., related to idle/inactive.  They think for the use of SIB12, a network not supporting sidelink would now have to send SIB12 and there are some mandatory sidelink-specific fields that would have to be included.

LG indicate that the mandatory parameters can be an empty container.

ZTE think the gNB may support n3c and not want to provide SIB12, so they would rather keep it in SIB1.

Ericsson agree with Nokia that SIB1 is not ideal, but they also see that SIB12 may not be the ideal place, and the best solution might be a new SIB, but they acknowledge that a new SIB for one bit is not so sensible.

LG want to amend their previous comment, because there are some places where SIB12 would force the UE to provide the SUI message.  So they think in principle a new SIB would make sense.

Huawei think a new SIB is not good if we only have one bit, so they would like to stick to the previous agreement to use SIB1.

Apple think we could remove the feature entirely.

OPPO agree with Huawei and note that SIB1 already contains some support indications for other features such as IAB.  MediaTek have a similar view and think SIB1 may be the least bad option.

Nokia understand that SIB12 is not a good option, but they think it is regrettable for all networks to have to add a bit for a feature that may not be widely supported.  They also understand that the existing support indications are for the UE to decide whether to select the cell.

InterDigital think neither SIB1 nor SIB12 is ideal, but a new SIB is not realistic, and SIB1 may be the lowest impact.  Ericsson have a similar view.

Agreement:

The indication of N3C MP support is confirmed in SIB1.

R2-2404733	RSRP thresholds for U2N relay selection and re-selection	Nokia	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2405237	Corrections on UE capability reporting in SUI for L2 U2U relay	Huawei, HiSilicon	discussion	Rel-18	NR_SL_relay_enh-Core

Proposal 1: Clarify that the sl-CapabilityInformationSidelink-r18 in the SUI message indicates the UE capability of the L2 U2U Relay UE.  Besides, add a new IE to indicates the UE capability of the target L2 U2U Remote UE indicated by the sl-TargetUE-Identity. Changes for these are shown in the text proposal in the Annex..

Discussion:

OPPO understand the intention, but they wonder if we should change the field description of a legacy IE; maybe we need to add a clarification for Rel-18.

Apple think the first change is correct, but they do not see the need for the new IE.  Huawei understand that we need to provide the capability of the target UE, which cannot be done with the current structure.

Nokia also do not see that the new IE is needed; they think there is no case where both capabilities are needed in a single message, so we could use the legacy procedure.

Ericsson are not sure about the intention of the proposal to send the capability of the end UE; they understand that the configuration for the end UE is derived by the UE itself, and the network only needs the capability of the peer UE.

Qualcomm also think only one connection is established at a time towards one target, so they wonder if there is a case where multiple relay UE capabilities should be reported.

OPPO also think the new field for e2e capability is needed, since we left it to UE implementation to indicate the other capabilities; they think a separate capability indication is clearer.

Ericsson wonder why the gNB needs to know the destination UE capability, since the higher-layer configurations are done by the UE.  OPPO understand at least the PDCP configuration comes from the network.  Ericsson think this is done by UE implementation, not the network.

OPPO think the AS release indicator might be needed as well.

R2-2405352	Discussion on n3c-BearerAssociated	ZTE, Sanechips	discussion	NR_SL_relay_enh-Core

[AT126][408][Relay] Relay RRC proposals with ASN.1 impact (Huawei)

Scope: F2F offline to check ASN.1 proposals on Rel-18 relay and determine if changes are needed.  Critical proposals without ASN.1 impact can be checked if time permits.

Intended outcome: Report to CB session in R2-2405876

Schedule: Wednesday 2024-05-22 1700-1800 in Brk2

Deadline:  Thursday 2024-05-23 1000 JST

R2-2405876		Report of [AT126][408][Relay] Relay RRC proposals with ASN.1 impact (Huawei)	Huawei, HiSilicon	discussion	Rel-18	NR_SL_relay_enh-Core

Proposal 1: to add the new field to indicate the UE capability of the target L2 U2U Remote UE, clarify only out-of-order delivery is included in the UE cap container.

Discussion:

Chair clarifies that the intention is to have a new container for just this field and future extensions. Huawei indicate that the existing capability can be reused.

Ericsson understood from the RRC rapporteur that an OCTET STRING can contain an IE and we could define a new format.

Proposal 2: In subclause 5.8.17.4 of TS 38.331, for integrated discovery, when remote UE perform SL-RSRP filtering, the SL-RSRP filtering parameters should be used instead of using SD-RSRP filtering parameters.

Discussion:

Nokia recall that there was an agreement to use the SD-RSRP threshold because we assume the measurement would have SD-RSRP characteristics, and they think this also means we should use the SD-RSRP filtering.  They agree with the typo correction in the TP.

CATT agree with Nokia and think we can just fix the typo.

P3: remove the “according to association between User Info and L2 ID as specified in TS 23.304 [65],” for local ID pair setting procedure

Agreements:

Add the new field to indicate the UE capability of the target L2 U2U Remote UE, clarify only out-of-order delivery is included in the UE cap container.  A new container format will be defined for this purpose containing only the out-of-order delivery and the extension marker, to be contained in an OCTET STRING.

Remove the “according to association between User Info and L2 ID as specified in TS 23.304 [65],” for local ID pair setting procedure.

Other documents

R2-2404252	Correction of local ID setting by U2U Relay UE	OPPO	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2404328	Clarification on the U2U Relay (Re-)selection Procedure	CATT	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2404663	Discussion on SLRB index in SUI for L2 U2U relay	Apple	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2404678	Discussion on the remaining issues for U2U relay	LG Electronics Inc.	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2405238	Discussion on the SLRB index in the SUI for U2U relay UE	Huawei, HiSilicon	discussion	Rel-18	NR_SL_relay_enh-Core

R2-2405322	Correction on setuprelease type sidelink fields handling	Google	CR	Rel-18	38.331	18.1.0	4821	-	F	NR_SL_enh-Core

R2-2405351	Discussion on relay UE traffic pattern reporting in UAI	ZTE, Sanechips	discussion	NR_SL_relay_enh-Core

R2-2405628	discussion on U2U relay related issues	Sharp	discussion	Rel-18	NR_SL_relay_enh-Core

Withdrawn/Not available

R2-2405368	Rapp RRC CR for Rel-18 SL relay enhancement	Huawei, HiSilicon	report	Rel-18	38.331	NR_SL_relay_enh-Core	Withdrawn

### 7.9.4	SRAP corrections

Impact to 38.351. A single CR with miscellaneous corrections is requested from the specification rapporteur. Minor and editorial issues should be coordinated with the rapporteur and merged into the miscellaneous CR. Larger issues can be discussed based on contributions.

Rapporteur CR (endorsed at RAN2#125bis)

R2-2404247	Corrections for NR sidelink relay enhancements	OPPO, ZTE	CR	Rel-18	38.351	18.1.0	0034	2	F	NR_SL_relay_enh-Core	R2-2403814

=> Revised in R2-2405881

[Post126][404][Relay] Rel-18 relay SRAP CR (OPPO)

Scope: Update the CR in R2-2404247 in line with decisions of this meeting.

Intended outcome: Agreed CR in R2-2405881

Deadline:  Short (for RP)

R2-2405881	Corrections for NR sidelink relay enhancements	OPPO, ZTE	CR	Rel-18	38.351	18.1.0	0034	3	F	NR_SL_relay_enh-Core

=> Agreed

Other documents

R2-2404253	Discussion on SRAP corrections	OPPO	discussion	Rel-18	NR_SL_relay_enh-Core

Proposal 1	For egress link determination at U2U Remote UE, R2 not pursue further detailed specification of UE implementation.

Proposal 2	For egress link determination at U2U Relay UE, not pursue the L2 ID-based link determination.

Discussion:

Apple think the impact of their CR is not very large, and without it there are restrictions on reusing IDs for different connected relay UEs that go beyond what we have agreed.

OPPO note that we agreed the local ID should be able to identify the source and target remote UE, so they understand that it should not also depend on disambiguation with the L2ID.

Huawei think nothing is broken without the changes.  Apple agree that the system works.

Apple think on P1, the remote UE has to identify the relay UE by L2ID initially as provided by upper layer.  They think some clarification would be useful.

Qualcomm understand the upper layer will use link IDs, not L2ID, so they assume the UE should maintain the L2ID mapping to link IDs and derive the L2ID for each link.

OPPO intend that there may be different internal UE implementations regarding the layer interaction, and we don’t need to specify the details.

Agreements:

For egress link determination at U2U Relay UE, RAN2 will not pursue the L2 ID-based link determination.

R2-2404662	Correction for egress link determination in L2 U2U Relay UE	Apple	CR	Rel-18	38.351	18.1.0	0036	-	F	NR_SL_relay_enh-Core

- Not pursued

### 7.9.5	MAC corrections

Impact to 38.321. A single CR with miscellaneous corrections is requested from the CR rapporteur. Minor and editorial issues should be coordinated with the rapporteur and merged into the miscellaneous CR. Larger issues can be discussed based on contributions.

### 7.9.6	RLC and PDCP corrections

Impact to 38.322 and 38.323. For each specification, a single CR with miscellaneous corrections is requested from the CR rapporteur. Minor and editorial issues should be coordinated with the rapporteur and merged into the miscellaneous CR. Larger issues can be discussed based on contributions.

Rapporteur CR

R2-2405856	Miscellaneous Rapporteur Corrections to 38.323 for SL Relay	InterDigital, ZTE	CR	Rel-18	38.323	18.1.0	0135	2	F	NR_SL_relay_enh-Core	Late

=> Revised in R2-2405882

[Post126][405][Relay] Rel-18 relay PDCP CR (InterDigital)

Scope: Update the CR in R2-2405856 in line with decisions of this meeting.

Intended outcome: Agreed CR in R2-2405882

Deadline:  Short (for RP)

R2-2405882	Miscellaneous Rapporteur Corrections to 38.323 for SL Relay	InterDigital, ZTE	CR	Rel-18	38.323	18.1.0	0135	3	F	NR_SL_relay_enh-Core	Late

=> Agreed

Other document

R2-2405353	Discussion on PDCP corrections for MP	ZTE, Sanechips	discussion	NR_SL_relay_enh-Core

Proposal 1: For duplicate PDU discard, add the following case for MP split DRB:

−	   If the primary path for a MP split DRB is on the indirect path, when deactivation of PDCP duplication is indicated for the DRB, the PDCP entity shall indicate to the RLC entities other than the primary path to discard all duplicated PDCP Data PDUs.

Discussion:

ZTE clarify the change may be needed if the primary RLC entity is not used for MP with primary path on the indirect path.

CATT think for scenario 1, the primary RLC entity can be used, but not for scenario 2, so they think some clarification is useful.

InterDigital understand that the primary RLC entity was used in the legacy specification to indicate the possibility of multiple RLC entities on the direct path, but on the indirect path we have only a single RLC entity.  So they think it makes sense to exclude the terminology “primary RLC entity” on the indirect path.

Huawei are OK with the change and agree with InterDigital’s comment.

Nokia think we already tried to specify MP primary RLC in the rapporteur CR, and we should use the structure we have there.  They also think there is no ambiguity in the current specification; if the primary path is on the indirect path, the primary RLC entity will be the RLC entity of the indirect path, otherwise it will be one of the RLC entities on the direct path side.  ZTE point out that there is no “primary RLC entity” on the indirect path.  Nokia understand that the primary RLC entity is defined by indicating a cell group and should always be aligned with the primary path.

InterDigital understand that the primary path can be on the indirect path, but we should not reuse the legacy “primary RLC entity” terminology for the indirect path.

OPPO agree with Nokia and think the primary RLC entity is defined in the RRC spec as being on the primary path.

Apple note that in any case the PDCP layer communicates with the SRAP entity, not any RLC entity.  Nokia understand that the discard action must be taken by the RLC entity.

InterDigital think it is a bit a matter of style.

Nokia think the concerned action is discard, which cannot be done in SRAP, and we need to indicate which RLC entities shall do the discard.  InterDigital agree that the SRAP entity does not do the discard, and they think the point here is to avoid the “primary RLC entity” terminology.

Nokia think PDCP should follow the terminology established in RRC, where the primary RLC entity is defined.  LG have some sympathy for Nokia’s view and would like to keep the primary RLC entity language.

InterDigital see no need to refer to the primary RLC entity on the indirect path in the PDCP spec.

Agreement:

For duplicate PDU discard, add the following case for MP split DRB:

−	   If the primary path for a MP split DRB is on the indirect path, when deactivation of PDCP duplication is indicated for the DRB, the PDCP entity shall indicate to the RLC entities other than the primary path to discard all duplicated PDCP Data PDUs.

Wording and structure to be determined in rapporteur CR implementation.

### 7.9.7	UE capabilities

Impact to 38.306 and capability-related impact to 38.331. A single CR with miscellaneous corrections is requested from the CR rapporteur. Minor and editorial issues should be coordinated with the rapporteur and merged into the miscellaneous CR. Larger issues can be discussed based on contributions.

To include handling of RIL O437

Documents discussed jointly

R2-2405236	UE capability corrections for multi-path operation	Huawei, HiSilicon	discussion	Rel-18	NR_SL_relay_enh-Core

Proposal 1: The new capability is needed to indicate whether UE supports UL transmission via direct path and DL reception via either direct path or indirect path for split SRB.

R2-2405287	Discussion on Open Issues in 38.306	Ericsson	discussion	Rel-18

Proposal 1	Do not support a new capability to indicate whether UE supports UL transmission via direct path and DL reception via either direct path or indirect path for split SRB.

Agreement:

O437 moves to Rejected.

### 7.9.8	Idle mode corrections

Impact to 38.304. A single CR with miscellaneous corrections is requested from the CR rapporteur. Minor and editorial issues should be coordinated with the rapporteur and merged into the miscellaneous CR. Larger issues can be discussed based on contributions.

## 7.10	IDC enhancements for NR and MR-DC

(NR_IDC_enh-Core; leading WG: RAN2; REL-18; WID: RP-221281)

Time budget: 0 TU

Tdoc Limitation: 1 tdocs

Corrections. For smaller corrections please contact CR editor / Rapporteur directly. For RRC corrections, only selected RIL can be submitted in the agenda  (i.e. only if RRC editor suggests to discuss the RIL under this agenda)

R2-2405484	Xiaomi, Nokia, Huawei, HiSilicon, ZTE Corporation, Sanechips, Ericsson, Samsung	CR	Rel-18	38.331	18.1.0	4838	-	F	NR_IDC_enh-Core

=> The CR is agreed

=> RAN2 understanding is that for the earlier agreement “the UE always provides FDM reporting when provides the TDM reporting to the network”, FDM reporting and TDM reporting need not always be included in the same UAI message.

[AT126][014][IDC] Miscellaneous corrections for IDC (Xiaomi)

Scope: To discuss the changes in R2-2405484

Intended outcome: Agreeable RRC CR R2-240xxxx

Deadline of company comments: Wednesday 2024-05-22 1800

Deadline of comments on the CR revision: Thursday 2024-05-23 1800

Note: The RRC CR will be agreed by email without CB.

Deadline:  05-24-24

## 7.11	Enhancements of NR Multicast and Broadcast Services

(NR_MBS_enh-Core; leading WG: RAN2; REL-18; WID: RP-231829)

Time budget: 0 TU

Tdoc Limitation: 1 tdoc

### 7.11.1	Organizational

LS in, rapporteur input (e.g. rapporteur CR, open issues list)

R2-2404411	RIL list for MBS	Huawei, HiSilicon	report	Rel-18	NR_MBS_enh-Core

Todo RILs: C150 N102 S731 X151

Noted

=> Revised in R2-2406026

R2-2405113	MBS Rapporteur CR for RRC	Huawei, HiSilicon, CATT, Samsung, LG Electronics Inc., CMCC, Xiaomi	CR	Rel-18	38.331	18.1.0	4688	2	F	NR_MBS_enh-Core	R2-2404019

To be reviewed during post-meeting e-mail discussion

=> Revised in R2-2406025

[POST126][607][eMBS] RRC CR (Huawei)

Scope: Update RRC CR according to the agreements from the meeting.

Intended outcome: Agreed CR

Deadline:  Two weeks

R2-2406025	Miscellaneous corrections for eMBS	Huawei, HiSilicon, CATT, Samsung, LG Electronics Inc., CMCC, Xiaomi	CR	Rel-18	38.331	18.1.0	4688	3	F	NR_MBS_enh-Core

=> Agreed

R2-2406026	RIL list for MBS	Huawei, HiSilicon	report	Rel-18	NR_MBS_enh-Core

### 7.11.2	RRC corrections

Corrections related to RILs from ASN.1 review.

ToDo RILs

R2-2404339	[C150] Issue on applying PTM configuration in RRC Release	CATT, Huawei, HiSilicon, CBN, China Broadnet	discussion	Rel-18	NR_MBS_enh-Core

Proposal 1: To support the case that PTM configuration for SCell can be included in RRCRelease message, “on the cell where the MBSMulticastConfiguration message was received” is to be removed in 5.10.3.2. TP in Annex 1 is agreed.

DISCUSSION:

- Nokia agrees with the proposal.

- Samsung thinks we can modify the wording instead of removing this description.

- Huawei thinks MRB establishment can happen not only when the config is received so it is better to remove. Nokia agrees, it is better to remove.

- ZTE would like to add an explicit indication.

- Ericsson indicates we have information in MCCH or we can rely on Paging in MCCH-less cell.

Remove “on the cell where the MBSMulticastConfiguration message was received” from 5.10.3.2 in RRC.

R2-2405078	[S731] SDT failure and multicast reception	Samsung	discussion	Rel-18

Proposal 1: Upon unsuccessful completion of the SDT procedure:

(a)	UE which is configured for multicast reception in RRC_INACTIVE, transits to RRC_IDLE (same as legacy spec).

(b)	UE forwards TMGI(s) to upper layers for multicast session(s) that UE is configured for multicast reception and receiving in RRC_INACTIVE. Adopt the text proposal given in the annex.

DISCUSSION:

- Nokia does not think this is required as it is clear from CT1 specs.

- LGE thinks it is possible in CT1 specs, but it causes service interruption.

- ZTE does not like LGE’s solution, it is a modification of legacy SDT behaviour.

- Ericsson does not think we need to change anything in RAN2 specs.

- Huawei shares views from Nokia, Ericsson, ZTE.

- Samsung thinks in NAS there is no list of active sessions, so they propose to send this.

S731 is rejected

R2-2405483	[X151] Discussion on frequency information reported for shared processing	Xiaomi, Huawei, HiSilicon	discussion	Rel-18	NR_MBS_enh-Core	R2-2402849

Proposal: RAN2 is kindly requested to clarify that the frequency band information reported is based on the UE capability supporting MBS.

DISCUSSION:

- Xiaomi clarifies there was some offline discussion and SA4 made some changes.

To be discussed during CR review whether it is OK to remove “acquired from the SIB21 or the USD” from freqInfoMBS field description.

R2-2405564	[N101] [N102] [N103] [N104] [N105] Control plane aspects of multicast reception in RRC_INACTIVE state	Nokia	discussion	Rel-18	NR_MBS_enh-Core

Proposal 1 [N102]: RAN2 discusses the following options:

1-	Default values are introduced to DMRS type, DMRS additional position and max length for multicast reception in RRC_INACTIVE state (no changes in specification required),

2-	RRC release/MCCH indicate DMRS type, DMRS additional position and max length for multicast reception in RRC_INACTIVE state. (requires changes in the specification).

DISCUSSION:

- Samsung is OK to use default values, but the values should be decided by RAN1.

- Huawei thinks default values are already defined in 38.214 and we can reuse those values.

- QCM prefers having default values, but thinks this needs to be clarified by RAN1.

- Ericsson thinks the default values are already there.

RAN2 thinks default values of DMRS type, DMRS additional position and max length should be used for multicast reception in RRC_INACTIVE state.

If companies think there is some RAN1 impact, the contributions can be brought to RAN1 directly.

Other

R2-2404992	MBS open issues	Ericsson	discussion	Rel-18	NR_MBS_enh-Core

R2-2405293	Multicast reception in RRC_INACTIVE when an SDT procedure fails	LG Electronics Inc.	discussion	Rel-18	NR_MBS_enh-Core

R2-2405681	[C150] Which cell to apply the PTM configuration in RRC_INACTIVE	ZTE, Sanechips	discussion	Rel-18	NR_MBS_enh-Core

Withdrawn

R2-2405472	Paging message	Sharp	discussion	Withdrawn

### 7.11.3	Other corrections

Corrections related to other specs, e.g. 38.300, 38.321, 38.323, UE capabilities.

R2-2404667	Clarification on MAC reset for multicast reception in RRC_INACTIVE	Apple, CATT	discussion	Rel-18	NR_MBS_enh-Core

Proposal 1: Clarify that during RRC Resume procedure, UE only perform the MBS multicast specific MAC reset with the following operations:

-	Stop the MBS multicast DRX timers;

-	Flush the soft buffers for all DL HARQ processes used for MBS multicast;

-	For each DL HARQ process used for MBS multicast, consider the next received transmission for a TB as the very first transmission.

Proposal 2: Reflect MBS multicast specific MAC reset operation in MAC spec, and agree the TP in Option 1.

Noted

R2-2404668	Clarification on MAC reset for multicast reception in RRC_INACTIVE	Apple, Samsung, CATT, Huawei, HiSilicon, Nokia, Sharp, Qualcomm Incorporated, Ericsson	CR	Rel-18	38.321	18.1.0	1842	-	F	NR_MBS_enh-Core

Update with the TP from R2-2405582.

Revised in R2-2405771 which is agreed unseen.

R2-2405771	Clarification on MAC reset for multicast reception in RRC_INACTIVE	Apple, Samsung, CATT, Huawei, HiSilicon, Nokia, Sharp, Qualcomm Incorporated, Ericsson	CR	Rel-18	38.321	18.1.0	1842	1	F	NR_MBS_enh-Core

Agreed (unseen)

R2-2405582	Error data handling for MBS	Langbo	discussion	Rel-18	NR_MBS_enh-Core

Proposal 1: The MAC entity shall discard the received subPDU containing an LCID or eLCID value associated with a suspended multicast MRB.

Proposal 2: Adopt one of the TP in the Annex.

DISCUSSION:

- LGE, Nokia support the change.

- Xiaomi asks whether this also applies to G-CS-RNTI. Huawei thinks the issue happen for the UE in RRC_INACTIVE, so there is no need to include this.

The MAC entity shall discard the received subPDU containing an LCID or eLCID value associated with a suspended multicast MRB.

Adopt the TP in the Annex of R2-2405582.

## 7.12	Mobile IAB (Integrated Access and Backhaul) for NR

( NR_mobile_IAB -Core; leading WG: RAN3; REL-18; WID: RP-232669)

Time budget: N/A

Tdoc Limitation: 1 tdocs (if you want to input beyond the tdoc limitation, please cooperate with CR Rapporteurs).

### 7.12.1	Organizational and Stage-2

LS in. Includes TS impacts 38300 and Stage-2 Centric issues (can also cover secondary impacts to other TSes)

Agreed-in-principle CRs

R2-2405556	Clarification on supporting two logical DUs and connecting via stationary IAB node	ZTE, Qualcomm, Ericsson, Samsung, Nokia	CR	Rel-18	38.300	18.1.0	0853	2	F	NR_mobile_IAB-Core	R2-2403959

R2-2405686	Mismatch of terminology between 38.304 and 38.331	Samsung	CR	Rel-18	38.304	18.1.0	0398	1	F	NR_mobile_IAB-Core	R2-2402936

R2-2404960	Miscellaneous corrections on Mobile IAB	Ericsson	CR	Rel-18	38.331	18.1.0	4701	2	F	NR_mobile_IAB-Core	R2-2404018

3 CRs agreed

Other

R2-2404961	RILs conclusions for mobile IAB	Ericsson	discussion	Rel-18	NR_mobile_IAB-Core

All RIL issues closed

Noted

### 7.12.2	Stage-3

For multi-TS input, it is allowed to input also here.

#### 7.12.2.1	BAP

TS impacts 38340 and BAP Centric issues (can also cover secondary impacts to other TSes if applicable)

#### 7.12.2.2	Control plane corrections

TS impacts 38331, ASN.1 RIL, UE capabilities and 38.304

#### 7.12.2.3	User plane corrections

TS impacts 38321

## 7.13	Further enhancement of data collection for SON MDT in NR and EN-DC

(NR_ENDC_SON_MDT_enh2-Core; leading WG: RAN3; REL-18; WID: RP-221825)

Includes LS in’s related to AI/ML for NG-RAN

WI is declared 100% complete

Time budget: 0 TU

Tdoc Limitation: 1 tdocs ?

### 7.13.1	Organizational

Ls in and Rapporteur input. WI/Spec Rapporteur(s) are invited to provide updated open issues lists that need to be handled.

R2-2405091	Corrections to 38331 for Rel-18 SONMDT	Ericsson	CR	Rel-18	38.331	18.1.0	4815	-	F	NR_ENDC_SON_MDT_enh2-Core

Endorsed and to be updated

=> Revised in R2-2405968

[Post126][753][SONMDT] NR RRC CR for SONMDT (Ericsson)

Scope:

    - Produce agreeable CR

Intended outcome:

    - Agreed CR in R2-2405968 (Ericsson)

Deadline:

    - Short

=> Agreed in R2-2405968

R2-2405968	Corrections to 38331 for Rel-18 SONMDT	Ericsson	CR	Rel-18	38.331	18.1.0	4815	1	F	NR_ENDC_SON_MDT_enh2-Core

=> Agreed

=> Coversheet update by MCC: wrong rev value.

R2-2406125	Corrections to 38331 for Rel-18 SONMDT	Ericsson	CR	Rel-18	38.331	18.1.0	4815	2	F	NR_ENDC_SON_MDT_enh2-Core

=> Agreed

R2-2405092	RIL list for SON	Ericsson	discussion	NR_ENDC_SON_MDT_enh2-Core

All PropAgree are agreed. All PropReject are rejected.

R2-2405343	WI RIL list for 36.331 for R18 SONMDT	Huawei, HiSilicon	discussion	NR_ENDC_SON_MDT_enh2-Core

All PropAgree are agreed. All PropReject are rejected.

R2-2405344	Corrections to TS 36.331 for R18 SONMDT	Huawei, HiSilicon	CR	Rel-18	36.331	18.1.0	5022	-	F	NR_ENDC_SON_MDT_enh2-Core

Endorsed and to be updated

[AT126][765][SONMDT] LTE RRC CR for SONMDT (Huawei)

Scope:

    - Produce agreeable CR

Intended outcome:

    - Agreed CR in R2-2405969 (Huawei)

Deadline:

    - Thursday lunch. Intention is to agree the CRs over email.

R2-2405969	Corrections to TS 36.331 for R18 SONMDT	Huawei, HiSilicon	CR	Rel-18	36.331	18.1.0	5022	1	F	NR_ENDC_SON_MDT_enh2-Core	R2-2405344

Agreed

### 7.13.2	Papers related to RILs

R2-2404947	[C307] Discussion on remaining issue on fast MCG recovery	CATT,Fujitsu, Ericsson	discussion	Rel-18	NR_ENDC_SON_MDT_enh2-Core

-	Ericsson prefers solution 1 described in the paper since its simpler. Huawei also prefers solution 1. Nokia and ZTE prefers option 2. Samsung prefers option 1 but can accept solution 2. Fujitsu think both solutions work, but prefer solution 1 since it has less signalling overhead. CMCC prefers solution 1. CATT prefers solution 1.

Introduce a one bit indication to indicate the SCG failure happens during T316 running.

R2-2405093	Addressing SONMDT RILs and miscellaneous corrections	Ericsson	discussion	NR_ENDC_SON_MDT_enh2-Core

Proposal 1	RAN2 agree to move the sdt-Failed-r18 field outside the ra-InformationCommon IE, as shown in the Annex of R2-2405093.

Proposal 2	RAN2 agree to clarify the procedural text of for logging the time elapsed between SCG failure and MCG failure according to R2-2405093.

Proposal 3	Cancel the agreement: Move the SPR determination procedure description out of the branch of scg-State, and adopt the TP in section 4 of R2-2402653, and use the original text as shown R2-2405093.

Proposal 4	RAN2 to agree on the CR to TS 38.300 in the Annex 2 - Miscellaneous corrections to TS 38.300.

On P1

-	ZTE think that current design is good enough even though it could have been done nicer. Ericsson think we can do the change now when we have the chance. CATT think that it’s a good time to fix this now before ASN.1 freeze.

On P3

-	CATT is OK with revering the agreement.

On P4

-	ZTE think that the first change is about NW behaviour and should not be discussed in RAN2, instead it should be discussed in RAN3. Nokia think that the first change is not needed and is not a change which is suitable for stage-2, instead it could be considered for stage-3. Ericsson thinks that RAN2 made the agreements related to the first change.

Move the sdt-Failed-r18 field outside the ra-InformationCommon IE, as shown in the Annex of R2-2405093.

Clarify the procedural text of for logging the time elapsed between SCG failure and MCG failure according to R2-2405093.

Revert the agreement: Move the SPR determination procedure description out of the branch of scg-State, and adopt the TP in section 4 of R2-2402653, and use the original text as shown R2-2405093.

Except for the first change, adopt the changes for TS 38.300 shown in the Annex 2 of R2-2405093.

[AT126][760][SONMDT] SON/MDT corrections for 38.300 (Ericsson)

Scope:

    - Produce agreeable CR for 38.300 according to the agreement.

Intended outcome:

    - Agreed CR in R2-2405840 (Ericsson)

Deadline:

    - Thursday lunch. Intention is to agree the CRs over email.

R2-2405840	SON/MDT corrections for 38.300	Ericsson	CR	Rel-18	38.300	18.1.0	0869	-	F	NR_ENDC_SON_MDT_enh2-Core

Agreed

=> Coversheet update by MCC: wrong CR number

R2-2406123	SON/MDT corrections for 38.300	Ericsson	CR	Rel-18	38.300	18.1.0	0869	1	F	NR_ENDC_SON_MDT_enh2-Core

=> Agreed

R2-2405166	[S524] Handling SPR configuration during CHO/LTM based recovery 	Samsung	discussion

Proposal 1: If attemptCondReconfig or attemptLTM-Switch are configured while initiating RRC Reestablishment procedure, UE keeps the successPSCell-Config configured by the PSCell.

Proposal 1a: If attemptCondReconfig and attemptLTM-Switch are not configured while initiating RRC Reestablishment procedure, UE releases successPSCell-Config configured by the PSCell.

Proposal 1b: If the CHO based recovery or LTM based recovery is not successful and attemptCondReconfig or attemptLTM-Switch are configured, UE releases successPSCell-Config configured by the PSCell.

Proposal 2: Adapt the given TP in section 5.

-	Huawei think that these proposals are signalling optimizations. Samsung think that there can be misalignment between the UE and the SN. Ericsson says that the UE releases the SN at reestablishment.

-	After further offline discussion, Samsung indicates that companies agree about the issue. But the solution can be discussed later.

Postponed.

R2-2405557	[J041][J042]RILs for fast MCG recovery MRO	SHARP Corporation	discussion

Proposal 1: UE includes the handover target cell identity in the RLF report upon RRCReconfiguration is received in response to MCGFailureInformation.

Proposal 2: UE includes target PSCell Id in RLF report when fast MCG recovery failure cause is PSCell change failure.

On P1

-	QC thinks this has been discussed before, it is also not a correction.

On P2

-	QC disagrees. Ericsson thinks this is critical and hence they support. Samsung questions the scenario. Nokia also does not see the need for this. Lenovo thinks this was discussed and rejected earlier.

R2-2405085	Consideration on SON/MDT remaining issues	ZTE Corporation, Sanechips	discussion	Rel-18	NR_ENDC_SON_MDT_enh2-Core

### 7.13.3	Other

R2-2404736	Correction of availability indication of logged MDT report for SNPN	Nokia	CR	Rel-18	37.320	18.1.0	0132	-	F	NR_ENDC_SON_MDT_enh2-Core

-	Fujitsu indicated word needed polishing.

[AT126][761][SONMDT] MDT report for SNPN (Nokia)

Scope:

    - Polish wording

Intended outcome:

    - Agreed CRs in R2-2405841 (Nokia)

Deadline:

    - Thursday lunch. Intention is to agree the CRs over email.

R2-2405841	Correction of availability indication of logged MDT report for SNPN	Nokia	CR	Rel-18	37.320	18.1.0	0132	-	F	NR_ENDC_SON_MDT_enh2-Core	R2-2404736

Agreed

R2-2405345	Discussion on stage-2 issues for R18 SONMDT	Huawei, HiSilicon	discussion	NR_ENDC_SON_MDT_enh2-Core

In TS 37.340, remove the following sentence: A UE while being in EN-DC and NGEN-DC can collect E-UTRA RA Reports and NR RA Reports upon performing RACH in MN and SN respectively.

[AT126][762][SONMDT] Stage-2 issues for R18 SONMDT (Huawei)

Scope:

    - Produce agreeable CR

Intended outcome:

    - Agreed CR in R2-2405842 (Huawei)

Deadline:

    - Thursday lunch. Intention is to agree the CRs over email.

R2-2405842	Corrections to TS 37.340 on collection of RA reports	CR

Agreed

## 7.14	Enhancement on NR QoE management and optimizations for diverse services

(NR_QoE_enh-Core; leading WG: RAN3; REL-18; WID: RP-223488)

Time budget: 0 TU

Tdoc Limitation: 1 tdoc

### 7.14.1	Organizational

LSs and rapporteur inputs (e.g. rapporteur CR, open issues list)

R2-2404145	Reply LS on area scope handling for QoE measurements (S5-241925; contact: Ericsson)	SA5	LS in	Rel-18	NR_QoE_enh-Core	To:RAN2	Cc:RAN3, SA4

Noted

R2-2404480	RIL issue list for QoE	Ericsson	discussion	Rel-18	NR_QoE_enh-Core

PropAgree: H173

PropReject: H170, H171

ToDo: E224, E214, H079, H174, E216

- Ericsson clarifies that RAN3 is discussing a correction for E216 and RAN2 may need to do something.

PropAgree and PropReject RIL conclusions are endorsed

For E216, RAN2 will only do something if requested by RAN3.

R2-2404479	Correction of Enhancement on NR QoE management and optimizations for diverse services	Ericsson	CR	Rel-18	38.331	18.1.0	4781	-	F	NR_QoE_enh-Core

To be reviewed via post-meeting e-mail discussion

[POST126][608][QoE] RRC CR (Ericsson)

Scope: Update RRC CR according to the agreements from the meeting.

Intended outcome: Agreed CR

Deadline:  Two weeks

=> Agreed in R2-2406091

R2-2406091	Correction of Enhancement on NR QoE management and optimizations for diverse services	Ericsson	CR	Rel-18	38.331	18.1.0	4781	1	F	NR_QoE_enh-Core

=> Agreed

### 7.14.2	RRC corrections

Corrections related to RILs from ASN.1 review.

R2-2404481	Open RIL issues for QoE measurements	Ericsson	discussion	Rel-18	NR_QoE_enh-Core

Proposal 1	RAN2 to clarify or solve the different parameter configuration behaviours between UE AS layer and UE application layer for RVQoE configuration. [E214]

Proposal 2	Include measConfigReportAppLayerAvailable in RRCReconfigurationComplete or RRCReestablishmentComplete message if the UE has stored QoE configuration with appLayerIdleInactiveConfig configured, but it has not been successfully transmitted since the UE entered RRC_CONNECTED. (Reference TP in Annex.3) [E224]

Proposal 3	RAN2 to keep the current description for QoE MBS area scope checking.

Proposal 4	UE releases all NR QoE configurations and reports upon successful completion of the mobility from NR.

DISCUSSION on P1:

- CATT prefers to fix this issue in NAS specs. Nokia agrees, clarification should be done in NAS specs.

- Huawei thinks this can be left to UE implementation.

- Ericsson thinks the issue is in RRC specifications. Samsung agrees with Ericsson.

- QCM thinks there is no QoE modification procedure. Setup/release can be used.

[E214] We add a note to clarify that modification of RVQoE config is not possible, i.e. release/setup procedure should be used.

DISCUSSION on P2:

- Samsung thinks this is an optimization and we don’t need this.

- Ericsson asks if then the UE will keep on repeating the same information. Samsung confirms.

- QCM agrees with the intention of the change, but perhaps there can be a better wording to capture it.

- CATT asks whether we should also add another limitation for the UE not to resend the indication.

- Ericsson thinks this can be discussed as part of the TP discussion.

[E224] RAN2 understanding is that UE should include measConfigReportAppLayerAvailable in RRCReconfigurationComplete or RRCReestablishmentComplete message if the UE has stored QoE configuration with appLayerIdleInactiveConfig configured, but it has not been successfully transmitted since the UE entered RRC_CONNECTED. FFS how to capture in specification.

DISCUSSION on P4:

- Nokia agrees with the intention. Also – should this be also handled for R17? Nokia thinks we should add a condition that this is only executed when there is valid QoE configuration.

- Huawei would like to avoid R17 CR.

In Rel-18, we correct the QoE config release procedure in the following way: UE releases all NR QoE configurations and reports upon successful completion of the mobility from NR.

Can consider applying this from Rel-17. Check offline whether applying this from Rel-17 causes IOT issues.

R2-2405341	Discussion on serving cell for MBS QoE collection	Huawei, HiSilicon	discussion	NR_QoE_enh-Core

Proposal 1: RAN2 discuss the following solutions

-	Solution 1: UE checks the area scope when the cells receiving the MBS broadcast service changes.

-	Solution 2: UE checks the area scope upon cell reselection.

Proposal 2: RAN2 to discuss the following rule for area scope checking:

if any of MBS service cells belong to the area scope, UE is within the area scope. Otherwise, the UE is out of the area scope.

DISCUSSION:

- Ericsson thinks there is no standardization in SA WGs for MBS in IDLE/INACTIVE, would prefer not to do anything at this stage. Nokia agrees that nothing is needed now. Nokia indicates area scope checking is only done during session start, so we do not have to optimize.

- CATT understands the intention from Huawei and is OK to fix it.

- QCM thinks that it is better to keep it simple since there is no clear definition from SA5 on area scope.

RAN2 to keep the current description for QoE MBS area scope checking.

R2-2405342	Discussion on the condition QoENRDC [H174]	Huawei, HiSilicon	discussion	NR_QoE_enh-Core

Proposal 1: For H174, RAN2 to agree on the following definition of the condition QoENRDC:

This field is optionally present, Need M, when SRB5 is configured. Otherwise, it is absent.

DISCUSSION:

- Ericsson thinks the proposal is not aligned with the agreement RAN2 made in the past. There is also impact on QoE pause mechanism. Prefer to keep current wording.

- Samsung prefers the current wording, there is already note about what UE does when there is no SRB.

- Nokia agrees.

- Ericsson thinks that maybe note needs to be updated but it can be discussed later.

RIL H174 is rejected

R2-2404604	Discussion on remaining QoE issues	Samsung Shenzhen	discussion	Rel-18	NR_QoE_enh-Core

Proposal 1. UE suspends sending MBS QoE reports to gNB, when QoE reporting is paused. Adopt the following proposed text.

Proposal 2. UE sets each entry of measReportAppLayerContainerList in the order of time. Adopt the following proposed text.

Proposal 3. UE uses measReportAppLayerContainerList-r18 only to include QoE reports for Rel-18 MBS QoE measurement. Adopt the following proposed text.

DISCUSSION on P1:

- Ericsson does not think the scenario can happen.

P1 is postponed to clarify whether the described scenario can happen.

DISCUSSION on P2:

- Ericsson indicates the time stamp is in the container (as opposed to RVQoE).

- Huawei agrees with Ericsson, the time stamp in container is enough. QCM agrees.

- Samsung indicates the timestamp is needed for MDT alignment.

P2 is not agreed unless critical issues are found

DISCUSSION on P3:

- Ericsson thinks it is not a problem as R17 gNB will just not read this IE. Samsung indicates that then some reports will be lost.

- Huawei thinks there should be no problem. Samsung clarifies that

Offline Samsung to discuss P1, P2 and P3. If companies think the issues exist, they solutions can be captured during the RRC CR review.

[AT126][604][QoE]  Remaining QoE issues (Samsung)

Scope: Discuss P1, P2, P3 from R2-2404604

Intended outcome: Report with agreeable proposals to be approved offline (report in R2-2405777)

Deadline:  Friday 2024-05-24 0900

R2-2405777 Report of [AT126][604][QoE] Remaining QoE issues (Samsung) Samsung

P2 and P3 in R2-2404604 are not agreed.

The intention of P1 in R2-2404604 is agreed. The exact change will be discussed during RRC CR review

R2-2405086	Consideration on QoE remaining issues	ZTE Corporation, Sanechips	discussion	Rel-18	NR_QoE_enh-Core

R2-2405443	Discussion on QoE left open issues	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_QoE_enh-Core

### 7.14.3	Other corrections

Corrections related to other specs, e.g. 38.300, 37.340, UE capabilities.

## 7.15	NR Sidelink evolution

(NR_SL_enh2; leading WG: RAN1; REL-18; WID: RP-230077)

Time budget: 0 TU

Tdoc Limitation: 2 tdocs

### 7.15.1	Organizational

Including incoming LSs and rapporteur inputs. CR rapporteurs are asked to continue maintaining an open issues list reflecting known issues to be handled during the maintenance phase.

R2-2404106	Reply to LS on IUC or DRX in co-channel co-existence (R1-2403573; contact: LGE)	RAN1	LS in	Rel-18	NR_SL_enh2-Core	To:RAN2

- Noted.

R2-2404108	LS on CPE starting position for S-SSB in SL-U (R1-2403578; contact: OPPO)	RAN1	LS in	Rel-18	NR_SL_enh2-Core	To:RAN2

- Noted.

[OPPO]: Already captured in the latest RRC CR.

R2-2404109	LS on updating RAN1 agreement about minimum time gap Z (R1-2403588; contact: Huawei)	RAN1	LS in	Rel-18	NR_SL_enh2-Core	To:RAN2

- Noted.

[Huawei]: Already captured in the latest MAC CR.

### 7.15.2	Control plane corrections

Including RRC corrections and ASN.1 RILs. A single CR with miscellaneous corrections is requested; minor and editorial issues should be coordinated with the CR rapporteur and merged into the miscellaneous CR. Note RRC CR rapporteur’s summary and suggestion may be provided.

R2-2404167	Correction on Release-18 SL Evolution	OPPO	CR	Rel-18	38.331	18.1.0	4646	2	F	NR_SL_enh2	R2-2403930

- Endorsed.

=> Revised in R2-2405899

[POST126][101][V2X/SL] (OPPO)

Scope: To capture all agreements into 38.331.

Intended outcome: Approve RRC CR in R2-2405899.

Deadline: Short email discussion.

R2-2405899	Correction on Release-18 SL Evolution	OPPO	CR	Rel-18	38.331	18.1.0	4646	3	F	NR_SL_enh2

=> Agreed

R2-2404168	Left issues on RRC	OPPO	discussion	Rel-18	NR_SL_enh2

Proposal 1	R2 confirm the configured carrier-set configuration for two RLC legs in case of PDCP duplication would not affect the usage of legacy-carrier before initial RRCReconfigurationCompleteSidelink message which confirms SL CA carrier(s) addition.

- Agreed.

R2-2404171	RIL list for R18 SL	OPPO	report	Rel-18	NR_SL_enh2

- All “Agreed”, “PropAgreed” and “Rejected” are agreed.

R2-2404200	[O324] Allowed carrier indication upon carrier addition and release	OPPO	discussion	Rel-18	NR_SL_enh2

Proposal 1	R2 discuss to capture the allowed-carrier indication to lower layer due to RRCReconfigurationCompleteSidelink message confirming SL CA carrier(s) addition/release into 5.8.9.1b.2.2 / 5.8.9.1b.1.2, and remove that in 5.8.9.1a.4.

- Agreed.

[OPPO]: From RRC CR rapporteur point of view, carrier additions due to CA/PDCP duplication would be good to move to the dedicated section that already exists. With Xioami’s TP, it is still not clear whether PDCP duplication is enabled during SL link establishment. [Xiaomi]: Understands CA/PDCP duplication is not allowed during SL link establishment.

R2-2404574	Correction on TS 38.331 for SL	Xiaomi	discussion

Proposal 1: RAN2 to agree to clarify that the existing procedure to indicate the allowed carriers for the original RLC bearer for SRB applies to the case when PDCP duplication is not enabled.

Proposal 2: RAN2 to agree TP1 in Annex.

Proposal 3: For STCH, when duplication is configured, when the UE is in RRC_IDLE / RRC_INACTIVE / OOC, a UE can use any carrier within the super-set of <legacy carrier, and the carriers that the QoS flows of the unicast link associate with> but has to ensure the two RLC legs are not mapped to the same carrier.

Proposal 4: RAN2 to agree TP2 in Annex.

- P1 and P2 are handled as part of RRC CR preparation.

[OPPO, Apple]: Understand “a UE can use any carrier within the super-set of <legacy carrier, and the carriers that the QoS flows of the unicast link associate with>” is applicable only to SL SRB (not for DRB). [OPPO]: Can have offline discussion with Xiaomi. [Xiaomi]: After offline discussion, it was agreed to handle P1 and P2 as part of RRC CR preparation.

R2-2404216	[H162][H163] Discussion on RRC issues	Huawei, HiSilicon	discussion	Rel-18	NR_SL_enh2

Proposal 1: [H162] RAN2 to confirm that SL CA can be applied when the PDCP duplication is not used.

Proposal 2: [H162] RAN2 to adopt TP1 in Annex if Proposal 1 is agreed.

- Proposal 1 is confirmed.

- TP1 is not pursued.

[OPPO]: TP1 and proposal 1 are not aligned. “sl-AllowedCarriers” is configured for PDCP duplication. TP seems proposing SL carriers configured for PDCP duplication is used for CA purpose. Note there is condition specified this IE is only for PDCP duplication.

Proposal 3: [H163] For RRC_IDLE/INACTIVE UE, if SL C-LBT failure is detected for all the RB sets in RP with PSFCH and even there are still available RB set(s) in RP without PSFCH, SL RLF should be triggered for all UC connections configured with HARQenabled LCH(s).

Proposal 4: [H163] If proposal 3 is agreed, RAN2 to agree TP2 in Annex.

- Not pursued.

[Ericsson]: We can count on SL LBT recovery timer instead of SL RLF declaration.

R2-2404318	Discussion on remaining issue of TS 38.304	NEC  Corporation	discussion	Rel-18	NR_SL_enh2

Proposal 1	It is suggested to capture the UE cell selection behaviour in TS 38.304 if the UE would like to perform V2X use case with multi-carrier.

Proposal 2	If propose 1 is agreed, it is suggested to adopt the corresponding TP.

- Noted. Companies can have more time to think if the current 304 is enough or not.

R2-2405230	[Z712][Z713]Discussion on control plane correction	ZTE Corporation, Sanechips	discussion	Rel-18	NR_SL_enh2

Proposal 1	For SRB, legacy carrier is used if the SL-TxProfile of at least one associated QoS flow for the destination indicates backwardsCompatible, and adopt the TP-1 in Annex.

- Agree to remove “where the carrier indicated in sl-FreqInfoList is used for the RLC bearer if the SL-TxProfile of at least one associated QoS flow for the sl-ServedRadioBearer indicates backwardsCompatible”

[OPPO]: Agree that the current text is not clear because TX Profile is only associated with GC/BC while SRB is associated with UC. Simple suggestion is to remove “where the carrier indicated in sl-FreqInfoList is used for the RLC bearer if the SL-TxProfile of at least one associated QoS flow for the sl-ServedRadioBearer indicates backwardsCompatible”.

Proposal 2	Suggest to add “after receiving RRCReconfigurationCompleteSidelink”, and add”1/2/3” after SRB, adopt the TP-2 in annex clause.

- Agreed.

R2-2405857	Miscellaneous Rapporteur Stage 2 Corrections for NR Sidelink Evolution	InterDigital	CR	Rel-18	38.300	18.1.0	0838	2	F	NR_SL_enh2	R2-2403926

- Endorsed.

=> Revised in R2-2405901

[POST126][103][V2X/SL] (IDC)

Scope: To capture all agreements into 38.300.

Intended outcome: Stage 2 CR in R2-2405901.

Deadline: Short email discussion.

R2-2405901	Miscellaneous Rapporteur Stage 2 Corrections for NR Sidelink Evolution	InterDigital	CR	Rel-18	38.300	18.1.0	0838	3	F	NR_SL_enh2

=> Agreed

### 7.15.3	User plane corrections

Including MAC corrections. A single CR with miscellaneous corrections is requested; minor and editorial issues should be coordinated with the CR rapporteur and merged into the miscellaneous CR. Note RRC CR rapporteur’s summary and suggestion may be provided.

R2-2404838	Correction on Release-18 Sidelink evolution	LG	CR	Rel-18	38.321	18.1.0	1830	1	F	NR_SL_enh2-Core	R2-2403931

- Tick for Y for Other core specifications, and N for others.

- Endorsed in R2-2405897 with the change above.

[ZTE]: Tick is missed for “Other specs affected”.

R2-2405897	Correction on Release-18 Sidelink evolution	LG	CR	Rel-18	38.321	18.1.0	1830	2	F	NR_SL_enh2-Core

[POST126][102][V2X/SL] (LG)

Scope: To capture all agreements into 38.321.

Intended outcome: Approve MAC CR in R2-2405900.

Deadline: Short email discussion.

R2-2405900	Correction on Release-18 Sidelink evolution	LG	CR	Rel-18	38.321	18.1.0	1830	3	F	NR_SL_enh2-Core

=> Agreed

R2-2405685	Summary on user plane corrections	LG Electronics Inc.	discussion	NR_SL_enh2	Late

- Noted. We’ll take them into account for discussion.

TP for SL CSI Reporting MAC CE:

R2-2404357	Correction on Sidelink CSI reporting MAC CE	LG Electronics Inc.	discussion	Rel-18	38.321	NR_SL_enh2

Adding related UE behaviour in 5.22.1.11 (TX carrier selection)?

- Add related UE behaviour in 5.22.1.11 (TX carrier selection). TP in R2-2404357 is baseline, but the detailed wordings can be further discussed in CR preparation.

[Xiaomi]: Wonder why there is a case TX carrier selection is triggered for SL CSI reporting MAC CE? [LG]: In the current specification, TX carrier selection is triggered when SL grant is generated. It’s aligned with the current specification structure. [OPPO, Vivo]: Prefer simple spec impact, i.e. not capturing it into TX carrier selection. The consequence would be the UE may not get a grant for SL CSI reporting MAC CE. [Lenovo, Toyota]: If the UE may not get a grant for SL CSI reporting MAC CE, why we agreed per carrier SL CSI reporting MAC CE? It is quite natural to consider it in TX carrier selection.

Whether CBR condition is applied for SL CSI reporting MAC CE transmission?

- CBR condition is not applied for standalone SL CSI reporting MAC CE.

[LG]: CBR condition is not needed. [Nokia]: Prefer applying the existing CBR condition to SL CSI reporting MAC CE also. However, we don’t have CBR threshold for MAC CE transmission now, which we need further discussion how to configure the corresponding threshold. [Huawei]: For SL CSI reporting MAC CE, there is only single carrier. Even though CBR condition is not met, there is no other candidate carrier for it. It is good either CBR condition is not applied or leaves it to UE implementation. [Ericsson]: Agree with LG given SL CSI reporting MAC CE is small enough. [OPPO]: If CBR condition is not applied, does it include a case SL CSI reporting MAC CE is embedded with a STCH? [ZTE]: With the current spec, CBR condition is already checked in LCP, which means if MAC CE is included with a STCH, it already means CBR condition is met. [Lenovo]: Agree with ZTE.

Adding related UE behaviour in 5.22.1.7

- Add related UE behaviour in 5.22.1.7

[Nokia]: Whether the UE behaviour is per carrier and/or the timer maintenance is per carrier?

Whether the existing UE behaviour is updated to specify it’s per carrier operation?

- Yes. The detailed wording can be further discussed in CR preparation

Whether the sl-CSI-ReportTimer is maintained per carrier?

- Yes.

Note 2 is removed in 5.22.1.7?

- Yes.

Adding related restriction to LCP

- No LCP change.

[Huawei, Nokia]: Consider it’s an essential aspect to make sure per carrier CSI reporting MAC CE. [Ericsson, Apple, IDC, Qualcomm, LG]: Without LCP change, it still works. Prefer no LCP change.

[Ericsson]: We may send LS to inform RAN2 agreements to RAN1. [Nokia, OPPO]: The ball has been passed to RAN2. No specific RAN1 action is expected from RAN2 point of view. No need of the LS to RAN1.

- No LS to RAN1.

- Coordinate with RAN1 delegates internally (e.g. decision on sl-CSI-ReportTimer per carrier).

R2-2404217	Correction on SL CSI reporting MAC CE	Huawei, HiSilicon	discussion	Rel-18	NR_SL_enh2

R2-2404358	Discussion on CSI report for Carrier Aggregation	SHARP Corporation	discussion	NR_SL_enh2-Core

R2-2404385	SL CSI report	Nokia	discussion	NR_SL_enh2

R2-2405232	Discussion on specification impact on SL CSI report	ZTE Corporation, Sanechips, Ericsson	discussion	Rel-18	NR_SL_enh2

Re-evaluation and pre-emption for MCSt:

R2-2404210	Remaining issue on Re-evaluation/Pre-emption for MCSt	CATT	discussion

Proposal 1: Specify how MAC entity shall determine the number of consecutive slots used for re-evaluation or pre-emption in the MCSt case by down-selecting the following two options:

	Option 1: Only the resources in the first slot or the resources in all the slots of the Multi-consecutive slots transmission shall be re-evaluated or checked for pre-emption;

	Option 2: Only the resources in the first or last M consecutive slots shall be re-evaluated or checked for pre-emption.

Proposal 2: Specify the agreed option in Proposal 1 via normative texts in subclause 5.22.1.2a in MAC Spec.

P2 in R2-2404169

Proposal 2	R2 not pursue P2 in R2-2403047.

[CATT]: Do not see any compatibility issue. [LG]: In the current specification, which resource to be selected for re-evaluation and pre-emption is up to implementation. Do not see why we need to change it now. [CATT]: MCSt has new characteristics, which should be new compared to R16.

- P2 in R2-2404169 is agreed.

P2 in R2-2404892

Proposal 2a. For Multi-consecutive slots transmission on unlicensed band, if the UE succeeds in LBT for any anterior slot within the MCSt, the MAC layer does not perform resource reselection upon reception of the re-evaluation/pre-emption indication from the PHY layer for the remaining slots within the MCSt for maintaining the channel occupancy.

Proposal 2b. Adopt the TP2 in TS 38.321 for proposal 2a.

- Noted.

[OPPO]: It seems against the resource re-evaluation and pre-emption. Now, if the resource is reselected after re-evaluation/pre-emption, the resource is used for retransmission, but the proposal is not to use it if it was Acked. [Qualcomm]: For multi-slots MCSt, we should fill all TBs. If we break the rule, we need to ask RAN1. [OPPO]: Understand the proposal is for the case a slot was Acked, re-evaluation or pre-emption was already done, then resource reselection is not triggered for the slot, which is against the current relation between re-evaluation/pre-emption and resource reselection. [Xiaomi]: Feel sympathy, but consider a different way-out, e.g. UE needs to retransmit using the slot after re-evaluation/pre-emption only when a condition is met.

HARQ A/N for GC and PSFCH resources:

P4 in R2-2405231 (ZTE)

Proposal 4	It’s up to UE implementation to determine the number of candidate PSFCH resources, and adopt the TP-4 in annex clause.

P2 in R2-2404218 (Huawei)

Proposal 2: No spec impact is needed as such issue can be avoided by NW implementation (i.e. NW can configure same number of candidate PSFCH resource for different PSFCH occasions).

- P2 in R2-2404218 is agreed.

SL LBT Failure reporting:

P1 in R2-2404218 (Huawei)

Proposal 1: RAN2 to discuss the following options to ensure that SL LBT failures are able to be reported:

	To define the start point for evaluating “SL LBT failure MAC CE(s) has not been generated”. The corresponding TP is provided in Annex – TP1 (option 1).

	UE evaluates whether the SL LBT failure MAC CE for a certain RB set has not been generated, and consider SL LBT failure MAC CE for the RB set has not been generated after the SL C-LBT failure is recovered. The corresponding TP is provided in Annex – TP2 (option 2).

P1 in R2-2404497 (Ericsson)

Proposal 1	For SL LBT failure report in case of Mode 2, no further spec change is pursued.

- Update the condition in 5.31.2 as follow.

“1>	if SL consistent LBT failure has been triggered, and not cancelled, in the RB set(s), and SL LBT failure MAC CE for the triggered SL LBT Failure has not been generated;”

[Nokia, Lenovo, LG]: Agree with Huawei. There is an issue to be fixed. [LG]: Prefer TP2. Wonder if “(s)” is still needed to cover multiple MAC CEs. [Lenovo]: We can simply update the condition as follow then we don’t need a second sentence in TP2. “1>	if SL consistent LBT failure has been triggered, and not cancelled, in the RB set(s), and SL LBT failure MAC CE for the triggered SL LBT Failure has not been generated;”

Configuration of both IUC and Co-Ex:

P3 and P4 in R2-2404169 (OPPO)

Proposal 3	For co-configuration of IUC scheme-1 and Co-Ex, revise TS 38.321 clause 5.22.1.1, so that for preferred-resource + full-sensing case, UE firstly select resource, following the additional requirement for Co-Ex, within the intersection between IUC preferred resource and full sensing resource candidates reported by lower layer, if available. Otherwise (if no available resource in the intersection), UE perform resource selection by ignoring IUC preferred resource filtering.

- Will be revisited next meeting. MAC CR rapporteur will prepare the corresponding MAC changes for both IUC + Co-Ex and DRX + Co-Ex, then we can determine whether RAN2 spec impacts are acceptable or not. If it needs to introduce a new functional change, we may not support them.

[OPPO]: In addition to proposal 3, clarify actually we also need to consider preferred resource + random selection. [LG]: In the current MAC, there are so many if conditions that are related to IUC and Co-Ex. Think it is not easy to change whole parts of MAC. [Vivo]: It is good to see the changes before the next meeting. [LG]: Can prepare TP and share it before the next meeting.

Proposal 4	For co-configuration of IUC scheme-2 and Co-Ex, revise TS 38.321 clause 5.22.1.2b, so that UE select resource within the full sensing resource candidates reported by lower layer following the additional requirement for Co-Ex, by excluding the conflict resource(s) for the removed resource.

Co-Ex Correction:

R2-2404496	Correction to resource selection for LTE-NR cochannel scenario	Ericsson, LG Electronics Inc.	CR	Rel-18	38.321	18.1.0	1807	1	F	NR_SL_enh2	R2-2402946

- “randomly the time and frequency resources” is changed to “randomly select the time and frequency resources”

- Agreed in R2-2405898 with the change above.

[Toyota]: “randomly the time and frequency resources” should be changed to “randomly select the time and frequency resources”

Others:

P4 and P5 in R2-2404218 (Huawei)

Proposal 4: When the MAC entity selects carrier(s) among the candidate carriers with increasing order of CBR from the lowest CBR, it can skip CCs that would cause the aggregated bandwidth to exceed UE capability once selected, and continue with selecting carriers among other candidate carriers with increasing order of CBR.

Proposal 5: If proposal 4 is agreed, RAN2 to adopt TP3 for TS 38.321.

- Not pursued.

[OPPO, Nokia]: We haven’t captured any RF capability restriction. Consider the note “ NOTE 2:	It is left to UE implementation how many carriers to select based on UE capability.” is already helpful.

P1 and P2 in R2-2404319 (NEC)

Proposal 1	Not support the feature of LTE Uu control NR sidelink for both SL-U and SL-CA.

Proposal 2	It is suggested to agree the corresponding TP1 and TP2.

- Proposal 1 is agreed.

- TPs in R2-2404319 are baseline, but leaves detailed wordings to MAC CR rapporteur.

[Session chair]: Existing LTE ASN.1 or configuration can support SL-U or SL-CA? [NEC]: For SL-U, the UE needs to report some SL-U related information. [Qualcomm]: For LTE Uu controlled NR SL, both mode 3 and mode 4 are supported while for NR SL CA, only mode 2 is supported. [Huawei]: Agree with proposal 1.

R2-2404320	Clarification on SL DRX RTT timer for SL-U	SHARP Corporation	discussion	Rel-18

- TP is agreed, and detailed wordings is up to MAC rapporteur.

[LG]: There is some typo in TP.

P7 and P8 in R2-2404575 (Xiaomi)

Proposal 7: RAN2 to agree to delete the corresponding description on sl-lbt-FailureDetectionTimer upon BWP deactivation.

Proposal 8: RAN2 to agree with TP3 in Annex.

- TP3 is agreed.

P1 and P2 in R2-2405231 (ZTE)

Proposal 1	If legacy carrier is indicated to be used by upper layer, UE should select legacy carrier directly without taking the CBR restriction into account, and adopt the TP-1.

- Noted.

[OPPO]: In LTE V2X CA, it was discussed and the conclusion was not to handle special handling. [ZTE]: Backward compatibility was not there in LTE V2X CA. [OPPO]: There was similar issue to security message in LTE V2X CA. [CATT]: Share the intention. Alternative way is to exclude legacy carrier from TX carrier (re)selection if certain condition (e.g. before SL link establishment) is met. [CATT]: Before SL link establishment, upper layer may indicate only legacy carrier, then there would be no issue. [Session chair]: What will happen if CBR condition is not met for the legacy carrier? [CATT]: Then the UE will not perform transmission on the legacy carrier. [Session chair]: Ok, then it may work in different direction compared to the proposal. [Xiaomi]: Last meeting, we discussed if the carrier for QoS flow is not met, it was decided we’ll inform it to the upper layer. Assume the situation is similar. No need of special handling for legacy carrier. [CATT]: It’s ok to note it this meeting, but since this meeting is the last meeting for ASN.1, we need to be careful to introduce new parameter(s) from the next meeting.

Proposal 2	Add the parameter of sl-AllowedCarriers in the clause 5.22.1.4.1.1 to align the description in RRC and MAC specification, and adopt the TP-2.

- TP-2 is agreed.

P3 in R2-2405231 (ZTE):

Proposal 3	Delete the description of carrier mapping restrictions for destination selection if the UE is configured with SL relay discovery or A2X communication related resource pool, correct typo, and adopt the TP-3.

- TP-3 is agreed (with keeping NOTE 0)

[LG]: Ok to delete most of them, but NOTE 0 should be kept since it is also applied to SL communication.

P5 in R2-2405231 (ZTE):

Proposal 5	Add “associated to the LCID” in sidelink RLC entity establishment clause, agree the draft CR-4 in Annex.

- Noted.

[Huawei, NEC]: Think “corresponding” already means “associated to the LCID”.

R2-2405462	Discussion on carrier selection for SL MAC CE(s)	LG Electronics Inc.	discussion	NR_SL_enh2

=> Revised in R2-2405698

R2-2405698	Discussion on carrier selection for SL MAC CE(s)	LG Electronics Inc.	discussion	NR_SL_enh2

Proposal 1-1. UE selects a carrier on which the SL IUC Request was received as a carrier for transmission of a SL IUC Information MAC CE.

Proposal 1-2. Carrier selection of SL IUC Request MAC CE, Condition based SL IUC Information MAC CE and SL DRX command MAC CE uses the same procedure as the carrier selection procedure of logical channel data.

- P1-1 is agreed.

- Revisit P1-2 next meeting.

[Lenovo, Ericsson]: For P1-2, we don’t need any new thing. For P1-1, we can consider similar approach as SL CSI reporting MAC CE. [Vivo]: Support the P1-1 and P1-2. [Apple]: We need to be careful on the determined carrier for IUC since IUC was mainly led by RAN1. [Lenovo]: For P1-2, without this additional change, it can still work with some delay. We had some special handling for SL CSI reporting MAC CE because it is sensitive to delay. Not sure for others. [LG]: In the current MAC, if the grant already configured is not enough to add MAC CE, MAC will generate new grant for MAC CE transmission and it can trigger TX carrier (re)selection. If we follow what Lenovo mentioned, it is not aligned with the current MAC.

R2-2404169	Left issues on MAC	OPPO	discussion	Rel-18	NR_SL_enh2

R2-2404218	MAC corrections for SL evolution	Huawei, HiSilicon	discussion	Rel-18	NR_SL_enh2

R2-2404319	Discussion on remaining issue of TS 38.321	NEC  Corporation	discussion	Rel-18	NR_SL_enh2

R2-2404497	Discussion on remaining UP issues	Ericsson	discussion	Rel-18	NR_SL_enh2

R2-2404575	Correction on TS 38.321 for SL	Xiaomi	discussion

R2-2404892	Discussion on re-evaluation and pre-emption check for MCSt	vivo	discussion	Rel-18

R2-2405228	On group size and PSFCH occasions for SL-U	Nokia	discussion	NR_SL_enh2

R2-2405231	Discussion on remaining issues on user plane for SL evo	ZTE Corporation, Sanechips	discussion	Rel-18	NR_SL_enh2

## 7.16	Void

## 7.17	Dual Transmission/Reception (Tx/Rx) Multi-SIM for NR

(NR_DualTxRx_MUSIM-Core; leading WG: RAN2; REL-18; WID: RP-233071)

Time budget: 0 TU

Tdoc Limitation: 2 tdocs

### 7.17.1	Organizational

Rapporteur input, i.e., WI/Spec Rapporteur(s) are invited to provide updated open issues lists that need to be handled.

Incoming LS.

Corrections to TS 38.300.

R2-2404386	RILs_conclusion_MUSIM	vivo(Rapporteur)	other	Rel-18	NR_DualTxRx_MUSIM-Core

RILs with status ‘PropAgree’ are agreed.

RILs with status ‘PropReject’ are rejected.

R2-2404387	Correction on NR MUSIM enhancements	vivo	CR	Rel-18	38.331	18.1.0	4776	-	F	NR_DualTxRx_MUSIM-Core

R2-2404388	Correction on NR MUSIM enhancements	vivo	CR	Rel-18	38.331	18.1.0	4777	-	F	NR_DualTxRx_MUSIM-Core

- Vivo explains the 1st one is what we already endorsed, and the 2nd one can be considered as baseline for further discussions. Vivo explains some new changes based on Huawei new RILs have been taken into account in the 2nd CR.

Both CRs are noted. R2-2404388 is taken as baseline for further updates.

=> Revised in R2-2406076

Post meeting email disc

[Post126][203][MUSIM] CR for TS 38.331 (vivo)

Scope: Update and review the CR for TS 38.331, update the RIL list based on agreements in this meeting.

Intended outcome: Agreed CR and RIL list

Deadline: Short

R2-2406076	Correction on NR MUSIM enhancements	vivo	CR	Rel-18	38.331	18.1.0	4777	1	F	NR_DualTxRx_MUSIM-Core

=> Agreed

R2-2406075	RILs_conclusion_MUSIM	vivo	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

=> Agreed

### 7.17.2	RRC

Corrections to RRC (other than UE capabilities, which should be submitted to 7.17.3).

Discussions and proposals on the RRC open issues if listed by Rapporteur(s) or triggered by LSs, etc..

RILs

R2-2404610	[Z103][Z115][Z117] Discussion on MUSIM RILs	vivo	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

R2-2404719	[RIL Z116] [RIL Z103] Consideration on the MUSIM UAI Reporting	ZTE Corporation, Sanechips	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

R2-2404744	[RIL Z115] [RIL Z117] Correction to the MUSIM Gap Configuration	ZTE Corporation, Sanechips, Samsung	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

The 3 contributions above are noted.

Z103

- ZTE points out some condition may be needed based on offline discussion. vivo do not think the added condition ‘the UE supports the capability of the nr-NeedForGap-Reporting-r16’ needed.

Samsung and Huawei think vivo changes are ok.

- Ericsson think it is better to replace ‘measurement gap requirement information’ with some filed names.

Z103 is agreed, and changes proposed in R2-2404610 taken as baseline. Can further improve the wordings (e.g., replacing some parts with exact field names)

Z115 & Z117

Both RILs are agreed. TPs in R2-2404744 taken as baseline. Can further improve the wording.

Z116

- Vivo agrees, because different behaviours are needed for the two cases, i.e., the parent IE does not exist or it is reported but it’s empty. Xiaomi also agree, and think there is a typo. Ericsson also agree. Samsung wonders whether there is anything wrong.

- Samsung and Nokia think some further checking is needed.

R2-2405735	WF on [RIL Z116]	ZTE Corporation, Sanechips, vivo, Samsung, Huawei, HiSilicon, Xiaomi, Ericsson	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

P1, P2 and P3 are agreed.

Timer related

R2-2404706	Discussion on stopping of the wait timer	Huawei, HiSilicon	discussion

- Samsung and LG E think P3 is not needed. Vivo think it is OK.

For P3, only the change ‘transmitted -> indicated’ is agreed, other parts not pursued.

R2-2405537	Wait Timer Stop Handling	LG Electronics	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

- Nokia think Huawei P1&P2 and its TP is good.

- LG E think we already agree to include something to the procedure text. also ok with using Huawei TP as baseline.

TP for 5.3.5.3 from R2-2404706 is taken as baseline, with ‘resulting from the RRCReconfiguration’ removed. Can further improve the wording.

R2-2405642	Discussion on UE behavior upon T348 stop and T348 expiry 	Samsung Electronics Czech	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

Proposal #1: RAN2 to confirm which understanding is correct upon T348 expriy:

-	Understanding #1: UE autonomously releases dedicated configuration associated with musim-CapRestriction.

-	Understanding #2: UE still considers the latest configuration as the current configuration but does not perform any required behaviors about dedicated configuration associated with musim-CapRestriction.

Discussions:

- QC, Intel, Huawei think #2 is correct.

- Samsung want to check the intended UE behaviour for the case of PScell release, and think this in this case UE can just release the SCG configuration. ZTE think this create issue if later UE receive more configurations for SCG.

- Intel think SCG release is a corner case and do not require a normative behaviour. Nokia, CATT agree.

- Samsung think we can confirm #2 and we need to remove ‘releases SCG’ from the informative table.

?? Understanding #2 is confirmed.

?? ‘UE may apply the temporary capability restriction that SCG is not supported if ServCellIndex of PSCell was included in indicated MUSIM-CellToRelease-r18, UE releases SCG.’ Is removed from the informative table.

?? No other changes required to RRC spec.

Chair: can discuss in later stage, with the understanding that this is mainly about improving the informative table and it does not impact the RRC procedure text.

R2-2405689	Discussion on clarification of the action upon T348 expiry	China Telecom Corporation Ltd.	discussion	NR_DualTxRx_MUSIM-Core

Other issues

R2-2405641	Discussion on PSCell release for MUSIM operation  	Samsung Electronics Czech	discusion	Rel-18	NR_DualTxRx_MUSIM-Core

- ZTE support P1.

In the ASN.1 of the IE MUSIM-CellToRelease, change "SCellIndex" into "ServCellIndex".

R2-2405191	Additional capability restrictions related to measurement gaps	Nokia	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

- Xiaomi think this is not urgent. HW agree. HW think this requires a lot of change, and it is enhancement. ZTE agree and think even if we do not change UE can still report these gap info.

- QC think it is fine to try to include this.

- Nokia think this is not enhancement and it is about interworking with other gap features. Xiaomi think it is not about interworking issue, and think NW can anyway request the UE to report the necessary info.

Chair: after discussions there seem to be no sufficient support to do these changes.

Not pursued.

R2-2404242	Discussion on restriction of per FR/UE report for maximum CC number	Huawei, HiSilicon	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

- Samsung, ZTE, QC and Xiaomi support P1

For the restriction on maximum CC numbers, it is up to UE implementation to report per-FR level, or per-UE level, or per-FR level and per-UE level together, and no need to define any restriction on this in the specification. No RRC spec impact.

R2-2404745	Consideration on the Reconfiguration Failure Processing When T348 is Running	ZTE Corporation, Sanechips	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

R2-2404792	SpCells in MUSIM capability restriction signalling	Ericsson	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

R2-2404793	Intra-band CA in MUSIM capability restriction signalling	Ericsson	discussion	Rel-18	NR_DualTxRx_MUSIM-Core

R2-2405192	Clarification on DAPS Handover for MUSIM Dual TX/RX operation	Nokia	discussion

### 7.17.3	Other

UE capabilities related corrections.

Corrections to TS 37.340.

Other issues if not covered by the previous agenda items.

R2-2404478	Clarification to R18 MUSIM UE Capabilities	Huawei, HiSilicon	draftCR	Rel-18	38.306	18.1.0	NR_DualTxRx_MUSIM-Core

- ZTE suggest to improve the wording as ‘For a UE supporting nr-NeedForGap-Reporting-r16, this field also indicates UE supports providing musim-NeedForGapsInfoNR-r18 with temporary capability restriction as defined in TS 38.331 [9].’

The CR is endorsed in R2-2405731 for inclusion in the mega CR, with the update ‘for a UE supporting nr-NeedForGap-Reporting-r16, this field also indicates UE supports providing musim-NeedForGapsInfoNR-r18 with temporary capability restriction as defined in TS 38.331 [9].’

R2-2405731	Clarification to R18 MUSIM UE Capabilities	Huawei, HiSilicon	draftCR	Rel-18	38.306	18.1.0	NR_DualTxRx_MUSIM-Core

The CR is endorsed for inclusion in the mega UE cap CR, as per the previous agreement

## 7.18	Mobile Terminated Small Data Transmission

(NR_NR_MT_SDT-Core; leading WG: RAN2; REL-18; WID: RP-222993)

Time budget: 0 TU

Tdoc Limitation: 1 tdoc

### 7.18.1	Organizational

LS in, rapporteur input (e.g. rapporteur CR, open issues list)

R2-2404542	SDT ASN.1 RIL list	ZTE Corporation(rapporteur)	report

=>	The following RILs as agreed: E075, H071 and S081

R2-2404537	[E075] SDT corrections for harmonizing T319a and SDT ongoing labels	ZTE Corporation (rapporteur), Ericsson	CR	Rel-18	38.331	18.1.0	4687	1	F	NR_SmallData_INACTIVE-Core, NR_MT_SDT-Core	R2-2402757

=>	the CR is revised to update naming convention of cg-MT-SDT-MaxDurationToNextCG-Occasion

=>	The CR is agreed in R2-2405953 with the change above

R2-2405953	[E075] SDT corrections for harmonizing T319a and SDT ongoing labels	ZTE Corporation (rapporteur), Ericsson	CR	Rel-18	38.331	18.1.0	4687	2	F	NR_SmallData_INACTIVE-Core, NR_MT_SDT-Core

=> Agreed

R2-2404543	[S081] Clarification for order of list in PagingRecordList	ZTE Corporation(rapporteur), Samsung	CR	Rel-17	38.331	17.8.0	4785	-	F	LTE_NR_MUSIM-Core

=>	The CR is agreed

R2-2404544	[S081] Clarification for order of list in PagingRecordList	ZTE Corporation(rapporteur), Samsung	CR	Rel-18	38.331	18.1.0	4786	-	A	LTE_NR_MUSIM-Core

R2-2404545	[S081] Clarification for order of list in PagingRecordList	ZTE Corporation(rapporteur), Samsung	CR	Rel-18	38.331	18.1.0	4787	-	F	NR_MT_SDT-Core

=>	Merge R2-2404544 into this CR and keep it as Cat F

=>	The CR is agreed with this merge in R2-2405952

R2-2405952	[S081] Clarification for order of list in PagingRecordList	ZTE Corporation (rapporteur), Samsung	CR	Rel-18	38.331	18.1.0	4787	1	F	NR_MT_SDT-Core

=> Agreed

R2-2404778	Rapportuer MAC CR for MT-SDT	Huawei, HiSilicon	CR	Rel-18	38.321	18.1.0	1846	-	F	NR_MT_SDT-Core

=>	fix the naming convention in RRC

=>	The CR is not pursued

### 7.18.2	Others

Essential corrections only (including any topics)

R2-2404258	Correction of RA-SDT procedure	Ericsson	CR	Rel-18	38.321	18.1.0	1834	-	F	NR_MT_SDT-Core

-	Nokia indicates that we have used this ‘else if or’ in other places in the specification, so nothing is broken.   ZTE agrees with Nokia and even in RRC we use similar construct.

-	Huawei thought it is clear how it was changed and how it is written now is confusing.   LG has some sympathy in Ericsson proposal and would like to change in all other places.

-	Nokia thinks that this would then not be a Cat F CR.

=>	The two changes are equivalent but it is not clear whether the change is needed.

=>	Not pursued

R2-2404486	Correction on sdt-LogicalChannelSR-DelayTimer applicability	Nokia, Nokia Shanghai Bell	CR	Rel-18	38.321	18.1.0	1774	2	F	NR_SmallData_INACTIVE-Core, NR_MT_SDT-Core	R2-2403083

=>	The CR is agreed

SDT Topics from TEI-18 agenda items (moved from 7.24.x)

RAN3 LS related

R2-2404123	Reply LS on SDT signalling optimization for partial context transfer (R3-242198; contact: ZTE)	RAN3	LS in	Rel-18	TEI18	To:RAN2

R2-2404541	SDT signalling optimization for partial context transfer	ZTE Corporation, Sanechips	LS out	To:RAN3

Proposal: RAN2 should inform RAN3 that sending RRCSetup during SDT procedure would result in the data loss due release of all UP radio bearers and signalling overhead due to extra signalling needed to reestablish the security context and to reconfigure all the radio bearers

=>	Noted

R2-2404488	SDT signalling optimization	Nokia	discussion	Rel-18	TEI18

Observation: RAN2 solution for SDT signaling optimization is applicable for Rel-18 UEs supporting SDT.

Proposal: Reply to RAN3 LS as in [1] indicating that there is no RAN2 impact foreseen by the network-based solution, and this is supported already by Rel-17 SDT UEs.

=>	Noted

R2-2405649	Discussion on RAN3 LS on SDT signalling optimization for partial context transfer	Huawei, HiSilicon	discussion	Rel-18	TEI18

Proposal 1: RAN2 replies to RAN3 that:

1.	Fallback from RRC Resume procedure to RRC connection establishment makes the UE release its security context, all of its data radio bearers and its entire RRC configuration (except default values) causing service and data interruption as well as additional procedures to be triggered in both AS and upper layers.

2.	Fallback from RRC Resume procedure to RRC connection establishment is supposed to be utilized by the network only in exceptional cases, i.e. in case the retrieval of the UE context from the anchor gNB is not possible.

3.	The solution which was introduced by RAN2 is very simple and its additional complexity on top of baseline SDT functionality is negligible.

4.	The benefits of the NW-based solution proposed by RAN3 over the legacy mechanism (i.e. release the UE to RRC INACTIVE and perform Paging) are unclear.

Discussion

-	Ericsson agrees with observations of ZTE and Huawei.  The reply should be that there are no impacts to RAN2 and the concerns are already clear.

-	Qualcomm has similar understanding as ZTE and Huawei and the impact is that there is data loss.

-	Intel thinks that RRCSetup clears the data and it doesn’t serve the purpose that RAN3 is trying to achieve.

-	CATT supports Nokia proposals and thinks that NAS layer retransmission would solve the issue.

-	LG thinks that there is no spec impact but there are RAN2 impact which is data loss.

-	Nokia thinks that some of these signaling issues have been there since Rel-15, but the new SDT issue is data loss.

=>	Reply to LS:

-	No specification impact (for Rel-17 and 18)

-	However, there are RAN2 issues, like data loss (explain the issue)  and Uu signaling overhead due to security being cleared (explain that as from Rel-15 …)

[POST126][033][SDT] LS reply to RAN3 (ZTE)

Intended outcome: agree to LS by email

Deadline:  short

=> The LS is postponed

R2-2404489	Draft reply LS on SDT signalling optimization for partial context transfer	Nokia	LS out	Rel-18	TEI18	To:TSG RAN WG3

R2-2406007	Draft-Reply LS on SDT signalling optimization for partial context transfer	ZTE	LS out	Rel-18	TEI18	To:RAN3

-	Intel thinks that there are some spec impact.

Other topics

R2-2404546	Paging monitoring for extended CG period [CG-SDT-Enh]	ZTE Corporation, Sanechips, Mediatek, Qualcomm	CR	Rel-18	38.331	18.1.0	4788	-	F	TEI18

-	Ericsson agrees with this proposal as there are benefits

-	Huawei thinks this forces the UE to implement two different behaviours and wonders if this is for redcap UEs only.  LG agrees that we shouldn’t have different behaviour.  While T319 is not running the UE monitors paging.

-	ZTE explains that the issue is for RA-SDT hence the reason for separating.

-	Qualcomm indicates that the main issues is that the UE should monitor paging before T319a starts when there is a long gap.  Huawei thinks that we are already monitoring paging when T319a is not running.  ZTE is just linking this behavior to long gap.  But maybe we can tie to UE capability. Further the redcap UEs cannot monitor this and also Rel-17 has a different behavior. At least we need to cover that case.

=>	The CR is postponed

=>	Noted

R2-2404490	SDT BFR timer being not configured [RA-SDT_BeamFailure]	Nokia, Sony, Nokia Shanghai Bell	CR	Rel-18	38.321	18.1.0	1814	1	F	TEI18	R2-2403087

=>	The CR is agreed

R2-2405648	Correction for resumeIndication [SDT_ReleaseEnh]	Huawei, HiSilicon, Nokia, ZTE Corporation, Sanechips, Intel Corporation	CR	Rel-18	38.300	18.1.0	0848	1	F	TEI18	R2-2403108

=>	The CR is agreed

## 7.19	Enhanced support of reduced capability NR devices

(NR_redcap_enh-Core; leading WG: RAN1; REL-18; WID: RP-232671)

WI is declared 100% complete

Time budget: 0 TU

Tdoc Limitation: 1 Tdocs

### 7.19.1	Organizational

Incoming LSs, CR rapporteur’s miscellaneous non-controversial corrections, etc.

LSs

R2-2404113	Reply LS on 2-step RACH for eRedCap (R1-2403646; contact: Ericsson)	RAN1	LS in	Rel-18	NR_redcap_enh-Core	To:RAN2

Noted

R2-2404135	Reply LS on Rel-18 RedCap enhancements to address remaining ENs in TS 23.502 (S2-2405421; contact: Huawei)	SA2	LS in	Rel-18	NR_redcap_enh-Core	To:RAN2, RAN3, CT1	Cc:CT4

Noted

Misc

R2-2404240	Miscellaneous corrections on TS 38.304 for eRedCap	Huawei, HiSilicon	CR	Rel-18	38.304	18.1.0	0401	-	F	NR_redcap_enh-Core

Agreed

R2-2404443	Miscellaneous corrections on TS 38.321 for eRedCap	vivo (Rapporteur)	CR	Rel-18	38.321	18.1.0	1835	-	F	NR_redcap_enh-Core

Noted, covered by R2-2405974

R2-2405318	Miscellaneous corrections for eRedCap	Ericsson	CR	Rel-18	38.331	18.1.0	4729	2	F	NR_redcap_enh-Core	R2-2403861

Endorsed and to be updated

=> Revised R2-2405979

[Post126][755][eRedCap] Miscellaneous corrections for eRedCap (Ericsson)

Scope:

    - Produce agreeable CRs

Intended outcome:

    - Agreed CR in R2-2405979 (Ericsson)

Deadline:

    - Short

R2-2405979	Miscellaneous corrections for eRedCap	Ericsson	CR	Rel-18	38.331	18.1.0	4729	3	F	NR_redcap_enh-Core

=> Agreed

R2-2405319	RIL List for eRedCap - after RAN2#125bis	Ericsson	discussion	Rel-18	NR_redcap_enh-Core	R2-2403397

Noted

### 7.19.2	Papers related to RILs

Papers related to identified RILs

R2-2404431	Clarification on RIL V179 for eRedcap and proposed TP to RRC	Xiaomi Communications	discussion

-	LG agrees with the intention and is OK with this if other companies want this. Ericsson think we discussed this earlier and agreed to not add more details, and that we should rely on the NW to give a proper configuration. Xiaomi think that if we don’t clarify this the spec is unclear in the situations described in the paper.

Not pursued unless most companies change their mind and want this.

### 7.19.3	Other

Critical corrections, if any.

2-step for eRedCap

R2-2404241	Discussion on 2-step RACH for eRedCap	Huawei, HiSilicon	discussion	Rel-18	NR_redcap_enh-Core

R2-2405071	2-step RACH for eRedCap	NEC	discussion	Rel-18	NR_redcap_enh-Core

-	QC wants to support 2-step for eRedCap. Vivo think that RAN1 suggest a different solution than we have agreed in RAN2. LG do not have a strong opinion whether 2-step should be supported or not for eRedCap. Ericsson reminds that there is no RAN1 impact on supporting 2-step for eRedCap but still RAN1 wants to optimise this further, instead we should not support 2-step for eRedCap. ZTE agrees that there is no impact in RAN1 so they are surprised that RAN1 just didn’t support 2-step eRedCap resources, it would be cleanest if we would just make RAN1 to support 2-step eRedCap resources. ZTE would like to support 2-step for eRedCap. Nokia thinks we already made a decision that if RAN1 does not add 2-step eRedCap resources then we should not support 2-step for eRedCap UEs.

We do not support eRedCap 2-step RA.

[AT126][763][eRedCap] 2-step RA for eRedCap (vivo)

Scope:

    - Produce agreeable CRs and LS if needed

Intended outcome:

    - Agreed CRs in R2-2405843 and R2-2405844 (vivo)

    - Approvable LS in R2-2405845 if needed (vivo)

Deadline:

    - Friday

R2-2405843	Miscellaneous corrections on TS 38.321 for eRedCap	vivo (Rapporteur)	CR	Rel-18	38.321	18.1.0	1835	1	F	NR_redcap_enh-Core

Remove the first change, update cover page accordingly, agreed unseen in R2-2405974.

R2-2405974	Miscellaneous corrections on TS 38.321 for eRedCap	vivo (Rapporteur)	CR	Rel-18	38.321	18.1.0	1835	2	F	NR_redcap_enh-Core

=> Agreed

=> Coversheet update by MCC: wrong rev value

R2-2406127	Miscellaneous corrections on TS 38.321 for eRedCap	vivo (Rapporteur)	CR	Rel-18	38.321	18.1.0	1835	3	F	NR_redcap_enh-Core

=> Agreed

R2-2405844	Correction on TS 38.331 for 2-step RACH for eRedCap	vivo	CR

Noted

R2-2405845	[Draft] Reply LS on 2-step RACH for eRedCap	vivo

Noted

Discussion on the thee papers above:

-	NEC wonders if RRC is really needed, MAC should be enough and is simpler.

R2-2404444	Discussion on remaining issues for eRedCap	vivo, Guangdong Genius	discussion	Rel-18	NR_redcap_enh-Core

R2-2404901	MAC corrections for supporting 2-step RACH for eRedCap	ZTE Corporation, Sanechips	discussion	Rel-18	NR_redcap_enh-Core

R2-2405326	Discussion on 2-step RA for eRedCap UEs	Ericsson	discussion	Rel-18	NR_redcap_enh-Core

R2-2405333	Draft LS reply on 2-step RA for eRedCap	Ericsson	LS out	Rel-18	NR_redcap_enh-Core	To:RAN1

CFRA for eRedCap

R2-2405540	On CFRA procedure for eRedCap UE	LG Electronics Inc., Xiaomi	discussion	Rel-18	NR_redcap_enh-Core

Proposal 1. In order to align the CFRA and CBRA procedure for eRedCap UE, do not support fallback from eRedCap to Redcap is supported for CFRA, once RRC indicates that eRedCap is applicable and RedCap is not applicable for eRedCap UE.

Proposal 2. Adopt the TP in Annex A.

R2-2404515	RA issues on eRedCap	Nokia	discussion	NR_redcap_enh-Core

Focus on P2.

Proposal 2: In selection of the set of RA resources for CFRA, no specification change is needed for eRedCap.

In selection of the set of RA resources for CFRA, no specification change is needed for eRedCap.

Barring

R2-2404471	Correction on (e)Redcap 1 Rx and 2 Rx barring	Nokia	CR	Rel-18	38.331	18.1.0	4632	3	F	NR_redcap-Core, NR_redcap_enh-Core	R2-2403841

The change is agreeable. The eRedCap parts are included in the eRedCap RRC CR (R2-2405979). The RedCap part captured in R2-2405970 (a revision of R2-2404471)

R2-2405970	Correction on Redcap 1 Rx and 2 Rx barring	Nokia	CR	Rel-18	38.331	18.1.0	4632	4	F	NR_redcap-Core	R2-2404471

Agreed

=> Coversheet update by MCC: NR_redcap-Core is a Rel-17 WI, It needs TEI18 to accompany it to make this CR a Rel-18 CR.

R2-2406142	Correction on Redcap 1 Rx and 2 Rx barring	Nokia	CR	Rel-18	38.331	18.1.0	4632	5	F	NR_redcap-Core, TEI18

=> Agreed

## 7.20	NR MIMO evolution

(NR_MIMO_evo_DL_UL-Core; leading WG: RAN1; REL-18; WID: RP-233028)

Time budget: 0TU

Tdoc Limitation: 2 tdoc

### 7.20.1	Organizational

Rapporteur input, i.e., WI/Spec Rapporteur(s) are invited to provide updated open issues lists that need to be handled.

Incoming LS.

Stage 2 corrections.

R2-2404215	Correction to MIMO Evolution	Ericsson	CR	Rel-18	38.331	18.1.0	4775	-	F	NR_MIMO_evo_DL_UL-Core

- Ericsson think this is basically the endorsed version, with one editorial change.

Noted, will be updated after the meeting.

=> Revised in R2-2406031

R2-2405589	Clarification of PDCCH ordered CFRA for 2TA	NTT DOCOMO, INC., Samsung	draftCR	Rel-18	38.300	18.1.0	F	NR_MIMO_evo_DL_UL-Core

- Xiaomi think the intention is OK, but suggest to change ‘second TA’ to ‘TA of another TAG’.

- Ericsson think the wording can improve.

- Nokia think this is not entirely correct and think the TA can also be obtained via CBRA. Samsung think this for the case when the timing of ‘2nd TA’ is initially established.

=> Updated in R2-2405732

R2-2405732	Clarification of PDCCH ordered CFRA for 2TA	NTT DOCOMO, INC., Samsung, Nokia	CR	Rel-18	38.300	18.1.0	0868	-	F

Agreed

Post meeting email disc

[Post126][204][MIMOevo] CR for TS 38.321 (Samsung)

Scope: Update and review the CR for TS 38.321.

Intended outcome: Agreed CR

Deadline:  Short

R2-2406031	Correction to MIMO Evolution	Ericsson	CR	Rel-18	38.331	18.1.0	4775	1	F	NR_MIMO_evo_DL_UL-Core

=> Coversheet updated by MCC in R2-2406128: "Clauses affected" empty, in "other specs affected" an incorrect reference

R2-2406128	Correction to MIMO Evolution	Ericsson	CR	Rel-18	38.331	18.1.0	4775	2	F	NR_MIMO_evo_DL_UL-Core

=> Agreed

R2-2406073	Miscellaneous Corrections on MIMOevo	Samsung	CR	Rel-18	38.321	18.1.0	1832	1	F	NR_MIMO_evo_DL_UL-Core

=> Agreed

[Post126][205][MIMOevo] CR for TS 38.331 (Ericsson)

Scope: Update and review the CR for TS 38.331, update the RIL list based on agreements in this meeting.

Intended outcome: Agreed CR and RIL list

Deadline:  Short

R2-2406032	RIL-List Q2 v147_Post126	Ericsson	discussion	Rel-18	NR_MIMO_evo_DL_UL-Core

=> Agreed

### 7.20.2	MAC

Corrections to MAC.

Discussions and propsoals on the open issues if listed by Rapporteur(s) or triggered by LSs, ect..

R2-2405171	Corrections on PHR	Samsung	discussion	Rel-18	NR_MIMO_evo_DL_UL-Core

Proposal 1: For Rel-18 multi-entry PHR MAC CE for STx2P, if conditions for PHR are met,

-	if the active BWP of a serving cell is configured with multipanelSchemeSDM or multipanelSchemeSFN for twoPHRmode, two type 1 PH values and the corresponding two Pcmax values are reported;

-	if the active BWP of a serving cell is configured with multiple TRP PUSCH repetition (i.e., not configured with multipanelSchemeSDM or multipanelSchemeSFN) for twoPHRmode, two type 1 value and the corresponding Pcmax are reported; (whether one type 3 PH instead of two type 1 values is reported can be discussed separately which depends on RAN1 reply)

-	otherwise, one type 1 or type 3 PH value and the corresponding Pcmax value are reported as Rel15/16.

Discussions:

- OPPO think it is safer to wait for R1 reply. LG E think P1 is aligned with R1 spec, so agreeable.

- LG E think with the TP for P1, there is misalignment btw R1 and R2 spec.

- ZTE think the TP should reflect ‘active BWP of a serving cell’.

- QC ok with P1 and think detailed changes can be further checked.

RAN2 assume P1 is agreeable, unless otherwise informed by R1 reply LS. Detailed changes can be checked further.

Proposal 2-1: In PHR procedure, the procedure similar to multi-entry PHR is applied for single-entry PHR, to capture “instruct the Multiplexing and Assembly procedure to generate and transmit the Enhanced Single Entry PHR for multiple TRP STx2P MAC CE”.

Proposal 2-2: For proposal 2-1, adopt the TP in Appendix 5.2 for TS 38.321 clause 5.4.6.

P2-1 is agreed. TP in Appendix 5.2 for TS 38.321 clause 5.4.6 is taken as baseline. Detailed changes can be checked further.

Proposal 3-1: Reporting one type 1 PH or one type 3 PH is not applicable if the serving cell is configured with multi-panel scheme and the associated MAC entity is configured with twoPHRmode.

Proposal 3-2: For proposal 3-1, adopt the TP in Appendix 5.3 for TS 38.321 clause 5.4.6.

- OPPO think it should be ‘if the active BWP is configured with….’ Samsung agrees.

- LG E think we need new procedural texts for multi panel case.

Proposal 4-1: In PHR procedure, for STx2P multi-entry PHR, move the steps of obtaining PCMAX,f,c,k and MPEk under the following conditions:

if this MAC entity has UL resources allocated for transmission on this Serving Cell; or

if the other MAC entity, if configured, has UL resources allocated for transmission on this Serving Cell and phr-ModeOtherCG is set to real by upper layers

Proposal 4-2: For multi-entry PHR, restructure the procedural text for obtaining Pcmax and MPE, by using the structure of single-entry PHR MAC CEs.

Proposal 4-3: For Proposal 4-1 and 4-2, adopt the TP in Appendix 5.4 for TS 38.321 clause 5.4.6.

Proposal 5: For STx2P multi-entry MAC CE with 8 serving cells and with 32 serving cell, two bitmaps are added:

•	one with each bit indicating whether the octet containing the second PH value is present or not for a reported serving cell

•	the second one with each bit indicating whether the octet containing the second Pcmax is present or not for a reported serving cell.

Discussions:

- ZTE think only one bitmap is needed, and do not want to change the current behaviour.

- CATT think there is no need to change, if we rely on inter node msg. LG E, OPPO, QC, Ericsson agree.

R2-2404555	Remaining issues on STx2P PHR	LG Electronics Inc.	discussion	Rel-18	NR_MIMO_evo_DL_UL-Core

Noted

Discuss the remaining issues for MAC in offline, and check the proposals in CB.

[AT126][201][MIMOevo] Offline discussion on the remaining MAC issues (Samsung)

Scope:  Discuss the remaining critical MAC issues that need to be handled in this meeting

Intended outcome: Summary and agreeable proposals in R2-2405733

Deadline:  before CB session

R2-2405733	Report of [AT126][201][MIMOevo] remaining MAC issues (Samsung)	discussion	Rel-18	NR_MIMO_evo_DL_UL-Core

Remove the PH 2 for Type 2 PH reporting from the figures of the Enhanced Multiple Entry PHR for multiple TRP STx2P MAC CE.

When the MAC entity transmitting the PHR is not configured with twoPHRmode, for Rel-18 multi-panel PHR, apply current Rel-17 procedure that report one type 1 PH with possible wording changes in the current steps (e.g., clarify/modify “first transmission” considering simultaneous tx for multi panel case, etc)

It’s up to NW implementation to make sure the current Rel-18 multi-entry PHR MAC CE format for STx2P can work (e.g., for dynamic BWP switch).

RAN2 assumes the inter-node msg can be used to make sure the current Rel-18 multi-entry PHR MAC CE format for STx2P can work for DC, i.e., need RRC changes (e.g,  by new parameter in inte-node msg)

The following will be included in the post meeting email disc for RRC CR: Introduce a new parameter in inter-node msg, e.g., twoSRS-MultipanelScheme, (similar to twoSRS-PUSCH-Repetition-r17), that indicates whether the indicated serving cell is configured with multiple panel simultaneous uplink transmission schemes of multipanelSchemeSDM or multipanelSchemeSFN corresponding to two SRS resource sets configured in either srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage 'codebook' or 'noncodebook'. The new parameter and twoSRS-PUSCH-Repetition-r17 are not configured together for a serving cell.

Discussion on P5:

- ASUSTeK suggest to add ‘other changes if needed, can be discussed’. Xiaomi do not see a need for further discussion. Nokia, ZTE ok to further check. Ericsson think we can discuss based on company contributions.

For 8Tx: Capture in MAC for UL HARQ “Each HARQ process supports one or two TBs.”

Proposal 6: For RIL C520, due to different views on whether/how to further clarify anything, no more changes for now and keep the changes in last meeting.

- Ericsson think this can be postponed, and it’s going to be BC change if there need any changes.

- Samsung think we already discussed on this, so would like to conclude and reject this RIL. Ericsson think we can reject this. LG E ok to reject.

- CATT wonders if we reject this, what if we found there is an issue. ZTE think we can close this RIL.

- LG E think the proposal from CATT is for NW config.

C520 is rejected.

R2-2404374	Discussion on PHR-Related Issues for STx2P	CATT	discussion	Rel-18	NR_MIMO_evo_DL_UL-Core

R2-2404487	Correction on multi-TRP STx2P PHR MAC CE	Nokia	discussion	Rel-18	NR_MIMO_evo_DL_UL-Core

R2-2405182	Cosideration On PHR and PHR MA CE for STxMP	ZTE Corporation	discussion	Rel-18	NR_MIMO_evo_DL_UL-Core

R2-2405489	RAN4 impacts of 2TA for SDT	Xiaomi	discussion	Rel-18	NR_MIMO_evo_DL_UL-Core

R2-2405426	Discussion on introducing 8Tx in MAC specification	ASUSTeK	discussion	Rel-18	38.321	NR_MIMO_evo_DL_UL-Core

The 5 contributions above are noted

### 7.20.3	RRC

Corrections to RRC, RILs.

Discussions and proposals on the open issues if listed by Rapporteur(s) or triggered by LSs, ect..

R2-2404214	Remaining aspects on RRC for MIMOevo	Ericsson	discussion	Rel-18	NR_MIMO_evo_DL_UL-Core

R2-2405222	[H169] Configuration of RACH for MIMO with 2TA	Huawei, HiSilicon	discussion	Rel-18	NR_MIMO_evo_DL_UL-Core

- Ericsson suggest to discuss the HW paper.

R2-2404375	[C520] [C521] [C522] [C523] [C524] Discussion on RRC Corrections for MIMO	CATT	discussion	Rel-18	NR_MIMO_evo_DL_UL-Core

- Ericsson not sure whether P1 is needed in RRC, think it can be discussed in MAC.

Chair: document missing for Rapp’s RIL resolution proposal.

Discuss the remaining issues for RRC in offline, and check the proposals in CB.

[AT126][202][MIMOevo] Offline discussion on the remaining RRC issues (Ericsson)

Scope:  Discuss the remaining critical RRC issues that need to be handled in this meeting

Intended outcome: Summary and agreeable proposals in R2-2405734

Deadline:  before CB session

R2-2405734		[AT126][202][MIMOevo] Offline discussion on the remaining RRC issues (Ericsson)		discussion	Rel-18	NR_MIMO_evo_DL_UL-Core

ToAgree: RILs C524, C521, C522

ToReject: RILs H152 H196

In current RRC signaling: replace rach-ConfigTwoTAIndex-r18 with AdditionalPCIIndex-r17 (with proper description), clarify how to release of RACH config: explicit by network

Adopt the TP in Annex of ZTE R2-2405183 (fix typos)

Adopt the TP in Annex of Huawei R2-2405690

R2-2405172	Clarification on UE capability enquiry with codebook type request	Samsung	discussion	Rel-18	NR_MIMO_evo_DL_UL-Core

- HW think the description might also impact R17. Samsung explains that if we go Approach 1 then no impact to R17.

Noted

R2-2405183	Consideration on 2TA RRC Aspect	ZTE Corporation	discussion	Rel-18	NR_MIMO_evo_DL_UL-Core

R2-2405690	Aperiodic CSI report with 2 resources for channel measurement and unified TCI framework	Huawei, HiSilicon	discussion	Rel-18	NR_MIMO_evo_DL_UL-Core

The 2 contributions above are noted.

## 7.21	Further NR coverage enhancements

(NR_cov_enh2-Core; leading WG: RAN1; REL-18; WID: RP-221858)

Time budget: 0 TU

Tdoc Limitation: 1 tdoc

### 7.21.1	Organizational

Incoming LSs, Rapporteur input etc.

Editorials/clarifications should not be included in any tdoc but sent to the WI spec rapporteurs, who can submit a rapporteur CR as part of this AI.

Rapporteur inputs and other pre-assigned documents in this AI do not count towards the tdoc limitation.

R2-2405056	Micellaneous MAC corrections for CE	ZTE Corporation	CR	Rel-18	38.321	18.1.0	1851	-	F	NR_cov_enh2-Core

- Rapporteur informs that no feedback was received during the email discussion

Use as baseline for updates from this meeting

Will be sent to post meeting email (for plenary)

=> Revised in R2-2406093

[AT126][802][CE_enh]  updated MAC CR (ZTE)

Scope: Update the MAC CR with the latest agreements and review

Expected output: Final MAC CR for plenary

Deadline: Short (for plenary)

R2-2406093	Micellaneous MAC corrections for CE	ZTE Corporation	CR	Rel-18	38.321	18.1.0	1851	1	F	NR_cov_enh2-Core

=> Agreed

R2-2405387	Updated RIL List for CE (based on v102)	Huawei, HiSilicon	discussion	NR_cov_enh2-Core

M871, M872, E169 : Agreed

S831: Agreed

=> Revised in R2-2406023

R2-2405388	RRC CR for agreed Coverage Enhancements RILs	Huawei, HiSilicon	CR	Rel-18	38.331	18.1.0	4833	-	F	NR_cov_enh2-Core

- Rapporteur informs that changes related S831 are not yet reflected in the running RRC.

- ZTE: for SI request there is a issue also for Rel-17 and have submitted a CR to Rel-17 with Cat A CR and there will be in issue if that is agreed.

Use as baseline for updates from this meeting

Will be sent to post meeting email (for plenary)

=> Revised in R2-2406024

[AT126][801][CE_enh]  updated RRC CR and RIL list (Huawei)

Scope:

Update the RRC CR with the latest agreements and review

Updated RIL list according the agreements from the meeting

Expected output:

Final RRC CR for plenary

Updated RIL list

Deadline: Short (for plenary)

R2-2406023	Updated RIL List for CE	Huawei, HiSilicon	discussion	NR_cov_enh2-Core

=> Noted

R2-2406024	Miscellaneous RRC corrections for Coverage Enhancements	Huawei, HiSilicon	CR	Rel-18	38.331	18.1.0	4833	1	F	NR_cov_enh2-Core

=> Agreed

### 7.21.2	Other Essential corrections

General Discussion on which IE (rach-ConfigCommon) to use to initialize the RACH parameters for SI request/PDCCH order/CFRA with/without MSG1 repetition

R2-2405389	[S831] Discussion on SI-RequestConfig for MSG1 repetition	Huawei, HiSilicon	discussion	NR_cov_enh2-Core

Proposal 1: For non-REDCAP specific initial BWP, UE use the parameters (e.g. rach-ConfigGeneric and ssb-perRACH-Occasion) in rach-ConfigCommon associated with the same repetition number only in case that rach-OccasionsSI is absent.

Proposal 1bis: For non-REDCAP specific initial BWP, UE use the other parameters (e.g. prach-RootSequenceIndex and msg1-SubcarrierSpacing etc) in rach-ConfigCommon associated with the same repetition number only regardless of whether or not rach-OccasionsSI is absent.

Proposal 2: For REDCAP specific initial BWP, UE use the parameters (e.g. rach-ConfigGeneric and ssb-perRACH-Occasion) in rach-ConfigCommon associated with the same repetition number and (e)RedCap indication only in case that rach-OccasionsSI is absent.

Proposal 2bis: For REDCAP specific initial BWP, UE use the other parameters (e.g. prach-RootSequenceIndex and msg1-SubcarrierSpacing etc) in rach-ConfigCommon associated with the same repetition number and (e)RedCap indication only regardless of whether or not rach-OccasionsSI is absent.

Proposal 3: clarify in SI-RequestConfigRepetition description that the UE then applies the corresponding parameters depending on the RACH resource set selected for SI request with repetition upon RACH initialization and a reference is added to MAC.

- Huawei clarify that there are some parameters (e.g. prach-RootSequenceIndex and msg1-SubcarrierSpacing etc) that are used from rach-ConfigCommon and this needs some clarification.

=> Noted

R2-2404948	[S831] Discussion on the remainging issues on Msg1 repetition	CATT	discussion	Rel-18	NR_cov_enh2-Core

Proposal 1: For SI request with MSG1 repetition on the non-REDCAP specific initial BWP, if rach-OccasionsSI is not configured, UE uses the rach-ConfigCommon associated with msg1-Repetitons feature only and with the same repetition number.

Proposal 2: For SI request with msg1 repetiton on Redcap-specific intial BWP, if rach-OccasionsSI is not configured, UE uses the rach-ConfigCommon associated with the same msg1 repetition number in initialUplinkBWP-RedCap.

Proposal 3: Introduce a new configure condition for rach-OccasionsSI in SI-RequestConfigRepetition to guarantee that only when there is available configuration of rach-ConfigCommon associated with same msg1 repetition number on initial BWP or Redcap specific intial BWP, the rach-OccasionsSI in SI-RequestConfigRepetition can be configured as optional, similiarly as 4StepCFRArep.

Proposal 4: It needed to make the clarification for (e)Redcap UEs from Rel 17 that if rach-OccasionsSI is not configured, the (e)Redcap UE uses the corresponding parameters configured in rach-ConfigCommon of the Redcap-specific initial uplink BWP.

=> Noted

R2-2405057	RACH-ConfigCommon for SI request with Msg1 repetition	ZTE Corporation	discussion	Rel-18	NR_cov_enh2-Core

Proposal 1	For all the SI request cases (irrespective of Msg1 repetition and RedCap), if rach-OccasionsSI is not configured in corresponding SI request configuration, UE applies the RACH-ConfigCommon that is not associated with any feature. (Apply the same principle for Rel-17 and Rel-18, revert previous RAN2 agreement)

=> Noted

R2-2405541	On RACH partition selection on RedCap-specific initial BWP for Msg1-based SI request	LG Electronics Inc.	discussion	Rel-18	NR_cov_enh2-Core

Proposal 1. When a RedCap UE performs Msg1-based SI request with Msg1 repetition in RedCap-specific initial UL BWP,  RACH partition associated with RedCap indication and Msg1 repetition indication with the same repetition number should be selected.

Proposal 2. When an eRedCap UE performs Msg1-based SI request with Msg1 repetition in RedCap-specific initial UL BWP,  RACH partition is selected as follows:

-	If the RACH partition associated with RedCap indication and Msg1 repetition indication with the same repetition number is configured, this RACH partition should be selected

-	Otherwise, RACH partition associated with eRedCap indication and Msg1 repetition indication with the same repetition number should be selected.

Rel-17 proposal??

Proposal 3. When a RedCap UE performs Msg1-based SI request (without Msg1 repetition) in RedCap-specific initial UL BWP,  RACH partition is selected as follows:

-	If the RACH partition associated with RedCap indication is configured, this RACH partition should be selected

-	Otherwise, legacy RACH partition should be selected.

Proposal 4. When an eRedCap UE performs Msg1-based SI request (without Msg1 repetition) in RedCap-specific initial UL BWP,  RACH partition is selected as follows:

-	If the RACH partition associated with RedCap indication is configured, this RACH partition should be selected.

-	Otherwise,

	If RedCap is not applicable for eRedCap UE, RACH partition associated with eRedCap indication is selected.

	Otherwise (i.e., if RedCap is applicable for eRedCap UE), legacy RACH partition is selected.

=> Noted

Common DISCUSSION on above papers (R2-2405389, R2-2404948, R2-2405057, R2-2405541)

- Samsung agree with Huawei proposals. For SI request there is no need to differentiate between RedCap and eRedCap. So, Huawei proposal is sufficient.

- Huawei think that there is an issue for separate RO case.

- LG explain that for RedCap specific BWP, RACH configuration for RedCap specific RACH partition will be selected if configured. But for eRedCap there may be some issue.

- ZTE explain that this is related to the Rel-17 discussions and explain that it is better to have unified solution. There are solutions proposed for Rel-17

- Samsung want to focus on Rel-18 and agree on Rel-18 even if it is different to Rel-17 and explain that it is okay to have a different solution since the partition for MSG1 is only introduced from Rel-18 onwards.

- Ericsson point out that there is open discussion for RedCap and we may need to revisit these agreements if they are not compatible with the agreements made there.

For SI request with MSG1 repetition:

Options 1: Do same as Rel-17

Option 2: Update Rel-17 and do same in Rel-18

Option 3: Do something different in Rel-18 for SI request but keep Rel-17 as it is

If any issues are found for the agreements below for (e)RedCap, after the discussion in the common session, we can comeback.

For non-REDCAP specific initial BWP, UE (both RedCap and non-RedCap UEs) use the parameters (e.g. rach-ConfigGeneric and ssb-perRACH-Occasion) in rach-ConfigCommon associated with the same repetition number only in case that rach-OccasionsSI is absent.

- ZTE are concerned that with the above agreement, we have different considerations for different features and is very complex. So, we should follow Rel-17 to have a common solution.

For non-REDCAP specific initial BWP, UE (both RedCap and non-RedCap UEs) use the other parameters (e.g. prach-RootSequenceIndex, msg1-SubcarrierSpacing etc) in rach-ConfigCommon associated with the same repetition number only regardless of whether or not rach-OccasionsSI is absent.

- LG explain that with the above agreement, different time offset may be needed for different RO because the RO density may be different, but indicate that they are okay with the agreement.

For REDCAP specific initial BWP, UE use the parameters (e.g. rach-ConfigGeneric and ssb-perRACH-Occasion) in rach-ConfigCommon associated with the same repetition number and (e)RedCap indication only in case that rach-OccasionsSI is absent.

- For the above, LG wonder if there is a need to differentiate between eRedCap + MSG1 repetition and RedCap+Msg1 repetition? But LG are okay with the above agreement

For REDCAP specific initial BWP, UE use the other parameters (e.g. prach-RootSequenceIndex and msg1-SubcarrierSpacing etc) in rach-ConfigCommon associated with the same repetition number and (e)RedCap indication only regardless of whether or not rach-OccasionsSI is absent.

Time offset for CFRA/SI request with Msg1 repetition

R2-2404156	Correction on CFRA with Msg1 Repetition	vivo	discussion	Rel-18	NR_cov_enh2-Core

Proposal 1: RAN2 confirms msg1-RepetitionTimeOffsetROGroup is applicable for CFRA with Msg1 repetition.

Proposal 2: RAN2 to discuss the following options, if it is confirmed that msg1-RepetitionTimeOffsetROGroup is applicable for CFRA with a given repetition number:

-	Option 1: msg1-RepetitionTimeOffsetROGroup associated with the same repetition number in FeatureCombinationPreambles is applied.

-	Option 2: msg1-RepetitionTimeOffsetROGroup can be indicated together with msg1-RepetitionNum via RACH-ConfigDedicated when dedicated ROs are configured for CFRA. If absent, two neighbour RO groups are consecutive.

DISCUSSION

- Ericsson, Huawei, Chinatelecom, Samsung, ZTE support option 1.

- LG and ETRI think that RO density is different for CBRA and CFRA and separate configuration may be beneficial.

For both RedCap and non-RedCap UEs, msg1-RepetitionTimeOffsetROGroup associated with the same repetition number in FeatureCombinationPreambles is applied (for both CFRA and SI request)

R2-2405583	Discussion on time offset of RO group for CFRA with Msg1 repetition	China Telecom	discussion	Rel-18	NR_cov_enh2-Core

Proposal 1: Time offset of RO group can be applied to CFRA with Msg1 repetition. No additional spec change is needed.

=> Noted

R2-2405068	Remaining issues for preamble repetitions	ETRI	discussion

Proposal 3: Introduce msg1-RepetitionTimeOffsetROGroup-r18 in CFRA.

Proposal 4: Introduce msg1-RepetitionNumTimeOffsetROGroup-r18 in SI-RequestConfigRepetition-r18 for each repetition number.

=> Noted

Other issues

R2-2404225	Correction to PHR MAC CE Design for assumed PUSCH reporting	Samsung, Nokia, Nokia Shanghai Bell, Lenovo, NEC, Interdigital, Ericsson, Qualcomm, Vivo	discussion	Rel-18	NR_cov_enh2-Core

Proposal 1: For the Multiple Entry PHR with assumed PUSCH MAC CE:

- Ek indicates the presence of a PCMAX,f,c for assumed PUSCH field of Serving Cells for which Ci field is set to 1 and PCell.

- The Serving Cells for which Ci field is set to 1 and PCell are indexed sequentially starting with PCell and followed by other Serving Cells in ascending order of ServCellIndex i.

- The Ek field set to 1 indicates that a PCMAX,f,c for assumed PUSCH field for the kth Serving Cell is reported.

DISCUSSION

- Huawei, CATT still think that this solution has limited applicability. The TP changes the definition of the Ei.

- Chair wonders if there are any strong objections since there is strong support and no technical concerns? Some concerns expressed but seems this may be acceptable as there are no strong objections.

For the Multiple Entry PHR with assumed PUSCH MAC CE:

- Ek indicates the presence of a PCMAX,f,c for assumed PUSCH field of Serving Cells for which Ci field is set to 1 and PCell.

- The Serving Cells for which Ci field is set to 1 and PCell are indexed sequentially starting with PCell and followed by other Serving Cells in ascending order of ServCellIndex i.

- The Ek field set to 1 indicates that a PCMAX,f,c for assumed PUSCH field for the kth Serving Cell is reported.

R2-2404259	Discussion on remaining issues for Coverage Enhancements	Ericsson	discussion	Rel-18	38.321	NR_cov_enh2-Core

Proposal 1	Correct the MAC specification so that only the particular repetition factor as measured by RSRP and compared to the threshold value is considered according to TP.

DISCUSSION:

- LG, Huawei, think that the TP is incorrect as it doesn’t consider the different feature combinations.

=> Noted

## 7.22	Void

## 7.23	Timing Resiliency and URLLC Enh

(NR_TRS_URLLC; leading WG: RAN3; REL-18; WID: RP-230754)

Time budget: 0 TU

Tdoc Limitation: 1 tdoc

### 7.23.1	Organizational

Incoming LSs, Rapporteur input etc.

R2-2404462	Corrections to URLLC and Timing Resiliency 	Ericsson, Nokia	CR	Rel-18	38.331	18.1.0	4667	2	F	TRS_URLLC-NR-Core	R2-2403722

=>	Update cover page and clean changes

=>	the CR is agreed in R2-2405927 with the changes above

R2-2405927	Corrections to URLLC and Timing Resiliency	Ericsson, Nokia	CR	Rel-18	38.331	18.1.0	4667	3	F	TRS_URLLC-NR-Core

=> Agreed

=> Coversheet update by MCC: "Source to TSG" field is empty.

R2-2406124	Corrections to URLLC and Timing Resiliency	Ericsson, Nokia	CR	Rel-18	38.331	18.1.0	4667	4	F	TRS_URLLC-NR-Core

=> Agreed

### 7.23.2	General

Essential corrections only.

## 7.24	TEI18

Specific items may be allocated to a breakout session for treatment.

Time budget: 1 TU

### 7.24.1	TEI proposals by Other Groups

Items initiated by other groups that is/has been communicated by LS, where the other group indicate this is TEI18. (Specific other-group-WIs should use the R18 Other Agenda Item below).

R2-2404147	Reply LS on the user consent for trace reporting (S5-242221; contact: Ericsson)	SA5	LS in	Rel-18	TEI18	To:RAN3	Cc:RAN2, SA2, SA3

SDT – Related to RAN3 LS – to be treated with SDT

R2-2404123	Reply LS on SDT signalling optimization for partial context transfer (R3-242198; contact: ZTE)	RAN3	LS in	Rel-18	TEI18	To:RAN2

R2-2404488	SDT signalling optimization	Nokia	discussion	Rel-18	TEI18

R2-2404489	Draft reply LS on SDT signalling optimization for partial context transfer	Nokia	LS out	Rel-18	TEI18	To:TSG RAN WG3

R2-2404541	SDT signalling optimization for partial context transfer	ZTE Corporation, Sanechips	LS out	To:RAN3

R2-2405649	Discussion on RAN3 LS on SDT signalling optimization for partial context transfer	Huawei, HiSilicon	discussion	Rel-18	TEI18

### 7.24.2	TEI proposals by RAN2

Items initiated in RAN2 for NR and LTE.

No contributions should be submitted under 7.24.2.  They should be submitted under 7.24.x

Tdoc limitation: 1 tdoc, limitation applicable to new proposals.  No new Cat. B proposals expected for this meeting

#### 7.24.2.0	In Principle agreed CRs

To be treated in SL relay and Positioning breakout session

R2-2404246	Remove of AS condition checking of SUI for U2N Relay communication	OPPO, Apple	CR	Rel-18	38.331	18.1.0	4645	1	F	TEI18, NR_SL_relay-Core	R2-2402210

- Agreed

R2-2404509	[B021] Missing posSibType2-17a in list of posSIB types [PosL2RemoteUE]	MediaTek Inc., Lenovo	CR	Rel-18	38.331	18.1.0	4767	1	F	TEI18	R2-2403792

- Agreed

R2-2405253	LPP support for sub 1s location information reporting periodicity [Sub_1s_periodicity]	Ericsson, AT&T, T-Mobile, Vivo, Deutsche Telekom, Huawei, HiSilicon, Vodafone	CR	Rel-18	37.355	18.1.0	0501	3	B	TEI18	R2-2403973

- Agreed

R2-2405254	Corrections for Bluetooth AoA/AoD [BT-AoA-AoD]	Ericsson, Intel Corporation	CR	Rel-18	37.355	18.1.0	0502	1	F	TEI18	R2-2403794

- Agreed

Withdrawn/Not available

R2-2404245	Remove of AS condition checking of SUI for U2N Relay communication	OPPO, Apple	CR	Rel-18	38.351	18.1.0	0035	-	F	TEI18, NR_SL_relay_enh-Core	R2-2402210	Withdrawn

To be treated in MBS breakout session

R2-2405114	Correction on MBS search spaces configuration for (e)Redcap [RedCapMBS_Bcast]	Huawei, CATT, Xiaomi, HiSilicon	CR	Rel-18	38.331	18.1.0	4689	1	F	TEI18, NR_MBS_enh-Core, NR_redcap_enh-Core	R2-2402770

Revised in R2-2405772 to use proper 3GPP styles

R2-2405772	Correction on MBS search spaces configuration for (e)Redcap [RedCapMBS_Bcast]	Huawei, CATT, Xiaomi, HiSilicon	CR	Rel-18	38.331	18.1.0	4689	2	F	TEI18, NR_MBS_enh-Core, NR_redcap_enh-Core

Revised in R2-2405784 to remove NR_MBS_enh-Core, NR_redcap_enh-Core from WI code field on the cover page

R2-2405784 Correction on MBS search spaces configuration for (e)Redcap [RedCapMBS_Bcast]	Huawei, CATT, Xiaomi, HiSilicon	CR	Rel-18	38.331	18.1.0	4689	3	F	TEI18

Agreed (unseen)

#### 7.24.2.1	2Rx XR

Contributions on signaling support for ‘2Rx non-REDCAP XR devices’ as per RP-234015.  Co-source contributions are highly encouraged.

R2-2404454	38331_Correction for the selected band for 2RX XR UE capability checking	Xiaomi Communications	draftCR	Rel-18	38.331	18.1.0	TEI18, NR_XR_enh-Core

=>	Not pursued

R2-2405646	Correction for cell barring for 2Rx XR UE [2Rx_XR_Device]	Huawei, HiSilicon	CR	Rel-18	38.331	18.1.0	4851	-	F	TEI18

-	Xiaomi is concerned as the UE only decide what band to use after this section.

-	Qualcomm is not sure about both CRs as this situation might never happen. Apple thinks we can add in the definition of 2RX XR “in the band”.  Nokia also thinks that nothing may be needed. LG thinks we can consider Apple’s approach, but we can also add a note to further clarify.

-	Vodafone and AT&T doesn’t think it should be captured as a NOTE.

=>	Not pursued

R2-2404520	[B020] Correction of cell barred indication value for 2Rx XR [2Rx_XR_Device]	Lenovo	draftCR	Rel-18	38.331	18.1.0	TEI18

=>	Revise to remove the brackets from B020, make it a real CR 4858, and agreed in R2-2405926

R2-2405926	[B020] Correction of cell barred indication value for 2Rx XR [2Rx_XR_Device]	Lenovo	CR	Rel-18	38.331	18.1.0	4858	TEI18

=> Agreed

R2-2405647	Correction on cell status for 2Rx XR UE [2Rx_XR_Device]	Huawei, HiSilicon	CR	Rel-18	38.304	18.1.0	0408	-	F	TEI18

=>	The CR is agreed

#### 7.24.2.2	Other RAN2 TEI-18

Contributions should focus only critical issues/corrections for already agreed TEI-18 topics.   NCo-sourcing of such proposals is encouraged.   Contributions on items that were explicitly downprioritized from Rel-18 WIs should not be brought as TEI18.  No new Cat. B proposals expected for this meeting

Including outcome of [POST125bis][019][Emergency Calls] Common solution (Lenovo)

Emergency calls

R2-2404257	Summary of [POST125bis][019][Emergency Calls] Common solution (Lenovo)	Lenovo	discussion

Proposal 1: Continue discussion in RAN2#126 on common solution for allowing EM Calls, following two points are to be resolved:

Question A: [Requirement] RAN2 kindly seek operator input to see if there’s a need to differentiate among features with respect to EM Calls support. Example: Allow EM calls to UE-A barred due to a feature-x but not to UE-B barred due to feature-y, if both UE-A and UE-B support EM calls on ‘this’ cell?

Question B: [Feasibility] RAN2 kindly clarify if there are some hardware/ technical limitations (beyond temporary performance degradation) prohibiting EM Calls by a UE on a certain cell, needing separate/ specific EM call exemption.

Proposal 2: Endorsed R17 RedCap CRs can be approved.

R2-2405645	Barring exemption for emergency calls	Huawei, HiSilicon	discussion	Rel-18	TEI18

Proposal 1: A solution enabling barring exemption for emergency calls should not allow the UEs to ignore MIB barring.

Proposal 2: RAN2 to introduce a general rule that barring exemption for emergency calls is allowed for eRedCap and 2Rx XR UEs (no other R18 features) in case there is no barring in MIB and discuss which of the barring exemption solutions to introduce:

•	Option 1:

o	Barring exemption bit is kept for RedCap UEs (as agreed)

o	There is no additional barring exempt bit in SIB1 for eRedCap and 2Rx XR UEs

o	New common CRs for eRedCap and 2Rx XR UEs are prepared while the endorsed CRs for RedCap UEs are kept as they are

•	Option 2:

o	barringExemptRedCap bit is replaced with a common barringExemptEmergencyCall bit applicable to RedCap, eRedCap and 2Rx XR UEs

o	Separate CRs are prepared for TS 38.304 for RedCap UEs (early implementable) and for eRedCap and 2Rx XR UEs.

-	Lenovo thinks option 2, with common bit is preferable. Apple prefers option 1 as that will complicate things, we have R17 CR and then Rel-18 different solution

-	TIM thinks that emergency call allowance should be done on a per use case basis. For example, if it is a robot it doesn’t make sense, but if it is a smartphone we can consider.

-	Nokia prefers option 1.

-	Qualcomm thinks option 2 will allow us to add more features in the future.

-	TMobile is not sure any of this options.

-	ZTE has a slight preference for option 2.

-	Ericsson also wonders if we need to differentiate.

-	Nokia and Oppo think we should have separate control to 2RX XR UEs. ZTE doesn’t understand why we won’t separate control. Nokia explains that redcap UEs can be 1Rx.

-	Vodafone thinks that 2RX XR should be default behaviour. CATT would like to have network control.

Discussion on whether to consider NES (i.e. ignore MIB barring)

-	Vodafone and Fraunhaufer thinks that it would be a fundamental change. Qualcomm thinks that this use case is not strong engouht. Nokia, Ericsson, ZTE, CATT,  thinks we can leave NES out of this discussion.

-	Apple thinks that in the future, the UE may not have a legacy layer.

-	Oppo also thinks that NES should be out of scope as the intention and logic is quite different

-	Lenovo thinks that all UEs should be allowed to make emergency calls.

-	BT is not ready to make a decision.

-	ZTE thinks that if we really want to fix NES is to not use MIB barring.  Since we have MIB barring we don’t want to have exceptions for that purpose.

-	Samsung and BT ask what happens if the NW doesn’t support this.  Apple explains that there are other conditions the UE will have to check before it gets to the EM call barring.   Huawei thinks that we can do this check in 304 CR.

Aftercome back on EM NES

-	BT will organize an offline to discuss MIB barring case for NES.  Vodafone thinks that we won’t be able to do much for MIB barring in Rel-18 and this would result in NBC.     Qualcomm thinks that it can be done in BC way.   Apple indicates yes but with addition of SIB1.   ZTE thinks that we may need to undo MIB barring and this would work even for Rel-15.  Qualcomm wouldn’t like that solution.

-	Tmobile thinks that any solution would be much worst than the actual problem.

-	BT wouldn’t want to change the MIB for all devices, just NES.   Huawei doesn’t thinks this is an issue in the field, if it does happen that there are only NES cells the network can just unbar the MIB and this would be ok for these very rare case.  We can reject them when they try to access. Tmobile agrees with Huawei and ZTE and this can be handled by network implementation.

Agreements

1	NES (i.e. ingoring MIB barring) will not be considered in our common solution discussion.  FFS if anything specific for NES will need to be done.  If anything needs to be done, it would not be part of the common solution.

2	For Rel-18, we introduce 1 bit that enables EM call for RedCap, eRedCap, and 2Rx XR. One RRC Rel-18 with magic sentence that it is early implementable in Rel-17.  A CR for 38.304 doesn’t need to have the magic sentence.

3	This replaces the previous agreement and we will notify RAN3 verbally via delegates

[AT126][002][XR] CR to 331 and 304(Apple)

Intended outcome: Review CRs

Deadline:  05-24-24

R2-2406069	Updates and Introduction of UE capabilities for Rel-18 WIs, including [HARQ-ACK MUX on PUSCH]	Intel Corporation	CR	Rel-18	38.306	18.1.0	1131	-	B	TEI18	NR_MIMO_evo_DL_UL-Core, NR_cov_enh2-Core, NR_MC_enh-Core, NR_Mob_enh2-Core, NR_FR2_multiRX_DL-Core, NR_DSS_enh-Core, NR_NTN_enh-Core, Netw_Energy_NR-Core, NR_FR1_lessthan_5MHz_BW-Core, NR_MG_enh2-Core, NR_SL_enh2-Core, NR_UAV-Core, NR_FR2_multiRX_DL-Core, NR_MBS_enh-Core, NR_demod_enh3-Core, NR_netcon_repeater-Core, NR_NTN_solutions-Core, NR_DualTxRx_MUSIM-Core, NR_pos_enh2-Core, NR_BWP_wor-Core, NR_mobile_IAB-Core, TEI18

=> Agreed

=> Coversheet update by MCC: wrong rev value

R2-2406129	Updates and Introduction of UE capabilities for Rel-18 WIs, including [HARQ-ACK MUX on PUSCH]	Intel Corporation	CR	Rel-18	38.306	18.1.0	1131	1	B	TEI18	NR_MIMO_evo_DL_UL-Core, NR_cov_enh2-Core, NR_MC_enh-Core, NR_Mob_enh2-Core, NR_FR2_multiRX_DL-Core, NR_DSS_enh-Core, NR_NTN_enh-Core, Netw_Energy_NR-Core, NR_FR1_lessthan_5MHz_BW-Core, NR_MG_enh2-Core, NR_SL_enh2-Core, NR_UAV-Core, NR_FR2_multiRX_DL-Core, NR_MBS_enh-Core, NR_demod_enh3-Core, NR_netcon_repeater-Core, NR_NTN_solutions-Core, NR_DualTxRx_MUSIM-Core, NR_pos_enh2-Core, NR_BWP_wor-Core, NR_mobile_IAB-Core, TEI18

=> Agreed

R2-2406070	Updates and Introduction of UE capabilities for Rel-18 WIs	Intel Corporation	CR	Rel-18	38.331	18.1.0	4861	-	B	TEI18	NR_MIMO_evo_DL_UL-Core, NR_cov_enh2-Core, NR_MC_enh-Core, NR_Mob_enh2-Core, NR_FR2_multiRX_DL-Core, Netw_Energy_NR-Core, NR_FR1_lessthan_5MHz_BW-Core, NR_MG_enh2-Core, NR_SL_enh2-Core, NR_ATG-Core, NR_MBS_enh-Core, NR_NTN_enh-Core, NR_pos_enh2-Core, NR_mobile_IAB-Core

=> Agreed

=> Coversheet update by MCC: wrong rev value

R2-2406130	Updates and Introduction of UE capabilities for Rel-18 WIs	Intel Corporation	CR	Rel-18	38.331	18.1.0	4861	1	B	TEI18	NR_MIMO_evo_DL_UL-Core, NR_cov_enh2-Core, NR_MC_enh-Core, NR_Mob_enh2-Core, NR_FR2_multiRX_DL-Core, Netw_Energy_NR-Core, NR_FR1_lessthan_5MHz_BW-Core, NR_MG_enh2-Core, NR_SL_enh2-Core, NR_ATG-Core, NR_MBS_enh-Core, NR_NTN_enh-Core, NR_pos_enh2-Core, NR_mobile_IAB-Core

=> Agreed

R2-2406071	Corrections and Updates to UE capabilities for RAN1 feature group 55-6	Intel Corporation	CR	Rel-18	38.306	18.1.0	1132	-	B	NR_L1enh_URLLC-Core, TEI18

=> Agreed

R2-2406072	Corrections and Updates to UE capabilities for RAN1 feature group 55-6	Intel Corporation	CR	Rel-18	38.331	18.1.0	4862	-	B	NR_L1enh_URLLC-Core, TEI18

=> Agreed

R2-2405940	Summary of Offline #002 ([AT126][002][XR] CR to 331 and 304)	Apple	discussion	Rel-18	TEI18

R2-2404461	Analysis on Common solution for EM calls	Lenovo	discussion	Late

R2-2404540	Enabling emergency calls in a barred cell	ZTE Corporation, Sanechips	discussion

R2-2404643	Introduction of barring exemption for RedCap UEs for emergency calls [RedCap_EM_Call]	Apple, China Telecom, Vodafone, Verizon, TMobile USA, ZTE, Vivo, Ericsson	CR	Rel-18	38.304	18.1.0	0380	3	B	TEI18	R2-2402903

R2-2405937	Introduction of barring exemption for RedCap UEs for emergency calls [RedCap_EM_Call]	Apple, China Telecom, Vodafone, Verizon, TMobile USA, ZTE, Vivo, Ericsson, Nokia	CR	Rel-18	38.304	18.1.0	0380	4	B	TEI18

=> Revised in R2-2405956

R2-2405956	Introduction of barring exemption for RedCap UEs for emergency calls [RedCap_EM_Call]	Apple, China Telecom, Vodafone, Verizon, TMobile USA, ZTE, Vivo, Ericsson, Nokia	CR	Rel-18	38.304	18.1.0	0380	5	B	TEI18

=> Agreed

R2-2404644	Introduction of barring exemption for RedCap UEs for emergency calls [RedCap_EM_Call]	Apple, China Telecom, Vodafone, Verizon, TMobile USA, ZTE, Vivo, Ericsson	CR	Rel-18	38.331	18.1.0	4570	2	B	TEI18	R2-2402902

R2-2405938	Introduction of barring exemption for RedCap UEs for emergency calls [RedCap_EM_Call]	Apple, China Telecom, Vodafone, Verizon, TMobile USA, ZTE, Vivo, Ericsson, Nokia	CR	Rel-18	38.331	18.1.0	4570	3	B	TEI18

=> Revised in R2-2405957

R2-2405957	Introduction of barring exemption for RedCap UEs for emergency calls [RedCap_EM_Call]	Apple, China Telecom, Vodafone, Verizon, TMobile USA, ZTE, Vivo, Ericsson, Nokia	CR	Rel-18	38.331	18.1.0	4570	4	B	TEI18

=> Agreed

=> FFS check in RAN2-127 if further clarifications are required to ensure that once the cell is considered acceptable according to new conditions, the UE doesn’t follow the procedures in 38.304 associated with cell barred case.

R2-2404645	Introduction of barring exemption for eRedCap UEs for emergency calls [eRedCap_EM_Call]	Apple, China Telecom, Vodafone, Verizon, TMobile USA, ZTE, Vivo, Ericsson	CR	Rel-18	38.304	18.1.0	0381	3	B	TEI18	R2-2403472

=> Revised in R2-2405939

R2-2405939	Introduction of barring exemption for (e)RedCap and 2RX XR UEs for emergency calls [EM_Call_Exemption]	Apple, China Telecom, Vodafone, Verizon, TMobile USA, ZTE, Vivo, Ericsson. Nokia	CR	Rel-18	38.304	18.1.0	0381	4	B	TEI18

=> Revised in R2-2405951

R2-2405951	Introduction of barring exemption for (e)RedCap and 2RX XR UEs for emergency calls [EM_Call_Exemption]	Apple, China Telecom, Vodafone, Verizon, TMobile USA, ZTE, Vivo, Ericsson. Nokia	CR	Rel-18	38.304	18.1.0	0381	5	B	TEI18

=> Revised in R2-2405958

R2-2405958	Introduction of barring exemption for (e)RedCap and 2RX XR UEs for emergency calls [EM_Call_Exemption]	Apple, China Telecom, Vodafone, Verizon, TMobile USA, ZTE, Vivo, Ericsson. Nokia	CR	Rel-18	38.304	18.1.0	0381	6	B	TEI18

=> Agreed

R2-2404646	Introduction of barring exemption for eRedCap UEs for emergency calls [eRedCap_EM_Call]	Apple, China Telecom, Vodafone, Verizon, TMobile USA, ZTE, Vivo, Ericsson	CR	Rel-18	38.331	18.1.0	4571	2	B	TEI18	R2-2402904

R2-2404647	Introduction of barring exemption for UEs for emergency calls [EM_Call_Exemption]	Apple, T-Mobile USA	CR	Rel-18	38.304	18.1.0	0402	-	B	TEI18

Redirection

R2-2404430	Correction on redirection to GERAN [Redirect_to_GERAN]	vivo, Qualcomm Incorporated, Samsung, Vodafone	CR	Rel-18	36.331	18.1.0	5000	1	F	TEI18	R2-2402330

=>	The CR is agreed

CIO_in_reportconfig

R2-2405596	Potential correction for cell individual offset in ReportConfig [CIO_in_ReportConfig]	NTT DOCOMO, INC., Qualcomm Incorporated, Ericsson	discussion	Rel-18

Proposal 1.	Discuss whether RAN2 should avoid limitation of CIO configuration in report configiguration and introduce indication of frequency in reportConfig.

Proposal 2.	If it is concluded that an indication of frequency in reportConfig is introduced, apply the changes in Annex A.

-	Huawei and LG thinks that this can be handled by NW implementation.  Samsung agrees but this would reduce signaling overhead, so it would be good to have this type of enhancement. Ericsson agrees with Samsung.

-	ZTE supports the proposal.

-	Qualcomm asks what happens if this is not provided.  Samsung explains that we can’t make it mandatory and if it is not signalled all MO apply the CIO.  It is to save signaling overhead.

=>	An indication of frequency in reportConfig is introduced, apply the changes in Annex A in the final CR

=>	Noted

R2-2404976	[E230] Clarification on CIO configured within ReportConfig [CIO_in_ReportConfig]	Ericsson, NTT DOCOMO, INC., Qualcomm Incorporated	CR	Rel-18	38.331	18.1.0	4707	1	F	TEI18	R2-2403184

=>	Update the CR with the agreement from R2-2405596

=>	Revised in R2-2405930 and reviewed over email

R2-2405930	Clarification on CIO configured within ReportConfig [CIO_in_ReportConfig]	Ericsson, NTT DOCOMO, INC., Qualcomm Incorporated	CR	Rel-18	38.331	18.1.0	4707	2	F	TEI18

=>	The CR is agreed

[AT126][008][CIO] CR to 38.331 (Ericsson)

Intended outcome: agree to CR by email

Deadline:  05-24-24

meas_report_enh

R2-2404973	Enhancements to measurement report [meas_report_enh]	Ericsson, T-Mobile USA, Turkcell, Rakuten Mobile, BT Plc., NTT Docomo, Deutsche Telekom, MediaTek Inc., Verizon, AT&T, Vodafone, Continental Automotive, KDDI, Charter, NEC, Telecom Italia, CATT, Reliance Jio, Qualcomm Incorporated	CR	Rel-18	38.331	18.1.0	4803	-	B	TEI18

=> Revised in R2-2405993

R2-2405993	Enhancements to measurement report [meas_report_enh]	Ericsson, T-Mobile USA, Turkcell, Rakuten Mobile, BT Plc., NTT Docomo, Deutsche Telekom, MediaTek Inc., Verizon, AT&T, Vodafone, Continental Automotive, KDDI, Charter, NEC, Telecom Italia, CATT, Reliance Jio, Qualcomm Incorporated	CR	Rel-18	38.331	18.1.0	4803	1	B	TEI18

=> Revised in R2-2406021

R2-2406021	Enhancements to measurement report [meas_report_enh]	Ericsson, T-Mobile USA, Turkcell, Rakuten Mobile, BT Plc., NTT Docomo, Deutsche Telekom, MediaTek Inc., Verizon, AT&T, Vodafone, Continental Automotive, KDDI, Charter, NEC, Telecom Italia, CATT, Reliance Jio, Qualcomm Incorporated	CR	Rel-18	38.331	18.1.0	4803	2	B	TEI18

=> Agreed

R2-2404974	Enhancements to measurement report [meas_report_enh]	Ericsson, T-Mobile USA, Turkcell, Rakuten Mobile, BT Plc., NTT Docomo, Deutsche Telekom, MediaTek Inc., Verizon, AT&T, Vodafone, Continental Automotive, KDDI, Charter, NEC, Telecom Italia, CATT, Reliance Jio, Qualcomm Incorporated	CR	Rel-18	38.306	18.1.0	1110	-	B	TEI18

=> Revised in R2-2405994

R2-2405994	Enhancements to measurement report [meas_report_enh]	Ericsson, T-Mobile USA, Turkcell, Rakuten Mobile, BT Plc., NTT Docomo, Deutsche Telekom, MediaTek Inc., Verizon, AT&T, Vodafone, Continental Automotive, KDDI, Charter, NEC, Telecom Italia, CATT, Reliance Jio, Qualcomm Incorporated	CR	Rel-18	38.306	18.1.0	1110	1	B	TEI18

=> Agreed

R2-2404975	Discussion on enhancements to measurement report [meas_report_enh]	Ericsson, T-Mobile USA, Turkcell, Rakuten Mobile, BT Plc., NTT Docomo, Deutsche Telekom, MediaTek Inc., Verizon, AT&T, Vodafone, Continental Automotive, KDDI, Charter, NEC, Telecom Italia, CATT, Reliance Jio, Qualcomm Incorporated	discussion	Rel-18	TEI18

Proposal 1	After performing measurements according to the existing RAN4 requirements, if a measurement report is triggered the UE reports a new indication within MeasResults IE to indicate if an event entering condition for the cell is satisfied for the first time.

Proposal 2	After performing measurements according to the existing RAN4 requirements, if a measurement report is triggered the UE reports a new indication within MeasResults IE to indicate which cell(s) fulfilled the event leaving condition.

Proposal 3	After performing measurements according to the existing RAN4 requirements, UE triggers a measurement report when the best cell among the ones previously reported changes.

Proposal 4	RAN2 to introduce new UE capabilities in case these new informations to be reported are agreed.

R2-2405141	On enhancing the measurement reporting (report when the order in maxReportCells is changed)	Nokia	discussion	Rel-18	TEI18

Proposal 1: To solve the problem of preparation of the best “x” cells and execution to the best cell RAN2 agrees to use either:

-	the solution for re-ordering mechanism triggering a new event or

-	use of multiple thresholds for the same event linked to same Measurement Object.

R2-2405490	Discussion on measurement reporting enhancements	LG Electronics Inc.	discussion

R2-2405635	Alternative solution for reporting best cell change	ZTE Corporation	discussion	Rel-18	TEI18

Proposal 1: For reporting best cell change, to introduce an indication (e.g. reportOnlyIfBestCellChange) in ReportConfigNR, used to indicate the UE to only send periodical event MR if the best neighbour cell has changed. The field is optionally configured when reportAmount >1.

-	Huawei thinks this is combining event based and periodical. Nokia explains that N would be configurable so it is up to the network.

Discussion

Proposal 3	After performing measurements according to the existing RAN4 requirements, UE triggers a measurement report when the best cell among the ones previously reported changes.

For reporting best cell change, to introduce an indication (e.g. reportOnlyIfBestCellChange) in ReportConfigNR, used to indicate the UE to only send periodical event MR if the best neighbour cell has changed. The field is optionally configured when reportAmount >1.

Consider up to N Best cell change

-	Huawei thinks it is more reasonable to skip and increment UE variable by 1.  ZTE thinks that both solutions work but chose this one as it is simpler.

-	LG asks if we consider CIO.  ZTE hasn’t considered it.

Agreements:

1	After performing measurements according to the existing RAN4 requirements, if a measurement report is triggered the UE reports a new indication within MeasResults IE to indicate if an event entering condition for the cell is satisfied for the first time.

2	After performing measurements according to the existing RAN4 requirements, if a measurement report is triggered the UE reports a new indication within MeasResults IE to indicate which cell(s) fulfilled the event leaving condition.

3	For reporting best cell change, to introduce an indication (e.g. reportOnlyIfBestCellChange) in ReportConfigNR, used to indicate the UE to only send periodical event MR if the best neighbour cell has changed. The field is optionally configured when reportAmount >1.

4	as a compromise we will limit N=2 and new capability. Send a report if either first best cell has changed or the second best cell has change

5	RAN2 to introduce new UE capabilities in case these new informations to be reported are agreed.

After CB one compromise is to make N to 2

-	Nokia can accept the compromise.   Huawei thinks that N 1 already solves the problem we were trying to address in the same place, but can compromise if all companies are ok.

-	Qualcomm is concerned that if N2 we have to do testing for both N=1 and 2 case.  So we would need to add another capability.  Apple and Oppo also thinks that we would need a new capability.  LG thinks that if we add another capability it can be greater than 2.

-	CATT and Oppo think that 1 is sufficient.

=>	as a compromise we will limit N=2 and new capability. Send a report if either first best cell has changed or the second best cell has change.

[AT126][007][Meas Enh] Review TP and CR (Ericsson)

Intended outcome: Review TP and agree to CR after meeting

Deadline:  Thursday

SDT – to be handled with SDT section

R2-2405648	Correction for resumeIndication [SDT_ReleaseEnh]	Huawei, HiSilicon, Nokia, ZTE Corporation, Sanechips, Intel Corporation	CR	Rel-18	38.300	18.1.0	0848	1	F	TEI18	R2-2403108

R2-2404490	SDT BFR timer being not configured [RA-SDT_BeamFailure]	Nokia, Sony, Nokia Shanghai Bell	CR	Rel-18	38.321	18.1.0	1814	1	F	TEI18	R2-2403087

R2-2404546	Paging monitoring for extended CG period [CG-SDT-Enh]	ZTE Corporation, Sanechips, Mediatek, Qualcomm	CR	Rel-18	38.331	18.1.0	4788	-	F	TEI18

To be handled in positioning breakout

R2-2405575	Corrections related to LPP RILs E001-E003 and Q033 due to agreed CT4 corrections [LocalCoords]	Ericsson	CR	Rel-18	37.355	18.1.0	0510	-	F	TEI18

- Revised in R2-2405858

R2-2405858	Corrections related to LPP RILs E001-E003 and Q033 due to agreed CT4 corrections [LocalCoords]	Ericsson	CR	Rel-18	37.355	18.1.0	0510	1	F	TEI18

- Revised in R2-2405861

R2-2405861	Corrections related to LPP RILs E001-E003 and Q033 due to agreed CT4 corrections [PosLocalCoords]	Ericsson, Qualcomm Incorporated	CR	Rel-18	37.355	18.1.0	0510	2	F	TEI18

- Agreed

Discussion:

Ericsson clarify this is for CT4 alignment.

Nokia wonder if this is a correction or a new function.

Huawei wonder if a RAN3 CR is also needed.  Qualcomm think there is probably some impact there, but it would be from Rel-16.

R2-2404758	Correction to on-demand SIB request [OdSIBReq]	Huawei, HiSilicon, Ericsson, Qualcomm	CR	Rel-18	38.331	18.1.0	4796	-	B	TEI18

- Postponed

Discussion:

Lenovo have concerns on the magic sentence; they think it will not work when facing a legacy network.  They also wonder how the UE knows if the network supports the periodic feature.

MBS related – to be treated in breakout session

R2-2404993	Clarifications for MBS RedCap CFR	Ericsson, CATT	CR	Rel-18	38.300	18.1.0	0864	-	F	TEI18, NR_MBS-Core, NR_redcap-Core, NR_redcap_enh-Core

Remove impact analysis

Remove the following part “The NG-RAN node also ensures that in case of a single DCI transmission that only the UE for which the transmission is intended can receive it.” and update the cover page accordingly.

Revised in R2-2405773

- ZTE asks why companies do not want the last sentence.

- Huawei thinks the last sentence is not aligned with the agreement and the previous sentence captures it already.

- Ericsson wonders whether another clarification should be added.

R2-2404994	Clarification for RedCap UE supporting MBS broadcast	Ericsson	CR	Rel-18	38.306	18.1.0	1111	-	F	TEI18, NR_MBS-Core, NR_redcap-Core, NR_redcap_enh-Core

Remove impact analysis

Should be submitted as a draft CR in R2-2405774

Can discuss the exact wording of the change during offline

The intention is agreed

- Huawei thinks existing text already is clear so we do not need this CR.

- QCM supports the intention of clarifying in 38.306, but prefers not to refer to RRC parameters, but talk about features.

- Xiaomi also supports the intention of the CR.

R2-2405558	MBS operation with eDRX MICO [TEI18 NR_MBS_enh]	Nokia, Ericsson	CR	Rel-18	38.304	18.1.0	0399	1	F	TEI18	R2-2403598

Add clauses affected

Avoid having hanging paragraph

Change the TEI18 identifier

Keep one Tdoc number

Revised in R2-2405775

- Ericsson clarified why they deviated from IPA CR.

- ZTE agrees with the current CR.

[AT126][605][TEI18] MBS TEI18 CRs (Ericsson)

Scope: Revise R2-2404993, R2-2404994 and R2-2405558 according to the agreements

Intended outcome: Agreeable CR in R2-2405773 (38.300), R2-2405774 (38.306 draft CR) and in R2-2405775 (38.304 CR)

Deadline: CRs available for offline approval: Friday 2024-05-24 0900

R2-2405773	Clarifications for MBS RedCap CFR	Ericsson, CATT, ZTE	CR	Rel-18	38.300	18.1.0	0864	1	F	TEI18, NR_MBS-Core, NR_redcap-Core, NR_redcap_enh-Core

=> Revised in R2-2406027

R2-2406027	Clarifications for MBS RedCap CFR [RedCapMBS_Bcast]	Ericsson, CATT, ZTE	CR	Rel-18	38.300	18.1.0	0864	1	F	TEI18

=> Agreed

R2-2405774	Clarification for RedCap UE supporting MBS broadcast	Ericsson	draftCR	Rel-18	38.306	18.1.0	F	TEI18, NR_MBS-Core, NR_redcap-Core, NR_redcap_enh-Core

=> Revised in R2-2406028

R2-2406028	Clarification for RedCap UE supporting MBS broadcast [RedCapMBS_Bcast]	Ericsson	draftCR	Rel-18	38.306	18.1.0	F	TEI18

=> Revised in R2-2406047

R2-2406047	Clarification for RedCap UE supporting MBS broadcast [RedCapMBS_Bcast]	Ericsson	CR	Rel-18	38.306	18.1.0	1111	1	F	TEI18

=> Agreed

R2-2405775	MBS operation with eDRX MICO [eMBS_eDRX_MICO]	Nokia, Ericsson	CR	Rel-18	38.304	18.1.0	0399	2	F	TEI18

=> Revised in R2-2406034

R2-2406034	MBS operation with eDRX MICO	Nokia, Ericsson	CR	Rel-18	38.304	18.1.0	0399	3	F	5MBS_Ph2

=> Agreed

R2-2404995	Scheduling restrictions with RedCap CFR and eRedCap Ues	Ericsson	discussion	Rel-18	TEI18, NR_MBS-Core, NR_redcap-Core, NR_redcap_enh-Core

R2-2405130	Correction on the configuration of Redcap CFR [RedCapMBS_Bcast]	Huawei, HiSilicon	CR	Rel-18	38.331	18.1.0	4816	-	F	TEI18, NR_MBS_enh-Core, NR_redcap_enh-Core

- QCM thinks that this CR will prevent some deployment cases.

- Huawei thinks having only MCCH and not having CFR should not happen.

- Nokia indicates the change would affect NEED code.

Offline Huawei

[AT126][606][TEI18] Correction on the configuration of Redcap CFR (Huawei)

Scope: Discuss whether to apply change as per R2-2405130

Intended outcome: Report with a proposal in R2-2405776

Deadline: Report available: Friday 2024-05-24 0900

R2-2405776	Correction on the configuration of Redcap CFR [RedCapMBS_Bcast]	Huawei, HiSilicon	CR	Rel-18	38.331	18.1.0	4816	1	F	TEI18, NR_MBS_enh-Core, NR_redcap_enh-Core

Remove NR_MBS-Core and NR_redcap-Core from WI code on the cover page

Revised in R2-2405783

R2-2405783 Correction on the configuration of Redcap CFR [RedCapMBS_Bcast]	Huawei, HiSilicon	CR	Rel-18	38.331	18.1.0	4816	2	F	TEI18

Agreed (unseen)

## 7.25	R18 Other

Specific items may be allocated to a breakout session for treatment.

Impacts from Other RAN WGs and TSGs that has no separate TU budget in RAN2. LS ins for Rel-18 specific WIs/SIs that has no RAN WI.

Clarification CRs should be discussed with spec rapporteurs of the topic prior to submission.

Time budget: 2 TU

Tdoc Limitation: -

### 7.25.1	RAN4 led items

#### 7.25.1.1	Lower MSD capability

#### 7.25.1.2	Intra-band non-collocated NR-CA. EN-DC

#### 7.25.1.3	TCI State Switch indication for HST

#### 7.25.1.4	FR2 Multi Rx operation

#### 7.25.1.5	FR2 SCell Enhancements

#### 7.25.1.6	ATG

R2-2405529	Clarification on ATG Timing advance Reporting procedures	Samsung	discussion	Rel-18	NR_ATG-Core

Proposal 1: RAN2 to discuss how to handle switching of SCSes via BWP-switching for timing advance reporting:

•	offsetThresholdTA-r18

•	Timing Advance field in TAR MAC CE

Proposal 2: RAN2 to choose between

1) Clarifying that SCS is based on SCS of active BWP

2) Clarifying that SCS is based on SCS of initial BWP

3) Any other solution

=>	we will address the issue next meeting (if needed)

-	Samsung would prefer option 2.  ZTE would prefer a simple solution. Huawei thinks it should be the active BWP.  CATT and Qualcomm think that the current spec already specifies option 1, so there is no issue in current spec.

#### 7.25.1.7	Other

Including Less than 5MHz, BWP operation without restrictions, measurement gaps, etc

R2-2404119	Reply LS on RRC network assistant signalling for advanced receiver on MU-MIMO scenario (R1-2403750; contact: China Telecom)	RAN1	LS in	Rel-18	NR_demod_enh3-Core	To:RAN2, RAN4

=>	Noted

R2-2405585	Correction on RRC signalling for advanced receiver	China Telecom, CATT	CR	Rel-18	38.331	18.1.0	4673	1	F	NR_demod_enh3-Core	R2-2402536

=>	The CR is agreed

R2-2405699	RIL list for Advanced receivers	CATT, China Telecom	discussion	Rel-18	NR_demod_enh3-Core

R2-2405213	E173, E174 Remaining issues on Advanced receivers	Ericsson	discussion	Rel-18	NR_demod_enh3-Core

E173

Proposal 1	RAN2 to agree to to delete the optionality of the BOOLEAN value for the parameters in IE AdvancedReceiver-MU-MIMO .

-	Qualcomm indicates that we discussed in last meeting and we agreed they are not needed. China telecom indicates that the current signaling has been confirmed by RAN4.

E174

Proposal 3	RAN2 to remove the parameters advReceiver-MU-MIMO-DCI-1-1 within the IE AdvancedReceiver-MU-MIMO and add in the corresponding field description in IE PDSCH-Config: “Configure the presence of the co-scheduled UE information filed in DCI format 1_1 (see TS 38.212 [17], clause 7.3.1.2.2).”.

-	Nokia has sympathy for these proposals.  QUlacomm thinks that this parameter is a requirement from RAN4 and they confirmed.

-	Nokia thinks we didn’t discussed this in a suitable way, we asked RAN4 but they are not signaling experts.

=>	E173 and 174 are rejected

R2-2405664	Discussion on remaining issues for Advanced receivers [E174]	Huawei, HiSilicon	discussion	Rel-18	NR_demod_enh3-Core

R2-2405491	[N143] On DM-RS power boosting assumption for advanced receivers	Nokia	discussion	NR_demod_enh3-Core

=>	N143 is agreed

R2-2404743	Discussion on the power boosting parameter for advance receivers	MediaTek Inc.	discussion	NR_demod_enh3-Core

=>	Noted

R2-2405584	Discussion on RRC signalling for advanced receiver on MU-MIMO scenario	China Telecom, CATT	discussion	Rel-18	NR_demod_enh3-Core

=>	Noted

R2-2405991	Reply LS to RAN2 on RRM enhancements for NR FR2 HST (R4-2410285; contact: Samsung)	RAN4	LS in	Rel-18	NR_HST_FR2_Enh	To:RAN2

NR_BWP_wor-Core

R2-2404128	Reply LS on BWP operation without bandwidth restriction (R4-2406500; contact: vivo, Vodafone)	RAN4	LS in	Rel-18		To:RAN2, RAN1

=>	Noted

R2-2404557	Discussion on reply LS from RAN4 on NCD-SSB for PSCell	vivo, Vodafone	discussion	Rel-18	NR_BWP_wor-Core

Proposal 1: RAN2 to discussion how to capture the restriction on NCD-SSB measurement applicable for PCell and PSCell:

-	Option 1: Keep RAN2 specification as it is, i.e. no change in RAN2.

-	Option 2: Add a restriction in stage-2 specification. Detailed TP is provided in Annex.

R2-2405515	Discussion on BWP operation without bandwidth restriction for PSCell	Huawei, HiSilicon	discussion	Rel-18	NR_BWP_wor

Proposal 2: Introduce a new UE capability for the NCD-SSB based L1/L3 measurement function for PSCell.

-	ZTE doesn’t support this capability as this is not the intention from RAN4 LS.

-	Ericsson thinks that if we have to test for both Pcell and PScell.

=>	No separate capability will be added

=>	Update ncd-SSB-BWP-Wor-r18 description to add PScell “NOTE: this feature applies only to PCell and PSCell”.

=>	UE capability rapporteur will capture change

=>	Noted

Discussion

-	Vodafone and Oppo would have a preference for option 2. Huawei and Ericsson thinks we don’t need to capture anything.

R2-2404445	Miscellaneous corrections on TS 38.300 for BWP operation without restriction	vivo	CR	Rel-18	38.300	18.1.0	0837	1	D	NR_BWP_wor-Core	R2-2402621

=>	The CR is agreed

R2-2404446	[V994][V995] Miscellaneous corrections on TS 38.331 for BWP operation without restriction	vivo	CR	Rel-18	38.331	18.1.0	4679	1	F	NR_BWP_wor-Core	R2-2402623

=>	The CR is agreed

R2-2404877	RIL list for BWP_Wor	vivo	discussion	Rel-18	NR_BWP_wor-Core

=>	Noted

4Rx_low_NR_band_handheld_3Tx_NR_CA_ENDC-Core

R2-2404129	LS on 3Tx SAR solution for inter-band CA with PC1.5 (R4-2406579; contact: Huawei)	RAN4	LS in	Rel-18		To:RAN2

=>	Noted

R2-2405511	Discussion on 3Tx SAR for inter-band CA with PC1.5	Huawei, HiSilicon	discussion	Rel-18	4Rx_low_NR_band_handheld_3Tx_NR_CA_ENDC-Core

=>	Noted

R2-2405512	Correction on 3Tx SAR for inter-band CA with PC1.5	Huawei, HiSilicon	CR	Rel-17	38.306	17.8.0	1122	-	F	4Rx_low_NR_band_handheld_3Tx_NR_CA_ENDC-Core

-	Huawei explains that there is no legacy network support PC1.5

=>	work on some rewording offline

[AT126][025][3Tx SAR] 38.306 CR (HUawei)

Intended outcome: agree to CR and shadow CR by email

Deadline:  05-24-24

R2-2405946	Correction on 3Tx SAR for inter-band CA with PC1.5	Huawei, HiSilicon	CR	Rel-17	38.306	17.8.0	1122	1	F	4Rx_low_NR_band_handheld_3Tx_NR_CA_ENDC-Core

=>	The CR is agreed

R2-2405513	Correction on 3Tx SAR for inter-band CA with PC1.5	Huawei, HiSilicon	CR	Rel-18	38.306	18.1.0	1123	-	A	4Rx_low_NR_band_handheld_3Tx_NR_CA_ENDC-Core

=>	Revised in R2405947

R2-2405947	Correction on 3Tx SAR for inter-band CA with PC1.5	Huawei, HiSilicon	CR	Rel-18	38.306	18.1.0	1123	1	A	4Rx_low_NR_band_handheld_3Tx_NR_CA_ENDC-Core

=>	The CR is agreed

R2-2405514	Draft LS reply on 3Tx SAR solution for inter-band CA with PC1.5	Huawei, HiSilicon	LS out	Rel-18	4Rx_low_NR_band_handheld_3Tx_NR_CA_ENDC-Core	To:RAN4

=>	The LS is approved in R2-2405943 with the final CR attached

R2-2405943	LS reply on 3Tx SAR solution for inter-band CA with PC1.5	RAN2	LS out	Rel-18	4Rx_low_NR_band_handheld_3Tx_NR_CA_ENDC-Core	To:RAN4

=> Approved

=> Coversheet revised by MCC: removing "Draft" from the LS title

=> Revised in R2-2406022

R2-2406022	LS reply on 3Tx SAR solution for inter-band CA with PC1.5	RAN2	LS out	Rel-18	4Rx_low_NR_band_handheld_3Tx_NR_CA_ENDC-Core	To:RAN4

=> Approved

NR_FR1_lessthan_5MHz_BW

R2-2404131	LS on UE Capability for Asymmetric BW for less than 5 MHz (R4-2406717; contact: Rakuten)	RAN4	LS in	Rel-18	NR_FR1_lessthan_5MHz_BW	To:RAN2, RAN1

=>	Noted

R2-2404278	Introduction of NR support for dedicated spectrum less than 5MHz for FR1	Qualcomm Incorporated	CR	Rel-18	38.331	18.1.0	4525	3	B	NR_FR1_lessthan_5MHz_BW-Core	R2-2403961

[AT126][028][less5MHz] RRC CR (Qualcomm)

Intended outcome: Agree to 38.331 CR by email

Deadline:  05-24-24

R2-2406010	Introduction of NR support for dedicated spectrum less than 5MHz for FR1	Qualcomm Incorporated	CR	Rel-18	38.331	18.1.0	4525	4	B	NR_FR1_lessthan_5MHz_BW-Core

=>	The CR is agreed

R2-2404279	Introduction of NR support for dedicated spectrum less than 5MHz for FR1	Qualcomm Incorporated	CR	Rel-18	36.331	18.1.0	4983	3	B	NR_FR1_lessthan_5MHz_BW-Core	R2-2403962

=> Revised in R2-2406011

R2-2406011	Introduction of NR support for dedicated spectrum less than 5MHz for FR1	Qualcomm Incorporated	CR	Rel-18	36.331	18.1.0	4983	4	B	NR_FR1_lessthan_5MHz_BW-Core

=>	The CR is agreed

R2-2405565	Less than 5Mhz operation	Nokia	discussion	Rel-18	NR_FR1_lessthan_5MHz_BW-Core

Proposal 1: Remove conditional presence of “LessThan5Mhz” from fields to Need R i.e. for fields dl-CarrierFreq-r18, frequencyBandList-r18

Proposal 2: Rename measidleCarrierListNR-LessThan5Mhz and measReselectionCarrierListNR-LessThan5Mhz to generic ones and not require using these field always when configuring cells supporting 12,15 or 20 PRBs.

=>	Address the issue by specifying: If neighbour cell list is being broadcasted in a cell with other than 3MHz then it is mandatory otherwise it is not mandatory.

Not treated – waiting for RAN1 discussion/input

R2-2404746	Consideration on the AsymmetricBandwidthCombinationSet for Less than 5M	ZTE Corporation, Sanechips	discussion	Rel-18	NR_FR1_lessthan_5MHz_BW-Core

=>	Some of these aspects should be brought up during mega CR review after RAN1 also has made some agreements.

=>	Noted

R2-2404831	Support of asymmetric bandwidth for less than 5 MHz	Huawei, HiSilicon	discussion	Rel-18	NR_FR1_lessthan_5MHz_BW

R2-2404832	Correction of 38.331 to support asymmetric bandwidth for less than 5 MHz (Opt-1)	Huawei, HiSilicon	CR	Rel-18	38.331	18.1.0	4800	-	F	NR_FR1_lessthan_5MHz_BW

R2-2404833	Correction of 38.306 to support asymmetric bandwidth for less than 5 MHz (Opt-1)	Huawei, HiSilicon	CR	Rel-18	38.306	18.1.0	1108	-	F	NR_FR1_lessthan_5MHz_BW

R2-2404834	Correction of 38.306 to support asymmetric bandwidth for less than 5 MHz (Opt-2)	Huawei, HiSilicon	CR	Rel-18	38.306	18.1.0	1109	-	F	NR_FR1_lessthan_5MHz_BW

R2-2405516	Introduction of UE capability for BWP operation without bandwidth restriction for PSCell	Huawei, HiSilicon	CR	Rel-18	38.306	18.1.0	1124	-	B	NR_BWP_wor

R2-2405517	Introduction of UE capability for BWP operation without bandwidth restriction for PSCell	Huawei, HiSilicon	CR	Rel-18	38.331	18.1.0	4843	-	B	NR_BWP_wor

R2-2405673	Correction to the AsymmetricBandwidthCombinationSet for Less than 5M Case	ZTE Corporation, Sanechips	CR	Rel-18	38.306	18.1.0	1125	-	F	NR_FR1_lessthan_5MHz_BW-Core

=> Revised in R2-2405712

R2-2405712	Correction to the AsymmetricBandwidthCombinationSet for Less than 5M Case	ZTE Corporation, Sanechips	CR	Rel-18	38.306	18.1.0	1125	1	F	NR_FR1_lessthan_5MHz_BW-Core

R2-2405692	Discussion on LS from RAN4 on 3Mhz Uplink Asymmetric BW Support	Rakuten Mobile, Inc	discussion	Rel-18

### 7.25.2	RAN1 led items

E.g. UL Tx Switching, MC enhancements, DSS

R2-2405992	LS on UE capability for multi-carrier enhancement (R4-2410299; contact: vivo)	RAN4	LS in	Rel-18	NR_MC_enh-Core	To:RAN2	Cc:RAN1

-	Vivo explains that companies opinions are split.

=>	implement option 2 in our CR.  Docomo will provide TP to Capability rapporteur and this will be implemented directly in mega CR

=>	Noted

R2-2405371	Rapp RRC CR for MC enhancement	Huawei, HiSilicon, NTT DOCOMO INC, Samsung, Ericsson, OPPO	CR	Rel-18	38.331	18.1.0	4832	-	F	NR_MC_enh-Core

[AT126][026][MC enh] CR to 331 (Huawei )

Intended outcome: Agree to CR

Deadline:  05-24-24

R2-2406008	Rapp RRC CR for MC enhancement	Huawei, HiSilicon, NTT DOCOMO INC, Samsung, Ericsson, OPPO	CR	Rel-18	38.331	18.1.0	4832	1	F	NR_MC_enh-Core

=>	The CR is agreed

R2-2405372	RIL list for MC enhancement	Huawei, HiSilicon	report	Rel-18	NR_MC_enh-Core

=>	Noted

R2-2405370	Clarification to the band pair reporting	Huawei, HiSilicon, Qualcomm Incorporated	CR	Rel-18	38.306	18.1.0	1115	-	F	NR_MC_enh-Core

=>	The CR is agreed

R2-2405597	Discussion on RAN4 agreement for Multi-carrier enhancements	NTT DOCOMO, INC.	discussion	Rel-18

=>	Confirm that RAN2 do not introduce UE capability to report preferred switching pattern.

=>	Noted

R2-2405394	Clarification on deactivated or dormant cells in multi-cell scheduling	Samsung	CR	Rel-18	38.321	18.1.0	1859	-	F	NR_MC_enh-Core

-	ZTE thinks that the CR is not needed.  In this section we don’t specify what the UE shall do, we specify what the UE should not do.  It is very clear what the UE should do based on RAN1 spec.

=>	Remove impact analysis from cover page

[AT126][027][MC enh] 38.321 CR (Samsung)

Intended outcome: Agree to CR (if no concerns after checking with RAN1 are identified)

Deadline:  05-24-24

R2-2405955	Clarification on deactivated or dormant cells in multi-cell scheduling	Samsung	CR	Rel-18	38.321	18.1.0	1859	1	F	NR_MC_enh-Core

=>	The CR is postponed

### 7.25.3	Other

RAN3, SA2, SA3, CT1 led items and others, e.g. eNPN, Slicing, NTN self evaluation issues, etc.

R2-2405682	Supporting 3MHz Uplink Bandwidth with 5MHz or wider Channel Bandwidth	Rakuten Mobile, Inc	draftCR	Rel-18	38.306	18.1.0	B	NR_FR1_lessthan_5MHz_BW_Ph2

# 8	Rel-19

## 8.0	General

This AI is reserved for Rel-19 LSs from other WGs.  No contributions are expected on these LSs for this meeting

LS of 5G Femto

R2-2404141	LS on Support of UE move between CAG cell of 5G Femto and CSG cell (S2-2405813; contact: NTT DOCOMO)	SA2	LS in	Rel-19	FS_5G_Femto, FS_NR_WAB_5GFemto	To:RAN2, RAN3

In TR 23.700-45, the WT#2 (mapped to KI#1 in the TR) investigates the support of the UE moving between CAG cell of 5G Femto and CSG cell, with no impact on the RAN. Therefore, some companies proposed below solutions for this KI:

1	The UE partitions CSG-CAG ID and constructs mapped CSG/CAG ID, and reports to the NG-RAN or E-UTRAN (depending on the considered mobility direction) as described in pCR (S2-2405814).

2	RAN recognizes the target CSG cell (or the target CAG cell) as an open cell during the handover (e.g., via local configuration) and the core network performs access control as described in pCR (S2-2405789).

-	Lenovo thinks that we don’t need to respond and SA2 didn’t expect changes for RAN2.

-	ZTE explains that they discussed whether there is UE impact or not and they couldn’t decide so they asked us.

-	Ericsson thinks solution 1 has UE impact but not solution 2. Samsung has some concerns on solution 2 as it can cause handover failure and we would have RAN impact as well.  Nokia agrees with the handover concerns.

-	ZTE, Nokia and Huawei think that current solution 2 has no UE impact.

-	Samsung indicates that if we want to address the handover failure case there will be UE impact

=>	Confirm there is UE impact for Solution 1

=> 	Majority of companies assume that solution2 is NW-based solution with no UE impact, from RAN2 perspective.  However, some concerns with handover failure have been raised, that may result in UE or RAN3 impact, but RAN2 hasn’t had time to have a detailed technical discussion.

=>	Noted

Reply LS draft

R2-2405936	DRAFT Reply LS on Support of UE move between CAG cell of 5G Femto and CSG cell	LG Electronics	LS out	Rel-19	FS_5G_Femto, FS_WAB_5GFemto_NR	To:SA2

=>	Add that RAN2 didn’t have time to discuss the technical aspects.

=>	Delete last part

=>	Update: Majority of companies assume that solution2 is NW-based solution with no UE impact, from RAN2 perspective.  However, some concerns with handover failure have been raised, that may result in UE or RAN3 impact, but RAN2 hasn’t had time to have a detailed technical discussion.

=>	The LS will be revised in  R2-2405942

[AT126][024][Femto] LS reply  (LG)

Intended outcome: approve LS by email

Deadline:  05-24-24

R2-2405942	Reply LS on Support of UE move between CAG cell of 5G Femto and CSG cell	LG Electronics Inc.	LS out	Rel-19	FS_5G_Femto, FS_WAB_5GFemto_NR	To:SA2	Cc:RAN3

=> Approved

Other LS

R2-2404142	LS on FS_VMR_Ph2 solution impacts to RAN (S2-2405822; contact: Qualcomm)	SA2	LS in	Rel-19	FS_VMR_Ph2	To:RAN3	Cc:RAN2

=>	Noted

R2-2404114	LS to RAN2 on CBSR for Rel-19 MIMO (R1-2403650; contact: Samsung)	RAN1	LS in	Rel-19	NR_MIMO_Ph5	To:RAN2

=>	Noted

R2-2404132	LS on 3T6R and 4T6R antenna switching SRS (R4-2406718; contact: Huawei)	RAN4	LS in	Rel-19	NR_ENDC_RF_Ph4-Core	To:RAN1	Cc:RAN2

=>	Noted

Not treated (No input was expected for this meeting)

R2-2405463	Discussion on UE handover between CAG cell of 5G Femto and CSG Cell	Samsung	discussion	Rel-19	FS_5G_Femto, FS_NR_WAB_5GFemto

R2-2405464	(Draft) reply LS on support of UE move between CAG cell of 5G Femto and CSG Cell	Samsung	LS out	Rel-19	FS_5G_Femto, FS_NR_WAB_5GFemto	To:SA2	Cc:RAN3

R2-2405502	RAN2 impact of SA2 solutions for handover between CSG and CAG cells	LG Electronics Inc.	discussion	FS_5G_Femto

R2-2405523	DRAFT Reply LS on Support of UE move between CAG cell of 5G Femto and CSG cell	LG Electronics Inc.	LS out	Rel-19	FS_5G_Femto	To:SA2

=> Revised in R2-2405936

R2-2405936	DRAFT Reply LS on Support of UE move between CAG cell of 5G Femto and CSG cell	LG Electronics Inc.	LS out	Rel-19	FS_5G_Femto	To:SA2

=> Revised in R2-2405942

R2-2405683	Discussion on CSG-CAG mobility	ZTE Corporation, Sanechips	discussion	Rel-19	FS_NR_WAB_5GFemto

R2-2405684	(Draft) reply LS on support of UE move between CAG cell of 5G Femto and CSG Cell	ZTE Corporation, Sanechips	LS out	Rel-19	FS_NR_WAB_5GFemto	To:SA2	Cc:RAN3

R2-2405713	Discussion on SA2 LS regarding UE mobility between CAG cell of 5G Femto and CSG cell	Huawei, HiSilicon	discussion	Rel-19

## 8.1	AI/ML for NR air interface

(NR_AIML_air-Core; leading WG: RAN1; REL-19; WID: RP-240774)

Time budget: 2 TU

Tdoc Limitation: 5 tdocs

### 8.1.1	Organizational

LS, Rapporteur input, including workplan, etc.

R2-2405264	Rapporteur input for Rel-19 WI on AI/ML for NR air interface	Ericsson	discussion	Rel-19	NR_AIML_air-Core

=>	Noted

### 8.1.2	Functionality based LCM

Contributions should focus on general understanding of LCM procedure (except for data collection and model transfer/delivery), what is required to enable the UE to perform different steps of the LCM procedure, what is the granularity of functionality, dependencies with RAN1 and what is needed from RAN1 to progress in RAN2

Contributions should be submitted in 8.1.2.x and aspects related to data collections should be submitted in data collection section

Two-sided model discussions are out of scope of this AI

.  Model identification and model transfer/delivery is out of scope of this AI and will be discussed in RAN2#127 after further RAN1 progress

#### 8.1.2.1	LCM for NW-sided model for Beam Management use case

LCM related to NW-sided model for beam management use case only

##### Additional conditions:

R2-2405024	Discussion on LCM for NW-sided model for BM	CMCC	discussion	Rel-19	NR_AIML_air-Core

Proposal 4: UE-side additional conditions (e.g. UE speed) can be reported to gNB for management. FFS on the details of necessary UE-side additional conditions.

=>	Noted

R2-2404274	On LCM for NW-sided model for Beam Management Use Case	Qualcomm Incorporated 	discussion	Rel-19

Proposal 2: For the network-side model development, the required additional condition is left up to the network implementation.

Proposal 3: For RAN2 waits for RAN1 conclusion on

-	Whether CSI reporting can be used for training data collection, and

-	Dependence of NW-sided models on UE-side additional condition

before discussing data collection configuration and reporting, and reporting of UE-side additional conditions for LCM (training and monitoring) for NW-side models for beam management use case.

=>	Noted

Discussion

-	Xiaomi, Apple and LG agree with Qualcomm. ZTE agrees with CMCC about UE speed.  Samsung thinks that it is likely there will be a need for additional condition but we can wait for RAN1.

##### Inference:

R2-2404150	LCM for NW-sided model for Beam Management use case	OPPO	discussion	Rel-19	NR_AIML_air-Core

Proposal 3: For network-sided model for BM use case, RAN2 confirms that UE inputs for network-sided model inference will rely on L1 signaling, RAN2 will not further spend time on this aspect.

=>	Noted

R2-2404690	Discussion on LCM for NW-sided model for Beam Management use case	CATT	discussion	Rel-19	NR_AIML_air-Core

Proposal 1: RAN2 wait for RAN1’s more progress on configuration of Set A and Set B for BM-Case1 and BM-Case2 before discussing the potential RRC configurations for radio measurements and the related reporting.

Proposal 2: For inference, RAN2 assumes for NW-sided model, no discussion is needed in RAN2 until triggered by RAN1 input.

=>	Noted

Discussion

-	CATT thinks that there are some ongoing discussions. LG thinks that OPPO’s proposal is aligned with RAN1 agreeement.

##### Monitoring:

R2-2404598	Discussion on LCM for NW-sided model for BM	Xiaomi	discussion

Proposal 1: NW can configure UE to report performance monitoring data to assist NW-sided model LCM.

Proposal 2: Performance monitoring data report can be triggered by periodic, event or NW request.

=>	Noted

R2-2405185	Consideration on LCM for NW side Model for AIML based BM	ZTE Corporation	discussion	Rel-19	NR_AIML_air-Core

Proposal 3: RAN2 assumes layer 1 measurement framework can be utilized for UE to report the performance metrics to NW for NW side functionality/model management, the detail shall be discussed in RAN1, whether there is any RAN2 spec impact is FFS.

=>	Noted

R2-2405265	LCM for NW-side models for beam management	Ericsson	discussion	Rel-19	NR_AIML_air-Core

Observation 2	The gNB should monitor its own performance according to the inference process for which it is also responsible.

Proposal 1	RAN2 will work on RAN2 specifications enhancements associated to gNB-side model monitoring, only based on RAN1 inputs, if any.

=>	Noted

Agreements for NW-sided model for Beam Management

1	For the network-side model, required network side additional condition is left up to the network implementation

2	RAN2 will wait for RAN1 for any required UE side additional conditions.

3	For network-sided model for BM use case, RAN2 confirms that UE inputs for inference at network-sided model will rely on L1 signaling, RAN2 will not further spend time on this aspect.

4	The gNB is responsible for monitoring its own performance.  RAN2 will work on RAN2 specifications enhancements associated to gNB-side model monitoring, only based on RAN1 inputs, if any

Not treated

R2-2404150	LCM for NW-sided model for Beam Management use case	OPPO	discussion	Rel-19	NR_AIML_air-Core

R2-2404184	Beam management NW-sided model LCM signaling	Intel Corporation	discussion	Rel-19	NR_AIML_air-Core

R2-2404220	LCM for Positioning use case	NEC	discussion

R2-2404274	On LCM for NW-sided model for Beam Management Use Case	Qualcomm Incorporated 	discussion	Rel-19

R2-2404340	Discussion on the LCM of NW-sided model for Beam Management	Fujitsu	discussion	Rel-19	NR_AIML_air-Core

R2-2404389	Discussion on LCM for NW-sided model for Beam Management	vivo	discussion	Rel-18	NR_AIML_air-Core

R2-2404502	LCM for NW-sided model for Beam Management use case	Interdigital Inc.	discussion	Rel-19	NR_AIML_air-Core

R2-2404598	Discussion on LCM for NW-sided model for BM	Xiaomi	discussion

R2-2404636	LCM procedure of NW-sided model for AI/ML based beam management	Apple	discussion	Rel-19	NR_AIML_air-Core

R2-2404690	Discussion on LCM for NW-sided model for Beam Management use case	CATT	discussion	Rel-19	NR_AIML_air-Core

R2-2404940	LCM related to NW-side models for Beam Management	Nokia	discussion	Rel-19	NR_AIML_air-Core

R2-2405024	Discussion on LCM for NW-sided model for BM	CMCC	discussion	Rel-19	NR_AIML_air-Core

R2-2405179	Functionality-based LCM for NW sided model	Samsung	discussion	Rel-19	NR_AIML_air-Core

R2-2405185	Consideration on LCM for NW side Model for AIML based BM	ZTE Corporation	discussion	Rel-19	NR_AIML_air-Core

R2-2405265	LCM for NW-side models for beam management	Ericsson	discussion	Rel-19	NR_AIML_air-Core

R2-2405272	Discussion on LCM for NW-Side Models	Futurewei Technologies	discussion	Rel-19

R2-2405336	Discussion on functionality based LCM for NW-sided model for BM	Huawei, HiSilicon	discussion	NR_AIML_air-Core

#### 8.1.2.2	LCM for UE-sided model  for Beam Management use case

Including functionality identification, additional conditions and further reporting of applicable functionalities

##### Terminology alignment:

R2-2404691	Discussion on LCM for UE-sided model for Beam Management use case	CATT	discussion	Rel-19	NR_AIML_air-Core

Proposal 6: RAN2 to adopt the following definitions of functionality selection, activation, deactivation, switching, and fallback operation.

Functionality/model activation: Enable an AI/ML functionality/model for a specific AI/ML-enabled feature.

Functionality/model deactivation: Disable an AI/ML functionality/model for a specific AI/ML-enabled feature.

Functionality/model selection: The process of selecting an AI/ML functionality/model for activation among multiple functionalities/models for the same AI/ML enabled feature. Note: Functionality/model selection may or may not be carried out simultaneously with functionality/model activation.

Functionality/model switching: Deactivating a currently active AI/ML functionality/model and activating a different AI/ML functionality/model for a specific AI/ML-enabled feature.

Functionality/model fallback: Deactivating a currently active AI/ML functionality/model and fallback to non-AI/ML operation.

=>	Noted

R2-2405180	Functionality-based LCM for UE sided model	Samsung	discussion	Rel-19	NR_AIML_air-Core

Proposal 2: RAN2 agree the following definition for functionality types as a starting point.

-	Supported/identified functionalities: this refers to functionalities that UE can indicate by using UE capabilities.

-	Configured functionalities: this refers to functionalities that gNB can configure UE for model inference and performing measurements for training purposes?. Depending on proactive/reactive approach, configured functionalities may or may not be applicable upon configuration.

-	Applicable functionalities: this refers to functionalities that the UE is ready to apply for model inference. It can be considered as candidates for functionality activation.

-	Activated functionalities: this refers to functionalities that the UE starts predicting beam results via model inference.

Proposal 6: RAN2 focus on functionality activation/deactivation assuming that switching and fallback are supported via activation/deactivation.

-	Nokia thinks this is a bit premature

-	Lenovo thinks that we can make this from the UE perspective, and how it is implemented can be FFS.   LG thinks that some procedure will be different.

=>	Noted

R2-2404390	Discussion on LCM for UE-sided model for Beam Management	vivo	discussion	Rel-18	NR_AIML_air-Core

Proposal 1 From the RAN2’s view, functionality fallback/switching can be achieved by functionality (re-)configuration/activation/deactivation. Thus, RAN2 does not need to define separate functionality fallback/switching procedures.

=>	Noted

Discussion

-	LG, ZTE and Apple thinks Samsung proposal’s make sense.  Lenovo thinks that we should align “configured” with “configurable”. Intel thinks we should add training as the UE will also need a configuration so it can do training.

-	FUturewith thinks that functionality based we don’t need de-activation.  Intel explains that de-activation is fallback to legacy.  Samsung this that we don’t know yet and it depends on what functionality is for what use case and it could refer to switching.

##### (Functionality)Availability:

R2-2404151	LCM for UE-sided model for Beam Management use case	OPPO	discussion	Rel-19	NR_AIML_air-Core

Proposal 2: For UE-sided model for BM use case, RAN2 assumes only if UE really has the model available UE can additionally report UE-sided applicable functionality associated with the available model via non-UE capability signaling.

=>	Noted

R2-2405266	LCM for UE-side models for beam management	Ericsson	discussion	Rel-19	NR_AIML_air-Core

Proposal 2	Introduce signalling for the UE to inform the gNB whether the AI/ML functionality is available for operation (e.g., whether there are trained models available within it). FFS whether the “availability indication” can be reported as part of the applicability-reporting information, or as a separate signalling.

-	Qualcomm thinks that the only thing that needs to be report is applicable functionality.   ZTE has the same understanding as Ericsson.

-	CATT agrees with Oppo as it is similar to applicability.

-	Samsung also agrees that it is similar to applicability.

=>	Noted

##### Network side additional conditions:

R2-2404185	Beam management UE-sided model LCM signaling	Intel Corporation	discussion	Rel-19	NR_AIML_air-Core

Proposal 8: Associated ID in relation with NW-side additional condition is transmitted via RRC signalling, together with data collection related configuration. RAN2 to wait further progress in RAN1 about other details of NW-side additional condition.

=>	Noted

R2-2405184	Considerations on  LCM for UE side Model for AIML Based BM	ZTE Corporation	discussion	Rel-19	NR_AIML_air-Core

Proposal 5: 	RAN2 to wait for RAN1 decision on the consistency of NW-side additional condition across training and inference for UE-sided model issue before start to discuss the impact on RAN2 on what is needed for UE to determine applicability of a functionality for network side additional conditions.

=>	Noted

Discussion

-	Qualcomm, Samsung, LG thinks that we can make some assumptions in RAN2.

##### UE side additional/applicability conditions:

R2-2405337	Discussion on function ality based LCM for UE-sided model for BM	Huawei, HiSilicon	discussion	NR_AIML_air-Core

Proposal 4: It is proposed RAN2 to agree that the UE considers UE-side additional conditions when determining functionality applicability, and there is no need to directly exchange UE-side additional conditions between UE and NW.

=>	Noted

R2-2404599	Discussion on LCM for UE-sided model for BM	Xiaomi	discussion

Proposal 13: NW can configure UE to report UE side additional condition.

=>	Noted

R2-2404185	Beam management UE-sided model LCM signaling	Intel Corporation	discussion	Rel-19	NR_AIML_air-Core

Proposal 2: Existing UAI framework is used at least for proactive reporting of applicable functionality.  FFS reactive

Proposal 3: Beam management UE-side applicable functionality includes the mapping between the measured number of beams in Set B and the number of predicted beams in Set A. FFS on other additional conditions.

Proposal 4: During proactive reporting of UE-side applicable functionality, network configures UE via RRC signalling about whether to allow UE reports assistance information.

Proposal 5: During reactive reporting of UE-side applicable functionality network configures the candidate values for UE-side applicable functionality on top of configuration for proactive reporting. FFS on the granularity of candidate values.

Proposal 5a: During reactive reporting of UE-side applicable functionality, network further configures reporting events (e.g. deployment scenario change, UE speed change, UE memory change, etc), UE reports its applicable functionality if its applicability/preference is changed and the reporting event(s) are met via UEAssitanceInformation. FFS on the reporting events for UE-side additional information reporting.

=>	Noted

Discussion

-	LG thinks that we should respond with RRC configuration complete for the reactive approach.   Intel thinks that there is no immediately respond with RRC configuration complete.

-	Qualcomm doesn’t think the UE needs to report additional conditions.  The first steps should be to understand the size of the information and whether SRB1 can be used.   Ericsson thinks that we should understand what reactive/proactive means.

-	NEC thinks that proactive reporting is more than one time reporting.

[POST126][032][AI/ML PHY] LCM (Intel/Samsung)

Intended outcome:

Phase 1: Agreable definitions (Samsung)

Phase 2: Reach common understanding of reactive/proactive framework for applicable functionality.

Deadline:  long

Agreements

1	RAN2 will support functionality activation/deactivation after inference configuration.   FFS initial state of configuration and how activation/deactivation is achieved.   FFS what Deactivation refers to:  examples discussed: 1) fallback to legacy 2) switching, etc.

2	We will work offline on the definitions for functionality types and define what is availability.

3	The UE will indicate the gNB/LMF whether the AI/ML functionality is available/applicable.   For a functionality to be applicable at least there should at least one model available within it.   FFS other details on what is applicability/non-applicability.

4	For NW-side additional conditions, RAN2 assumes that RRC signaling from gNB to UE can be designed for consistency between inference and training.  RAN2 will wait for RAN1 input for further details.   FFS if the same applies to positioning

5	For BM use case, As a baseline the UE determines whether a functionality is applicable.  Existing UAI framework is used at least for proactive reporting of applicable functionality.  FFS reactive

Applicable functionalities (determination and indication to the network):

R2-2404817	Discussion on applicability-related information and additional condition	Lenovo	discussion	Rel-19

Proposal 1	RAN2 is suggested to clarify that the last meeting agreement about UE-side applicable functionality reporting is essentially about UE-side functionality “applicability-related information” reporting, which could mean the following according to the TR

a.	Condition/Information (e.g., scenarios, locations, configuration, deployment) under which a functionality is applicable

b.	whether functionality(es) are (non)applicable under the current condition (e.g., scenarios, locations, configuration, deployments)

Proposal 3	For the “NW-decision, NW-initiated” LCM scenario for UE-sided model, RAN2 considers the following three sub-scenarios with respect to the determination of applicable functionality, exchange of applicability-related information and UE/NW-side additional condition.

a.	Sub-Scenario #1: UE reports conditions under which a functionality is applicable; UE may report UE-side additional condition; gNB determines the applicable functionality according to the conditions reported from UE.

b.	Sub-Scenario #2: UE may receive NW-side additional condition; UE determines the applicable functionality; UE reports if a functionality is applicable.

c.	Sub-Scenario #3: UE reports conditions under which a functionality is applicable; UE reports if a functionality is determined applicable from UE point of view; gNB determines the applicable functionality according to the conditions reported from UE and among those determined to be appliable from UE point of view.

=>	Noted

R2-2404275	On LCM for UE-sided model  for Beam Management use case	Qualcomm Incorporated 	discussion	Rel-19

Proposal 2: UE determines the applicable functionalities, using

-	Model availability information for supported/configured functionalities,

-	UE-side additional condition information for supported/configured functionalities, and

-	Network side additional condition information for supported/configured functionalities.

=>	Noted

R2-2404691	Discussion on LCM for UE-sided model for Beam Management use case	CATT	discussion	Rel-19	NR_AIML_air-Core

Proposal 3: What is applicable functionality and how the UE determines which functionality is applicable or not (if it is needed) are up to RAN1.

Proposal 4: RAN2 assumes the UAI signaling is used for applicable functionality reporting no matter whether the proactive reporting or reactive reporting is used.

=>	Noted

Discussions

-	Xiaomi think it is up to the UE to decide applicability

-	LG thinks it is not clear what is UE side condition.  If it is related to configuration it can be specified in our spec.

##### Not treated

Inference:

R2-2405025	Discussion on LCM for UE-sided model for BM	CMCC	discussion	Rel-19	NR_AIML_air-Core

Proposal 9: For the configuration of Set A and Set B for UE-sided model inference, the legacy CSI-MeasConfig including CSI-ReportConfig can be enhanced based on RAN1 input.

R2-2404691	Discussion on LCM for UE-sided model for Beam Management use case	CATT	discussion	Rel-19	NR_AIML_air-Core

Proposal 10: For beam management use case, RAN2 postpones the discussion for UE-sided model inference until more progress made in RAN1.

Monitoring:

R2-2405266	LCM for UE-side models for beam management	Ericsson	discussion	Rel-19	NR_AIML_air-Core

Proposal 10	RAN2 waits for RAN1 progress related to performance monitoring reporting of UE-side functionality.

R2-2404185	Beam management UE-sided model LCM signaling	Intel Corporation	discussion	Rel-19	NR_AIML_air-Core

Proposal 9: During performance monitoring, the UE reuses CSI framework to report either ground truth label of Set A of beams or inference intermediate KPIs calculated by UE to network, together with the associated ID. It is up to RAN1 whether to report the measured RSRP of Set A of beams or other inference intermediate KPIs.

R2-2404151	LCM for UE-sided model for Beam Management use case	OPPO	discussion	Rel-19	NR_AIML_air-Core

R2-2404185	Beam management UE-sided model LCM signaling	Intel Corporation	discussion	Rel-19	NR_AIML_air-Core

R2-2404275	On LCM for UE-sided model  for Beam Management use case	Qualcomm Incorporated 	discussion	Rel-19

R2-2404341	Discussion on the LCM for UE-sided model for Beam Management	Fujitsu	discussion	Rel-19	NR_AIML_air-Core

R2-2404370	LCM for UE-sided model  for Beam Management use case	TCL	discussion	Rel-19

R2-2404390	Discussion on LCM for UE-sided model for Beam Management	vivo	discussion	Rel-18	NR_AIML_air-Core

R2-2404503	LCM for UE-sided model for Beam Management use case	Interdigital Inc.	discussion	Rel-19	NR_AIML_air-Core

R2-2404599	Discussion on LCM for UE-sided model for BM	Xiaomi	discussion

R2-2404637	LCM procedure of UE-sided model for AI/ML based beam management	Apple	discussion	Rel-19	NR_AIML_air-Core

R2-2404691	Discussion on LCM for UE-sided model for Beam Management use case	CATT	discussion	Rel-19	NR_AIML_air-Core

R2-2404816	LCM for AIML based beam management with UE-sided model	Lenovo	discussion	Rel-19

R2-2404817	Discussion on applicability-related information and additional condition	Lenovo	discussion	Rel-19

R2-2404902	UE side model functionality definition	Sony	discussion	Rel-19	NR_AIML_air-Core

R2-2404933	Discussion on LCM for UE-sided model	Spreadtrum Communications	discussion	Rel-19

R2-2404941	Functionality based LCM related to UE-side models for BM	Nokia	discussion	Rel-19	NR_AIML_air-Core

R2-2404957	Functionality identification and applicability related report	LG Electronics	discussion	Rel-19	NR_AIML_air-Core

R2-2404958	Functionality management for UE-sided model	LG Electronics	discussion	Rel-19	NR_AIML_air-Core

R2-2405025	Discussion on LCM for UE-sided model for BM	CMCC	discussion	Rel-19	NR_AIML_air-Core

R2-2405073	Discussion on LCM for UE-sided model	NEC	discussion	Rel-19	NR_AIML_air-Core

R2-2405180	Functionality-based LCM for UE sided model	Samsung	discussion	Rel-19	NR_AIML_air-Core

R2-2405184	Considerations on  LCM for UE side Model for AIML Based BM	ZTE Corporation	discussion	Rel-19	NR_AIML_air-Core

R2-2405246	LCM Framework for UE-sided model	SHARP Corporation	discussion

R2-2405266	LCM for UE-side models for beam management	Ericsson	discussion	Rel-19	NR_AIML_air-Core

R2-2405274	Discussion on LCM for UE-Side Models	Futurewei Technologies	discussion	Rel-19

R2-2405337	Discussion on functionality based LCM for UE-sided model for BM	Huawei, HiSilicon	discussion	NR_AIML_air-Core

R2-2405536	Considerations for moving forward with Functionality-based LCM	Kyocera	discussion	R2-2403574

R2-2405665	Discussion on applicable functionality	NTT DOCOMO, INC.	discussion	Rel-19

#### 8.1.2.3	LCM for Positioning use case

Contributions should focus on UE-sided model, but can discuss NW-sided model and should focus on 1st priority positioning use cases

Network side model:

R2-2404942	Functionality based LCM for Positioning	Nokia	discussion	Rel-19	NR_AIML_air-Core

Proposal 9: RAN2 to await RAN1 progress that indicates a need for specification work on AI/ML positioning Case 3a and Case 3b.

=>	Noted

R2-2404504	LCM for Positioning use case	Interdigital Inc.	discussion	Rel-19	NR_AIML_air-Core

Proposal 1: For the LCM of positioning Case 3a, no specification enhancements are needed for LPP.

Proposal 2: For the LCM of positioning Case 3a, no specification enhancements are needed for RRC.

Proposal 3: For the LCM of positioning Case 3b, no specification enhancements are needed for LPP.

Proposal 4: For the LCM of positioning Case 3b, no specification enhancements are needed for RRC.

=>	Noted

R2-2404616	Discussion on LCM for positioning use case	ZTE Corporation	discussion	Rel-19	NR_AIML_air-Core

Proposal 8: For AI/ML positioning use case 3a and 3b, RAN2 assumes the following approaches can be used for functionality-LCM:

    - LMF activates/deactivates the functionality

    - gNB activates/deactivates the functionality

Proposal 9: For AI/ML positioning use case 3a, RAN2 assumes that LMF can allow gNB to activate/deactivate the AI model of the functionality for model-LCM.

Proposal 10: For use case 3a, RAN2 assumes that gNB should to know the PRU side additional condition to determine a AI model. FFS on how gNB get such information.

Proposal 11: Send LS to RAN3 about RAN2’s agreement on use case 3a and use case 3b.

=>	Noted

Discussions

-	Samsungs agrees that for now there are no RAN2 impact. Vivo thinks we should wait for RAN1 and not agree that there are no enhancements and the LS is not needed.

-	Apple, Intel thinks that it is too early to make an agreement no enhancements as there are still things being discussed. gNB cannot play a role in the activation and deactivation so it can be either LMF or CN and we should make a decision that CN can manage the functionality.

Agreements for NWside model

1	RAN2 to await RAN1 progress to determine need for any specification work on AI/ML positioning Case 3a and Case 3b.

UE side model (Capability/Functionality Identification):

R2-2404152	LCM for Positioning use case	OPPO	discussion	Rel-19	NR_AIML_air-Core

Proposal 1: For AI/ML based positioning of Case 1, RAN2 assumes functionality ID is used by UE to indicate UE-sided applicable functionality. The reported functionality ID refers to a UE-sided supported sub-use case.

=>	Noted

R2-2405070	LCM for positioning use case	Qualcomm Incorporated	discussion

Proposal 9:	The LPP Capability Transfer procedures (RequestCapabilities/ProvideCapabilities messages) are used to dynamically indicate supported AI/ML positioning capabilities.

Proposal 5:	For AI/ML functionality identification of UE-side models, if needed, existing LPP Capability Transfer procedures (RequestCapabilities/ProvideCapabilities messages) are used to indicate to the network which AI/ML functions for positioning are supported by a UE.

=>	Noted

Discussion

-	Vivo, Samsung shares same view as qualcomm that capability can be use to transfer this information. Samsung indicates that there is functionality already that provides unsolicited capability reporting.

-	Intel thinks that we can also use assistance information similar to inventory use case, so we should wait for further progress to understand what capability would be reported.   Qualcomm thinks UAI is not suitable.

-	Ericsson thinks functionality ID is not needed.

-	CATT thinks we should use capability for reporting and there are only two models supported so no need for functionality.

-	ZTE thinks that there are other ways to identify models.

-	Qualcomm explains that we have already mechanisms that if the UE capability changes we can send another capability and there is no motivation to do new procedures.

-	Nokia thinks that while we have dynamic reporting we still need to understand how the dynamic AI/ML functionality would work.

-	Ericsson thinks that it would be good to handle things similarly for BM, so we can have common unified.

UE side model (Additional conditions/applicability):

R2-2404186	Positioning use case LCM signaling	Intel Corporation	discussion	Rel-19	NR_AIML_air-Core

Proposal 3: Associated ID representing NW-side additional conditions is configured/reported together with training data collection, inference configuration, inference result reporting (if needed) and performance metrics reporting over LPP signaling.

=>	Noted

R2-2404616	Discussion on LCM for positioning use case	ZTE Corporation	discussion	Rel-19	NR_AIML_air-Core

Proposal 2: RAN2 do not specify the mapping between UE/NW additional conditions and AI models

=>	Noted

R2-2405262	LCM for positioning use case	Ericsson	discussion	Rel-18

Proposal 5	RAN2 to support UE to provide AI/ML model applicability/validity/availability conditions to LMF via assistance data message in LPP.

=>	Noted

Discussion

-	Samsung, NEC, Qualcomm, Apple, shares the view with ZTE and we don’t need any enhancement for exchanging additional conditions.  Apple thinks that it is sufficient to say UE side model it is up to UE implementation.  Huawei thinks that how the UE checks availalabity is up to UE implenetation. Vivo thinks that this is the same for BM.

UE side model (Inference):

R2-2404942	Functionality based LCM for Positioning	Nokia	discussion	Rel-19	NR_AIML_air-Core

Proposal 4: For Case 1, inference operation the inference result is a position estimate similar to the output for a legacy UE-based positioning and hence we can re-use LPP RequestLocationInformation/ProvideLocationInformation messages. It is FFS whether any extensions to these messages are necessary to support AI/ML inference operation for Case 1.

=>	Noted

R2-2405070	LCM for positioning use case	Qualcomm Incorporated	discussion

Proposal 6:	Existing LPP procedures related to Location Information Transfer (RequestLocationInformation/ ProvideLocationInformation messages) are used for providing the results of the UE sided model inference operation.

=>	Noted

UE side model (Monitoring):

R2-2404692	Discussion on LCM for Positioning use case	CATT	discussion	Rel-19	NR_AIML_air-Core

Proposal 5: For Pos case 1, UE or LMF could perform performance monitoring. Detailed specification impact could be discussed after down selection of the monitoring sub-options by RAN1.

=>	Noted

R2-2404391	Discussion on functionality LCM for AI/ML enhanced positioning	vivo	discussion	Rel-18	NR_AIML_air-Core

Proposal 3	To support monitoring metric calculation at target UE, UE obtains ground truth label, position calculation assistance data, PRU measurement or PRU location from LMF via LPP Request/Provide Assistance Data.

Proposal 4	To support monitoring metric calculation at LMF, LMF obtains model inference output from UE via LPP Request/Provide Location Information.

Proposal 5	To help UE with model-level LCM, LMF may provide performance metrics or functionality management decision via LPP Request/Provide Assistance Data.

=>	Noted

R2-2404186	Positioning use case LCM signaling	Intel Corporation	discussion	Rel-19	NR_AIML_air-Core

For Case 1

Proposal 7: Existing LPP signalling can be used to transfer measurement information and assistance data for UE-side ground truth label calculation. LPP signalling is enhanced to support sending ground truth label from LMF to UE during monitoring at least for Option A-1. FFS whether to reuse existing Assistance Data Transfer/Delivery procedure or new procedure.

-	ZTE thinks that this also can include PRU ground truth label.  Nokia understands that UE includes PRU.  Qualcomm thinks that this is positioning procedures already to obtain UE positioning. ZTE explains that this is to give the target UE from LMF multiple PRU ground truth label.

Proposal 8: LPP signaling is enhanced to support sending inference results (e.g. predicted measurement, predicted UE location coordinates, etc) from UE to LMF during monitoring in Option B-1 and Option B-2. FFS whether to use Location Information Transfer/Delivery procedure or a new procedure.

=>	Noted

Agreements

- The LPP Capability Transfer procedures (RequestCapabilities/ProvideCapabilities messages) are used to indicate supported AI/ML positioning capabilities.  FFS how to handle dynamic capabilities, depending on further RAN1 progress and understanding of the functionality.

- wait for RAN1 for associate ID discussion

- At least for Case 1, existing LPP procedures related to Location Information Transfer (RequestLocationInformation/ ProvideLocationInformation messages) are used for providing the results of the UE sided model inference operation.  FFS further details on signaling enhancements

Not treated

R2-2404152	LCM for Positioning use case	OPPO	discussion	Rel-19	NR_AIML_air-Core

R2-2404186	Positioning use case LCM signaling	Intel Corporation	discussion	Rel-19	NR_AIML_air-Core

R2-2404219	LCM for NW-sided model  for Beam Management	NEC	discussion

R2-2404230	Lifecycle management for positioning use-cases	Fraunhofer IIS, Fraunhofer HHI	discussion

R2-2404342	Discussion on the LCM of Positioning Use Case	Fujitsu	discussion	Rel-19	NR_AIML_air-Core

R2-2404391	Discussion on functionality LCM for AI/ML enhanced positioning	vivo	discussion	Rel-18	NR_AIML_air-Core

R2-2404504	LCM for Positioning use case	Interdigital Inc.	discussion	Rel-19	NR_AIML_air-Core

R2-2404616	Discussion on LCM for positioning use case	ZTE Corporation	discussion	Rel-19	NR_AIML_air-Core

R2-2404638	LCM procedure of AI/ML based positioning	Apple	discussion	Rel-19	NR_AIML_air-Core

R2-2404692	Discussion on LCM for Positioning use case	CATT	discussion	Rel-19	NR_AIML_air-Core

R2-2404699	Discussion on LCM for Positioning use case	Samsung	discussion	Rel-19	NR_AIML_air-Core	Late

R2-2404818	LCM for AIML based positioning with UE-sided model	Lenovo	discussion	Rel-19

R2-2404888	Discussion on LCM for positioning	Baicells	discussion

R2-2404942	Functionality based LCM for Positioning	Nokia	discussion	Rel-19	NR_AIML_air-Core

R2-2405038	Discussion on LCM for positioning	CMCC	discussion	Rel-19	NR_AIML_air-Core

R2-2405070	LCM for positioning use case	Qualcomm Incorporated	discussion

R2-2405098	Discussion on LCM for AI positioning	Xiaomi	discussion

R2-2405262	LCM for positioning use case	Ericsson	discussion	Rel-18

R2-2405338	Discussion on LCM for Positioning use case	Huawei, HiSilicon	discussion	NR_AIML_air-Core

R2-2405547	Discussion on Functionality-based LCM for Positioning Use Case	CEWiT	discussion	Rel-19	NR_AIML_air-Core

### 8.1.3	NW side data collection

Contributions should focus on the mechanisms and principles identified for data collection for network side model training during rel-18

gNB centric collection:

R2-2404343	Discussion on NW side data collection	Fujitsu	discussion	Rel-19	NR_AIML_air-Core

Proposal 1: For gNB-centric data collection for beam management, RAN2 enhances the existing L3 measurement and reporting procedures as baseline and consider the inputs from RAN1.

=>	Noted

R2-2404819	Discussion on training data collection for NW-sided model	Lenovo	discussion	Rel-19

Proposal 3	RAN2 supports the logging of L1 measurement results at UE, and UE reports the logged multiple instances of L1 measurement result to gNB via RRC message as configured by gNB.

=>	Noted

Discussion

-	Qualcomm indicates that RAN1 is discussing to extend the framework for training so we should wait. Vivo agrees.

-	Apple thinks that from RAN2 perspective we can go with enhanced MDT.  Apple and ZTE thinks that the gNB centric and OAM centric solution can be the same.  CMCC, Huawei, also agrees that MDT can considered.

-	Ericsson indicates that L1 is mainly for inference but if it later agreed for training we can revisit.  And the logging is optional so the proposal from Lenovo makes sense.

-	Oppo think that from RAN2 we can still make some assumptions.

-	Xiaomi also thinks we should support logging but it should be optional. Intel agrees as it will increase power consumption. If the UE doesn’t support logging for L3 reporting then L1 reporting can be used, so it is not precluded from RAN1.

-	NEC agrees with proposal 1 and with apple and ZTE

-	Samsung thinks that we can go with enhanced MDT.

-	Ericsson explains that it is an optional feature and today from L1 reporting this is not possible.

-	Nokia doesn’t thinks that enhancement of MDT is not linked to this discussions.

-	MEdiatek is concerned with saying single RRC message as it depends on the size. Qualcomm has the same concern.

OAM centric collection:

R2-2404505	Data Collection for Network-Sided Model Training	Interdigital Inc.	discussion	Rel-19	NR_AIML_air-Core

Proposal 2: Immediate MDT is the baseline framework for OAM-centric data collection for the training of a network-sided model.

Proposal 3: Enhance the immediate MDT framework to support measurement logging and on-demand reporting.

Proposal 4: Immediate MDT measurement logging is an optional feature (i.e., based on UE capability).

=>	Noted

R2-2404693	Consideration on NW side data collection	CATT	discussion	Rel-19	NR_AIML_air-Core

Proposal 1: Enhance logged MDT in RRC_CONNECTED state for training data collection framework of BM use case.

Proposal 2: As an enhancement based on the logged MDT framework, NW could notify the UE about termination node for training data collection.

Proposal 3: Signaling based MDT and management based MDT are both applicable for the data collection of AI/ML based model training, regardless of the final decision made by RAN2 to enhance logged MDT or immediate MDT.

=>	Noted

R2-2404148	Data Collection for Network Side Model Training	OPPO	discussion	Rel-19	NR_AIML_air-Core

Proposal 2: Introduce a new MDT framework for data collection for NW-sided model training, which fulfills the following requirements:

-	Multiple samples can be reported within one report, i.e. support UE data logging;

-	Have the flexibility to collect data from any RRC states;

-	The UE can be configured by the gNB to report the logged data periodically, or event-driven, or on gNB request;

-	Extendable for future use cases.

=>	Noted

Discussions

-	InterDigital, Qualcomm, Samsung, Intel, Ericsson thinks that immediate MDT make since we agreed to support logging for the RRC messages.  Intel agrees and wouldn’t want to have a new framework. Ericsson also indicates that logged MTD doesn’t work and there are architectural problems.

-	Mediatek wonders if RAN1 should be made aware. ZTE is concerned with immediate MDT  and wants to add this should not impact RRM measruements for mobility.  Interdigital explains that we have mechanism and tools to avoid impact.

Enhance the immediate MDT framework support measurement logging (based on agreement 1), and periodical, event-based and based on network request.  Logging feature is optional (similar to agreement 1).

-	Huawei thinks that we should start with periodical reporting only.  Apple explains that with MDT framework we can do this so we shouldn’t put any other restrictions.

-	Mediatek thinks that if we want to log things in the UE we shouldn’t do immediate MDT.  InterDigital explains that the intention is to log in the UE and send the information.

-	Nokia thinks that we should assume that we use the existing CSI framework and then provide the same information in the RRC message.

-	Nokia doesn’t think this loging needs to be part of the immediate MDT. The gNB can collect the data and report it.

-	Xiaomi thinks that we should use the same framework for both gNB and OAM.

Agreements for beam management

1. 	For gNB centric and OAM centric (for RRC signaling between UE and gNB), reporting multiple instances of logged L1 measurement result from UE to gNB via a RRC message as configured by gNB is an optional feature. FFS how to handle case when single RRC message is not sufficient. FFS if there will be any further enhancement needed pending RAN1 agreement.

2.	Immediate MDT is the baseline framework for OAM-centric data collection for the training of a network-sided model

3.	Enhance the immediate MDT framework to support periodical reporting.00 FFS whether and what event-based reporting is supported and FFS on network request reporting

Data collection for positioning:

R2-2404639	Further discussion on NW-sided data collection	Apple	discussion	Rel-19	NR_AIML_air-Core

Proposal 8: For NW-sided model of case 3b of AI/ML based positioning, extend NPPRa to report dataset from gNB to LMF for model training.

Proposal 9: For NW-sided model of case 3a of AI/ML based positioning, it doesn’t need to enhance data collection because there is no dataset transfer between different entities.

R2-2405339	Discussion on  NW-sided data collection for training	Huawei, HiSilicon	discussion	NR_AIML_air-Core

Proposal 7: LPP protocol can be reused for case 3b for training data collection in the positioning case.

Proposal 8: For case 3a in the positioning case, it is proposed to re-use the conclusion of training data collection for the BM case as much as possible.

R2-2404148	Data Collection for Network Side Model Training	OPPO	discussion	Rel-19	NR_AIML_air-Core

R2-2404187	Enhancements for NW-side data collection	Intel Corporation	discussion	Rel-19	NR_AIML_air-Core

R2-2404276	On Network Side Data Collection 	Qualcomm Incorporated 	discussion	Rel-19

R2-2404317	AIML data collection for NW-side model 	NEC	discussion	Rel-19	NR_AIML_air-Core

R2-2404343	Discussion on NW side data collection	Fujitsu	discussion	Rel-19	NR_AIML_air-Core

R2-2404371	Discussion on NW side data collection	TCL	discussion	Rel-19

R2-2404392	Discussion on NW side data collection framework	vivo	discussion	Rel-18	NR_AIML_air-Core

R2-2404477	Data Collection for Network Side Model Training	MediaTek  Inc.	discussion	R2-2402363

R2-2404505	Data Collection for Network-Sided Model Training	Interdigital Inc.	discussion	Rel-19	NR_AIML_air-Core

R2-2404600	Discussion on NW side data collection	Xiaomi	discussion

R2-2404639	Further discussion on NW-sided data collection	Apple	discussion	Rel-19	NR_AIML_air-Core

R2-2404693	Consideration on NW side data collection	CATT	discussion	Rel-19	NR_AIML_air-Core

R2-2404819	Discussion on training data collection for NW-sided model	Lenovo	discussion	Rel-19

R2-2404934	Discussion on NW side data collection	Spreadtrum Communications	discussion	Rel-19

R2-2404943	Data Collection for Training of NW-side models	Nokia	discussion	Rel-19	NR_AIML_air-Core

R2-2404959	NW side data collection	LG Electronics	discussion	Rel-19	NR_AIML_air-Core

R2-2405026	Discussion on NW side data collection	CMCC	discussion	Rel-19	NR_AIML_air-Core

R2-2405186	Consideration on NW side Data Collection	ZTE Corporation	discussion	Rel-19	NR_AIML_air-Core

R2-2405200	Discussion on NW-side data collection 	Samsung	discussion

R2-2405339	Discussion on  NW-sided data collection for training	Huawei, HiSilicon	discussion	NR_AIML_air-Core

R2-2405548	Discussion on NW Side Data Collection	CEWiT	discussion	Rel-19	NR_AIML_air-Core

R2-2405667	NW-side data collection for beam management use cases	Ericsson	discussion

### 8.1.4	UE side data collection

Study part of WID - Contributions should focus on the mechanisms identified for data collection for UE side model training during rel-18

Including outcome of [POST125bis][020][AI/ML PHY] UE side data collection (Mediatek)

R2-2404476	Report of [POST125bis][020][AI/ML PHY] UE side data collection	MediaTek Inc.	discussion

=>	Revised in R2-2405931

R2-2405931	Report of [POST125bis][020][AI/ML PHY] UE side data collection	MediaTek Inc.	discussion

Proposal 1: [27/29] The term ‘server for data collection for UE-side models training’ is introduced in the definitions/descriptions of solution 1b, 2, and 3, represents that the server inside of MNO's network. The server for data collection for UE-side model training may or may not be used for training of UE-side models.  It is called as ’OTT server’ if the server is outside of MNO’s network.

-	Tmobile finds the wording very confusing and we should use trusted vs. untrusted and where the server is is not important.

-	Mediate explains that before we had terminology of OTT and we never discussed this.

-	Samsung agrees with Tmobile and we should care whether it is trusted or untrusted and we don’t talk about ownership but just security aspects.

-	Apple thinks that new wording is more confusing, it is trusted by network vendors or operators.   Samsung explains that you can have trusted WiFi and that means that there needs to be secure tunnel.

-	Samsung thinks that even the server is outside, it can have a secure tunnel and it is the same as if the UE is inside the MNO.

-	Verizon thinks that if we say inside we mean that it is a trusted one and it doesn’t matter where it is physically.

-	Huawei asks if we can assume that if the server is inside the MNO it means it is owned by the MNO.

-	Oppo thinks that we are talking about ownership.  If it is owned by the operator it can be inside or outside.   The server can also be owned by a third party.

-	Verzion thinks that we should just keep inside/outside and inside always means trusted.  Futurewei thinks that we should still take into account ownership.

=>	Noted

Agreements

1	Solution 1a has no specification impact.

Discussion on termination point

-	Nokia and Samsung is not clear what first termination point is.  Vivo agrees and thinks that we should discuss all the nodes involved in the process.   Oppo thinks we need an anchor point, like for OAM we have the TCE.

-	Samsung thinks that define the entity that communicates with the server? Huawei thinks that the most important point is which is the final MNO point.

-	Ericsson asks if we need to define what is the termination entity as these entities need to communicated.

[AT126][020][AI/ML PHY] UE side data collections (Mediatek)

Intended outcome: Agreable table for UE side data collection and clarification of visibility, levels of visibility, and standardized vs. non-standarize

Deadline:  05-24-24

R2-2406000	Report of [AT126][020][AI/ML PHY] UE side data collections	Mediatek Inc.	discussion	Rel-19	NR_AIML_air-Core

Table 1 Characteristics of different options for training data collection for UE-side models

| Aspects | 1a) OTT (3GPP Transparent) | 1b) The server for training data collection for UE-side models (3GPP non-transparent) | 2. Transfer via Core Network | 3. Transfer via OAM |
| --- | --- | --- | --- | --- |
| Inside/outside MNO’s network (FFS) | Outside | FFS: Inside or Outside | Inside FFS: Outside | Inside FFS: Outside |
| FFS whether we talk about Trusted/untrusted Ownership |  |  |  |  |
| First termination entity | OTT server | The server for data collection for UE-side model training | Inside the CN | gNB if CP tunnel OAM |
| UP/CP tunnel | UP tunnel (Note: data collection may be charged as normal traffic.) | UP tunnel (Note: data collection may be charged as normal traffic.) | CP tunnel (provided the data volume remains within the NAS signalling capacity) FFS: UP tunnel | CP tunnel (provided the data volume remains within the RRC signalling capacity) FFS: UP tunnel |
| Protocol layer for data transfer | Application layer | Application layer | NAS layer for CP tunnel FFS: the protocol layer for UP tunnel | RRC layer for CP tunnel FFS: the protocol layer for UP tunnel UP tunnel |
| Controllability of MNO on data transfer | No specific controllability | Has controllability FFS: level of controllability | Full controllability (Note 1) | Full controllability (Note 1) |
| Control granularity by NW | NA, the OTT server can directly request data from the UE. | Example: per PDU sessions based on SLA | NAS procedure | RRC procedure |
| Visibility of data content in MNO | No visibility | FFS No visibility, partial visibility, Full visibility | Full visibility (Note 2) Partial Visibility? | Full visibility (Note 2) |
| Data format | Out of 3GPP scope | FFS | Standardized FFS: non-standardized ? | Standardized FFS: non-standardized |
| Involved WGs | No, out of 3GPP scope | SA2, RAN2 | SA2, RAN2 | SA5, SA2, RAN2 |
| - Note 1: Full controllability: The MNO has the capability to manage data transfer to the server for UE-side data collection. This includes initiating, terminating, and fully managing the volume of data. (Subject to refinement and modification) - Note 2: Visibility of data content signifies the capability of the MNO to, at least, be aware of, access, and comprehend the data during transfer. (Subject to refinement and modification, the scope does not exclude additional requisites, such as the ability to modify the collected data.) |  |  |  |  |

R2-2406000	Report of [AT126][020][AI/ML PHY] UE side data collections

-	Qualcomm asks what does the FFS on 2,4 mean as we can’t resolve it in RAN2. Mediatek explains that it is not clear.

-	Ericsson thinks that these bullets are a fact and we should keep all of them.

-	Tmobile explains that for partial visibility we can have parts of the data standardized and other

Agreements

For the options identified to realize the different levels of data content visibility

1.	Full visibility for standardized data content.

2.	Partial visibility for partially standardized data content.

3.	No standardized visibility

NOTE in the TR that RAN2 discussed that visibility can be achieved as per SLA only but is outside of the scope of our discussions.

Discussions

-	Qualcomm thinks that Option 1b) can have full visibility and partial visibility.  Futurewei thinks that full visibility is not possible with option 1b).    Oppo thinks that for option 1b the visibility is only via SLA.  The Visibility doesn’t come for free.   Qualcomm thinks that we have standardized data for 25.532.   Oppo thinks that the referenced spec 25.532 is about API and the operator needs to cooperate to get it otherwise it is not for free.

-	Ericsson thinks that Visibility from RAN2 means that the requirement

Agreements

1	Capture table below in TR (Rapporteur will clean up and refine and review over email)

2	Capture the privacy concerns from different stakeholders as informative annexes in the TR. All the options (1a, 1b, 2, 3) should respect those privacy concerns. Details are up to TR rapporteur.

| Aspects | 1a) OTT (3GPP Transparent) | 1b) The server for training data collection for UE-side models (3GPP non-transparent) | 2. Transfer via Core Network | 3. Transfer via OAM |
| --- | --- | --- | --- | --- |
| First termination entity | OTT server | The server for data collection for UE-side model training | Inside the CN (e.g., LMF) | Inside OAM domain |
| AI/ML-specific Data Transfer Path | UE to OTT server via either 3GPP or non-3GPP network | UE-> CN ->Server for data collection for UE-side model training/OTT server (Note 4) | UE-> CN -> Server for data collection for UE-side model training/OTT server (Note 4) | UE->gNB->OAM-> Server for data collection for UE-side model training/OTT server |
| UP/CP tunnel | UP tunnel | UP tunnel | CP tunnel (provided the data volume remains within the NAS signalling capacity) FFS: UP tunnel | CP tunnel (provided the data volume remains within the RRC signalling capacity) FFS: UP tunnel |
| Protocol layer for data transfer | Application layer | Application layer | NAS layer for CP tunnel FFS: the protocol layer for UP tunnel | RRC layer for CP tunnel FFS: the protocol layer for UP tunnel |
| Controllability of MNO on data transfer | No AI/ML specific controllability | Has controllability FFS: level of controllability | Full controllability (Note 1) | Full controllability (Note 1) |
| Control granularity by NW | NA, the OTT server can directly request data from the UE. | Example: per PDU sessions | NAS procedure, FFS impact to other layers | RRC procedure |
| Possible Options for Visibility of data content in MNO and Data format (Note 2, Note 3) | No standardized visibility | FFS | Opt A) Full visibility for standardized data content. FFS Opt B) Partial visibility for partially standardized data content. FFS Opt C) No standardized visibility FFS: meaning of ‘partial/partially’ SAME for OPTION 3 | Full visibility for standardized data content. Partial visibility for partially standardized data content No standardized visibility FFS: meaning of ‘partial/partially’ and how to achieve different levels of visibility |
| Involved WGs | NA | SA2, SA3, RAN2 | SA2, SA3, RAN3 RAN2, CT1 and CT3 | RAN2, RAN3, SA3, SA5, FFS SA2 |
| - Note 1: Full controllability: The MNO has the capability to manage data transfer to the server for UE-side data collection. This includes initiating, terminating, and fully managing data transfer. (Subject to refinement and modification) - Note 2: Visibility of data content signifies the capability of the MNO to, at least, be aware of, access, and comprehend the data during transfer. (Subject to refinement and modification, the scope does not exclude additional requisites, such as the ability to modify the collected data.) - (To be updated with definition in agreement) Note 3: For Solution 1b, 2/3, the following options are identified to realize the different levels of data content visibility if different levels of data content visibility to MNO are considered. FFS on the data content visibility via SLA. - Full visibility for standardized data content. - Partial visibility for partially standardized data content. - No visibility for non-standardized data content. - Note 4: The potential involvement of NF or other higher layers entities/functionalities should be discussed in other WGs. |  |  |  |  |

.

[POST126][034][AIML PHY] TP for data collection  (Ericsson)

Intended outcome: Prepare and review TP capturing table in minutes and agreements on data collection

Deadline:  long

Not treated

R2-2405634	Discussion on UE side data collection	China Unicom	discussion	NR_AIML_air-Core

R2-2405271	Discussion on Data Collection for UE-side Model Training	Futurewei Technologies	discussion	Rel-19

R2-2404149	Data Collection for UE Side Model Training	OPPO	discussion	Rel-19	NR_AIML_air-Core

R2-2404188	Discussion on UE-sided  training data collection	Intel Corporation	discussion	Rel-19	NR_AIML_air-Core

R2-2404196	Data collection for UE side model training	Xiaomi	discussion	Rel-19	NR_AIML_air-Core

R2-2404221	Data Collection for UE side Model training	NEC	discussion

=> Revised in R2-2405851

R2-2405851	Data Collection for UE side Model training	NEC	discussion

R2-2404277	On UE-side Data Collection	Qualcomm Incorporated 	discussion	Rel-19

=> Revised in R2-2405716

R2-2405716	On UE-side Data Collection	Qualcomm Incorporated	discussion	Rel-19

R2-2404393	Discussion on UE side data collection	vivo	discussion	Rel-18	NR_AIML_air-Core

R2-2404476	Report of [POST125bis][020][AI/ML PHY] UE side data collection	MediaTek Inc.	discussion

=> Revised in R2-2405931

R2-2405931	Report of [POST125bis][020][AI/ML PHY] UE side data collection	MediaTek Inc.	discussion

R2-2404506	Data Collection for UE-Sided Model Training	Interdigital Inc.	discussion	Rel-19	NR_AIML_air-Core

R2-2404640	Further discussion on UE-sided data collection	Apple	discussion	Rel-19	NR_AIML_air-Core

R2-2404694	Consideration on UE side data collection	CATT	discussion	Rel-19	NR_AIML_air-Core

R2-2404820	Other aspects related to training data collection for UE-sided model	Lenovo	discussion	Rel-19

R2-2404935	Discussion on UE side data collection	Spreadtrum Communications	discussion	Rel-19

R2-2404944	Data Collection for Training of UE-side models	Nokia	discussion	Rel-19	NR_AIML_air-Core

R2-2405027	Discussion on data collection for UE-sided model training	CMCC	discussion	Rel-19	NR_AIML_air-Core

R2-2405187	Consideration on UE side Data Collection	ZTE Corporation	discussion	Rel-19	NR_AIML_air-Core

R2-2405271	Discussion on Data Collection for UE-side Model Training	Futurewei Technologies	discussion	Rel-19

R2-2405340	Discussion on  UE-sided data collection for training	Huawei, HiSilicon	discussion	NR_AIML_air-Core

R2-2405549	Discussion on UE Side Data Collection	CEWiT	discussion	Rel-19	NR_AIML_air-Core

R2-2405602	AI/ML Data Collection Requirements	"	T-Mobile USA, Verizon, Charter, NTT DOCOMO, Deutsche Telekom, Turkcell, BT, AT&T, Nokia, Telecom Italia, CMCC"	discussion	Rel-19	NR_AIML_air	Late

R2-2405634	Discussion on UE side data collection	China Unicom	discussion	NR_AIML_air-Core

R2-2405656	Data collection for UE-side model training	Samsung	discussion

R2-2405669	UE-side Data Collection	Ericsson	discussion

## 8.2	Ambient IoT

(FS_Ambient_IoT_solutions,leading WG: RAN1; REL-19; SID: RP-240826)

Time budget: 2 TU

Tdoc Limitation: 4 tdocs

NOTE: contributions should focus on technical aspects of topology 1 only in RAN2#126 (or common aspects of topology1/topology2) to progress on some basic required functionality.

### 8.2.1	Organizational

LS, Rapporteur input, including workplan, etc.

R2-2405615	TP for TR 38.769 update and terminologies	Huawei, CMCC, T-Mobile USA	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 1:	RAN2 to use the following terminologies for discussion in the study phase/TR:

	A-IoT paging: The function to be used for the initial trigger message to indicate device(s) that need to respond.

	A-IoT random access procedure

Proposal 2:	The Text Proposal in section 2 to capture the last meeting agreements is for information.

=>	Noted

[POST126][021][AIoT] TP (Huawei)

Intended outcome: Capture agreed solutions/options from this meeting

Deadline:  long

### 8.2.2	Stage 2 General aspects

Stage 2 overall procedure/message flow for three different cases: inventory, command only, inventory and command, taking into account SA2 and other WGs progress into account

Definitions of Inventory and Command

R2-2404192	Overall procedures for Inventory and Command use cases	Intel Corporation	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 2: To add the following terminology of Inventory and Command in the TR 38.769:

-	Inventory: A procedure used by Reader to discover and acquire the identifier of a single or group of AIoT device(s);

-	Command: A procedure used by Reader to send an operation request (e.g. Read, Write) to an AIoT device.

=>	Noted

R2-2404231	Discussion on stage 2 overall procedures for Ambient IoT	CATT, CEPRI	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 1: RAN2 to discuss the definition of “inventory” and “command” respectively:

-	Inventory: Refers to collect the identities of all or a group of or one of AIoT devices in range of a reader.

- Command: Refers to an instruction sent by an AF to a group of or an AIoT device(s). The following instructions may be supported:

•	Read: Reading data from the required AIoT device(s);

•	Write: Writing data to the required AIoT device(s);

•	Disable/enable: Disable/ enable the required AIoT device(s).

=>	Noted

Cases to support

R2-2404569	Ambient-IoT General Aspects	NEC	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 3:	RAN2 assume “Inventory only”, “command only” and “inventory + command” cases can be supported by above procedure consisting Step A (or Modified Step A), Step B and Step C.

-	Vodafone is concerned with security for option inventory + command.  Apple thinks that inventory + command is an optimization so let’s start with inventory and command.

=>	Noted

Inventory only Procedure

R2-2405041	General aspects and overall procedure	Huawei, HiSilicon, Wiliot Ltd., Orange, LG Uplus, NTT DOCOMO, INC.	discussion

Proposal 1a:	As baseline, the “inventory only” case is supported by the procedure (this doesn’t preclude the possibility that some messages can be combined in stage-3):

	Step A: A-IoT paging;

	Step B: A-IoT random access, if needed;

	Step C1: device to reader transmission including the device ID for inventory/identification;

=>	Noted

R2-2404809	Considerations on general aspects for Ambient IoT	Lenovo	discussion	Rel-19

Proposal 1: Procedure for inventory only case, includes Step A and Step B baseline procedure but without Step C baseline procedure, as depicted in the Figure 1.

=>	Noted

R2-2404628	Outstanding stage-2 issues of Ambient IoT design	Apple	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 2: to amend the agreement on the baseline procedure as follows:

a.	Step A: Based on the indication from 5GC (NG-AP message for topology 1, NAS message for topology 2) or its own determination, the reader sends the Initial Trigger Message indicating device(s) that need to respond. The message carries: device id(s), or device group id(s), or no idea (to trigger all the devices in the area); Details FFS

b.	Step B: Triggered device(s) performs the random access-like procedure, if needed; Details FFS

c.	Step C: The device may perform the data communication (i.e. UL/DL A-IoT MAC PDU(s) exchange) with the reader as needed,: Details FFS

=>	Noted

Discussion

-	Oppo thinks that step C would depend on the RA procedure and it could be FFS if device ID will be part of the procedure.

-	Lenovo would like to clarify that only device ID will be included in the message.

-	Mediatek and CATT think that we should have a unified procedure.

=>	AIoT paging terminology will be discussed and refined later

Inventory + Command Procedure

R2-2405041	General aspects and overall procedure	Huawei, HiSilicon, Wiliot Ltd., Orange, LG Uplus, NTT DOCOMO, INC.	discussion

Proposal 1b:	As baseline, the “inventory and command” case is supported by the procedure (this doesn’t preclude the possibility that some messages can be combined in stage-3):

Step A: A-IoT paging;

Step B: A-IoT random access, if needed;

Step C1: device to reader transmission including the device ID for inventory/identification;

Step C2:

C2a: reader to device data transmission (e.g. the DL command), and

C2b: corresponding device to reader data transmission (e.g. the feedback).

-	Mediatek explains that this should be part of a single procedure. Huawei confirms that we would capture a unified procedure at the end in the TP after this discussion.

-	Intel has a different opion as it will be simpler to capture the procedures separaterely.

-	Qualcomm ask what does inventory and command mean. The only use case for this is that if there is just DL command then the reader can trigger inventory and then command Samsung explains that the UE does first inventory and then does command.

-	Intel explains that there is a case in SA2 where both inventory and command are sent at the same time to the reader.

=>	Noted

Command Procedure

(whether to support command in the initial trigger message)

R2-2405039	General considerations on A-IoT	CMCC	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 2: RAN2 assumes the end-to-end protection of Commands and Command Results are addressed by SA3 and SA2 via NAS security.

Proposal 3: Command message can be included in A-IoT paging message, and RAN2 supports command only procedure.

=>	Noted

R2-2404498	General aspects for Ambient IoT	Ericsson	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 3	RAN2 assumes that command procedure targets only registered devices.

Proposal 4	RAN2 to study command only procedure unless SA3 indicates a security concern on command only procedure.

=>	Noted

R2-2405598	Discussion on Stage 2 aspects for Ambient IoT	NTT DOCOMO, INC.	discussion	Rel-19

Proposal 1.	Do not support Read and Write command in Initial Trigger Message or leave discussion to check by SA3.

=>	Noted

Discussion (on supporting R2D only command)

-	Intel thinks that we should include this scenario and add some condition on SA2 conclusion and we shouldn’t link to SA3

-	Docomo asks how it would work in the case of multiple devices. Interdigital thinks that there are advantage and whether we support it it depends on SA2/SA3.

-	Huawei thinks we need to assume that there will have to be security.

-	ZTE doesn’t thinks that we should be able to send a write command to all UEs in agroupd.

Agreements 1 As baseline, the “inventory only” case is supported by the procedure: - Step A: A-IoT paging; - Step B: Device ID transmission (via Random Access or without using RA). Details are FFS 2 As baseline, the “inventory and command” case is supported by the procedure: - Step A: A-IoT paging; - Step B: Device ID transmission (via Random Access or without using RA). Details are FFS - Step C: reader to device data transmission (e.g. the R2D command), and - Step D: corresponding device to reader data transmission (e.g. the feedback). FFS whether this is optional, pending other WG discussions. Clarify in TR that inventory and command doesn’t mean that AIoT paging includes both Inventory and Command in the same message. This doesn’t mean that inventory and command are received by the reader at the same time from upper layer. 3 From RAN2 point of view we will study “Command only” use case. FFS the options on how to support it : Initial trigger message from the reader contains the command. Final feasibility depends on SA2 and SA3 work/conclusions. Use baseline procedure for “inventory and command”(i.e. first triggers inventory procedure and then sends command)

(signaling of the command and support of group)

R2-2404192	Overall procedures for Inventory and Command use cases	Intel Corporation	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 8: To support Command procedure for single device, the message “Command” and “Device Response” shall be specified in Data link layer (e.g. as part of A-IoT MAC). FFS on the relationship between the message defined in RAN2 and SA2.

=>	Noted

R2-2404533	General Aspects on Ambient IOT	InterDigital	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 7:	The “command” phase of the procedure consists of the reader transmitting a message containing at least upper layer data, and the device responding with upper layer data or acknowledgement.  FFS whether the acknowledgement is an upper layer acknowledgement or AS layer acknowledgement.

=>	Noted

R2-2404983	Discussion on general aspects for Ambient IoT	HONOR	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 2: RAN2 to take the following description of the command use case as baseline for further study

CN sends the command to the Reader.

Reader transmits the command related message to target device(s).

(Optional) Reader receives and transmits the feedback (data for read, FFS UL ack for read/write/disable) to CN.

FFS support of DL command for one/group of/all device(s).

=>	Noted

R2-2405383	Overall procedures for Ambient IoT 	Kyocera 	discussion	Rel-19

Proposal 9	RAN2 should aim to define one common procedure for all use cases, the “inventory”, the “read command” and the “write command”.

=>	Noted

Not treated

Assumptions on device type

R2-2405212	General aspects of Ambient IoT	Qualcomm Incorporated	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 1:  	RAN2 will aim to have common protocol and procedures for different device types (with/without DL/UL amplification and/or UL transmission generation) as much as possible.

R2-2404628	Outstanding stage-2 issues of Ambient IoT design	Apple	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 1: RAN2 to consider two A-IoT device types (e.g. low-end and high-end) in terms of their baseband/RAN2-related capabilities.

R2-2404344	Discussions on General Aspect of Ambient IoT	Fujitsu	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2404381	Discussion on general aspects of AIoT	Futurewei	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2404394	General discussion on ambient IoT	vivo	discussion	Rel-18	FS_Ambient_IoT_solutions

R2-2404507	Control and user plane modelling for ambient IoT interface	MediaTek Inc.	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2404587	Stage 2 overall procedure flow	OPPO	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2404873	Considerations on the general aspects of the Ambient IOT	Beijing Xiaomi Software Tech	discussion	Rel-19

R2-2404876	Further consideration on general aspects for AIoT	ZTE Corporation, Sanechips	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2404929	Discussion on general aspects of A-IoT	Spreadtrum Communications	discussion	Rel-19

R2-2405244	Further on general aspects of AIoT	Nokia	discussion	FS_Ambient_IoT_solutions

R2-2405291	Discussion on general aspects of ambient IoT	LG Electronics Inc.	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2405305	Discussion on stage-2 aspects for Ambient IoT	China Telecom	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2405465	General aspects for AIoT	Samsung	discussion	Rel-19	FS_Ambient_IoT_solutions

### 8.2.3	Functionality aspects

Contributions should focus on the functionalities required for A-IoT devices, including security related questions to SA3, need of resource allocation, segmentation/reassembly (pending RAN1 progress), QoS handling, higher layer repetition, BSR, SR, etc.?

MAC or AIOT Layer Functionality

(Logical channels and multiplexing)

R2-2405040	Discussion on protocol stack of A-IoT	CMCC	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 5: Logical channel is not considered for A-IoT.

Proposal 6: Multiplexing and LCP function are not supported for A-IoT.

=>	Noted

R2-2405214	Views on Functionality Aspects of Ambient IoT	Qualcomm Incorporated	discussion	FS_Ambient_IoT_solutions

Proposal 4:	The concept of logical channels as in NR MAC is also supported for AIoT.

=>	Noted

R2-2404879	Ambient-IoT Functionality Aspects	NEC	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 2:	RAN2 to define two mac layer channels:

−	One for data;

−	Another one for signaling.

=>	Noted

Discussions on logical channels

-	Fujitsu thinks that the terminology should be there for logical channels for MAC specification perspective.  Interdigital thinks that we should exclude supporting multiple logical channel and multiplexing rather than the concept.

-	Qualcomm asks how we would differentiate control MAC CE. This is a modeling issue, and there can be a code point to indicate the type of message.  Huawei agrees as if we use the concept of logical channel will be confusing.

-	Samsung thinks that we need to discuss the need to differentiate data.  Lenovo thinks that there is no need for multiple logical channel.

-	MEdiatek thinks device complexity is bad. No need for multiple logical channel and no need to differentiate between data and control on channel basis.

-	Kyocera asks how to handle the reader in case of multiple devices.

(SR)

R2-2404586	Discussion on needed functionalities for Ambient IoT communication	OPPO	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 4	SR is not needed for A-IoT communication while focusing on DT and DO-DTT.

=>	Noted

(BSR)

R2-2405466	Discussions on functionalities required for AIoT	Samsung	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 9: RAN2 is kindly asked to discuss the BSR-like function to facilitate the resource allocation at the reader.

=>	Noted

R2-2404345	Discussions on AIoT Functionalities	Fujitsu	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 4: If the buffer size is larger than the TBS, the A-IoT device may piggyback a BSR or SR in the uplink data transmission.

=>	Noted

R2-2404659	Disucssion on functional aspects for Ambient IoT	Apple	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 7	BSR in A-IoT air interface to solicit UL grant is not supported for DO-DTT.

=>	Noted

Discussions on need of BSR-like

-	Vivo thinks BSR is useful. Mediatek sees some value for Fujitsu’s case where the reader doesn’t know the expected size of the response, but we shouldn’t reproduce the NR SR/BSR concept. Nokia agrees with Mediatek wrt legacy BSR, however there may be some cases where it can be beneficial. Ericsson agrees that we shouldn’t support the full legacy BSR as it is.

-	Qualcomm thinks that there is benefits to indicating some message size, but it can be done differently than BSR. Xiaomi thinks that there are two cases 1) signaling and 2) segmentation so key point is whether we support segmation. 1 bit BSR would be enough.

-	Intel and CMCC doesn’t think there is BSR-based legacy scheduling is supported. Interdigital and Samsung doesn’t think that we are signaling message size, just status indication.

-	Huawei and Ericsson thinks that we can keep it open as maybe one bit may not be sufficient.

-	Google thinks padding is needed.

(Duplication discard and sequence number)

R2-2404395	Discussion on functionality aspects for Ambient IoT	vivo	discussion	Rel-18	FS_Ambient_IoT_solutions

Proposal 15	Data duplication discard is not supported for AIoT service.

Proposal 16	No MAC layer sequence number is needed for service data transmitted in MAC PDU.

=>	Noted

(Capability Reporting)

R2-2404155	Discussion on functions in protocol stack for ambient IOT	Xiaomi	discussion	Rel-19

Proposal 17: The device capability is reported to reader including static device capability and dynamic device capability.

=>	Not treated

R2-2404395	Discussion on functionality aspects for Ambient IoT	vivo	discussion	Rel-18	FS_Ambient_IoT_solutions

Proposal 6	RAN2 assumes that device type and device radio capability information reporting from device to reader are not necessary.

=>	Not treated

(Segmentation)

R2-2404395	Discussion on functionality aspects for Ambient IoT	vivo	discussion	Rel-18	FS_Ambient_IoT_solutions

Proposal 11	RAN2 should determine whether segmentation is needed based on the following input from RAN 1:

-	The maximum sustainable time based on the AIoT device energy storage;

-	The required duration for R2D/D2R transmission of single packet (e.g. 1000bit).

Proposal 12	Segmentation, if needed, should be considered as a functionality in MAC layer.

=>	Not treated

(Repetition)

R2-2404795	Functionality aspects for A-IoT	Ericsson	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 11	Study the need for retransmissions and repetition in MAC or in a new AS layer above MAC.

=>	Noted

R2-2405042	A-IoT functionalities	Huawei, HiSilicon	discussion

Proposal 6:	RAN2 will not support AS higher-layer repetition. (This does not preclude R2D message retransmission by reader implementation.)

=>	Noted

Discussion

-	Intel and Qualcomm think that legacy repetition shouldn’t be supported. ZTE is concerned that if you are doing segmentation the reader would have to retransmit everything. At least MAC retransmissions should be allowed.

-	Qualcomm explains that RAN1 support PHY retransmissions, but we can agree to no “RLC-like” repetition. Huawei would like to clarify no buffer based retransmission is supported.Samsung thinks that a failure may require a retransmission, but R2D upper layer retransmission can be done by implementation.

-	Lenovo sees benefits from R2D repetition if the device doesn’t receive it.

-	Ericsson would like to ensure we don’t preclude retransmitting the payload.

-	ZTE would like to ensure that this doesn’t preclude retransmitting a segment in case of segmentation.

Agreements on functionality

1	Multiple “AIoT logical channels” for upper layer data are not supported. FFS if AIoT logical channel concept is used depending on final modeling issue.

2	legacy NR BSR/SR is not needed for A-IoT communication.

3	FFS whether further indication of device message size/status is needed

4	AS-layer (above PHY layer) RLC-like sretransmission/repetition is not supported. This doesn’t preclude the reader and device sending the payload again as new transmission from MAC perspective. FFS how we handle segmentation case (if needed)

Visibility of inventory/command information at the AS layer

R2-2404795	Functionality aspects for A-IoT	Ericsson	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 4	Study whether Reader is aware of the below information:

a.	Message and service type (e.g., inventory or command etc)

b.	Information on targeted devices (e.g., group ID, device ID and number of devices)

c.	Device capabilities (e.g., device type (1, 2a or 2b) and/or whether device supports frequency shift)

d.	FFS other information.

=>	Not treated

Security and SA3/SA2 LS

ON need for sending LS

-	Huawei and Intel thinks we can just include our agreements

-	Vivo thinks we should ask some questions.

-	Ericsson indicates that all groups are working at the same time. Suggests to ask if we can assume certain things, like is device ID protected, etc. Vodafone agrees that we should at least indicate what we did assume so far. Also ask if we should also provide our agreements to SA2.

-	Tmobile thinks that we should at least ask if NAS security assumption.

-	CMCC also ask if the ID can be protected.  Huawei doesn’t want to ask what they already started to discuss.

-	Mediatek indicates that there are two reasons, 1) provide information from RAN2 and 2) ask if the information we are sending over AS needs to be protected

=>	Send an LS to SA2/SA3

-	provide agreements and RAN2 assumptions so far and ask if they have any feedback

[POST126][035][AIoT] LS to SA2/SA3  (Oppo)

Intended outcome: Agree to LS to SA2/SA3 ccRAN3 capturing some relevant agreements and RAN2 assumptions.

Deadline:  1 month

= Approved in R2-2406150

Not treated

R2-2404586	Discussion on needed functionalities for Ambient IoT communication	OPPO	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 1	RAN2 to send LS to SA3 asking the above questions.

Proposal 2	In the LS, include a question to SA2 asking whether NAS layer will be introduced for Ambient IoT.

R2-2404395	Discussion on functionality aspects for Ambient IoT	vivo	discussion	Rel-18	FS_Ambient_IoT_solutions

(Configuration Signalling and validity)

R2-2404501	AIoT read and write operations	MediaTek Inc.	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 2: Signalling messages between the reader and the device are modelled as read and write operations whose addressing indicates specific functionality.  FFS if the addresses are logical locations with specified behaviour or a separate form of data.

(Consideration of Stored Energy)

R2-2405269	Considerations for functionality aspects	Semtech Neuchatel SA	discussion

Proposal 1:  	RAN2 to study mitigation procedures of when an A-IoT device has insufficient energy storage to complete the transaction (for MO and MT transactions).

R2-2405233	On AIoT functionality aspect	Nokia	discussion	FS_Ambient_IoT_solutions

Proposal 9: RAN2 to study energy-level reporting by the AIoT device(s) as assistance information.

R2-2404193	Required functions for A-IoT	Intel Corporation	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2404232	Discussion on the Functionality Aspects for Ambient IoT	CATT, CEPRI	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2404382	On not supporting segmentation and reassembly in Layer 2 for AIoT	Futurewei	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2404508	Transfer of upper-layer AIoT information	MediaTek Inc.	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2404521	Considerations on functionality aspects for Ambient IoT	Lenovo	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2404534	Functions for Ambient IOT	InterDigital	discussion	Rel-19	FS_Ambient_IoT_solutions

=> Revised in R2-2405697

R2-2405697	Functions for Ambient IOT	InterDigital	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2404538	A-IoT Functionality	ZTE Corporation, Sanechips	discussion

R2-2404925	Discussion on the functionalities required for Ambient IOT	Spreadtrum Communications	discussion	Rel-19

R2-2404981	Discussion on functionality aspects of ambient IoT	KT Corp.	discussion

R2-2405306	Discussion on functionalities required for A-IoT devices	China Telecom	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2405384	Functionalities for Ambient IoT 	Kyocera 	discussion	Rel-19

R2-2405520	Discussion on user plane aspects for Ambient IoT	LG Electronics Inc.	discussion	FS_Ambient_IoT_solutions

R2-2405691	Discussion on functionality aspects for Ambient IoT	Philips International B.V.	discussion	Rel-19

### 8.2.4	A-IoT Paging

Contributions should focus on paging aspects and content required for Ambient IoT for the different identified procedures (i.e. inventory, inventory + command, command only), including monitoring of DL message and determination of transmission/access occasion.

Terminology

R2-2405495	Discussion on A-IoT paging	Samsung	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 1: RAN2 to use a unified terminology to call the A-IoT paging-like message, between ‘initial trigger message’ and ‘A-IoT paging message’, during SI phase.

-	Intel feels that companies are trying to put too many things into AIoT paging and that may be bit complicated.  We should try to distinguish between Inventory and command (read/write).

-	ZTE thinks that for convenience it’s ok to keep this terminology for now but this paging can include initial and we can have some other DL messages.

-	Mediatek prefers the initial trigger message as it has legacy implications. Qualcomm also prefers initial trigger message. CMCC thinks that initial trigger message implication that this is only for one case first access and not follow up access. LG thinks we should align the terminology.

-	Huawei thinks that in the TR we can refer to paging/initial trigger message but for simplicity during discussions we can call it AIoT paging. Oppo is concerned that paging is for one or some UEs, but in AIoT there is a case that the reader doesn’t know all the UEs.

=>	Noted

Contents of the paging message

(selection criteria)

R2-2405227	Discussion on DL messages for Ambient IoT UEs	Ericsson	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 4	The initial trigger message (for inventory) should address the following cases:

•  a message containing an ID of a single A-IoT device.

•  a message containing multiple IDs of A-IoT devices.

•  a message containing a group ID that maps to multiple A-IoT devices. FFS what group ID is (This group ID may be a separate ID or an ID generated by masking IDs.)

•  a message that does not contain an ID, i.e., inventory for all devices that can receive the AIoT message in the coverage area.

=>	Noted

-	Xiaomi thinks that for the third case it is up to SA2 and for fourth we don’t know.

-	Intel thinks that these should be applied to inventory.  Mediatek asks why it is specific to inventory. Intel thinks that the command would include the ID and that would be per device, group ID cannot work for command. Oppo thinks that it can also apply to command. ZTE thinks that the reader can still combine multiple command in one initial trigger message.

-	 LG thinks that we may need to consider segmentation.

-	NEC asks whether this is one message or multiple messages. Intel thinks we can discuss that later.

-	Huawei thinks that both case 2 and 3 address the same case so they can be merged. CATT agrees but we would need to further discuss what this ID is and it should be designed by SA2.

-	MEdiatek is that case 2 should be further studied as there may not be a use case for it.

-	Ericsson explains that you may have a group ID but you may have separate group IDs.

-	Nokia wonders what multiple ID would imply (i.e. flexible MAC headers). Interdigital thinks that there may be scenario that SA2 wants to support that would require multiple IDs.

-	Vodafone wonders if we should ask questions to SA2 on this and if the group ID has the same size as a device ID. Intel understands that the device IDs may be different.

-	Xiaomi thinks that one reason for multiple ID is to provide CFRA resources for multiple UEs.

-	Continental asks if a device can have more than one device ID or multiple group IDs. QC thinks that one device ID but it can have multiple group IDs. Oppo doesn’t think that multiple UEs share the same group ID.

-	CMCC thinks that the use case for multiple IDs is for paging both known and unknonw

-	NEC thinks that we should cover the case where we use a special value of ID to imply that we are paging all UEs. Qualcomm indicates that this is stage 3, when we decide it we can decide to put all bits to 0 or 1.

R2-2404369	Ambient IoT device paging	TCL	discussion	Rel-19

Proposal 4: delta paging should be introduced to avoid duplicated access procedure in the same transaction of paging.

Proposal 5: introduce a transaction ID in the paging message to identify the same transaction of paging, the devices who have already accessed to the network successfully in the previous round of random access should not access to the network again, if the transaction ID is the same compared to the previous paging message after which it has already accessed to the network.

=>	Noted

R2-2404881	Discussion on paging-like procedure for AIoT	ZTE Corporation, Sanechips	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 8a: A concept of inventory area can be introduced with intention that each inventory can only be performed for the devices within a certain inventory area.

=>	Noted

(control/resource information)

R2-2405030	Discussion on A-IoT paging	CMCC	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 8: The D2R transmission resources (e.g. dedicated or shared D2R resources) can be indicated in A-IoT paging.

=>	Noted

R2-2404346	Discussions on AIoT paging	Fujitsu	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 1: For inventory, in addition to the optional identifier, Paging for Ambient IoT includes:

Indication that the Ambient IoT device should determine initial D2R transmission occasion, or Paging type, e.g., for inventory.

Parameter(s) for the Ambient IoT device(s) to determine initial D2R transmission occasion.

=>	Noted

R2-2404579	Discussion on paging procedure for Ambient IoT	OPPO	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 2: RAN2 to agree on randomizing the access occasions for different AIOT devices via the paging message.

Proposal 3: RAN2 to agree on including a parameter to distribute access occasions for different AIOT devices in the paging message.

Proposal 5: RAN2 to wait for further RAN1 progress on TDM/FDM/CDM to decide whether and how related resource allocation parameters are included in the paging message.

=>	Noted

Discussions

-	Intel thinks that this is up to RAN1 discussion and they are discussing wether this will be in upper layer.

-	Apple thinks that shared resources that can be preconfigured but for CFRA we can include a dedicate message. CMCC indicates that even for shared resources the reader has to provide at least total shared slots

-	Vodafonen asks how the reader can provide dedicated resources for initial.

-	Oppo thinks that the resources can be indicate implicitly or explicitly.

-	Mediatek and Interdigital thinks that the UE needs to determine the resources in which to respond for access. AIoT paging message indicates information from which the device can determine resources from which to respond.

When the device monitors paging

R2-2404930	Discussion on paging functionality of A-IoT	Spreadtrum Communications	discussion	Rel-19

Proposal 3: Each A-IoT device should always monitor A-IoT paging message as long as the A-IoT device has sufficient energy.

=>	Noted

R2-2404579	Discussion on paging procedure for Ambient IoT	OPPO	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 6: Regarding monitoring of paging message, RAN2 to wait for further RAN1 progress on the discussion on energy harvesting.

=>	Noted

Discussion

-	Qualcomm, Xiaomi, Nokia has same view as Oppo.

-	Apple, Huawei, thinks that we should at least to make an assumption that the UE always receives as long as there is enough energy and wait for RAN1 for further progress.  Samsung and CATT thinks that from RAN2 perspective we can make an assumption.

-	Qualcomm asks what we are adding with respect to the previous agreements. Intel explains that we are saying that if the UE has enough power to complete the procedure it should monitor continoulsy.

-	Sony has some concerns with device energy

-	Mediatek indicates that this means that the reader doesn’t need to do anything special.

Agreements 1 RAN2 will study the following cases for AIoT paging message: - a message containing an ID of a single A-IoT device. - a message containing a group ID that maps to multiple A-IoT devices. - a message that does not contain an ID, i.e., addressed for all devices that can receive the AIoT message. - a message containing multiple IDs of A-IoT devices. Need to confirm the need for this use case based on SA2 discussion. What device ID and group ID and scenarios is depending on SA2 discussion. 2 AIoT paging message indicate information from which the device can determine resources to be used for response (D2R message). FFS how (e.g. implicit/explicit/configured/preconfigured) and what resources (dedicated and/or shared) are provided to the device taking into account RAN1 discussion. 3 From RAN2 perspective, we assume the device can receive as long as there is enough energy. We will wait for RAN1 further progress on device monitoring details.

Not treated

Permanent ID and Temporary ID

R2-2404578	Discussion on A-IOT paging procedure	Xiaomi	discussion

Proposal 2: A-IOT paging message from BS reader to device for a single device should include a device identifier e.g., temporary ID and/or permanent ID and resource/slot for the acknowledgement message from the triggered device.

R2-2404500	Ambient IoT identifiers and "paging" procedure	MediaTek Inc.	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 1: RAN2 will downselect between two solutions for the “AS identifier” used between the reader and the device: (1) random value selected by the device, (2) locally unique value assigned by the reader.

Proposal 2: RAN2 assume that the AS identifier may be used for multiple data transactions (read/write operations) between the reader and the device.  FFS how long the same identifier might be in use.

R2-2404689	Discussion on A-IoT paging functionality	LG Electronics Inc.	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 5. In order to support temporary identifier in A-IoT paging functionality, the A-IoT system should support the configuration of corresponding temporary identities for A-IoT devices. CN should be responsible for allocating temporary identities to A-IoT devices.

R2-2404194	Consideration on initial trigger message	Intel Corporation	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2404233	Discussion on Paging for Ambient IoT	CATT, CEPRI	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2404383	Discussion on AIoT paging	Futurewei	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2404396	Discussion on AIoT Paging	vivo	discussion	Rel-18	FS_Ambient_IoT_solutions

R2-2404398	Discussion on Paging Design for Ambient IoT	China Telecom	discussion

R2-2404535	Paging for Ambient IOT	InterDigital	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2404570	Ambient-IoT Paging	NEC	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2404660	Discussion on Ambient IoT Paging	Apple	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2404810	Discussion on paging procedure for Ambient IoT	Lenovo	discussion	Rel-19

R2-2404891	Ambient IoT Paging Method	Wiliot Ltd.	discussion

R2-2404903	Considerations on paging for Ambient IoT	Sony	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2405043	A-IoT paging functionality	Huawei, HiSilicon	discussion

R2-2405194	On Paging procedure for Ambient IoT	Nokia	discussion

R2-2405215	Views on Paging for Ambient IoT	Qualcomm Incorporated	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2405603	Discussion on initial trigger message (paging-like message) on Ambient IoT	NTT DOCOMO, INC.	discussion	Rel-19

### 8.2.5	A-IoT Random Access

Contributions should focus on A-IoT random access steps for both 2-step and 4-steps RA, content required for the different procedures, and any additional aspects related to CFRA and CBRA procedures.

Terminology

R2-2405015	Further discussion on random access for Ambient IoT	CMCC	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 1.1: The terminology of ‘access occasion’ refers to each occasion(slot) for device to initiate CBRA or CFRA access in slotted-ALOHA

Proposal 1.2: The terminology of ‘access round’ refers to a whole access procedure which consist of multiple access occasions.

=>	Not treated

4-step CBRA procedure

R2-2404926	Discussion on random access of Ambient IOT	Spreadtrum Communications	discussion	Rel-19

Proposal 2: For CBRA random access procedure for A-IoT based on slotted-ALOHA as follows can be supported in R19.

Msg1: A-IOT device sends a sequence like RN16 to reader.

Msg2: Reader acknowledgement the access of A-IOT device.

Msg3: A-IoT device sends device ID to reader.

Msg4: Reader acknowledgement the reception of A-IoT device ID.

=>	Noted

R2-2404811	Discussion on random access for Ambient IoT	Lenovo	discussion	Rel-19

Proposal 3: 4-step A-IoT random access includes contention resolution and device ID report. Detailed steps are as in the following:

A-IoT Msg0: trigger message from the reader

A-IoT Msg1: the device sends an ID to the reader.

A-IoT Msg2: the reader confirm the ID received in Msg1.

A-IoT Msg3: the device report device ID

A-IoT Msg4: the reader send ACK to the device.

=>	Noted

R2-2405107	Further on AIoT random access	Nokia	discussion	FS_Ambient_IoT_solutions

Proposal 2: For the 4-step contention-based access procedure, the very first D2R transmission consists of a random device identifier generated by the device.

Proposal 3: For the 4-step contention-based access procedure, device considers that the D2R Msg1 is successfully received when the random identifier received in the R2D Msg2 is similar to that it transmitted in Msg1.

Proposal 4: For the 4-step contention-based access, Msg3 includes the device ID and the D2R data transmission based on the received indication from the reader in Msg2.

Proposal 5: For the 4-step contention-based access procedure, the device considers the contention is resolved and the access procedure is successful when it receives a device ID in Msg4 similar to the device ID it sent in Msg3.

=>	Noted

Discussion

A-IoT Msg1: the device sends an ID to the reader (Options random number? device ID?).

-	CATT thinks it should be 16 bits random number. Samsung thinks size should be FFS but it should be random and generated by UE. Ericsson thinks it should be contention resolution ID. Qualcomm thinks that we need to wait for SA2. Intel doesn’t want to waste resources. CMCC thinks we should use random ID and device ID is not feasible for contention as the initial bits are common. Docomo agrees with CMCC and thinks that this procedure is terminated in reader and has security concerns with devices ID.

-	Vodafone, Huawei also thinks it needs to be random and device ID can be 100 bits and don’t want to send 100 bits, 16 is already too much.

-	Oppo thinks we should include device ID like RFID. For 2stepwe can use RF ID.

-	Lenovo and Qualcomm  also thinks a partial device ID can be used.   LG agrees with oppo.

-	Apple just thinks it should be a short ID. Interdigital thinks random ID is more determistic.

-	Huawei asks what is partial ID device, how can we ensure it is random enough.

A-IoT Msg2: the reader confirm the ID received in Msg1.

-	Xiaomi thinks that we may need to include some additional information like resources.

A-IoT Msg3: the device report device ID

-	Xiaomi thinks that report device ID is for contention resolution.  Intel thinks that device ID is for responding to inventory and whether it is for contention resolution it depends on msg4.

-	Ericsson thinks that we shouldn’t include contention reolution in msg3. CATT agrees but wonders if device ID is visible or it is an upper layer message. Qualcomm thinks that it depends on the use cases and it is not always device ID.

-	Huawei also agrees that contention resolution can be done by message 2.

A-IoT Msg4: the reader send ACK to the device.

[AT126][022][AIoT] CB on 4 step RA (Huawei)

Intended outcome: Discuss msg 4 details and contention resolution.

Deadline:  05-24-24

R2-2405950	Report of [AT126][022][AIoT] CB on 4 step RA	Huawei	discussion	Rel-19		FS_Ambient_IoT_solutions

=>	Noted

“4-step” CBRA procedure

Proposal 1:	The device considers the contention resolution as successful, if the Msg2 including the same random ID in Msg1 is received. RAN2 assumes the size of random ID in Msg1 should be sufficient for contention resolution purpose.

-	CMCC ask if this means that we would need a longer random ID number in first message for contention resolution purposes. Huawei thinks that we can make decision in next meeting and 16 bits may already be good enough. Intel, Qualcomm thinks we should discuss this later.

-	Qualcomm thinks that there may be collision even after message 2, but it should still ok

Proposal 2:	“Msg4” (i.e. the subsequent R2D transmission after D2R transmission) does not need to be always sent in random access. “Msg4” can be considered to handle the Msg3 transmission failure (due to various reasons). “Msg4” usage/presence can be further discussed.

	RAN2 will not use “Msg4” term for further discussion of the random access.

-	Vodafone asks what we will use instead. Intel thinks we can use message name.

“2-step” CBRA procedure

Proposal 3:	For the formulation of “2-step-like” CBRA, if RAN2 intends to further study:

	A-IoT Msg1: The device sends Device ID and/or any other upper layer data (depending on upper layer request). FFS on the need of random ID.

	A-IoT Msg2: the reader echos the ID received in Msg1.

-	Ericsson thinks that random ID and device ID may be the same. Samsung thinks that from reader point of view a single ID is sufficient so we need to understand whether this is the same as 4 step or something else. CATT is concerned that device ID will be combined with upper layer data and there is a concern the message size may be big. Qualcomm thinks that in this case you would do 4-step like RA.

-	Qualcomm explains that RAN1 is discussing a preamble like sequence so it shouldn’t preclude this case.

-	Telit is asking whether we are planning on supporting both, we have to keep in mind that this is a low complexity device. Intel explains that we can decide that later.

-	Vodafone agrees with Telit and also we don’t have capability so if the device has optionality the network would have to support both.

-	ZTE thinks that this is different from NR as paging may provide a grant, and if the paging message gives you a resource just for ID then you do 4-step and if it is bigger you would fill it and do 2-step, so it is easier to support both.

-	Mediatek continues to think the complexity is bad.  It may be more power efficient to do 2 step than 4 step. There is valid motivation to study both and we need to keep power/complexity tradeoff in place.

For the formulation of “2-step-like” CBRA, if RAN2 intends to further study:

	A-IoT Msg1: The device sends Device ID and/or any other upper layer data (depending on upper layer request). FFS what device ID is and whether an additional random ID (or other RAN1 information) is needed

-	Qualcomm and ZTE thinks that msg2 may not be needed

Agreements on “4 step” RA

1	A-IoT Msg1: the device sends an ID to the reader.  ID is a random ID generated by device (FFS how it is generated, e.g. randomly generated or generated based on Device ID).  FFS on ID size.  This doesn’t preclude any other RAN1 agreed information

2	A-IoT Msg2: the reader echos the ID received in Msg1. Further information may be included in mgs2 based on RAN1 agreements

3	A-IoT Msg3: device sends Device ID and/or any other upper layer data (depending on upper layer request)

- The device considers the contention resolution as successful, if the Msg2 including the same random ID in Msg1 is received. RAN2 assumes the size of random ID in Msg1 should be sufficient for contention resolution purpose.

- “Msg4” (i.e. the subsequent R2D transmission after D2R transmission) does not need to be always sent in random access. “Msg4” can be considered to handle the Msg3 transmission failure (due to various reasons). “Msg4” usage/presence can be further discussed.

RAN2 will not use “Msg4” term for further discussion of the random access.

Agreements on 2 step CB RA

- A-IoT Msg1: The device sends Device ID and/or any other upper layer data (depending on upper layer request). FFS what device ID is and whether an additional random ID is needed.  This doesn’t preclude any other RAN1 agreed information

- A-IoT Msg2: the reader may echo some information from Msg1.  FFS what some information is.  “Msg2” usage/presence can be further discussed

2-step CBRA procedure

R2-2404811	Discussion on random access for Ambient IoT	Lenovo	discussion	Rel-19

Proposal 4: For 2-step A-IoT random access procedure, device includes device id information and data in the very first message to the reader. Detailed steps are as following:

A-IoT Msg0: trigger message from the reader

A-IoT MsgA: the device sends an ID, as well as data (e.g. device ID) to the reader.

A-IoT MsgB: the reader confirm the ID received in MsgA.

=>	Noted

R2-2404536	Access Procedure for Ambient IOT	InterDigital	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 3:	Contention-based access procedure using 2-step RACH like procedure consists of the following messages: 1) A first (D2R) message containing at least a data PDU and a device ID (FFS how to determine the ID); 2) A second (R2D) message containing at least the contention resolution ID.

=>	Noted

R2-2405107	Further on AIoT random access	Nokia	discussion	FS_Ambient_IoT_solutions

Proposal 6: For the 2-step contention-based access, the very first D2R transmission in Msg1 consists of a random device identifier generated by the device and the device ID.

Proposal 7: For the 2-step contention-based access, the device considers the contention is resolved and the access procedure is successful when it receives a device ID in Msg2 similar to the device ID it sent in Msg1.

=>	Noted

CFRA procedures

R2-2404243	A-IoT random access procedure	Huawei, HiSilicon	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 11: The contention-free random access procedure is only for the case that single device is triggered for access.

=>	Noted

R2-2404811	Discussion on random access for Ambient IoT	Lenovo	discussion	Rel-19

Proposal 6: CFRA procedure can be triggered if specific device id(s) is included in initial trigger message.

=>	Noted

R2-2405604	Discussion on random access-like procedure for Ambient IoT	NTT DOCOMO INC.	discussion	Rel-19

Proposal 1.	Ambient IoT Reader can determine to conduct CFRA if the number of targeted devices indicated by the CN is one.

=>	Noted

Discussion

-	Oppo, Qualcomm, Apple,  thinks that we should not limit to a single device.  Samsung sees a use case for multiple devices.  CATT thinks we should support single device and further study multiple devices.

-	Intel thinks we should not use CFRA terminology, either contention-free or rach-less.

-	Vodafone doesn’t want to study multi-device as it would add complexity since reader needs to guarantee that there is no contention. Qualcomm explains that this is from reader perspective.

-	Mediatek that there may be complexity and we should prioritize single device to start with.

-	Chair indicates that in the next meeting we will start by discussing single device case.

Agreement

-	From reader perspective, contention-free access procedure we will study single and multi-device case (depending on RAN1 discussion).

Not treated

R2-2404243	A-IoT random access procedure	Huawei, HiSilicon	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 12: In contention-free random access procedure, A-IoT device directly sends the upper layer data (e.g. device ID) in the very first D2R message after being triggered (i.e. skip contention resolution steps).

R2-2405107	Further on AIoT random access	Nokia	discussion	FS_Ambient_IoT_solutions

Proposal 12: For the 1-step contention-free access, the very first D2R transmission Msg1 consists of the data transmission and is transmitted using the contention-free access resource indicated by the reader in Msg0.

Determing CBRA or CFRA

R2-2404661	Discussion on Ambient IoT Random Access	Apple	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 3	A triggered A-IoT device attempts contention-based access unless the A-IoT paging message indicates the contention-free access.

Proposal 4	An A-IoT-Paging message can indicate contention-free access by providing the contention-free access resources in PDRCH in the paging message.

Determing 2-step RACH or 4-step RACH

R2-2404536	Access Procedure for Ambient IOT	InterDigital	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 6: RAN2 studies reader-based and device-based criteria for selection between 2-step vs 4-step AIOT access procedure.

R2-2404811	Discussion on random access for Ambient IoT	Lenovo	discussion	Rel-19

Proposal 5: RAN2 further discuss whether and how to select different A-IoT RA types if both 4-step RA and 2-step RA are adopted.

Access failure handling

R2-2404499	Discussion on UL multiple access	Ericsson	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 13 For handling contention resolution failure and access failure, RAN2 to study the below options:

a. Option 1: a device which experiences contention-failure or access failure, re-accesses in the same round.

b. Option 2: a device which experiences contention-failure or access failure, re-accesses in the next round.

c. Option 3: the round length is adaptive. One round can be terminated earlier upon detection of too high collision. A new round with more time occasions is initiated.

R2-2405550	Discussion on random access for Ambient IoT	CEWiT	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 3: RAN 2 to study the A-IoT device behavior for RACH failure.

R2-2404536	Access Procedure for Ambient IOT	InterDigital	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 8: When access failure is detected, RAN2 considers performing both re-access in the additional occasions or waiting for next inventory round. FFS conditions for each case.

Power ramping or repetition for RACH

R2-2405358	Discussion on random access for ambient IoT 	Google Inc.	discussion	FS_Ambient_IoT_solutions

Proposal 3:  Support power ramping for Msg1 transmission.

Multiple access for RACH

R2-2404243	A-IoT random access procedure	Huawei, HiSilicon	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 8a:	Both TDMA and FDMA can apply to A-IoT Msg1.

R2-2404585	Discussion on random access for Ambient IoT	OPPO	discussion	Rel-19	FS_Ambient_IoT_solutions

Proposal 14	RAN2 wait for RAN1 progress on FDM/TDM/CDM based random access.

R2-2404154	Discussion on access procedure for ambient IOT	Xiaomi	discussion	Rel-19

R2-2404195	Consideration on contention based and contention free based Access	Intel Corporation	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2404224	Initial Access procedure for Ambient IoT device	NEC	discussion

R2-2404234	Discussion on the Random Access for Ambient IoT	CATT, CEPRI	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2404347	Discussions on AIoT Random Access	Fujitsu	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2404373	Random Access for Ambient IOT	TCL	discussion	Rel-19

R2-2404384	Discussion on AIoT random access	Futurewei	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2404397	Random Access Procedure for A-IoT Device	vivo	discussion	Rel-18	FS_Ambient_IoT_solutions

R2-2404499	Discussion on UL multiple access	Ericsson	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2404539	Random Access procedure for A-IoT	ZTE Corporation, Sanechips	discussion

R2-2404571	Discussion on the Random Access for Ambient IoT	LG Uplus	discussion	Rel-19

R2-2404864	Discussion on A-IoT random access procedure	ETRI	discussion	Rel-19

R2-2404904	Considerations on random access aspects for Ambient IoT	Sony	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2404956	Discussion on random access aspects for Ambient-IoT	Continental Automotive	discussion

R2-2405140	Random access aspects of Ambient IoT	Qualcomm Incorporated	discussion	FS_Ambient_IoT_solutions

R2-2405270	Considerations for Random Access	Semtech Neuchatel SA	discussion

R2-2405307	Discussion on random access for Ambient IoT	China Telecom	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2405427	Discussion on Ambient IoT random access conditions	ASUSTeK	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2405496	Discussion on A-IoT random access	Samsung	discussion	Rel-19	FS_Ambient_IoT_solutions

R2-2405518	Discussion on random access aspects for Ambient IoT	LG Electronics Inc.	discussion	FS_Ambient_IoT_solutions

R2-2405688	On Random Access for Ambient IoT	Philips International B.V.	discussion	Rel-19	FS_Ambient_IoT_solutions

## 8.3	AI/ML for Mobility

(FS_NR_AIML_Mob; leading WG: RAN2; REL-19; SID: RP-240082)

Time budget: 2 TUs

Tdoc Limitation: 4 tdocs

### 8.3.1	Organizational

LS, Rapporteur input, including workplan, etc.

R2-2404711	Discussion on work plan of AI mobility SI	OPPO,Nokia,Mediatek	discussion	Rel-19	FS_NR_AIML_Mob

R2-2404712	38.744 TR Skeleton of AI mobility NR	OPPO	draft TR	Rel-19	38.744	0.0.0	FS_NR_AIML_Mob	Withdrawn

R2-2405693	TR 38.744 Skeleton of AI mobility NR	OPPO	draft TR	Rel-19	38.744	0.0.1	FS_NR_AIML_Mob

=> Revised in R2-2406096

R2-2406096	TR 38.744 Skeleton of AI mobility NR	OPPO	draft TR	Rel-19	38.744	0.0.2	FS_NR_AIML_Mob

=> Endorsed

### 8.3.2	RRM measurement prediction

NOTE: papers from 8.3.2.1 and 8.3.2.2 have been grouped for discussion under 8.3.2.1

#### 8.3.2.1	Simulation assumptions and evaluation methodology for RRM measurement prediction

Including outcome of [POST125bis][021][AI/ML mobility ] Simulation assumptions and methodology  (Oppo)

No additional contributions are expected on aspects discussed in email discussion. Contributions, if any, should only focus on new aspects related to evaluation methodology including inter-frequency evaluation aspects.

Email discussion

R2-2404955	Summary of [POST125bis][021][AIML mobility ] Simulation assumptions and methodology	OPPO	discussion	Rel-19	FS_NR_AIML_Mob	Late

=>	Revised in R2-2405941

R2-2405941	Summary of [POST125bis][021][AIML mobility ] Simulation assumptions and methodology	OPPO	discussion	Rel-19	FS_NR_AIML_Mob	Late

=>	Noted

Agreements on simulation assumptions 1 For 2nd study goal i.e. to enhance handover performance, evaluation exercise will focus on FR2 to FR2 intra-frequency scenario. 2 For the evaluation exercise for 2nd study goal, RAN2 should assume that there is no reduction in measurement overhead 3 Prediction accuracy metric for RRM measurement cell level prediction is defined as “RSRP difference between predicted L3 cell level measurement result and actual L3 cell level measurement result of the same cell” for all RRM sub cases 4 for RRM sub case 1, it is up to company to report L1 RSRP difference 5 Definition of measurement reduction for intra-frequency scenario is defined as:(20/20) Measurement reduction rate in temporal domain (MRRT) assuming same length of measurement time instances: MRRT= skipped measurement time instances / total measurement time instances Measurement reduction rate in spatial domain (MRRS): MRRS = skipped beams to be measured/ total beams to be measured 6 For RRM sub case 1 and 3, it is up to company’s implementation whether L1 filtering is applied for input L1 beam level measurement. Companies are expected to report what L1 filtering they use in their simulation. 7 To agree on methodology of Intra_F_C_T_Case A as following: Intra-frequency intra-cell temporal domain case A prediction is done by predicting measurement result(s) in prediction window based on measurement results in observation window of the same cell for both FR1_to_FR1 and FR2_to_FR2. FFS aligning the prediction window 8 To agree on methodology of Intra_F_C_T_Case B: Intra-frequency intra-cell temporal domain prediction is done by predicting sub set measurement (case B) instances in temporal domain of the same cell for both FR1_to_FR1 and FR2_to_FR2. Several measurement reduction rates should be aligned among companies. The detail values for measurement reduction rate are FFS. 9 Intra-frequency intra-cell temporal domain prediction can be applied for all RRM sub cases. And it is up to company to report applied RRM sub case together with simulation result. 10 Methodology of Intra_F_C_S: Intra-frequency intra-cell spatial domain prediction is done by measuring sub set of configured SSB as input to the model to predict L3 cell level measurements for every instance of the same cell. It is only evaluted for FR2 intra-frequency scenario and RRM sub case 1 and 3. Several measurement reduction rates should be aligned among company without defining detail pattern. The detail rate values are FFS. 11 For both Intra-frequency and inter-frequency inter-cell prediction, the measurement on cell for measurement should not be reduced in both temporal and spatial domain 12 For Inter_F_C (inter-frequency inter-cell), RAN2 start evaluation from co-located scenario 13 for Inter_F_C(inter-frequency inter-cell), RAN2 should focus on the case where cell for measurement and cell for prediction are in the same sector. 14 FR1 to FR1 inter-frequency inter-cell prediction is applicable for all RRM sub cases. And it is up to company to report applied RRM sub cases together with their simulation result. 16 Intra_F_Inter_C (intra-frequency inter-cell) prediction will not be evaluated at least in early stage. Intra_F_Inter_C(Intra-frequency inter-cell) refers to both co-located and non-colocated neighbouring cell prediction. 17 No traffic model is simulated 18 During simulation UE is dropped 100% outdoor 19 It is up to company’s implementation to select how to distribute the UE 20 Fast fading is necessary for RRM sub case 1 and 3. FFS case 2 21 To agree not consider Oxygen absorption (7.6.1), Time-varying Doppler shift (7.6.6), Explicit ground reflection model (7.6.8) and blockage (7.6.4) for channel modelling (38.901) 22 LOSsoft is optionally modelled in the channel modelling 23 To agree on following parameters for FR2: 1, 30GHz as central frequency 2, 200m as ISD 3, UMi with distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901 as baseline channel modelling 24 For FR1, following parameters are agreed: 1, Table 6.2.1-1 template is taken as starting point ( to be updated by individual proposal later on) 3, to set up 2-tier model (7 sites, 3 sectors/cells per site) (20/20) 4, 500m as ISD 5, channel modelling is UMa with distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901 6, 20MHz as bandwidth 7, The recommendated value in yellow in table 2.3.3-1 ( by removing wording “At least for BM-Case1,”) 25 Section 7.6.5 in 38.901 is taken as baseline for inter-frequency correlation model. Whether inter-frequency correlation model is used is optional and companies can report what they use. FFS on the understanding shadowing correlation in inter-freq. For now companies should report what assumption they have made 26 Following RRC parameters need be aligned as simulation parameters: (17/20) - RRC parameters for measurement consolidation - RRC parameters for L3 filtering (filter coefficient, measurement period) 27 We apply same L1 sampling period for both intra and inter (i.e. we don’t simulate existence of measurement gap as starting point) 28 Simulation assumptions discussed in section 2.3.1~2.3.3 is taken as baseline also for use cases other than RRM measurement prediction. Any update is subject to further discussion on other use cases. 29 The sample period(s) are aligned among companies for intra-frequency intra-cell temporal domain prediction. We can start with 20ms for FR2 and 40ms for FR1. Measurement period: FFS – suggestion from rapporteur is to start with 480ms for FR2 and 200ms for FR1 30 Simulation parameters in table 2.3.4-1 (by removing Table A.2.1-7 and Table 2.1-10) are taken as starting point for both UE sided model and network sided model for FR2. The number of beams could be left for company to report. FFS Use 100Mhz for channel BW? 31 For FR1, following parameters are agreed: 2, {4GHz,30KHz} as frequency for intra-frequency scenario and {2GHz, 15KHz(FDD)/30GHz(TDD)} as another frequency for inter-frequency scenario

[AT126][030][AIMob] Simulation assumptions (Oppo)

Intended outcome: Proposals on simulation assumptions remaining from email discussion.

Deadline:  05-24-24

R2-2405960	Draft summary of [AT126][030][AIMob] Simulation assumptions (Oppo)	OPPO (rapporteur)	discussion	Rel-19	FS_NR_AIML_Mob

=>	Noted

Agreements 1 average RSRP difference is taken as prediction accuracy metric for RRM measurement prediction. Note the RSRP difference values should be an absolute value before they are averaged 2: Measurement reduction rates e.g. 1/2, for both intra-frequency intra-cell temporal domain prediction case B and spatial domain prediction. Revision in RAN2#127 is open. 3: One prediction window for FR1 and FR2 respectively as starting point. The detail value to be decided in the post email discussion. 4: UE trajectory option is up to company’s implementation and report 5: UE trajectory boundary processing is up to company’s implementation and report 6: For study goal 2, the candidate speeds are 60,90,120 km/h and company can report UE speed along with simulation result 7: For study goal 1, the candidate speeds are 30,60,90km/h and company can report UE speed along with simulation result 8: To decide on the values in table 2.4-2/3/4 in post email discussion. L3 filtering parameter for both FR1 and FR2; Recommended value. FilterCoefficient; 4. Table 2.4-2 Measurement period; Recommended value. FR1 to FR1 intra-frequency w.o. gap; 200ms. FR1 to FR1 inter-frequency with gap; 120ms. FR2 to FR2 intra-frequency w.o. gap; 480ms. Table 2.4-3 Consolidation parameter; Recommended value. nrofSS-BlocksToAverage for FR1; 1. nrofSS-BlocksToAverage for FR2; 3. absThreshSS-BlocksConsolidation; FFS. Table 2.4-4 9: To agree on 80MHz as bandwidth for FR2 10: To agree on 10m for BS antenna height for FR2 11: Fast fading is necessary for RRM sub case 2 too. 12: The shadowing fading of two different frequency layers can be reported by company 13: To define cluster approach at least based on the number of input cells, number of output cells and their relationship in temporal domain, spatial domain and frequency domain

Scenario priority

R2-2405933	Way forward on prioritized evaluation scenarios for RRM measurement prediction	Oppo, Nokia	discussion	Rel-19	FS_NR_AIML_Mob

Proposal 1: To approve the scenarios in table 2-2 for simulation evaluation

Proposal 2: Different simulation assumption(s) from combination cases in table 2-2 should be provided together with simulation result, if provided.

-	ZTE, Docomo, Huawei and Ericsson think that Case 3 is highest priority. Samsung thinks that case 3 thinks it is the most complicated.

-	ZTE thinks that we need to discuss speed and prediction window and we should try to prioritize some speed for each sub-case. Mediatek suggests to start with high speed scenario.

=>	Noted

Agreements:

The following is agreed

| Case number | Prioritization | Evaluation scenario combination | target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal | TBD |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal | TBD |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1st goal | Intra-cell |

##### KPIs:

R2-2404630	On measurement prediction	Apple	discussion	Rel-19	FS_NR_AIML_Mob

Proposal 1: Accuracy (i.e. RSRP difference) is always reported together with a time window, i.e. the KPI becomes a “paired KPI” <RSRP difference, prediction window>.

Proposal 2: measurement reduction rate is defined as “skipped measurements/total configured measurements” and it is reported in pairs together with accuracy, i.e. as <measurement reduction rate, accuracy>.

Proposal 3: no additional measurement related KPIs are needed; if time permits, additional KPIs can be considered in the future.

=>	Noted

R2-2404307	Discussion on system-level performance KPI for RRM measurement prediction	vivo	discussion	Rel-19	FS_NR_AIML_Mob

Proposal 1: System-level performance evaluation is needed to reflect the impact on mobility performance after RRM measurement prediction is introduced.

Proposal 2: The system-level performance KPIs for RRM measurement prediction include Ping-pong HO rate, short ToS rate, HOF rate, RLF frequency, handover interruption.

=>	Noted

Discussion

-	Apple is not sure how we can simulate time of stay etc.  Let’s start with prediction accuracy and then we can see what we simulate for system level performance.  Xiaomi, Ericsson agrees with Apple.

-	Mediatek thinks that at least model complexity needs to be added.

=>	We will start the evaluation with measurements prediction accuracy and model complexity.  We can discuss system performance after we see which scenarios have good measurements prediction accuracy.

[POST126][031][AIMob] Simulations (Oppo)

Intended outcome: Agree to evaluation documentation and small simulation related FFS (needed to start simulation evaluation for August meeting)

Endorse Skeleton TR

Deadline:  2 weeks

R2-2406095	Summary of [POST126][031][AIMob] Simulations (OPPO)	OPPO (Rapporteur)	discussion	Rel-19	FS_NR_AIML_Mob

=> Noted

##### L3 filtering, beam consolidation, etc.

R2-2405065	Discussion on RRM measurement prediction	ZTE Corporation	discussion	Rel-19	FS_NR_AIML_Mob

Proposal 1: For the cell level measurement prediction, suggest to consider the following three phases:

Phase1: To not consider L3 filtering, or consider filterCoefficient ki=0 in RRM measurement prediction simulation;

Phase 2: To consider a fixed value (e.g. filterCoefficient ki=4) for L3 filtering in simulation.

Phase 3: To consider variable values of filterCoefficient for L3 filtering in simulation, including both data collection and data inference.

For Phase2 and Phase3, RAN2 needs to discuss the assumed L3 filtering periodicity.

-	ZTE explains that some companies think we should start the ki=4. Apple thinks that a fixed coefficient works.   Xiaomi thinks that the last phase is not really needed.

=>	Noted

Agreements

- For the cell level measurement prediction, start with consider a fixed value for L3 filtering in simulation. FFS which fixed value

- For cell level prediction, RSRP difference to the actual measurement is calculated based on L3 filtered measurement result

R2-2404601	Discussion on simulation and evaluation methodology	Xiaomi	discussion

Proposal 2: For cell level prediction, RSRP difference to the actual measurement is calculated based on L3 filtered measurement result.

Proposal 3: RAN2 to discuss how to generate cell level results based on measured and predicted beams in case 1

=>	Noted

Not treated

R2-2404630	On measurement prediction	Apple	discussion	Rel-19	FS_NR_AIML_Mob

Observation 2: there is already 36 scenarios for measurement predictions, counting based on the RAN2 agreements so far.

Proposal 5: Significant down-selection/prioritization of all the measurement reduction scenarios is required.

R2-2404715	Discussion on open issue of RRM measurement use case	OPPO	discussion	Rel-19	FS_NR_AIML_Mob

Proposal 2: To deprioritize evaluation on FR1 to FR1 intra-frequency temporal domain case A

Proposal 3: To prioritize evaluation on FR2 to FR2 intra-frequency temporal domain case A

Proposal 4: To prioritize evaluation on FR2 to FR2 intra-frequency spatial domain

R2-2405480	RRM measurement prediction for UE sided prediction and NW-sided prediction 	LG Electronics Inc.	discussion	FS_NR_AIML_Mob

Proposal 2: Temporal domain prediction is prioritized over spatial/frequency domain prediction if TU is insufficient to treat all of them with equal priority.

Cluster based approach:

R2-2404700	Discussion on RRM measurement prediction	Samsung	discussion	Rel-19	FS_NR_AIML_Mob	Late

Proposal. 2: RAN2 to consider the following two approaches for RRM measurement prediction.

For Approach 1 (1-to-1 approach):

The model input is the measurement results for a single cell B

The model output is the prediction results for a single cell A

For temporal domain prediction: Cell A = Cell B

For spatial/frequency domain prediction: Cell A ≠ Cell B

For Approach 2 (N-to-K approach, aka Cluster approach)

The model input is the measurement results for SET B of N cells

The model output is the prediction results for SET A of K cells

For general temporal domain prediction: SET A ⊆ SET B (1<= K<=N) or 
For pure temporal domain prediction: SET A = SET B (1<=K=N)

For spatial/frequency domain prediction: SET A ⊈ SET B

R2-2404713	Discussion on simulation assumptions of RRM measurement	OPPO	discussion	Rel-19	FS_NR_AIML_Mob

Proposal 3: If the simulation is based on cluster approach, the evaluated cases should be limited in order to keep simulation workload at a reasonable level

Proposal 4: The cluster methodology is defined as follows:

Cluster approach is applicable only for co-located cells.

The number of input cells and output cells should be no more than 3 and the number of output cells should be <= the number of input cells.

The input measurement result should be L3 cell level measurement results

For intra-frequency scenario, the output cells should be full or subset of the input cells

For inter-frequency scenario, the input cells are from measured frequency layer and the output cells are from predicted frequency layer

Proposal 5: To start the evaluation with cluster approach in late stage

R2-2404999	Discussion on other aspects related to RRM measurement prediction	NTT DOCOMO, INC.	discussion	Rel-19

Proposal 1

-	For all 3 cases, study the cluster-based prediction with the following detailed schemes,

Input the measurement results from multiple cells to the AI/ML models, including the serving and neighboring cells.

Output the predicted measurements corresponding to the same cells of inputs.

##### Beam prediction:

R2-2405653	Remaining aspects of simulation assumptions for RRM measurement prediction	Huawei, HiSilicon	discussion	Rel-19	FS_NR_AIML_Mob

Proposal 1: L3 beam level measurement prediction should be considered, and simulation assumptions for cell level measurement prediction can be reused.

Proposal 2: For L3 beam level measurement prediction model, at least consider the following cases:

Case1: To predict L1 beam level measurement results, then generate L3 beam level measurement results based on the predicted L1 beam level measurement results

Case2: To directly predict L3 beam level measurement results based on L3 beam level measurement results

Case3: To directly predict L3 beam level measurement results based on L1 beam level measurement results

R2-2405028	Discussion on AIML based RRM measurement prediction	CMCC	discussion	Rel-19	FS_NR_AIML_Mob

Proposal 6: The temporal domain prediction and spatial domain can be applied for L3 beam level results prediction.

R2-2404308	Other aspects related to RRM measurement prediction	vivo	discussion	Rel-19	FS_NR_AIML_Mob

Proposal 2: For RRM measurement prediction, besides Case1-3, a new sub use case (Case 4) is considered: To directly predict cell-level results and beam-level results of best K beams based on beam-level results and optional cell-level results.

Proposal 10: For Case 4, consider the following two sub-cases:

Case 4-1: input are L1-filtered beam measurements, output include cell level qualities without L3 filtering and L1-filtered beam qualities;

Case 4-2: input include L3-filtered beam measurements and L3-filtered cell measurements, output include L3-filtered cell level qualities and L3-filtered beam qualities.

##### Model input/output:

R2-2404630	On measurement prediction	Apple	discussion	Rel-19	FS_NR_AIML_Mob

Observation 1: even if we agree on common model inputs (per use case), companies should be allowed to use other inputs as well.

Proposal 4: there is no need to specify model inputs at all, companies should be free to use (and report) whatever inputs they consider useful.

R2-2404700	Discussion on RRM measurement prediction	Samsung	discussion	Rel-19	FS_NR_AIML_Mob	Late

Proposal 7. For input/output of temporal domain prediction model, the measurement/prediction result at each time instance can be assumed as below.

| Prediction cases | Measurement result for input | Prediction result for output |
| --- | --- | --- |
| Cell-level (Case 1) | L1-RSRP per beam | L1-RSRP per beam |
| Cell-level (Case 2) | L1/L3-RSRP per cell | L3-RSRP per cell |
| Cell-level (Case 3) | L1-RSRP per beam | L3-RSRP per cell |
| Beam-level | L1-RSRP per beam | L1-RSRP per beam |

R2-2404308	Other aspects related to RRM measurement prediction	vivo	discussion	Rel-19	FS_NR_AIML_Mob

Proposal 6: For RRM measurement prediction, input and output should be with the same level of filtering, i.e., L1-filtering or L3-filtering:

- If the input are not L3-filtered measurements (i.e., L1-filtered beam measurements or Cell measurements without L3 filtering), the output should be not L3-filtered;

- If the input are L3-filtered measurements (i.e., L3-filtered beam measurements or L3-filtered cell measurements), the output should be L3-filtered.

Proposal 7: For Case 1, the input and output are L1-filtered beam-level qualities.

Proposal 8: For Case 2, consider the following two sub-cases:

- Case 2-1: input and output are cell-level qualities without L3 filtering;

- Case 2-2: input and output are L3-filtered cell-level qualities.

Proposal 9: For Case 3, the input are L1-filtered beam measurements, and the output are cell-level qualities without L3 filtering.

R2-2404475	Discussion on Other Aspects related to RRM Prediction	MediaTek Inc.	discussion

Proposal 3: RAN2 discuss the derivation of RSRP difference = |actual RSRP – predicted RSRP|, where RSRP values refer to the actual and prediction value of the same predicted cell. The predicted RSPR value can be direct L3 RSRP prediction or derived from L1 RSRP prediction indirectly.

Proposal 4: RAN2 discuss the calculation of statistic L3 cell-level RSRP difference (includes average RSRP, CDF of RSRP, RMSE RSRP,…):

Option 1: Consider the average over both observation and prediction instance

Option 2: Consider the average over prediction instance only

Proposal 5: RAN2 align the L3 filter parameters and discuss the calculation of L3 filtering result with following options

Option 1: Only include the observation results into L3 filtering average.

Option 2: include both the observation results and prediction results into L3 filtering average

R2-2404270	Additional consideration for RRM prediction simulation assumption	Intel Corporation	discussion	Rel-19	FS_NR_AIML_Mob

R2-2404307	Discussion on system-level performance KPI for RRM measurement prediction	vivo	discussion	Rel-19	FS_NR_AIML_Mob

R2-2404472	Evaluation and Preliminary Results on AIML for RRM Measurement Prediction	MediaTek Inc.	discussion

R2-2404485	Simulation based evaluation of AIML aided mobility	Ericsson	discussion	Rel-19	FS_NR_AIML_Mob

R2-2404601	Discussion on simulation and evaluation methodology	Xiaomi	discussion

R2-2404713	Discussion on simulation assumptions of RRM measurement	OPPO	discussion	Rel-19	FS_NR_AIML_Mob

R2-2405005	Discussion on the simulation assumptions and evaluation methodology for RRM measurement prediction	NTT DOCOMO, INC.	discussion	Rel-19

R2 R2-2404338	RRM Measurement Prediction for Enhanced Handover	Lekha Wireless Solutions 	discussion	Rel-19	Late

R2-2405551	Discussion on framework for RRM measurement	CEWiT	discussion	Rel-19	FS_NR_AIML_Mob

-2405064	Discussion on simulation assumption for RRM measurement prediction	ZTE Corporation	discussion	Rel-19	FS_NR_AIML_Mob

R2-2405205	Considerations for mobile UE trajectory generation and channel modelling for simulation evaluation	Qualcomm Incorporated	discussion	Rel-19

R2-2405206	Simulation assumptions and evaluation methodology for RRM measurement prediction	Interdigital Inc.	discussion	Rel-19	FS_NR_AIML_Mob

R2-2405444	Further discussion on simulation assumptions and methodology for RRM measurement prediction	Nokia, Nokia Shanghai Bell	discussion	Rel-19	FS_NR_AIML_Mob

R2-2405592	Considerations on Simulation for AI/ML Mobility	Samsung	discussion	Rel-19	FS_NR_AIML_Mob

R2-2405653	Remaining aspects of simulation assumptions for RRM measurement prediction	Huawei, HiSilicon	discussion	Rel-19	FS_NR_AIML_Mob

#### 8.3.2.2	Other aspects related to RRM measurement prediction

Including definitions, metrics, additional discussions on sub use cases etc.

NOTE: documents from this sections have been grouped with other documents above for discussion

R2-2404338	RRM Measurement Prediction for Enhanced Handover	Lekha Wireless Solutions 	discussion	Rel-19	Late

R2-2405551	Discussion on framework for RRM measurement	CEWiT	discussion	Rel-19	FS_NR_AIML_Mob

R2-2404260	Other aspects related to RRM measurement prediction	Fraunhofer HHI, Fraunhofer IIS	discussion

R2-2404283	Discussion on AI/ML based RRM measurement prediction	China Telecom Corporation Ltd.	discussion	Rel-19

R2-2404308	Other aspects related to RRM measurement prediction	vivo	discussion	Rel-19	FS_NR_AIML_Mob

R2-2404313	AIML Mob RRM measurement prediction 	NEC	discussion	Rel-19	FS_NR_AIML_Mob

R2-2404368	AI/ML RRM measurement prediction	TCL	discussion	Rel-19

R2-2404474	Discussion on Evaluation Methodology of RRM Prediction	MediaTek Inc.	discussion

R2-2404475	Discussion on Other Aspects related to RRM Prediction	MediaTek Inc.	discussion

R2-2404558	Discussion on AI aided RRM measurement prediction	HONOR	discussion	Rel-19	FS_NR_AIML_Mob

R2-2404609	Consideration on AI/ML based RRM measurement prediction	Xiaomi	discussion	Rel-19	FS_NR_AIML_Mob

R2-2404629	On evaluation methodology	Apple	discussion	Rel-19	FS_NR_AIML_Mob

R2-2404695	Consideration on inter-frequency RRM Measurement Prediction	CATT, Turkcell	discussion	Rel-19	FS_NR_AIML_Mob

R2-2404700	Discussion on RRM measurement prediction	Samsung	discussion	Rel-19	FS_NR_AIML_Mob	Late

R2-2404715	Discussion on open issue of RRM measurement use case	OPPO	discussion	Rel-19	FS_NR_AIML_Mob

R2-2404824	Further Discussion on Other Aspects related to RRM Measurement Prediction	Continental Automotive	discussion	Rel-19

R2-2404936	Discussion on other aspects related to RRM measurement prediction	Spreadtrum Communications	discussion	Rel-19

R2-2404999	Discussion on other aspects related to RRM measurement prediction	NTT DOCOMO, INC.	discussion	Rel-19

R2-2405028	Discussion on AIML based RRM measurement prediction	CMCC	discussion	Rel-19	FS_NR_AIML_Mob

R2-2405065	Discussion on RRM measurement prediction	ZTE Corporation	discussion	Rel-19	FS_NR_AIML_Mob

R2-2405095	On AI based RRM measurement predictions	Ericsson	discussion	FS_NR_AIML_Mob

R2-2405161	RRM measurement prediction scenarios and sub-use cases	Nokia	discussion	Rel-19	FS_NR_AIML_Mob	Withdrawn

R2-2405162	Considerations on the predicted measurements	Nokia	discussion	Rel-19	FS_NR_AIML_Mob	Withdrawn

R2-2405165	RRM measurement prediction	Lenovo	discussion	Rel-19

R2-2405207	Beam prediction related aspects	Interdigital Inc.	discussion	Rel-19	FS_NR_AIML_Mob

R2-2405480	RRM measurement prediction for UE sided prediction and NW-sided prediction 	LG Electronics Inc.	discussion	FS_NR_AIML_Mob

R2-2405650	Discussion on RRM measurement prediction	Huawei, HiSilicon	discussion	Rel-19	FS_NR_AIML_Mob

R2-2405671	RRM measurement prediction scenarios and sub-use cases	Nokia Corporation	discussion	Rel-19

R2-2405674	Considerations on the predicted measurements	Nokia Corporation	discussion	Rel-19

### 8.3.3	Measurement event predictions

Contributions should focus on measurement event prediction use cases/scenarios to focus during the study and relevant performance metrics/KPIs to evaluate

This AI will not be treated this meeting

R2-2404222	Target scenarios for measurement event prediction	NEC	discussion

R2-2404372	Discussion on measurement event prediction	TCL	discussion	Rel-19

R2-2404473	Evaluation on AI/ML for Measurement Event Prediction	MediaTek Inc.	discussion

R2-2404559	Discussion on AI aided measurement events prediction	HONOR	discussion	Rel-19	FS_NR_AIML_Mob

R2-2404823	Further Discussion on Measurement Event Prediction 	Continental Automotive	discussion	Rel-19

R2-2404905	Data collection for event prediction	Sony	discussion	Rel-19	FS_NR_AIML_Mob

R2-2405544	Discussion on measurement event predictions	ETRI	discussion

R2-2405612	Discussion on measurement event predictions	III	discussion	FS_NR_AIML_Mob

### 8.3.4	RLF/HO failure prediction

#### 8.3.4.1	Simulation assumptions and evaluation methodology for RLF failure prediction

Contributions should focus on discussing RLF specific methodology and simulation assumptions.

##### Scenario Prioritization:

R2-2404716	Discussion on simulation assumptions of RLF	OPPO	discussion	Rel-19	FS_NR_AIML_Mob

Proposal 4: RLF prediction is only evaluated on FR2 carrier in temporal domain

R2-2404602	Discussion on RLF prediction simulation and evaluation methodology	Xiaomi	discussion

Proposal 6: RAN2 can consider both FR1 and FR2.

Proposal 7: In FR1, sudden radio channel condition change can be introduced by topology, e.g. Manhattan topology.

R2-2405651	Simulation assumptions and evaluation methodology for RLF/HOF predictions	Huawei, HiSilicon	discussion	Rel-19	FS_NR_AIML_Mob

Proposal 3: HOF/RLF prediction should focus on FR2_to_FR2 and high-speed scenarios (90 km/h, 120 km/h).

##### Channel modelling:

R2-2404268	simulation assumption for RLF prediction	Intel Corporation	discussion	Rel-19	FS_NR_AIML_Mob

Proposal 3: RLF/HO failure prediction shall follow the same simulation assumptions in RRM measurement prediction in FR2, including e.g. UE trajectory, UE distribution, UE speed, channel modelling, etc.

R2-2405591	Simulation for HOF and RLF Prediction	Samsung	discussion	Rel-19	FS_NR_AIML_Mob

Proposal 3. In HOF/RLF prediction, RAN2 shall consider LOSsoft state as a spatial consistency of LOS-NLOS transition, according to TR 38.901.

##### Other simulation parameters:

R2-2404602	Discussion on RLF prediction simulation and evaluation methodology	Xiaomi	discussion

Proposal 2: For RLF monitoring, L1 samples filtered linearly over a sliding window of 200ms (i.e. 20 samples) for Qout and 100 ms (i.e. 10 samples) for Qin, respectively.

Proposal 3: reuse the parameters in table 5.2.1.3.1 in 36.839 to determine RLF. T310 can be set to shorter values in FR2.

R2-2404309	Simulation assumptions and evaluation methodology for RLF failure prediction	vivo	discussion	Rel-19	FS_NR_AIML_Mob

Observation 1: If reusing the simulation assumptions for RRM measurement prediction without changes, no RLF occurs in the initial simulation result.

Proposal 2: To acquire the dataset for RLF prediction, RAN2 should further discuss the simulation assumptions. The following two options can be considered:

- Option 1: Adjust the simulation parameters for RRM prediction that have impacts on the coverage or handover.

- Option 2: Do not consider handover in the simulation, and the trajectory of UE will be terminated until RLF occurs.

Proposal 5: Take the mobility-specific simulation parameters of HetNet in TR 36.839 as the baseline for system-level performance evaluation of RLF prediction:

| Parameter | Value |
| --- | --- |
| L1 measurement period | 40ms |
| Filtering Factor K | 4 |
| A3 Offset | 3 dB |
| TimeToTrigger | 480 ms |
| Ping-Pong-Time/short time of stay | 1 s |
| Handover preparation (decision) delay | 50ms |
| Handover execution time | 40ms |

R2-2404716	Discussion on simulation assumptions of RLF	OPPO	discussion	Rel-19	FS_NR_AIML_Mob

Proposal 5: the assumption of T310 and N311 should be set as such that RLF could be a bit easy to happen and a bit hard to cancel to facilitate training and inference

Proposal 6: RAN2 is kindly asked to discuss the detailed values of T310 and N331 so that they can be aligned among companies. Our recommendation: T310 = 200ms, N311=4.

R2-2404223	Simulation assumption for RLF prediction	NEC	discussion

R2-2404268	simulation assumption for RLF prediction	Intel Corporation	discussion	Rel-19	FS_NR_AIML_Mob

R2-2404309	Simulation assumptions and evaluation methodology for RLF failure prediction	vivo	discussion	Rel-19	FS_NR_AIML_Mob

R2-2404602	Discussion on RLF prediction simulation and evaluation methodology	Xiaomi	discussion

R2-2404696	Simulation assumptions and evaluation methodology for RLF prediction	CATT, Turkcell	discussion	Rel-19	FS_NR_AIML_Mob

R2-2404716	Discussion on simulation assumptions of RLF	OPPO	discussion	Rel-19	FS_NR_AIML_Mob

R2-2404825	Discussions on simulation assumptions and evaluation methodology for RLF/HOF prediction	TCL	discussion	Rel-19

R2-2405031	Discussion on Simulation Assumption and Methodology for RLF prediction	CMCC	discussion	Rel-19	FS_NR_AIML_Mob

R2-2405066	Discussion on simulation assumption for RLF prediction	ZTE Corporation	discussion	Rel-19	FS_NR_AIML_Mob

R2-2405096	Discussion on methodology for evaluation of RLF/HOF predictions	Ericsson	discussion	FS_NR_AIML_Mob

R2-2405208	Simulation assumptions and evaluation methodology for RLF prediction	Interdigital Inc.	discussion	Rel-19	FS_NR_AIML_Mob

R2-2405431	Discussion on simulation assumption for RLF/HO failure prediction	KDDI Corporation	discussion	Rel-19

R2-2405591	Simulation for HOF and RLF Prediction	Samsung	discussion	Rel-19	FS_NR_AIML_Mob

R2-2405651	Simulation assumptions and evaluation methodology for RLF/HOF predictions	Huawei, HiSilicon	discussion	Rel-19	FS_NR_AIML_Mob

R2-2405696	Discussion on RLF/HO failure prediction 	Indian Institute of Tech (M), IIT Kanpur	discussion	Rel-19

#### 8.3.4.2	Other aspects related to RLF/HO failure prediction

Including definition of RLF and HO prediction sub use cases, scenarios, metrics/KPIs, prioritizations etc.

##### Use case/modelling:

R2-2405004	Discussions on other aspects related to RLF/HOF prediction	NTT DOCOMO, INC.	discussion	Rel-19

Observation 1

There are two approaches for AI/ML-based HoF/RLF prediction,

Case 1: HoF/RLF prediction based on the temporal domain RRM measurement predictions.

The temporal domain RRM measurement predictions of multiple cells (including serving and neighbouring) are necessary for the HoF/RLF prediction.

Case 2: Directly HoF/RLF prediction by AI/ML models.

The detailed schemes are unclear so far and need further study in RAN2.

=>	Noted

R2-2405652	Discussion on HOF and RLF prediction	Huawei, HiSilicon	discussion	Rel-19	FS_NR_AIML_Mob

Proposal 3: For HOF/RLF prediction, RAN2 should study the feasibility and benefits for joint mode and independent mode, where the modes are defined as follows:

-	Indirect/Joint mode: the measurement prediction is used to predict HOF/RLF, i.e. the measurement prediction can be used as inputs for the HOF/RLF prediction

-	Direct/Independent mode: the historical HOF/RLF and other parameters other than measurement results are collected and they can be used for HOF/RLF prediction

=>	Noted

R2-2405203	RLF/HO failure prediction	Qualcomm Incorporated	discussion	Rel-19

Proposal 1. For simulation evaluation purposes, we propose the following framework for RLF prediction on the source cell.

RLF prediction is performed on the UE side using an AI/ML model and based on UE measurements of source cell beams (RLM RSs).

The AI/ML model for RLF prediction may produce as output one or more of the following:

- Prediction regarding whether RLF will occur in a certain window of time into the future.

- Predicted time of occurrence of RLF.

- Predicted times of occurrence of RLF and associated probabilities.

- E.g., Probability of occurrence of RLF is 10% 100ms into the future, 20% 150ms into the future, etc.

=>	Noted

Discussion on the approaches

Case 1: Indirect: HoF/RLF prediction based on the temporal domain RRM measurement predictions.

The temporal domain RRM measurement predictions of multiple cells (including serving and neighbouring) are necessary for the HoF/RLF prediction.

Case 2: Direct: Directly HoF/RLF prediction by AI/ML models.

- Vivo thinks that we should start with RRM measurement prediction.  And think that the simplest approach Prediction regarding whether RLF will occur in a certain window of time into the future.

- Lenovo thinks that we should both direct and indirect prediction.

- Huawei indicates that we can have other parameters for direct case.

- Apple thinks that we can have both and we report what the companies are using. The essence of this study would be the direct case so we shouldn’t deprioritize.

- Samsung thinks we should do both can consider the first case as baseline, and it is unclear how to evaluate the benefit. Apple thinks we have accuracy, prediction window etc.

- Ericsson thinks that case 2 is more complex. Oppo and Apple that there many means to address the problem. Oppo thinks that both direct and indirect.

- Nokia actually thinks that direct approach is simpler as we predict right away what we want to predict, with indirect we need to identify the measurements we need to determine out.

- Qualcomm is not sure what direct approach.  Huawei indicates that the difference is the data set you use to determine the model and for direct mode it can be based on historical RLF events.

- Xiaomi thinks that we can do direct, but need to discuss HOF separately from RLF and it would be difficult to determine the intermediate metric.  ZTE agrees.

- ZTE would like to understand how to simulate the direct prediction

- Qualcomm is asking how do we introduce RLF events in the simulations. ZTE explains that we would need system level simulations as we need SINR.

- Mediatek thinks we can study both.

##### Prioritization:

R2-2405004	Discussions on other aspects related to RLF/HOF prediction	NTT DOCOMO, INC.	discussion	Rel-19

Proposal 1

Deprioritize the study of HoF/RLF for FR1.

- The priority of the HoF/RLF study for FR2 is lower than that of the other two prediction cases due to the lower potential gain. The study can be started during the later stage of the SI.

- The first issues to be studied are the details of the prediction schemes and usages of the predictions.

-	Apple thinks that we need to do FR1 as FR2 is more of an academic study as there no standalone deployments. Qualcomm thinks we can see the benefits better in FR2.

-	ZTE, Huawei, Intel, Samsung, CMCC, Ericsson thinks we should do FR2 as it is the more challenging scenario. There are not many RLFs in FR1.  Intel thinks that there will be limited gains in FR1.

-	Apple thinks that RLFs happen in FR1

=>	Noted

R2-2404631	On RLF and HO failure prediction	Apple	discussion	Rel-19	FS_NR_AIML_Mob

Proposal 1: the study should focus on RLF failure due to T310 expiry as the representative RLF case.

-	ZTE thinks that this is only apply to indirect. For the direct it is not clear as we don’t know the metric and the timer is based on measurements. Apple explains that the procedure will be based on real measurements.  Oppo, Xiaomi agree with Apple.

Proposal 2: for HO failure study consider only the case of T304 expiry.

=>	Noted

R2-2405477	HOF prediction at UE side	LG Electronics Inc.	discussion	FS_NR_AIML_Mob

Proposal 1: RAN2 to review observation1 to 4, and discuss whether HOF prediction at UE side is feasible, considering both training and online inference aspects, and clarify if and how HOF prediction at UE side can provide benefit, compared to HOF prediction at network side.

Proposal 2: If HOF prediction at UE side is considered non-viable or the exclusive benefit of HOF prediction at UE side is not clear, HOF prediction is excluded from the scope of SI.

=>	Noted

Discussion on HOF

-	Huawei thinks that HOF can be predicted based on RLF so we don’t need separate simulations.  ZTE is concerned that we have 6 cases just for RRM and many other sub-cases and then we have RLF, so we really don’t have time. Xiaomi also thinks we can depriotize. Interdigital thinks that given we have RRM, Measurement event, and RLF there may not be much additional gains.  Both complexity and feasibility is not very clear. Lenovo thinks that it is too soon to exclude as maybe we don’t need to do separate simulations.

-	ZTE and Ericsson remind everyone that the scope of SI clearly indicates that we need to follow legacy mobility framework.

##### Input/Output:

R2-2404560	Discussion on HO failure/RLF prediction	HONOR	discussion	Rel-19	FS_NR_AIML_Mob

Proposal 2: Discuss the following model input/output of serving cell RLF prediction:

Model input:

- Measured serving cell L1 RSRP of SSB/CSI-RS

- RLF parameters (FFS)

  - T310

  - N310

  - N311

Model output:

  - Predicted RLF with timing information

=>	Noted

R2-2404597	Discussion on RLF/HOF prediction	Samsung Shenzhen	discussion	Rel-19	FS_NR_AIML_Mob

Proposal 2. For RLF prediction, the baseline input is RRM measurement results of SpCell. No restriction on other inputs.

Proposal 4. The RLF prediction results (UE reports to NW) includes 1) the RLF probability and/or 2) the expected RLF  time, as baseline.

=>	Noted

Discussion

-	Apple asks if we need to discuss model inputs, are we precluding certain inputs.  WE should leave it up to companies.  Vivo, Xiaomi, Qualcomm and Samsung agrees with apple, but if we really want to discuss input SINR is better than RSRP.

-	Nokia asks what “the expected RLF  time”.  Samsung explains it is the actual time that the UE predict. Interdigital thinks that 1 and 2 are combined, RLF probability within a time window.

-	Lenovo thinks that the expected RLF time is not needed.

-	LG and Vivo assumes the second one is the baseline and from that we can predict 1. Xiaomi thinks that the second one is feasible as with direct prediction it can predict the RLF. Nokia thinks we should start with 1 and can do 2 if we understand it a bit better. Oppo and Intel thinks that the time is easier to predict than the probability. Huawei thinks that we should start with first as a baseline and then use the probability to predict the time.

-	Oppo thinks that getting probability or time is different and depends on the model. Huawei thinks that there has to be both time and time window are needed.  Qualcomm thinks that expected time is not in scope.

-	Samsung explains that the second case is for indirect case and we may not be able to get the probability. Xiaomi and ZTE agree

Agreements

1	Study Indirect: RLF prediction based on the temporal domain serving cell measurement predictions (e.g. SINR).

2	Study Direct: Directly RLF prediction by AI/ML models.

3	FR2 study will be prioritized for RLF prediction.

4	The study should focus on RLF due to T310 expiry (i.e. in-synch/out-of-synch case) as the representative RLF case for direct and indirect prediction.

5	HOF prediction is downprioritized in our study.  NO simulations/evaluations should be done/submitted

6	RLF prediction result is the RLF probability within a time window or at time instance, at least for direct case.  FFS on expected RLF time and indirect case.

7	No evaluation/simulations are expected for August meeting for RLF

8	Simulation assumption specific to RLF will be discussed in August. The assumption is that we will reuse RRM simulation assumptions (where possible).

##### KPIs:

R2-2404631	On RLF and HO failure prediction	Apple	discussion	Rel-19	FS_NR_AIML_Mob

Proposal 6: consider additional KPIs (beyond RLF and HO prediction accuracy and prediction window) once it has been established that AI/ML is capable of predicting at least RLF with sufficiently high accuracy and sufficiently long prediction window.

R2-2405029	Discussion on other aspects related to RLF and HOF prediction	CMCC	discussion	Rel-19	FS_NR_AIML_Mob

Proposal 4: The following performance metrics/KPIs could be considered for HOF/RLF prediction:

- The performance metrics/KPIs defined for RRM measurement prediction can be reused for sub use case 1, e.g. RSRP difference to the actual measurement

- The prediction accuracy for the occurrence of HOF/RLF, e.g. precision, recall or F1-score

- Timing difference between timing of actual HOF/RLF and predicted HOF/RLF

=>	Noted

R2-2404269	Areas of interest for RLF/HO failure prediction	Intel Corporation	discussion	Rel-19	FS_NR_AIML_Mob

Proposal 6: For RLF/HO failure prediction using AI/ML for L3 based handover, following performance KPIs are considered as baseline:

- Reduction in HO failure rate compared to non-AI/ML (%)

- Reduction in RLF rate compared to non-AI/ML (%)

- Precision and Recall for HO failure/RLF predictions

- =>	Noted

Not treated

R2-2404269	Areas of interest for RLF/HO failure prediction	Intel Corporation	discussion	Rel-19	FS_NR_AIML_Mob

R2-2404310	Other aspects related to RLF/HOF prediction	vivo	discussion	Rel-19	FS_NR_AIML_Mob

R2-2404428	Discussion on further considerations for AI/ML-based mobility	Continental Automotive	discussion

R2-2404560	Discussion on HO failure/RLF prediction	HONOR	discussion	Rel-19	FS_NR_AIML_Mob

R2-2404597	Discussion on RLF/HOF prediction	Samsung Shenzhen	discussion	Rel-19	FS_NR_AIML_Mob

R2-2404603	Discussion on RLF and HOF prediction assumptions	Xiaomi	discussion

R2-2404697	Consideration on RLF and HO Failure Prediction	CATT, Turkcell	discussion	Rel-19	FS_NR_AIML_Mob

R2-2404717	Discussion on RLF and HOF use case	OPPO	discussion	Rel-19	FS_NR_AIML_Mob

R2-2404807	Prediction for HO failure and RLF	Lenovo	discussion	Rel-19

R2-2405004	Discussions on other aspects related to RLF/HOF prediction	NTT DOCOMO, INC.	discussion	Rel-19

R2-2405029	Discussion on other aspects related to RLF and HOF prediction	CMCC	discussion	Rel-19	FS_NR_AIML_Mob

R2-2405067	Discussion on RLF and HO failure prediction	ZTE Corporation	discussion	Rel-19	FS_NR_AIML_Mob

R2-2405075	Discussion on failure prediction	NEC	discussion	Rel-19	FS_NR_AIML_Mob

R2-2405097	Discussion on AI/ML based RLF and HOF predictions	Ericsson	discussion	FS_NR_AIML_Mob

R2-2405203	RLF/HO failure prediction	Qualcomm Incorporated	discussion	Rel-19

R2-2405209	Other aspects related to RLF/HOF prediction	Interdigital Inc.	discussion	Rel-19	FS_NR_AIML_Mob

R2-2405303	Discussion on HO failure/RLF prediction	Nokia	discussion	FS_NR_AIML_Mob

R2-2405382	Potential scenarios for RLF/HOF prediction 	Kyocera 	discussion	Rel-19	R2-2403420

R2-2405477	HOF prediction at UE side	LG Electronics Inc.	discussion	FS_NR_AIML_Mob

R2-2405478	RLF prediction	LG Electronics Inc.	discussion	FS_NR_AIML_Mob

R2-2405545	Discussion on RLF and HO failure prediction	ETRI	discussion

R2-2405652	Discussion on HOF and RLF prediction	Huawei, HiSilicon	discussion	Rel-19	FS_NR_AIML_Mob

R2-2404348	Discussion on RLF/HOF prediction	Fujitsu	discussion	Rel-19	FS_NR_AIML_Mob

R2-2404366	AI/ML HO failure prediction	TCL	discussion	Rel-19

R2-2404367	AI/ML RLF prediction	TCL	discussion	Rel-19

R2-2404631	On RLF and HO failure prediction	Apple	discussion	Rel-19	FS_NR_AIML_Mob

## 8.4	Low-power wake-up signal and receiver for NR (LP-WUS/WUR)

(NR_LPWUS-Core; leading WG: RAN1; REL-19; WID: RP-240801)

Time budget: 1 TU

Tdoc Limitation: 3 tdocs

### 8.4.1	Organizational

LS, Rapporteur input, including workplan, etc.

On potential LS to R1

- ZTE, Nokia, CATT do not see a need to send LS.

- Ericsson think R2 already agree and think we can inform. No strong view on whether to send LS but if we send an LS, it should include the concern that was discussed.

- Rapp think the other WGs can anyway check the agreements.

Chair: based on the discussions in CB, we decided not to send LS to R1 after this meeting. The previous agreement that we should inform R1 is reverted.

On post meeting email discussion

Chair: after discussions, it is concluded that there is no need for any post meeting email disc for this topic.

### 8.4.2	Procedure and configuration of LP-WUS in RRC_IDLE/INACTIVE

Procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring

Entry/exit condition

R2-2404460	Entering/Exit conditions, relaxed serving cell measurements on the main receiver and offload of  measurements to LP-WUR	Vodafone 	discussion	Rel-19

Proposal 2: It is proposed to define LP-WUR entering and exit conditions as follow:

•	LP-WUR entering conditions are satisfied if corresponding thresholds as provided over broadcast using SIBs or dedicated signalling are fulfilled and the corresponding functionality is taken over by the LP-Receiver in accordance with the performance as defined by RAN WG4.

•	LP-WUR exit conditions are satisfied if corresponding thresholds as provided over broadcast using SIBs or dedicated signalling are fulfilled and the corresponding functionality are taken over by the main receiver in accordance with the performance as defined by RAN WG4.

R2-2404562	Discussion on LP-WUS in RRC_IDLE/INACTIVE	HONOR	discussion	Rel-19	NR_LPWUS-Core

Proposal 2: If the serving cell quality, e.g. legacy RSRP, (FFS the new measurement result based on LP-SS), is above configured threshold(s), UE can monitor LP-WUS and decide whether to monitor legacy (PEI) PO as indicated by LP-WUS.

Proposal 3: If the serving cell quality, e.g. legacy RSRP and / or the new measurement result based on LP-SS, is below a configured threshold, UE should monitor (PEI) PO as in legacy.

Discussions on general direction

- Xiaomi, Samsung, Huawei, Sony generally fine with the direction of having seving cell quality as the base of those conditions, but not sure why we need to discuss more procedures as in VDF’s paper. VDF clarifies that the paper discuss more than paging procedure.

- OPPO think R1 already have assumptions on entry/exit conditions, so not sure if we need to repeat the discussions in R2. QC share this view.

- Apple think R2 can first confirm R1’s working assumption on the conditions, and then see if in R2 we can progress further on the FFS parts.

- Huawei think it is convenient to use the terminologies such as MR/LR. CATT agrees.

- IDT, Ericsson agree with Honor, and think it is up to UE whether to monitor LP-WUS. Ericsson think we should first discuss what the UE does when the conditions are fulfilled, e.g., it is related to how measurements are done.

- Sony think entry condition is clear and it is based on MR, for exit condition it is based on LR.

- Vivo think from the comments almost all agree that R1’s assumption is agreeable. ZTE agree. ZTE think R1 assume that it is R2 to discuss the conditions, and ZTE think in R2 we should confirm that those conditions are provided by gNB to UE.

RAN2 will further discuss the details about LP-WUS monitoring entry/exit conditions based on RAN1’s existing working assumptions.

Discussion on configuration of entry/exit in CB:

- Chair asks whether it is possible to remove the FFS from pervious WA.

- Ericsson ask what is the relationship btw the configuration of LP-WUS and LP-SS.

- Apple and HW suggest making it agreement, not WA.

The LP-WUS related configuration in SIB at least include the following information for IDLE/INACTIVE:

- -	LP-SS configuration

- -	LP-WUS configuration

- -	Entry/exit condition for LP-WUS monitoring (FFS if it is always configured)

Discussions based on Honor P2/3

-	Xiaomi wonders whether ‘low mobility’ type of condition is included in these proposals or not. NEC think it should be considered. Honor think it is open for discussion. OPPO not sure why we need to consider low mobility, as now what matters is mainly coverage.

-	ZTE, LG E fine with these proposals and ask do we also add RSRQ.

-	LG E think we cannot force UE to do measurement based on LR before the entry condition is met.

-	VDF think we do need to mention UE implementation in the definition of the conditions.

-	VDF and Nokia think after the condition is fulfilled the UE behaviour starts with ‘shall’, not ‘may’. QC, Apple, IDT think we should align with R1 wording, which is ‘may’.

-	Vivo think the only new thing we are discussing here is about the metrics, i.e., RSRP/RSRQ.

-	Ericsson think we need to discuss measurements.

-	QC and Apple think for the exit condition R2 should not discuss the metrics from LR, and it is R1 discussion.  Vivo think R1 already agreed on RSRP/RSRQ from LR, and they are discussing on possibility of using SINR.

Baseline for entry condition definition: If the serving cell quality, e.g. RSRP, RSRQ from MR, is above threshold(s) (if configured), UE may start to monitor LP-WUS, if UE monitors LP-WUS, it may stop monitoring the legacy PO. FFS if any measurement from LR is needed.

Baseline for exit condition definition: If the serving cell measurement result based on LR is below a threshold (if configured), UE monitors PO as in legacy and it may stop monitoring the LP-WUS.

Sub-grouping

R2-2404376	LP-WUS Operation in RRC_IDLE/INACTIVE	CATT	discussion	Rel-19	NR_LPWUS-Core

Proposal 1: The maximum number of subgroups for LP-WUS depends on RAN1 conclusion.

Proposal 2: For CN assigned subgrouping for LP-WUS, the similar procedure for CN assigned subgrouping for PEI is reused.

Proposal 3: Send an LS to SA2/CT1/RAN3 for the design of CN assigned subgrouping for LP-WUS.

Proposal 4: For UE_ID based subgrouping for LP-WUS, similar formula for PEI subgrouping is reused, i.e.,

SubgroupID = (floor (UE_ID/(N*Ns)) mod subgroupsNumForUEID_LP) + (subgroupsNumPerPO_LP – subgroupsNumForUEID_LP), where

UE_ID is related to 5G-S-TMSI, detail FFS,

N is the number of total paging frames in DRX cycle,

Ns is the number of the PO for a PF,

subgroupsNumForUEID_LP and subgroupsNumPerPO_LP are the subgroup number for UE_ID based subgrouping for LP-WUS and the total subgroup number for LP-WUS, respectively.

Proposal 5: It is up to network implementation that UEs belong to the same LP-WUS subgrouping are expected to be assigned to different PEI subgroupings. No specification impact is needed.

Discussion on P4 in CB session:

- Ericsson suggest to go for P4. ZTE fine and suggest to make it baseline.

- OPPO think exact signalling is up to CT1.

- Lenovo think this is too vague and we already had similar agreement from last meeting. Xiaomi also has similar question.

- Xiaomi wonders if UE support PEI and it’s already assigned with subgroup ID, what is the procedure for LPWUS subgroup.

?? For CN assigned subgrouping for LP-WUS, the similar procedure for CN assigned subgrouping for PEI is taken as baseline.

Discussion on P5 in CB session:

- NEC think P5 is ok, but want to add a note that ‘UE is not required to monitor both…’, so think it is too early to say that ‘no spec impact’.

- Samsung wonders whether this is for both CN and UE-ID based? CATT think yes.

- QC not sure whether the UE behaviour is the same for CN and UE-ID based subgroup. Thinks it is too early to conclude on this.

- HW think UE-ID based subgroup require further discussion.

- Apple think these two features are not linked, e.g., UE can just support LPWUS not PEI. Apple suggest to first focus on LPWUS, not discuss them together.

- Ericsson think this is not clear.

- VDF ask if for CN based way, the subgroup can be the same for LP-WUS and PEI, wants to minimize the impact to CN spec.

- Sony think we can say we reuse the concept/method of CN assigned subgroup.

RAN2 understand that if UE is configured with CN-based LP-WUS subgrouping, it is up to CN to assign the LP-WUS subgroup ID to the UE.

R2-2405637	Procedure and Configuration of LP-WUS in RRC IDLE/INACTIVE	Lenovo	discussion	Rel-19	NR_LPWUS-Core

Proposal 6: The maximum number of subgroups that can be configured for LP-WUS subgrouping is 8.

Proposal 7: RAN2 to discuss the inclusion of a LP-WUS Subgrouping Configuration in the SIB signalling.

Proposal 8: RAN2 to discuss the configuration of 2-level subgrouping to reduce false alarm rates in comparison to legacy PEI.

Discussions:

- OPPO, ZTE, NEC think maximum # of subgroups should wait for R1. Vivo think for PEI R2 decided, so now R2 can conclude as well.

- Ericsson think 3 bits is not enough to reduce the false alarm. Lenovo think we do not need to talk about # of bits, which is designed by R1.

- LG E think we can decide, and think 8 is sufficient, think we can inform R1. Spreadrum agree it is a balanced value.

- HW think at least 16 to reduce false alarm.

- QC think this is also controversial in R1 since it impact signal design/payload.

- Sony think we do not need to limit in R2 and think we should consider what is the impact for different values.

RAN2 assume the maximum number of subgroups that can be configured for LP-WUS subgrouping is no less than 8.

SI reception

R2-2404459	Discussion on procedure and configuration of LP-WUS in RRC_IDLE/INACTIVE	Huawei, HiSilicon	discussion

Proposal 9: Only the necessary system information needs to be maintained when UE is working with LP-WUR in RRC_IDLE/INACTIVE modes.

Proposal 10: Introduce a common LP-WUS to wake up all the UEs in RRC_IDLE/INACTIVE if the subgrouping information in LP-WUS is indicated in a codepoint way.

R2-2405577	LP-WUS operation in IDLE/Inactive state	Qualcomm Incorporated	discussion	NR_LPWUS-Core

Proposal 12	From RAN2 perspective, LP-WUS should be able to wake up all UEs to monitor paging for SI update/ETWS/CMAS notification. How to support LP-WUS to indicate all the UEs is left to RAN1 discussion.

Discussion:

- Samsung wonders if P9 in HW paper requires new functionality regarding SI reception. HW think we can further discuss.

- Nokia, ZTE, Apple do not think new functionality is needed.

- LG E not sure what is necessary SI in P9.

From RAN2 perspective, no new procedure is introduced for SI reception/updates.

R2-2404295	General considerations on the procedure for RRC_IDLE_INACTIVE	Xiaomi Communications	discussion

R2-2404314	LP-WUS procedure in RRC_IDLE INACTIVE 	NEC	discussion	Rel-19	NR_LPWUS-Core

R2-2404418	Discussion on LP-WUS WUR in RRC_IDLE INACTIVE	vivo	discussion	Rel-19	NR_LPWUS-Core

R2-2404469	LP-WUS in IDLE and INACTIVE	Nokia	discussion	Rel-19	NR_LPWUS-Core

R2-2404588	Discussion on procedure and configuration of LP-WUS in RRC_IDLE/INACTIVE	OPPO	discussion	Rel-19	NR_LPWUS-Core

R2-2404674	Procedure and configuration of LP-WUS in RRC_IDLE/INACTIVE	Apple	discussion	Rel-19	NR_LPWUS-Core

R2-2404860	Procedure and configuration of LP-WUS for IDLE and INACTIVE mode	ZTE Corporation, Sanechips	discussion	Rel-19	NR_LPWUS-Core

R2-2404906	RAN2 aspects on LP-WUS/WUR in RRC Idle/Inactive mode	Sony	discussion	Rel-19	NR_LPWUS-Core

R2-2404927	Discussion on LP-WUS operation in IDLEI/NACTIVE mode	Spreadtrum Communications	discussion	Rel-19

R2-2404996	WUR in Idle and Inactive	Ericsson	discussion	Rel-19	NR_LPWUS-Core

R2-2405223	LP-WUS in RRC_IDLE and RRC_INACTIVE	LG Electronics Inc.	discussion	Rel-19	NR_LPWUS-Core

R2-2405308	Discussion on procedure and configuration of LP-WUS in RRC_IDLE/INACTIVE	China Telecom	discussion	Rel-19	NR_LPWUS-Core

R2-2405325	Discussion on LP-WUS operation in RRC_IDLE/INACTIVE modes	InterDigital, Inc.	discussion	Rel-19	NR_LPWUS-Core

R2-2405354	Discussion on entry exit conditions for LP-WUS monitoring	Sharp	discussion

R2-2405409	Procedure and Configuration of LP-WUS in RRC Idle Inactive Mode	Samsung	discussion	Rel-19

R2-2405497	LP-WUS operation in IDLE/INACTIVE modes	CMCC	discussion	Rel-19	NR_LPWUS-Core

R2-2405638	Discussion on Procedure and configuration in RRC_IDLE-INACTIVE	NTT DOCOMO INC..	discussion	Rel-19	NR_LPWUS-Core

R2-2405695	Discussion on LP-WUS_WUR entry and exit conditions for RRC Idle_Inactive mode	KT Corp.	discussion	Rel-19	NR_LPWUS-Core	Late

### 8.4.3	RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE

RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions

Serving cell measurements related

R2-2404315	LP-WUS RRM measurement relaxation 	NEC	discussion	Rel-19	NR_LPWUS-Core

Proposal 1: RAN2 to consider the following solutions on serving cell measurement during LP-WUS monitoring mode:

Option-1: serving cell measurement fully offloaded to LR (i.e., no serving cell measurement via MR is required), the LR serving cell measurement requirement is decided by RAN4/1.

Option-2: serving cell measurement partially offloaded to LR (i.e., relaxed serving cell measurement via MR is still required), the LR and MR serving cell measurement requirement is decided by RAN4/1

Proposal 2: if proposal 1 is agreed, send an LS to RAN4/1 to ask discussing those feasibilities and related requirements.

R2-2405224	RRM relaxation and RRM offloading	LG Electronics Inc.	discussion	Rel-19	NR_LPWUS-Core

Proposal 1	The RRM relaxation in this WI means the RRM measurements using MR with relaxed measurement requirements to be defined by RAN4 without any involvement of LP-WUS/LP-WUR.

Proposal 2	The RRM offloading in this WI means performing LP-SS or PSS/SSS measurement using LP-WUR and not performing serving cell measurements using MR.

Proposal 3	For serving cell measurement relaxation, RAN2 should focus on specifying the relaxed measurement criterion for serving cell, and assume that RAN4 will define the relaxed measurement requirements for serving cell.

Proposal 4	Consider the two relaxed measurement criteria in Rel-16, i.e. criterion for UE with low mobility and criterion for UE not at cell edge, as baseline for relaxed measurement criterion for serving cell.

Proposal 6	RAN2 specifies the criterion for serving cell measurement offloading.

Discussion on LG E P3

- OPPO think there may be the case when both MR and LR are doing measurement.

- NEC think for relaxation it is possible to have measurements from both MR and LR. NEC think we can discuss whether fully loading to LR is supported. Apple also agree.

- Lenovo think R2 should first discuss on condition and support P4 from LG E paper as baseline.

- Ericsson think P3 should only be limited to UE that support LP-WUS. Apple agree. Vivo, QC has different view.

- VDF think relaxation is only used when UE is within LP-WUS coverage.

- QC think both cases should be included, whether LR is involved or not.

- Lenovo think P3 is fine.

- Xiaomi think the TR says that relaxation only happens when measurement if offloaded to LR. Samsung share this view.

- vivo suggest to discuss relx part in the next meeting, and think offloading part already progress in R4 so we can also check. CATT agree.

- Ericsson think in further discussions we should clarify how does this relate to entry/exit conditions of LP-WUS monitoring.

For serving cell measurement offloading (i.e., serving cell measurement fully offloaded to LR  and no serving cell measurement via MR is required), RAN2 should focus on specifying the offloading criterion for serving cell for UEs supporting LP-WUS, and assume that RAN4 will define the measurement offloading requirements for serving cell.

Neighboring cell measurements related

R2-2405013	Discussion on RRM measurement relaxation/offloading in IDLE/INACTIVE modes	CMCC	discussion	Rel-19	NR_LPWUS-Core

Proposal 8: The RRM measurement of the neighboring cell can only be performed by MR.

Proposal 9: The neighbor cell measurement relaxation mechanism in Release 16 can be reused(e.g., not at cell edge and low mobility).

Proposal 10a: For high-priority neighboring cells, if Threshold1>SSearchThresholdP, the “low mobility” relaxation entry condition uses the LR-based serving cell measurement results.

Proposal 10b: For high-priority neighboring cells, if Threshold1<SSearchThresholdP, both “low mobility” relaxation entry condition and “not at cell edge” relaxation entry condition uses the LR-based serving cell measurement results.

Proposal 11a: For equal or low priority neighboring cells, if SnonIntraSearchP>Threshold1>SSearchThresholdP, the “low mobility” relaxation entry condition uses the LR-based serving cell measurement results.

Proposal 11b: For equal or low priority neighboring cells, if SSearchThresholdP>Threshold1, both “low mobility” relaxation entry condition and “not at cell edge” relaxation entry condition uses the LR-based serving cell measurement results.

Proposal 11c: For equal or low priority neighboring cells, if Threshold1> SnonIntraSearchP, there is no effect on existing conditions.

Discussions on general aspects:

- Xiaomi, Ericsson, CATT fine with P8/9, and think the other proposals change legacy procedure and do not support. CATT think the other proposals can be further discussed.

- NEC think we postpone high prioiry neighbour cell.

- Ericsson have different view of modelling regarding P10-11, and think most company agree in this case MR is switched off and there is no neighboring measurement. Otherwise MR is waken up to measure the neighbor cell. MR handle the neighbor related measurements.

Discussion on P8/9:

- QC think R1 is discussing on LP-SS/OFDM based measurement, we may be able to measure the neighbour cell using LR. Sony do not support P8 either.

- Sony think we do not need to discuss MR meas, since it is legacy.

- VDF agree with CMCC that neighbour cell measurement is from MR. IDT agree.

- ZTE suggest to make P8 a R2 assumption for now and maybe wait for R1 info.

- Lenovo fine with P8, think P9 is not clear.

- Apple and LG E think we have this limitation in the WID.

- Nokia think the exit criteria is based on LR for serving, and suggest to make it clear in the agreement. Sony share this view. CATT think entry condition is based on LR.

- Huawei think it is better to say ‘we reuse R16 mechanism as baseline.’

- OPPO not ready to agree the addition on exit condition based on LR.

RAN2 understand that the RRM measurement of the neighboring cell can only be performed by MR. Can discuss again if RAN1 inform us otherwise.

RAN2 will further discuss the neighbor cell measurement relaxation criteria (if the UE is using LR to measure the serving cell), e.g., considering reuse Rel-16 criteria for ‘not at cell edge’ and ‘low mobility’.

R2-2404301	Discussion on RRM measurement relaxation for RRC_IDLE_INACTIVE	Xiaomi Communications	discussion

R2-2404323	Discussion on RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE	Huawei, HiSilicon	discussion	Rel-19	NR_LPWUS-Core

R2-2404377	RRM Relaxation and Offloading in RRC_IDLE/INACTIVE	CATT	discussion	Rel-19	NR_LPWUS-Core

R2-2404399	Discussion on RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE	China Telecom 	discussion

R2-2404419	Discussion on RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE	vivo	discussion	Rel-19	NR_LPWUS-Core

R2-2404470	RRM measurement relaxation in RRC_IDLE/INACTIVE	Nokia	discussion	Rel-19	NR_LPWUS-Core

R2-2404583	Discussion on RRM measurement in RRC IDLE and INACTIVE	OPPO	discussion	Rel-19	NR_LPWUS-Core

R2-2404675	RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE	Apple	discussion	Rel-19	NR_LPWUS-Core

R2-2404808	RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE	Lenovo	discussion	Rel-19

R2-2404861	RRM measurement relaxation for IDLE and INACTIVE mode	ZTE Corporation, Sanechips	discussion	Rel-19	NR_LPWUS-Core

R2-2404907	Discussion on RRMRAN2 aspects foron LP-WUS/WUR	Sony	discussion	Rel-19	NR_LPWUS-Core

R2-2404928	Discussion on RRM measurement relaxation and offloading in IDLE/INACTIVE mode	Spreadtrum Communications	discussion	Rel-19

R2-2404997	WUR and RRM measurements	Ericsson	discussion	Rel-19	NR_LPWUS-Core

R2-2405328	Discussion on RRM measurement relaxation and offloading	InterDigital, Inc.	discussion	Rel-19	NR_LPWUS-Core

R2-2405355	Discussion on serving cell RRM measurement offloading	Sharp	discussion

R2-2405410	RRM measurement relaxation and offloading in RRC Idle Inactive Mode	Samsung	discussion	Rel-19

R2-2405579	LP-WUS RRM measurement relaxation and offloading	Qualcomm Incorporated	discussion	NR_LPWUS-Core

### 8.4.4	Procedures for LP-WUS in RRC_CONNECTED

Procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring.

R2-2404420	Discussion on LP-WUS WUR in RRC_Connected	vivo	discussion	Rel-19	NR_LPWUS-Core

=> Revised in R2-2405935

R2-2405935	Discussion on LP-WUS/WUR in RRC_CONNECTED	vivo	discussion	Rel-19	NR_LPWUS-Core

P1:

- Nokia think it is better to stick to TR, instead of going into detailed solutions identified by R1.

- CATT think it is better to progress based on R1 progress, and think we should avoid duplicated discussions. Ericsson agree, and think R1 is focusing on PDCCH monitoring, and think in connected state UE should continue with legacy CSI report.

- Apple think the options in R1 agreements are not against TR, and think we in R2 can start with O1-1.

- Lenovo want to know whether in R2 we need to down-select?

- Sony think P1 is quite general and wonders how do we progress in R2.

In RRC_CONNECTED mode, RAN2 to further discuss the impacts of LP-WUS operation methods identified in RAN1.

O1-1, P2:

- ZTE ok with P2. But for P3, ZTE think in R1 there are some proposals to shift the timer so not sure if we can agree reuse DCP mechanism.

- LG E want to clarify that DCP and LP WUS do not co-exist, as that is duplication. OPPO, CATT agree.

- Ericsson, CATT fine with P2 and P3, and think it is useful and simple way to also use LPWUS in connected. Ericsson think it can be that O1-1 is a special case for O1-2-2.

- CATT think we can first go to P2.

- Nokia think how PDCCH is monitored is R1 disc, so we should postpone disc on O1-1.

- Samsung think P2 from vivo is clear and do not think it useful to further rewording.

- VDF think P2 is mainly about PDCCH monitoring.

For Option 1-1 (as described in RAN1 agreement), the LP-WUS monitoring occasion locates at a configured time offset before the start of drx-onDurationTimer. The range of time offset can be determined by RAN1.

P3:

- QC think this is too restrictive, and think we can say that DCP mechanism is taken as baseline.

- NEC do not see a need to for P3.

- Ericsson think original P3 is good, but OK to move with the compromise wording. Lenovo, LG E also ok with it, and want to add that DCP and O-1 here do not work together. LG E wants to clarify that LP-WUS replace DCP.

- Rapp observes that most of the companies are ok with reworded P3.

For Option 1-1, RAN2 assumes the solutions/ operations introduced for DCP mechanism is taken as baseline.

RAN2 assume that legacy DCP and Option 1-1 is not configured simultaneously for a UE.

P5:

- NEC generaly OK but think wording can be simplified.

- Ericsson want to understand better what is the spec impact with O1-2-2, and think it does not give further power saving gain over O1-1.

- Apple think maybe we need new definition of active time.

- LG E think the point of 1-2-2 is about defining new active time outside the legacy active time. Nokia agree and think once UE start active time it follows legacy behaviour.

- VDF wonders what is the relationship btw 1-2-2 and 1-1, do we choose btw them.

- CATT think original P5 is better and clear.

- OPPO think the intention is to understand what 1-2-2 means. OPPO think 1sts aspects is LPWUs triggers UE’s mornitoring of PDCCH. And 2nd aspect LPWUS also triggers drx-onDurationTimer.

?? RAN2 will further discuss Option 1-2-2 based on the following understanding for it:

- LP-WUS does not impact PDCCH monitoring in legacy active time other than legacy drx-onDurationTimer.

Or??

- For LP-WUS procedure to trigger PDCCH monitoring Option 1-2-2, LP-WUS only impacts the PDCCH monitoring outside C-DRX Active Time and during legacy drx-onDurationTimer, i.e. UE should keep monitoring PDCCH during the C-DRX Active Time other than the time when drx-onDurationTimer is running, even no LP-WUS has been detected.

Or??

Upon the detection of LP-WUS, the UE starts active time regardless of C-DRX config. How exactly this is done is FFS. Once UE starts PDCCH monitoring, it follows legacy active time definition.

Proposal 6: For LP-WUS procedure to trigger PDCCH monitoring Option 1-2-2, the following solutions on how to stop the PDCCH monitoring could be considered:

−	The UE stops the PDCCH monitoring upon receiving the PDCCH skipping indication;

−	Using a timer for PDCCH monitoring triggered by LP-WUS, and the UE performs PDCCH monitoring until the timer is expired. FFS whether a new timer or reuse legacy DRX timer, e.g. drx-InactivityTimer.

Proposal 7: For LP-WUS procedures to trigger PDCCH monitoring Option 1-3, UE monitors the LP-WUS within the drx-onDurationTimer duration and the UE starts to monitor PDCCH only if the detected LP-WUS indicates UE to monitor PDCCH.

Proposal 8: For LP-WUS procedure to trigger PDCCH monitoring Option 1-3, LP-WUS only impacts the PDCCH monitoring during legacy drx-onDurationTimer, i.e. UE should keaep monitoring PDCCH during the C-DRX Active Time other than the time when drx-onDurationTimer is running, even no LP-WUS has been detected.

Proposal 9: RAN2 assumes Option 1-2-2 should be supported for LP-WUS in RRC_CONNECTED mode.

Proposal 10: In RRC_CONNECTED mode, PDCCH monitoring triggered by LP-WUS is enabled/disabled by the corresponding configuration of LP-WUS. No additional assistance information from UE is needed.

Proposal 11: LP-WUS configuration is configured in RRCResume/RRCReconfiguration message.

Proposal 12: In RRC_CONNECTED mode, in case LP-WUS monitoring is enabled, activation/deactivation of LP-WUS monitoring by L1/L2 signaling is introduced. Details FFS.

R2-2404862	Procedure for LP-WUS in RRC_CONNECTED	ZTE Corporation, Sanechips	discussion	Rel-19	NR_LPWUS-Core

Proposal 1: LP-WUS configuration for RRC-CONNECTED should be provided by UE specific signaling, and FFS on what information is configured.

Proposal 2: The provision of LP-WUS configuration via dedicated signaling is seen as an implicit activation to apply LP-WUS for UE in RRC_CONNECTED state.

Proposal 3: RAN2 can postpone the discussion on whether to support L1/L2 activation/deactivation of LP-WUS in RRC-CONNECTED until there is agreement for the LP-SS and LP-WUS design in RAN1.

Proposal 4: RAN2 consider the option 1-1(e.g. LP-WUS replace DCP) as baseline for LP-WUS monitoring in RRC_CONNECTED state.

Proposal 5: Whether to support Option 1-2(monitoring LP-WUS outside the C-DRX active time) or Option 1-3(monitoring LP-WUS inside the C-DRX active time) can wait for RAN1’s further agreement.

R2-2405411	Procedures for LP-WUS in RRC Connected Mode	Samsung	discussion	Rel-19

Proposal 1. As a baseline scenario, RAN2 supports LP-WUS configuration before C-DRX on duration, similar as DCP (option 1-1).

Proposal 2. For the support of LP-WUS and associated PDCCH monitoring outside of C-DRX on duration, we need to wait for RAN1 progress.

P3:

- OPPO think we should further clarify what are the configurations.

- Ericsson, QC think P3 is ok, and detailed content of the config can be discussed and informed to R1.

- Nokia ask if this means we exclude other ways e.g., MAC. Sony think we are not excluding MAC. Samsung think we can remove e.g. part.

- ZTE think configuration can only be provided by RRC. HW agree, and think we just use RRC message.

The LP-WUS related configuration for RRC CONNECTED state UE is provided via dedicated RRC message.

R2-2404244	Discussion on LP-WUS for RRC_CONNECTED mode	Huawei, HiSilicon	discussion	Rel-19	NR_LPWUS-Core

R2-2404302	Discussing on LP-WUS monitoring for RRC_Connected	Xiaomi Communications	discussion

R2-2404316	LP-WUS procedure in RRC_CONNECTED 	NEC	discussion	Rel-19	NR_LPWUS-Core

R2-2404378	Analysis on LP-WUS in RRC_CONNECTED	CATT	discussion	Rel-19	NR_LPWUS-Core

R2-2404380	LP-WUS in RRC_CONNECTED	Nokia	discussion	NR_LPWUS-Core

R2-2404584	Discussion on LP-WUS in RRC_CONNECTED	OPPO	discussion	Rel-19	NR_LPWUS-Core

R2-2404676	Procedures for LP-WUS in RRC_CONNECTED	Apple	discussion	Rel-19	NR_LPWUS-Core

R2-2404908	Considerations on LP-WUS/WUR in RRC Connected mode	Sony	discussion	Rel-19	NR_LPWUS-Core

R2-2404998	WUR in Connected	Ericsson	discussion	Rel-19	NR_LPWUS-Core

R2-2405033	Discussion on LP-WUS operation in CONNECTED mode	CMCC	discussion	Rel-19	NR_LPWUS-Core

R2-2405309	Discussion on procedures for LP-WUS in RRC_CONNECTED	China Telecom	discussion	Rel-19	NR_LPWUS-Core

R2-2405332	Discussion on LP-WUS operation in RRC_CONNECTED mode	InterDigital, Inc.	discussion	Rel-19	NR_LPWUS-Core

R2-2405468	Discussion on LP-WUS in RRC_CONNECTED mode	LG Electronics Inc.	discussion	Rel-19

R2-2405578	LP-WUS operation in CONNECTED state	Qualcomm Incorporated	discussion	NR_LPWUS-Core

R2-2405639	LP-WUS in RRC Connected Mode	Lenovo	discussion	Rel-19	NR_LPWUS-Core

R2-2405687	Discussion on LP-WUS in RRC_CONNECTED	NTT DOCOMO INC..	discussion	Rel-19	NR_LPWUS-Core

## 8.5	Network Energy Saving Enh.

(Netw_Energy_NR_enh-Core; leading WG: RAN1; REL-19; WID: RP-240170)

Time budget: 1 TU

Tdoc Limitation: 3 tdocs

### 8.5.1	Organizational

LS, Rapporteur input, including workplan, etc.

R2-2404121	LS on the conditions for triggering UL WUS transmission to request on-demand SIB1 (R1-2403779; contact: MediaTek)	RAN1	LS in	Rel-19	Netw_Energy_NR_enh	To:RAN2

- Noted.

### 8.5.2	On-demand SSB SCell operation

Scenarios/use cases, RAN2 spec impacts and high-level solutions.

R2-2404170	Discussion on On-Demand SSB	OPPO	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2404201	Discussion on on-demand SSB for NES	Ericsson	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2404227	On-demand SSB SCell Operation	Samsung	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2404261	RAN2 impacts to enable on-demand SSB SCell	Intel Corporation	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2404576	Discussion on on-demand SSB	Xiaomi	discussion

R2-2404633	Discussion on RAN2 work of on-demand SSB for Scell	Apple	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2404821	Issues on the procedure of on-demand SSB SCell operation	Lenovo	discussion	Rel-19

R2-2404857	Further consideration on on-demand SSB SCell operation in connected mode	ZTE Corporation, Sanechips	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2404893	Discussion on on-demand SSB SCell operation	vivo	discussion	Rel-19

R2-2404909	On-demand SSB Scell operation discussion	Sony	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2404931	Discussion on on-demand SSB SCell operation	Spreadtrum Communications	discussion	Rel-19

R2-2404949	Consideration on on-demand SSB SCell operation	CATT	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2405034	Discussion on on-demand SSB SCell operation	CMCC	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2405076	Discussion on On-demand SSB for SCell	NEC	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2405122	Discussion on on-demand SSB SCell operation for NES	Huawei, HiSilicon	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2405138	On demand SSB transmission for SCell	InterDigital	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2405225	On-demand SSB SCell operation	LG Electronics Inc.	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2405289	Discussion on on-demand SSB SCell operation	Fujitsu	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2405294	Discussion on On-demand SSB SCell Operation	Qualcomm Incorporated	discussion

R2-2405310	On-demand SSB SCell operation	China Telecom	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2405566	On demand SSB handling	Nokia	discussion	Rel-18	Netw_Energy_NR_enh-Core

R2-2405660	Discussion on on-demand SSB procedure	Quectel	discussion	Rel-19	Late

### 8.5.3	On-demand SIB1

Any further consideration on scenarios/use cases, UL WUS configuration, triggering conditions the UE to request on-demand SIB1, procedure of on-demand SIB1 request by RACH, other impacts on RRC idle/inactive UEs (including cell barring, etc.), any impact on RRC connected UEs, etc.

1. Scenario 1a (case 2 in RAN1)

- Further scenario clarification (P1: R2-2404213: Ericsson, P2: R2-2404634: Apple)

Proposal 1: Study on-demand SIB1 provisioning for NES Cell(s) in versions of Scenario 1a with multiple Cells A and/or NES Cells:

- More than one Cell A may provide configuration for the same NES cell.

- The same Cell A may assist more than one NES Cells.

- Agreed.

[Intel, Huawei, Samsung]: Support the proposal. [Vodafone]: Understand if NES cell is the same cell, same WUS configuration will be configured even though it is configured by multiple cells A. [Apple]: NW implementation will ensure it.

Proposal 2 (modified): RRC release message assisted intra-cell WUS can be discussed as option of signaling details in stage 3.

- Agreed.

[LG]: Once RRC release message was transmitted, if the information is updated, how to update the UE of the latest WUS configuration? [Xiaomi]: If RRC release message includes the same WUS configuration as SIB, why not counting on SIB? [Huawei, Nokia]: Understand RRC release based option is supplementary option on top of SIB. [Apple]: Agree that it is supplementary option to reduce signalling overhead due to periodic SIB transmission, however WUS configuration is same as the one in SIB. [KDDI]: Think it is a kind of optimization, so we can discuss it later.

- Information of WUS configuration (P2: R2-2404213: Ericsson, P2: R2-2405295: Qualcomm)

Proposal 2 (modified): Can use the PCI and frequency of a NES Cell to associate the UL WUS configuration with a NES Cell.

- Agreed.

[Huawei]: Proposal 2 is aligned with RAN2 agreement last meeting. [Apple, ZTE]: Even for single NES cell, WUS configuration needs to be associated with PCI and frequency.

Proposal 2 (modified). For Message 1 based on-demand SIB1 request, the on-demand SI request configuration that currently included in SIB1 may be used as the design baseline.

- Agreed.

- Transmission of WUS configuration (P9a, P9b: R2-2404153: Xiaomi)

Proposal 9a (modified): Cell A’s SIB can be used to configure on-demand SIB1 related configuration for neighbour NES cells, e.g., via new SIB or the existing SIB.

- Agreed.

[Lenovo]: Why not configure in the legacy SIBs (e.g. SIB3/4)? [NEC]: The UE needs to check new SIB whenever cell reselection happens? Wording “For cell reselection” gives some confusion.

Proposal 9b (modified): After the UE camps on NES cell, UE can acquire WUS configuration from the NES cell.

- Postponed.

[Session chair]: Understand the scenario is the UE reselected from cell A to NES cell. While UE stays in the NES cell, the UE may need to re-acquire SIB1. Then the question is how the UE gets WUS configuration? E.g. performs cell reselection to cell A and reacquires WUS configuration or reacquire from NES cell itself? [Rakuten]: If NW tries to transmit SIB1 always to guarantee 3 hours invalidity, how NW can have energy saving? [Nokia]: Understand NES cell does not transmit SIB1, then how its SIB1 can be transmitted to cover this concern?

- Triggering condition of WUS transmission (P3-P4.2: R2-2404262: Intel, P9: R2-2405619: Huawei)

Proposal 4 (modified): (1) If the UE chooses the NES cell using legacy intra-F/inter-F cell re-selection procedure (as baseline), the UE triggers WUS transmission.

- Agreed.

[Lenovo]: Does the proposal include both cell selection and cell reselection? For cell selection, cell A is not available. [OPPO]: Understands 3) is applied before camping the NES cell. [Nokia]: With the previous agreement, if RRC release is used, cell selection needs to be also covered.

Proposal 4.1 (modified): After UE successfully receives OD-SIB1 for that NES Cell and if it is a suitable cell, UE camps in the NES Cell “similar” to a legacy Cell.

- Agreed.

[Nokia]: The UE will receive paging and updated system information in NES cell after camping there? [Lenovo]: Yes, and the question would be how long the UE performs them.

Proposal 9: In addition to the already agreed conditions (i.e. UE doesn’t have a valid SIB1, the cell doesn’t broadcast SIB1 and supports on-demand SIB1) introduce a new RSRP triggering condition for the NES UE to be allowed to send WUS to trigger on-demand SIB1. FFS what is the triggering condition and whether the triggering condition is a new threshold.

[Samsung, Xiaomi, OPPO, NEC, Lenovo]: Think the existing cell reselection criterion is enough. [Nokia]: May be useful, but reluctant to agree with them now. We can discuss it in stage 3. [ZTE]: Support the proposal. It is good for NW is allowed to configure separate threshold than cell reselection criterion threshold. [LG]: Current cell reselection criterion is not absolute threshold, it’s relative to the serving cell’s radio condition. Support the proposal. [IDC]: If we have new threshold and the threshold is not met, anyway the existing cell reselection criterion should be used to trigger WUS transmission. [Vivo]: With the new threshold, it will make UE behaviour more complicated. We need to consider many combination cases, e.g. new threshold is {met, not met} + existing cell reselection criterion is {met, not met}. [Samsung]: Rather discussing whether new threshold is required or not, it’s better to discuss whether we’ll allow earlier or later WUS transmission triggering (than cell reselection criterion). The UE may achieve it w/o new threshold. [Apple]: Seems each company has different understanding on the use of the threshold. It’s good to see overall companies’ understanding on how the threshold is used or works.

- UE request SIB1 to perform initial access in NES cell (P8: R2-2404449: Fujitsu)

Proposal 8 (modified): RAN2 not to support on-demand SIB1 request that is combined with an initial access to perform RRC connection establishment/resume on the NES cell.

- Agreed.

- Legacy UE impact (P4, P5: R2-2405049: OPPO)

Proposal 4: NW need to bar the legacy UE from accessing the on-demand SIB1 cell (e.g. based on the existing barring mechanism).

- Agreed.

[CATT]: Ok with the proposal, however to use ssb-SubcarrierOffset needs RAN1 confirmation.

Proposal 5 (modified): How to avoid/deprioritize the legacy UE camping at the Cell A attempting to switch to the NES Cell (but allowing the R19 NES UE to do that).

- Agreed.

[OPPO]: Clarified that intention is to set different cell reselection priority to legacy UE and R19 NES UE. [Xiaomi]: First we need to understand whether NES cell dedicated carrier is realistic scenario or not. [Huawei]: Agree with Xiaomi. Understand NES cell is per cell turn-on and off. [OPPO]: There is no restriction for NES cell dedicated carrier. [Apple]: Even with mixed carrier case, assume the number of macro cell is small on that carrier, so still to deprioritize the carrier makes a sense. [Samsung]: For the NES cell dedicated carrier, why not the existing UE dedicated cell reselection priority work? For the mixed carrier case, we can utilize the black cell list for the legacy UE. [Vivo]: Similar discussion happened for the dedicated cell reselection priority in R18, it was not pursued. [Rakuten]: If NES cell turns on, question is whether we consider it normal cell for legacy UE or not. [Session chair]: We’ve just agreed NW need to bar the legacy UE. [OPPO]: Regardless of barring, the intention is to deprioritize the NES carrier, which doesn’t mean the legacy UE will never attempt that carrier. [Nokia]: Clarified intention is not aligned with P5 itself. Think P5 itself is ok to companies.

- Barring relaxation for R19 NES UE (P8: R2-2404634: Apple)

Proposal 8 (modified): If the NES UE is unable to acquire the SIB1 of NES Cell before UE initiates OD-SIB1 procedure, it will not consider it as barred at that moment. R19 NES UE bars the cell if the UE fails to acquire SIB1 via on-demand SIB1 for NES cell.

- Agreed.

[Nokia]: Intention is correct, but in the current specification, we don’t specify how long the UE attempts to acquire SIB1. We may not need anything in the spec in the stage-3. [Fujitsu]: Agree with the proposal. In addition, the UE first should know whether that cell is NES cell or not. [Lenovo]: Agree with Nokia.

- Paging and SIB1 update in NES cell (P7: R2-2404634: Apple)

Proposal 7 (modified): After Rel-19 NES UEs camp in NES cell, the UE behaviour is same as the one defined as legacy normal camped state, e.g. paging reception, SIB1 update, etc.

- Agreed.

- RAR monitoring for MSG1 based OD-SIB1 REQ (P2: R2-2405275: NEC)

Proposal 2 (modified): RAN2 assumes the UE is expected to receive the RAR responding to the preamble transmission for Msg1-based on-demand SIB1 procedure, as the baseline.

- Agreed.

[Vivo]: SIB1 request is common to all UEs, what’s the real use of RAR for SIB1? [Nokia]: Without RAR, how long the UE wait for SIB1? Rather than introducing new mechanism (e.g. a new timer-based solution), it’s good to count on what we already have. [Fraunhofer]: Without RAR, we can save signalling overhead in Cell A (because WUS configuration does not need to include a configuration to receive RAR). [Apple]: RAN1 discusses a need of UE group common signalling. From RAN2 point of view, we support the proposal. [Intel]: Agree with the proposal, if RAN1 introduces something new for their world, we can revisit it. Otherwise we need to keep the proposal. [Sony]: It is good to design both SIB1 and other SIB in common as much as possible. Support the proposal.

- UE behaviour on RACH failure of OD-SIB1 REQ (P12: R2-2404634: Apple)

Proposal 12 (modified): As baseline, upon random access procedure failure of OD-SIB1 request, UE regards OD-SIB1 can’t be acquired in the NES cell and considers it as barred. It doesn’t exclude the option to leave the determination to the UE implementation.

- Agreed.

[Fujitsu]: Ok with the proposal, but want to understand what “RACH failure of OD-SIB1 request” means exactly? [Apple]: Same as OD-SIB REQ procedure. [LG]: In the current spec, we don’t specify how long the UE continues to acquire SIB1. If RACH fails, what’s the problem if the UE retries RACH again? [Nokia]: Ok with the proposal, but nothing may not need to be further specified. [Xiaomi]: Prefer leaving it to UE implementation. [Samsung]: “RACH failure” doesn’t mean just one preamble transmission failure, understand it means a whole random access procedure failure. [Apple]: Confirms Samsung understanding.

- SIB1 Validity (P8: R2-2405619: Huawei)

Proposal 8: SIB1 validity is supported, i.e. if a UE has a valid version of SIB1 stored, UE does not initiate the on-demand SIB1.

- Postponed.

[Samsung]: Question is how it really works. If it operates based on timer only, the NW may not be able to update the SIB1 (e.g. NW does not know when a UE acquires SIB1, so the NW does not know when 3 hours will end). If timer + value tag, where is the value tag sent? [Huawei]: Intention is to agree with the principle and to discuss details later. [Intel]: Ok with the proposal, but we need further discussion on many FFS. However, we don’t want to include value tag of SIB1 into MIB. [Samsung]: Understands the acquired SIB1 is valid until the reception of SIB change notification. Don’t understand why it is not enough and we attempt to change the UE behaviour.

2. Case 3 in RAN1 (P1: R2-2404226: Samsung)

Proposal 1 (modified): RAN2 to consider the scenario where SIB1 of Cell B can be directly obtained from Cell A; Cell A provides SIB1 of Cell B as a container in a new SIB (say SIB X of Cell A); UE requests for SIB X using the legacy SI request mechanism.

- Postponed.

[Fraunhofer]: Ok with the proposal. In addition, we need to consider how to reduce the signalling overhead. [Nokia, Ericsson]: Case 3 requires quite close coordination between two cells in order to guarantee the latest SIB1. [Ericsson]: Once the UE gets SIB1 from cell A and camps on NES cell, if the UE requires OD-SIB1 again for the camped NES cell, will the UE back to cell A and request it again? [Samsung]: Once camped on the NES cell, the SIB1 will be maintained by SIB change notification, so the scenario Ericsson mentioned will not happen.

- Once the NES UE camps on the NES cell, if the UE receives SIB change notification, the UE is expected to receive SIB1 from NES cell.

3. Scenario 3 (case 1 in RAN1) (P1: R2-2405619: Huawei)

Proposal 1: RAN2 to wait for RAN1’s progress whether to support scenario 3.

- Agreed.

[Lenovo]: There may be some RAN2 impact, e.g. cell selection perspective. [Nokia]: Think operator don’t want to make a coverage hole for the legacy UEs with the standalone scenario. Think there should be no big issue for cell selection. [Session chair]: Understand the following steps (SIB1 REQ to NES cell and SIB1 reception from NES cell) would be similar to the scenario 1a. Main outstanding difference is how the UE knows WUS configuration, which is under RAN1 discussion. It is not easy to discuss RAN2 aspect without RAN1 progress. Let’s check companies’ views.

Ok with proposal 1?

- No: Fraunhofer, Lenovo

- Yes: All other companies

Agreements on on-demand SIB1:

Further clarification

- Study on-demand SIB1 provisioning for NES Cell(s) in versions of Scenario 1a with multiple Cells A and/or NES Cells:

- More than one Cell A may provide configuration for the same NES cell.

- The same Cell A may assist more than one NES Cells.

- RRC release message assisted intra-cell WUS can be discussed as option of signaling details in stage 3.

Information of WUS configuration

- Can use the PCI and frequency of a NES Cell to associate the UL WUS configuration with a NES Cell.

- For Message 1 based on-demand SIB1 request, the on-demand SI request configuration that currently included in SIB1 may be used as the design baseline.

Transmission of WUS configuration

- Cell A’s SIB can be used to configure on-demand SIB1 related configuration for neighbour NES cells, e.g., via new SIB or the existing SIB.

Triggering condition of WUS transmission

- If the UE chooses the NES cell using legacy intra-F/inter-F cell re-selection procedure (as baseline), the UE triggers WUS transmission.

- After UE successfully receives OD-SIB1 for that NES Cell and if it is a suitable cell, UE camps in the NES Cell “similar” to a legacy Cell.

UE request SIB1 to perform initial access in NES cell

- RAN2 not to support on-demand SIB1 request that is combined with an initial access to perform RRC connection establishment/resume on the NES cell.

Legacy UE impact

- NW need to bar the legacy UE from accessing the on-demand SIB1 cell (e.g. based on the existing barring mechanism)

- How to avoid/deprioritize the legacy UE camping at the Cell A attempting to switch to the NES Cell (but allowing the R19 NES UE to do that).

Barring for R19 NES UE

- If the NES UE is unable to acquire the SIB1 of NES Cell before UE initiates OD-SIB1 procedure, it will not consider it as barred at that moment. R19 NES UE bars the cell if the UE fails to acquire SIB1 via on-demand SIB1 for NES cell.

Paging and SIB1 update in NES cell

- After Rel-19 NES UEs camp in NES cell, the UE behaviour is same as the one defined as legacy normal camped state, e.g. paging reception, SIB1 update, etc.

RAR monitoring for MSG1 based OD-SIB1 REQ

- RAN2 assumes the UE is expected to receive the RAR responding to the preamble transmission for Msg1-based on-demand SIB1 procedure, as the baseline.

UE behaviour on RACH failure of OD-SIB1 REQ

- As baseline, upon random access procedure failure of OD-SIB1 request, UE regards OD-SIB1 can’t be acquired in the NES cell and considers it as barred. It doesn’t exclude the option to leave the determination to the UE implementation.

SIB1 acquisition upon SIB change notification in NES cell

- Once the NES UE camps on the NES cell, if the UE receives SIB change notification, the UE is expected to receive SIB1 from NES cell.

Scenario 3 (case 1 in RAN1)

- RAN2 to wait for RAN1’s progress whether to support scenario 3.

R2-2404153	Discussion on on-demand SIB1	Xiaomi	discussion	Rel-19

R2-2404213	Discussion on on-demand SIB1 for NES	Ericsson	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2404226	On-demand SIB1	Samsung	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2404262	UE behaviour when (re)selecting to a NES Cell including further solution details and scenarios to support OD-SIB1	Intel Corporation	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2404449	Discussion on on-demand SIB1 transmission for network energy savings	Fujitsu Limited..	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2404458	Scenarios, configuration, and camping	Lenovo	discussion	Netw_Energy_NR_enh-Core

R2-2404565	Discussion on scenarios and procedure of on-demand SIB1	HONOR	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2404634	Further discussion on on-demand SIB1	Apple	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2404829	Discussion on on-demand SIB1 for NES	Rakuten Mobile, Inc	discussion	Rel-19

R2-2404858	Further consideration on on-demand SIB1 in idle and inactive mode	ZTE Corporation, Sanechips	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2404886	Discussion on the scenarios for on-demand SIB1	Google Inc.	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2404894	Discussion on  on-demand SIB1 for RRC IDLE and INACTIVE UE	vivo	discussion	Rel-19

R2-2404910	UL WUS for on-demand SIB1	Sony	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2404911	On-demand SIB1 for IDLE/INACTIVE UEs	Sony	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2404950	Consideration on on-demandSIB1 issues	CATT	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2405035	Discussion on on-demand SIB1	CMCC	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2405049	Consideration on on-demand SIB1	OPPO	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2405136	On-demand SIB1 request and reception	InterDigital	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2405226	On-demand transmission of SIB1	LG Electronics Inc.	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2405275	Discussion on On-demand SIB1 and RACH handling	NEC Telecom MODUS Ltd.	discussion

R2-2405295	Discussion on On-demand SIB1	Qualcomm Incorporated	discussion

R2-2405311	On-demand SIB1 for UEs in idle/inactive mode	China Telecom	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2405356	Discussion on on-demand SIB1	Sharp	discussion

R2-2405552	Discussion on on-demand SIB1 for NES	CEWiT	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2405567	On demand SIB1 handling	Nokia	discussion	Rel-18	Netw_Energy_NR_enh-Core

R2-2405611	On-demand SIB1 for NES	Fraunhofer IIS	discussion	Rel-19

R2-2405619	Discussion on on-demand SIB1 operation for NES	Huawei, HiSilicon	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2405640	On-demand SIB1 for Idle/Inactive mode UEs	III	discussion	Netw_Energy_NR_enh

### 8.5.4	Adaptation of common signal/channel transmissions

Further consideration of adapation of paging occasions in time domain, legacy UE impact (including barring aspect for paging adaptation), configuration aspect for paging adaptation, RAN2 spec impact and solutions for RACH adaptation (with consideration of RAN1 progress, note study of RACH adaptation in spatial domain needs to be concluded), etc.

1. RACH adaptation in spatial domain (P8: R2-2404577: Xiaomi)

Proposal 8 (modified): RAN2 counts on RAN1 evaluation and conclusion

- Agreed.

[Apple]: It is good to simply conclude RAN2 counts on RAN1 evaluation and conclusion.

RACH adaptation in spatial domain

- RAN2 counts on RAN1 evaluation and conclusion.

R2-2404183	Discussion on adaptation of common signal/channel transmissions	OPPO	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2404228	Adaptation of common signal/channel transmissions	Samsung	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2404263	RAN2 impacts to enable adaptation of paging and RACH in time	Intel Corporation	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2404349	Adaptation of common signal or channel	Fujitsu	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2404566	Discussion on adaptation of common signal/channel transmissions	HONOR	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2404577	Discussion on common signal adaptation	Xiaomi	discussion

R2-2404635	Further discussion on RAN2 work of common signal transmission adaptation	Apple	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2404822	Paging and PRACH adaptation for NES operation	Lenovo	discussion	Rel-19

R2-2404851	Discussion on the paging occasion adaptation	ITRI	discussion	Netw_Energy_NR_enh-Core

R2-2404859	Further consideration on paging occasion adaptation	ZTE Corporation, Sanechips	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2404895	Discussion on adaptation on common signal transmissions	vivo	discussion	Rel-19

R2-2404932	Discussion on adaptation of common signal/channel transmissions	Spreadtrum Communications	discussion	Rel-19

R2-2404951	Consideration on adaptation of common signalchannel transmissions	CATT	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2405036	Discussion on adaptation of common signalchannel transmissions	CMCC	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2405123	Discussion on adaptation of common signal/channels transmissions	Huawei, HiSilicon	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2405137	Time domain adaptation of common signalling and channels	InterDigital	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2405276	Paging enhancements and legacy UE barring	NEC Telecom MODUS Ltd.	discussion

R2-2405290	Adaptation of common signal/channel transmissions for NES	Ericsson	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2405296	Discussion on Adaptation of Common Signal/Channel Transmissions	Qualcomm Incorporated	discussion

R2-2405428	Discussion on paging adaptation	ASUSTeK	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2405568	Common signal aspects of NES WI	Nokia	discussion	Rel-18	Netw_Energy_NR_enh-Core

R2-2405576	Discussion on adaptation of paging signal/channel 	III	discussion

R2-2405621	Adaptation of Common Signals and Channels for NES	Fraunhofer IIS	discussion	Rel-19

R2-2405694	Discussion on common signal and channel adaptation	LG Electronics Inc.	discussion	Rel-19	Netw_Energy_NR_enh

## 8.6	Mobility Enhancement Ph4

(NR_Mob_Ph4-Core; leading WG: RAN2; REL-19; WID: RP-240299)

Time budget: 2 TU

Tdoc Limitation: 2 tdocs

### 8.6.1	Organizational

LS, Rapporteur input, including workplan, etc.

### 8.6.2	Inter-CU LTM

Any further consideration on scenarios/use cases, signalling flows, spec impacts and solutions in LTM preparation phase (e.g. to what extend the RRC procedure, RRC modeling, reference configuration of R18 can be reused, etc.), early sync phase, and LTM cell switch execution phase, any further consideration on security key handling, and details on subsequent inter-CU LTM and subsequent intra-CU LTM after an inter-CU or intra-CU LTM switch, etc.

1. Scenarios

- Clarification of DC (P1: R2-2404464: Ericsson)

Proposal 1: An LTM configuration with inter-CU LTM candidate cells can be configured either by the MCG or SCG (but not for both simultaneously) and it is up to the network to handle this (further details up to RAN3, if any). No restriction for intra-CU LTM candidate cells.

- Agreed.

[CMCC]: RAN3 has already agreed with P1 last meeting. [CATT]: If configuration includes mixture of intra-CU and inter-CU LTM candidate cells? At least inter-CU LTM candidate cell is included, it is configured either by MCG or SCG. [Huawei]: No stage 3 RAN2 impact from P1? [Ericsson]: Confirms no. [ZTE]: Understand if MN configures both inter-CU and intra-CU LTM, it would impact SN. However, if SN configures inter-CU, it would not impact on MN.

- Xn-based and N2-based inter-CU LTM (P1: R2-2404421: Vivo)

Proposal 1 (modified): Xn-based inter-CU LTM is prioritized in Rel-19.

- Agreed.

2. Stage-2 signaling flows and procedures

- Stage-2 signaling flows (Figure 2, P2-P4: R2-2405062: ZTE)

Proposal 2 (modified): The preparation of inter-CU LTM configuration is initiated by the source gNB-CU.

- Agreed.

[Ericsson]: For the figure 2, it is still under discussion whether one message is used for each candidate cell or for each candidate CU.

Proposal 3: For each candidate cell, the preparation of lower layer configuration is initiated by the candidate gNB-CU, based on the LTM request from the source gNB-CU. RAN2 assumes the interaction between the candidate gNB-CU and candidate gNB-DU follows the same signaling procedure for intra-CU LTM.

- Agreed.

Proposal 4: The source gNB-CU is responsible to collect the configurations and information of candidate cells from multiple candidate gNB-CUs and generates the common CSI resource configuration for L1 measurement on candidate cells.

- Agreed.

Proposal 5 (modified): In order to support subsequent LTM, the source gNB-CU needs to inform the candidate gNB-CU(s) about the common CSI resource configuration and the collected information of candidate cells from multiple candidate gNB-CUs. The candidate gNB-CU(s) responds with the candidate configuration to the source gNB-CU accordingly (if needed).

- Agreed.

3. Preparation

- RRC Configuration/structure (P9: R2-2405062: ZTE)

Proposal 9: The RRC signalling structure and modelling for Rel-18 LTM is taken as the baseline for inter-CU LTM.

- Agreed.

- LTM Candidate ID (P5: R2-2404165: CATT)

Proposal 5: For inter-CU LTM, LTM candidate ID is unique across all the participating gNB-CUs.

- Agreed.

- Max number of LTM candidate IDs (P2: R2-2404464: Ericsson)

Proposal 2: The maximum number of LTM candidate cell configuration is 8, regardless of whether these are intra-CU or inter-CU LTM candidate configurations.

- Agreed.

- Reference configuration

Single reference configuration as Rel-18 LTM (P3: R2-2404642: Apple, China Telecom)

Proposal 3: A single reference config is used for inter-CU LTM, and this reference configuration is the same as Rel-18 reference configuration. RAN2 expects additions to the Rel-18 reference configuration structure for Rel-19 LTM.

Multiple reference configuration, e.g. a reference configuration per CU (P5: R2-2404826: LG)

Proposal 5. Multiple reference configurations are provided per CU for inter-CU LTM.

[Ericsson]: Single reference configuration sounds simpler, but each CU may have different capabilities or default configurations. We need to see each solution’s pros and cons. [Rakuten]: Considering one CU covers many DUs, two reference configurations should be enough. [MediaTek]: Considering max 8 candidate cells, one reference configuration should be enough. [Qualcomm]: It is not easy to determine which one is better in the signalling overhead. It will be different case by case. [KDDI]: We don’t need coordinate with multiple CUs if we have multiple reference configurations, e.g. one reference per CU. [Huawei]: Don’t agree with KDDI. Even with single configuration, the source CU doesn’t need the coordination with others. [Nokia]: With the mixture of subsequent inter-CU and intra-CU LTM, a reference configuration per CU will reduce the signalling overhead in the end.

4. Early sync

- RAR based option (P1: R2-2405037: CMCC, P5: R2-2404421: Vivo)

Proposal 1 (modified): RAR-based TA acquisition is not supported for inter-CU LTM for non-conditional LTM. FFS on conditional LTM.

- Proposal 1 is agreed.

Proposal 5: Early RACH with RAR is supported for inter-CU LTM.

[CATT, Lenovo]: If early TA acquisition is started earlier than 20ms, it doesn’t bring any problem. Note RAR based solution was discussed in R18 and it was excluded in the end. [Vodafone]: Where 20ms delay comes from? [Samsung]: If we introduce RAR based option for conditional LTM, are we going to block use of RAR based option for inter-CU LTM? [OPPO]: If RAR is received from the source-CU, it will anyway have 20ms delay. If RAR is received from the target-CU, it will interrupt the current data reception/transmission. Prefer no RAR based option. [LG]: Even for conditional LTM, we may survive w/o RAR based option. [Nokia, ASUSTeK]: Want to open the possibility to use RAR based TA acquisition for inter-CU LTM. [ZTE]: In RAN4, it was already defined the UE behaviour or requirement to transmit preamble w/o interruption of the serving’s data reception/transmission. With it, there would be no issue with RAR based option. [Apple]: It was only about preamble transmission, not about RAR reception. [Session chair]: Let’s see how companies are thinking.

Introduce RAR-based option (w/o consideration of conditional LTM)?

- Yes: Samsung, Vivo, Nokia, ZTE, ASUSTeK, Rakuten, ITRI, Honor

- No: Much more companies

- UE based TA measurement and early DL sync using CSI-RS (P8: R2-2404165: CATT)

Proposal 8: RAN2 sends LS to RAN1 to ask confirmation on the following issues for inter-CU LTM

‐	Whether to support UE based TA measurement

‐	Whether to support early DL sync using CSI-RS

- No LS to RAN1/4.

[Samsung]: No need to send LS to RAN1. It is clear RAN1 will be involved for CSI-RS. [CATT]: RAN1 is not included in the scope of inter-CU LTM. [Apple]: Company can submit contribution referring RAN2 decision w/o LS. [Lenovo]: UE based TA was introduced in Rel-18 and RAN4 is not included in the scope of inter-CU LTM. Better to send an LS to RAN4. [Xiaomi]: Support sending LS to RAN1 and RAN4. [OPPO, MediaTek]: RAN4 already defined UE requirement for UE based TA measurement. Think it is applied for both intra-CU and inter-CU. [NEC]: Is common RAN2 understanding same RAN4 requirements will be applied to inter-CU LTM? If so, it would be good to capture it here. [Apple]: Read WID carefully, it includes some related RAN4 job.

5. Execution

- LTM Cell Switch Command (P5: R2-2405221: Huawei)

Proposal 5 (modified): R18 LTM CSC MAC CE is baseline to trigger LTM cell switch for inter-CU LTM.

- Agreed.

[OPPO]: We are still waiting for SA3 response LS. We may need to put security key related information into MAC CE. [Xiaomi]: Do we need to also include CFRA resource? [Huawei]: CFRA resource aspect was discussed for inter-DU case in Rel-18, but it was concluded no special handling. Why do we need it now for inter-CU case? [Ericsson]: We can update the proposal in more general way, e.g. R18 LTM CSC MAC CE is baseline.. blabla..

- Support of mixture of inter-CU and intra-CU LTMs (P12: R2-2405221: Huawei, P13: R2-2405062: ZTE)

Proposal 12 (modified): Introduce a new RRC configured Rel-19 ID: if the Rel-19 ID is different for the source cell and the target cell, the UE performs PDCP re-establishment, including security key update.

Proposal 13: For the mixture of inter-CU and intra-CU LTM, if the security key update is not required, the NoResetID is reused to determine whether the RLC re-establishment and PDCP data recovery (for AM DRB) is required.

[CATT, Nokia]: Agree with proposal 12. [Xiaomi]: We should wait for SA3 response LS. [Qualcomm]: Agree with proposal 13. [Ericsson]: P13 is just to repeat how R18 intra-CU LTM works. No additional meaning for inter-CU LTM.

- Postponed.

6. LTM Cell switch completion

- RACH-less LTM (P7: R2-2405221: Huawei)

Proposal 7: Support CG-based RACH-less and DG-based RACH-less procedures for inter-CU LTM.

- Agreed.

[Huawei]: Notification is also needed for inter-CU LTM.

- Determination of the completion (P11: R2-2404416: OPPO)

Proposal 11: The LTM completion defined for intra-CU LTM is followed for R19 LTM.

- Agreed.

7. Inter-CU LTM w/o security key change (R2-2405469, P4: R2-2405316: China Telecom)

R2-2405469	Inter-gNB LTM and moving PDCP anchor	Qualcomm Incorporated, NTT DOCOMO, Sony	discussion

Proposal 1: Upon inter-gNB LTM cell switch, a UE may have RRC/DRBs connect over PDCP to a gNB-CU of one LTM candidate gNB using an RLC connection to a gNB-DU of another LTM candidate gNB.

[Qualcomm]: Confirms it is supplementary option in addition to the option we agreed (i.e. with the security key change). Confirms for this option, it is free lunch from RAN2 point of view (i.e. no RAN2 spec impact). [China Telecom]: It was discussed in RAN3 (i.e. whether it’s logical deployment scenario or not), it was considered not as a logical deployment scenario. Consider it is a corner case. [CATT]: With this option, there would be performance issue because of many interactions between nodes. Also considering we already have security key change based option, the motivation to have additional option is not clear. [Vodafone]: Understand a benefit of this solution is to avoid CN change for security key change. [Samsung]: With this option, in the figure, whenever blue DU want to change configuration, blue DU needs to connect to red CU, which would bring the more inter-node interactions. Note there is restriction each DU can only connect to one CU-CP. [Ericsson]: Understand there should be RAN2 spec impact. [Huawei]: It’s like UP relocation procedure in UMTS, which impacts RAN2. [Nokia]: If the serving cell is not a LTM candidate, it is not clear how it works.

- Noted.

Proposal 4: Regarding the security key handling for inter-CU LTM, RAN2 to focus on the solution for the basic scenarios of inter-CU LTM with both CU-CP and CU-UP change.

Agreements on inter-CU LTM:

Clarification of DC

- An LTM configuration with inter-CU LTM candidate cells can be configured either by the MCG or SCG (but not for both simultaneously) and it is up to the network to handle this (further details up to RAN3, if any). No restriction for intra-CU LTM candidate cells.

Xn-based and N2-based inter-CU LTM

- Xn-based inter-CU LTM is prioritized in Rel-19.

Stage-2 signaling flows and procedures

- The preparation of inter-CU LTM configuration is initiated by the source gNB-CU.

- For each candidate cell, the preparation of lower layer configuration is initiated by the candidate gNB-CU, based on the LTM request from the source gNB-CU. RAN2 assumes the interaction between the candidate gNB-CU and candidate gNB-DU follows the same signaling procedure for intra-CU LTM.

- The source gNB-CU is responsible to collect the configurations and information of candidate cells from multiple candidate gNB-CUs and generates the common CSI resource configuration for L1 measurement on candidate cells.

- In order to support subsequent LTM, the source gNB-CU needs to inform the candidate gNB-CU(s) about the common CSI resource configuration and the collected information of candidate cells from multiple candidate gNB-CUs. The candidate gNB-CU(s) responds with the candidate configuration to the source gNB-CU accordingly (if needed).

Preparation:

RRC Configuration/structure

- The RRC signalling structure and modelling for Rel-18 LTM is taken as the baseline for inter-CU LTM.

LTM Candidate ID

- For inter-CU LTM, LTM candidate ID is unique across all the participating gNB-CUs.

Max number of LTM candidate IDs

- The maximum number of LTM candidate cell configuration is 8, regardless of whether these are intra-CU or inter-CU LTM candidate configurations.

Early sync:

RAR based option

- RAR-based TA acquisition is not supported for inter-CU LTM for non-conditional LTM. FFS on conditional LTM.

Execution:

LTM Cell Switch Command

- R18 LTM CSC MAC CE is baseline to trigger LTM cell switch for inter-CU LTM.

LTM Cell switch completion

- Support CG-based RACH-less and DG-based RACH-less procedures for inter-CU LTM.

- The LTM completion defined for intra-CU LTM is followed for R19 LTM.

R2-2404641	Important topics for further progress of the WI	Apple, China Telecom (rapporteurs)	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2404165	Discussion on inter-CU LTM	CATT	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2404271	Supporting inter-CU LTM with intra-CU LTM	Intel Corporation	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2404296	Further discussion on Inter-CU LTM	MediaTek inc.	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2404337	Inter CU LTM Discussion in Predictive mobility Scenarios	Lekha Wireless Solutions	discussion	Rel-19	Late

R2-2404416	Discussion on inter-CU LTM	OPPO	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2404421	Discussion on inter-CU LTM	vivo	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2404464	Important aspects regarding inter-CU LTM	Ericsson	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2404563	Discussion on inter-CU LTM	HONOR	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2404608	Discussion on Inter-CU LTM	Xiaomi	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2404642	Discussion on Inter-CU LTM topics	Apple	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2404780	Discussion on inter-CU LTM 	Panasonic	discussion

R2-2404796	Discussions security update on inter-CU LTM	KDDI Corporation	discussion	Rel-19

R2-2404806	Discussion on Inter-CU LTM	Lenovo	discussion	Rel-19

R2-2404826	Discussion on inter-CU LTM	LG Electronics	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2404835	Radio Resource aspects for intra-CU and inter-CU LTM	Rakuten Mobile, Inc	discussion	Rel-19

R2-2404836	Initial considerations for inter-CU LTM	Rakuten Mobile, Inc	discussion	Rel-19

R2-2404852	Discussion on subsequent inter-CU LTM	ITRI	discussion	NR_Mob_Ph4-Core

R2-2404912	LTM for Inter-CU	Sony	discussion	Rel-19	NR_Mob_Ph4

R2-2404921	Discussion on inter-CU LTM	NEC	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2404923	Discussion on Inter-CU LTM	Spreadtrum Communications	discussion	Rel-19

R2-2404980	Discussion on Inter-CU LTM	KT Corp.	discussion

R2-2404984	Further discussion on Inter-CU LTM cell switch	Transsion Holdings	discussion	Rel-19

R2-2405037	Discussion on Inter-CU LTM	CMCC	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2405062	Discussion on inter-CU LTM	ZTE Corporation	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2405111	Discussion on Inter-CU LTM	Interdigital, Inc.	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2405163	On inter-CU aspects for LTM	Nokia	discussion	Rel-19	NR_Mob_Ph4	Withdrawn

R2-2405221	Inter-CU LTM	Huawei, HiSilicon	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2405316	Discussion on Inter-CU LTM	China Telecom	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2405362	Potential issues on inter-CU LTM 	Kyocera 	discussion	Rel-19	R2-2403422

R2-2405391	Further Considerations to Support Inter-CU LTM	Samsung	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2405460	Discussion on security and procedures for inter-gNB LTM	Qualcomm Incorporated	discussion

R2-2405519	Discussion on inter-CU LTM	ITL	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2405553	LTM Enhancements for Inter-CU mobility	CEWiT	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2405588	Discussion on Inter-CU LTM	ETRI	discussion	Rel-19

R2-2405620	Discussion on issues for supporting inter-CU LTM	Sharp	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2405666	On inter-CU aspects for LTM	Nokia	discussion	Rel-19

### 8.6.3	Measurement enhancements for LTM

Details of event-triggered L1 measurement reporting including configuration aspect, beam level and/or cell level measurement?, event definitions, need of L1 filtering and timeToTrigger, Hys, etc.

1. Use cases (P1: R2-2404677: Apple)

Proposal 1: Event triggered L1 measurement should be designed for the following LTM purposes:

- Select the candidate beam/cell to trigger early synchronization.

- Select the target beam/cell and trigger LTM cell switch procedure.

- Agreed.

[Xiaomi]: Purpose of early synchronization is to perform LTM cell switch. Not sure if we need a) in separate in addition to b). [MediaTek]: Agree with intention, but no need to be specified. [CATT]: Support the proposals. Regardless of the relation between a) and b), we need to understand how to use measurement reporting event and it will help to make decision to define events. [Huawei]: Agree with the intention. [Nokia, Ericsson]: Agree with proposals.

2. Beam or cell level measurement

Beam level measurement (P5: R2-2404677: Apple)

Proposal 5 (modified): For event triggered L1 measurement, use of beam level measurement result for event evaluation is baseline. FFS for the cell level measurement.

- Agreed.

Cell level measurement (P3: R2-2405063: ZTE)

Proposal 3: For the input of the L1 event evaluation, RAN2 to consider both beam-level measurement result and cell-level measurement result (e.g. based on the consolidation of multiple beams of a cell).

[Huawei, LG, CATT, ZTE, IDC]: Good to have beam-level measurement as baseline, but may be not good to exclude cell level measurement. Otherwise it may bring ping-pong between two CUs. [CATT]: We can agree with beam level measurement as baseline, but we need to open cell level measurement. [ZTE]: Without cell level measurement, the frequent measurement report can happen. We need to discuss what beam(s) will be used for events. [Nokia]: We need to have common understanding on cell-level measurement. Think it’s not L3 filtered measured result. [Huawei, Ericsson]: We can further discuss how beams will be consolidated.

3. Event definition (P6-P8: R2-2404677: Apple)

Proposal 6 (modified): Support the following LTM events based on beam specific quality of serving cell and candidate cells as the L1 LTM measurement events.

-	Event LTM2: Beam of serving cell becomes worse than absolute threshold;

-	Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell;

-	Event LTM4: Beam of candidate cell becomes better than absolute threshold;

-	Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2.

FFS on what beam(s) of the serving cell and neighboring cell is used for event evaluation. FFS on the need of Event LTM1.

- Agreed.

[IDC]: Agree with A3, A4 and A5, but wonder what’s use case of A1 and A2. [Apple]: A2 + A4 can be used for LTM switch and A1 can be used to stop LTM switch. [ZTE]: RAN1 discuss whether to activate/deactivate LTM measurement resources, A2 can be helpful. [NTTDCM]: Also think A2 is helpful. Even A2 + A3 is useful. [Nokia]: Wonder if single beam is used for event evaluation or if multi beams can be used for event evaluation? [LG]: Think A2 + Leaving can be used. [Ericsson]: Support the proposals. [Huawei]: A1 is used to trigger A5 measurement configuration. [Xiaomi]: Shouldn’t the serving cell be changed to SPCell?

Proposal 8 (modified): For beam specific Ax events in P6, the beam corresponding to the indicated TCI state of serving cell is used for the event evaluation.

- Postponed.

[Apple]: For MIMO, RAN1 uses the current beam for evaluation. [Vivo]: RAN1 uses the beam corresponding to the indicated TCI state. [LG]: Understand the beam corresponding to the activated TCI state is still open in RAN1. [OPPO]: Unified TCI state is applied, so we don’t need to restrict it to PDSCH reception. [CATT]: Understand the best beam can be used (instead of the current beam). [Apple]: NW will always try to select the best beam as the beam corresponding to the indicated TCI state. We don’t need to consider the best beam in separate. [ZTE]: For MIMO, it is to select the beam. For mobility, we may need to consider the best beam.

4. Configuration

- Measurement RS (P2-P3: R2-2404677: Apple)

Proposal 2: For event triggered L1 measurement, the L1 measurement resource configuration does not include the resource of serving cell.

Proposal 2a: UE derives the RS resource for current beam of serving cell implicitly from a QCL RS of indicated TCI state.

Proposal 3: Support the beam config of both SSB and CSI-RS in L1 measurement resource configuration in LTM config. Working assumption: Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5.

- Agreed.

[ZTE]: Support the proposal and if possible, we can agree with that same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5. [Samsung]: Assuming we use the current beam corresponding to the indicated TCI state, it can dynamically change RS type. Then how to guarantee always same RS type can be used between the serving cell and neighbouring cell? [Ericsson]: Similar discussion happened in the past and there was a way to achieve it. We can discuss more details in the stage-3.

- L1 Filtering (P5: R2-2404166: CATT)

Proposal 5: RAN2 assumes filtering of the L1 measure results is needed. It’s up to RAN1 whether the specified L1 filtering is needed or ok to leave it to UE implementation.

- Agreed.

[CATT, Apple]: Still good to agree with proposal 5, and let RAN1 decide whether it will be specified or not. [MediaTek]: Suggest to send LS to RAN1 to inform it. [Ericsson, CMCC]: Support the proposal, but think no need of the LS to RAN1. [LG]: Agree with P5.

- TTT, Hysteresis, Offset (P5: R2-2405610: KDDI)

Proposal 5: To avoid “Ping Pong” cell switch in LTM, when defining the events for triggering LTM L1 measurement report, the following aspects could be considered:

- hysteresis, beam specific offset, candidate cell specific offset;

- entering condition, leaving condition and time-to-trigger.

For LTM event evaluation,

Whether TTT is applied?

- Yes

Whether hysteresis for entering/leaving is applied?

- Yes

[ZTE]: hysteresis is per beam or per cell? [Nokia, LG]: Understand it’s per event.

Whether beam specific (FFS for cell specific) offset is applied?

- Agreed.

FFS on the need of measurement reporting once leaving condition is met.

- Agreed.

[LG]: For certain cell, we have event specific offset in R18. We can further consider it.

5. Contents in measurement report (P10: R2-2404166: CATT)

Proposal 10: As baseline, the following information can be included in the content of the event-triggered measure results,

‐Beam measurement results

‐Beam information (e.g., SSB index, candidate configuration id), explicitly or implicitly

‐Information of the triggered event, explicitly or implicitly

6. MAC CE or UCI for measurement reporting?

MAC CE (P6: R2-2404463: Ericsson, T-Mobile)

Proposal 6: Event-triggered L1-measurements are reported by the UE to the network via a MAC CE.

UCI (P14: R2-2405492: LG)

Proposal 14: RAN2 to consider transmission procedure adopted for UE-initiated/event-driven beam reporting in R19 MIMO as baseline for event-triggered L1 report for LTM. That is, Mode A (dynamically scheduling UCI by gNB) and Mode B (UCI in pre-configured resource(s) for second UL channel) are supported.

Agreements on measurement enhancements for LTM:

- Event triggered L1 measurement should be designed for the following LTM purposes:

- Select the candidate beam/cell to trigger early synchronization.

- Select the target beam/cell and trigger LTM cell switch procedure.

2. 	For event triggered L1 measurement, use of beam level measurement result for event evaluation is baseline. FFS for the cell level measurement.

3.	Support the following LTM events based on beam specific quality of serving cell and candidate cells as the L1 LTM measurement events.

-	Event LTM2: Beam of serving cell becomes worse than absolute threshold;

-	Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell;

-	Event LTM4: Beam of candidate cell becomes better than absolute threshold;

-	Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2.

FFS on what beam(s) of the serving cell and neighboring cell is used for event evaluation. FFS on the need of Event LTM1.

4.	Support the beam config of both SSB and CSI-RS in L1 measurement resource configuration in LTM config. Working assumption: Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5.

5.	RAN2 assumes filtering of the L1 measure results is needed. It’s up to RAN1 whether the specified L1 filtering is needed or ok to leave it to UE implementation.

6.	For LTM event evaluation, TTT, hysteresis for entering/leaving, and/or beam specific (FFS for cell specific) offset can be applied. FFS on the need of measurement reporting once leaving condition is met.

R2-2404166	Event-triggered L1 measurement reporting	CATT	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2404297	Discussion on event-triggered L1 measurement reporting	MediaTek inc.	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2404350	Measurement enhancements for LTM	Fujitsu	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2404417	Discussion on event-triggered L1 measurement reporting	OPPO	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2404422	Discussion on measurement enhancement for LTM	vivo	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2404457	L1 Measurement enhancements	Lenovo	discussion	NR_Mob_Ph4-Core

R2-2404463	Important aspects regarding event triggered L1 measurements	Ericsson, T-Mobile USA	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2404564	Discussion on L1 measurement enhancements	HONOR	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2404677	Measurement enhancements for LTM	Apple	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2404779	Event-Triggered L1 Report for LTM	Huawei, HiSilicon	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2404924	Discussion on measurement enhancements for LTM	Spreadtrum Communications	discussion	Rel-19

R2-2404985	Discussion on measurement enhancement for LTM	Transsion Holdings	discussion	Rel-19

R2-2405014	Discussion on LTM measurement related enhancements	CMCC	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2405063	Discussion on event-triggered L1 measurement reporting	ZTE Corporation	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2405077	Discussion on event triggered L1 measurement reporting	NEC	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2405112	Event triggered L1 reporting for LTM	Interdigital, Inc.	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2405149	On Measurement-related Enhancements for Rel-19 LTM	Nokia	discussion	Rel-19	NR_Mob_Ph4	R2-2403305

R2-2405317	Discussion on event-triggered L1 measurement reporting for LTM	China Telecom	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2405385	Consideration on event-triggered L1 measurement reporting 	Kyocera 	discussion	Rel-19	R2-2403423

R2-2405392	Support of Event-Triggered L1 Measurement Enhancements for LTM	Samsung	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2405487	Discussion on event-triggered L1 measurement reporting	Xiaomi	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2405492	Measurement related enhancements for LTM	LG Electronics Inc.	discussion	NR_Mob_Ph4-Core

R2-2405522	Discussion on measurement enhancements for LTM	ITL	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2405607	Proposals for event triggered L1 measurement report	Sharp	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2405610	Discussion on event triggered L1 measurement reporting	KDDI Corporation	discussion	Rel-19

## 8.7	XR Enhancements Ph3

(NR_XR_Ph3-Core; leading WG: RAN2; REL-19; WID: RP-240791)

Time budget: 2 TU

Tdoc Limitation: 4 tdocs

### 8.7.1	Organizational

LS, Rapporteur input, including workplan, etc.

R2-2404288	XR Workplan	Nokia, Qualcomm (Rapporteurs)	Work Plan	Rel-19	NR_XR_Ph3-Core

- Rapporteur suggests we start work on RRM gaps in October.

Noted

R2-2404289	XR Agreements	Nokia, Qualcomm (Rapporteurs)	discussion	Rel-19	NR_XR_Ph3-Core

Noted

R2-2404290	SA2 Overview	Nokia, Qualcomm (Rapporteurs)	discussion	Rel-19	NR_XR_Ph3-Core

Noted

R2-2404291	RAN3 Overview	Nokia, Qualcomm (Rapporteurs)	discussion	Rel-19	NR_XR_Ph3-Core

Noted

R2-2404138	LS on Application-Layer FEC Awareness at RAN (S2-2405604; contact: Qualcomm)	SA2	LS in	Rel-19	FS_XRM_Ph2	To:RAN2, SA4	Cc:RAN3

- RAN2 is expected to answer the question on PSI as well.

- Lenovo thinks this is more in the expertise of SA4. LGE, Nokia, MTK agrees.

- OPPO thinks that for DL there is no RAN2 impact, if UL is in scope, then we might need to reply.

- Vivo thinks we may need to reply in case we think UL is affected.

- Huawei think it is better to leave this to SA4.

- Intel thinks we should indicate we only use two levels of PSI and we can indicate this.

RAN2 thinks the question on using PSI as PDU set ratio is in SA4 scope at least for DL.

R2-2404139	LS on FS_XRM Ph2 (S2-2405625; contact: vivo)	SA2	LS in	Rel-18	FS_XRM_Ph2	To:SA4, RAN2, RAN3

Noted

#### 8.7.1.1	Discussion on LSs from SA2

Discussion on RAN2 replies to SA2 LS on FS_XRM Ph2 (S2-2405625) and SA2 LS on Application-Layer FEC Awareness at RAN (S2-2405604)

Draft reply LSes

R2-2404174	Draft reply LS on FS_XRM PH2	Qualcomm Incorporated	LS out	Rel-19	NR_XR_Ph3-Core	To:SA2; Cc:SA4, RAN3

R2-2404175	Reply LS on application-layer FEC awareness at RAN	Qualcomm Incorporated	LS out	Rel-19	NR_XR_Ph3-Core	To:SA2; Cc:SA4, RAN3

R2-2404424	Draft reply LS to SA2 on FS_XRM PH2	vivo	LS out	Rel-19	NR_XR_Ph3-Core	To:SA2; Cc:RAN3, SA4

R2-2405301	Draft Reply LS to SA2 on XR	CMCC	LS out	Rel-19	NR_XR_Ph3-Core	To:SA2

Discussion on XRM Phase 2 LS

R2-2404423	Discussion on LS from SA2 on FS_XRM Ph2	vivo	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 1: RAN2 understands adding inter-PDU set correlation information would assist RAN making PDU set discarding decision as comparing to the existing Rel-18 PDU Set discarding with some complexity.

Proposal 2: It is feasible for the NG-RAN to provide available data rate for the (non-)GBR QoS flows. Detail is up to RAN3.

Proposal 3: RAN2 understands the size of incoming burst is not useful for RAN resource scheduling.

Proposal 4: Regarding PDU Set Delay measurement and exposure, RAN2 understands the Alt 1 and Alt 2-1 are feasible and have no impact on the Uu interface, while Alt 2-2 has some impacts on the UE behaviour and Uu interface.

-	Alt1: DL PDU Set Delay based on T2 minus T1

-	Alt2: PDU Set Delay = Tend_N – T1_i

	Alt2-1: Tend_N is the sending of the last PDU of the PDU Set to the UE

	Alt2-2: Tend_N is the reception time of the last PDU of the PDU Set at the UE.

Proposal 5: Regarding PDU Set Loss Rate measurement and exposure, RAN2 understands it is feasible only if PSIHI is indicated and RLC AM mode is configured. In case RLC UM mode is configured, it is not feasible to measure and expose the PDU Set Loss Rate. If PSIHI is not configured, it is up to SA4 to decide.

Proposal 6: In the reply LS to SA2, RAN2 should inform SA2 that: if any of the above topics impacts RAN, e.g. on UE behavour or Uu interface, SA should inform RAN and RAN WG to extend the corresponding Rel-19 XR WID in RAN.

DISCUSSION on P1:

- Xiaomi thinks it is more to SA4 to discuss this solution.

- QCM thinks SA4 can decide whether this is useful, RAN2 can discuss RAN2 impacts.

- Ericsson does not think this is useful.

- Intel thinks that would be helpful, we do not have to mention complexity.

- Huawei thinks this is related to RAN2, this is old issue discussed in Rel-18. There is complexity and impact on PSER. We should mention complexity and impact on PSER.

- LGE is not sure whether this is useful, but considering this is for DL this can be up to gNB implementation.

- Nokia thinks it is beneficial to avoid sending useless PDU sets. We can mention complexity. App layer issues should be up to SA4. Lenovo agrees.

- Mediatek thinks it is unclear whether this is useful.

- Apple asks whether there are RAN2 impacts if we just focus on DL?

- CMCC thinks we can study both UL and DL. CMCC thinks the correlation information can also be used for scheduling.

- Smasung agrees there is no spec impact for DL. Samsung would prefer not to mention complexity.

RAN2 thinks adding inter-PDU set correlation information can potentially help RAN to avoid sending of unnecessary PDU sets provided that there is correlation between the discarded PDU sets.

RAN2 thinks it is up to SA4 to reply whether such correlation truly exists.

RAN2 thinks there is additional complexity with the solution and impact on PSER.

DISCUSSION on P2:

- Vodafone wonders if we already have mechanism for this, e.g. L4S. Wonders how often it has to be done as data rate changes dynamically.

- CATT thinks it is RAN3 scope. QCM agrees. RAN2 can only provide data rate at granularity at DRB level, but not QoS flow basis.

- Samsung thinks this is RAN2 scope as we are responsible for scheduling. Samsung thinks network should be able to estimate data rate. Huawei agrees, there are already mechanisms which utilize such information so gNB has to be able to do it.

- OPPO thinks it is more RAN3 scope, similar to QNC, it impacts gNB only.

- Nokia wonders what is the available bit rate.

- Lenovo agrees to leave this up to RAN3.

- Apple agrees this is up to RAN3 whether this can be provided to app layer.

- Xiaomi agrees this is up to RAN3.

- LGE this can be totally up to RAN3.

RAN2 thinks that whether it can be estimated at QoS flow level is up to RAN3 to answer.

DISCUSSION on P3:

- Nokia thinks this is useful. Huawei agrees this is useful.

- QCM is not clear why this is useful.

- Samsung thinks this is for DL only but thinks this is not useful unless there is also arrival time.

- Ericsson thinks this is useful if it is provided up front. ZTE agrees.

- Intel thinks for UL that could also be useful. ZTE agrees.

- QCM indicates we should not impact latency.

RAN2 only discussed the question for DL only

RAN2 understands the size of incoming burst is useful for gNB resource scheduling if it can be provided early enough, e.g. in the first packet of the burst.

DISCUSSION on P4/P5 (PSDB/PSER):

- Samsung thinks it is feasible for both PSDB and PSER by simple extension of PDB and PER measurements.

- Intel thinks there is significant work to make it possible as it involves UE and PDU set info is not transmitted over the air.

- Huawei thinks we need to discuss for DL and UL separately. For UL, we would need to add in-band marking. For DL there is complexity. Not clear about the benefits.

- LGE thinks we can focus on DL. It can be measured by gNB implementation but it may be inaccurate especially for PSER.

- Lenovo does not think this can be reliably done and also wonders about the purpose of this info.

- Ericsson agrees with Lenovo and LGE.

For DL, RAN2 thinks that some PSDB/PSER estimation by gNB implementation is possible, but its accuracy and reliability is unclear based on existing mechanism. However, RAN3 is in a better position to reply this question.

For UL, RAN2 thinks currently it is not possible to have info about PSDB/PSER and such mechanism would add significant complexity.

RAN2 is not clear about the benefits and usage of such information by application layer.

[AT126][601][XR] Reply LS to SA2 on FS_XRM PH2 (vivo)

Scope:  Prepare and review the reply LS to SA2 based on the agreements and discussion during the meeting

Intended outcome: Agreeable LS

Deadline:  LS ready for offline approval: Friday 0900

R2-2405779 Reply LS to SA2 on FS_XRM PH2	RAN2 LS out	Rel-19	NR_XR_Ph3-Core	To:SA2; Cc:RAN3, SA4

Approved

Discussion on AL-FEC LS

R2-2405199	Discussion on AL-FEC Awareness at RAN	NEC	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 1:  Reply SA2 that NG-RAN can determine whether a PDU was successfully delivered over an unacknowledged mode data bearer, and NG-RAN get this information sufficiently early to decide whether or not to drop subsequent AL-FEC packets.

Proposal 2: Inform SA2 that RAN2 is discussing to enhance RLC AM in Rel-19, with feature of skipping/speeding up further retransmission of a packet, which could be used in conjunction with AL-FEC awareness.

Proposal 3: Reply to SA2 that no additional impact on NG-RAN to support dynamic redundancy ratios comparing to support static redundancy ratio, and it could be up to network implementation on how to use the AL-FEC information.

R2-2405546	Discussion on SA2 LS on AL-FEC Awareness at RAN	Futurewei	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 1. RAN2 do not see a major benefit of dropping AL-FEC packets based on a known ratio and a reception status of a PDU Set for XR traffic that can justify the complexity to be added to RAN and the following potential drawbacks:

•	The transmitter needs to intentionally transmit some but not all AL-FEC packets of the same PDU Set and then stop and wait for feedback before deciding whether to transmit any of the remaining AL-FEC packets.

•	Delay due to the stop-and-wait reduces the delay budget that can be used for the gNB’s scheduling. The more efficiency gain being targeted at, the more rounds of stop-and-wait cycle may be needed to add redundancy incrementally, and hence the more delay. To not to violate the PSDB, the delay budget for the gNB to schedule the transmission of each individual round needs to be cut short significantly, putting more constrains on the gNB’s scheduling and potentially resulting in a lower radio efficiency in the transmissions.

•	A potential need for more frequent sending of PDCP SN gap report.

DISCUSSION on whether “NG-RAN can determine whether a PDU was successfully delivered over an unacknowledged mode data bearer? If so, does NG-RAN get this information sufficiently early to decide whether or not to drop subsequent AL-FEC packets?”:

- Nokia thinks HARQ is unreliable, otherwise why would we have RLC AM.

- Ericsson agrees this cannot be done reliably.

- LGE agrees it is unreliable and cannot be timely. There are HARQ errors.

- QCM thinks that for DL, relying on HARQ is good enough, the error rate is not significant. QCM indicates in MBS we rely on HARQ already.

- Intel agrees with QCM, this is a good estimation, even though not 100% reliable.

- ZTE indicates there are impact on, e.g. in CU-DU split. RAN3 would need to specify network interactions.

- Lenovo thinks it is clear HARQ is not reliable, we cannot discard something based on such unreliable means.

- Vivo thinks HARQ accuracy is sufficient. Feedback is immediately after receiving data at PHY layer and it is fast enough.

- CMCC indicates multiple LCHs can be multiplexed in one TB which is another issue.

- MTK agrees HARQ is not reliable enough. NACK->ACK misdetection is dangerous.

- Nokia indicates that HARQ error is only one issue and there are other issues if we want to be precise, e.g. PDU to TB mapping etc.

RAN2 thinks it is not possible for NG-RAN to reliably determine whether a PDU was successfully delivered over an unacknowledged mode data bearer.

[AT126][602][XR] Reply LS to SA2 on application-layer FEC awareness at RAN (Qualcomm)

Scope:  Prepare and review the reply LS to SA2 based on the agreements and discussion during the meeting

Intended outcome: Agreeable LS

Deadline:  LS ready for offline approval: Friday 0900

R2-2405780 Reply LS on Application-Layer FEC Awareness at RAN Qualcomm Incorporated

Replace “RAN2 do not believe it is possible for NG-RAN to determine with 100% reliability” with “RAN2 do not believe it is possible for NG-RAN to reliably determine“

- QCM thinks we should mention that it is not 100% reliable.

- Intel thinks we could mention that HARQ gives some rough idea.

- Nokia, Samsung, Ericsson, LGE, Lenovo think we should just say is cannot be done reliably. If we add something on HARQ, then we need to mention other issues, e.g. additional delay etc.

- Vivo thinks we can mention HARQ.

For Q2:

- Nokia thinks RAN2 should reply to this question. Nokia thinks for DL solution might be easy, but for UL there are additional impacts.

- OPPO think that we can reply for DL it is up to NW implementation, for UL it is more complex.

- Ericsson think that even for DL it can impact NW implementation.

- Ericsson thinks we should mention impact of FEC on capacity.

- Intel thinks that dynamic ratios might imply that different PDU sets should be treated differently over the air.

- Ericsson thinks that there is also an issue with how we interpret intentional drop vs non-intentional drop.

- Nokia proposes to delay the reply.

For Q2 (on dynamic ratios), RAN2 reply that:

    - RAN2 did not have time to analyse the proposal in detail

    - Based on initial analysis:

      - For both UL and DL, there is additional complexity for RAN

      - For DL, there is no RAN2 specifications impact but there is an impact on network implementation complexity

      - For UL, there might be additional impact on specifications

      - This may also have negative impact on RAN performance due to intentional dropping of packets, but RAN2 did not asses the details

R2-2405781	Reply LS on Application-Layer FEC Awareness at RAN 	RAN2	LS out To: SA2 Cc: SA4, RAN3

Approved

R2-2404264	RAN2 responses to SA2 LSs on Rel-19 XR	Intel Corporation	discussion	Rel-19	NR_XR_Ph3-Core

R2-2404292	Discussing SA2 Liaisons	Nokia, Nokia Shanghai Bell	discussion	Rel-19	NR_XR_Ph3-Core

R2-2404294	Discussion on SA2 LSs for Rel-19 XR	Xiaomi	discussion	Rel-19	NR_XR_Ph3-Core

R2-2404329	Discussion on SA2 LSs	CATT	discussion	Rel-19	NR_XR_Ph3-Core

R2-2404333	Discussion on SA2 Liaisons on Rel-19 XR	Meta	discussion

R2-2404511	Discussion on LSs from SA2	Ericsson	discussion	Rel-19	NR_XR_Ph3-Core

R2-2404552	Discussion on LSs from SA2	ZTE Corporation, Sanechips	discussion

R2-2404775	Discussion on LSs from SA2 for XR	Huawei, HiSilicon	discussion	Rel-19	NR_XR_Ph3-Core

R2-2404812	Discussion on LS from SA2	Lenovo	discussion	Rel-19

R2-2405003	Discussion on LSs from SA2	InterDigital	discussion	Rel-19	NR_XR_Ph3-Core

R2-2405050	Discussion on RAN2 impact based on SA2 LS	OPPO	discussion	Rel-19	NR_XR_Ph3-Core

R2-2405300	Discussion on SA2 LS on XR	CMCC	discussion	Rel-19	NR_XR_Ph3-Core

R2-2405542	Discussion on SA2 LSs for XR	LG Electronics Inc.	discussion	Rel-19	NR_XR_Ph3-Core

R2-2405593	RAN2 Aspects of SA2 LSes on XRM and AL-FEC	Samsung	discussion	Rel-19	NR_XR_Ph3-Core

R2-2405662	Discussion on LS from SA2 on AL-FEC awareness	vivo	discussion	Rel-19	NR_XR_Ph3-Core

### 8.7.2	Multi-modality support

Objective: Study and if justified, specify aspects related to multi-modality (intra-UE) (with coordination with SA2/SA4 as needed by LS request). Aim to facilitate efficient and effective support for XR application with Multiple QoS flows with multi-modal inter-dependencies, meeting multi-modal QoS requirements, e.g. synchronization and/or coordination. Efficiency enhancements are expected to be visible in terms of capacity or power consumption.

Including aspects such as:

- what kind of multi-modality information is useful at the gNB and/or UE

- how is this information used by the gNB/UE and what benefits this brings

- what are the potential benefits and enhancements from multi-modal awareness depending on traffic direction (UL/DL)

- identification of potential impacts on other WGs due to multi-modal awareness enhancements

- other enhancements for multi-modal traffic, e.g. power saving, scheduling

Multi-modality awareness at RAN - benefits

R2-2404549	RAN enhancements for Multi-Modality support	ZTE Corporation, Sanechips, China Telecom, Meta, Sony, China Unicom	discussion

Observation 1: Static PDB based scheduling may not satisfy the synchronization requirements in some scenarios

Observation 2: If multi-modal dependencies and synchronization requirements are not known to RAN, transmission of dependent data may be delayed beyond the synchronization requirements with respect to the associated data and this may result in

-	capacity loss at RAN (due to unnecessary transmissions) and/or

-	unnecessary power consumption at the UE (as the UE has to transmit/receive and process the packets which will eventually be discarded at the upper layers)

Proposal 1: Support Multi-Modality awareness in RAN in Rel-19 for UL and DL

Proposal 2: Multi-modal dependencies (including the multi-modal service ID – MMSID) and associated synchronization requirements between multi-modal flows should be visible to RAN to satisfy the corresponding QoS requirements and to improve the radio capacity and reduce power consumption at the UE.

Proposal 3: Send an LS to SA2/SA4 to inform about the above conclusions and ask SA2/SA4 to define the necessary signalling to provide multi-modality awareness in RAN

R2-2404403	Multi-modality support	Nokia, Nokia Shanghai Bell	discussion	NR_XR_Ph3-Core

Observation 1: Current 3GPP specification is sufficient to achieve the goal of synchronized delivery of multi-modal flows.

Observation 2: Current available mechanism should be sufficient to handle different modals with different QoS requirements.

Proposal 1: For synchronized delivery of multi-modal flows, no additional/new RAN mechanism is required.

Proposal 2: For QoS insurance of multi-modal flows, no additional/new RAN mechanism is required.

DISCUSSION on whether to support multi-modality awareness:

- Xiaomi thinks that application layer alone cannot achieve synchronization requirement and some RAN assistance is needed.

- QCM so far is not sure about whether there is an issue with synchronization as the sync requirement is quite relaxed. QCM thinks that sync related does not have to be studied by RAN2.

- Meta disagrees with QCM, there are use cases where this is needed and current framework does not support it.

- Huawei thinks there are use cases where the flows needs to be synchronized.

- Mediatek thinks there is some info in RTP, but perhaps such information should be also available at RAN layer.

- OPPO supports MM awareness, otherwise we cannot meet the requirements.

- Lenovo also supports MM awareness, gNB should have this knowledge. It can be used at lest for scheduling.

- LGE agrees with the scenario from ZTE paper and support MM awareness. We should request SA2 to deliver this information.

- Ericsson does not think there is a problem.

- Intel thinks companies show a lot of potential enhancements thanks to MM awareness. We can ask SA2 what information they can give us.

- Ericsson thinks there were no capacity gains shown. ZTE clarifies that if we transmit unnecessary packets, then it is clear there is negative impact to capacity. This is similar to dropping correlated packets.

- Meta thinks user capacity is what matters most.

- Spreadtrum thinks that awareness can also be used for admission control.

- Apple thinks that we cannot always assume that we can discard related packets.

- Nokia thinks we can ask about MMSID, but is not clear whether synchronization thresholds are useful.

- MTK is not sure about UL, but for DL this can be done by gNB implementation.

- Vivo thinks we can mention both UL and DL.

- ZTE thinks we can indicate what information we find useful, but details solution are still FFS, perhaps up to WI phase.

Support Multi-Modality awareness in RAN in Rel-19 for UL and DL.

Ask SA2 whether/what information could be provided to RAN

Clarify in the LS how RAN2 considers to use this information, e.g. coordinated handling of multi-modal flows.

Can consider while drafting the LS to indicate the potential benefits

[AT126][603][XR] LS to SA2 on multi-modality awareness at RAN (Nokia)

Scope:  Prepare and review the reply LS to SA2 based on the agreements and discussion during the meeting

Intended outcome: Agreeable LS

Deadline:  LS ready for offline approval: Friday 0900

R2-2405778 LS on multi-modality awareness at RAN TSG RAN WG2

Put SA4, RAN3 in cc

Remove “Although discussions are still ongoing in RAN2 about the potential RAN mechanisms and its gains“

Add “and their gains in next RAN2 meeting” in the second paragraph

Change “to continue the discussions about the potential RAN” to “to continue the discussions which potential RAN”

We clarify that we are still in study phase and that we need to understand whether/what information we can get from SA2 (e.g. MMSID, synchronization thresholds) to conclude the study.

Revised in R2-2405945

- Nokia clarifies that companies want to mention benefits. Nokia thinks we should just mention that RAN2 will continue analysing those.

- Intel would like to add SA4 in the LS (in To field and also request them to provide feedback).

- Huawei has no strong view on sending this to SA4, perhaps adding them in cc is sufficient.

- Huawei thinks we made an agreement on supporting MM, so we should mention benefits.

- ZTE thinks potential gains depend on what information we get.

- ZTE proposes we state that we agreed to support MM awareness and that gains depend on what information we can get from CN. Meta agrees to list the potential gains, potential solutions also how we intend to use it.

- Nokia thinks the reason to agree MM awareness is to check further the gains.

- Mediatek, LGE thinks current LS structure is not logical.

- Interdigital thinks we should at least give SA2 some idea on what we have in mind.

- ZTE proposes to mention MMSID and synchronization thresholds as examples. OPPO agrees.

- Nokia thinks we should clarify that we are still in study phase, but we need information from SA2 to continue it.

R2-2405945 LS on multi-modality awareness at RAN TSG RAN WG2

Change “In this meeting, as part of this study, RAN2 has agreed that RAN awareness of multi-modality is supported for both UL and DL.” To “In this meeting, as part of this study, RAN2 has agreed that multi-modality awareness at RAN is supported for both UL and DL.”

Change “MMSID or synchronization thresholds” to “MMSID and/or synchronization thresholds”

Remove “CC SA4, RAN3” from actions

With the changes above, the LS is approved unseen in R2-2405782

- Meta and CMCC would like to delete “as part of this study”.

- Nokia thinks that it is a fact that we are in the study phase.

- Apple supports the current version.

- Nokia does not want to mention “in Rel-19”. Intel does not think mentioning it is critical. Samsung also agrees, we should send the LS. LGE thinks we mention “study” too many times.

R2-2405782 LS on multi-modality awareness at RAN	TSG RAN WG2 LS out	Rel-19	NR_XR_Ph3-Core	To:SA2 Cc: TSG RAN WG3, TSG SA WG4

Approved (unseen)

Multi-modality awareness at RAN – how is it delivered?

R2-2404913	multi modal flows and DRB mapping	Sony	discussion	Rel-19	NR_XR_Ph3

Proposal 1: RAN2 assumes that multi modal service ID is received in RAN from the core network i.e. there is no need for UE to provide this information to the gNB. Send an LS to SA2.

R2-2404400	Discussion on multi-modality support for XR	China Telecom	discussion

Proposal 2: The Rel-19 XR UE can also provide UL assistance information which may include the synchronization threshold and dependency information to the network.

Multi-modality awareness at RAN – LCP/DSR/discarding enhancements

R2-2404880	Enhancements for support of Multi-Modal XR applications 	Lenovo	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 4: RAN2 to discuss enhancements to the LCP procedure, where the absolute remaining time and in addition the relative remaining time, e.g. enforcing the multi-modal synchronization requirements between PDU sets of LCHs of a multi-modal application, of data is considered when determining the order in which data of LCHs is multiplexed in a TB/UL grant.

Proposal 5: RAN2 to discuss DSR enhancements for multi-modal applications, e.g. UE providing information to gNB on data for which the relative remaining delay, e.g. enforcing the multi-modal synchronization requirement, becomes lower than a threshold.

R2-2404937	Discussion on XR Multi-modality	Spreadtrum Communications	discussion	Rel-19

Proposal 6: Study discard enhancement considering inter dependency among multi-modal QoS flows.

Other enhancements related to multi-modal traffic

R2-2404512	Discussion on multi-modality	Ericsson	discussion	Rel-19	NR_XR_Ph3-Core

Observation 1	A single DRX configuration matched to a traffic flow may not be suitable to fulfil the PDBs of other traffic flows, resulting in down to zero capacity.

Observation 2	Multiple active DRX configurations, each matching a traffic flow, are suitable to achieve both high UE power saving gains and many satisfied UEs, if a single DRX configuration matched to one flow does not satisfy the PDBs of other flows.

Proposal 1	Support multiple active DRX configurations to limit the delay and optimize power saving of UEs with multi-flow XR services.

R2-2404774	Discussion on multi-modal XR	Huawei, HiSilicon	discussion	Rel-19	NR_XR_Ph3-Core

Observation 6: Legacy DG/CG may be not efficient for the transmission of haptic data which have stringent PDB, unpredictable burst size and irregular periodicity.

Observation 7: The transmission of haptic data may impact the transmission of video and audio, and decrease the resource efficiency.

Observation 8: Under the existing scheduling mechanism, the network capacity may be decreased if there is haptic traffic.

Proposal3: RAN2 to confirm whether the existing scheduling mechanism is sufficient to support multi-modal XR services with haptic traffic, from both the haptic KPI and the network capacity perspective.

R2-2404265	Justification and Enhancements for Multi-modal Services	Intel Corporation	discussion	Rel-19	NR_XR_Ph3-Core

R2-2404330	Discussion on Multi-Modality	CATT	discussion	Rel-19	NR_XR_Ph3-Core

R2-2404334	Discussion on Multi-Modality XR	Meta	discussion

R2-2404351	Discussions on Multi-modality Awareness	Fujitsu	discussion	Rel-19	NR_XR_Ph3-Core	R2-2402278

R2-2404425	Discussion on Multi-modality	vivo	discussion	Rel-19	NR_XR_Ph3-Core

R2-2404455	Discussion on Multi-modality support for XR traffic	Xiaomi Communications	discussion

Proposal 4 For multi-modal QoS requirements, LCP enhancement will not be considered until the requirement of multi-modal QoS is clear enough.

Proposal 5 PDU set based discarding across PDU sets/QoS flows should not be considered until we get requirement from SA2.

R2-2404556	Discussion on Multi-modal support for XR	LG Electronics Inc.	discussion	Rel-19	NR_XR_Ph3-Core

R2-2404572	Discussion on Multi-modality support for XR	TCL	discussion

R2-2404649	Views on Support of Multi-Modality Services in Rel-19 XR	Apple	discussion	Rel-19	NR_XR_Ph3-Core

R2-2404866	Multi-modality support for XR	Google Inc.	discussion

R2-2404937	Discussion on XR Multi-modality	Spreadtrum Communications	discussion	Rel-19

R2-2405000	Multi-modality support for XR	InterDigital	discussion	Rel-19	NR_XR_Ph3-Core

R2-2405016	Further discussion on multi-modality support for XR	CMCC	discussion	Rel-19	NR_XR_Ph3-Core

R2-2405051	Discussion on the multi-modality support	OPPO	discussion	Rel-19	NR_XR_Ph3-Core

R2-2405072	Discussion on multi-modality support	NEC	discussion	Rel-19	NR_XR_Ph3-Core

R2-2405158	Further aspects of multi-modality support in RAN	Samsung	discussion

R2-2405439	Draft LS to SA2 on XR multi-modality	CMCC	LS out	Rel-19	NR_XR_Ph3-Core	To:SA2

R2-2405614	Discussion on multi-modality	MediaTek Inc.	discussion	Rel-19

### 8.7.3	RRM measurement gaps/restrictions related enhancements

Objective: Specify enhancements to enable transmission/reception in gaps/restrictions that are caused by RRM measurements (from inter-frequency RRM measurement gaps, or intra-frequency measurements, or other scheduling restrictions etc).

This agenda item will not be treated during RAN2#126 and no contributions should be submitted for this AI for this meeting.

### 8.7.4	Scheduling enhancements

Objective: For the UL, Study and if justified, Specify enhancements using delay/deadline information, for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs.

Including aspects such as:

- whether/how to resolve the issue of data with low remaining time being delayed due to other data from LCHs with higher LCH priority

- enhancing DSR with additional information, e.g. what is additional information, can it refer to non-delay critical data etc.

LCP enhancements – LCH prioritization

R2-2404182	Discussion on scheduling enhancements for XR	OPPO	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 1	Delay-aware LCP enhancement to resolve the issue of data with low remaining time being delayed due to other data from LCHs with higher LCH priority is supported in Rel-19 XR.

Proposal 2	For delay-aware LCP enhancement, RAN2 discuss the following options to override/adjust the priority of LCH based on delay/deadline information:

- Option 1: Adjust the LCH with delay-critical data to the highest priority below the priorities of all SRBs

- Option 2: Use additional priority configured to LCHs in case of these LCHs with delay-critical data

DISCUSSION on P1:

- LGE is wondering whether we need to down-select between LCH restrictions and LCH prioritization or we can keep both is study phase.

- OPPO clarifies that with P1 they cover both.

- Ericsson wonders about the gains of this delay approach, in their simulations capacity is lost.

- CMCC thinks this approach helps in the situation where non-delay critical data takes grant of delay-critical data.

- QCM supports proposal 1. Would like to add that we need to consider UE complexity.

- Vivo thinks that this solution helps to increase UE satisfaction rate. OK with the proposal.

- Intel is OK with the proposal, would like to clarify this is for inter-LCH prioritization.

- Futurewei indicates we need to be careful not to prioritize low priority packets.

Delay-aware LCP enhancement to resolve the issue of data with low remaining time being delayed due to data from other LCHs with no delay critical data is supported in Rel-19 XR.

The solution should consider impact on UE complexity (as already indicated in SI objective description)

DISCUSSION on P2:

- Apple thinks option 1 is not flexible enough, it is better to have it configurable.

- QCM thinks option 2 is more flexible, no need for option 1. QCM wonders if priority is adjusted only for delay-critical data or for the whole LCH.

- Intel agrees with QCM.

- Xiaomi thinks that configurability brings additional complexity, it would be simpler with static priority.

- Lenovo thinks the network should be above to control when and for which LCH this is used.

- Vivo is fine with option 2, but option 1 is simpler.

- Spreadtrum thinks network should configure the priority.

- Nokia thinks option 2 is OK, there is not additional complexity.

- LGE thinks option 2 is preferable, but option 1 is also OK.

- QCM thinks option 1 is not flexible enough, e.g. if there are different LCH with delay critical data.

- Fujitsu is OK, but wonders whether this new priority applies to al ldata or only delay critical data.

- ZTE thinks PBR will still apply.

- QCM thinks all other parameters stay the same, including PBR. Lenovo agrees.

- Lenovo thinks we should apply the higher priority to the whole LCH

For delay-aware LCP enhancement, RAN2 considers the following option to override/adjust the priority of LCH based on delay/deadline information as a baseline:

- - Use additional priority configured to LCHs in case of these LCHs with delay-critical data

FFS whether the priority only applies to delay-critical data within the LCH or for the whole LCH

LCP enhancements – LCH restrictions

R2-2405543	Discussion on Scheduling enhancement for XR	LG Electronics Inc.	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 2. The data with short remaining time should be prioritized only if it is needed, e.g., when it is indicated by the network or when DSR is transmitted.

Proposal 4. Support the enhancement of LCP restrictions to prioritize delay-critical data, e.g., select only the logical channels with delay-critical data, when the delay-critical data needs to be prioritized.

DISCUSSION on P2:

- Apple thinks that such indication may be too late for UE to process it. No need to couple with DSR.

- Lenovo has concerns with P2, it has impacts on RAN1.

- ZTE does not think indication from the network is not a good idea. It should be static configuration, i.e. based on threshold.

- Xiaomi also does not think this is good idea, it is better to base on conditions such as DSR triggering.

- CMCC sees no need for this.

- Huawei thinks this can be done with no RAN2 impact, e.g. whenever delay critical is occurring then the UE uses LCH restriction.

We try to avoid RAN1 impacts.

RAN2 assumes no dynamic indications are needed for triggering the delay-aware LCP mechanism. RAN2 assumes this mechanism is configured in a semi-static way.

R2-2404331	Consideration on XR-specific Scheduling Enhancement	CATT	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 1: For enhanced LCP restrictions/LCH selection, dedicated UL grant only for data with low remaining time can be considered.

Proposal 2: An indication in DCI can be used to indicate the UL dynamic grant is dedicated for data with low remaining time.

Proposal 3: New LCP restriction can be introduced to indicate whether a logical channel is allowed to use the dedicated UL grant for data with low remaining time.

- ZTE thinks that without RAN1 work, LCP restrictions based solutions are not that interesting.

- Nokia would like to exclude solutions where we disallow completely non-delay critical data from using an UL grant.

- LGE thinks relaxing LCH restrictions has further impact on UE.

For LCP restrictions based solutions, RAN2 will not discuss solutions requiring RAN1 work. FFS whether other LCP restrictions based approaches are needed/beneficial

The solutions should not disallow non-delay critical data from using an UL grant.

LCP enhancements – granularity

R2-2404456	Discussion on scheduling enhancements of XR traffic	Xiaomi Communications	discussion

Proposal 3 Prioritization within a logical channel (e.g., by remaining timer or by importance) will not be considered

DISCUSSION:

- QCM thinks we do not have to discuss anything for intra-LCH prioritization.

- Intel, Mediatek agrees with Qualcomm.

- LGE would also like to clarify this is about MAC prioritization.

- Spreadtrum agrees with the proposal.

LCP prioritization within a logical channel will not be considered in RAN2 discussions

R2-2404550	Scheduling enhancements for XR	ZTE Corporation, Sanechips	discussion

Proposal 3: Introduce a remaining time threshold based LCP mechanism, e.g. At the stage for allocation of resources in LCP, UL grant resource is allocated for the data with remaining time below a threshold firstly; and the remaining resource, if any, is allocated for the non delay-critical data based on legacy LCH priority.

DISCUSSION on whether we have a new remaining time threshold for delay-aware LCP:

- Interdigital thinks we can reuse R18 threshold.

- Vivo prefers separate configuration, up to NW whether value is the same or different.

- Xiaomi, Fujitsu thinks we can reuse DSR threshold.

- Nokia, ZTE, QCM, Lenovo prefers to have a separate threshold. Time to trigger for DSR can be more, but this is stage-3 detail.

- Xiaomi does not see the value of separating. Fujitsu agrees.

- Lenovo clarifies the threshold for DSR might be higher, e.g. to allow SR.

FFS whether a separate remaining time threshold can be configured for delay aware LCP (i.e. different from the one used for DSR).

DSR enhancements – multiple thresholds and grouping of data

R2-2404176	Discussion on delay-aware scheduling	Qualcomm Incorporated	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 5. 	Network can configure an LCG with multiple DSR reporting thresholds. For each threshold, UE reports data volume whose remaining time is below that threshold.

Proposal 6. 	Enhanced DSR MAC CE does not need to include any delay information related to PDU Set Importance.

R2-2404514	Discussion on scheduling enhancements	Ericsson	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 1	Enhance DSR to report with multiple pairs of remaining time and buffer size.

Proposal 2	Grouping of data in the DSR should be based on PDU Sets.

DISCUSSION on whether to have multiple with multiple pairs of remaining time and buffer size:

- Intel asks whether new threshold are only for delay-critical. Ericsson thinks we cannot refer to delay-critical only.

- Lenovo asks if we report data multiple times.

- Google asks if this means that we will have multiple thresholds for adjusting the priority.

- Nokia is fine with the proposal. Nokia also supports indication of importance.

- Vivo is OK with the proposal, but would also like to clarify about triggering.

- Ericsson thinks one trigger is enough.

- CMCC wonders if this means that we have data with different priority

- Huawei clarifies that we can have data with different PDBs arriving at different times, support the proposal.

- LGE supports the first proposal. We need to consider time gap between DSR triggering and transmission time.

Enhance DSR to report with multiple pairs of remaining time and buffer size for the LCG.

FFS whether DSR triggering is impacted

FFS whether PDU set importance needs to be included

DSR enhancements – whether to include non-delay critical data

R2-2405535	Discussion on UL scheduling enhancements	MediaTek Inc.	discussion	Rel-19	38.321	NR_XR_Ph3, NR_XR_Ph3-Core

Proposal 1: For DSR enhancement, RAN2 only consider information related to delay-critical data. Non-delay critical data can be handled by BSR.

R2-2404426	Discussion on scheduling enhancement for XR	vivo	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 5	DSR should be enhanced to comprise both delay-critical data information and non-delay-critical data information.

R2-2404266	LCH prioritization and DSR enhancements	Intel Corporation	discussion	Rel-19	NR_XR_Ph3-Core

R2-2404335	Discussion on Scheduling Enhancement for XR	Meta	discussion

R2-2404352	Discussions on delay-aware LCP	Fujitsu	discussion	Rel-19	NR_XR_Ph3-Core

R2-2404401	Scheduling enhancements for XR traffic	China Telecom	discussion

R2-2404567	Discussion on UL scheduling enhancements	HONOR	discussion	Rel-19	NR_XR_Ph3-Core

R2-2404573	Discussion on LCP enhancement in XR	TCL	discussion

R2-2404650	Complementary RLC Mechanisms for LCP Enhancements	Apple, Lenovo, CATT, OPPO	discussion	Rel-19	NR_XR_Ph3-Core

R2-2404651	Views on DSR Enhancements for Rel-19 XR	Apple	discussion	Rel-19	NR_XR_Ph3-Core

R2-2404708	Scheduling Enhancements for XR	Nokia, Nokia Shanghai Bell	discussion	Rel-19	NR_XR_Ph3-Core

R2-2404849	Discussion on the LCP enhancements for XR	ITRI	discussion	NR_XR_Ph3-Core

R2-2404878	Enhanced uplink scheduling for XR 	Lenovo	discussion	Rel-19	NR_XR_Ph3-Core

R2-2404887	Considerations on delay-sensitive scheduling for XR	NEC	discussion	Rel-19	NR_XR_Ph3-Core

R2-2404914	UL Scheduling enhancements for XR	Sony	discussion	Rel-19	NR_XR_Ph3

R2-2404938	Discussion on XR scheduling enhancement	Spreadtrum Communications	discussion	Rel-19

R2-2405001	Scheduling enhancements for XR	InterDigital	discussion	Rel-19	NR_XR_Ph3-Core

R2-2405017	Further discussion on scheduling enhancement for XR	CMCC	discussion	Rel-19	NR_XR_Ph3-Core

R2-2405119	Delay-aware scheduling enhancements	Huawei, HiSilicon	discussion	Rel-19	NR_XR_Ph3-Core

R2-2405357	Discussion on scheduling enhancement for XR	Google Inc.	discussion	NR_XR_Ph3-Core

R2-2405404	Discussion on scheduling enhancements for XR	DENSO CORPORATION	discussion	Rel-19	NR_XR_Ph3-Core

R2-2405446	Solutions for DSR enhancement	TCL	discussion	Rel-19

R2-2405473	LCP enhancement	Sharp	discussion

R2-2405481	Discussion on XR scheduling enhancements	III	discussion	NR_XR_Ph3-Core

R2-2405594	Scheduling Enhancements for Rel-19 XR	Samsung	discussion	Rel-19	NR_XR_Ph3-Core

R2-2405654	Discussion on XR uplink Scheduling enhancements	Rakuten Mobile, Inc	discussion	Rel-19

### 8.7.5	RLC enhancements

Objective: RLC re-transmission related enhancements for operation of RLC Acknowledged Mode (AM) with small packet delay budget.

Including aspects such as:

- enhancements to ensure timely RLC retransmission(s) for XR

- how to avoid unnecessary retransmissions (e.g. of out-dated packets)

Avoiding unnecessary retransmissions

R2-2404596	Discussion on RLC AM enhancements	Huawei, HiSilicon	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 1:	RAN2 to enhance the RLC AM by adopting enhancements from one of the following perspectives:

•	The Rx RLC entity does not send NACK to the Tx RLC entity for the outdated data even if the data has not been received successfully;

•	The Tx RLC entity stops unnecessary RLC retransmission and notifies the Rx RLC entity which RLC SDU will not be retransmitted.

DISCUSSION:

- Mediatek thinks the first option is not feasible as the timer is maintained at the tx side. Vivo agrees.

- LGE thinks there will be no issue with unnecessary retransmissions if we have a proper solution for timely retransmission. Prefer to have Rx based solution.

- Apple thinks for Rx solution we need some timer at Rx side which can be dynamic in XR and not visible at Rx side.

- Apple thinks that if the number of retransmissions is configured properly, then there might be no issue.

- Ericsson think at Rx side we can use reordering timer, we do not have to have visibility to other traffic related timers.

- Intel thinks we should agree that unnecessary Tx avoidance should consider delay budget. Intel has slight preference for Rx but thinks we should study both.

- QCM would like to prioritize Tx side approach.

- Nokia has the opposite view. Thinks Rx side approach will be much simpler.

- KDDI shares the view with Nokia, Rx approach is simpler. Xiaomi agrees.

- LGE thinks we might not need any solution. Ericsson disagrees.

- Samsung disagrees with LGE, thinks the solution are complimentary.

For avoiding unnecessary RLC AM retransmissions, RAN2 to enhance the RLC AM by adopting enhancements from one of the following perspectives:

- Rx initiated approach

- Tx initiated approach

RAN2 will discuss details of both approaches, compare them and choose one once the details are clearer.

R2-2405032	Discussion on RLC retransmission enhancements in XR	CMCC	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 1: The transmitting side of AM RLC entity could notify the receiving RLC side with the SN gap when the transmitting side abandons the retransmission of an obsolete RLC SDU or an RLC SDU segment.

DISCUSSION:

- LGE thinks this is complex.

- Lenovo thinks we could also just inform about obsolete PDUs.

For Tx initiated approach:

    - The transmitting side of AM RLC entity notifies the receiving RLC side about the obsolete SDUs

    - Tx side stops retransmit obsolete SDUs

    - Rx side updates state variables according to the information from Tx side

R2-2404293	RLC AM enhancements for XR	Nokia, Nokia Shanghai Bell	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 4: For proper advancing of the transmitting window, RLC AM is enhanced with a way for the receiver to indicate abandoned SDUs to the transmitter.

For Rx initiated approach:

    - For proper advancing of the transmitting window, RLC AM is enhanced with a way for the receiver to indicate abandoned SDUs to the transmitter.

    - Tx side just processes the status report as in legacy

    - FFS how Rx side determines that an SDU should be abandoned

Autonomous retransmissions

R2-2404212	Discussion on RLC AM Enhancements 	CANON Research Centre France	discussion	Rel-19	NR_XR_Ph3-Core

Observation 1: In RLC AM, the retransmission of PDUs can experience large delay

Observation 2: Relying on RLC parameter tunning such as polling frequency or t_reassembly timer cannot solve the issue.

Proposal 1: Consider anticipated retransmission of PDUs with unknown status.

Proposal 2: Further study the criteria for triggering anticipated retransmission of PDUs with unknown status.

DISCUSSION:

- Intel asks what is meant by “anticipated”? Is it autonomous? CANON confirms.

- Nokia, QCM agrees with observations and supports the proposal.

- QCM indicates RTT for RLC AM is too long.

- Mediatek disagrees with the observations. Think we can reconfigure, e.g. polling timer. Autonomous reTx is bad for capacity.

- Ericsson shares an understanding of Mediatek. We can have an earlier feedback to trigger reTx and we should avoid unnecessary duplicate transmissions.

- LGE agrees with the observations, just using shorter timer is not good. It will require a lot of status reports and impacts processing and it can delay actual data.

- Ericsson thinks that timers are always adjusted to traffic requirements.

- Vivo thinks that we need to be careful with autonomous reTx, especially during congestion.

- Samsung think we should study autonomous reTx, it can be complimentary to status reports.

- Xiaomi is also concerned about capacity, gNB implementation can avoid the issues in the paper.

- Apple thinks we need to analyse more what we already have, e.g. repetition, duplication. If something is needed, we should look at polling timer.

- Huawei thinks we need to have some status report to avoid capacity loss.

- KDDI is also concerned about capacity.

- Nokia thinks the whole point was to have RLC retransmissions for cases where HARQ does not work. Nokia thinks we can have a general agreement which covers also solutions including SRs. But the reTx needs to be quick.

- LGE thinks that polling and SR cannot work for delay-critical data. If we retransmit delay critical data timely then we solve both data loss issue and unnecessary reTx issue.

- Ericsson thinks we first need to understand why the current mechanism does not work.

- QCM thinks that even if you reduce timers, that will not be enough.

- NEC thinks we should consider RLF due to autonomous reTx

To achieve timely retransmissions on RLC layer for XR traffic, RAN2 will consider the following options:

    - Autonomous retransmission (i.e. without status report) of PDUs based on some triggers (existing or new triggers can be considered)

    - Retransmission based on enhanced status report

    - Retransmission based on enhanced polling

- FFS whether any enhancements are needed or this can be solved with proper configuration and current mechanism

Impact on capacity should be considered

RAN2 focuses on the enhancements for UL traffic

R2-2404177	Discussion on RLC enhancements	Qualcomm Incorporated	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 3.	If configured by network, a RLC AM transmitter can retransmit a RLC PDU if one of the following conditions is met:

-	after the remaining time of the PDU has dropped below a configured threshold; or

-	after the PDU has failed a configured number of HARQ transmissions; or

-	if the PDU is in the RLC retransmission buffer and there are spare PUSCH resources available after the LCP procedure.

R2-2405285	Discussion on RLC AM Enhancements	Ericsson	discussion	Rel-19

Observation 1	For traffic in the UL, network can decide when to send the RLC STATUS report and for shorter PDBs, it can send it more often.

Observation 2	For traffic in the DL, the network can configure smaller values for the traffic with shorter PDBs thereby enabling faster feedback/timely retransmissions.

Proposal 1	No enhancements are needed for the RLC STATUS PDU as it is up to network implementation for traffic in the UL and network configuration for traffic in the DL.

Proposal 2	Not pursue solutions relating to autonomous RLC retransmissions based on timers or without the STATUS PDUs.

R2-2405002	RLC enhancements for XR	InterDigital	discussion	Rel-19	NR_XR_Ph3-Core

Proposal 2:	Study polling enhancements to inform the UE of the reception status of each uplink PDU set.

Proposal 3:	Support the UE to stop the t-StatusProhibit timer to allow timely reporting of the RLC status. FFS the scenario(s) when the UE stop the timer.

R2-2404197	RLC AM retransmission enhancements	Xiaomi	discussion	Rel-19	NR_XR_Ph3-Core

R2-2404255	Discussion on RLC re-transmission related enhancements	OPPO	discussion	Rel-19	NR_XR_Ph3-Core

R2-2404267	RLC (re)transmission enhancements	Intel Corporation	discussion	Rel-19	NR_XR_Ph3-Core

R2-2404332	Consideration on XR-specific RLC Enhancement	CATT	discussion	Rel-19	NR_XR_Ph3-Core

R2-2404336	Discussion on RLC AM Enhancements for XR	Meta	discussion

R2-2404353	Discussions on RLC enhancements	Fujitsu	discussion	Rel-19	NR_XR_Ph3-Core

R2-2404359	Discussion on RLC enhancements	SHARP Corporation	discussion	NR_XR_Ph3-Core

R2-2404402	Discussion on RLC enhancements for XR	China Telecom	discussion

R2-2404427	Discussion on RLC enhancement for XR	vivo	discussion	Rel-19	NR_XR_Ph3-Core

R2-2404551	RLC enhancements for XR	ZTE Corporation, Sanechips	discussion

R2-2404568	Discussion on RLC enhancements for XR	HONOR	discussion	Rel-19	NR_XR_Ph3-Core

R2-2404627	Considerations on RLC re-transmission related enhancements for XR	KDDI Corporation	discussion

R2-2404652	Views on RLC-AM Enhancements for Rel-19 XR	Apple	discussion	Rel-19	NR_XR_Ph3-Core

R2-2404813	AM RLC Enhancement	Lenovo	discussion	Rel-19

R2-2404850	RLC re-transmission enhancements for XR	ITRI	discussion	NR_XR_Ph3-Core

R2-2404915	RLC periodic poll based on RLC SN	Sony	discussion	Rel-19	NR_XR_Ph3

R2-2404939	Discussion on timely RLC retransmission(s)	Spreadtrum Communications	discussion	Rel-19

R2-2405195	RLC AM Enhancement	NEC	discussion	Rel-19	NR_XR_Ph3-Core

R2-2405329	Discussion on details of RLC enhancements for XR	LG Electronics Inc.	discussion	Rel-19	NR_XR_Ph3-Core

R2-2405380	Discussion on RLC enhancements on small packet delay budget scenario	MediaTek Inc.	discussion	Rel-19	38.322	NR_XR_Ph3, NR_XR_Ph3-Core

R2-2405445	Solutions for RLC AM retransmission enhancement	TCL	discussion	Rel-19

R2-2405493	Discussion on RLC enhancements for XR	Samsung	discussion	Rel-19	NR_XR_Ph3-Core

## 8.8	NTN for NR Ph3

(NR_NTN_Ph3-Core; leading WG: RAN2; REL-19; WID: RP-240775

LTE_TN_NR_NTN_mob, leading WG: RAN2, Rel-19 WID: RP-240846)

Time budget: 2 TU

Tdoc Limitation: 4 tdocs

### 8.8.1	Organizational

LS, Rapporteur input, including workplan, etc.

R2-2404207	Updated work plan for NR NTN Ph3	CATT, Thales	Work Plan

Noted

R2-2404137	LS on Support of Regenerative-based Satellite Access (S2-2405600; contact: vivo)	SA2	LS in	Rel-19	FS_5GSAT_Ph3_ARCH	To:RAN3	Cc:RAN2

Noted

### 8.8.2	Downlink coverage enhancements

Contributions should focus on RAN2 aspects of DL coverage enhancements (e.g. cell level / beam level DTX/DRX mechanism, etc.).

Applicable UE states / impact to legacy UEs

R2-2405229	On the applicability of downlink coverage enhancements	Nokia, Nokia Shanghai Bell	discussion	NR_NTN_Ph3-Core

Observation 1: having a switching on-off granularity different from cell level is very power inefficient.

Proposal 1: RAN2 to assume that the work on signalling for downlink coverage enhancement will be on cell level. Up to RAN1 whether a cell is covered by multiple beams or not.

Observation 2: Rel-18 NES allows for independent cell DTX/DRX to enhance power saving.

Observation 3: Legacy NES cell DTX/DRX is defined as RAN1’s “N2” state within the downlink coverage enhancement study, i.e. it is not allowed to impact idle mode operations, including SSB transmission of RACH occasions.

Observation 4: The existing framework of NES cannot be used to which cells completely off and new, non backwards compatible, signalling/procedures are required.

Observation 5: Utilising NTN downlink coverage enhancement suitable NES including turning SSB transmission off or increasing periodicity, will trigger a cell (re)selection procedure in legacy UEs and thus compromising energy consumption.

Observation 6: New signalling/procedures to handle UEs in RRC_IDLE/ RRC_INACTIVE will only work for Rel19 UEs and will not be backwards compatible, leading to potential increased energy consumption.

Proposal 2: It is RAN2s understanding that legacy UEs will not be compatible with any new DL coverage enhancement signalling procedures.

-	Ericsson thinks we should also consider power sharing as part of this discussion. NEC agrees

-	IDC thinks we first need to ask questions to RAN1 to get a number of clarifications before assessing the impact on legacy UEs

Based on the solution being investigated in RAN1, RAN2 will further discuss whether/how legacy UEs might operate in a cell supporting DL coverage enhancements.

Proposal 3: RAN2 to agree that support of system level downlink coverage enhancements in Rel-19 will not be trivial as there will be high spec impact, including on legacy NES

Proposal 4: RAN2 to send the above conclusions in an LS to RAN1

R2-2404204	Discussion on downlink coverage enhancements	CATT	discussion

Observation 1: If the SSB periodicity is not extended, cell/beam level DTX/DRX mechanism cannot concentrate the beam energy to raise the EIRP of each beam, and thus does not help the DL coverage enhancements target as specified in the WID.

Observation 2: If the SSB periodicity is extended, there would be possibility to concentrate the beam energy to raise the EIRP of each beam which is the goal of the WID with potential beam active/inactive status applied. NES-like DTX/DRX mechanism could be used/adapted to implement the different cell/beam status.

Proposal 1: Ask RAN1 whether the existing SSB pattern for an NR cell (e.g. SSB position in burst, SSB index number, etc.) is changed in Rel-19 NR NTN, and whether the SSB periodicity is extended compared with existing TN values.

-	CEWIT thinks that RAN1 is already working on this and they will inform us

Ask RAN1 to keep us informed on their progress on whether the existing SSB pattern for an NR cell (e.g. SSB position in burst, SSB index number, etc.) is changed in Rel-19 NR NTN, and whether the SSB periodicity is extended compared with existing TN values.

More in general, ask RAN1 whether/how the solution they are investigating is expected to impact common control signalling for UEs in RRC idle / RRC inactive

Also remind RAN1 that satellite beams are currently not visible to UEs and any decision about different beam status will have to relate to beams visible to the UE (e.g. SSB beams)

Observation 3: Whether UL beam hopping that is separate from DL beam hopping is within the scope of WI needs to be clarified by RAN1, and this leads to impacts on whether RAN2 needs to consider a separate cell/beam DRX mechanism.

Proposal 2: Ask RAN1 whether UL beam hopping that is separate from DL beam hopping is within the scope of the WI (which relates to whether separate cell/beam DRX needs to be further considered by RAN2).

Also ask RAN1 whether they are also working on UL beam hopping (and whether this is separate from DL beam hopping)

Proposal 3: RAN2 assumes that no matter whether the SSB periodicity is extended or not, the N1/N2/N3 state of beam footprint should not impact the periodic broadcast of SSB (i.e. N1 beam footprint should not cover any SSB transmission opportunities).

Proposal 4: RAN2 assumes that via proper NW configuration, the UE can determine the common search space (for paging/SI acquisition/RA) and PRACH occasions, and perform related IDLE/INACTIVE procedure based on legacy mechanisms. No impact to UE behaviour is foreseen.

Proposal 5: Ask RAN1 to confirm the assumption in Proposal 3/4.

Proposal 6: Cell/beam DTX/DRX mechanism is not supported for RRC_IDLE/RRC_INACTIVE.

Proposal 7: Send LS to RAN1 with the following questions for clarification:

-	Whether the existing SSB pattern for an NR cell (e.g. SSB position in burst, SSB index number, etc.) is changed in Rel-19 NR NTN, and whether the SSB periodicity is extended compared with existing TN values.;

-	Whether UL beam hopping that is separate from DL beam hopping is within the scope of Rel-19 NR NTN WI;

-	RAN2 assumes that N1/N2/N3 states of beam footprint should not impact the periodic broadcast of SSB, and UE can determine the common search space (for paging/SI acquisition/RA) and PRACH occasions and perform related IDLE/Inactive procedure as in legacy. Whether RAN1 has any concern on this.

R2-2404739	Discussion on system level enhancement for downlink coverage enhancements for NTN	Xiaomi	discussion	Rel-19	NR_NTN_Ph3-Core

Observation 1: For SI transmission alone, with legacy parameter of SI-SchedulingInfo, the total number of beam footprints can be reached by beam hopping within the SI periodicity for set1-1, set1-2 and set1-3.

Observation 2: For Paging alone, with legacy parameter of PCCH and PDCCH and SSB configuration, total number of beam footprints can be reached by beam hopping within the paging cycle for set1-1, set1-2 and set1-3.

Observation 3: When considering SSB, SI message and Paging transmission all together, the total number of beam footprint requirement for set 1-1 and set 1-3 (except set 1-2) can still be met with legacy parameter.

Proposal 1	Send LS to RAN1 to ask, for SSB/SI/paging, whether N1 off state is the period of time outside the configured periodic occasions for SSB/SI/paging broadcasting, resulting from the legacy SSB/SI/Paging occasion configuration (doesn't preclude the possibility of extending the value range of existing parameters). I.e. from UE point of view, satellite beam illumination plan is transparent to UE, and UE doesn't needs to know the dwell time and revisit time interval of beam illumination for SSB/SI/Paging across the coverage. Or, whether it is possible that new TDM pattern (except only extending periodicity) may be introduced for SSB/SI/Paging.

Proposal 2	Send LS to RAN1 to ask, whether the total number of beam footprints and the total number of simultaneously activated beam footprints of set 1-1, 1-2 and 1-3 only refers to satellite Tx beam, or refers to both Tx beam and Rx beam. If it is the first case, then if there is limitation on the total number of beam footprints and total number of simultaneously activated beam footprints for satellite Rx beam. If it is the latter case, what the ratio between them.

Observation 4: For random access, to support beam hopping, TDM pattern of beam illumination is better to be known to UE. And TDM pattern of beam illumination needs to be translated into NR cell or beam TDM pattern.

Proposal 3	For random access to support beam hopping, network is better to provide UE with TDM pattern of NR cell/ NR cell beam on/off representing TDM pattern of beam illumination hopping.

Proposal 4	For active traffic to support beam hopping, it shares the same TDM pattern of NR cell/ NR cell beam on/off as random access.

Proposal 5	Send LS to RAN1 to ask, whether the transition between N2 and N3 is automatic, i.e. based on whether currently there are users with ongoing active traffic, or, is based on network configuration.

More on mapping among cells / satellite beams / SSB beams and other questions to RAN1

R2-2405124	Discussion on DL coverage enhancements	Huawei, HiSilicon, Turkcell	discussion	Rel-19	NR_NTN_Ph3-Core

Proposal 1: The UE can be configured with multiple DTX/DRX patterns associated to different beams, each pattern is characterized by active and non-active periods.

Proposal 2: The UE determines which pattern to apply based on the DL SSB beam.

Proposal 3: Similar to R18 NES, dynamic activation/deactivation mechanism via L1 signalling is introduced.

Proposal 4: Send an LS to RAN1 to confirm which DL signals/channels are affected or need to be determined by RAN2, e.g. SPS PDSCH, UE-specific PDCCH, Periodic/semi-persistent CSI-RS for CSI reporting, Group-common PDCCHs.

Proposal 5: Send an LS to RAN1 to ask whether cell/beam level DRX is needed in NTN and which UL signals/channels are affected or need to be determined by RAN2.

Proposal 6: Send an LS to RAN1 to ask how the common signaling design as a result of beam hopping would affect the beam level DTX/DRX design, e.g. whether a separate pattern for common signaling is needed.

R2-2404159	Discussion on Downlink Coverage Enhancements	vivo	discussion	Rel-19	NR_NTN_Ph3-Core

Observation 1: In NR-NTN deployment, both option a) same PCI for several satellite beams and option b) one PCI per satellite beam are supported. It is up to NW implementation to adopt either option.

Observation 2: In the case one PCI per satellite beam, satellite beam “on”/”off” switching means cell “on”/”off” switching. Then, Rel-18 NES cell DTX techniques can be considered.

Observation 3: In the case same PCI for several satellite beams, the network may turn on/off all the satellite beams associated with the same cell simultaneously. Alternatively, it only activates a subset of satellite beams of a cell.

Proposal 1: RAN2 assumes the following satellite beam deployment options can be considered in system-level DL coverage enhancements:

-	option a) same PCI for several satellite beams;

-	option b) one PCI per satellite beam.

Proposal 2: Send an LS to check with RAN1 on RAN2's assumption of the satellite beam deployment options.

Proposal 3: RAN2 checks with RAN1 whether the case where only a subset of satellite beams belonging to the same cell are “on” while the other satellite beams are “off” is possible.

Proposal 4: RAN2 further discusses the DTX pattern is on a per beam or per cell basis (pending RAN1 input).

Proposal 5: The NW provides DTX information through SIB.

Proposal 6: RAN2 sends an LS to ask RAN1 the UE behaviour impacts on common message reception when a satellite beam is in N1 state (i.e. state “off”).

Proposal 7: For system-level DL coverage enhancements, RAN2 assumes both quasi-earth-fixed system and earth-moving system are considered.

-	IDC thinks there is no need to ask this

RAN2 assumes that both EFC and EMC are supported (no need to send this in the LS to RAN1)

Proposal 8: RAN2 to discuss and check with RAN1 whether any UL transmission is allowed when a satellite beam is in N1 state (i.e. state “off”).

Proposal 9: RAN2 to discuss and check with RAN1 whether UE dedicated UL transmission is allowed when a satellite beam is in N2 state (i.e. state “common messages only”).

Agreements;

- Based on the solution being investigated in RAN1, RAN2 will further discuss whether/how legacy UEs might operate in a cell supporting DL coverage enhancements.

- RAN2 assumes that both EFC and EMC are supported

[AT126][305][R19 NR NTN] LS to RAN1 (CMCC)

Scope: draft an LS to RAN1 based on meeting agreements. Can consider additional questions if there is support (if there is no consensus we just stick to the questions agreed online)

Intended outcome: Approved LS

Deadline for rapporteur's summary (in R2-2405762):  Friday 2024-05-24 08:00

R2-2405762	LS on DL coverage enhancements	CMCC	LS out		LS out	Rel-19	NR_NTN_Ph3-Core	To:RAN1

Revised in R2-2405767

R2-2405767	LS on DL coverage enhancements	CMCC	LS out		LS out	Rel-19	NR_NTN_Ph3-Core	To:RAN1

Approved

R2-2404354	Discussions on beam and cell level DTX DRX	Fujitsu	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2404582	Discussion on DL coverage enhancement in NTN	OPPO	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2404654	DL coverage enhancement in NTN	Apple	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2404682	Discussion on cell DTX/DRX	Qualcomm Incorporated	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2404797	Downlink coverage enhancement in NTN	Lenovo	discussion	Rel-19

R2-2405081	Consideration on downlink coverage enhancements	ZTE Corporation, Sanechips	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2405173	Discussion on Downlink Coverage Enhancement	Samsung	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2405240	Discussion on RAN2 Aspects for Downlink Coverage Enhancements in NR NTN evolution	THALES	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2405299	Questions identified to RAN1 on Downlink Coverage Enhancement	CMCC	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2405312	Downlink Coverage in NR NTN	China Telecom	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2405320	Consideration on downlink coverage enhancement	NEC Corporation.	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2405375	Downlink coverage enhancement for NTN	InterDigital	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2405376	[Draft] LS on DL coverage enhancements	InterDigital	LS out	Rel-19	NR_NTN_Ph3-Core	To:RAN1

R2-2405449	Downlink coverage enhancements	Ericsson	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2405613	Downlink coverage enhancements for NTN	NERCDTV	discussion

R2-2405626	Discussion for DL coverage enhancement	Sharp	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2405636	Discussion on downlink coverage enhancements in NR NTN	ETRI	discussion	Rel-19	NR_NTN_Ph3-Core

Withdrawn

R2-2405600	Downlink coverage enhancements for NTN	NERCDTV	discussion	Withdrawn

### 8.8.3	Uplink Capacity/Throughput Enhancement

No contributions are expected for this AI at this meeting.

### 8.8.4	Support of Broadcast service

Contributions should address the signaling of the intended service area of a broadcast service.

R2-2404841	Support for broadcast services in NR NTN	Ericsson	discussion	Rel-19	NR_NTN_Ph3-Core

Proposal 1	For MBS broadcast service, both EFC and EMC are supported.

Agreed

Proposal 2	Even if the intended MBS broadcast service area is limited to a portion of an NTN cell, the content of the service can be received within the whole cell.

RAN2 will not define means for the NW to prevent the reception of the content of the service outside of the intended service area.

Proposal 3	A UE does not establish MRB(s) associated with a service limited to an intended service area when it is not located within the area.

-	CATT thinks the UE behaviour would be up to UE implementation.

-	Nokia is fine with p3 and p4 and thinks we should have a clear behaviour.

-	HW thinks that the UE should not monitor MCCH/MTCH if the UE is outside the service area

-	Ericsson thinks there is some normative text already now for the TN case about releasing MRBs when leaving the cell

Come back next meeting to check whether a UE may not (or should not) establish MRB(s) associated with a service limited to an intended service area when it is not located within the area (up to UE implementation how the UE detects it’s not in the intended service area)

Proposal 4	A UE releases its established MRB(s) associated with a service limited to an intended service area when exiting the area.

Come back next meeting to check whether a UE may (or should) release its established MRB(s) associated with a service limited to an intended service area when exiting the area (up to UE implementation how the UE detects it’s not in the intended service area)

Proposal 5	MBS broadcast intended service area is provided via system information.

Agreed

Proposal 6	RAN2 considers the following possibilities for including the service area information: SIB20/ SIB21/ MBSBroadcastConfiguration.

-	LG thinks we should consider the possibility of a new SIB

For MBS broadcast RAN2 considers the following possibilities for including the service area information: SIB20/ SIB21/ MBSBroadcastConfiguration. FFS for ETWS

R2-2405082	Consideration on broadcast service enhancements	ZTE Corporation, Sanechips	discussion	Rel-19	NR_NTN_Ph3-Core

Observation 1: It is difficult to adjust the satellite footprint in order to match the MBS broadcast service area, especially considering there could be multiple MBS broadcast service with different service area being provided simultaneously in the cell.

Proposal 1: RAN2 doesn’t pursue solution to limit the broadcast transmission to the intended service area only (i.e. no transmission happens outside of the intended service area).

Observation 2: TN coverage like service area could provide approximately coverage with less signalling overhead, and less specs impact, while geographical area information could better matches with the intended service area of MBS broadcast service area with higher signalling overhead

Observation 3:NW can based on implementation to configure either TN coverage and Geographical area information for different MBS service area to fit with different service requirement.

Proposal 2: Specify signalling support to allow indicating service area in either circles (like for TN coverage) or geographical area information, e.g. via polygons. Similar to TN coverage, an area identity is used to identify a geographical area.

Observation 4: Existing specs already allows provide MBS service over different cells if the MSB service area is a list of cells/TAIs, there is no need to provide additional information to UE.

Observation 5: No need to mix the discussion on how UE is provided with intended broadcast service area and how NW is provided with broadcast service area.

Observation 6: It is already legacy principle that the mapping between mapped cell id and fixed geographical area is only maintained at NW’s side, and NW can based on its implementation to provide suitable information over uu interface, e.g., service area, CGI.

Proposal 3: No need to provide intended service area in the format of cell list/TAI list or mapped cell id over uu interface .

Observation 7: Association between MBS broadcast session with broadcast service area is needed, so that UE can identify MBS services for its location, and select properly the MBS service.

Observation 8: There could be two different options to associated with intended service area (e.g., geographical area/TN coverage) with MBS session:

- Opt1: One intended service area (e.g., geographical area/TN coverage) is mapped to one or more MBS session

- Opt2: One or more intended service area (e.g., geographical area/TN coverage) is mapped to one MBS session

Proposal 4: When intended service area (e.g., geographical area/TN coverage) is provided for MBS broadcast service, it needs to be associated with MBS session.

Agreed (FFS on the details)

Proposal 4a: RAN2 discusses and selects between below two options to allow mapping between broadcast service area information and MBS service session

- Opt1: One intended service area (e.g., geographical area/TN coverage) is mapped to one or more MBS session

- Opt2: One or more intended service area (e.g., geographical area/TN coverage) is mapped to one MBS session

Observation 9: Decision on whether new or existing SIBs are considered can be postpone to when RAN2 proceeds more on the details of the broadcast service contents.

Observation 10: It is beneficial to clarify UE behavior on MBS service reception when intended service area is provided.

Proposal 5: UE can take into account its location information together with the intended service area information when receiving/releasing MBS services, e.g., UE can not acquire or release the MBS service when its location is outside the intended service area of the MBS service.

Agreements:

- For MBS broadcast service, both EFC and EMC are supported.

- RAN2 will not define means for the NW to prevent the reception of the content of the service outside of the intended service area.

- MBS broadcast intended service area is provided via system information

- For MBS broadcast RAN2 considers the following possibilities for including the service area information: SIB20/ SIB21/ MBSBroadcastConfiguration. FFS for ETWS

- When intended service area (e.g., geographical area/TN coverage) is provided for MBS broadcast service, it needs to be associated with MBS session (FFS on the details)

R2-2404580	Discussion on providing MBS service area in NTN network	OPPO	discussion	Rel-19	NR_NTN_Ph3-Core

Observation 1: if one cell covers a large area, different MBS sessions may be available in different sub-areas.

Proposal 1: If broadcast transmission is confirmed to be limited to the intended service area only, RAN2 to agree that different MBS sessions should be associated with different geographical areas, and such information should be indicated in the SIB.

Proposal 2: RAN2 to agree that ETWS in the NTN network could be associated with only certain geographical areas, and such areas could be indicated in associated SIBs, e.g., SIB6 or SIB7, if the satellite footprint consists of one cell or multiple cells with large coverage area.

Observation 2: for the NTN network where the satellite footprint consists of one cell or multiple cells and each of them covers a big area, regarding location dependent broadcast service, more than one contents for the same particular MBS session could be distributed in each cell.

Proposal 3: RAN2 to agree that, for the location dependent broadcast service, the area session ID of each distinguished content of the MBS session and the associated area range should be provided in the MBS configuration information.

Proposal 4: RAN2 to agree that, to support MBS services/contents in cells area of more than one NTN cells (or portions thereof), only the service area covering portions of cells need to be explicitly indicated.

Observation 3: it is common for a UE to receive broadcast service outside the intended service area, for example if its Rx performance is good, or unintended coverage extension of the NTN cell occurs.

Proposal 5: RAN2 to discuss how to capture in the spec for not letting UEs to receive MBS service outside the intended service area.

Proposal 6: RAN2 to agree to reuse the coverage area information included in the SIB25 to provide the information of the intended area for the MBS services and/or MBS session content for the NTN network in the SIB.

R2-2404655	Broadcast service support over NTN	Apple	discussion	Rel-19	NR_NTN_Ph3-Core

Proposal 1: It is possible that the broadcast transmission is limited to the intended service area only.

Proposal 2: If UE outside of intended service area receives the broadcast service, RAN does not enforce any rule for UE to discard the broadcast data (leave to application layer to handle).

Proposal 3: The intended broadcast service area can be defined by one or multiple satellite beam(s) or by a geographical area represented by referenceLocation and radius.

Proposal 4: RAN2 to discuss how to enhance the service continuity matter with intended service area taken into account.

Proposal 5: RAN2 to discuss about service area specific MCCH/MTCH to replace cell specific MCCH/MTCH.

R2-2404160	Discussion on MBS Broadcast Provision in NTN	vivo	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2404206	Discussion on support of broadcast service via NR NTN	CATT	discussion

R2-2404282	Discussion on support of a broadcast service in NR NTN	ETRI	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2404355	Discussions on signaling of the intended service area of a broadcast service	Fujitsu	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2404429	Discussion on support of broadband services	Continental Automotive	discussion

R2-2404621	Discussion on MBS Broadcasting Control over NTN access	TCL	discussion

R2-2404679	MBS broadcast service area information	Qualcomm Incorporated	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2404798	On broadcast service area indication in NTN	Lenovo	discussion	Rel-19

R2-2404853	Discussions on the configuration of intended service areas	ITRI	discussion	NR_NTN_Ph3-Core

R2-2404854	Discussions on limiting broadcast service in the intended service areas	ITRI	discussion	NR_NTN_Ph3-Core

R2-2404916	Broadcast service area signaling	Sony	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2404982	Discussion on the support of broadcast service	HONOR	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2405020	Support of MBS broadcast service for NTN	CMCC	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2405099	Discussion on the service area of a broadcast service	Xiaomi	discussion

R2-2405125	Discussion on supporting MBS broadcast over NTN	Huawei, HiSilicon, Turkcell	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2405147	On the Signalling Aspects of MBS over Rel-19 NR NTN	Nokia	discussion	Rel-19	NR_NTN_Ph3	R2-2403306

R2-2405174	Discussion on Broadcast Service Area in NTN	Samsung	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2405204	Discussions on MBS in Rel-19 NTN	TOYOTA Info Technology Center	discussion	Rel-19

R2-2405211	Discussion on support of broadcast service in NTN	LG Electronics France	discussion	Rel-19	NR_NTN_Ph3	R2-2403121

R2-2405239	Discussion on MBS broadcast additional features for NR NTN Evolution	THALES	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2405277	Clarification on intended service area	NEC Telecom MODUS Ltd.	discussion

R2-2405377	Support for broadcast service in NTN	InterDigital	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2405525	Discussion on support of broadcast service	ITL	discussion	Rel-19

R2-2405627	Discussion on MBS service support for NR NTN	Sharp	discussion	Rel-19	NR_NTN_Ph3-Core

### 8.8.5	Support of regenerative payload

Contributions should focus on the needed updates for Stage 2 description and on whether any existing essential features would be affected - and potentially need any modifications - in a regenerative payload architecture.

Possibly impacted features (SDT, Re-establishment, satellite switch with resync, NW verified UE location)

R2-2405156	Regenerative payload for NR NTN	Samsung	discussion	Rel-19	NR_NTN_Ph3-Core

Proposal 2: RAN2 to study the issue of RRC re-establishment in a regenerative payload scenario.

-	Samsung thinks that there are no additional issues wrt TN case. LG agrees

-	vivo thinks that if there are any issues they should be discussed in RAN3 first. CATT agrees

-	Lenovo thinks either we ask RAN3 or we can think about possible enhancements in RAN3

-	Ericsson indicates that RAN3 is looking into this and so far the issues is considered similar to the one for TN

Regarding potential issues with RRC re-establishment, we wait for RAN3 input, if any

Proposal 3: Do not optimize RA-SDT procedure for regenerative payload architecture.

-	QC thinks we could do something to prevent the UE from wasting power

-	IDC agrees with p3 and wonders about the difference with transparent case

-	LG agrees with p3. Nokia also agrees

-	HW thinks that there could be some possible cases when there is a power astin issue but does not fully agree with the QC use case

Proposal 4: Satellite switch with resync feature is not supported in a regenerative payload architecture.

Proposal 5: Do not consider any optimization to network-verified location for regenerative payload.

R2-2404799	Considerations on regenerative payload in NTN	Lenovo	discussion	Rel-19

Proposal 1: RAN2 to discuss how to avoid RRC reestablishment to a gNB that cannot retrieve UE context from last serving gNB due to that no connection can be maintained between gNBs.

Proposal 2: RAN2 to discuss how to optimize for UE in RRC_INACTIVE in case the UE context cannot be retrieved from last serving gNB due to that no connection can be maintained between gNBs.

Regarding potential issues with support of Inactive state, we wait for RAN3 input, if any

R2-2405126	Discussion on support of regenerative payload	Huawei, HiSilicon, Turkcell	discussion	Rel-19	NR_NTN_Ph3-Core

Observation 1: There may not always be a DL SDT packet associated with the UL SDT transmission and the SDT procedure might be terminated by the network due to inactivity in the UL so that the UE stops monitoring PDCCH after sending the ACK for the contention resolution MAC CE and then misses the RRCRelease message.

Proposal 1: Add stage-2 description to clarify that the satellite switch with re-sync is not supported in the regenerative payload.

R2-2405021	Support of regenerative payload	CMCC	discussion	Rel-19	NR_NTN_Ph3-Core

Observation 1: PCI unchanged solution is beneficial for signaling overhead reduction in NTN system due to without L3 mobility.

Proposal 2: To support PCI unchanged for regenerative payload with gNB on board, keeping same security key between source satellite gNB and target satellite gNB based on ISL coordination is demanded.

-	LG wonders if different satellites can use the same PCI. CMCC thinks this is possible

-	Nokia thinks satellite switch with resync could be supported in this case, but with impacts on RAN3 and SA3 (and this would probably need a new WI). Ericsson agrees

-	Thales thinks we should support this feature in a clean way, involving RAN3 and SA3. CMCC agrees

-	Ericsson thinks we should not support satellite switch with resync with regenerative payload

-	Ericsson thinks that the possible involvement of other groups needs to be discussed in the plenary. LG agrees

Companies interested to support satellite switch with resync with regenerative payload are invited to bring this discussion to the RAN plenary

Proposal 3: The following 2 options could be discussed for UE context transmission from source satellite gNB to target satellite gNB:

- Option 1: transmission whole UE context (PDCP/RLC/MAC/PHY related configuration information)

- Option 2: only transmission static part of UE context (PDCP/RLC/MAC/PHY related configuration information) and flush&reestablish dynamic part of UE context (e.g., HARQ buffer status in MAC, timer value, counter value in MAC, PHY related information)

Proposal 4: Both hard and soft satellite switch could be discussed or at least hard switch which is more simple in quasi-earth fixed cell case could be the starting point.

R2-2405148	On the feasibility of the existing NTN features over regenerative architecture	Nokia	discussion	Rel-19	NR_NTN_Ph3

Observation 1: Satellite switching with resynchronization was specified in Rel-18 for transparent payloads to reduce radio signalling load.

Observation 2: In a regenerative architecture, satellite switching under the assumption of single gNB is no longer valid since gNBs are on board of the satellites.

Observation 3: In a regenerative architecture and regardless of the mobility procedure, the UE will frequently switch between gNBs on board of different satellites, triggering additional AMF/UPF signalling (e.g., for path switching).

Observation 4: Connected mobility with regenerative payloads is possible with existing mobility mechanisms, i.e. CHO and HO procedures.

Observation 5: During the satellite switching, the UE behaviour remains unchanged regardless of the NTN architecture (apart for the security config).

Observation 6: The source and target gNBs can use the same cell configuration.

Observation 7: Large part of the coordination between gNBs will be also required for conventional L3 mobility.

Observation 8: For satellite switching with regenerative payload, Rel-19 UE can benefit from similar gains as in transparent architecture while NW complexity may increase compared with regular L3 procedures (to be checked with RAN3 and SA3 for security issues).

Proposal 1: RAN2 to discuss the feasibility of the satellite switching with resync with regenerative payload.

Proposal 2: If satellite switching with resync with regenerative payload is found to be feasible, RAN2 to endorse Figure 4 as baseline for the satellite switching with regenerative payload.

Observation 9: NW may send scheduling resources over PDCCH for a UE that has not yet completed the satellite switching.

Observation 10: The RACH less handover feature works for regenerative architecture, while few optimisations are possible, like on informing the target gNB when the UE is present in the target cell.

Proposal 3: RAN2 to discuss optimisation to support RACH-less handover for regenerative architecture.

RAN2 aims at supporting RACH-less handover for regenerative architecture. Can further discuss whether any optimization is needed

-	LG thinks that RACH-less HO optimizations have been excluded. Nokia thinks this is true but for R18.

Observation 11: Network verified UE location works also under regenerative architecture as none of the assumptions for that feature to work have changed in this architecture option.

Proposal 4: RAN2 to conclude that network verified UE positioning is supported for regenerative architecture without adjustments.

RAN2 assumes that network verified UE positioning is supported for regenerative architecture

R2-2404680	Discussion on regenerative payload	Qualcomm Incorporated	discussion	Rel-19	NR_NTN_Ph3-Core

Observation 1.	The satellite switch with resync feature may be supported in regenerative payload architecture by network implementation of mirror satellite at t-Service.

Proposal 1	From RAN2 perspective, whether to indicate the support of satellite switch with resync for regenerative payload is left to network implementation. The only RAN2 specification impact is to clarify the +ve gap of satellite switch i.e., t-ServiceStart can occur after t-Service.

Proposal 2	For UE power saving when using RA SDT in regenerative payload, the PDCCH monitoring for further RRC message is delayed by UE-gNB RTT after sending HARQ feedback of the contention resolution MAC CE.

-	Apple thinks would change the RA SDT procedure

-	vivo thinks a similar discussion was done for RA-SDT in TN and there was no consensus to change

Proposal 3	RAN2 study if frequent gNB switch is an issue for network verified UE location.

R2-2405196	Support  Regenerative Payload	NEC	discussion	Rel-19	NR_NTN_Ph3-Core

Proposal 1: common TA is zero in case of regenerative mode with full gNB on board

-	CATT agrees with p1. Ericsson thinks we don’t have to specify this.

-	QC thinks we can say that common RA “can” be zero. Also Kmac

-	CATT thinks this should be zero.

RAN2 assumes that the typical setting for common TA and Kmac would be zero in case of regenerative mode with full gNB on board. FFS whether we capture in the spec that common TA and Kmac shall/should be set to zero.

Proposal 2a: RAN2 discuss if we support RP is not at gNB with regenerative mode, , i.e., Kmac can be configured as a non-zero value.

-	CATT thinks the RP is at the satellite

Proposal 2b: RAN2 send LS to RAN1 ask for any necessary RAN1 spec update in order to support RP not at gNB with regenerative mode

Agreements:

- Regarding potential issues with RRC re-establishment, RAN2 will wait for RAN3 input, if any

- Regarding potential issues with support of Inactive state, RAN2 will wait for RAN3 input, if any

- RAN2 aims at supporting RACH-less handover for regenerative architecture. Can further discuss whether any optimization is needed

- RAN2 assumes that network verified UE positioning is supported for regenerative architecture

- RAN2 assumes that the typical setting for common TA and kmac would be zero in case of regenerative mode with full gNB on board. FFS whether we capture in the spec that common TA and Kmac shall/should be set to zero.

Stage 2 updates

R2-2405448	Stage 2 updates for regenerative payload	Ericsson	discussion	Rel-19	NR_NTN_Ph3-Core

Proposal 1	Clarify that the figure in 16.14.2.1 is for transparent payload and add on sentence “With regenerative payload, the network may configure Kmac and common TA to zero.”

Proposal 2	Consider the text proposal in section 4

Noted

R2-2405100	Discussion on the support of regenerative payload	Xiaomi	discussion

Proposal 1: We suggest remove the ISL in the section 16.14.1 and wait for the conclusion on ISL from RAN3.

Proposal 2: We suggest remove the Figure B.4-2 and wait for the RAN3 input for the figure on NTN based NG-RAN with regenerative NTN payload.

R2-2404205	Further discussion on regenerative payload	CATT	discussion

R2-2405241	Regenerative NTN payload support in NR NTN Evolution	THALES	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2405313	Stage-2 updates for regenerative payload	China Telecom	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2404161	Discussion on Mobility with Regenerative Payload	vivo	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2404256	Further discussion on regenerative payload for satellite switch with resync	NTU	discussion	Rel-19	Late

R2-2404590	Discussion on satellite switch with resynch for regenerative payload	OPPO	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2404917	Satellite switch with re-sync in regenerative payload	Sony	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2405083	Consideration on support of regenerative payload	ZTE Corporation, Sanechips	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2405629	Discussion for regenerative payload	Sharp	discussion	Rel-19	NR_NTN_Ph3-Core

Withdrawn

R2-2404622	Discussion on Support of NTN Regenerative Architecture	TCL	discussion	Withdrawn

### 8.8.6	LTE to NR NTN mobility

Support for idle mode mobility between LTE and NR NTN.

R2-2404211	Support of Idle Mode Mobility from EUTRA TN to NR NTN	CATT	discussion

Observation 1: Mobility from LTE to NR is not supported for NB-IoT UE and BL UE in the current Specs.

Proposal 1:	For idle mode mobility from EUTRA TN to NR NTN, neither eMTC UEs nor NB-IoT UEs are considered in the scope.

-	IDC is fine to remove NB-IoT. Removing eMTC would have additional specification impacts

-	Telit thinks that we should support p1 as NB-IoT and eMTC are only connected to the EPC. QC thinks this about idle mode support so no issue

For idle mode mobility from EUTRA TN to NR NTN, NB-IoT UEs are considered not in the scope.

For idle mode mobility from EUTRA TN to NR NTN, we don’t consider specific optimizations for BL UEs and UEs in CE.

Observation 2: The cell reselection from NR TN to NR NTN is already supported in NR Spec, and consists of the following key parts:

-	Step 0: SI reception (cell reselection related information from SIB2/3/4 and satellite assistance information in SIB19);

-	Step 1: Neighbor cell measurement initiation (based on RRM measurement results i.e. same as cell reselection to TN cells in legacy);

-	Step 2: NTN neighbor cell measurement (with UE autonomous SMTC adjustment based on SMTC configuration in SIB2/4 and satellite assistance information in SIB19);

-	Step 3: Cell reselection evaluation (as in legacy based on RRM measurement results).

Proposal 2: SIB24 is reused to provide the NR NTN cell reselection related information (e.g. frequency information, SMTC config, etc.).

-	HW supports this

SIB24 is reused to provide the NR NTN cell reselection related information (e.g. frequency information, SMTC config, etc.), introducing a satellite ID list in per frequency. The EUTRA cell provides the satellite assistance information for NR neighbor cell per satellite, as identified by the satellite ID.

Proposal 3: To support the idle mode mobility from EUTRA TN to NR NTN, the satellite assistance information for NR NTN neighbor cells is needed and should include the following parameters:

-	Satellite ephemeris information

-	TA common information

-	k-Mac

-	epoch time

-	validity duration

-	ntn-PolarizationDL.

-	CATT highlights that UL-Polarizaiton is not needed

-	QC wonders if we can say that we include NTN-config

-	Ericsson supports the proposal but thinks ntn-PolarizationDL is not strictly needed and could be made optional

Agreed (FFS if ntn-PolarizationDL is mandatory or optional)

Observation 3: Current satellite assistance information defined in SIB33 in TS36.331 does not include the ntn-PolarizationDL, which is needed for NR satellite.

Observation 4: Current satellite assistance information defined in SIB33 for IoT NTN has the same value range as that defined in TS38.331 for NR NTN, except for the TA common related configurations.

Proposal 4: The Ephemeris information/epoch time/k-mac/validity duration defined in SIB33 specified in TS36.331 should be reused for NR satellite assistance information.

-	QC wonders if this would increase the TB size for eMTC

-	Xiaomi thinks using SIB19 would be better

-	ZTE supports this proposal.

The Ephemeris information/epoch time/k-mac/validity duration IEs defined in SIB33 specified in TS36.331 should be reused for NR satellite assistance information.

Proposal 5: The ntn-PolarizationDL and TA common related configurations within NTN-Config specified in TS38.331 should be introduced in TS36.331 for NR satellite assistance information.

The signalling format for ntn-PolarizationDL and TA common related configurations within NTN-Config specified in TS38.331 should be introduced in TS36.331 for NR satellite assistance information.

Proposal 6: The EUTRA cell provides the satellite assistance information for NR neighbor cell per satellite, as identified by the satellite ID. Introduce a satellite ID list in the SIB24 per frequency.

Proposal 7: RAN2 further down-selects the following options on how to provide the NR satellite assistance information:

-	Option 1: Introduce a new SIB to include the NR satellite assistance information.

-	Option 2: Define new IE for NR satellite assistance information and define separate neighbour satellite information list to provide the NR satellite information in SIB33.

-	Option 3: Extend the NeighSatelliteInfo defined for IoT NTN to include the parameters needed for NR satellite, and reuse the neighSatelliteInfoList defined in SIB33 to provide either NR or IoT NTN information.

-	QC support option 1

-	Lenovo thinks we should preclude option 3.

-	Nokia thinks that option 1 should be the last option and think we could pack the new IEs in existing SIBs. Vivo agrees

-	HW thinks we should not exclude option 3

-	Samsung thinks we could go for option 3

-	CATT wonders if we need to clarify that only some UEs will consider the new extension (if we fo for option 3)

Take a final decision between different options in the next meeting after seeing the actual TP

Proposal 8: NR NTN neighbor cell measurement initiation is performed as in legacy, i.e. based on RRM measurement as in cell reselection from EUTRA TN to NR TN. No spec impact is needed on cell reselection procedure in TS 36.304.

-	Samsung thinks we should check this more until the next meeting. Ericsson agrees

Come back in the next meeting

Proposal 9: Confirm the working assumption "WA: we don’t introduce multiple SMTCs in LTE".

Proposal 10: Introduce the clarification in the field description of measTimingConfig (configured via SIB24 in TS 36.331) that it is configured based on the assumption that the gNB-UE propagation delay equals to 0 ms, and UE can adjust the offset based on the actual propagation delay, when the corresponding frequency is associated with a satellite ID.

-	Ericsson agrees and thinks we should add this is for NTN only in the field description

-	HW also supports this

Agreed

Proposal 11: NR NTN cell reselection evaluation is based on RRM measurements as legacy; no spec impact foreseen for EUTRA TN to NR NTN cell.

-	Ericsson suggest to come back to this in the next meeting

Working Assumption: NR NTN cell reselection evaluation is based on RRM measurements as legacy; no spec impact foreseen for EUTRA TN to NR NTN cell (can come back in the next meeting to see if the WA can be confirmed)

Agreements:

- For idle mode mobility from EUTRA TN to NR NTN, NB-IoT UEs are considered not in the scope.

- For idle mode mobility from EUTRA TN to NR NTN, we don’t consider specific optimizations for BL UEs and UEs in CE.

- SIB24 is reused to provide the NR NTN cell reselection related information (e.g. frequency information, SMTC config, etc.), introducing a satellite ID list in per frequency. The EUTRA cell provides the satellite assistance information for NR neighbor cell per satellite, as identified by the satellite ID.

4.	To support the idle mode mobility from EUTRA TN to NR NTN, the satellite assistance information for NR NTN neighbor cells is needed and should include the following parameters:

-	Satellite ephemeris information

-	TA common information

-	k-Mac

-	epoch time

-	validity duration

-	ntn-PolarizationDL (FFS if mandatory or optional)

5.	The Ephemeris information/epoch time/k-mac/validity duration IEs defined in SIB33 specified in TS36.331 should be reused for NR satellite assistance information.

6.	The signalling format for ntn-PolarizationDL and TA common related configurations within NTN-Config specified in TS38.331 should be introduced in TS36.331 for NR satellite assistance information.

7.	RAN2 will decide in the next meeting which of the following options to adopt for the provision of the NR satellite assistance information (based on TPs provided by the WI RRC Rapporteur):

Option 1: Introduce a new SIB to include the NR satellite assistance information.

Option 2: Define new IE for NR satellite assistance information and define separate neighbour satellite information list to provide the NR satellite information in SIB33.

Option 3: Extend the NeighSatelliteInfo defined for IoT NTN to include the parameters needed for NR satellite, and reuse the neighSatelliteInfoList defined in SIB33 to provide either NR or IoT NTN information.

8.	Introduce the clarification in the field description of measTimingConfig (configured via SIB24 in TS 36.331) that it is configured based on the assumption that the gNB-UE propagation delay equals to 0 ms, and UE can adjust the offset based on the actual propagation delay, when the corresponding frequency is associated with a satellite ID.

Working Assumption:

- NR NTN cell reselection evaluation is based on RRM measurements as legacy; no spec impact foreseen for EUTRA TN to NR NTN cell (can come back in the next meeting to see if the WA can be confirmed)

R2-2404681	Idle mode mobility from LTE to NR NTN	Qualcomm Incorporated	discussion	Rel-19	NR_NTN_Ph3-Core

Proposal 1	eMTC UE and NB-IoT UE can also be considered to be in scope.

Proposal 2	Confirm that LTE to ATG NR cell mobility, i.e., broadcasting information of SIB22 is not in scope.

Proposal 3	Introduce satellite ID in SIB24 to indicate the NR frequency is associated with the satellite frequency.

Proposal 4	Introduce a new SIB to broadcast NR NTN satellite information.

Proposal 5	Confirm that multiple satellites associated with the same frequency can have different SMTC offsets.

R2-2404162	Discussion on LTE TN to NR NTN Mobility	vivo	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2404198	Support for LTE to NR-NTN idle mode mobility	Telit Communications S.p.A. ; Thales	discussion	R2-2403066

R2-2404591	Discussion on LTE to NR NTN idle mode mobility	OPPO	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2404656	Mobility from LTE TN to NR NTN	Apple	discussion	Rel-19	LTE_TN_NR_NTN_mob-Core

R2-2404759	Discussion on cell reselection from E-UTRA TN to NR NTN	MediaTek Inc.	discussion	NR_NTN_Ph3-Core	R2-2403226

R2-2404800	On LTE to NR-NTN IDLE mobility	Lenovo	discussion	Rel-19

R2-2404840	E-UTRAN TN to NR-NTN mobility	Ericsson	discussion	Rel-19	LTE_TN_NR_NTN_mob

R2-2404986	Further discussion on idle mode cell reselection form LTE to NR NTN	Transsion Holdings	discussion	Rel-19

R2-2405022	Considerations on cell reselection enhancements from E-UTRAN TN to NR-NTN	CMCC	discussion	Rel-19	LTE_TN_NR_NTN_mob

R2-2405084	Consideration on idle mode mobility between LTE TN and NR NTN	ZTE Corporation, Sanechips	discussion	Rel-19

R2-2405101	Discussion on the cell reselection from LTE to NR NTN	Xiaomi	discussion

R2-2405108	Discussion on LTE to NR NTN mobility	Interdigital, Inc.	discussion	Rel-19	LTE_TN_NR_NTN_mob

R2-2405127	Discussion on LTE to NR NTN mobility	Huawei, HiSilicon, Turkcell	discussion	Rel-19	LTE_TN_NR_NTN_mob-Core

R2-2405146	On How to Address E-UTRA TN to NR NTN Mobility in IDLE mode	Nokia	discussion	Rel-19	NR_NTN_Ph3

R2-2405155	E-UTRAN TN to NR NTN mobility basic scenario and signalling	Samsung	discussion	Rel-19	LTE_TN_NR_NTN_mob-Core

R2-2405210	Discussion on support of LTE to NR NTN cell reselection	LG Electronics France	discussion	Rel-19	LTE_TN_NR_NTN_mob	R2-2403123

R2-2405314	Consideration of LTE TN to NR NTN mobility	China Telecom	discussion	Rel-19	NR_NTN_Ph3-Core

R2-2405630	Discussion for LTE to NR NTN mobility	Sharp	discussion	Rel-19	LTE_TN_NR_NTN_mob-Core

## 8.9	IoT NTN Ph3

(IoT_NTN_Ph3-Core; leading WG: RAN2; REL-19; WID: RP-240776)

Time budget: 1 TU

Tdoc Limitation: 2 tdocs

### 8.9.1	Organizational

LS, Rapporteur input, including workplan, etc.

R2-2404144	LS to SA2 and RAN2 on selected satellite architecture for Store and Forward (S3-241567; contact: InterDigital)	SA3	LS in	Rel-19	FS_5GSAT_Ph3_SEC	To:SA2, RAN2	Cc:SA, RAN3, SA3-LI

-	Oppo and MTK think there is no need to reply to SA3 and also not to SA2

-	HW thinks we can decide whether to send a reply LS based on the progress of this meeting. Nokia agrees.

-	QC thinks we should send an LS to SA2

Noted

R2-2405378	[Draft] Reply LS on selected satellite architecture for Store and Forward	InterDigital	LS out	Rel-19	IoT_NTN_Ph3-Core	To:SA3	Cc:SA2

-	Ercisson thinks we don’t need to sned na LS now.

### 8.9.2	Support of Store & Forward

Contributions should focus on possible impacts to the radio interface.

R2-2404321	Overview of the Store and Forward satellite operation	Huawei, HiSilicon, Turkcell 	discussion	Rel-19	IoT_NTN_Ph3-Core

- Basic procedure for S&F satellite operation

Proposal 1: RAN2 can start from Modelling 1, i.e., after sending the RRC connection request, UE is served by the same satellite/eNB before being released to IDLE. Modelling 2 (involving change of serving eNB) can be discussed after sufficient progress is made in RAN3/SA2.

-	MTK supports this

-	ZTE wonders if this means we take single satellite as a baseline. HW thinks this is not the intention but think this is where we can start from, pending further progress in other groups

-	Oppo and CATT think we first need to discuss when the UE will be sent to idle.

-	ZTE thinks we should not discuss the Attach procedure but only on AS signalling.

-	Nokia thinks this proposal does not assume a single satellite

Proposal 2: For the uplink/downlink messages transmission, the following steps are taken as baseline for S&F satellite operation:

1) The UE sends uplink data/signalling to eNB when service link is available and the eNB stores it.

2) When feeder link is available, the eNB sends the uplink data/signalling to the CN.

3) The eNB receives the downlink data/signalling from the CN and stores it when feeder link is available.

4) The eNB sends the downlink data/signalling to the UE when service link is available again.

-	Apple thinks this is more for MO, what about MT?

-	CATT supports this

For the uplink/downlink messages transmission for MO, from RAN2 perspective the following steps are taken as baseline for S&F satellite operation (in case only eNB is on the satellite):

- 1) The UE sends uplink data signalling to eNB when service link is available and the eNB stores it.

- 2) When feeder link is available, the eNB sends the uplink data/NAS signalling to the CN.

- 3) The eNB (same or different) receives the downlink data/NAS signalling from the CN and stores it when feeder link is available (and service link is not available).

- 4) The eNB (same or different) sends the downlink data/signalling to the UE when service link is available again.

-	S&F indication and access control

Proposal 3: S&F indication via AS is supported. FFS if any S&F indication via NAS is needed.

-	Samsung thinks this is fine but it could be optional for the NW to signal this

-	Oppo thinks we don’t need a NAS indication

S&F indication can be provided by SIB (FFS on the details).

RAN2 assumes that no NAS indication is needed

Proposal 4: A new S&F barring indication is broadcasted in the SIB which can implicitly indicate the S&F satellite operation.

-	Oppo supports p4 and wonders if this the same as the indication in p3. HW thinks this could be the same. Telit thinks it could be different

-	Novamint thinks we should not have a barring indication

-	Samsung supports p4 and thinks that when not in S&F mode the barring is not set

-	ZTE thinks there is no need for a new barring indication for R19 UEs supporting this feature. IDC agrees. Xiaomi also agree

Proposal 5: Rel-19(+) UEs not supporting S&F should consider a cell performing S&F satellite operation as barred.

-	vivo thinks we should check this with SA2

-	Lenovo/Novamint think that SA2 is considering to support legacy UEs during S&F operation

-	QC thinks this might also depend on whether (part of) the CN will also be on board or not

Proposal 6: Legacy UEs should be barred by cellBarred and cellBarredNTN in a cell performing S&F satellite operation.

RAN2 understands legacy UEs may be barred by legacy cellBarred and cellBarredNTN

Proposal 7: RAN2 to discuss UE’s RRC state and the related behaviours when service link is not available for S&F satellite operation.

-	ZTE thinks we can keep the existing UE RRC states

Agreements:

1.	For the uplink/downlink messages transmission for MO, from RAN2 perspective the following steps are taken as baseline for S&F satellite operation (in case only eNB is on the satellite):

1) The UE sends uplink data signalling to eNB when service link is available and the eNB stores it.

2) When feeder link is available, the eNB sends the uplink data/NAS signalling to the CN.

3) The eNB (same or different) receives the downlink data/NAS signalling from the CN and stores it when feeder link is available (and service link is not available).

4) The eNB (same or different) sends the downlink data/signalling to the UE when service link is available again

2.	S&F indication can be provided by SIB (FFS on the details). RAN2 assumes that no NAS indication is needed

3.	RAN2 understands legacy UEs may be barred by legacy cellBarred and cellBarredNTN

R2-2405193	Radio Interface Aspects for Store And Forward mode operation of IoT-NTN	Nokia, Nokia Shanghai Bell	discussion

- SF Mode indication

Proposal 1:  SF mode indication via AS signalling (i.e system information) is considered as baseline. RAN2 to exclude NAS based signalling for SF mode indication.

Proposal 2:  RAN2 to discuss the options for indication of mode transition (from SF to Normal and vice-versa) towards UE.

- Scenarios And Access Control

Proposal 3:  RAN2 to identify the application scenarios that require specific changes for the radio interface operation in S&F mode.

Proposal 4:  RAN2 to investigate access control and scheduling related changes for S&F operation.

- Multi-satellite Operation for SF IoT-NTN

Proposal 5:  RAN2 to confirm multi-satellite operation as basis for further analysis of store & forward mode.

Proposal 6:  Direct ISL between NTN-eNBs is not considered for multi-satellite operation in store & forward mode.

Proposal 7:  RAN2 to consider paging enhancements in SF mode for delivering ACK for MO Traffic and MT Traffic towards IoT-NTN UE.

- Security Aspects

Proposal 8:  For User Plane solution for IoT-NTN in S&F mode, security handling of same UE context across two successive NTN nodes without direct connectivity needs to be investigated further.

R2-2404683	S&F satellite operation with full eNB as regenerative payload	Qualcomm Incorporated	discussion	Rel-19	IoT_NTN_Ph3-Core

Observation 1.	eNB only onboard satellite solution has the issue of very long attach or TAU update delay in addition to MT data delivery/paging, roaming and security threat.

Proposal 1	Send LS to SA2 asking on any core network impacts and security threat of eNB only onboard satellite solution and also asking whether RAN2 can start working on the solution.

R2-2404199	Considerations on S&F operation from device perspective	Telit Communications S.p.A.	discussion	Revised

Revised in R2-2404979

R2-2404979	Considerations on S&F operation from device perspective	Telit Communications S.p.A., Novamint, Sateliot	discussion	R2-2404199

Proposal 1: UE should receive an indication whether S&F is supported by the satellite.

Proposal 2: UE should receive an indication whether RT is momentarily supported by the satellite.

Proposal 3: For S&F Satellite operation expected maximum delivery time should be indicated to the UE.

Proposal 4: In case both transfer methods are available, UE should be able to decide/ indicate whether data transfer should be performed by S&F or RT.

Proposal 5: Satellite should be able to indicate (broadcast) whether S&F or RT operation are momentarily available or unavailable/barred.

R2-2404885	Discussion on the store and forward operation	Google Inc.	discussion	Rel-19

Proposal 1	The UE RRC should be able to identify a cell being in one of the following S&F scenarios:

S-1) The cell does not support the S&F satellite operation (i.e., feeder link is always available);

S-2) The cell supports the S&F satellite operation, and the feeder link is currently available;

S-3) The cell supports the S&F satellite operation, and the feeder link is currently not available.

Proposal 2	The legacy UE and the Rel-19 UE not supporting the S&F satellite operation shall refrain from accessing a cell under S-3 (i.e., a cell with no feeder link available).

Proposal 3	When the legacy NTN barring indication is set as ‘barred’, Rel-19 UEs need to check a new indication (in SIB1) reflecting the feeder link status to determine whether the cell can be accessed or not.

Proposal 4	Rel-19 UE can inform eNB of its capability in supporting the S&F satellite operation via MSG3.

Proposal 5	When the feeder link is not available, eNB can suspend UE’s RRC connection by transmitting an RRC Connection Release message to the UE supporting the S&F satellite operation. FFS whether a new release cause is needed.

R2-2404589	Discussion on Store & Forward satellite operation	OPPO	discussion	Rel-19	IoT_NTN_Ph3-Core

Observation 1: In S&F scenario, no inter-satellite link would be assumed otherwise when connecting with the UE via service link, serving satellite can still reach the ground network via other satellite’s forwarding operation and thus no storing operation at satellite is needed.

Proposal 1	For UP solution, UE only connects to its anchor satellite which stores UE’s context.

Proposal 2	Anchor satellite is identified by the satellite ID associated with the serving cell which releases UE to IDLE mode with suspend configuration.

Proposal 3	The legacy cellbarred-NTN should be set as barred in the cell with S&F satellite operation.

Proposal 4	Separate cellbarred indication is introduced in SIB for Rel-19 UEs supporting S&F operation.

R2-2404163	RAN2 Aspects for Store & Forward	vivo	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2404202	Discussion on support of store and forward operation	CATT	discussion

R2-2404436	Elements of Store & Forward Operation	PANASONIC	discussion

R2-2404657	Support of S&F operation in IoT NTN	Apple	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2404801	Store and Forward support in IoT NTN	Lenovo	discussion	Rel-19

R2-2404882	RAN2 impacts of supporting Store&Forward operation in IoT NTN	ZTE Corporation, Sanechips	discussion	Rel-19	IoT_NTN_Ph3-Core	R2-2402380

R2-2404987	Discussion on support of Store&Forward	Transsion Holdings	discussion	Rel-19

R2-2405012	Discussion on IoT NTN Store and Forward	CMCC	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2405102	Discussion on the support of store and forward satellite operation	Xiaomi	discussion

R2-2405132	RAN2 impact on S&F mode	MediaTek Inc.	discussion	IoT_NTN_Ph3-Core	R2-2402942

R2-2405153	On RAN2 aspects of Store and Forward	Samsung	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2405197	Support  of Store and Forward	NEC	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2405242	Considerations on Store & Forward Satellite Operation	SHARP Corporation	discussion	Rel-19

R2-2405315	The impact of access for Store & Forward in IoT NTN	China Telecom	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2405429	Discussion on information for Store & Forward	ASUSTeK	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2405450	Support for store and forward	Ericsson	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2405657	Discussion of the Store and Forward satellite operation	TCL	discussion

Withdrawn

R2-2404409	Overview of the Store and Forward satellite operation	Huawei, HiSilicon	discussion	Rel-19	IoT_NTN_Ph3-Core	Withdrawn

### 8.9.3	Uplink Capacity Enhancement

At this meeting contributions should only focus on the possible enhancements to reduce the necessary uplink and downlink signaling to complete an EDT transaction (Msg3 transmission without msg1/RAR; efficient delivery of msg4 / RRCEarlyDataComplete).

R2-2405202	Discussion on Msg3-EDT enhancements	ESA	discussion	Rel-19

Observation 1: DSA is improving the Msg3 EDT (without msg1) throughput by about a factor of 6 without any impacts on the current receiver implementation at the network, and minimal specification impact overall.

Observation 2 CRDSA is improving the legacy SA-based throughput of about a factor of 80 for balanced power packets and more than a factor of 100 for unbalanced packets power, which is highly relevant in NTN IoT scenarios.

Proposal 1: RAN2 to study enhanced random access techniques, such as Diversity Slotted ALOHA (DSA) and Contention Resolution Diversity Slotted Aloha (CRDSA), for improved random-access performance of Msg3-EDT transmissions without msg1/ Random Access Response (RAR).

-	Xiaomi thinks this is interesting but more applicable for 6G and not only for IoT NTN

-	Oppo thinks this should be discussed in RAN1

-	vivo thinks we should first discuss whether to support contention based UL transmission

-	Samsung thinks there is quite some gain and it’s worth continuing discussing this.

-	Inmarsat welcomes this proposal, using a technique already used in satellite communication. Also thinks there is limited impact to the NW and almost no impact to the UE. Novamint agrees

-	Ericsson strongly supports the proposal and suggests to continue the study

-	ZTE thinks this is probably too new for R19 and cannot exclude RAN1 impacts. Also thinks this scheme might have drawbacks in terms of latency. Nokia agrees and thinks that in case we need to consult with RAN1

-	QC thinks existing mechanism could eventually lead to even higher latency due to collisions (ESA agrees). Also thinks we could send an LS to RAN1 but we could also progress in RAN2 independently.

-	Iridium thinks this has a lot of value and supports continuing studying this. Eutelsat Group agrees

-	HW thinks more evaluation is needed in RAN1

-	MTK thinks that RAN2 can continue the study on this and if needed send an LS to RAN1 later. ESA agrees

Can continue the discussion on this proposal, evaluating possible impacts on the specification, in the next RAN2 meeting (we might send an LS to RAN1 later on this)

R2-2404842	UL capacity enhancements objectives for IoT NTN	Ericsson	discussion	Rel-19	IoT_NTN_Ph3-Core

Observation 1	Enhancements to PUR are not with the Rel-19 IoT NTN WID’s scope.

Observation 2	Enhancing PUR for NTN may require substantial changes that could contradict legacy PUR enabling conditions, thereby increasing complexity.

Proposal 1	RAN2 prioritizes enhancements to the EDT procedure in Rel-19.

-	Ericsson thinks that PUR has a lot of preconditions that might not be valid in NTN and then we should focus on EDT

-	Xiaomi thinks this is more a terminology issue. QC has a similar view, the final solution could be a combination of EDT and PUR (maybe with a different name)

-	Lenovo supports this proposal

-	Although ZTE had a different proposal, thinks we could focus on the procedure rather than on the name of the feature

-	Toyota thinks this is not only a matter of terminology, as PUR refers to pre-allocated resources

RAN2 focusses the study on contention-based Msg3 transmission to complete an EDT-like transaction (FFS on the details of Msg3. FFS on the procedural steps, e.g. how much we reuse of EDT and PUR procedures. FFS on allocation of resources).

R2-2404884	Consideration on enhanced early data transmission in IoT NTN	ZTE Corporation, Sanechips	discussion	Rel-19	IoT_NTN_Ph3-Core	R2-2402381

Proposal 1: RAN2 clarify that the main task of the objective “to reduce the necessary uplink and downlink signaling to complete an Early Data Transmission (EDT) transaction” is to support contention-based Msg3 transmission or contention-based Shared PUR.

Proposal 2a: It’s suggested that, if an IoT NTN UE in IDLE state is activated to use R19 contention-based Shared PUR, the UE needs to verify/update the TA just before triggering PUR.

-	vivo supports this and is ok to send an LS to RAN1/RAN4 on this if needed

-	IDC wonders if this is applicable also for the CRDSA proposal. ESA confirms

-	CATT is generally fine and is ok to check with RAN1/4. Thinks we should ask about NTA and UE specific TA

-	Nokia is fine but thinks that an LS is needed

-	QC wonders if we really need to send an LS

-	ZTE thinks we can send the LS because the requirements for sending msg3 could be different than for msg1

If an IoT NTN UE in IDLE state is to use the new R19 contention-based procedure, the UE needs to verify/update the uplink synchronization (e.g. get GNSS fix, acquire TA) just before sending msg3.

Check offline if we need to send an LS to RAN1/RAN4 on this

Proposal 2b: If the proposal 2a can be agreed, RAN2 further discuss whether the pur-TimeAlignmentTimer is no longer needed.

Proposal 3: It’s suggested that, if contention-based Shared PUR is supported, when the UE changes the camping cell, the UE doesn’t need to stop using this enhanced PUR function.

Proposal 4a: With reference to legacy PUR configuration, RAN2 can take the following resources types as start point for the discussion on shared PUR resources configuration:

•	Time domain resources, e.g., Periodicity, Offset, start time (H-SFN, frame, SFN etc.) of PUR occasion

•	Frequency domain resources, e.g., Carrier, SubCarrier

•	TBS

•	PUR MPDCCH/NPDCCH search space window

•	OCC resource

Proposal 4b: RAN2 is suggested to open discuss the following alternatives for configuring shared PUR resources:

•	Alt1: To provide a common PUR resource configuration via SIB.

•	Alt2: To provide shared PUR resources via dedicated signaling. It can be allowed to configure some different resources for different UEs while the other resources may be same for all the UEs.

•	Alt3: To provide multiple sets of shared PUR resources via the SIB and NW can indicate index of the resource set for a specific UE via the dedicated signaling.

Proposal 5: The existing types of “Msg4” for normal PUR process, e.g., Physical layer L1 ACK, Timing Advance Command MAC CE, and RRCEarlyDataComplete/RRCConnectionRelease/RRCConnectionSetup/ RRCConnectionResume, need to be supported for shared PUR scheme. The very simple Physical layer L1 ACK can be used in suitable cases to achieve the expected more efficient Msg4 transmission.

Proposal 6: RAN2 is suggested to open discuss the following alternatives for RNTI design and for the scheduling of Msg4:

•	Alt1: Common PUR-RNTI configured in the common PUR resources configuration.

•	Alt2: UE-specific PUR-RNTI allocated by NW that is similar as the one for dedicated PUR.

•	Alt3: Resource-specific PUR-RNTI which can be respectively calculated by UE and eNB according to the resources used/selected by the UE.

Proposal 7: The contention resolution scheme in legacy random access procedure can be reused for shared PUR, e.g., a UE Contention Resolution Identity can be included in the DL MAC PDU and UE checks whether the UE Contention Resolution Identity matches the 48 first bits of the CCCH SDU transmitted in Msg3.

Proposal 8: RAN2 is suggested to discuss the following other issues:

•	Issue#1: How to handle the coexistence of PUR and contention-based shard PUR

•	Issue#2: Whether a backoff scheme is needed when Msg3 collision occurs

Agreements:

- RAN2 focusses the study on contention-based Msg3 transmission to complete an EDT-like transaction (FFS on the details of Msg3. FFS on the procedural steps, e.g. how much we reuse of EDT and PUR procedures. FFS on allocation of resources).

- RAN2 can continue the discussion on Diversity Slotted ALOHA (DSA) and Contention Resolution Diversity Slotted Aloha (CRDSA) for Msg3-EDT transmissions without msg1/ RAR, evaluating possible impacts on the specification, in the next RAN2 meeting (RAN2 might send an LS to RAN1 later on this)

- If an IoT NTN UE in IDLE state is to use the new R19 contention-based procedure, the UE needs to verify/update the uplink synchronization (e.g. get GNSS fix, acquire TA) just before sending msg3.

[AT126][306][R19 IoT NTN] LS to RAN1/RAN4 (ZTE)

Scope: discuss the need and content of an LS to RAN1/RAN4 to confirm RAN2 agreement on the need to verify/update the uplink synchronization just before sending contention based msg3.

Intended outcome: draft LS (if needed)

Deadline for rapporteur's summary (in R2-2405763):  Friday 2024-05-24 08:00

R2-2405763	LS on uplink synchronization for contention based msg3 transmission	ZTE	LS out		LS out	Rel-19	IoT_NTN_Ph3-Core	To:RAN1,RAN4

Change “legacy” to “random access for IoT NTN”

Revised in R2-2405769

R2-2405769	LS on uplink synchronization for contention based msg3 transmission	ZTE	LS out		LS out	Rel-19	IoT_NTN_Ph3-Core	To:RAN1,RAN4

Approved

R2-2404581	Discussion on enhanced EDT for IoT NTN	OPPO	discussion	Rel-19	IoT_NTN_Ph3-Core

Observation 1	Compared to EDT, the feature of PUR can further improve the uplink transmission efficiency and save UE power consumption by skipping the random access procedure.

Observation 2	In legacy PUR, only dedicated uplink resource is supported, which is less resource efficient than shared uplink resource.

Proposal 1	RACH-less EDT, i.e. direct Msg3 transmission without Msg1/Msg2 is supported in Rel-19 IoT NTN.

Proposal 2	For RACH-less EDT, support shared uplink resource for Msg3 transmission, which are configured by eNB via system information.

Observation 3	Support of shared uplink resource for Msg3 transmission would bring the risk of collision among different UEs.

Proposal 3	RACH-less EDT can be enabled/disabled by network per UE via UE-specific signalling.

Proposal 4	For the shared Msg3 resource pool, support per CE level configuration.

Proposal 5	For each CE level, the shared Msg3 resource pool can be configured on anchor and/or non-anchor carriers.

Proposal 6	For each CE level, multiple Msg3 resource pools can be configured to support different numerologies (e.g. 3.75kHz and 15kHz SCS, single tone and multi-tone).

Proposal 7	An IDLE UE can initiate RACH-less EDT if all of the following conditions are fulfilled:

-	RACH-less EDT is enabled for the UE in this cell

-	the UE has a valid timing alignment value

-	the upper layers request establishment or resumption of an RRC connection

-	the establishment or resumption request is for mobile originating calls

-	the size of the resulting MAC PDU is expected to be no larger the TBS configured for RACH-less EDT.

Proposal 8	UE performs Msg3 resource selection from the multiple resource pools. The following aspects needs to be considered.

-	CE level determination

-	numerology determination

-	carrier selection

Observation 4	If the RNTI for Msg3 PUSCH scrambling is allocated by network, network needs to decode Msg3 blindly using all possible RNTIs, which would bring large complexity at network.

Proposal 9	In RACH-less EDT, the RNTI used for PUSCH/PDCCH/PDSCH scrambling is calculated based on the Msg3 PUSCH time-frequency resource.

Observation 5	For PUR, besides RRC response message, L1/L2 based ACK can also be used to terminate the PUR procedure, which could improve the performance of resource efficiency, UE power consumption as well as MO latency compared to EDT.

Proposal 10	L2 signalling can be used for acknowledging the reception of a transmission using EDT.

Proposal 11	The feature of Msg4 enhancement can be applied to both RACH based EDT and RACH-less EDT.

Proposal 12	RAN2 discusses the following fallback mechanism:

-	For RACH-less EDT, fallback between different CE levels

-	Fallback from PUR to RACH-less EDT

-	Fallback from RACH-less EDT to RACH-based EDT

R2-2405110	Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete	Interdigital, Inc.	discussion	Rel-19	IoT_NTN_Ph3-Core

Proposal 1: Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete only applies to the C-plane solution.

Proposal 2: Confirm that it is already possible with the existing PUR feature to terminate the EDT procedure without using RRCEarlyDataComplete, by using Layer 1 ACK or Timing advance MAC CE, if eNB is aware that there is no pending downlink data or signalling.

Proposal 3: RAN2 to discuss how eNB knows that there is no pending downlink data from the application layer.

Proposal 4: Confirm that it is already possible with the existing PUR feature for UE to indicate in PURConfigurationRequest whether it expects a downlink response by RRCEarlyDataComplete.

Proposal 5: Confirm that the existing PUR feature may not be optimal in an NTN deployment as there is currently no mechanism for the eNB to determine whether UE expects a downlink application layer response unless the UE moves to RRC_CONNECTED in every cell to perform dedicated PUR configuration.

Proposal 6: RAN2 to consider whether the following enhancements are beneficial:

1)	Introducing network signalling so that PUR configuration request information to be transferred across cells.

2)	Indication by the UE in RRCEarlyDataRequest an “rrc-ACK” parameter (whether UE expects a downlink application layer response)

3)	Enabling EDT termination without any downlink ACK, or using a common ACK for multiple transmissions or UEs

R2-2404164	Discussion on EDT Enhancement for IoT-NTN	vivo	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2404203	On signalling overhead reduction for EDT in IoT NTN	CATT	discussion

R2-2404322	Overview of capacity enhancement for uplink	Huawei, HiSilicon, Turkcell 	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2404561	Discussion on EDT optimisation in IoT-NTN	HONOR	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2404658	Uplink capacity enhancement in IoT NTN	Apple	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2404684	Discussion on EDT enhancements	Qualcomm Incorporated	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2404740	Discussion on uplink capacity enhancements for IOT NTN	Xiaomi	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2404802	EDT for uplink capacity enhancement in NTN	Lenovo	discussion	Rel-19

R2-2404922	Uplink Capacity Enhancement for EDT transaction	Spreadtrum Communications	discussion	Rel-19

R2-2405023	Considerations on EDT enhancements for IoT-NTN	CMCC	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2405109	Msg3 transmission without msg1/RAR	Interdigital, Inc.	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2405133	Discussion on enhanced EDT	MediaTek Inc.	discussion	IoT_NTN_Ph3-Core	R2-2402943

R2-2405154	Scenarios and procedures for IoT NTN uplink capacity enhancements	Samsung	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2405201	Efficient delivery of RRCEarlyDataComplete message	TOYOTA Info Technology Center	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2405321	Consideration on EDT enhancement for IoT-NTN	NEC Corporation.	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2405442	Discussion on UL capacity enhancement for IoT NTN	Nokia, Nokia Shanghai Bell	discussion	Rel-19	IoT_NTN_Ph3-Core

R2-2405655	Consideration on enhanced early data transmission in IoT NTN	TCL	discussion

Withdrawn

R2-2404410	Overview of capacity enhancement for uplink	Huawei, HiSilicon	discussion	Rel-19	IoT_NTN_Ph3-Core	Withdrawn

## 8.10	SON MDT Ph4

(NR_ENDC_SON_MDT_Ph4-Core; leading WG: RAN3; REL-19; WID: RP-234038)

Time budget: 0.5 TU

Tdoc Limitation: 2 tdocs

### 8.10.1	Organizational

LS, Rapporteur input, including workplan, etc.

LS

R2-2404122	LS on support of MRO for MR-DC SCG failure (R3-242195; contact: ZTE)	RAN3	LS in	Rel-19	NR_ENDC_SON_MDT_Ph4-Core	To:RAN2

Noted

Workplan

R2-2405631	Workplan for Rel-19 SON_MDT Enhancement	China Unicom	Work Plan	NR_ENDC_SON_MDT_Ph4-Core

Endorsed

### 8.10.2	MRO enhancements for Rel-18 mobility features

LTM, CHO with candidate SCGs, subsequent CPAC

Subsequent CPAC

R2-2404875	MRO for CHO with candidate SCG(s) and SCPAC	NEC	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Proposal 4: RAN2 studies failure and near failure scenarios for subsequent execution of SCPAC.

Proposal 5: For near failure of subsequent execution of SCPAC, RAN2 discusses how and which node generates SPR triggers for subsequent CPAC.

On P4

-	Ericsson think we should agree on what we try to achieve. LG thinks existing reports are sufficient, but are open to study further. Lenovo supports the proposal. Samsung thinks current spec works for failure and near failure. Ericsson thinks its unclear what enhancements are needed at this moment. CMCC thinks this is in scope and hence we need to study. QC are OK to study.

RAN2 will look into failure and near failure scenarios for subsequent execution of SCPAC and see if/what enhancements are needed.

CHO with candidate SCG

R2-2404734	MRO for CHO with candidate SCG	Nokia	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Proposal 1: RAN2 to enhance the RLF report and SCGFailureInformation with additional information regarding the state of the two execution conditions.

Proposal 2: RAN2 to enhance SHR and SPR with new triggers (e.g. based on time gap between condition fulfilment) and content that allows the network to identify the execution condition that delayed the handover.

Proposal 3: Scenarios where the UE is additionally configured with CHO-only configuration should be down-prioritized and studied only if time allows, as they do not cause loss of connectivity.

On P1

-	Huawei is OK with P1, but would like to understand the scenario and the issue first, and P1 is very general. ZTE are unsure of the motivation and what should be reported. Nokia explains that the proposal is that the existing reports are enhanced with more info.

On P2

-	ZTE thinks we have done something similar for CHO but that’s only for the failure case, and if we do P2 we may trigger too many SHRs and SPRs. Nokia thinks there will be no unnecessary reports. CATT supports P2. Huawei supports but want to remove the example in the proposal. LG agrees with Huawei. Samsung and Ericsson are unsure of the usefulness. Lenovo wants a new triggers since it is about near-failure and hence important.

On P3:

-	Samsung thinks that CHOs will be in place when CHO with candidate SCG is deployed. Huawei think that CHO-only is out of scope. CMCC disagrees. Lenovo thinks its important to consider the combination.

RAN2 to enhance the RLF report with additional information regarding the state of the two execution conditions. We see later if we also can enhance the SCGFailureInformation report.

R2-2404814	Discussion on MRO for R18 mobility	Lenovo	discussion	Rel-19

LTM

R2-2404735	MRO for LTM	Nokia	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Proposal 1: RAN2 to correct the definition of scenario 3a as follows: Case 3a: the UE detects HOF/RLF in the LTM target cell and performs reestablishment procedure in a cell other than the source cell and the target cell.

We correct the definition of scenario 3a as follows: Case 3a: the UE detects HOF/RLF in the LTM target cell and performs reestablishment procedure in a cell other than the source cell and the target cell.

R2-2405094	SON support for MRO	Ericsson	discussion	NR_ENDC_SON_MDT_Ph4-Core

Proposal 1	If configured the log the L1 measurements for serving cell, target cell and other LTM candidate cells in RLF report, upon RLF or mobility failure.

Proposal 2	RAN2 agree to reuse the timeConnFailure and the reconnectCellId in RLF-report for the LTM failures.

Proposal 3	RAN2 agree to define a new field in RLF-report to log the LTM recovery cell ID upon selecting an LTM candidate cell.

Proposal 4	Extend lastHO-Type in RLF-Report to indicate the LTM cell switch as last executed mobility procedure.

On P3

-	Nokia wonders if we need to differentiate CHO vs LTM, or can the UE simply log the recovery cell without indicating it as CHO or LTM. Ericsson explains the reasoning and Nokia can agree with it.

-	Fujitsu think we should discusss if we should support the scenario with CHO and LTM.

If available, log the L1 measurements for serving cell, target cell and other LTM candidate cells in RLF report, upon RLF or mobility failure.

Reuse the existing approach of using timeConnFailure and the reconnectCellId in RLF-report also for LTM failures, details TBD.

Log the LTM cell ID upon performing recovery an LTM candidate cell, details TBD e.g. which field.

Extend lastHO-Type in RLF-Report to indicate the LTM cell switch as last executed mobility procedure.

R2-2404952	Discussion on MRO Enhancements for Mobility	CATT	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

R2-2404977	MRO for Rel-18 mobility	ZTE, Sanechips	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

R2-2405018	Discussion on MRO enhancements for R18 mobility features	CMCC	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

R2-2405298	SON/MDT reports for LTM 	Kyocera 	discussion

R2-2405334	MRO enhancement for Rel-18 mobility	Huawei, HiSilicon	discussion	NR_ENDC_SON_MDT_Ph4-Core

R2-2405430	Discussion on random access report for LTM	ASUSTeK	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

R2-2405538	Failure and Near failure cases for CHO with Candidate SCGs	LG Electronics	discussion	Rel-18	NR_ENDC_SON_MDT_Ph4-Core

R2-2405539	SHR for MCG LTM	LG Electronics	discussion	Rel-18	NR_ENDC_SON_MDT_Ph4-Core

R2-2405569	Discussion on MRO enhancement for R18 mobility features	SHARP Corporation	discussion

R2-2405580	MRO enhancement for SON and MDT	Qualcomm Incorporated	discussion	NR_ENDC_SON_MDT_Ph4-Core

R2-2405632	Discussion on MRO enhancement for mobility	China Unicom	discussion	NR_ENDC_SON_MDT_Ph4-Core

R2-2405150	MRO enhancements for Rel-18 mobility features	Samsung	discussion

R2-2404311	MRO for Rel-18 mobility features	vivo	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

R2-2404356	Discussion on MCG LTM MRO enhancement	Fujitsu	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

R2-2404867	Discussion on the MRO enhancements for R18 mobility features	Beijing Xiaomi Software Tech	discussion	Rel-19

R2-2404874	MRO enhancements for LTM	NEC	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

### 8.10.3	SON/MDT for Slicing

No contributions are expected and this AI will not be treated in RAN2#126, in wait for RAN3 progresses

### 8.10.4	SON/MDT for NTN

No contributions are expected and this AI will not be treated in RAN2#126, in wait for RAN3 progresses

### 8.10.5	Leftovers from Rel-18

RACH optimization for SDT, MHI Enhancement for SCG Deactivation/Activation, MRO for MR-DC SCG failure

SDT

R2-2405633	Discussion on RACH optimization for SDT	China Unicom	discussion	NR_ENDC_SON_MDT_Ph4-Core

Proposal 1: RAN2 to support SDT-related enhancement when SDT initiation fails and fallback to a legacy RA procedure, due to network-configured thresholds being set too restrictively.

Proposal 2: RAN2 to support SDT-related enhancement upon SDT initiation stage to prevent receiving excessively loose thresholds configured by the network.

Proposal 3: RAN2 to support including RSRP/data volume related information in RA report for two scenarios: SDT initiation fails and fallback to Legacy RA scenario, and SDT initiation scenario. The detailed information can be:

Option 1: Downlink RSRP value and pending uplink data volume;

Option 2: The gap between the actual value of downlink RSRP/ pending uplink data volume to the configured threshold received from the network.

Proposal 4: When SDT failure happens, the UE can indicate the failure cause of SDT to the network, e.g. T319a expiration.

On P3

-	 QC thinks that the NW knows the thresholds so the gap does not need to be reported. Ericsson thinks that option 1 is already supported in the spec today for 2-step. ZTE thinks that the NW may forget a previous threshold. CATT agrees with ZTE and hence wants option 2. Nokia think that option 1 is good enough. CMCC agrees with Nokia.

On P4

-	Samsung does not support this. Ericsson supports the proposal as the NW can optimize the T319a duration, etc. ZTE supports the proposal for the reasons explained by Ericsson. Huawei also supports P4 in general but details can be discussed later. QC thinks that also for this case the UE shall report the RSRP and data volume so that the NW can set the related thresholds better. Ericsson agrees with QC.

For the purpose of SON enhancements for SDT, include RSRP/data volume related information

Downlink RSRP value and buffered uplink data volume at the time when the UE evaluates if it should perform SDT.

When SDT failure happens, the UE can indicate the failure cause of SDT to the network, e.g. T319a expiration. Details are TBD, e.g. if RSRP and data volume can also be included in such report.

R2-2404953	Consideration on leftovers from Rel-18 SONMDT	CATT	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Focus on P1-P5

Proposal 1: Besides the MO-SDT, the scenario of MT-SDT based RA procedure is considered in RACH optimization for SDT.

On P1

-	Samsung thinks it is not in scope since the objective is about R18 leftovers. ZTE wants to support MT-SDT. NEC wonders what there is to optimize since the resources are the same.

MHI

R2-2405019	MHI Enhancement for SCG Deactivation/Activation	CMCC, CATT, Ericsson, ZTE, Huawei, HiSilicon	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

-	QC thinks that the NW knows how long time the SCG was activated. Samsung wonders what the NW does with this info. CMCC explains that if the time of activation is low, the NW could have skipped configuring the SCG completely. Samsung thinks the NW knows this without a UE report. CMCC thinks the NW report is optional. Nokia agrees with Samsung. Huawei highlights that we are talking about a mobility case and with mobility the NW cannot easily know. CATT thinks that he NW can change how much it activates/deactivates the SCG based on this info. QC, Samsung, and Nokia are not convinced of the use case.

Postponed

SCG failure

R2-2404978	Rel-18 leftovers for SON MDT	ZTE, Sanechips	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

Focus on P5 and P6

R2-2405335	Discussion on support of the Rel-18 leftovers	Huawei, HiSilicon	discussion	NR_ENDC_SON_MDT_Ph4-Core

Focus on P5

On P6 from ZTE paper

-	Ericsson thinks this was discussed in R18 and deprioritized since there was no time and now in R19 we have even less time so we don’t want to anything more than NR-DC. Lenovo think it is simple to support all architecture options. Samsung and CATT also want to support all architecture options. Qualcomm wants to only do EN-DC now, and if time allows we can do other architecture options later. Ericsson think that RAN3 asks RAN2 to decide and are OK if we only do EN-DC.

Reply to RAN3 that we will only do EN-DC. RAN2 understands that whether also supporting (NG)EN-DC has no additional RAN2 impact hence RAN3 can decide. If later we get time we can consider other options.

[AT126][764][SONMDT] Architecture options for R19 SONMDT (ZTE)

Scope:

    - Produce approvable draft LS

Intended outcome:

    - Approvable LS in R2-2405846 (ZTE)

Deadline:

    - Thursday lunch. Intention is to agree the CRs over email.

R2-2405846	Reply LS on support of MRO for MR-DC SCG failure	RAN2

Approved

R2-2405164	SON/MDT enhancements for leftover topics from R18	Samsung	discussion

R2-2405581	SON and MDT Rel-18 leftover issues	Qualcomm Incorporated	discussion	NR_ENDC_SON_MDT_Ph4-Core

R2-2405668	On Rel.18 leftovers	Ericsson	discussion

R2-2405560	Discussion on RACH enhancement for SDT	SHARP Corporation	discussion

R2-2404312	RACH optimization for SDT	vivo	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

R2-2404815	Discussion on MRO for MR-DC SCG failure	Lenovo	discussion	Rel-19

R2-2404868	Discussion on the RACH optimization for SDT	Beijing Xiaomi Software Tech	discussion	Rel-19

# 9	Breakout session reports

No documents shall be submitted to this AI or its sub-AIs. It is only for at-meeting-generated contents.

## 9.1	Session on V2X/SL, R19 NES and MOB

R2-2405701	Report from session on V2X/SL, R19 NES and MOB	Vice Chairman (Samsung)	discussion

=>	The report is approved

## 9.2	Session on R18 MIMOevo, R18 MUSIM, and R19 LP-WUS

R2-2405702	Report from session on R18 MIMOevo, R18 MUSIM, and R19 LP-WUS	Vice Chairman (CATT)	discussion

=>	The report is approved

## 9.3	Session on NR NTN and IoT NTN

R2-2405703	Report from Break-Out Session on NR NTN and IoT NTN	Session chair (ZTE)	discussion

=>	The report is approved

## 9.4	Session on positioning and sidelink relay

R2-2405704	Report from session on positioning and sidelink relay	Session chair (MediaTek)	discussion

=>	The report is approved

## 9.5	Session on Mobility Enh and Mobile IAB

R2-2405705	Report from session on Mobility Enh and Mobile IAB	Session chair (MediaTek)	discussion

=>	The report is approved

## 9.6	Session on R18 MBS, R18 QoE and R19 XR

R2-2405706	Report from session on R18 MBS, R18 QoE and R19 XR	Session chair (Huawei)	discussion

=>	The report is approved

## 9.7	Session on maintenance, SON/MDT and eRedCap

R2-2405707	Report from maintenance, SON/MDT and eRedCap breakout session	Session chair (Ericsson)	discussion

=>	The report is approved

## 9.8	Session on further NR coverage enhancements

R2-2405708	Report from Further NR coverage enhancements session	Session chair (ZTE)	discussion

=>	The report is approved

# Closing of the meeting

The meeting was closed by the chair at 13:20 UTC on Friday, 24th of May.

# Annex A:	List of participants

RAN2#126 participants list is attached to this report.

Total number of participants: 447

# Annex B:	List of Tdocs

The list of tdocs from RAN2#126 is attached to this report.

Total of 1996 tdoc numbers were allocated of which 1975 tdocs were made available.

# Annex C:	Incoming liaison statements

| TDoc | Title | Source | Status | Rel | Related WIs | To | Cc | Original LS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R2-2404104 | Reply LS on positioning MAC agreements (R1-2403536; contact: Huawel) | RAN1 | noted | Rel-18 | NR_pos_enh2-Core | RAN2 |  | R1-2403536 |
| R2-2404105 | Reply LS on SRS BW aggregation impact on other channels/signals (R1-2403539; contact: Huawei) | RAN1 | noted | Rel-18 | NR_pos_enh2-Core | RAN4 | RAN2, RAN3 | R1-2403539 |
| R2-2404106 | Reply to LS on IUC or DRX in co-channel co-existence (R1-2403573; contact: LGE) | RAN1 | noted | Rel-18 | NR_SL_enh2-Core | RAN2 |  | R1-2403573 |
| R2-2404107 | LS on UE’s reporting SL PRS CBR measurement to gNB (R1-2403577; contact: Qualcomm) | RAN1 | noted | Rel-18 | NR_pos_enh2-Core | RAN2 |  | R1-2403577 |
| R2-2404108 | LS on CPE starting position for S-SSB in SL-U (R1-2403578; contact: OPPO) | RAN1 | noted | Rel-18 | NR_SL_enh2-Core | RAN2 |  | R1-2403578 |
| R2-2404109 | LS on updating RAN1 agreement about minimum time gap Z (R1-2403588; contact: Huawei) | RAN1 | noted | Rel-18 | NR_SL_enh2-Core | RAN2 |  | R1-2403588 |
| R2-2404110 | Reply LS on Parallel Tx Capability (R1-2403619; contact: ZTE) | RAN1 | noted | Rel-17 | TEI17, NR_newRAT-Core, NR_2step_RACH, NR_IIOT_URLLC_enh | RAN2 |  | R1-2403619 |
| R2-2404111 | Reply LS on decisions on SLPP (R1-2403622; contact: vivo) | RAN1 | noted | Rel-18 | NR_pos_enh2-Core | RAN2 |  | R1-2403622 |
| R2-2404112 | Reply LS on questions on RAN1 parameter list (R1-2403636; contact: CATT) | RAN1 | noted | Rel-18 | NR_pos_enh2-Core | RAN2 | RAN3, RAN4 | R1-2403636 |
| R2-2404113 | Reply LS on 2-step RACH for eRedCap (R1-2403646; contact: Ericsson) | RAN1 | noted | Rel-18 | NR_redcap_enh-Core | RAN2 |  | R1-2403646 |
| R2-2404114 | LS to RAN2 on CBSR for Rel-19 MIMO (R1-2403650; contact: Samsung) | RAN1 | noted | Rel-19 | NR_MIMO_Ph5 | RAN2 |  | R1-2403650 |
| R2-2404115 | "LS on the identification of the power control parameters after LTM cell switch ( | R1-2403683; contact: Fujitsu)" | noted | Rel-18 | NR_Mob_enh2-Core | RAN2 |  | R1-2403683 |
| R2-2404116 | LS on Rel-18 RAN1 UE features list for NR after RAN1#116bis (R1-2403705; contact: NTT DOCOMO, AT&T) | RAN1 | noted | Rel-18 | NR_MIMO_evo_DL_UL, NR_pos_enh2, Netw_Energy_NR, NR_netcon_repeater, NR_NTN_enh, NR_Mob_enh2, NR_SL_enh2, NR_redcap_enh, NR_MC_enh, NR_XR_enh, NR_FR1_lessthan_5MHz_BW, NR_DSS_enh, NR_BWP_wor, NR_cov_enh2, TEI18 | RAN2, RAN4 |  | R1-2403705 |
| R2-2404117 | LS on PRS resource ID for bandwidth aggregation (R1-2403728; contact: ZTE) | RAN1 | noted | Rel-18 | NR_pos_enh2-Core | RAN2 |  | R1-2403728 |
| R2-2404118 | LS on the dci-FormatsSL and DCI format 3_2 (R1-2403732; contact: Qualcomm) | RAN1 | noted | Rel-18 | NR_pos_enh2-Core | RAN2 |  | R1-2403732 |
| R2-2404119 | Reply LS on RRC network assistant signalling for advanced receiver on MU-MIMO scenario (R1-2403750; contact: China Telecom) | RAN1 | noted | Rel-18 | NR_demod_enh3-Core | RAN2, RAN4 |  | R1-2403750 |
| R2-2404120 | LS on the capability mux-HARQ-ACK-withoutPUCCH-onPUSCH-r16 (R1-2403760; contact: MediaTek) | RAN1 | noted | Rel-17 | TEI17, NR_newRAT-Core | RAN2 |  | R1-2403760 |
| R2-2404121 | LS on the conditions for triggering UL WUS transmission to request on-demand SIB1 (R1-2403779; contact: MediaTek) | RAN1 | noted | Rel-19 | Netw_Energy_NR_enh | RAN2 |  | R1-2403779 |
| R2-2404122 | LS on support of MRO for MR-DC SCG failure (R3-242195; contact: ZTE) | RAN3 | noted | Rel-19 | NR_ENDC_SON_MDT_Ph4-Core | RAN2 |  | R3-242195 |
| R2-2404123 | Reply LS on SDT signalling optimization for partial context transfer (R3-242198; contact: ZTE) | RAN3 | noted | Rel-18 | TEI18 | RAN2 |  | R3-242198 |
| R2-2404124 | Reply LS on combination of HST and RRM relaxation (R4-2403532; contact: Apple) | RAN4 | noted | Rel-16 | NR_HST, NR_UE_pow_sav-Core | RAN2 |  | R4-2403532 |
| R2-2404125 | LS on SL positioning measurements (R4-2406386; contact: Huawei) | RAN4 | noted | Rel-18 | NR_pos_enh2-Core | RAN1, RAN2 |  | R4-2406386 |
| R2-2404126 | Reply LS on n-TimingAdvanceOffset for PDCCH order RACH (R4-2406444; contact: Apple) | RAN4 | noted | Rel-18 | NR_Mob_enh2-Core | RAN2 | RAN1 | R4-2406444 |
| R2-2404127 | LS on UE capability for NW verified location (R4-2406496; contact: Huawei) | RAN4 | noted | Rel-18 | NR_NTN_enh-Core | RAN1, RAN2 |  | R4-2406496 |
| R2-2404128 | Reply LS on BWP operation without bandwidth restriction (R4-2406500; contact: vivo, Vodafone) | RAN4 | noted | Rel-18 | NR_BWP_wor-Core | RAN2, RAN1 |  | R4-2406500 |
| R2-2404129 | LS on 3Tx SAR solution for inter-band CA with PC1.5 (R4-2406579; contact: Huawei) | RAN4 | noted | Rel-18 | 4Rx_low_NR_band_handheld_3Tx_NR_CA_ENDC-Core | RAN2 |  | R4-2406579 |
| R2-2404130 | LS on RAN4 UE feature list for Rel-18 (version 4) (R4-2406679; contact: CMCC) | RAN4 | noted | Rel-18 | NR_ENDC_RF_FR1_enh2, NR_channel_raster_enh, NR_FR2_multiRX_DL, NR_RRM_enh3, NR_MG_enh2, NonCol_intraB_ENDC_NR_CA, NR_HST_FR2_enh, NR_ATG, NR_demod_enh3, NR_pos_enh2, NR_MC_enh, NR_Mob_enh2, NR_NTN_enh, NR_cov_enh2, Netw_Energy_NR | RAN2 | RAN1 | R4-2406679 |
| R2-2404131 | LS on UE Capability for Asymmetric BW for less than 5 MHz (R4-2406717; contact: Rakuten) | RAN4 | noted | Rel-18 | NR_FR1_lessthan_5MHz_BW | RAN2, RAN1 |  | R4-2406717 |
| R2-2404132 | LS on 3T6R and 4T6R antenna switching SRS (R4-2406718; contact: Huawei) | RAN4 | noted | Rel-19 | NR_ENDC_RF_Ph4-Core | RAN1 | RAN2 | R4-2406718 |
| R2-2404133 | LS on 5GS missing CBC support for shared networks (S2-2405210; contact: Ericsson) | SA2 | noted | Rel-15 | 5GS_Ph1 | CT1 | RAN2, RAN3, SA1 | S2-2405210 |
| R2-2404134 | Reply LS on L2ID and user info for L2 based U2U (S2-2405379; contact: OPPO) | SA2 | noted | Rel-18 | NR_SL_relay_enh-Core | RAN2, CT1 |  | S2-2405379 |
| R2-2404135 | Reply LS on Rel-18 RedCap enhancements to address remaining ENs in TS 23.502 (S2-2405421; contact: Huawei) | SA2 | noted | Rel-18 | NR_redcap_enh-Core | RAN2, RAN3, CT1 | CT4 | S2-2405421 |
| R2-2404136 | Reply LS on U2U relay selection (S2-2405531; contact: OPPO) | SA2 | noted | Rel-18 | NR_SL_relay_enh-Core | RAN2 | CT1 | S2-2405531 |
| R2-2404137 | LS on Support of Regenerative-based Satellite Access (S2-2405600; contact: vivo) | SA2 | noted | Rel-19 | FS_5GSAT_Ph3_ARCH | RAN3 | RAN2 | S2-2405600 |
| R2-2404138 | LS on Application-Layer FEC Awareness at RAN (S2-2405604; contact: Qualcomm) | SA2 | noted | Rel-19 | FS_XRM_Ph2 | RAN2, SA4 | RAN3 | S2-2405604 |
| R2-2404139 | LS on FS_XRM Ph2 (S2-2405625; contact: vivo) | SA2 | noted | Rel-18 | FS_XRM_Ph2 | SA4, RAN2, RAN3 |  | S2-2405625 |
| R2-2404140 | LS reply on LCS user plane connection binding to the UE (S2-2405797; contact: CATT) | SA2 | noted | Rel-18 | 5G_eLCS_Ph3 | CT1 | CT4, RAN2 | S2-2405797 |
| R2-2404141 | LS on Support of UE move between CAG cell of 5G Femto and CSG cell (S2-2405813; contact: NTT DOCOMO) | SA2 | noted | Rel-19 | FS_5G_Femto, FS_NR_WAB_5GFemto | RAN2, RAN3 |  | S2-2405813 |
| R2-2404142 | LS on FS_VMR_Ph2 solution impacts to RAN (S2-2405822; contact: Qualcomm) | SA2 | noted | Rel-19 | FS_VMR_Ph2 | RAN3 | RAN2 | S2-2405822 |
| R2-2404143 | LS on data collection to enable ML model training and inference in 5GC for Direct AI/ML based positioning (S2-2405833; contact: vivo) | SA2 | available | Rel-19 | FS_AIML_CN | RAN1, RAN2 | RAN3 | S2-2405833 |
| R2-2404144 | LS to SA2 and RAN2 on selected satellite architecture for Store and Forward (S3-241567; contact: InterDigital) | SA3 | noted | Rel-19 | FS_5GSAT_Ph3_SEC | SA2, RAN2 | SA, RAN3, SA3-LI | S3-241567 |
| R2-2404145 | Reply LS on area scope handling for QoE measurements (S5-241925; contact: Ericsson) | SA5 | noted | Rel-18 | NR_QoE_enh-Core | RAN2 | RAN3, SA4 | S5-241925 |
| R2-2404146 | LS reply on OAM requirements for UE location verification (S5-242198; contact: CATT) | SA5 | noted | Rel-18 | OAM_NTN | RAN3 | SA2, RAN1, RAN2 | S5-242198 |
| R2-2404147 | Reply LS on the user consent for trace reporting (S5-242221; contact: Ericsson) | SA5 | noted | Rel-18 | TEI18 | RAN3 | RAN2, SA2, SA3 | S5-242221 |
| R2-2405928 | LS on higher layer parameters for Rel-18 Positioning (R1-2405403; contact: Intel) | RAN1 | noted | Rel-18 | NR_pos_enh2-Core | RAN2 |  | R1-2405403 |
| R2-2405929 | Reply LS on SRS bandwidth aggregation (R1-2405456; contact: ZTE) | RAN1 | noted | Rel-18 | NR_pos_enh2-Core | RAN2 | RAN3 | R1-2405456 |
| R2-2405991 | Reply LS to RAN2 on RRM enhancements for NR FR2 HST (R4-2410285; contact: Samsung) | RAN4 | available | Rel-18 | NR_HST_FR2_enh | RAN2 |  | R4-2410285 |
| R2-2405992 | LS on UE capability for multi-carrier enhancement (R4-2410299; contact: vivo) | RAN4 | noted | Rel-18 | NR_MC_enh-Core | RAN2 | RAN1 | R4-2410299 |
| R2-2405997 | Reply LS on parameters used for CG RACH-less Handover (R1-2405563; contact: Samsung) | RAN1 | noted | Rel-18 | NR_NTN_enh-Core, NR_mobile_IAB-Core, TEI18 | RAN2 |  | R1-2405563 |
| R2-2406009 | Reply LS on parameters used for CG RACH-less Handover from LTM perspective (R1-2405671; contact: Fujitsu) | RAN1 | noted | Rel-18 | NR_Mob_enh2-Core | RAN2 |  | R1-2405671 |

50 incoming LS, of which 48 LS were noted. The remaining non-treated LSin will be treated in RAN2#127.

# Annex D:	Outgoing liaison statements

| TDoc | Title | Rel | Related WIs | To | Cc |
| --- | --- | --- | --- | --- | --- |
| R2-2405766 | LS on RACH during uplink transmission extension | Rel-18 | IoT_NTN_enh-Core | RAN1 | RAN4 |
| R2-2405767 | LS on DL coverage enhancements | Rel-19 | NR_NTN_Ph3-Core | RAN1 |  |
| R2-2405769 | LS on UL synchronization for contention based Msg3 transmission without Msg1/Msg2 | Rel-19 | IoT_NTN_Ph3-Core | RAN1, RAN4 |  |
| R2-2405779 | Reply LS to SA2 on FS_XRM PH2 | Rel-19 | NR_XR_Ph3-Core | SA2 | RAN3, SA4 |
| R2-2405781 | Reply LS on Application-Layer FEC Awareness at RAN | Rel-19 | NR_XR_Ph3-Core | SA2 | SA4, RAN3 |
| R2-2405782 | LS on multi-modality awareness at RAN | Rel-19 | NR_XR_Ph3-Core | SA2 | SA4, RAN3 |
| R2-2405846 | Reply LS on support of MRO for MR-DC SCG failure | Rel-19 | NR_ENDC_SON_MDT_Ph4-Core | RAN3 |  |
| R2-2405872 | Reply LS on SL positioning measurements | Rel-18 | NR_pos_enh2 | RAN4 |  |
| R2-2405942 | Reply LS on Support of UE move between CAG cell of 5G Femto and CSG cell | Rel-19 | FS_5G_Femto, FS_WAB_5GFemto_NR | SA2 | RAN3 |
| R2-2405986 | Reply LS on SLPP parameters provision | Rel-18 | NR_pos_enh2-Core | RAN1 |  |
| R2-2405987 | LS on relative velocity | Rel-18 | NR_pos_enh2-Core | SA2 |  |
| R2-2405988 | LS on the maximum number of devices supported in SLPP | Rel-18 | NR_pos_enh2-Core | CT1, CT4 | SA2 |
| R2-2406022 | LS reply on 3Tx SAR solution for inter-band CA with PC1.5 | Rel-18 | 4Rx_low_NR_band_handheld_3Tx_NR_CA_ENDC-Core | RAN4 |  |
| R2-2406111 | Reply LS on IE supportedBandwidthCombinationSetIntraENDC and IE intraBandENDC-Support | Rel-17 | TEI17 | RAN4 |  |
| R2-2406148 | Reply LS on Rel-18 higher-layers parameter list | Rel-18 | NR_MC_enh-Core, NR_MIMO_evo_DL_UL-Core, NR_pos_enh2-Core, Netw_Energy_NR, NR_cov_enh2, NR_XR_enh-Core, NR_Mob_enh2, NR_BWP_wor-Core, NR_NTN_enh, IoT_NTN_enh-Core, NR_SL_enh2-Core, NR_netcon_repeater-Core, NR_DSS_enh, NR_redcap_enh-Core, TEI18 | RAN1 | RAN3, RAN4 |
| R2-2406149 | Reply LS on RAN4 vs RAN2 Cricket Match | Rel-19 | TEI19 | RAN4 |  |
| R2-2406150 | LS on RAN2 agreements and assumptions for Ambient IoT | Rel-19 | FS_Ambient_IoT_solutions | SA3, SA2 | RAN3 |

17 outgoing LS.

# Annex E:	List of agreed CRs

| TDoc | Title | Source | Rel | Spec | Related WIs | CR | Rev | Cat |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R2-2404240 | Miscellaneous corrections on TS 38.304 for eRedCap | Huawei, HiSilicon | Rel-18 | 38.304 | NR_redcap_enh-Core | 0401 |  | F |
| R2-2404246 | Remove of AS condition checking of SUI for U2N Relay communication | OPPO, Apple | Rel-18 | 38.331 | TEI18, NR_SL_relay-Core | 4645 | 1 | F |
| R2-2404404 | Terminology alignment on the types of NR NTN deployment | CATT, Nokia, Nokia Shanghai Bell, Ericsson, Huawei, HiSilicon, ZTE Corporation, Sanechips | Rel-17 | 38.300 | NR_NTN_solutions-Core | 0861 |  | F |
| R2-2404405 | Terminology alignment on the types of NR NTN deployment | CATT, Nokia, Nokia Shanghai Bell, Ericsson, Huawei, HiSilicon, ZTE Corporation, Sanechips | Rel-18 | 38.300 | NR_NTN_solutions-Core | 0862 |  | A |
| R2-2404430 | Correction on redirection to GERAN [Redirect_to_GERAN] | vivo, Qualcomm Incorporated, Samsung, Vodafone | Rel-18 | 36.331 | TEI18 | 5000 | 1 | F |
| R2-2404445 | Miscellaneous corrections on TS 38.300 for BWP operation without restriction | vivo | Rel-18 | 38.300 | NR_BWP_wor-Core | 0837 | 1 | D |
| R2-2404446 | [V994][V995] Miscellaneous corrections on TS 38.331 for BWP operation without restriction | vivo | Rel-18 | 38.331 | NR_BWP_wor-Core | 4679 | 1 | F |
| R2-2404447 | Correction on RACH resource set availability check | vivo, Guangdong Genius | Rel-17 | 38.321 | NR_redcap-Core, NR_cov_enh-Core, NR_SmallData_INACTIVE-Core | 1827 | 2 | F |
| R2-2404465 | Clarification of configured grant in shared spectrum | Xiaomi | Rel-16 | 38.331 | NR_unlic-Core | 4778 |  | F |
| R2-2404486 | Correction on sdt-LogicalChannelSR-DelayTimer applicability | Nokia, Nokia Shanghai Bell | Rel-18 | 38.321 | NR_SmallData_INACTIVE-Core, NR_MT_SDT-Core | 1774 | 2 | F |
| R2-2404490 | SDT BFR timer being not configured [RA-SDT_BeamFailure] | Nokia, Sony, Nokia Shanghai Bell | Rel-18 | 38.321 | TEI18 | 1814 | 1 | F |
| R2-2404509 | [B021] Missing posSibType2-17a in list of posSIB types [PosL2RemoteUE] | MediaTek Inc., Lenovo | Rel-18 | 38.331 | TEI18 | 4767 | 1 | F |
| R2-2404529 | Miscellaneous non-controversial rapporteur corrections | Intel Corporation | Rel-16 | 38.306 | NR_newRAT-Core, NR_eMIMO-Core, NR_unlic-Perf, 5G_V2X_NRSL-Core | 1096 |  | F |
| R2-2404543 | [S081] Clarification for order of list in PagingRecordList | ZTE Corporation (rapporteur), Samsung | Rel-17 | 38.331 | LTE_NR_MUSIM-Core | 4785 |  | F |
| R2-2404553 | Correction to BCS5 bandwidth capabilities | Nokia, Qualcomm Incorporated, ZTE Corporation, Sanechips, Huawei, HiSilicon, Ericsson | Rel-17 | 38.306 | NR_BCS4-Core | 1080 | 1 | F |
| R2-2404554 | Correction to BCS5 bandwidth capabilities | Nokia, Qualcomm Incorporated, ZTE Corporation, Sanechips, Huawei, HiSilicon, Ericsson | Rel-18 | 38.306 | NR_BCS4-Core | 1081 | 1 | A |
| R2-2404670 | Clarification on the combination of HST and RRM measurement relaxation | Apple, Ericsson, Nokia (Rapporteur) | Rel-16 | 38.300 | NR_HST, NR_UE_pow_sav-Core | 0839 | 1 | F |
| R2-2404702 | Correction on prerequisite feature for csi-ReportingCrossPUCCH-Grp-r16 | Qualcomm Incorporated | Rel-16 | 38.306 | TEI16 | 1018 | 2 | F |
| R2-2404703 | Correction on prerequisite feature for csi-ReportingCrossPUCCH-Grp-r16 | Qualcomm Incorporated | Rel-17 | 38.306 | TEI16 | 1019 | 2 | A |
| R2-2404704 | Correction on prerequisite feature for csi-ReportingCrossPUCCH-Grp-r16 | Qualcomm Incorporated | Rel-18 | 38.306 | TEI16 | 1020 | 2 | A |
| R2-2404720 | Clarification on the srs-AntennaSwitchingBeyond4RX-r17 | ZTE Corporation, Sanechips | Rel-17 | 38.306 | NR_FeMIMO-Core | 1075 | 1 | F |
| R2-2404721 | Clarification on the srs-AntennaSwitchingBeyond4RX-r17 | ZTE Corporation, Sanechips | Rel-18 | 38.306 | NR_FeMIMO-Core | 1076 | 1 | A |
| R2-2404722 | Correction on the supportedBandwidthDL/UL-v1780 for the NR-DC (r17) | ZTE Corporation, Sanechips | Rel-17 | 38.306 | NR_BCS4-Core | 1077 | 1 | F |
| R2-2404784 | Miscellaneous non-controversial corrections Set XXI | Ericsson | Rel-15 | 38.331 | NR_newRAT-Core, TEI15 | 4797 |  | F |
| R2-2404785 | Miscellaneous non-controversial corrections Set XXI | Ericsson | Rel-16 | 38.331 | NR_newRAT-Core, TEI16 | 4798 |  | F |
| R2-2404845 | Clarification on usage of LEO or NGSO | MediaTek Inc. | Rel-17 | 38.331 | NR_NTN_solutions-Core | 4745 | 1 | F |
| R2-2404846 | Clarification on usage of LEO or NGSO | MediaTek Inc. | Rel-18 | 38.331 | NR_NTN_solutions-Core | 4746 | 1 | A |
| R2-2404847 | Clarification on usage of LEO or NGSO | MediaTek Inc. | Rel-17 | 38.306 | NR_NTN_solutions-Core | 1082 | 1 | F |
| R2-2404848 | Clarification on usage of LEO or NGSO | MediaTek Inc. | Rel-18 | 38.306 | NR_NTN_solutions-Core | 1083 | 1 | A |
| R2-2404960 | Miscellaneous corrections on Mobile IAB | Ericsson | Rel-18 | 38.331 | NR_mobile_IAB-Core | 4701 | 2 | F |
| R2-2404962 | Dummy the rrc-TransactionIdentifier field from IABOtherInformation | Ericsson | Rel-16 | 38.331 | NR_IAB-Core | 4702 | 1 | F |
| R2-2404963 | Dummy the rrc-TransactionIdentifier field from IABOtherInformation | Ericsson | Rel-17 | 38.331 | NR_IAB-Core | 4703 | 1 | A |
| R2-2404964 | Dummy the rrc-TransactionIdentifier field from IABOtherInformation | Ericsson | Rel-18 | 38.331 | NR_IAB-Core | 4704 | 1 | A |
| R2-2404988 | Clarification on TRS in idle and inactive | Ericsson, MediaTek, ZTE, Nokia, Huawei, HiSilicon, Apple, CATT | Rel-17 | 38.331 | NR_UE_pow_sav_enh-Core | 4754 | 2 | F |
| R2-2405055 | Miscellaneous RRC corrections for NCR | ZTE Corporation (Rapporteur), Nokia | Rel-18 | 38.331 | NR_netcon_repeater-Core | 4809 |  | F |
| R2-2405135 | Miscellaneous MAC corrections for network energy savings | InterDigital | Rel-18 | 38.321 | Netw_Energy_NR-Core | 1855 |  | F |
| R2-2405145 | Miscellaneous UAV-related Stage-2 Corrections | Nokia, Ericsson, Qualcomm | Rel-18 | 38.300 | NR_UAV-Core | 0867 |  | F |
| R2-2405175 | Clarification on SIB1 reception for ETWS/CMAS | Samsung, Ericsson | Rel-15 | 38.331 | NR_newRAT-Core | 4817 |  | F |
| R2-2405176 | Clarification on SIB1 reception for ETWS/CMAS | Samsung, Ericsson | Rel-16 | 38.331 | NR_newRAT-Core | 4818 |  | A |
| R2-2405177 | Clarification on SIB1 reception for ETWS/CMAS | Samsung, Ericsson | Rel-17 | 38.331 | NR_newRAT-Core | 4819 |  | A |
| R2-2405178 | Clarification on SIB1 reception for ETWS/CMAS | Samsung, Ericsson | Rel-18 | 38.331 | NR_newRAT-Core | 4820 |  | A |
| R2-2405253 | LPP support for sub 1s location information reporting periodicity [Sub_1s_periodicity] | Ericsson, AT&T, T-Mobile, Vivo, Deutsche Telekom, Huawei, HiSilicon, Vodafone | Rel-18 | 37.355 | TEI18 | 0501 | 3 | B |
| R2-2405254 | Corrections for Bluetooth AoA/AoD [BT-AoA-AoD] | Ericsson, Intel Corporation | Rel-18 | 37.355 | TEI18 | 0502 | 1 | F |
| R2-2405255 | Introduction of NR UE Rx-Tx time difference measurement in NR UL E-CID | Ericsson, Polaris Wireless, China Telecom, NTT Docomo, AT&T, FirstNet, Intel, Comtech, Nokia, Nokia Shanghai Bell, Verizon Wireless, Huawei, ZTE | Rel-18 | 38.305 | NR_pos_enh-Core, TEI18 | 0164 | 2 | F |
| R2-2405263 | Clarification to Network-Controlled Repeaters Stage-2 description | Ericsson, Nokia | Rel-18 | 38.300 | NR_netcon_repeater | 0808 | 3 | F |
| R2-2405278 | Reference for User Service Description | Nokia (Rapporteur), Ericsson, Xiaomi, Qualcomm Incorporated | Rel-17 | 38.300 | NR_MBS-Core | 0865 |  | F |
| R2-2405279 | Reference for User Service Description | Nokia (Rapporteur), Ericsson, Xiaomi, Qualcomm Incorporated | Rel-18 | 38.300 | NR_MBS-Core | 0866 |  | A |
| R2-2405280 | Reference for User Service Description | Nokia, Ericsson, Xiaomi, Qualcomm Incorporated | Rel-17 | 38.304 | NR_MBS-Core | 0404 |  | F |
| R2-2405281 | Reference for User Service Description | Nokia, Ericsson, Xiaomi, Qualcomm Incorporated | Rel-18 | 38.304 | NR_MBS-Core | 0405 |  | A |
| R2-2405346 | Corrections for SL RLC mode indication | ZTE, Sanechips | Rel-16 | 38.331 | 5G_V2X_NRSL-Core | 4825 |  | F |
| R2-2405347 | Corrections for SL RLC mode indication | ZTE, Sanechips | Rel-17 | 38.331 | 5G_V2X_NRSL-Core | 4826 |  | A |
| R2-2405348 | Corrections for SL RLC mode indication | ZTE, Sanechips | Rel-18 | 38.331 | 5G_V2X_NRSL-Core | 4827 |  | A |
| R2-2405359 | Miscellaneous corrections | Samsung (Rapporteur) | Rel-16 | 38.321 | 5G_V2X_NRSL-Core | 1810 | 1 | F |
| R2-2405360 | Miscellaneous corrections | Samsung (Rapporteur) | Rel-17 | 38.321 | 5G_V2X_NRSL-Core, NR_SL_enh-Core, NR_FeMIMO-Core | 1811 | 1 | F |
| R2-2405361 | Miscellaneous corrections | Samsung (Rapporteur) | Rel-18 | 38.321 | 5G_V2X_NRSL-Core, NR_SL_enh-Core, NR_FeMIMO-Core, NR_HST_FR2_enh-Core, NR_MT_SDT-Core | 1812 | 1 | F |
| R2-2405363 | Clarification to s-MeasConfig for L2 U2N relay case | Huawei, HiSilicon, Sharp, CATT, LG | Rel-17 | 38.331 | NR_SL_relay-Core | 4830 |  | F |
| R2-2405364 | Clarification to s-MeasConfig for L2 U2N relay case | Huawei, HiSilicon, Sharp, CATT, LG | Rel-18 | 38.331 | NR_SL_relay-Core | 4831 |  | A |
| R2-2405365 | Miscellaneous RRC corrections for SL relay | Huawei, HiSilicon | Rel-17 | 38.331 | NR_SL_relay-Core | 4682 | 2 | F |
| R2-2405366 | Miscellaneous RRC corrections for SL relay | Huawei, HiSilicon | Rel-18 | 38.331 | NR_SL_relay-Core | 4683 | 2 | A |
| R2-2405370 | Clarification to the band pair reporting | Huawei, HiSilicon, Qualcomm Incorporated | Rel-18 | 38.306 | NR_MC_enh-Core | 1115 |  | F |
| R2-2405408 | Correction on SidelinkUEInformationNR | Philips International B.V. | Rel-17 | 38.331 | NR_SL_relay-Core | 4731 | 1 | F |
| R2-2405412 | Correction on SidelinkUEInformationNR | Philips International B.V. | Rel-18 | 38.331 | NR_SL_relay-Core | 4732 | 1 | A |
| R2-2405457 | Miscellaneous Corrections (Rapporteur) | Qualcomm Incorporated, Nokia, CATT | Rel-17 | 38.304 | NR_UE_pow_sav_enh-Core, NR_redcap-Core | 0406 |  | F |
| R2-2405459 | Miscellaneous Corrections (Rapporteur) | Qualcomm Incorporated, Ericsson, Nokia, CATT | Rel-18 | 38.304 | NR_UE_pow_sav_enh-Core, NR_redcap-Core, NR_UAV-Core | 0407 |  | F |
| R2-2405484 | Miscellaneous corrections for IDC | Xiaomi, Nokia, Huawei, HiSilicon, ZTE Corporation, Sanechips, Ericsson, Samsung | Rel-18 | 38.331 | NR_IDC_enh-Core | 4838 |  | F |
| R2-2405534 | Miscellaneous correction for IoT-NTN | Nokia | Rel-18 | 36.304 | IoT_NTN_enh-Core | 0873 | 2 | F |
| R2-2405556 | Clarification on supporting two logical DUs and connecting via stationary IAB node | ZTE, Qualcomm, Ericsson, Samsung, Nokia | Rel-18 | 38.300 | NR_mobile_IAB-Core | 0853 | 2 | F |
| R2-2405585 | Correction on RRC signalling for advanced receiver | China Telecom, CATT | Rel-18 | 38.331 | NR_demod_enh3-Core | 4673 | 1 | F |
| R2-2405618 | Network energy savings for NR miscellaneous RRC CR | Huawei, HiSilicon | Rel-18 | 38.331 | Netw_Energy_NR-Core | 4692 | 2 | F |
| R2-2405624 | Clarification on the combination of HST and RRM measurement relaxation | Apple, Ericsson, Nokia (Rapporteur) | Rel-17 | 38.300 | NR_UE_pow_sav-Core, NR_HST | 0840 | 2 | A |
| R2-2405625 | Clarification on the combination of HST and RRM measurement relaxation | Apple, Ericsson, Nokia (Rapporteur) | Rel-18 | 38.300 | NR_UE_pow_sav-Core, NR_HST | 0841 | 2 | A |
| R2-2405647 | Correction on cell status for 2Rx XR UE [2Rx_XR_Device] | Huawei, HiSilicon | Rel-18 | 38.304 | TEI18 | 0408 |  | F |
| R2-2405648 | Correction for resumeIndication [SDT_ReleaseEnh] | Huawei, HiSilicon, Nokia, ZTE Corporation, Sanechips, Intel Corporation | Rel-18 | 38.300 | TEI18 | 0848 | 1 | F |
| R2-2405686 | Mismatch of terminology between 38.304 and 38.331 | Samsung | Rel-18 | 38.304 | NR_mobile_IAB-Core | 0398 | 1 | F |
| R2-2405717 | Clarification RLM/BFD relaxation and short DRX | Ericsson, Nokia, Qualcomm, Huawei, HiSilicon | Rel-17 | 38.331 | NR_UE_pow_sav_enh-Core | 4770 | 2 | F |
| R2-2405718 | CEF and RLF reporting for RedCap UEs | MediaTek Inc., Qualcomm Incorporated, Nordic Semiconductor ASA, Ericsson | Rel-17 | 38.306 | NR_SON_MDT-Core, NR_redcap-Core | 1060 | 2 | F |
| R2-2405720 | CEF and RLF reporting for RedCap UEs | MediaTek Inc., Qualcomm Incorporated, Nordic Semiconductor ASA, Ericsson | Rel-17 | 38.331 | NR_SON_MDT-Core, NR_redcap-Core | 4647 | 2 | F |
| R2-2405722 | Mandating the capability mux-HARQ-ACK-withoutPUCCH-onPUSCH-r16 | MediaTek Inc., Ericsson | Rel-17 | 38.306 | NR_newRAT-Core, TEI17 | 1112 | 1 | F |
| R2-2405723 | Mandating the capability mux-HARQ-ACK-withoutPUCCH-onPUSCH-r16 | MediaTek Inc., Ericsson | Rel-18 | 38.306 | NR_newRAT-Core, TEI18 | 1113 | 1 | F |
| R2-2405727 | Correction to UE capability description for fallback BC behavior | Ericsson, Nokia (Rapporteur) | Rel-15 | 38.300 | NR_newRAT-Core | 0843 | 2 | F |
| R2-2405728 | Correction to UE capability description for fallback BC behavior | Ericsson, Nokia (Rapporteur) | Rel-16 | 38.300 | NR_newRAT-Core | 0844 | 2 | A |
| R2-2405729 | Correction to UE capability description for fallback BC behavior | Ericsson, Nokia (Rapporteur) | Rel-17 | 38.300 | NR_newRAT-Core | 0845 | 2 | A |
| R2-2405730 | Correction to UE capability description for fallback BC behavior | Ericsson, Nokia (Rapporteur) | Rel-18 | 38.300 | NR_newRAT-Core | 0846 | 2 | A |
| R2-2405732 | Clarification of PDCCH ordered CFRA for 2TA | NTT DOCOMO, INC., Samsung, Nokia | Rel-18 | 38.300 | NR_MIMO_evo_DL_UL-Core | 0868 |  | F |
| R2-2405752 | IoT NTN measurement corrections | Ericsson, Huawei | Rel-18 | 36.300 | IoT_NTN_enh-Core | 1401 | 2 | F |
| R2-2405753 | Miscellaneous corrections to Rel-18 NR NTN | Ericsson | Rel-18 | 38.331 | NR_NTN_enh-Core | 4761 | 4 | F |
| R2-2405755 | Terminology alignment in 38.304 for NR-NTN | ZTE Corporation, Sanechips, Ericsson, CATT, Nokia, Intel, Huawei, HiSilicon, Samsung | Rel-18 | 38.304 | NR_NTN_enh-Core | 0403 | 2 | F |
| R2-2405758 | Corrections to IOT NTN | Huawei, HiSilicon | Rel-18 | 36.331 | IoT_NTN_enh-Core | 5021 | 1 | F |
| R2-2405759 | Corrections on UE behaviour on DRX for IoT NTN | MediaTek | Rel-18 | 36.321 | IoT_NTN_enh-Core | 1585 | 3 | F |
| R2-2405768 | Correction on NR NTN | CATT (Rapporteur) | Rel-18 | 37.355 | NR_NTN_enh-Core | 0504 | 1 | F |
| R2-2405771 | Clarification on MAC reset for multicast reception in RRC_INACTIVE | Apple, Samsung, CATT, Huawei, HiSilicon, Nokia, Sharp, Qualcomm Incorporated, Ericsson, Langbo | Rel-18 | 38.321 | NR_MBS_enh-Core | 1842 | 1 | F |
| R2-2405783 | Correction on the configuration of Redcap CFR [RedCapMBS_Bcast] | Huawei, HiSilicon | Rel-18 | 38.331 | TEI18 | 4816 | 2 | F |
| R2-2405784 | Correction on MBS search spaces configuration for (e)Redcap [RedCapMBS_Bcast] | Huawei, CATT, Xiaomi, HiSilicon | Rel-18 | 38.331 | TEI18 | 4689 | 3 | F |
| R2-2405785 | RRC corrections for XR | Huawei, HiSilicon | Rel-18 | 38.331 | NR_XR_enh-Core | 4700 | 3 | F |
| R2-2405801 | IoT NTN Kmac correction | Ericsson | Rel-17 | 36.300 | LTE_NBIOT_eMTC_NTN-Core | 1402 | 1 | F |
| R2-2405802 | IoT NTN Kmac corrections | Ericsson | Rel-18 | 36.300 | LTE_NBIOT_eMTC_NTN-Core | 1403 |  | A |
| R2-2405809 | Clarification on the SRS Carrier Switching for the PUSCH-less Cell (r15) | ZTE Corporation, Sanechips, Ericsson, Samsung, Qualcomm Incorporated | Rel-15 | 38.306 | NR_newRAT-Core | 1100 | 1 | F |
| R2-2405810 | Clarification on the SRS Carrier Switching for the PUSCH-less Cell (r16) | ZTE Corporation, Sanechips, Ericsson, Samsung, Qualcomm Incorporated | Rel-16 | 38.306 | NR_newRAT-Core | 1101 | 1 | A |
| R2-2405811 | Clarification on the SRS Carrier Switching for the PUSCH-less Cell (r17) | ZTE Corporation, Sanechips, Ericsson, Samsung, Qualcomm Incorporated | Rel-17 | 38.306 | NR_newRAT-Core | 1102 | 1 | A |
| R2-2405812 | Clarification on the SRS Carrier Switching for the PUSCH-less Cell (r18) | ZTE Corporation, Sanechips, Ericsson, Samsung, Qualcomm Incorporated | Rel-18 | 38.306 | NR_newRAT-Core | 1103 | 1 | A |
| R2-2405813 | Clarification on the parallel Tx Capability (r15) | ZTE Corporation, Sanechips, Ericsson | Rel-15 | 38.306 | NR_newRAT-Core | 1104 | 1 | F |
| R2-2405817 | Clarification on xDD differentiation for SDL bands | Huawei, HiSilicon | Rel-15 | 38.306 | NR_newRAT-Core | 1116 | 1 | F |
| R2-2405818 | Clarification on xDD differentiation for SDL bands | Huawei, HiSilicon | Rel-16 | 38.306 | NR_newRAT-Core | 1117 | 1 | A |
| R2-2405819 | Clarification on xDD differentiation for SDL bands | Huawei, HiSilicon | Rel-17 | 38.306 | NR_newRAT-Core | 1118 | 1 | A |
| R2-2405820 | Clarification on xDD differentiation for SDL bands | Huawei, HiSilicon | Rel-18 | 38.306 | NR_newRAT-Core | 1119 | 1 | A |
| R2-2405821 | Adding PCI and ARFCN of target cell for intra-RAT SHR | ZTE Corporation, Sanechips, Ericsson, Huawei, CATT, Samsung | Rel-17 | 38.331 | NR_ENDC_SON_MDT_enh-Core | 4811 | 1 | F |
| R2-2405823 | Misclassification of RLF reports as Too Early HO failure | Ericsson | Rel-17 | 38.331 | NR_ENDC_SON_MDT_enh-Core | 4813 | 1 | F |
| R2-2405825 | UE capability for misclassification of RLF reports as Too Early HO failure | Ericsson | Rel-17 | 38.306 | NR_ENDC_SON_MDT_enh-Core | 1127 |  | F |
| R2-2405827 | Clarification on the RSRP measurement for SDT | Xiaomi, Ericsson | Rel-17 | 38.331 | NR_SmallData_INACTIVE-Core | 4839 | 1 | F |
| R2-2405828 | Clarification on the RSRP measurement for SDT | Xiaomi, Ericsson | Rel-18 | 38.331 | NR_SmallData_INACTIVE-Core | 4840 | 1 | A |
| R2-2405829 | Correction to PDCP configuration for multicast MRB | MediaTek inc. | Rel-17 | 38.331 | NR_MBS-Core | 4652 | 2 | F |
| R2-2405830 | Correction to PDCP configuration for multicast MRB | MediaTek Inc. | Rel-18 | 38.331 | NR_MBS-Core | 4651 | 2 | A |
| R2-2405833 | Clarification on RACH-ConfigCommon for PDCCH order based CFRA and SI request | ZTE Corporation, Samsung | Rel-17 | 38.331 | NR_redcap-Core | 4807 | 1 | F |
| R2-2405834 | Clarification on RACH-ConfigCommon for PDCCH order based CFRA and SI request | ZTE Corporation, Samsung | Rel-18 | 38.331 | NR_redcap-Core, NR_cov_enh2, NR_redcap_enh | 4808 | 1 | F |
| R2-2405837 | Clarification on srs-SwitchingAffectedBandsListNR | Huawei, HiSilicon | Rel-17 | 38.306 | TEI17 | 1120 | 1 | F |
| R2-2405838 | Clarification on srs-SwitchingAffectedBandsListNR | Huawei, HiSilicon | Rel-18 | 38.306 | TEI17 | 1121 | 1 | A |
| R2-2405841 | Correction of availability indication of logged MDT report for SNPN | Nokia | Rel-18 | 37.320 | NR_ENDC_SON_MDT_enh2-Core | 0132 | 1 | F |
| R2-2405842 | Corrections to TS 37.340 on collection of RA reports | Huawei, HiSilicon | Rel-18 | 37.340 | NR_ENDC_SON_MDT_enh2-Core | 0393 |  | F |
| R2-2405849 | Miscellaneous non-controversial corrections Set XXI | Ericsson | Rel-17 | 38.331 | NR_newRAT-Core, TEI17 | 4718 | 2 | F |
| R2-2405852 | Missing Conditionally mandatory features without UE radio access capability parameters for 80ms scheduling offset for positioning SI acquisition | Ericsson | Rel-18 | 38.306 | NR_pos-Core | 1086 | 3 | A |
| R2-2405853 | Missing Conditionally mandatory features without UE radio access capability parameters for 80ms scheduling offset for positioning SI acquisition | Ericsson | Rel-17 | 38.306 | NR_pos-Core | 1088 | 3 | A |
| R2-2405854 | Missing Conditionally mandatory features without UE radio access capability parameters for 80ms scheduling offset for positioning SI acquisition | Ericsson | Rel-16 | 38.306 | NR_pos-Core | 1087 | 3 | F |
| R2-2405859 | Correcting Figures | Nokia (Rapporteur) | Rel-18 | 38.300 | TEI18, NR_newRAT-Core | 0859 | 1 | F |
| R2-2405861 | Corrections related to LPP RILs E001-E003 and Q033 due to agreed CT4 corrections [PosLocalCoords] | Ericsson, Qualcomm Incorporated | Rel-18 | 37.355 | TEI18 | 0510 | 2 | F |
| R2-2405862 | Correction on the UL TEG report | Huawei, HiSilicon, Ericsson | Rel-17 | 38.331 | NR_pos_enh-Core | 4794 | 1 | F |
| R2-2405864 | Correction on posSIB(s) acquisition [SI-SCHEDULING] | Philips International B.V., Ericsson | Rel-17 | 38.331 | TEI17 | 4467 | 3 | F |
| R2-2405865 | Correction on posSIB(s) acquisition [SI-SCHEDULING] | Philips International B.V., Ericsson | Rel-18 | 38.331 | TEI17 | 4725 | 2 | A |
| R2-2405877 | RRC corrections for Rel-18 SL relay enhancements | Huawei, HiSilicon, ASUSTeK, Nokia, OPPO, Ericsson, ZTE, CATT, Sharp, Lenovo, Apple, MediaTek, Xiaomi, Samsung | Rel-18 | 38.331 | NR_SL_relay_enh-Core | 4684 | 3 | F |
| R2-2405881 | Corrections for NR sidelink relay enhancements | OPPO, ZTE | Rel-18 | 38.351 | NR_SL_relay_enh-Core | 0034 | 3 | F |
| R2-2405882 | Miscellaneous Rapporteur Corrections to 38.323 for SL Relay | InterDigital, ZTE | Rel-18 | 38.323 | NR_SL_relay_enh-Core | 0135 | 3 | F |
| R2-2405883 | Corrections to TS 37.355 | CATT | Rel-18 | 37.355 | NR_pos_enh2-Core | 0500 | 3 | F |
| R2-2405884 | Miscellaneous and RRC Positioning RILs based Corrections | Ericsson | Rel-18 | 38.331 | NR_pos_enh2-Core | 4759 | 3 | F |
| R2-2405885 | Miscellaneous Stage 2 Corrections | Qualcomm Incorporated | Rel-18 | 38.305 | NR_pos_enh2-Core | 0163 | 2 | F |
| R2-2405886 | Corrections to TS 38.300 for SL positioning | vivo, Huawei, HiSilicon, CATT | Rel-18 | 38.300 | NR_pos_enh2-Core | 0881 |  | F |
| R2-2405887 | Miscellaneous corrections to SLPP specification | Intel Corporation | Rel-18 | 38.355 | NR_pos_enh2-Core | 0003 | 3 | F |
| R2-2405888 | Rapporteur MAC CR for R18 positioning | Huawei, HiSilicon | Rel-18 | 38.321 | NR_pos_enh2 | 1844 | 1 | F |
| R2-2405891 | Correction to MAC on cast type | Ericsson | Rel-17 | 38.321 | NR_SL_enh-Core | 1837 | 1 | F |
| R2-2405892 | Correction to MAC on cast type | Ericsson | Rel-18 | 38.321 | NR_SL_enh-Core | 1838 | 1 | A |
| R2-2405893 | Correction on tx profile for SL DRX | ZTE Corporation, Sanechips | Rel-17 | 38.331 | NR_SL_enh-Core | 4757 | 3 | F |
| R2-2405894 | Correction on tx profile for SL DRX | ZTE Corporation, Sanechips | Rel-18 | 38.331 | NR_SL_enh-Core | 4758 | 3 | A |
| R2-2405895 | Correction on NR SL discovery transmission | Philips International B.V. | Rel-17 | 38.331 | NR_SL_enh-Core | 4834 | 1 | D |
| R2-2405896 | Correction on NR SL discovery transmission | Philips International B.V. | Rel-18 | 38.331 | NR_SL_enh-Core | 4835 | 1 | A |
| R2-2405898 | Correction to resource selection for LTE-NR cochannel scenario | Ericsson, LG Electronics Inc. | Rel-18 | 38.321 | NR_SL_enh2 | 1807 | 2 | F |
| R2-2405899 | Correction on Release-18 SL Evolution | OPPO | Rel-18 | 38.331 | NR_SL_enh2 | 4646 | 3 | F |
| R2-2405900 | Correction on Release-18 Sidelink evolution | LG | Rel-18 | 38.321 | NR_SL_enh2-Core | 1830 | 3 | F |
| R2-2405901 | Miscellaneous Rapporteur Stage 2 Corrections for NR Sidelink Evolution | InterDigital | Rel-18 | 38.300 | NR_SL_enh2 | 0838 | 3 | F |
| R2-2405924 | Correction on RACH resource set availability check | vivo, Guangdong Genius | Rel-18 | 38.321 | NR_redcap-Core, NR_cov_enh-Core, NR_SmallData_INACTIVE-Core | 1828 | 3 | A |
| R2-2405926 | B020 Correction of cell barred indication value for 2Rx XR [2Rx_XR_Device] | Lenovo | Rel-18 | 38.331 | TEI18 | 4858 |  | F |
| R2-2405930 | Clarification on CIO configured within ReportConfig [CIO_in_ReportConfig] | Ericsson, NTT DOCOMO, INC., Qualcomm Incorporated | Rel-18 | 38.331 | TEI18 | 4707 | 2 | F |
| R2-2405932 | PDCP SN gap reporting | LG Electronics Inc. (Rapporteur) | Rel-18 | 38.323 | NR_XR_enh-Core | 0139 |  | F |
| R2-2405934 | Miscellaneous XR Corrections | Nokia (Rapporteur) | Rel-18 | 38.300 | NR_XR_enh-Core | 0860 | 1 | F |
| R2-2405946 | Correction on 3Tx SAR for inter-band CA with PC1.5 | Huawei, HiSilicon | Rel-17 | 38.306 | 4Rx_low_NR_band_handheld_3Tx_NR_CA_ENDC-Core | 1122 | 1 | F |
| R2-2405947 | Correction on 3Tx SAR for inter-band CA with PC1.5 | Huawei, HiSilicon | Rel-18 | 38.306 | 4Rx_low_NR_band_handheld_3Tx_NR_CA_ENDC-Core | 1123 | 1 | A |
| R2-2405952 | [S081] Clarification for order of list in PagingRecordList | ZTE Corporation (rapporteur), Samsung | Rel-18 | 38.331 | NR_MT_SDT-Core, LTE_NR_MUSIM-Core | 4787 | 1 | F |
| R2-2405953 | SDT corrections for harmonizing T319a and SDT ongoing labels | ZTE Corporation (rapporteur), Ericsson | Rel-18 | 38.331 | NR_SmallData_INACTIVE-Core, NR_MT_SDT-Core | 4687 | 2 | F |
| R2-2405956 | Introduction of barring exemption for RedCap UEs for emergency calls [RedCap_EM_Call] | Apple, China Telecom, Vodafone, Verizon, TMobile USA, ZTE, Vivo, Ericsson, Nokia, Huawei | Rel-18 | 38.304 | TEI18 | 0380 | 5 | B |
| R2-2405957 | Introduction of barring exemption for RedCap UEs for emergency calls [RedCap_EM_Call] | Apple, China Telecom, Vodafone, Verizon, TMobile USA, ZTE, Vivo, Ericsson, Nokia, Huawei | Rel-18 | 38.331 | TEI18 | 4570 | 4 | B |
| R2-2405958 | Introduction of barring exemption for (e)RedCap and 2RX XR UEs for emergency calls [EM_Call_Exemption] | Apple, China Telecom, Vodafone, Verizon, TMobile USA, ZTE, Vivo, Ericsson. Nokia, Huawei | Rel-18 | 38.304 | TEI18 | 0381 | 6 | B |
| R2-2405959 | Miscellaneous stage-2 corrections for network energy savings | Ericsson | Rel-18 | 38.300 | Netw_Energy_NR-Core | 0870 |  | F |
| R2-2405966 | Introduction of Inter-node Coordination on the Aggregated Bandwidth for the NR-DC (r17) | ZTE Corporation, Sanechips,Ericsson,Nokia,Huawei, HiSilicon | Rel-17 | 38.331 | NR_BCS4-Core | 4735 | 3 | F |
| R2-2405967 | Introduction of Inter-node Coordination on the Aggregated Bandwidth for the NR-DC (r18) | ZTE Corporation, Sanechips,Ericsson,Nokia,Huawei, HiSilicon | Rel-18 | 38.331 | NR_BCS4-Core | 4736 | 3 | A |
| R2-2405969 | Corrections to TS 36.331 for R18 SONMDT | Huawei, HiSilicon | Rel-18 | 36.331 | NR_ENDC_SON_MDT_enh2-Core | 5022 | 1 | F |
| R2-2405972 | Capabilities for Rel-18 Enhanced LTE Support for UAV WI | Huawei, HiSilicon, CATT, Qualcomm Inc., Ericsson | Rel-18 | 36.306 | LTE_UAV_enh-Core | 1884 | 2 | F |
| R2-2405975 | Introduction of new intra-band EN-DC capabilities for inter-band EN-DC | Google Inc., CATT | Rel-17 | 38.331 | NR_newRAT-Core, TEI17 | 4750 | 4 | F |
| R2-2405976 | Introduction of new intra-band EN-DC capabilities for inter-band EN-DC | Google Inc., CATT | Rel-17 | 38.306 | NR_newRAT-Core, TEI17 | 1084 | 4 | F |
| R2-2405977 | Introduction of new intra-band EN-DC capabilities for inter-band EN-DC | Google Inc., CATT | Rel-18 | 38.331 | NR_newRAT-Core, TEI17 | 4751 | 4 | A |
| R2-2405978 | Introduction of new intra-band EN-DC capabilities for inter-band EN-DC | Google Inc., CATT | Rel-18 | 38.306 | NR_newRAT-Core, TEI17 | 1085 | 4 | A |
| R2-2405979 | Miscellaneous corrections for eRedCap | Ericsson, Nokia | Rel-18 | 38.331 | NR_redcap_enh-Core | 4729 | 3 | F |
| R2-2405981 | Miscellaneous corrections on LPP for Rel-18 positioning UE capabilities | Xiaomi | Rel-18 | 37.355 | NR_pos_enh2-Core | 0503 | 2 | F |
| R2-2405982 | CR 38.355 for SLPP capability | Xiaomi | Rel-18 | 38.355 | NR_pos_enh2-Core | 0004 | 2 | B |
| R2-2405983 | Correction to Positioning SRS Configuration | Huawei, HiSilicon | Rel-16 | 38.331 | NR_pos-Core | 4791 | 2 | F |
| R2-2405984 | Correction to Positioning SRS Configuration | Huawei, HiSilicon | Rel-17 | 38.331 | NR_pos-Core | 4792 | 2 | A |
| R2-2405985 | Correction to Positioning SRS Configuration | Huawei, HiSilicon | Rel-18 | 38.331 | NR_pos-Core | 4793 | 2 | A |
| R2-2405994 | Enhancements to measurement report [meas_report_enh] | Ericsson, T-Mobile USA, Turkcell, Rakuten Mobile, BT Plc., NTT Docomo, Deutsche Telekom, MediaTek Inc., Verizon, AT&T, Vodafone, Continental Automotive, KDDI, Charter, NEC, Telecom Italia, CATT, Reliance Jio, Qualcomm Incorporated, ZTE Corporation, Apple, Nokia | Rel-18 | 38.306 | TEI18 | 1110 | 1 | B |
| R2-2405998 | Correction on resource pool selection for A2X communication | Samsung, Sharp, ZTE, Sanechips | Rel-18 | 38.321 | NR_UAV-Core | 1813 | 3 | F |
| R2-2406008 | Rapp RRC CR for MC enhancement | Huawei, HiSilicon, NTT DOCOMO INC, Samsung, Ericsson, OPPO | Rel-18 | 38.331 | NR_MC_enh-Core | 4832 | 1 | F |
| R2-2406010 | Introduction of NR support for dedicated spectrum less than 5MHz for FR1 | Qualcomm Incorporated | Rel-18 | 38.331 | NR_FR1_lessthan_5MHz_BW-Core | 4525 | 4 | B |
| R2-2406011 | Introduction of NR support for dedicated spectrum less than 5MHz for FR1 | Qualcomm Incorporated | Rel-18 | 36.331 | NR_FR1_lessthan_5MHz_BW-Core | 4983 | 4 | B |
| R2-2406013 | Correction to mandatory 80ms scheduling offset for positioning SI acquisition | Huawei, HiSilicon, Ericsson | Rel-15 | 36.306 | LCS_LTE_acc_enh-Core | 1885 | 1 | F |
| R2-2406014 | Correction to mandatory 80ms scheduling offset for positioning SI acquisition | Huawei, HiSilicon, Ericsson | Rel-16 | 36.306 | LCS_LTE_acc_enh-Core | 1886 | 1 | A |
| R2-2406015 | Correction to mandatory 80ms scheduling offset for positioning SI acquisition | Huawei, HiSilicon, Ericsson | Rel-17 | 36.306 | LCS_LTE_acc_enh-Core | 1887 | 1 | A |
| R2-2406016 | Correction to mandatory 80ms scheduling offset for positioning SI acquisition | Huawei, HiSilicon, Ericsson | Rel-18 | 36.306 | LCS_LTE_acc_enh-Core | 1888 | 1 | A |
| R2-2406018 | RACH-less support generalization [RACH-lessHO] | Nokia | Rel-18 | 38.300 | TEI18, NR_NTN_enh-Core, NR_mobile_IAB-Core | 0799 | 4 | B |
| R2-2406020 | Clarification on HARQ RTT Timer operation | Apple, Nokia, Nokia Shanghai Bell, MediaTek Inc., ZTE, Huawei, HiSilicon, Samsung | Rel-18 | 38.321 | TEI18 | 1843 | 2 | F |
| R2-2406021 | Enhancements to measurement report [meas_report_enh] | Ericsson, T-Mobile USA, Turkcell, Rakuten Mobile, BT Plc., NTT Docomo, Deutsche Telekom, MediaTek Inc., Verizon, AT&T, Vodafone, Continental Automotive, KDDI, Charter, NEC, Telecom Italia, CATT, Reliance Jio, Qualcomm Incorporated, ZTE Corporation, Apple, Nokia | Rel-18 | 38.331 | TEI18 | 4803 | 2 | B |
| R2-2406024 | Miscellaneous RRC corrections for Coverage Enhancements | Huawei, HiSilicon | Rel-18 | 38.331 | NR_cov_enh2-Core | 4833 | 1 | F |
| R2-2406025 | Miscellaneous corrections for eMBS | Huawei, HiSilicon, CATT, Samsung, LG Electronics Inc., CMCC, Xiaomi | Rel-18 | 38.331 | NR_MBS_enh-Core | 4688 | 3 | F |
| R2-2406027 | Clarifications for MBS RedCap CFR [RedCapMBS_Bcast] | Ericsson, CATT, ZTE | Rel-18 | 38.300 | TEI18 | 0864 | 2 | B |
| R2-2406030 | Miscellaneous corrections for further mobility enhancements | Huawei, HiSilicon | Rel-18 | 38.321 | NR_Mob_enh2-Core | 1817 | 3 | F |
| R2-2406034 | MBS operation with eDRX MICO | Nokia, Ericsson | Rel-18 | 38.304 | 5MBS_Ph2 | 0399 | 3 | B |
| R2-2406043 | Corrections for NR Support for UAV (Uncrewed Aerial Vehicles) | Qualcomm Incorporated | Rel-18 | 38.331 | NR_UAV-Core | 4693 | 3 | F |
| R2-2406044 | Corrections to Enhanced LTE Support for UAV (Uncrewed Aerial Vehicles) | Qualcomm Incorporated, Nokia | Rel-18 | 36.331 | LTE_UAV_enh-Core | 5004 | 3 | F |
| R2-2406047 | Clarification for RedCap UE supporting MBS broadcast [RedCapMBS_Bcast] | Ericsson | Rel-18 | 38.306 | TEI18 | 1111 | 1 | B |
| R2-2406048 | Correction on the MT communication handling procedure | R3 (Huawei, ZTE, Ericsson, Nokia, CATT, Qualcomm Incorporated, China Telecom) | Rel-18 | 38.300 | NR_redcap_enh-Core | 0871 |  | F |
| R2-2406049 | Correction on SDT RRC Release with resume indication [SDT_ReleaseEnh] | R3 (ZTE, Huawei, China Telecom, Qualcomm, LG Electronics, China Unicom, Samsung, CATT) | Rel-18 | 38.300 | TEI18 | 0872 |  | B |
| R2-2406050 | Clarification on non-homogeneous deployment | R3 (Nokia, Nokia Shanghai Bell, ZTE, Huawei, CATT, Ericsson, Qualcomm Inc, CMCC, Xiaomi, Samsung, Lenovo, CATT) | Rel-18 | 38.300 | NR_XR_enh-Core | 0873 |  | F |
| R2-2406051 | Correction on DL PSI based discard | R3 (Huawei, CATT, CMCC, Qualcomm, Ericsson, Nokia, Nokia Shanghai Bell, ZTE, China Telecom) | Rel-18 | 38.300 | NR_XR_enh-Core | 0874 |  | F |
| R2-2406052 | Introduction of Measurement Quality and Time Stamp Information to E-CID [ECIDQualTimeStamp] | R3 (Ericsson, Comtech, Verizon Wireless, Polaris Wireless, Nokia, ZTE) | Rel-18 | 38.305 | TEI18 | 0170 |  | B |
| R2-2406053 | Correction on burst arrival time in handover | R3 (Lenovo, Huawei, CATT, CMCC, ZTE, Xiaomi, Samsung, Qualcomm, Nokia, Nokia Shanghai Bell, Ericsson) | Rel-18 | 38.300 | NR_XR_enh-Core | 0875 |  | F |
| R2-2406054 | Clarification of the bearer menagement in case of CHO with SCG | R3 (Nokia, LG Electronics) | Rel-18 | 37.340 | NR_Mob_enh2-Core | 0394 |  | F |
| R2-2406055 | Stage2 correction on direct data forwarding | R3 (Samsung, CATT, Huawei, Cybercore, ZTE, Nokia, LG Electronics, Ericsson) | Rel-18 | 37.340 | NR_Mob_enh2-Core | 0395 |  | F |
| R2-2406056 | Correction of Slice Availability and Support | R3 (Nokia, Deutsche Telekom, Ericsson, Huawei, CATT, ZTE) | Rel-18 | 38.300 | eNS_Ph3-NR-Core | 0876 |  | F |
| R2-2406057 | Addition of CPAC detection in stage 2 | R3 (Samsung, Lenovo, CATT, Cybercore) | Rel-18 | 37.340 | NR_ENDC_SON_MDT_enh2-Core | 0396 |  | F |
| R2-2406058 | Correction to OAM requirement for UE location verification | R3 (CATT, Huawei, ZTE, Samsung, Nokia) | Rel-18 | 38.305 | NR_NTN_enh-Core | 0171 |  | F |
| R2-2406059 | Addition of SON for fast MCG recovery in stage 2 | R3 (Samsung, Lenovo, CATT, Nokia, ZTE) | Rel-18 | 38.300 | NR_ENDC_SON_MDT_enh2-Core | 0877 |  | F |
| R2-2406060 | Stage 2 update on AI/ML for NG-RAN | R3 (CATT, Nokia, ZTE, Ericsson, Huawei, Samsung) | Rel-18 | 38.300 | NR_AIML_NGRAN-Core | 0878 |  | F |
| R2-2406061 | Correction on Handover Cancel in CHO with SN for EN-DC | R3 (Samsung, ZTE, Ericsson, Huawei) | Rel-18 | 37.340 | LTE_NR_DC_enh2-Core, TEI18 | 0397 |  | F |
| R2-2406062 | Correction on MDT for PNI-NPN | R3 (ZTE, Ericsson, Huawei, CATT) | Rel-18 | 37.320 | NR_ENDC_SON_MDT_enh2-Core | 0133 |  | F |
| R2-2406063 | Correction on Inter-system MRO | R3 (Samsung, ZTE, Nokia, CMCC) | Rel-18 | 38.300 | NR_SON_MDT-Core, TEI18 | 0879 |  | F |
| R2-2406064 | Correction on Inter-system MRO | R3 (Samsung, Nokia, ZTE, CMCC) | Rel-18 | 36.300 | NR_SON_MDT-Core, TEI18 | 1404 |  | F |
| R2-2406065 | Stage 2 corrections for AI/ML for NG-RAN | R3 (Huawei, Nokia, Ericsson, InterDigital, NEC, Deutsche Telekom, Telecom Italia, CATT, Lenovo, ZTE, Samsung) | Rel-18 | 38.300 | NR_AIML_NGRAN-Core | 0880 |  | F |
| R2-2406066 | Indicator on MDT configuration in MR-DC | R3 (ZTE) | Rel-18 | 37.320 | NR_ENDC_SON_MDT_enh2-Core | 0134 |  | F |
| R2-2406067 | Correction on user consent for trace reporting | R3 (Ericsson, AT&T, Deutsche Telekom, ZTE, CMCC, Nokia, Huawei) | Rel-18 | 37.320 | NR_SON_MDT-Core, TEI18 | 0135 |  | F |
| R2-2406071 | Corrections and Updates to UE capabilities for RAN1 feature group 55-6 | Intel Corporation | Rel-18 | 38.306 | NR_L1enh_URLLC-Core, TEI18 | 1132 |  | B |
| R2-2406072 | Corrections and Updates to UE capabilities for RAN1 feature group 55-6 | Intel Corporation | Rel-18 | 38.331 | NR_L1enh_URLLC-Core, TEI18 | 4862 |  | B |
| R2-2406073 | Miscellaneous Corrections on MIMOevo | Samsung | Rel-18 | 38.321 | NR_MIMO_evo_DL_UL-Core | 1832 | 1 | F |
| R2-2406074 | Miscellaneous corrections from ASN.1 review Q2 | Ericsson | Rel-18 | 38.331 | NR_MT_SDT-Core, NR_MBS-Core | 4717 | 2 | F |
| R2-2406076 | Correction on NR MUSIM enhancements | vivo | Rel-18 | 38.331 | NR_DualTxRx_MUSIM-Core | 4777 | 1 | F |
| R2-2406084 | Stage-2 corrections for LTM | Mediatek Inc. | Rel-18 | 38.300 | NR_Mob_enh2-Core | 0842 | 3 | F |
| R2-2406086 | Miscellaneous corrections on further mobility enhancements in NR | Ericsson | Rel-18 | 38.331 | NR_Mob_enh2-Core | 4705 | 2 | F |
| R2-2406087 | Rapporteur corrections on RRC for the generalization of RACH-less [RACH-lessHO] | Ericsson | Rel-18 | 38.331 | NR_mobile_IAB-Core, NR_Mob_enh2-Core, NR_NTN_enh-Core, TEI18 | 4706 | 2 | F |
| R2-2406089 | Clarification on RETX_COUNT during an LTM cell switch | Ericsson | Rel-18 | 38.322 | NR_Mob_enh2-Core | 0056 |  | B |
| R2-2406091 | Correction of Enhancement on NR QoE management and optimizations for diverse services | Ericsson | Rel-18 | 38.331 | NR_QoE_enh-Core | 4781 | 1 | F |
| R2-2406092 | Corrections for XR | Qualcomm Incorporated | Rel-18 | 38.321 | NR_XR_enh-Core | 1833 | 1 | F |
| R2-2406093 | Micellaneous MAC corrections for CE | ZTE Corporation | Rel-18 | 38.321 | NR_cov_enh2-Core | 1851 | 1 | F |
| R2-2406094 | Miscellaneous corrections for IoT NTN capabilities | Qualcomm Incorporated | Rel-18 | 36.306 | IoT_NTN_enh-Core | 1889 | 1 | F |
| R2-2406097 | Rapporteur correction for RACH-less HO [RACH-lessHO] | Huawei, HiSilicon | Rel-18 | 38.321 | TEI18, NR_Mob_enh2-Core, NR_NTN_enh-Core, NR_mobile_IAB-Core | 1845 | 1 | F |
| R2-2406099 | Correction on the UL TEG report | Huawei, HiSilicon, Ericsson | Rel-18 | 38.331 | NR_pos_enh-Core | 4795 | 2 | A |
| R2-2406101 | Correction on of PSCell List of RA report | R3 (ZTE, CATT, China Telecom, Nokia, Huawei, Ericsson, CMCC, China Unicom, Samsung) | Rel-18 | 37.340 | NR_ENDC_SON_MDT_enh2-Core | 0398 | 1 | F |
| R2-2406102 | Miscellaneous corrections for mobility enhancements | ZTE Corporation | Rel-18 | 37.340 | NR_Mob_enh2-Core | 0392 | 1 | F |
| R2-2406103 | Corrections for Non-terrestrial Networks | InterDigital | Rel-18 | 38.321 | NR_NTN_enh-Core | 1858 | 2 | F |
| R2-2406104 | Corrections on NR sidelink relay enhancements | LG Electronics (Rapporteur) | Rel-18 | 38.300 | NR_SL_relay_enh-Core | 0857 | 2 | F |
| R2-2406106 | Miscellaneous Corrections for TS 36.331 | Samsung, Qualcomm | Rel-14 | 36.331 | TEI14, TEI10 | 5023 | 2 | F |
| R2-2406107 | Miscellaneous Corrections for TS 36.331 | Samsung, Qualcomm | Rel-15 | 36.331 | TEI14, TEI10, TEI15, NB_IOTenh2-Core | 5024 | 2 | F |
| R2-2406108 | Miscellaneous Corrections for TS 36.331 | Samsung, Qualcomm | Rel-16 | 36.331 | TEI14, TEI10, TEI15, NB_IOTenh2-Core | 5025 | 2 | A |
| R2-2406109 | Miscellaneous Corrections for TS 36.331 | Samsung, Qualcomm | Rel-17 | 36.331 | TEI14, TEI10, LTE_NBIOT_eMTC_NTN, TEI17, NR_ext_to_71GHz-Core, TEI15, NB_IOTenh2-Core | 5026 | 2 | F |
| R2-2406113 | Correction CR for QoE measurements and conditional handover | Ericsson, China Unicom | Rel-18 | 38.331 | NR_QoE-Core, TEI18 | 4713 | 3 | F |
| R2-2406114 | Correction on the supportedBandwidthDL/UL-v1780 for the NR-DC (r18) | ZTE Corporation, Sanechips | Rel-18 | 38.306 | NR_BCS4-Core | 1078 | 2 | A |
| R2-2406115 | Stage-2 corrections on NR NTN | THALES | Rel-18 | 38.300 | NR_NTN_enh-Core | 0858 | 3 | F |
| R2-2406116 | Clarification on the parallel Tx Capability (r16) | ZTE Corporation, Sanechips, Ericsson | Rel-16 | 38.306 | NR_newRAT-Core | 1105 | 3 | A |
| R2-2406117 | Clarification on the parallel Tx Capability (r17) | ZTE Corporation, Sanechips, Ericsson | Rel-17 | 38.306 | NR_newRAT-Core | 1106 | 3 | A |
| R2-2406118 | Clarification on the parallel Tx Capability (r18) | ZTE Corporation, Sanechips, Ericsson | Rel-18 | 38.306 | NR_newRAT-Core | 1107 | 3 | A |
| R2-2406119 | Adding PCI and ARFCN of target cell for intra-RAT SHR | ZTE Corporation, Sanechips, Ericsson, Huawei, CATT, Samsung | Rel-18 | 38.331 | NR_ENDC_SON_MDT_enh-Core | 4812 | 3 | A |
| R2-2406120 | UE capability for misclassification of RLF reports as Too Early HO failure | Ericsson | Rel-18 | 38.306 | NR_ENDC_SON_MDT_enh-Core | 1128 | 1 | A |
| R2-2406121 | Correction of the range of DL-DataToUL-ACK for ATG | CMCC | Rel-18 | 38.331 | NR_ATG | 4857 | 3 | F |
| R2-2406122 | Miscellaneous non-controversial rapporteur corrections | Intel Corporation | Rel-17 | 38.306 | NR_newRAT-Core, NR_eMIMO-Core, NR_unlic-Perf, 5G_V2X_NRSL-Core, NR_IIOT_URLLC_enh-Core, NR_pos_enh-Core, NR_cov_enh-Core, NR_FeMIMO-Core, NR_ext_to_71GHz-Core, NR_MBS-Core, NR_demod_enh2-Perf, NR_SL_enh-Core, NR_NTN_solutions-Core | 1097 | 2 | F |
| R2-2406123 | SON/MDT corrections for 38.300 | Ericsson | Rel-18 | 38.300 | NR_ENDC_SON_MDT_enh2-Core | 0869 | 1 | F |
| R2-2406124 | Corrections to URLLC and Timing Resiliency | Ericsson, Nokia | Rel-18 | 38.331 | TRS_URLLC-NR-Core | 4667 | 4 | F |
| R2-2406125 | Corrections to 38331 for Rel-18 SONMDT | Ericsson | Rel-18 | 38.331 | NR_ENDC_SON_MDT_enh2-Core | 4815 | 2 | F |
| R2-2406126 | Misclassification of RLF reports as Too Early HO failure | Ericsson | Rel-18 | 38.331 | NR_ENDC_SON_MDT_enh-Core | 4814 | 3 | A |
| R2-2406127 | Corrections on TS 38.321 for 2-step RACH for eRedCap | vivo (Rapporteur) | Rel-18 | 38.321 | NR_redcap_enh-Core | 1835 | 3 | F |
| R2-2406128 | Correction to MIMO Evolution | Ericsson | Rel-18 | 38.331 | NR_MIMO_evo_DL_UL-Core | 4775 | 2 | F |
| R2-2406129 | Updates and Introduction of UE capabilities for Rel-18 WIs, including [HARQ-ACK MUX on PUSCH] | Intel Corporation | Rel-18 | 38.306 | NR_MIMO_evo_DL_UL-Core, NR_cov_enh2-Core, NR_MC_enh-Core, NR_Mob_enh2-Core, NR_FR2_multiRX_DL-Core, NR_DSS_enh-Core, NR_NTN_enh-Core, Netw_Energy_NR-Core, NR_FR1_lessthan_5MHz_BW-Core, NR_MG_enh2-Core, NR_SL_enh2-Core, NR_UAV-Core, NR_MBS_enh-Core, NR_demod_enh3-Core, NR_netcon_repeater-Core, NR_NTN_solutions-Core, NR_DualTxRx_MUSIM-Core, NR_pos_enh2-Core, NR_BWP_wor-Core, TEI18, NR_mobile_IAB-Core, NR_FR2_multiRX_DL | 1131 | 1 | B |
| R2-2406130 | Updates and Introduction of UE capabilities for Rel-18 WIs | Intel Corporation | Rel-18 | 38.331 | NR_MIMO_evo_DL_UL-Core, NR_cov_enh2-Core, NR_MC_enh-Core, NR_Mob_enh2-Core, NR_FR2_multiRX_DL-Core, Netw_Energy_NR-Core, NR_FR1_lessthan_5MHz_BW-Core, NR_MG_enh2-Core, NR_SL_enh2-Core, NR_ATG-Core, NR_MBS_enh-Core, NR_NTN_enh-Core, NR_pos_enh2-Core, NR_mobile_IAB-Core | 4861 | 1 | B |
| R2-2406131 | Correction on multicast DRX to support NTN | LG electronics Inc., Samsung, Xiaomi, Ericsson, vivo | Rel-17 | 38.321 | NR_MBS-Core, NR_NTN_solutions-Core | 1820 | 4 | F |
| R2-2406132 | Correction on multicast DRX to support NTN | LG electronics Inc., Samsung, Xiaomi, Ericsson, vivo | Rel-18 | 38.321 | NR_MBS-Core, NR_NTN_solutions-Core | 1821 | 4 | A |
| R2-2406133 | Correction on multicast DRX to support NTN | LG electronics Inc., Samsung, Xiaomi, Ericsson, vivo | Rel-17 | 38.306 | NR_MBS-Core, NR_NTN_solutions-Core | 1129 | 2 | F |
| R2-2406134 | Correction on multicast DRX to support NTN | LG electronics Inc., Samsung, Xiaomi, Ericsson, vivo | Rel-18 | 38.306 | NR_MBS-Core, NR_NTN_solutions-Core | 1130 | 2 | A |
| R2-2406135 | Correction on multicast DRX to support NTN | LG electronics Inc., Samsung, Xiaomi, Ericsson, vivo | Rel-17 | 38.331 | NR_MBS-Core, NR_NTN_solutions-Core | 4859 | 2 | F |
| R2-2406136 | Correction on multicast DRX to support NTN | LG electronics Inc., Samsung, Xiaomi, Ericsson, vivo | Rel-18 | 38.331 | NR_MBS-Core, NR_NTN_solutions-Core | 4860 | 2 | A |
| R2-2406137 | Clarification of configured grant in shared spectrum | Xiaomi, OPPO | Rel-17 | 38.331 | NR_unlic-Core, TEI17 | 4779 | 2 | F |
| R2-2406138 | Miscellaneous non-controversial corrections Set XXI | Ericsson | Rel-18 | 38.331 | NR_newRAT-Core, TEI18 | 4799 | 2 | F |
| R2-2406139 | CEF and RLF reporting for (e)RedCap UEs | MediaTek Inc., Qualcomm Incorporated, Nordic Semiconductor ASA, Ericsson | Rel-18 | 38.306 | NR_SON_MDT-Core, NR_redcap-Core | 1061 | 3 | A |
| R2-2406140 | CEF and RLF reporting for (e)RedCap UEs | MediaTek Inc., Qualcomm Incorporated, Nordic Semiconductor ASA, Ericsson | Rel-18 | 38.331 | NR_SON_MDT-Core, NR_redcap-Core | 4648 | 3 | A |
| R2-2406141 | Clarification of configured grant in shared spectrum | Xiaomi, OPPO | Rel-18 | 38.331 | NR_unlic-Core, TEI17 | 4780 | 1 | A |
| R2-2406142 | Correction on Redcap 1 Rx and 2 Rx barring | Nokia | Rel-18 | 38.331 | NR_redcap-Core, TEI18 | 4632 | 5 | F |
| R2-2406143 | Correction on TRS for idle and inactive UEs | CATT, Ericsson | Rel-18 | 38.300 | NR_UE_pow_sav_enh-Core, TEI18 | 0836 | 3 | F |
| R2-2406144 | Correction on TRS in idle and inactive | Ericsson, MediaTek, ZTE, Nokia, Huawei, HiSilicon, Apple, CATT | Rel-18 | 38.331 | NR_UE_pow_sav_enh-Core, TEI18 | 4755 | 3 | F |
| R2-2406145 | Clarification RLM/BFD relaxation and short DRX | Ericsson, Nokia, Qualcomm, Huawei, HiSilicon | Rel-18 | 38.331 | NR_UE_pow_sav_enh-Core, TEI18 | 4771 | 2 | F |
| R2-2406146 | Miscellaneous Corrections for TS 36.331 | Samsung, Qualcomm | Rel-18 | 36.331 | TEI14, LTE_NBIOT_eMTC_NTN, TEI10, TEI17, NR_ext_to_71GHz-Core, TEI15, NB_IOTenh2-Core, TEI18 | 5027 | 3 | F |

270 Agreed CRs.

# Annex F:	List of email discussions during the meeting

Discussions during the R2-126 meeting:

List of AT meeting email discussions

[AT126][002][XR] EM call in barred cell CR to 331 and 304(Apple)

[AT126][003][R17 MBS-NTN] Agree to CRs (LG)

[AT126][004][R17 UP] HARQ RTT offline (Apple)

[AT126][005][UAV] CR to 321 (Samsung)

[AT126][007][Meas Enh] Review TP and CR (Ericsson)

[AT126][008][CIO] CR to 38.331 (Ericsson)

[AT126][011][NES] 38.300  (Ericsson)

[AT126][012][RACHless] Stage 2 CR  (Nokia)

[AT126][014][IDC] Miscellaneous corrections for IDC (Xiaomi)

[AT126][015][NCR] Miscellaneous corrections (Apple)

[AT126][016][XR] Stage 2 CR (Nokia)

[AT126][018][XR] CR to 38.331 (Huawei)

[AT126][019][XR] CR to 38.323 (LG)

[AT126][020][AIML PHY] UE side data collections (Mediatek)

[AT126][022][AIoT] CB on 4 step RA (Huawei)

[AT126][023][RRC parameters] LS to RAN1 (Ericsson)

[AT126][024][Femto] LS reply  (LG)

[AT126][025][3Tx SAR] 38.306 CR (Huawei)

[AT126][026][MC enh] CR to 331 (Huawei )

[AT126][027][MC enh] 38.321 CR (Samsung)

[AT126][028][less5MHz] RRC CR (Qualcomm)

[AT126][030][AIMob] Simulation assumptions (OPPO)

[AT126][033][SDT] LS reply to RAN3 (ZTE)

[AT126][201][MIMOevo] remaining MAC issues (Samsung)

[AT126][301][NR NTN Enh] SMTC impacts for soft satellite switch (Sequans)

[AT126][302][IoT NTN Enh] T390 issues (Samsung)

[AT126][303][IoT NTN Enh] MAC issues (Ericsson)

[AT126][304][NR NTN Enh] MAC CR (Interdigital)

[AT126][305][R19 NR NTN] LS to RAN1 (CMCC)

[AT126][306][R19 IoT NTN] LS to RAN4 and RAN1 on TA for Msg3 (ZTE)

[AT126][307][IoT NTN Enh] LS to RAN1 on T390 (Samsung)

[AT126][401][POS] Rel-15 LTE positioning CR (Huawei)

[AT126][402][POS] Slot offset for positioning SRS configuration (Huawei)

[AT126][403][POS] Tx UE configuration

[AT126][404][POS] LS to RAN4 on SL positioning measurements (Huawei)

[AT126][405][POS] SLPP error causes (Lenovo)

[AT126][406][POS] Remaining SLPP issues (Intel)

[AT126][407][POS] Rel-18 SRSp enhancement fields in INMs (Huawei)

[AT126][408][Relay] Relay RRC proposals with ASN.1 impact (Huawei)

[AT126][502][R18MobE] UE cap Early TA acquisition

[AT126][503][R18MobE] Early Measurements (Nokia)

[AT126][504][R18MobE] Conditional Mobility (OPPO)

[AT126][601][XR] Reply LS to SA2 on FS_XRM PH2 (vivo)

[AT126][602][XR] Reply LS to SA2 on application-layer FEC awareness at RAN (Qualcomm)

[AT126][603][XR] LS to SA2 on multi-modality awareness at RAN (Nokia)

[AT126][604][QoE] Remaining QoE issues (Samsung)

[AT126][605][TEI18] MBS TEI18 CRs (Ericsson)

[AT126][606][TEI18] Correction on the configuration of Redcap CFR (Huawei)

[AT126][751][Maint] IoT NTN Kmac correction (Ericsson)

[AT126][752][Maint] Clarification on the SRS Carrier Switching for the PUSCH-less Cell (ZTE)

[AT126][753][Maint] Clarification on xDD differentiation for SDL bands(Huawei)

[AT126][754][Maint] Misclassification of RLF reports as Too Early HO failure (Ericsson)

[AT126][755][Maint] Correction of the range of DL-DataToUL-ACK-v1700 (CMCC)

[AT126][756][Maint] PDCCH order based CFRA and SI request (ZTE)

[AT126][757][Maint] Intra-band EN-DC capabilities for inter-band EN-DC (Google)

[AT126][758][Maint] Clarification on srs-SwitchingAffectedBandsListNR (Huawei)

[AT126][759][Maint] Miscellaneous non-controversial rapporteur corrections (Intel)

[AT126][760][SONMDT] SONMDT corrections for 38.300 (Ericsson)

[AT126][761][SONMDT] MDT report for SNPN (Nokia)

[AT126][762][SONMDT] Stage-2 issues for R18 SONMDT (Huawei)

[AT126][763][eRedCap] 2-step RA for eRedCap (vivo)

[AT126][764][SONMDT] Architecture options for R19 SONMDT (ZTE)

[AT126][765][SONMDT] LTE RRC CR for SONMDT (Huawei)

63 at-meeting offline discussions.

# Annex G:	Post-meeting email discussions

# Guidelines for email discussions:

General guidelines for SHORT email discussions, to be concluded approved endorsed at current meeting.

- Aim to have the final version of the agreed documents provided by the rapporteur at or shortly after the deadline.

- Please provide comments on the first version of the document in good time before the deadline. This allows the rapporteur to make an update addressing all companies' comments and there still be time for a quick round of comments on the update.

- If you have provided comments in the discussion then please indicate to the rapporteur if you are ok with the update provided (preferably via reflector). This avoids the rapporteur having to wait before they can conclude that their update is acceptable to you.

- Rapporteurs, if not already available, please request your tdoc number from the Secretary when you initiate your email discussion and then provide the final version as soon as you are confident that it is agreeable. You do not need to wait for a reminder from chair, session chair or Secretary before sending the final version.

- To avoid any confusion, Secretary, chair, or session chair will send an email to confirm the final status of the document.

For emails discussion to the next meeting (long):

- Rapporteurs, feel free to set an intermediate deadline for companies to provide initial comments, so that the conclusions and proposals can be prepared and distributed before the final deadline.

- Participants, please respect any intermediate deadline indicated by the rapporteur, and preferably provide your feedback as soon as possible.

# Inactive periods and other planning comments

June 6th 	1000 UTC	Deadline short email discussions

Aug 2nd	1000 UTC	Deadline long email discussions

Aug 9th 	1000 UTC	Submission Deadline RAN2#127

Weekends are inactive periods.

It is recommended to not send emails or update files on the server during inactive periods while It is not strictly prohibited. Rapporteurs may kick-off discussions during inactive period. However, no intermediate deadlines and no interactive discussion, no decision making may occur during the inactive period. It shall be possible for a delegate to stay away from reflector and 3GPP server during the inactive period, and still fully participate. Rapporteur announcements during the inactive period, if any, or other updates, can be taken into account after the inactive period.

# Short email discussions, Deadline June 6th, 10:00 UTC (unless otherwise stated)

Please request R2-126 TDoc numbers for the following email discussions from MCC if not already allocated.

NOTE: R18 UE capability CRs have to be endorsed by Friday, May 31st

Approval / endorsement will be declared at or shortly after the deadline.

63 email approvals/discussions scheduled after the RAN2#126 meeting (58 short and 5 long).

[POST126][002][UE Cap] Mega CRs (Intel)

Intended outcome: Agree to Mega UE capability CRs implementing all other WG capabilities and RAN2 endorsed CRs

Deadline:  short

=> Agreed in:

R2-2406069 (38.306)

R2-2406070 (38.331)

R2-2406071 (38.306)

R2-2406072 (38.331)

[POST126][003][R17 MBS-NTN] Agree to CRs (LG)

Intended outcome: Agree to CRs implementing agreements from this meeting

Deadline:  short

=> Agreed in:

R2-2406077 (38.321 Rel-17)

R2-2406078 (38.321 Rel-18)

R2-2406079 (38.306 Rel-17)

R2-2406080 (38.306 Rel-18)

R2-2406081 (38.331 Rel-17)

R2-2406082 (38.331 Rel-18)

=> Noted in R2-2406083

[POST126][004][ASN.1 common] 38.331 CR (Ericsson)

Intended outcome: Agree to CR capturing ASN.1 common issues – revision of R2-2404791

Deadline:  short

=> Agreed in R2-2406074

[POST126][006][UAV] CR to 38.331 and 36.331 (Qualcomm)

Intended outcome: Agree to CRs and RIL list

Deadline:  short

=> Agreed in R2-2406043 and R2-2406044

=> Noted in R2-2406045 and R2-2406046

[POST126][009][NES] RRC CR (Huawei)

Intended outcome: Agree to RRC CR with any additional changes needed and updated RIL List

Deadline:  Short

=> Agreed in R2-2405618

=> Noted in R2-2406088

[POST126][010][NES] UE capabilities (Vivo)

Intended outcom: endorse CRs

Deadline:  Friday, May 31st

=> Endorsed in:

R2-2406039 (38.306)

R2-2406040 (38.331)

[POST126][017][XR] CR to 38.321  (Qualcomm)

Intended outcome: Agree to CR

Deadline:  Short

=> Agreed in R2-2406092

[POST126][031][AIMob] Simulations (Oppo)

Intended outcome: Agree to evaluation documentation and small simulation related FFS (needed to start simulation evaluation for August meeting)

Endorse Skeleton TR

Deadline:  short

=> Endorsed in R2-2406096 (TR skeleton)

=> Noted in R2-2406095 (Summary)

[POST126][033][SDT] LS reply to RAN3 (ZTE)

Intended outcome: agree to LS by email

Deadline:  short

=> The LS is postponed

[POST126][036][RACHless] RRC CR  (Ericsson)

Intended outcome: Agree to RRC CR

Deadline:  short

=> Agreed in R2-2406087

[POST126][037][RACHless] MAC CR (Huawei)

Intended outcome: agree to CR

Deadline:  short

=> Agreed in R2-2406097

[Post126][038][RACHless] UE capability (Samsung)

Intended outcome: Endorse CRs

Deadline:  Friday, May 31st

=> Endorsed and merged to the mega CR in:

R2-2406041 (38.306)

R2-2406042 (38.331)

[POST126][101][V2X/SL] (OPPO)

Scope: To capture all agreements into 38.331.

Intended outcome: Approve RRC CR in R2-2405899.

Deadline: Short email discussion.

=> Agreed in R2-2405899

[POST126][102][V2X/SL] (LG)

Scope: To capture all agreements into 38.321.

Intended outcome: Approve MAC CR in R2-2405900.

Deadline: Short email discussion.

=> Agreed in R2-2405900

[POST126][103][V2X/SL] (IDC)

Scope: To capture all agreements into 38.300.

Intended outcome: Stage 2 CR in R2-2405901.

Deadline: Short email discussion.

=> Agreed in R2-2405901

[Post126][203][MUSIM] CR for TS 38.331 (vivo)

Scope: Update and review the CR for TS 38.331, update the RIL list based on agreements in this meeting.

Intended outcome: Agreed CR and RIL list

Deadline: Short

=> Agreed in R2-2406076 (38.331) and R2-2406075 (RIL list)

[Post126][204][MIMOevo] CR for TS 38.321 (Samsung)

Scope: Update and review the CR for TS 38.321.

Intended outcome: Agreed CR

Deadline:  Short

=> Agreed in R2-2406073

[Post126][205][MIMOevo] CR for TS 38.331 (Ericsson)

Scope: Update and review the CR for TS 38.331, update the RIL list based on agreements in this meeting.

Intended outcome: Agreed CR and RIL list

Deadline:  Short

=> Agreed in R2-2406031 (38.331) and R2-2406032 (RIL List)

[Post126][301][NR-NTN Enh] 38.331 CR (Ericsson)

Scope: update the RRC CR with meeting agreements

Intended outcome: Agreed CR

Deadline for agreed CR (in R2-2405753): short

=> Agreed in R2-2405753

[Post126][302][NR-NTN Enh] 38.321 CR (Interdigital)

Scope: update the MAC CR with meeting agreements

Intended outcome: Agreed CR

Deadline for agreed CR (in R2-2406103): short

=> Agreed in R2-2406103

[Post126][303][NR-NTN Enh] 37.355 CR (CATT)

Scope: update the 37.355 CR with meeting agreements

Intended outcome: Agreed CR

Deadline for agreed CR (in R2-2405768): short

=> Agreed on R2-2405768

[Post126][304][NR-NTN Enh] UE Caps CRs (Intel)

Scope: update CRs with meeting agreements

Intended outcome: Endorsed CRs

Deadline for CR: very short

=> Endorsed and to be merged to the mega CR in:

R2-2406037 (38.306)

R2-2406038 (38.331)

[Post126][305][IoT-NTN Enh] 36.331 CR (Huawei)

Scope: update the RRC CR with meeting agreements

Intended outcome: Agreed CR

Deadline for agreed CR (in R2-2405758): short

=> Agreed in R2-2405758

[Post126][306][IoT-NTN Enh] 36.321 CR (Mediatek)

Scope: update the MAC CR with meeting agreements

Intended outcome: Agreed CR

Deadline for agreed CR (in R2-2405759): short

=> Agreed in R2-2405759

[Post126][307][IoT-NTN Enh] 36.306 CR (Qualcomm)

Scope: update the 36.306 CR with meeting agreements

Intended outcome: Agreed CR

Deadline for agreed CR (in R2-2406097): short

=> Agreed in R2-2406094

[Post126][401][Relay] Rel-18 relay RRC CR (Huawei)

Scope: Update the CR in R2-2405367 in line with decisions of this meeting.

Intended outcome: Agreed CR in R2-2405877

Deadline:  Short (for RP)

=> Agreed in R2-2405877

=> Noted in R2-2406085 (RIL list)

[Post126][402][Relay] Rel-18 relay capability CRs (Samsung)

Scope: Update the CRs in R2-2405532 and R2-2405533 in line with decisions of this meeting.

Intended outcome: Endorsed CRs in R2-2405878 (38.331) and R2-2405879 (38.306)

Deadline:  Friday, May 31st (for merge into mega CRs)

=> Endorsed and to be merged to the mega CR in:

R2-2405878 (38.331)

R2-2405879 (38.306)

[Post126][403][Relay] Rel-18 relay stage 2 CR (LG)

Scope: Develop and check a relay CR to 38.300 in line with decisions of this meeting, taking the endorsed CR from RAN2#125bis in R2-2403974 as baseline.

Intended outcome: Agreed CR in R2-2405880

Deadline:  Short (for RP)

=> Agreed in R2-2406104

[Post126][404][Relay] Rel-18 relay SRAP CR (OPPO)

Scope: Update the CR in R2-2404247 in line with decisions of this meeting.

Intended outcome: Agreed CR in R2-2405881

Deadline:  Short (for RP)

=> Agreed in R2-2405881

[Post126][405][Relay] Rel-18 relay PDCP CR (InterDigital)

Scope: Update the CR in R2-2405856 in line with decisions of this meeting.

Intended outcome: Agreed CR in R2-2405882

Deadline:  Short (for RP)

=> Agreed in R2-2405882.

[Post126][406][POS] Rel-18 positioning LPP CR (CATT)

Scope: Update the CR in R2-2404434 in line with decisions of this meeting.  Late-arriving parameter updates from RAN1 can be taken into account if possible.

Intended outcome: Agreed CR in R2-2405883

Deadline:  Short (for RP)

=> Agreed in R2-2405883

=> Noted in R2-2406100

[Post126][407][POS] Rel-18 positioning RRC CR (Ericsson)

Scope: Update the CR in R2-2405257 in line with decisions of this meeting, including implementation of constraints on configuration of SL-PRS carrier in SIB23/preconfiguration.  Late-arriving parameter updates from RAN1 can be taken into account if possible.

Intended outcome: Agreed CR in R2-2405884

Deadline:  Short (for RP)

=> Agreed in R2-2405884

=> Noted in R2-2406033

[Post126][408][POS] Rel-18 positioning stage 2 (305) CR (Qualcomm)

Scope: Update the CR in R2-2405247 in line with decisions of this meeting.

Intended outcome: Agreed CR in R2-2405885

Deadline:  Short (for RP)

=> Agreed in R2-2405885

[Post126][409][POS] Rel-18 positioning stage 2 (300) CR (vivo)

Scope: Develop a positioning CR to 38.300 in line with decisions of this meeting.

Intended outcome: Agreed CR in R2-2405886

Deadline:  Short (for RP)

=> Agreed in R2-2405886

[Post126][410][POS] Rel-18 positioning SLPP CR (Intel)

Scope: Update the CR in R2-2404191 in line with decisions of this meeting.  Late-arriving parameter updates from RAN1 can be taken into account if possible.

Intended outcome: Agreed CR in R2-2405887

Deadline:  Short (for RP)

=> Agreed in R2-2405887

[Post126][411][POS] Rel-18 positioning MAC CR (Huawei)

Scope: Update the CR in R2-2404762 in line with decisions of this meeting.

Intended outcome: Agreed CR in R2-2405888

Deadline:  Short (for RP)

=> Agreed in R2-2405888

=> Noted in R2-2406098

[Post126][412][POS] Rel-18 positioning RRC capability CRs (Xiaomi)

Scope: Update the CRs in R2-2404623 and R2-2404624 in line with decisions of this meeting.  Late-arriving parameter updates from RAN1 can be taken into account if possible.

Intended outcome: Endorsed CRs in R2-2405889 (38.331) and R2-2405890 (38.306)

Deadline:  Friday, May 31st (for merge into mega CRs)

=> Endorsed and merged to the mega CR in:

R2-2405889 (38.331)

R2-2405890 (38.306)

[Post126][413][POS] Rel-18 positioning LPP capability CR (Xiaomi)

Scope: Update the CR in R2-2404595 in line with decisions of this meeting.  Late-arriving parameter updates from RAN1 can be taken into account if possible.

Intended outcome: Agreed CR in R2-2405981

Deadline:  Short (for RP)

=> Agreed in R2-2405981

[Post126][414][POS] Rel-18 positioning SLPP capability CR (Xiaomi)

Scope: Update the CR in R2-2404760 in line with decisions of this meeting.  Late-arriving parameter updates from RAN1 can be taken into account if possible.

Intended outcome: Agreed CR in R2-2405982

Deadline:  Short (for RP)

=> Agreed in R2-2405982

[Post126][415][POS] LS to SA2 on relative velocity (Nokia)

Scope: Draft an LS to SA2 in line with the conclusion of RIL Rapp024, indicating what RAN2 have agreed on relative velocity and inviting SA2 to comment.

Intended outcome: Approved LS in R2-2405987

Deadline:  Short (not for RP)

=> Approved in R2-2405987

[Post126][416][POS] LS to CT1/CT4 on maximum devices supported in SLPP (Huawei)

Scope: Draft an LS to CT1/CT4, Cc: SA2, indicating our requirement in SLPP to support up to 256 devices and requesting them to take it into account.

Intended outcome: Approved LS in R2-2405988

Deadline:  Short (not for RP)

=> Approved in R2-2405988

* [Post126][417][POS] Update of mirror CR in R2-2405863 (Huawei)

Scope: Update the category A CR R2-2405863 to correct coversheet formalities and bring the changes in line with the agreed Rel-17 category F CR in R2-2405862.

Intended outcome: Agreed CR

Deadline: Short (for RP)

=> Agreed in R2-2406099

[Post126][510][R18MobE] 37340 CR (ZTE)

Scope: Based on endorsed CR in R2-2405058, add agreeable impact of / capture of agreements from current meeting.

Intended outcome: Agreed CR

Deadline: Short

=> Agreed in R2-2406102

[Post126][511][R18MobE] PDCP SN gap at LTM fast recovery (MediaTek)

Scope: Converge on the note and where to put it. If it is agreed to have the note in 38300, revise the agreed 38300 CR, if it is agreed to have the note in 38331 then add to RRC discussion, if other TS without ongoing CR then make new CR.

Intended outcome: Agreed CR (if applicable)

Deadline: Short

=> Agreed in R2-2406084

[Post126][512][R18MobE] 38331 (Ericsson)

Scope: Reflect agreements and progress in the CR, resolve remaining points as discussed at the meeting.

Intended outcome: Agreed CR

Deadline: Short

=> Agreed in:

R2-2406086 (38.331)

R2-2406089 (38.322)

[Post126][513][R18MobE] UE capabilities (Intel)

Scope: Reflect agreements and progress in the CRs, resolve points as discussed at the meeting.

Intended outcome: endorsed CRs 38306 38331 (for merge)

Deadline: Friday, May 31st (for merge)

=> Endorsed and merged to the mega CR in:

R2-2406035 (38.306)

R2-2406036 (38.331)

[Post126][515][R18MobE] 38321 (Huawei)

Scope: Reflect agreements and progress in the CR, resolve remaining points as discussed at the meeting.

Intended outcome: Agreed CR

Deadline: Short

=> Agreed in R2-2406030 (38.321 CR#1817r3)

[POST126][605][TEI18] MBS TEI18 CRs (Ericsson)

Scope: Revise R2-2404993, R2-2404994 and R2-2405558 according to the agreements

Intended outcome: Agree to CRs including UE capability (to be submitted separately)

Deadline: Two weeks

=> Agreed in:

R2-2406047 (38.306)

R2-2406027 (38.300)

R2-2406034 (38.304)

[POST126][607][eMBS] RRC CR (Huawei)

Scope: Update RRC CR according to the agreements from the meeting.

Intended outcome: Agreed CR

Deadline:  short

=> Agreed in R2-2406025 (CR#4688r2)

=> Available in R2-2406026 (RIL list for MBS)

[POST126][608][QoE] RRC CR (Ericsson)

Scope: Update RRC CR according to the agreements from the meeting.

Intended outcome: Agreed CR

Deadline:  short

=> Agreed in R2-2406091

[Post126][751][Maint] Miscellaneous Corrections for TS 36.331 (Samsung)

Scope:

    - Review 36.331 rapporteurs misc CRs.

Intended outcome:

    - Agreed CRs in R2-2405803, R2-2405804, R2-2405805, R2-2405806, R2-2405807 (Samsung)

Deadline:

    - Short

=> Agreed in:

R2-2406106 (Rel-14)

R2-2406107 (Rel-15)

R2-2406108 (Rel-16)

R2-2406109 (Rel-17)

R2-2406110 (Rel-18)

[Post126][752][Maint] Miscellaneous non-controversial corrections for NR RRC (Ericsson)

Scope:

    - Produce RRC rapporteurs misc CRs

Intended outcome:

    - Agreed CRs in R2-2405847, R2-2405848, R2-2405849, R2-2405850 (Ericsson)

Deadline:

    - Short

=> Agreed in R2-2404784, R2-2404785, R2-2405849, R2-2405850

[Post126][753][SONMDT] NR RRC CR for SONMDT (Ericsson)

Scope:

    - Produce agreeable CR

Intended outcome:

    - Agreed CR in R2-2405968 (Ericsson)

Deadline:

    - Short

=> Agreed in R2-2405968

[Post126][754][Maint] Intra-band EN-DC capabilities for inter-band EN-DC (Google)

Scope:

    - Produce agreeable CRs

Intended outcome:

    - Agreed CRs in R2-2405975, R2-2405976, R2-2405977, and R2-2405978 (Google)

Deadline:

    - Short

=> Agreed in:

R2-2405975 (38.331 Rel-17)

R2-2405976 (38.306 Rel-18)

R2-2405977 (38.331 Rel-17)

R2-2405978 (38.306 Rel-18)

=> Approved in R2-2406111 (LS out)

[Post126][755][eRedCap] Miscellaneous corrections for eRedCap (Ericsson)

Scope:

    - Produce agreeable CRs

Intended outcome:

    - Agreed CR in R2-2405979 (Ericsson)

Deadline:

    - Short

=> Agreed in R2-2405979

[POST126][801][CE_enh]  updated RRC CR and RIL list (Huawei)

Scope:

Update the RRC CR with the latest agreements and review

Updated RIL list according the agreements from the meeting

Expected output:

Final RRC CR for plenary

Updated RIL list

Deadline: Short (for plenary)

=> Agreed in R2-2406024: 38.331 CR#4833r1

=> Noted in R2-2406023: Updated RIL List for CE

[POST126][802][CE_enh]  updated MAC CR (ZTE)

Scope: Update the MAC CR with the latest agreements and review

Expected output: Final MAC CR for plenary

Deadline: Short (for plenary)

=> Agreed in R2-2406093

[Post126][999] Review of RAN3 endorsed CRs (MCC)

Scope: Review RAN3 endorsed stage 2 CR (21 of them).

Expected output: Agreed CRs for plenary

Deadline: Short (for plenary)

=> See clause 2.5 for agreed CRs

# Long email discussions, Deadline August 2nd, 10:00 UTC (unless otherwise stated)

Please request R2-127 TDoc numbers for the following email discussions by 3GU according to normal tdoc submission procedure.

[POST126][005][Cricket Challenge] Reply LS to R2-2406204

Intended outcome:

- Agree to reply LS R2-2406204 on RAN4 vs RAN2 Cricket Match

- Agree to team composition and RAN2 cheering audience

Deadline for inputs: (Long - July 26th, 10:00 UTC)

=> Approved in R2-2406149

[POST126][021][AIoT] TP (Huawei)

Intended outcome: Capture agreed solutions/options from this meeting

Deadline: long

[POST126][032][AI/ML PHY] LCM (Intel/Samsung)

Intended outcome:

Phase 1: Agreable definitions (Samsung)

Phase 2: Reach common understanding of reactive/proactive framework for applicable functionality.

Deadline: long

[POST126][034][AIML PHY] TP for data collection (Ericsson)

Intended outcome: Prepare and review TP capturing table in minutes and agreements on data collection

Deadline: long

[POST126][035][AIoT] LS to SA2/SA3 (Oppo)

Intended outcome: Agree to LS to SA2/SA3 ccRAN3 capturing some relevant agreements and RAN2 assumptions.

Deadline: June 28th, 2024

=> Approved in R2-2406150

[Post126][514][R18MobE] UE capabilities Open Issues (intel)

Scope: progress open issues

Intended outcome: Report, TP/draftCRs if applicable

Deadline: Long

# Annex H:	History

| Document history |  |  |
| --- | --- | --- |
| Date | TSG RAN WG2 Tdoc | Subject |
| 18.06.2024 |  | First draft version |
| 17.07.2024 |  | Second draft version |
| 09.08.2024 | R2-2406202 | Submitted for approval in RAN2#127 |
| Author: Dr. Juha Korhonen ETSI Mobile Competence Centre (MCC) Tel. +33 (0)4 92 94 42 00 email: Juha.Korhonen@etsi.org |  |  |