3GPP TSG RAN WG2#127	R2-2406231

Maastricht, Netherlands, 19th - 23rd August 2024

3GPP TSG-RAN WG4 Meeting #111	R4-2410693

Fukuoka, Japan, May 20 – 24, 2024

Title:	LS on intra-band EN-DC channel spacing

Response to:

Release:	Rel-18

Work Item:	TEI18

Source:	RAN4

To:	RAN2

Cc:

Contact Person:

Name: 	Hu Dan

E-mail Address: hudan11@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

In Rel-15, RAN4 sent the LS R4-1813862 to RAN2 for intra-band MR-DC band combination signaling, with the assumptions that if a UE supports intra-band non-contiguous EN-DC, it also supports contiguous in the same band.

And then the capability of intraBandENDC-Support was introduced in TS 38.306 as follows, with three states, including: contiguous (not include), non-contiguous and ‘both’. However, from RAN4’s perspective, UE implementing ‘non-contiguous’ needs more stringent capability. In this case, if a UE supports ‘non-contiguous’, it should not preclude the support of ‘contiguous’ in the same band. The capability UE reports should reflect the implementation UE can support, rather than just for configurations.

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| intraBandENDC-Support Indicates whether the UE supports intra-band (NG)EN-DC with only non-contiguous spectrum, or with both contiguous and non-contiguous spectrum for the (NG)EN-DC combination as specified in TS 38.101-3 [4]. If the UE does not include this field for an intra-band (NG)EN-DC combination the UE only supports the contiguous spectrum for the intra-band (NG)EN-DC combination. If intrabandENDC-Support-UL is absent and the band combination supports intra-band (NG)EN-DC only in DL, this field indicates the DL capability. If intrabandENDC-Support-UL is absent and the band combination supports intra-band (NG)EN-DC in DL and UL, this field indicates the common capability for both DL and UL. If intrabandENDC-Support-UL is included, intraBandENDC-Support indicates the DL capability. | BC | No | N/A | N/A |

With the UE capability, it is noticed that in the field, some UEs indicating ‘non-contiguous’ via intraBandENDC-Support cannot access to the NW configuring intra-band contiguous EN-DC. The UE can only access to single carrier rather than intra-band EN-DC, which degrades the UE performance.

Considering the capability of intraBandENDC-Support has been implemented for several releases, to make up for the problem, RAN4 agrees to introduce a new capability to support intra-band non-contiguous with nominal channel spacing. In RAN4 spec, nominal channel spacing between E-UTRA carrier and an adjacent NR carrier is defined for intra-band contiguous EN-DC in TS 38.101-3.

2. Actions:

To RAN2:

ACTION: RAN4 respectfully asks RAN2 to introduce the capability to support intra-band non-contiguous EN-DC with nominal channel spacing.

3. Date of Next TSG WG RAN4 Meetings:

TSG-RAN4 Meeting#112 	19 – 23 August 2024	Maastricht, Netherlands

TSG-RAN4 Meeting#112bis	14 - 18 October 2024	China