3GPP TSG-RAN WG2 Meeting #127	R2-2406246

Maastricht, Netherlands, August 19th– 23rd, 2024

Agenda Item:	8.8.2

Source: 	vivo

Title:  	Further Considerations on DL Coverage Enhancements

Document for:	Discussion and Decision

- Introduction

In the updated NTN WID [1], studying whether and how to extend the SSB periodicity is added within the scope of the NTN DL coverage enhancement.

- Study and specify if beneficial downlink coverage enhancements targeting support for additional reference satellite payload parameters covering both GSO and NGSO constellations operating in FR1-NTN or FR2-NTN [RAN1, RAN2, RAN4] - Define additional reference satellite payload parameters assuming power sharing among satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint, such that satellite beams may not all be simultaneously active or may be active below the nominal EIRP density per satellite beam (see section 6.1.1 in TR 38.821) due to limited power and limited feeder link bandwidth. - Define the corresponding power sharing assumptions and necessary link level and system level evaluation methodology and relevant KPIs for evaluations of the coverage, to allow for identification of physical channels/signals and system-level aspects that need enhancements and the corresponding needed improvements. - Study and if needed specify solutions, including link level enhancements for FR1-NTN (e.g. for PDCCH, PDSCH) and/or system level enhancements for FR1-NTN and/or FR2-NTN, allowing dynamic and flexible power sharing between satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint. - RAN1 to report at the latest by RAN#106 with the list of targeted physical channels/signals for link level enhancements (if any), and with the targeted system-level enhancements (if any) - RAN1 should report on impact to backward compatibility, if any, for potential extension of the SSB periodicity at the latest by RAN#106, in conjunction with the targeted system-level enhancements. - Notes for this objective: - SSB channel enhancement other than SSB periodicity extension is not considered - RAN1 should consider issues such as UE’s cell search complexity and impact to initial cell selection, latency and success rate, for the above extension - The SSB periodicity enhancements potentially defined in this WID only apply to NTN operation - Antenna gain of UE shall be assumed to be -5.5dBi in case of smartphone in FR1-NTN, the UE is assumed to be a full duplex UE, and at least 2Rx are considered at the UE - NGSO to be considered in priority: LEO Set-1 @ 600 km - Rel-18 network energy saving techniques should be considered as baseline in the system level study

Therefore, in this contribution, we would like to first discuss our consideration on the extension of the SSB periodicity from the RAN2 spec point of view. And also share our consideration on the support of cell/beam level DTX mechanism.in NR-NTN network with dynamic power sharing.

- Discussion

  - 2.1	Potential RAN2 impact due to SSB periodicity extension

To support the dynamic power sharing in the NR-NTN network with beam hopping, the SSB periodicity may be extended. As a result, there might be potential RAN2 spec impacts, as described in the following part.

According to the TS 38.213 [2], UE in IDLE/INACTIVE state will consider the half frames with SSB occur with a periodicity of 20ms, and it is up to NW to ensure the periodicity of the SSB burst for the cell supporting broadcasting is not larger than 20ms. Besides, during initial cell selection, UE does not have any cell information. If we extend the SSB periodicity, the legacy UE may not successfully camp on the cell, e.g., among initial cell selection. Specifically, the legacy UE blindly searches SSB with 20ms search duration in the time domain based on sync raster in the frequency band. However, within the search duration, there may be not any SSB bursts transmitted if the cell transmits the SSB in a periodicity larger than 20ms.

- 4.1 Cell search Cell search is the procedure for a UE to acquire time and frequency synchronization with a cell and to detect the physical layer Cell ID of the cell. ……Skip unrelated part…… For initial cell selection, a UE may assume that half frames with SS/PBCH blocks occur with a periodicity of 2 frames.

Further, even R19 UE can search the SSB with a larger SSB periodicity assumption, with doubt, more UE power consumption and larger latency are needed to find the initial cell since UE has to search the SSB in 160ms search duration in each possible frequency location in the frequency band.

Observation 1: Extending the SSB periodicity may be non-backward compatible for legacy UE, and it leads to more UE power consumption and initial cell search latency.

Besides, extending the SSB periodicity may also have impacts on the RLM/BFD. For example, if the radio link condition becomes poor during the SSB transmission interval, the UE cannot detect the poor radio condition since there is no available SSB that can be measured. Consequently, the RLM/BFD will also not be triggered in time. And UE may still transmit data in this case, which may cause unnecessary data transmission or radio resource consumption since it is very probable the transmission will not succeed.

Observation 2: Extending the SSB periodicity may have impacts on RLM/BFD detection, since the UE may not determine the RLF/beam failure in time.

Regarding the RRM measurement, whether the extended SSB periodicity has impacts on the measurement accuracy requirements is up to RAN4. However, according to TS38.331 [3], the SMTC configuration and the SSB periodicity configuration for RRC_CONNECTED UE is not enough if RAN1 decides to support SSB periodicity larger than 160ms, since the maximum value range is 160ms now.

-- ASN1START -- TAG-SSB-MTC-START SSB-MTC ::= SEQUENCE { periodicityAndOffset CHOICE { sf5 INTEGER (0..4), sf10 INTEGER (0..9), sf20 INTEGER (0..19), sf40 INTEGER (0..39), sf80 INTEGER (0..79), sf160 INTEGER (0..159) }, duration ENUMERATED { sf1, sf2, sf3, sf4, sf5 } }

ssb-periodicityServingCell ENUMERATED { ms5, ms10, ms20, ms40, ms80, ms160, spare2, spare1 }

Similarly, the issue also exists for the periodicity of the SIB1, since the currently specified value is 160ms.

- the SIB1 is transmitted on the DL-SCH with a periodicity of 160 ms and variable transmission repetition periodicity within 160 ms as specified in TS 38.213 [13], clause 13. The default transmission repetition periodicity of SIB1 is 20 ms but the actual transmission repetition periodicity is up to network implementation. For SSB and CORESET multiplexing pattern 1, SIB1 repetition transmission period is 20 ms. For SSB and CORESET multiplexing pattern 2/3, SIB1 transmission repetition period is the same as the SSB period (TS 38.213 [13], clause 13). SIB1 includes information regarding the availability and scheduling (e.g. mapping of SIBs to SI message, periodicity, SI-window size) of other SIBs with an indication whether one or more SIBs are only provided on-demand and, in that case, the configuration needed by the UE to perform the SI request. SIB1 is cell-specific SIB;

Observation 3: The current value range of the SMTC periodicity and offset, the service cell SSB periodicity, and the periodicity of the SIB1 requires new signalling if the extended SSB periodicity is larger than 160ms.

Although some potentiaal RAN2 impacts can be identified for SSB periodicity extenstion, we think whether and how to extend the SSB periodicity in R19 is totally up to RAN1. RAN2 should postpone further studying potential impacts and just wait for final RAN1 decision.

Proposal 1: Whether and how to extend the SSB periodicity is up to the RAN1 decision.

Proposal 2: RAN2 postpones the discussion of potential RAN2 impact on SSB periodicity extension.

  - 2.2	Cell/beam level DTX

In the last meeting, we mainly discussed the RAN1-related issues and checked with them via LS. But there are still some RAN2 centric issues that may not depend on RAN1’s response LS, i.e., how to provide the cell level/beam level DTX/DRX configuration.

In R18 NES, the cell level DTX/DRX configuration is configured via dedicated signalling, that is because even for the DTX/DRX off state, the common procedure like RACH/paging/SI is not affected. But for the cell level/beam level DTX in NTN, there may be some N1 beams where neither SSB/paging/SI nor dedicated transfer/receiving is available as defined by RAN1 [4]. Since this may influence the IDLE/INACTIVE UEs in the NTN cell, we think the DTX information should be provided through system information. Considering that a cell consists of multiple beams or single beam, the beam level DTX information can be applied to all use cases, which is preferable to us. For example, if a cell just include one beam, the beam-level DTX configuration can be used for indicate the cell DTX inforamtion.

Such DTX configuration is useful for IDLE/INACTIVE UEs that need to monitor paging in case UE is camped on the cell in the corresponding monitoring occasion in each PF/PO. For example, in case the UE is located in the beam for which it is in N1 state or switched to N1 state, UE need to keep monitoring in each monitoring occasion in each PF/PO even there is not any paging will be send by NW, but this may cause UE power consumption. To our consideration, such issue can be avoided with enhanced UE handling for this case, i.e., UE skip to monitor the paging occasions if UE determines these paging occasions is in the N1 state beam duration.

Proposal 3: The NW provides the beam level DTX information through SIB.

- Conclusions

This contribution discussed the DL coverage enhancements in Rel-19 NR NTN. Observations and Proposals are summarized as follows,

Observation 1: Extending the SSB periodicity may be non-backward compatible for legacy UE, and it leads to more UE power consumption and initial cell search latency.

Observation 2: Extending the SSB periodicity may have impacts on RLM/BFD detection, since the UE may not determine the RLF/beam failure in time.

Observation 3: The current value range of the SMTC periodicity and offset, the service cell SSB periodicity, and the periodicity of the SIB1 requires new signalling if the extended SSB periodicity is larger than 160ms.

Proposal 1: Whether and how to extend the SSB periodicity is up to the RAN1 decision.

Proposal 2: RAN2 postpones the discussion of potential RAN2 impact on SSB periodicity extension.

Proposal 3: The NW provides the beam level DTX information through SIB.

- Reference

- RP-241667, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3, Thales, CATT.

- TS38.213, NR; Physical layer procedures for control.

- TS38.331, NR; Radio Resource Control (RRC); Protocol specification.

- Final_Minutes_report_RAN1#116_v100.