3GPP TSG-RAN WG2 Meeting #127                                                                   R2-2406247

Maastricht, Netherlands, August 19th – 23rd, 2024

Agenda item:	8.8.4

Source:	vivo

Title:	Further Discussion on MBS Broadcast Provision in NTN

Document for:	Discussion and Decision

1 Introduction

In past RAN2 meetings, there was a lot of progress on MBS broadcast provision within the intended service area. The achieved agreements are listed below for reference [1][2]:

RAN2#125bis Agreements:

- For MBS broadcast service we don’t restrict the work to any satellite constellation type

- We prioritize working on a solution for MBS broadcast but we don’t preclude other broadcast services, namely ETWS

- We will cover at least the case where the indicated intended service area covers a portion of a NTN cell

- The intended service area can cover the area of more than one NTN cells (or portions thereof)

- Can discuss next time whether the broadcast transmission can be limited to the intended service area only (i.e. no transmission happens outside of the intended serive area)

- At least the following geographical area formats to model service area can be further considered (the signalling of other information than the geographical information can be considered):

- Circles (like for TN coverage)

- Geographical area information, e.g. via polygons, to better approximate the intended shape of service area

RAN2#126 Agreements:

1.	For MBS broadcast service, both EFC and EMC are supported.

2.	RAN2 will not define means for the NW to prevent the reception of the content of the service outside of the intended service area.

3.	MBS broadcast intended service area is provided via system information

4.	For MBS broadcast RAN2 considers the following possibilities for including the service area information: SIB20/ SIB21/ MBSBroadcastConfiguration. FFS for ETWS.

5.	When intended service area (e.g., geographical area/TN coverage) is provided for MBS broadcast service, it needs to be associated with MBS session (FFS on the details).

6. 	Come back next meeting to check whether a UE may not (or should not) establish MRB(s) associated with a service limited to an intended service area when it is not located within the area (up to UE implementation how the UE detects it’s not in the intended service area).

7. 	Come back next meeting to check whether a UE may (or should) release its established MRB(s) associated with a service limited to an intended service area when exiting the area (up to UE implementation how the UE detects it’s not in the intended service area).

In this contribution, we would like to further discuss the detailed solutions to MBS broadcast provision in NTN from network signalling and UE behavior perspectives.

2 Discussion

  - 2.1	Network signalling

Based on the achieved agreements, we assume the following cases depicted in Figure 1 can be considered for MBS broadcast provision within an intended service area, where the intended service area can be partially or fully overlapped with at least one NTN cell area.

Figure 1: MBS broadcast provision within the intended service area

Then the first question that comes to us is whether a cell should signal the whole intended service area or just signal the portion of it that is within the cell area. In our understanding, considering the sake of service continuity (i.e. the UE may consider the frequency providing the MBS broadcast service the UE interested in to be the highest priority during the MBS broadcast session), a cell should signal the whole intended service area of a broadcast session. With that, the UE can determine whether it can keep receiving the ongoing broadcast session within the intended service area on the reselected cell based on broadcast frequency prioritization. For example, in Case 1 mentioned above, Cell#1 should indicate the both left (i.e. within Cell#2) and right (i.e. within Cell#2) elliptical areas in green). Then the UE can determine if reselected to Cell#2, it cannot receive the broadcast service within the intended area. As a result, it may not prioritize the frequency associated with service#1. For example, in Case 2 mentioned above, Cell#1 should indicate the whole intended service area in green not just the cell area of Cell#1. Then the UE can determine if reselected to Cell#2, it can continue to receive the broadcast service within the intended area. As a result, it may prioritize the frequency associated with service#1. Therefore, we have the following proposal,

Proposal 1: For a cell, the network signals the whole intended service area of a broadcast session.

Next, we would like to discuss how to model the intended service area. In Rel-18 NR NTN, SIB25 is introduced to contain TN coverage information (i.e. geographical cycle with reference location and radius) for neighbour cell measurements. Given that the motivation for TN coverage indication is quite similar to that for the intended service area indication, it seems a spontaneous logic to reuse the Rel-18 framework. Some may argue that the SIB25 cycle modeling may be not efficient or precise for specific service area fitting. However, it is not RAN2’s responsibility to conclude the accuracy requirement for the service area description. It is hard to make any decision if RAN2 keeps on with the accuracy requirement discussion. So, for simplicity and progress, we think the TN coverage configuration structure (i.e. area information list) can be reused to signal the intended service area.

Proposal 2: Circles format (like for TN coverage) is reused to model the intended service area.

Furthermore, regarding which message (i.e. SIB20, SIB21, or broadcast MCCH) to carry the service area information, we understand that broadcast MCCH is the best choice. For SIB20, it is mainly introduced for MCCH scheduling. If service area information is included in SIB20, it seems a bit weird as service area information is more like session-level configuration but not MCCH scheduling. Meanwhile, the change of service area may also lead to system information change notifications, resulting in unnecessary SIB reacquisition regarding the legacy UEs. Some may think it should be SIB20’s duty. The benefit may be that if the UE is not in any service area, then it can not acquire the MCCH. However, we don’t that is the case. This is because the NW may provide broadcast sessions associated with a service area and some other broadcast sessions that are not associated with the service area.UE anyway has to check the MCCH to figure out whether the service that the user is interested in is provided or not. As for SIB21, it is used to inform the MBS FSAI of the intra/inter frequencies for service continuity. Providing service area information for the broadcast service not provided in the current serving cell may offer additional benefits for service continuity. So we can use the existing broadcast MCCH message and SIB 21 to carry the service area information for broadcast reception and service continuity, respectively. Note that a new MCCH message is not needed since RAN2 agrees not to define means to prevent the reception of the content of the service outside of the intended service area.

With this, both broadcast session PTM configuration and service area information (e.g. a list of areas where each area is defined using center location coordinates and radius, similar to TN coverage) are included in the broadcast MCCH message. Then we can use the area ID(s) to associate the broadcast session and the service area.

Proposal 3: Existing broadcast MCCH is used to include a list of service areas, where each area is associated with a service area ID.

Proposal 4: The intended serviced area of a broadcast session is indicated by adding the service area ID list in the broadcast MCCH.

Proposal 5: The network can include intended serviced area information in SIB21 for broadcast service continuity.

  - 2.2	UE behavior

With the network siganlling as mentioned above, UE is able to interpret the intended service area for the MBS broadcast session. When the UE is located within the service area of a broadcast session that the user is interested in, broadcast PTM configuration can be applied for broadcast reception. On the opposite, when the UE is outside the intended service area, it should release the MRB and stop the corresponding DL-SCH reception. That is because, from the UE point of view, the network may not schedule broadcast MCCH/MTCH outside that area (i.e. the satellite beam with broadcast MCCH/MTCH scheduling is not available in the intended service area, as illustrated in Figure 2). For power saving and also to obey with intention of the service area, the UE should stop broadcast reception.

Figure 2: satellite beam with MBS broadcast scheduling

Besides, MRB handling, service continuity is also a key part of NR MBS broadcast feature. For the frequency prioritization, as analyzed in section 2.1, we think the UE may set one frequency associated with broadcast service provision with the highest frequency priority only when it will be in the intended service area after the cell reselection. Specifically, if UE is receiving or interested in receiving an MBS broadcast service associated with the intended service area and can only receive this MBS broadcast service by camping on a frequency on which it is provided, the UE may consider that frequency to be the highest priority during the MBS broadcast session when its location is within the intended service area of the corresponding session after cell reselection.

Proposal 6: A UE may establish MRB(s) associated with a service limited to an intended service area only when it is located within that service area.

Proposal 7: A UE should release its established MRB(s) and stop DL-SCH reception for a service limited to an intended service area when exiting that service area.

Proposal 8: Frequency prioritization may be performed only when the UE is located within that service area of the concerned broadcast service after cell selection.

3 Conclusion

In this contribution, we have further discussed network siganlling and UE broadcast reception behavior when MBS broadcast service is provided within the intended service area. And we make the following proposals:

Network signalling:

Proposal 1: For a cell, the network signals the whole intended service area of a broadcast session.

Proposal 2: Circles format (like for TN coverage) is reused to model the intended service area.

Proposal 3: Existing broadcast MCCH is used to include a list of service areas, where each area is associated with a service area ID.

Proposal 4: The intended serviced area of a broadcast session is indicated by adding the service area ID list in the broadcast MCCH.

Proposal 5:  RAN2 to discuss whether to include intended serviced area information in SIB21 for service continuity.

UE behavior:

Proposal 6: A UE may establish MRB(s) associated with a service limited to an intended service area only when it is located within that service area.

Proposal 7: A UE should release its established MRB(s) and stop DL-SCH reception for a service limited to an intended service area when exiting that service area.

Proposal 8: Frequency prioritization may be performed only when the UE is located within that service area of the concerned broadcast service after cell selection.

4 Reference

[1] Draft_R2_125bis_Meeting_Report_v4.

[2] Draft_R2_126_Meeting_Report_v2.