3GPP TSG-RAN WG2 Meeting #127	R2-2406248

Maastricht, Netherlands, August 19th– 23rd, 2024

Agenda Item:	8.8.5

Source: 	vivo

Title:  	Discussion on RACH-less Handover with Regeneration Payload

Document for:	Discussion and Decision

- Introduction

In RAN2#126 [1], RACH-less handover for regenerative payload is discussed and the following agreement is made.

- RAN2 aims at supporting RACH-less handover for regenerative architecture. Can further discuss whether any optimization is needed

In this contribution, we would like to further discuss the impacts of supporting the RACH-less handover for regenerative payload.

- Discussion

To make RACH-less support generalization, a section under 9.2.3 about RACH-less handover is added according to the CR R2-2406018 approved in the RAN2#126 meeting, which also has been captured in the latest TS 38.300 as below. It can be found that the RACH-less HO for NTN is introduced in Rel-18 NTN, and for now, it is restricted to the intra-gNB case [2].

#### 9.2.3.6 RACH-less handover During intra-gNB HO procedure, RACH-less handover can be configured for a UE. The RACH-less handover procedure applies the following functionality: - The UE uses the same timing advance value at the target cell as in the source cell or timing advance value of 0. - The handover command for the UE may contain a beam identifier for the beam to be used by the UE at the target cell. The beam may be determined based on a UE measurement report and/or left up to gNB implementation, e.g., using the target cell's knowledge about the beam(s) used by the UE at the co-located source cell. - The handover command may include a configured UL grant. UE can fallback to RACH when there is no valid configured uplink grant. Alternatively, an UL grant is dynamically signalled by the target cell. - The UE transmits the RRCReconfigurationComplete message using the configured or dynamically signalled UL grant. Successful UL data reception on the target cell terminates the RACH-less handover execution.

Observation 1: In the latest TS 38.300, RACH-less HO is restricted in intra-gNB case.

However, after reviewing the discussion of RACH-less for Rel-18 NTN based on the Agreements below, there is no restriction that RACH-less is only supported for intra-gNB case.

RAN2#121bis Agreements via email – from offline 109:

- NTN RACH-less HO is supported for Intra-satellite handover with the same feeder link. i.e., with same gateway/gNB;

- NTN RACH-less HO can be supported for intra-satellite handover with different feeder links, i.e., with gateway/gNB switch, inter-satellite handover with gateway/gNB switch, and inter-satellite handover with same gateway/gNB.

The above agreements have been sent to RAN1 via LS R2-2304271 in RAN2#1261bis meeting. And no negative feedback on the deployment scenario of RACH-less has been received. We think this technical logic is also applicable to NTN wetwork with regenerative payload. Hence, the stage 2 spec should be updated to allow NTN RACH-less support for both intra-gNB and inter-gNB cases in Rel-19.

Proposal 1: Update stage 2 spec to make NTN RACH-less support for both intra-gNB and inter-gNB cases.

The following is the procedure for RACH-less HO in inter-gNB case. Compared with the normal RACH-based inter-gNB handover procedure, the only difference is in step 5, step 6, step 9, step 10.

In step 5/6: the target gNB includes the RACH-Less HO configuration within its configuration, and optionally the pre-allocated configured grant which is used for the first uplink transmission.

In step 9: optionally, if the configured grant is not included in the target gNB’s configuration, the target gNB schedule the dynamic UL grant, and UE receives the uplink grant via the PDCCH of the target gNB;

In step 10: UE sends the first uplink transmission, i.e., the RRCReconfigurationComplete, to the target gNB using either the pre-allocated configured grant, or the acquired dynamic grant.

But for above differences, they have already been supported in the spec so as to support the intra-gNB RACH-less handover in R18. So it is clear there is not any issue to support inter-gNB RACH-less handover.

Figure 1: RACH-less handover procedure for inter-gNB case

Observation 2: From stage 3 aspect, inter-gNB RACH-less handover can be naturally supported based on the already introduced enhancement for intra-gNB RACH-less handover.

As for the issue in [3] that there may be some waste of resources as the target cell may try to schedule the UE unsuccessfully before it has switched in a dynamic grant case, the benefit is quite limited if we do the optimization. For example, under the case that time-based CHO is configured with RACH-less, targe gNB can schedule UE during [T1, T2] based on Conditional Handover Time Based Information IE (i.e., t1-Threshold and duration) provided by source gNB via XN/NG interface. For legacy HO command with RACH-less configuration case, target gNB can schedule UE based on NW implementation, e.g., based on minimum UE to satellite (at specific orbit) propagation delay. Specifically, after completion of the handover preparation procedure, target gNB can start to schedule UE after waiting for a time duration that equals the minimum UE to satellite (at specific orbit) propagation delay. Even though this is not the accurate timing that UE switches to targe gNB, further optimazation to obatineobtain the accurate scheduling time has limited benefit for reducing resource. Moreover, similar issue also exists for R18 RACH-less based handover in transparent payload architecture, this is not a specific issue regarding regeneration payload. It is quite confusing why we need to do specific handling for RACH-less handover in the regeneration payload architecture.

Based on the above analysis, from our perspective, no further optimization for regenerative payload architecture is needed. And supporting the RACH-less handover for regeneration payload has no stage 3 spec impacts.

Proposal 2: For RACH less support for regenerative payload architecture, no further optimization is needed, and no stage 3 spec impacts.

- Conclusions

This contribution discussed the RACH-less handover for regenerative payload in Rel-19 NR NTN. Observations and Proposals are summarized as follows,

Observation 1: In the latest TS 38.300, RACH-less HO is restricted in intra-gNB case.

Observation 2: From stage 3 aspect, inter-gNB RACH-less handover can be naturally supported based on the already introduced enhancement for intra-gNB RACH-less handover.

Proposal 1: Update stage 2 spec to make NTN RACH-less support for both intra-gNB and inter-gNB cases.

Proposal 2: For RACH less support for regenerative payload architecture, no further optimization is needed, and no stage 3 spec impacts.

- Reference

- Draft_R2_126_Meeting_Report_v2.

- TS38.300, NR; NR and NG-RAN Overall description; Stage-2.

- R2-2405148, On the feasibility of the existing NTN features over regenerative architecture, Nokia