3GPP TSG-RAN WG2 Meeting #127	R2-2406249

Maastricht, Netherlands, August 19th– 23rd, 2024

Agenda Item:	8.8.6

Source: 	vivo

Title:  	Further Discussion on LTE TN to NR NTN Mobility

Document for:	Discussion and Decision

- Introduction

In RAN2#126 [1], the LTE TN to NR NTN mobility was discussed but there are still some open issues left for further discussion, as we highlight in the following.

Agreements: 1.To support the idle mode mobility from EUTRA TN to NR NTN, the satellite assistance information for NR NTN neighbour cells is needed and should include the following parameters: - Satellite ephemeris information - TA common information - k-Mac - epoch time - validity duration - ntn-PolarizationDL (FFS if mandatory or optional) 2. RAN2 will decide in the next meeting which of the following options to adopt for the provision of the NR satellite assistance information (based on TPs provided by the WI RRC Rapporteur): Option 1: Introduce a new SIB to include the NR satellite assistance information. Option 2: Define new IE for NR satellite assistance information and define separate neighbour satellite information list to provide the NR satellite information in SIB33. Option 3: Extend the NeighSatelliteInfo defined for IoT NTN to include the parameters needed for NR satellite, and reuse the neighSatelliteInfoList defined in SIB33 to provide either NR or IoT NTN information. Working Assumption: 1. NR NTN cell reselection evaluation is based on RRM measurements as legacy; no spec impact foreseen for EUTRA TN to NR NTN cell (can come back in the next meeting to see if the WA can be confirmed)

Besides, there are also other issues, e.g., how to distinguish the NR TN cell and NR NTN cell, and also the UE capability needed for the feature.

In this contribution, we would like to further discuss the above remaining open issues on supporting the LTE TN to NR NTN IDLE mode mobility.

- Discussion

  - Provision of the ntn-PolarizationDL

Singling the polarization mode is beneficial for the UE capable of differentiating RHCP and LHCP with the circularly or linearly polarized antennas to match suitable polarization mode to avoid 3dB polarization loss [1]. Since not all scenarios it is useful and it is optimization, it is optional for NW to provide the information in NR NTN [2].

ntn-PolarizationDL If present, this parameter indicates polarization information for downlink transmission on service link: including Right hand, Left hand circular polarizations (RHCP, LHCP) and Linear polarization.

Observation 1: The polarization information is optional provided to UE in NR NTN and this is beneficial just for certain scenarios.

Similarly, we think providing the polarization information for NR NTN neighbour cell in EUTRA TN should also be optional and whether to provide it is totally up to NW.

Proposal 1: The ntn-PolarizationDL for NR NTN neighbour cells is optionally provided to UE.

  - Signalling for NR NTN neighbour satellite information provision

As for the signalling for provision of the NR NTN neighbour satellite information, RAN2 already agreed to down select from the following three options in last meeting.

  - Option 1: Introduce a new SIB to include the NR satellite assistance information.

  - Option 2: Define new IE for NR satellite assistance information and define separate neighbour satellite information list to provide the NR satellite information in SIB33.

  - Option 3: Extend the NeighSatelliteInfo defined for IoT NTN to include the parameters needed for NR satellite, and reuse the neighSatelliteInfoList defined in SIB33 to provide either NR or IoT NTN information.

Option 1 has more spec impacts compared with the option 2/option3, but the benefits of option 1 is UE not supporting the LTE TN to NR NTN mobility can skip to obtain the new SIB. Option 2 and Option 3 aims to reuse the current SIB33 but additionally add the NR NTN satellite information in the SIB33, thus the spec signalling is quite limited. However, the maximum number of satellites within the NeighSatelliteInfoList in SIB 33 is 4, if we adopt option 3, the number may be not enough for both EUTRA NTN neighbour cell reselection and NR NTN neighbour cell reselection. Given the analysis, it is preferred to use option 2, which has fewer spec impacts and also not impacts the satellite assistance information provision for EUTRA NTN neighbour cells.

Proposal 2: Option 2 is taken i.e., Define new IE for NR satellite assistance information and define separate neighbour satellite information list to provide the NR satellite information in SIB33.

  - Cell reselection for ENTRA TN to NR NTN

For NTN-to-NTN cell reselection, the time based or the location-based cell reselection is introduced for NTN. However, different like NTN cell, the TN cell is fixed and can always provide cell coverage, so the time-based cell reselection is not suitable for neighbour cell measurement initiation. Also, there is clear near far effect in EUTRA TN cell, so the legacy RRM based cell reselection mechanism can work well. Hence, there is no need to introduce the location-based cell reselection for TN-NTN cell reselection case.

Proposal 3: Confirm the work assumption: NR NTN cell reselection evaluation is based on RRM measurements as legacy; no spec impact foreseen for EUTRA TN to NR NTN cell.

  - Distinguishment of the NR TN or NTN cell

Regarding how UE distinguish NR TN frequency and NR NTN frequency, the legacy mechanism can be reused. UE discriminates whether a frequency provided in SIB24 is for NR TN or NR NTN in an implicit way, i.e., if the satellite ID is absent for a frequency in SIB24 (i.e., there is no corresponding satellite assistance information for this frequency), the UE assumes this frequency is TN frequency and performs TN frequency measurement.

Proposal 4: For an NR frequency in SIB24, if no corresponding satellite assistance information in SIB33 is present, UE assumes this frequency is TN frequency and performs TN frequency measurement.

  - UE capability

There may be UEs supporting the feature of LTE TN to NR NTN IDLE mode mobility or the UEs not supporting the feature camped on the same EUTRA NTN cell, and there are different UE behaviours. E.g., only UEs supporting the feature is required to obtain the new assistance information for NR NTN neighbour cells, while the UEs not supporting this does not need to acquire it. Moreover, as we already agreed in last meeting, upon performing NR NTN neighbour cell measurement, UE supporting the feature can automatically adjust the SMTC offset based on the actual propagation delay, when the corresponding frequency is associated with a satellite ID, but this is not required for UEs not supporting the feature. Thus, from test purpose, it is beneficial to define such UE capability.

Proposal 5: Introduce a new UE capability without signalling for LTE TN to NR NTN mobility.

- Conclusions

This contribution discussed the LTE TN to NR NTN mobility in Rel-19 NR NTN. Observations and Proposals are summarized as follows,

Observation 1: The polarization information is optional provided to UE in NR NTN and this is beneficial just for certain scenarios.

Proposal 1: The ntn-PolarizationDL for NR NTN neighbour cells is optionally provided to UE.

Proposal 2: Option 2 is taken i.e., Define new IE for NR satellite assistance information and define separate neighbour satellite information list to provide the NR satellite information in SIB33.

Proposal 3: Confirm the work assumption: NR NTN cell reselection evaluation is based on RRM measurements as legacy; no spec impact foreseen for EUTRA TN to NR NTN cell.

Proposal 4: For an NR frequency in SIB24, if no corresponding satellite assistance information in SIB33 is present, UE assumes this frequency is TN frequency and performs TN frequency measurement.

Proposal 5: Introduce a new UE capability without signalling for LTE TN to NR NTN mobility.

- Reference

- Draft_R2_126_Meeting_Report_v2.

- TS38.821, Solutions for NR to support Non-Terrestrial Networks (NTN).

- R2-2406250, LTE to NR NTN Mobility Capability CR