3GPP TSG-RAN WG2 Meeting #127	R2-2406251

Maastricht, Netherlands, August 19th– 23rd, 2024

Agenda Item:	8.9.2

Source: 	vivo

Title:  	RAN2 Aspect for S&F Operation

Document for:	Discussion and Decision

- Introduction

In RAN2#126 [1], RAN2 discussed and made the following aspects on S&F.

Agreements: 1. For the uplink/downlink messages transmission for MO, from RAN2 perspective the following steps are taken as baseline for S&F satellite operation (in case only eNB is on the satellite): 1) The UE sends uplink data signalling to eNB when service link is available and the eNB stores it. 2) When feeder link is available, the eNB sends the uplink data/NAS signalling to the CN. 3) The eNB (same or different) receives the downlink data/NAS signalling from the CN and stores it when feeder link is available (and service link is not available). 4) The eNB (same or different) sends the downlink data/signalling to the UE when service link is available again 2. S&F indication can be provided by SIB (FFS on the details). RAN2 assumes that no NAS indication is needed 3. RAN2 understands legacy UEs may be barred by legacy cellBarred and cellBarredNTN

In this contribution, we would like to further discuss the S&F operation.

- Discussion

  - On cell access control

For UE which support S&F operation, if both the cellBarred and cellBarred-NTN are set as barred, the UE cannot identify whether the cell is set to “barred” to bar the UE not supporting S&F operation or other reasons. So a new S&F barred indication need to be introduced to control whether the R19 UE supporting S&F operation is allowed to access the cell operating in the S&F mode or not.

Proposal 1: Introduce a new S&F cell barred indication for S&F operation in the SIB1.

Then based on the newly introduced cell barred indication, the UE behaviours can be like:

- For legacy UEs, use the legacy cellBarred and cellBarredNTN to decide whether to access the cell as per RAN2 agreements in RAN2#126;

- For R19 UEs not supporting the S&F operation, they can also use the legacy cellBarred and cellBarredNTN to decide whether to access the cell.

- For R19 UEs supporting the S&F cell operation, if the new S&F cell barred indication is present, they can decide the cell works at S&F mode, and the UE access is based on the value within the S&F cell barred indication, e.g., the cell is allowed to access if the new S&F cell barred indication is set to “barred”, otherwise the cell is not allowed to access. If the new S&F cell barred indication is not present, UE can follow the legacy cellBarred and cellBarredNTN.

Proposal 2: For R19 UEs not supporting the S&F operation, the UE shall follow the legacy cellBarred and cellBarredNTN indication.

Proposal 3: For R19 UEs supporting the S&F operation, the UE shall:

- follow the legacy cellBarred and cellBarredNTN indication if the new S&F cell barred indication is not present;

- consider the cell is not allowed for S&F operation if the new S&F cell barred indication is set to “barred”;

- consider the cell is allowed for S&F operation if the new S&F cell barred indication is set to “not barred”.

  - On S&F indication

In RAN2#126 meeting, regrading to S&F indication, RAN2 achieved the following agreements:

S&F indication can be provided by SIB (FFS on the details). RAN2 assumes that no NAS indication is needed

Based on the S&F conclusion in the latest SA2 TR 23.700-29 [2] below, the design of S&F indication depends on RAN. Hence, RAN can confirm that no NAS indication is needed.

NOTE 9: How the UE is aware that a satellite supports S&F mode of operation depends on RAN.

Proposal 4: RAN2 confirms that no NAS indication for S&F operation is needed.

Regarding the details of the S&F indication, this mainly depend on the usage of the S&F indication.

Firstly, The S&F indication can be used to bar UEs not supporting the S&F operation, since S&F satellite operation works very differently from legacy operation and is only applicable for the delay tolerant IoT NTN UEs. And if UEs not supporting the S&F operation try to access the S&F cell, it is quite probably that the cell access will be failed since access to the cell operating in S&F operation may need longer time due to the unavailable service link or the feeder link. In general, such information can be enabled via new S&F cell barred indication, but it is FFS whether to additionally indicate whether the cell support the S&F mode, e.g., especially for the neighbour cell so as to let UE to decide whether to perform neighbour cell measurement.

Further, along with the satellite movement, it is probably that cell in the satellite is operating in S&F mode during some time for which the service link and the feeder link is not available at the same time, while operating as normal cell for which the service link and the feeder link is available during another time. And it would be beneficial for NW to provide such information, e.g., time to operate in S&F operation, so that UE can decide whether/when to (re)select the cell to establishment connectivity, or perform neighbour cell measurement.

Moreover, in case the cell is operating in the S&F mode, it is also predictable when the service link is available, and when the feeder link is available. If UE can have such information, UE can decide when to trigger the establishment procedure, or when to initiate the transmission.

Proposal 5: Except the S&F barred indication, RAN2 to discuss which of the following information can be additionally provided via SIB:

- The info for current satellite operation, i.e., S&F operation and regenerative operation;

- The time info for the service link status;

- The time info for the feeder link status.

  - On UE behaviour in case service link is unavailable

According to the conclusion of S&F operation in SA2 TR 23.700-29, the following architectures are supported for S&F operation.

-Architecture 1: Store and Forward operation with a full CN onboard the satellite

- The whole CN including eNB, MME, SGW, PGW, HSS, E-SMLC, SMSC etc are on board each satellite. Proxies are deployed on the satellite and the ground for application traffic, including support of MT traffic, MO traffic, SMS, etc

-Architecture 2: Store and Forward operation with a Split MME architecture;

- In the Split MME architecture, HSS is on the ground.

- MME functionality is split into two parts: MME-onboard - the MME part which is onboard the satellite and MME-ground - the MME part which is on the ground network with an interface out of scope of 3GPP

For architecture 1, even though the whole CN is on the satellite, there still exists a case that UE cannot complete the whole NAS procedure before the service link is unavailable due to the service time of the satellite is not enough. For architecture 2, due to the part of split MME is on the ground, there always exist that UE cannot complete the whole NAS procedure before the service link is unavailable. Then, how to treat UE which has ongoing NAS procedure should be solved. In the legacy procedure, if the PCell link is weak or lost, UE will declare RLF and trigger RRC Reestablishment. If RRC Reestablishment procedure fails, UE will finally go to idle and discard all UE context. However, under S&F operation, the satellite will move back again and then UE can continue the ongoing NAS procedure. From this perspective, UE shall store UE context (e.g., UE NAS context).

Observation 1: From RAN2 perspective, UE with ongoing NAS procedure should store UE context when the service link is unavailable. Then, UE can continue the ongoing NAS procedure when the service link is available again.

In the current spec, RRC Release with suspend configuration can be used to make UE keep UE context. However, this mechanism is not applied to all UE (e.g., CP only UE doesn’t support RRC resume/suspend procedure) and all case (e.g., RRC resume/suspend procedure cannot be used before AS security is activated). To make UE distinguish the RRC Release due to S&F operation and keep the UE context, a new RRC release cause value can be introduced, which has less spec impact.

Proposal 6: A new RRC release cause value is introduced for S&F operation.

When the service link is available again, UE can continue the previous ongoing procedure. The main issue is how UE know the service link is available again. From our perspective, some assistance info can be carried by RRC Release message, e.g., the time info for next service link available to assistant UE to recovery RRC connection.

Proposal 7: Assistance info for service link status (e.g., the time info for the next service link available) can be additionally carried by RRC Release.

- conclusions

This contribution discussed the S&F operation in Rel-19 NR NTN. Observations and Proposals are summarized as follows,

Observation 1: From RAN2 perspective, UE with ongoing NAS procedure should store UE context when the service link is unavailable. Then, UE can continue the ongoing NAS procedure when the service link is available again.

Proposal 1: Introduce a new S&F cell barred indication for S&F operation in the SIB1.

Proposal 2: For R19 UEs not supporting the S&F operation, the UE shall follow the legacy cellBarred and cellBarredNTN indication.

Proposal 3: For R19 UEs supporting the S&F operation, the UE shall:

- follow the legacy cellBarred and cellBarredNTN indication if the new S&F cell barred indication is not present;

- consider the cell is not allowed for S&F operation if the new S&F cell barred indication is set to “barred”;

- consider the cell is allowed for S&F operation if the new S&F cell barred indication is set to “not barred”.

Proposal 4: RAN2 confirms that no NAS indication for S&F operation is needed.

Proposal 5: Except the S&F barred indication, RAN2 to discuss which of the following information can be additionally provided via SIB:

- The info for current satellite operation, i.e., S&F operation and regenerative operation;

- The time info for the service link status;

- The time info for the feeder link status.

Proposal 6: A new RRC release cause value is introduced for S&F operation.

Proposal 7: Assistance info for service link status (e.g., the time info for the next service link available) can be additionally carried by RRC Release.

- Reference

- Draft_R2_126_Meeting_Report_v2.

- TR 23.700-29, , Study on integration of satellite components in the 5G architecture.