3GPP TSG-RAN WG2 Meeting #127                                                                   R2-2406252

Maastricht, Netherlands, August 19th – 23rd, 2024

Agenda item:	8.9.3

Source:	vivo

Title:	Further Discussion on EDT Enhancement for IoT-NTN

Document for:	Discussion and Decision

1 Introduction

In the RAN2#125bis and RAN2#126 meetings, technical discussions were initiated to clarify the objective scope of EDT enhancement and study the potential solutions. As a result, RAN2 agreed to support contention-based Msg3 transmission. The following agreements were achieved:

RAN2#125bis Agreements: 1. Both NB-IoT and eMTC are within scope of uplink capacity enhancements. 2. Both C-plane and U-plane solutions are within scope of uplink capacity enhancements. 3. Only CIoT EPS is within scope of uplink capacity enhancements. RAN2#126 Agreements: 1. RAN2 focusses the study on contention-based Msg3 transmission to complete an EDT-like transaction (FFS on the details of Msg3. FFS on the procedural steps, e.g. how much we reuse of EDT and PUR procedures. FFS on allocation of resources). 2. RAN2 can continue the discussion on Diversity Slotted ALOHA (DSA) and Contention Resolution Diversity Slotted Aloha (CRDSA) for Msg3-EDT transmissions without msg1/ RAR, evaluating possible impacts on the specification, in the next RAN2 meeting (RAN2 might send an LS to RAN1 later on this). 3. If an IoT NTN UE in IDLE state is to use the new R19 contention-based procedure, the UE needs to verify/update the uplink synchronization (e.g. get GNSS fix, acquire TA) just before sending msg3.

In this contribution, we will further discuss the contention-based Msg3 transmission to complete an EDT-like transaction for IoT-NTN.

2 Discussion

Based on our understanding of the agreements, an exemplary overall procedure for contention-based Msg3 (CB Msg3) PUSCH transmission is shown below in Figure 1.

Figure 1: CB Msg3 transmisison procedure

CB Msg3 PUSCH configuration:

Firstly, it can be assumed that it is up to the network to decide whether to provide CB Msg3 PUSCH configuration applicable to a UE. The network may only enable a set of UEs amongst all UEs within the cell area to use that configuration or allow any UE camp on the current cell to use it. And that CB Msg3 PUSCH configuration may be cell-specific or area-specific. In our understanding, the CB Msg3 PUSCH configuration should be included in the SIB message so that it can be broadcast and shared to multiple UEs, similar to EDT. Further, the eNB should configure the CB Msg3 PUSCH in a periodic manner (similar way as NPRACH) so that the UE with unpredictable UL bursty data can use the next available CB Msg3 PUSCH. Another option is that the CB Msg3 PUSCH resources are only valid for one shot (similar to one-shot DPUR) or for N shots. As the CB Msg3 PUSCH is shared amongst multiple UEs, resource waste might be not a big issue compared with DPUR considering the NW can have some solutions to control the CB Msg3 PUSCH usage ratio (e.g. controlling the total number of UEs allowing to use CB Msg3 PUSCH). So we may not consider one-shot or N-shot configuration.

Aside from time domain resource allocation, configuration in other aspects, such as frequency, should be considered as well. In our understanding, from PHY transmission point of view, the UL transmission and corresponding response reception via CB Msg3 PUSCH is similar to DPUR. Thus we assume it is simplest to reuse DPUR allocation configuration for CB Msg3 PUSCH configuration. For example, the PUR-related PUSCH and PDCCH configurations can be reused.

Regarding cell-specific or area-specific configuration, we think the motivation for area-specific configuration is not clear as the UE enters a reselected cell, it may re-acquire the configuration (assuming cell-specific) again for further CB Msg3 PUSCH transmission. Meanwhile, it might be a bit difficult to coordinate the resource allocation amongst multiple regenerative eNBs onboard or eNBs on earth, also considering the complexity of area-specific PUSCH configuration.

Proposal 1: The eNB indicates contention-based Msg3 PUSCH configuration via SIB on a per-cell basis.

Proposal 2: Taking DPUR configuration structure as a reference for contention-based Msg3 PUSCH configuration.

CB Msg3 transmission triggering:

Regardless of the configuration method outcome, from the UE point of view, a UE aware of CB Msg3 PUSCH configuration anyway has to determine whether it is suitable to use that configuration. Considering the WI scope is still related to EDT, we think some EDT initiating conditions can be reused for the CB Msg3 transmission procedure. Specifically, at the RRC level, the UE shall check to figure out if the cell supports CB Msg3 transmission and whether the size of pending UL data is smaller than TBS threshold for efficient transmission, etc.

Proposal 3: The UE triggers contention-based Msg3 transmission only if the size of pending UL data is less than TBS threshold.

CB Msg3 PUSCH slot access:

Once several UEs are triggering CB Msg3 transmission, for throughput maximization, how to control the collision and congestion on the same CB Msg3 PUSCH resource becomes an important issue (i.e. The collision and congestion should be controlled under a specific level, as once the collision and congestion is higher than that level, generally, the system throughput will degrade dramatically due to inter-UE interference). In the previous RAN2 meeting, Diversity Slotted ALOHA (DSA) and Contention Resolution Diversity Slotted Aloha (CRDSA), which are widely adopted for satellite communication (e.g., CRDSA, which is enhanced from DSA, has been approved for the DVB-RCS2 specification [3]), were proposed as an efficient random access method for CB Msg3 PUSCH transmission.

Herein, we would like to compare the technical details of how DSA, CRDSA, and existing MAC slot selection (i.e. randomly select a slot for pending bursts transmission) work. These 3 schemes are shown in Figure 2.

Figure 2: Slot access scheme

MAC slot selection, in our understanding, is a technique with which the UE can randomly select an available slot to transmit the pending burst without any eNB assignment or UE-eNB coordination. For example, it is used for the PUR occasion selection and PRACH slot selection. It basically works well within a very moderate channel congestion case. If the transmitted data is not successfully decoded, then the network can send a retransmission assignment, at the cost of larger latency and DCI overhead. In conclusion, this method is simple and can achieve an average throughput while ensuring acceptable delay and data-loss probability.

DSA was originally designed for satellite communications. The key point is to allow UE to transmit multiple copies of the same packet (i.e. in a repetition manner), where the multiple copies can be either transmitted via different sub-carriers (frequency hopping for frequency diversity) or be transmitted on the same sub-carriers but spaced apart by random time intervals (time diversity) [4]. DSA solution enables a lower transmission delay, compared to the MAC slot selection solution (throughput performance is similar to MAC slot selection, best close to 0.36 copy per slot). That is repetition retransmission is performed along with the initial transmission without waiting for NW’s retransmission assignment and the collision on the second/third copy will be much lower if the channel load is light (e.g. G=0.1~0.2). Even though the NW cannot successfully decode the first copy, it can continue to decode the second/third one.

CRDSA is enhanced based on the transmission of a chosen number of copies as in DSA. The difference is that the frequency hopping pattern and the time-domain diversity pattern are well-designed. That is each copy has a pointer to the location of the other copies of the same content. Also, the pattern guarantees there will be a copy free from collision (e.g. the purple copy in Slot#7 shown in Figure 2) so that the network receiver to accomplish the Successive Interference Cancellation (SIC) process. Take Figure 2 as an example, the network can first decode the purple copy in Slot#7 for UE#1 and then decode the Slot#4 copy by removing the content of the already decoded Slot#7 copy, where Slot#4 purple copy can be regarded as the same as Slot#7 purple copy, that is a collision interference to Slot#4 yellow copy). With this, it is possible to successfully decode Slot#4 yellow copy and then do the same for Slot#2 and Slot#5. In theory, with CRDSA, the throughput can be up to 1 copy per slot with the benefit of flexible UL data access to the network (i.e., latency reduction). NOTE that the existing NPRACH design is quite similar to the logic of CRDSA without SIC. That is preamble transmission is based on group transmission as per TS 36.211 (i.e. equivalent to copy in CRDSA) while the time-frequency pattern is designed for collision avoidance. An example of NPRACH transmission is illustrated in Figure 3.

According to Table 10.1.6.1-1 in TS36.211, one NPRACH preamble for format 0 and format 1 consists of 4 symbol groups, with each symbol group comprising one CP and 5 symbols. Preamble format. 0; 4; 4; 5. 1; 4; 4; 5. 2; 6; 6; 3; 3∙24576Ts.

Figure 3. NPRACH transmission

Based on the above description, the main characteristics of these 3 thress schemes are categorized here:

| Characteristics | MAC slot selection | DSA | CRDSA |
| --- | --- | --- | --- |
| UE slot selection behavior | UE randomly selects a slot | UE randomly selects the first slot and selects the next slot with a time-frequency hopping. | UE selects the slots based on the time-frequency hopping pattern, similar to NPRACH hopping. |
| NW decoding | Individual decoding | Individual decoding | Decoding with SIC |
| UE response reception | Start the response window for contention resolution | May start multiple response windows for each copy transmission | Start the response window for contention resolution |
| Peak throughput [4, 5] | Up to the value close to 0.36 copy per slot | Up to the value close to 0.38 copy per slot | Up to almost 1 copy per slot with ideal interference cancellation |
| Packet loss rate [4, 5] | 10%@G≈0.13 | 10%@G≈0.21 (2 copy) | 10%@G≈0.55 (2 copy) |
| Retransmission Latency | It may be large due to retransmission relying on DL assignment | Improved due to multiple copies | Greatly improved with multiple copies and less interference |
| Spec Impacts | None, existing scheme. | RAN2 modification is needed, e.g. support multiple CR windows | PHY signal new design is needed (e.g. defining the pattern, similar to NPRACH). Maybe RAN2 modification is needed for UL grant handling |

In conclusion, the following observations summarize the differences among the solutions explained in the above table:

Observation 1: DSA and CRDSA enable a much lower retransmission delay with the help of copy repetition.

Observation 2: DSA enables a slightly lower packet loss rate when the channel load is light (G=0.1-0.25) while CRDSA enables a significantly lower packet loss rate even when the channel load is high (G=0.5-0.6).

Observation 3: DSA can hardly bring peak throughput performance gain (i.e. negligible) while peak throughput performance gain by using CRDSA is significant.

Observation 4: Adopting DSA or CRDSA for contention-based Msg3 transmission requires specific RAN1/RAN2 spec modification.

Frankly, adopting DSA or CRDSA for contention resolution Msg3 transmission does bring additional performance gains in terms of delay and throughput. It seems RAN2 can agree to introduce low-complexity DSA and CRDSA solutions if there is majority support. However, we would like to point out that the OCC solution for the NPUSCH discussed in RAN1 might achieve higher performance gain than DSA or CRDSA in all KPI domains (e.g. delay, throughout, PLR), thanks to PHY channel enhancement with OCC in the repetition transmission case [6]. Thus, we think RAN2 should send an LS to RAN1, asking RAN1 to evaluate which solution (i.e. DSA or CRDSAor OCC) is the best choice for CB Msg3 PUSCH transmission.

Observation 5: Adopting OCC for NPUSCH in contention-based Msg3 transmission may achieve higher performance gain than using DSA or CRDSA.

Proposal 4: Before further discussing DSA and CRDAS, RAN2 asks RAN1 whether the OCC solution that they are working on is also applicable to contention-based Msg3 PUSCH transmission and which options (i.e. OCC, DSA, and CRDAS) will best increase UL capacity.

Message content:

For the initial transmission in the CB Msg3 transmission procedure, we think the UL transmission in DPUR procedure can be reused. That is, after triggering CB Msg3 transmission at RRC lay, the UE includes RRCEarlyDataRequest message in which the NAS container includes traffic data in CP CIoT Optimization case. Further, for UP CIoT Optimization case, the UE resumes the SRB and DRBs, thus containing both RRCConnectionResumeRequest message and UL UP data from DRB in the CB Msg3 PUSCH. Therefore, we propose

Proposal 5: The RRC messages that are applicable for DPUR can be reused for the contention-based Msg3 transmission.

Response reception:

After the initial UL transmission, the UE needs to start a PDCCH monitoring window for response reception. After sending to RRC_IDLE, the UE may select the cell that sends the RRC Release message to camp. In such case, to reduce Msg4 overhead, it is supposed to reuse the efficient CP-PUR reception procedure. Specifically, PDCCH addressed to UE-specific RNTI (e.g. C-RNTI) should be used for contention resolution, which can assign PDSCH to include RRC RRCEarlyDataComplete or RRCConnectionRelease potentially with DL UP data to terminate the contention-based Msg3 transmission procedure. Besides, as overhead reduction should be also considered as per WID, we see it is straightforward to reuse the efficient DL delivery for CP transmission using PUR. That is UE terminates CP PUR procedure if L1 ACK or the MAC PDU contains only TAC MAC CE is received. For example, when the UE determines to initiate CP-based contention-based Msg3 transmission in the cell where the previous RRC Release message is sent, it will start to monitor UE-specific RNTI scrambled PDCCH for response. As long as the corresponding L1 ACK is received or MAC PDU consisting of only TAC MAC CE is received, the UE considers as if an empty RRCEarlyDataComplete is received and then ends the ongoing RRC procedure, as shown in Figure 4.

Figure 4: CP-based contention-based Msg3 transmission

Proposal 6: After the initial UL transmission via contention-based Msg3, UE starts a window for response reception taking UE-eNB RTT into account.

Proposal 7: For CP-based contention-based Msg3 transmission, DPUR response for CP transmission is reused (e.g. the eNB sends L1 ACK or TAC MAC CE to acknowledge the initial UL transmission) if the UE is in the cell where the previous RRC Release message was sent.

3 Conclusion

In this contribution, we have provided our understanding of the contention-based Msg3 transmission to complete an EDT-like transaction for IoT-NTN. And we made the following observations and proposals:

CB Msg3 PUSCH configuration:

Proposal 1: The eNB indicates contention-based Msg3 PUSCH configuration via SIB on a per-cell basis.

Proposal 2: Taking DPUR configuration structure as a reference for contention-based Msg3 PUSCH configuration.

CB Msg3 transmission triggering:

Proposal 3: The UE triggers contention-based Msg3 transmission only if the size of pending UL data is less than TBS threshold.

CB Msg3 PUSCH slot access:

Observation 1: DSA and CRDSA enable a much lower retransmission delay with the help of copy repetition.

Observation 2: DSA enables a slightly lower packet loss rate when the channel load is light (G=0.1-0.25) while CRDSA enables a significantly lower packet loss rate even when the channel load is high (G=0.5-0.6).

Observation 3: DSA can hardly bring peak throughput performance gain (i.e. negligible) while peak throughput performance gain by using CRDSA is significant.

Observation 4: Adopting DSA or CRDSA for contention-based Msg3 transmission requires specific RAN1/RAN2 spec modification.

Observation 5: Adopting OCC for NPUSCH in contention-based Msg3 transmission may achieve higher performance gain than using DSA or CRDSA.

Proposal 4: Before further discussing DSA and CRDAS, RAN2 asks RAN1 whether the OCC solution that they are working on is also applicable to contention-based Msg3 PUSCH transmission and which options (i.e. OCC, DSA, and CRDAS) will best increase UL capacity.

Message content:

Proposal 5: The RRC messages that are applicable for DPUR can be reused for the contention-based Msg3 transmission.

Response reception:

Proposal 6: After the initial UL transmission via contention-based Msg3, UE starts a window for response reception taking UE-eNB RTT into account.

Proposal 7: For CP-based contention-based Msg3 transmission, DPUR response for CP transmission is reused (e.g. the eNB sends L1 ACK or TAC MAC CE to acknowledge the initial UL transmission) if the UE is in the cell where the previous RRC Release message was sent.

4 Reference

[1] Draft_R2_125bis_Meeting_Report_v4.

[2] Draft_R2_126_Meeting_Report_v2.

[3] Second generation DVB interactive satellite system; part 2 lower layers for satellite standard, in Digital Video Broadcasting (DVB), ETSI EN 101 545-2 V1.1.1.

[4] G. L. Choudhury and S. S. Rappaport, Diversity ALOHA—A random access scheme for satellite communications.

[5] A. Meloni and M. Murroni, Random Access in DVB-RCS2: Design and Dynamic Control for Congestion Avoidance.

[6] R1-2404197, Discussions on IoT-NTN uplink capacity enhancement, vivo.