3GPP TSG-RAN WG2 Meeting #127	R2-2406254

Maastricht, Netherlands, Aug 19~23, 2024

Agenda item:	8.7.1.1

Source:	Qualcomm Incorporated, Nokia, Nokia Shanghai Bell

Title:	Discussion on reply LS on PSI-based SDU discard in NR-DC

WID/SID:	NR_XR_Ph3-Core

Document for:	Discussion and decision

# Introduction

In the LS R3-243957 [1], RAN3 asks RAN2 to clarify the UE behavior for handling uplink PSI based SDU discard when MN and/or SN sends a PSI-Based SDU Discard Activation/Deactivation MAC CE.

In this paper, we first present our understanding on the current spec (Rel-18). We then discuss why the current UE behavior is not desirable and should be enhanced. Based on that discussion, we propose enhanced conditions for UE to de-/activate PSI-based SDU discard for a UL split bearer and enhanced procedure for UE to report its data volume based on the state of PSI-based SDU discard.

# Discussion

RAN3 has clarified in their LS that

In case of NR-DC split bearer, MN DU and SN DU congestion is local to each node. Based on congestion situation in MN DU and/or SN DU, each of the nodes independently send PSI-Based SDU Discard Activation/Deactivation MAC CE to UE for activation/deactivation of UL PSI based PDU set discarding of the split bearer.

Observation 1.	According to RAN3, MN and SN can independently send PSI-Based SDU Discard Activation/Deactivation MAC CE to UE, based on their own states of congestion.

According to the current RAN2 specs (clause 5.18.34 in TS 38.321 and clause 5.2.1 and 5.6 in TS 38.323), UE de-/activates PSI-based SDU discard on a split UL bearer once it receives a de-/activation MAC CE. This de-/activation is applied independently from which node sends the MAC CE and does not depend on the current congestion state of the two nodes.

Observation 2.	According to the current RAN2 spec, UE de-/activates PSI-based SDU discard on a split UL bearer once it receives a PSI-Based SDU Discard De-/Activation MAC CE. This de-/activation is applied independently from which node sends the MAC CE and does not depend on the current congestion state of the two nodes.

Based on the above two facts, one can identify a contradicting UE behavior in the following two scenarios:

- Scenario #1: Suppose initially both MN and SN are in congestion and consequently UE has activated PSI-based SDU discard. Then UE receives a PSI-Based SDU Discard De-/Activation MAC CE deactivating discards from one of the nodes. According to the current specs, this MAC CE triggers UE to deactivates PSI based SDU discard, even though one of the nodes is still in congestion.

- Scenario #2: Suppose initially neither MN nor SN is in congestion. Then UE receives a PSI-Based SDU Discard De-/Activation MAC CE activating discard from one of the nodes. According to the current specs, this MAC CE triggers UE to activate PSI based SDU discard.

It is not hard for one to notice that in the two scenarios above, the UE exhibits different SDU discard behaviors, although in both cases only one node is congested. That is because in the current procedure, whether PSI-based SDU discard should be activated also depends on whether MN and SN previously were in congestion or not.

Observation 3.	As a result, UE may have different behaviors for PSI-based SDU discard when only one of the network nodes is congested, depending on whether MN and SN previously are in congestion or not.

We do not think such a UE behavior is desirable, as congestion-based SDU discard should depend only on the current congestion state of network nodes. In addition, if only one of the network nodes is congested, UE does not need to activate PSI-based discard, because UE can still use the other network node without problem.

A more effective approach is for UE to decide whether to de-/activate PSI-based discard by aggregating congestion indication from both MN and SN. This approach allows network to issue congestion indication based on only their local congestion status, eliminating the need for coordination between MN and SN. As a result, this approach obviates the need for new inter-gNB signaling and simplifies deployment.

Observation 4.	Empowering UE to determine the de-/activation of PSI-based discard by aggregating congestion indication from both MN and SN can eliminate the need for coordination between MN and SN.

Therefore, we think the following corrections to the current specs should be considered:

Proposal 1. 	UE activates PSI-based SDU discard on a UL split-bearer only after it has received MAC CEs activating the discard from both MN and SN.

Proposal 2.	UE deactivates PSI-based SDU discard on a UL split-bearer once it has received a MAC CE deactivating the discard from either MN or SN.

A related issue is PDCP data volume reporting in case of congestion. If a network sends a PSI-Based SDU Discard Activation MAC CE, it implies that specific node is congested. The UE hence should avoid sending data or sending less data through that node. We think a simple change to implement that is to apply a different data volume reporting threshold (i.e. ul-DataSplitThreshold) in this case. More specifically,

Proposal 3.	After receiving a PSI-Based SDU Discard De-/Activation MAC CE on the primary path that activates SDU discard, UE applies a smaller ul-DataSplitThreshold.

Proposal 4.	After receiving a PSI-Based SDU Discard De-/Activation MAC CE on the secondary path that activates SDU discard, UE applies a larger ul-DataSplitThreshold.

If the UE has received PSI-Based SDU Discard De-/Activation MAC CEs on both the primary and secondary paths, both network nodes are congested. Hence UL traffic does not need to be routed differently from that in the default case. Therefore, the same ul-DataSplitThreshold used in the normal case can be applied.

Proposal 5.	UE applies the legacy ul-DataSplitThreshold if both primary and secondary paths are in congestion state.

Although the discussion above is for Rel-19, we think the proposed UE behaviors are relevant and correct in Rel-18 too. Therefore, we think that the above enhancements (Proposal 1~5) can be considered for Rel-18 corrections too.

Proposal 6.	Adopt the above enhancements (Proposal 1~5) as Rel-18 corrections too.

If the above proposals can be agreed, RAN2 should include them in the reply LS to RAN3.

Proposal 7.	Include the above enhancements and Observation 4, if agreed, in the reply LS to RAN3.

# Conclusion

We respectfully request RAN2 to discuss and agree to the following proposals:

Observation 1.	According to RAN3, MN and SN can independently send PSI-Based SDU Discard Activation/Deactivation MAC CE to UE, based on their own states of congestion.

Observation 2.	According to the current RAN2 spec, UE de-/activates PSI-based SDU discard on a split UL bearer once it receives a PSI-Based SDU Discard De-/Activation MAC CE. This de-/activation is applied independently from which node sends the MAC CE and does not depend on the current congestion state of the two nodes.

Observation 3.	As a result, UE may have different behaviors for PSI-based SDU discard when only one of the network nodes is congested, depending on whether MN and SN previously are in congestion or not.

Observation 4.	Empowering UE to determine the de-/activation of PSI-based discard by aggregating congestion indication from both MN and SN can eliminate the need for coordination between MN and SN.

Proposal 1. 	UE activates PSI-based SDU discard on a UL split-bearer only after it has received MAC CEs activating the discard from both MN and SN.

Proposal 2.	UE deactivates PSI-based SDU discard on a UL split-bearer once it has received a MAC CE deactivating the discard from either MN or SN.

Proposal 3.	After receiving a PSI-Based SDU Discard De-/Activation MAC CE on the primary path that activates SDU discard, UE applies a smaller ul-DataSplitThreshold.

Proposal 4.	After receiving a PSI-Based SDU Discard De-/Activation MAC CE on the secondary path that activates SDU discard, UE applies a larger ul-DataSplitThreshold.

Proposal 5.	UE applies the legacy ul-DataSplitThreshold if both primary and secondary paths are in congestion state.

Proposal 6.	Adopt the above enhancements (Proposal 1~5) as Rel-18 corrections too.

Proposal 7.	Include the above enhancements, if agreed, in the reply LS to RAN3.

# Reference

- R3-243957, LS on UL PSI based PDU discarding in NR-DC, RAN3, May 2024.