3GPP TSG-RAN WG2 Meeting #127	R2-2406255

Maastricht, Netherlands, Aug 19~23, 2024

Agenda item:	8.7.1.1

Source:	Qualcomm Incorporated

Title:	Discussion on reply LS to SA2 on FS_XRM Ph2

WID/SID:	NR_XR_Ph3-Core

Document for:	Discussion and decision

# Introduction

In the LS S2-2407351, SA2 asks RAN2 to provide feedback on two questions:

- indicating periodicity via in-band signaling (i.e. in GTP-U) for dynamic changes of the periodicity;

- whether the time to next burst is useful for RAN resource scheduling

We share our views on the above two questions in this paper.

# Discussion

Question 1

Traffic periodicity has direct impact on RAN radio configurations such as DRX. When traffic changes periodicity, those configurations need to be adjusted accordingly so that RAN can continue its operations efficiently. For example, DRX cycle should match traffic periodicity to maximize UE power savings. When traffic changes its periodicity, DRX cycle needs to be updated immediately so that UE can avoid unnecessary active time or increased scheduling latency. Therefore, we think that from RAN2’s perspective, it is useful for RAN to be informed about changes of traffic periodicity, if they are dynamic enough. But whether the indication should be in band or out of band is outside RAN2’s scope and is better to be decided by other WGs.

Proposal 1. 	It is useful for RAN to be informed of dynamic changes of traffic periodicity, as they have impact on RAN configurations such as DRX.

Question 2

If RAN has the information of the time to next burst, it can configure an optimal DRX start offset based on that, i.e. UE starts its DRX on duration timer right at when the next burst starts. Of course, this is possible only in the idealized scenario, as the path between RAN and the Application Server can introduce jitters. However, we think that information can still help RAN reduce its estimate on the range of the end-to-end jitters. An improved estimate on jitter can help RAN find a better DRX start offset than without that information. Therefore, we believe RAN can benefit from the knowledge of the time to next burst.

Proposal 2. 	RAN can benefit from the knowledge of the time to next burst, because it can use that knowledge to configure/signal a better DRX start offset for next DRX cycle and save more UE power.

# Conclusion

We respectfully request RAN2 to discuss and agree to the following proposals:

Proposal 1. 	It is useful for RAN to be informed of dynamic changes of traffic periodicity, as they have impact on RAN configurations such as DRX.

Proposal 2. 	RAN can benefit from the knowledge of the time to next burst, because it can use that knowledge to configure/signal a better DRX start offset for next DRX cycle and save more UE power.

# Reference

- S2-2407351, LS on FS_XRM Ph2, SA2, May 2024.