3GPP TSG-RAN WG2 Meeting #127	R2-2406256

Maastricht, Netherlands, Aug 19 – 23, 2024

Agenda item:	8.7.4

Source:	Qualcomm Incorporated

Title:	Discussion on delay-aware scheduling

WID/SID:	NR_XR_enh-Core

Document for:	Discussion and decision

# Introduction

In the Rel-19 XR SID, the following objective is specified:

For the UL, Study and if justified, specify enhancements using delay/deadline information, for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs. [RAN2]. Note: LCP implementation complexity need to be taken into account when evaluating solutions.

In this paper, we discuss three enhancements for delay-aware scheduling:

- Dynamic priority in LCH prioritization

- Delay-aware LCP restriction

- Delay status report

based on the agreements on these three topics so far.

# Discussion

## Delay-aware LCH prioritization

In RAN2#126, the following agreements on LCH prioritization were made:

- For delay-aware LCP enhancement, RAN2 considers the following option to override/adjust the priority of LCH based on delay/deadline information as a baseline: - Use additional priority configured to LCHs in case of these LCHs with delay-critical data - FFS whether the priority only applies to delay-critical data within the LCH or for the whole LCH - FFS whether a separate remaining time threshold can be configured for delay aware LCP (i.e. different from the one used for DSR).

Before we discuss technical details of the enhancement and its potential impacts on LCP and other scheduling procedures, we think it is worth discussing how it should be modelled in the spec. For instance,

- In legacy, after data from an LCH is multiplexed in a TB, the Bj of that LCH is updated (i.e. it is deduced by the size of the data). If this data has had its priority adjusted, it needs to be discussed which LCH’s Bj should be adjusted? Because Bj is associated with an LCH, not a priority. Similarly, since LCP restrictions are associated with LCH, not priority, which LCP restrictions should be applied after data has priority adjustment, the ones configured for its original LCH or another one?

- If after priority adjustment, the new priority of the data is the highest among all the buffered data. In that case, should a BSR be triggered? If yes, when the BSR MAC CE is sent, should the size of the data be included in the data volume calculation of its original LCG or another LCG? Although LCHs with the same priority are likely to be associated with the same LCG, it is more direct and clearer, at least from modelling perspective, to specify which new LCH instead of new priority that the data is associated with.

- After a BSR is triggered due to priority adjustment for some data, UE needs to select a SR configuration to request UL grant. This selection depends on the mapping between SR configurations and LCHs. For the same reason as for BSR, it is also more direct and clearer to specify which new LCH instead of new priority that the data is associated with, so that UE can choose the right SR configuration accordingly.

Observation 1.	Priority adjustment may have impact on Bj update, LCP restriction, triggering and reporting in BSR and selection of SR configuration, all of which are associated with LCH instead of priority.

We therefore think that it makes more sense, at least from spec modelling perspective, to assign data to another LCH instead of another priority.

Proposal 1. 	To simplify spec modelling, data is reassigned to another LCH, rather than to another priority, after its remaining time falls below a configured adjustment threshold.

Next, we discuss configuration related aspects of the enhancements, e.g. the two FFS issues.

DSR triggering and LCH/priority adjustment serve different purposes in the UL scheduling process. DSR is triggered to inform network that buffered data starts running low in its remaining time and needs to be scheduled sooner. Priority adjustment is needed only when remaining time of an SDU becomes critical and more/higher scheduling opportunities are needed to ensure it will not miss its deadline.

Based on the differences in their purposes, it is not hard to see that it makes more sense for these two procedures to happen sequentially instead of simultaneously. Or network should at least have the option of being able to separate them (i.e. configuring different triggering thresholds for them). Therefore, a separate threshold should be configured for the LCH/priority adjustment.

Since the value of this threshold is more likely to depend on how fast gNB can schedule UE than the delay budget of an LCH, we think it is sufficient to configure only one adjustment threshold per LCH. In addition, for the same reason, we think it is sufficient for the value of this adjustment threshold to be configured per MAC instead of per LCH.

Proposal 2.	The LCH adjustment threshold is configured separately from the DSR triggering threshold, and the value of this adjustment threshold is configured per MAC entity.

The reason that delay-critical data is given a higher priority than its default is because it is at risk of missing its delay deadline and needs to be scheduled sooner than normal. Otherwise, if data has enough remaining time, it is sufficient to schedule it at its default priority first. This decision does not need to depend on whether there is delay-critical data in the same LCH or not.

To illustrate this point further, consider the following example: suppose a very large data burst arrives in its LCH in two pieces. A small SDU arrives first and has become delay critical when the rest of the burst arrives. It does not make any sense that the whole data burst is given a higher priority immediately upon its arrival only because a small SDU in the same LCH has become delay critical.

Therefore, priority adjustment should be applied to only delay-critical data.

Proposal 3. 	Within an LCH, only data whose remaining time is below the adjustment threshold can be reassigned to another LCH.

We think it is necessary to add an additional restriction to the LCH/priority adjustment. Within an LCH, only data which has not been reassigned from other LCHs can be considered for reassignment, i.e. data can be reassigned for at most once. This is because otherwise, reassigned data can be adjusted to another LCH again as its remaining time meets the reassignment threshold. This behavior is not a result of the configuration in which the adjustment threshold is per MAC entity. It can still happen if each LCH has a different adjustment threshold, i.e. once the remaining time of reassigned data meets the adjustment threshold of its current LCH, it can reassigned to another LCH again.

Proposal 4.	Within an LCH, only data which has not been reassigned from other LCHs can be considered for reassignment, i.e. data can be reassigned for at most once.

Next, we discuss the impact of LCH/priority adjustment on the LCP and other scheduling procedures.

In the LCH prioritization procedure, Bj controls how much data per unit time can be scheduled from an LCH. Now consider an LCH (denoted as LCH#1) that has some data which is reassigned from another LCH (denoted as LCH#2). After the reassigned data is multiplexed in a TB, if Bj of LCH #1 is deducted as a result of that, then it is not fair to the data whose original LCH is LCH#1, because the reassigned data takes away some bandwidth from LCH#1’s PBR. Therefore, even after data is reassigned to a different LCH, only the Bj of its original LCH is updated when the data is transmitted.

Proposal 5. 	When reassigned data is transmitted, the Bj of its original LCH is updated. No other LCHs need to update their Bj.

The reason that is reassigned to a different LCH is to give it more scheduling opportunities. Therefore, after its reassignment, as far as scheduling is concerned, it should be treated the same way as the data incumbent in its reassigned LCH. By this argument, this equal treatment should include LCP restriction too.

Proposal 6. 	After data is reassigned to a new LCH, it should be treated the same in the LCH prioritization and apply the same LCP restriction as the incumbent data in that LCH.

After data is reassigned to a new LCH, if it has a priority higher than any other data in UE’s buffer, then according to the current BSR triggering criteria, a regular BSR should be triggered. We think this behavior makes sense, because in this case network should be informed of the new buffer status as a result of the reassignment, so that the reassigned data can get a proper UL grant.

Proposal 7. 	After data is assigned to a new LCH, a BSR is triggered if its new priority is higher than that of any other data in the buffer.

After a BSR is triggered, SR is transmitted if no PUSCH resource is available. If there are multiple SR configurations, UE chooses the SR configuration which is associated with the LCH which triggered the BSR. If the BSR is triggered by reassigned data, we think UE should choose an SR configuration based on the priority of the new LCH of the reassigned data. The reason is the same as argued in Proposal 6, i.e. the reassigned data should be scheduled with its new priority.

Proposal 8. 	If an SR is triggered due to data reassigned to a new LCH, UE should use the SR configuration associated with the new LCH.

## Delay-aware LCP restrictions

In this section, we describe two uses case in which UE can benefit from making LCP restrictions conditional on remaining time of data.

Use Case 1.

Since Rel-15, a few LCP restrictions, such as maxPUSCH-Duration and configuredGrantType1Allowed have been introduced to ensure that certain UL grants are used only by delay-sensitive data, because those UL grants are optimized for low latency instead of spectral efficiency.

Those LCP restrictions can be relevant for XR traffic too. For example, if remaining time of XR data has not dropped below a threshold, the data should be subject to LCP restrictions such as maxPUSCH-Duration. But after the remaining time of data drops below a threshold, low latency scheduling is needed to meet its delay requirement. Hence LCP restrictions such as maxPUSCH-Duration can be relaxed or removed.

Observation 2. 	Delay-critical data can have more scheduling opportunities if the decision to apply LCP restriction also depends on an SDU’s remaining time.

Use Case 2.

XR applications can generate different types of traffic flows, which may have different traffic characteristics and QoS requirements. Therefore, it can be more efficient to serve different traffic flows on different radio resources. For example, UL TRPs deployed near cell edge can help improve coverage for UEs when they move away from the cell center. On the other hand, traffic routed through them may incur additional delay. Since video traffic is more delay tolerant, it can be routed through a cell-edge UL TRP instead of directly through gNB to take advantage of its higher throughput. On the other hand, it is better to route haptic/control traffic directly through gNB, because those flows do not require high throughput but are much more delay sensitive.

Similar arguments can be applied to data with different delay status within the same LCH. For example, when the remaining time of an SDU drops below a threshold, it should be routed through radio resources with low delay (e.g. directly to gNB instead of via a UL TRP near cell edge). Otherwise, it should be routed through radio resources optimized for throughput.

Routing traffic through different radio resources can be implemented through LCP restrictions, e.g. by configuring different allowedServingCells for different LCHs. However, legacy LCP restrictions are “static”, i.e. a restriction does not change with states of the LCH (e.g. delay status, buffer size). Thus, it can’t be used to steer traffic based on their delay status, as described above. One way to fix that is to make LCP restrictions conditional on the remaining time of an SDU. For example, an LCH can be configured with two sets of LCP restrictions:

- Default restrictions: This set of LCP restrictions is used by SDUs whose remaining times are above a threshold;

- Delay-critical restrictions: This set of LCP restrictions is used by SDUs whose remaining time are below a threshold.

Observation 3. 	Delay-aware scheduling can be implemented by applying different LCP restrictions to different data within an LCH based on their delay status, e.g. restrict delay-critical data to use only UL grants on low-latency radio resources.

Based on the analysis of the two use cases above, we can see that LCHs with delay-sensitive traffic can be better supported if they can apply a different set of LCP restrictions or no restrictions based on their delay status.

Proposal 9.	Study enhancements that allow different data within an LCH to apply different LCP restrictions based on their delay status. FFS which LCP restrictions to include.

## Delay-aware UL scheduling in DC

In DC configurations, different cell groups may have different delay performance. For example, suppose the secondary PCell is a high-band carrier not co-located with the PCell. As a result, it has higher throughput but a longer delay for its traffic. Then it is not hard to see that the same use case described in the previous section applies too, i.e. it is more efficient to route video traffic through the SN but haptic/control traffic through the MN.

Observation 4. 	If different cell groups have different delay and throughput performance, it is desirable to route different types of traffic through different cell groups.

In legacy, network controls how UE routes its UL traffic by data volume. More specifically,

- Network configures one of the cell groups to be the primary path and the other secondary path.

- If a DRB’s data volume is below a configured threshold, ul-DataSplitThreshold, UE reports its data volume only to the primary path. Otherwise, it reports data volume to both paths.

However, in some scenarios, preferred routes based on delay (or remaining time) may be different from those based on data volume. For example, when a data burst just starts to arrive (thus data volume is low) and SDUs still have enough remaining time left, data can be reported and sent to either cell group. When buffer starts to build up (thus data volume is high) and the remaining time of some data is approaching zero, then the delay-critical data should be routed only through the cell group with small delays. Therefore, delay sensitive traffic should be routed based on its delay status instead of its data volume.

Observation 5. 	Delay-sensitive traffic should be routed based on delay status instead of data volume.

Based on the above analysis, we think it is well justified to study new rules for scheduling different types of XR traffic with UL split bearers. This study can include what information UE should report to network (e.g. DSR vs BSR or both) about a delay-sensitive DRB and how UE should route delay-sensitive traffic based on its delay status.

Proposal 10. 	Study enhancements for UE to report delay-sensitive data on a UL split bearer in DC configuration based on remaining time threshold instead of data volume threshold.

## Enhancements to DSR

In RAN2#126, the following agreements on DSR were made:

- Enhance DSR to report with multiple pairs of remaining time and buffer size for the LCG. - FFS whether DSR triggering is impacted - FFS whether PDU set importance needs to be included

Triggering threshold and reporting thresholds for DSR serve different purposes. The motivation behind multiple reporting thresholds is to increase granularity of delay information for the network. On the other hand, the benefit of having multiple triggering thresholds is only marginal, because after network obtains remaining time of a PDU in a DSR, it can estimate the remaining time of that PDU in any sequent slot. Therefore, triggering threshold and reporting thresholds can be configured separately, and the number of reporting thresholds should not affect the configuration of triggering threshold.

Proposal 11.	DSR is triggered only based on the triggering threshold and is not impacted by the presence of multiple reporting thresholds.

At the last meeting, it was also discussed whether enhanced DSR MAC CE should include information related to PDU Set Importance (PSI). In our view, that is not necessary. Other than discarding during congestion, PDU sets with difference importance levels do not need other differentiated handling. In fact, SA2 discussed QoS handling for different PSIs in Rel-18 and at the end agreed that PDU sets with different importance levels should NOT be mapped to different logical channels. That means that PSIs have the same delay and reliability requirements during normal operations.

Proposal 12. 	Enhanced DSR MAC CE does not need to include any delay information related to PDU Set Importance.

After PSI-based SDU discard is activated, UE applies a different (shorter) PDCP discard timer for PDU sets with low importance, to alleviate congestion. However, this mechanism has an unintended impact on DSR. For example, with a shorter discard timer, a PDU set with low importance would trigger DSR sooner than PDU sets with high importance but arriving around the same time. Such a pre-mature request for scheduling can mis-lead gNB to allocate more radio resources to PDU sets with low importance than necessary, because they are more subject to discard during congestion.

A simple way to avoid the negative impacts described above is not to include PDU set with low importance in DSR operations when PSI-based discard has been activated. More specifically,

- PDU sets with low importance does not trigger DSR;

- Data volume reported in a DSR MAC CE does not include SDUs in PDU sets with low importance.

It is also worth noting that the classification of low importance PDU sets is up to UE implementation. Hence UE can control which PDU sets should be subject to more discarding based on its estimate on the severity of congestion. Therefore, excluding low-importance PDU sets from DSR triggering and DSR MAC CE will not impact much on the application performance.

Proposal 13. 	If PSI-based SDU discard has been activated, PDU sets with low importance do not trigger DSR and are not reported in DSR MAC CE.

# Conclusion

Based on the above analysis, we respectively request RAN2 to discuss and agree to the following proposals:

Delay-aware LCH prioritization

Observation 1.	Priority adjustment may have impact on Bj update, LCP restriction, triggering and reporting in BSR and selection of SR configuration, all of which are associated with LCH instead of priority.

Proposal 1. 	To simplify spec modelling, data is reassigned to another LCH, rather than to another priority, after its remaining time falls below a configured adjustment threshold.

Proposal 2.	The LCH adjustment threshold is configured separately from the DSR triggering threshold, and the value of this adjustment threshold is configured per MAC entity.

Proposal 3. 	Within an LCH, only data whose remaining time is below the adjustment threshold can be reassigned to another LCH.

Proposal 4.	Within an LCH, only data which has not been reassigned from other LCHs can be considered for reassignment, i.e. data can be reassigned for at most once.

Proposal 5. 	When reassigned data is transmitted, the Bj of its original LCH is updated. No other LCHs need to update their Bj.

Proposal 6. 	After data is reassigned to a new LCH, it should be treated the same in the LCH prioritization and apply the same LCP restriction as the incumbent data in that LCH.

Proposal 7. 	After data is assigned to a new LCH, a BSR is triggered if its new priority is higher than that of any other data in the buffer.

Proposal 8. 	If an SR is triggered due to data reassigned to a new LCH, UE should use the SR configuration associated with the new LCH.

Delay-aware LCP restrictions

Observation 2. 	Delay-critical data can have more scheduling opportunities if the decision to apply LCP restriction also depends on an SDU’s remaining time.

Observation 3. 	Delay-aware scheduling can be implemented by applying different LCP restrictions to different data within an LCH based on their delay status, e.g. restrict delay-critical data to use only UL grants on low-latency radio resources.

Proposal 9.	Study enhancements that allow different data within an LCH to apply different LCP restrictions based on their delay status. FFS which LCP restrictions to include.

Delay-aware UL scheduling in DC

Observation 4. 	If different cell groups have different delay and throughput performance, it is desirable to route different types of traffic through different cell groups.

Observation 5. 	Delay-sensitive traffic should be routed based on delay status instead of data volume.

Proposal 10. 	Study enhancements for UE to report delay-sensitive data on a UL split bearer in DC configuration based on remaining time threshold instead of data volume threshold.

Enhancements to DSR

Proposal 11.	DSR is triggered only based on the triggering threshold and is not impacted by the presence of multiple reporting thresholds.

Proposal 12. 	Enhanced DSR MAC CE does not need to include any delay information related to PDU Set Importance.

Proposal 13. 	If PSI-based SDU discard has been activated, PDU sets with low importance do not trigger DSR and are not reported in DSR MAC CE.