3GPP TSG-RAN2 Meeting #127	R2-2406261

# Maastricht, Netherlands, 19th August 2024 - 23rd August 2024

Agenda Item:	7.25.1

Source:	OPPO

Title:	Discussion on LS R4-2410693

Document for:	Discussion, Decision

- Introduction

This is to discuss left issue from R4 LS.

- Discussion

In this LS, R4 claimed that

With the UE capability, it is noticed that in the field, some UEs indicating ‘non-contiguous’ via intraBandENDC-Support cannot access to the NW configuring intra-band contiguous EN-DC. The UE can only access to single carrier rather than intra-band EN-DC, which degrades the UE performance.

I.e., the problem is

1) There is indeed UE in the field only supports non-contiguous but not support contiguous configuration

2) There is network in the field that does not based on the UE capability but directly configure contiguous case

Considering the current capability has supported all three cases

- Contiguous case only: the field is absent

- Non-contiguous case only: the field is present, and = non-contiguous

- Both cases are support: this fields is present, and = both

The current field can cover all possible UE implementation (i.e., including the UE supporting both cases), so there seems no need to introduce a new UE capability.

Furthermore, it is worth to note that this issue is about UEs and network in the fields, a new capability cannot solve the issue in the field (if any) anyway.

- R2 reply R4 that the current UE capability can already support the UE supporting both contiguous and non-contiguous cases, so no need to introduce new UE capability.

- Conclusion

Proposal 1	R2 reply R4 that the current UE capability can already support the UE supporting both contiguous and non-contiguous cases, so no need to introduce new UE capability.