3GPP TSG-RAN2 Meeting #127												R2-2406264

# Maastricht,  Netherlands, 19th August 2024 - 23rd August 2024

Agenda Item:	7.15.1

Source:	OPPO

Title:	Discussion on LS R1-2405530

Document for:	Discussion, Decision

- Introduction

This is to discuss left issue from R1 LS.

- Discussion

During 125bis, R2 agreed that

Proposal 1: From R2 perspective, UE is not expected to be (pre)configured with 1) both partial sensing and Co-Ex in the same resource pool, 2) both random-selection and Co-Ex in the same resource pool in Rel-18.

Proposal 2: From R2 perspective, UE is not expected to be (pre)configured to perform partial sensing operation over an unlicensed spectrum using interlace RB based transmission, in Rel-18.

Proposal 3: From R2 perspective, UE is not expected to be (pre)configured with a LTE/NR-SL co-existence resource pool over an unlicensed spectrum, in Rel-18.

- Agree with all proposals. However, we don’t need to capture them in the spec. We can leave them into NW implementation. We’ll send a LS to RAN1 to inform and check if they have a concern.

For P1, R1 replied that

Regarding the 2nd part of the first agreement on UE is not expected to be (pre)configured with both random-selection and Co-Ex in the same resource pool, in Rel-18, …

RAN1 response: It is RAN1 understanding that the RAN2’s agreement on random-selection and Co-Ex are not expected to be co-configured in the same resource pool applies only to NR SL normal resource pool. That is, this agreement does not apply to the NR SL exceptional pool.

Therefore, R2 spec TS 38.321 clause 5.22.1.1 needs to be revised, e.g.,

4>	if transmission based on random selection is configured by upper layers:

5>	if the selected resource pool is not SL-PRS dedicated resource pool:

6>	randomly select the time and frequency resources for one transmission opportunity from the resource pool which occur within the SL DRX Active time, if configured, as specified in clause 5.28.2 of the destination UE selected for indicating to the physical layer the SL DRX Active time above, and the pool(s) in which all RB sets had Sidelink consistent LBT failure detected and not cancelled and the resources of which the lowest sub-channel includes intra cell guard band PRBs if sl-transmissionStructureForPSCCHandPSSCH is set to 'contiguousRB' are excluded, if configured, according to the amount of selected frequency resources, the remaining PDB of SL data available in the logical channel(s), and the remaining SL-PRS delay budget of the SL-PRS transmission(s), if available, allowed on the carrier.

And also clause 5.22.1.2b needs to be revised, e.g.,

If the MAC entity has been configured with Sidelink resource allocation mode 2 or Sidelink resource allocation scheme 2 to transmit using pool(s) of resources in a carrier as indicated in TS 38.331 [5] based on full sensing, or partial sensing or random selection or any combination(s), the MAC entity shall for each Sidelink process:

1>	if sl-interUECoordinationScheme2 enabling reception/transmission of a resource conflict indication is configured by RRC; and

1>	if the next resource of the selected sidelink grant which has been indicated by a prior SCI is overlapped with conflict resource(s) indicated by the physical layer as specified in clause 16.3.1 of TS 38.213 [6]:

2>	remove the resource from the selected sidelink grant associated to the Sidelink process;

2>	randomly select the time and frequency resource from the resources indicated by the physical layer as specified in clause 8.1.4 of TS 38.214 [7] excluding the conflict resource(s) for the removed resource, according to the amount of selected frequency resources, the selected number of HARQ retransmissions and the remaining PDB of either SL data available in the logical channel(s), and the remaining SL-PRS delay budget for SL-PRS transmission, if available, by ensuring the minimum time gap between any two selected resources of the selected sidelink grant in case that PSFCH is configured for this pool of resources, and that a resource can be indicated by the time resource assignment of an SCI for a retransmission according to clause 8.3.1.1 of TS 38.212 [9];Please note that by this change, clearly both MAC and RRC spec supports configuring random-selection + Co-Ex in the same normal pool, R2 needs to discuss whether

- Allow configuration of random-selection + Co-Ex in the same normal pool (for which, some companies at R2 had the view that it will lead to performance loss in the normal pool, especially it will cause the resource collision with LTE SL UE in normal RP)

- Disallow this configuration, but it has to be visible somewhere in the specification.

For P2, R1 view is

Regarding the second agreement on UE is not expected to be (pre)configured to perform partial sensing operation over an unlicensed spectrum using interlace RB based transmission, in Rel-18, …

RAN1 response: RAN1 would like to inform that the CR in R1-2405528 (attached) to support partial sensing operation over an unlicensed spectrum using interlace RB based transmission has been agreed for the Mode 2 resource selection procedure in TS 38.214 Section 8.1.4.

There seems no R2 spec change due to this.

- R2 revise MAC spec (e.g., clause 5.22.1.1/2b) to reflect R1 view that the co-configuration of random-selection and Co-Ex in the same exceptional RP.

- R2 discuss whether to allow the co-configuration of random-selection and Co-Ex in the same normal RP. And if not, capture it in the specification.

- Conclusion

Proposal 1	R2 revise MAC spec (e.g., clause 5.22.1.1/2b) to reflect R1 view that the co-configuration of random-selection and Co-Ex in the same exceptional RP.

Proposal 2	R2 discuss whether to allow the co-configuration of random-selection and Co-Ex in the same normal RP. And if not, capture it in the specification.