3GPP TSG-RAN2 Meeting #127	R2-2406266

# Maastricht, Netherlands, 19th August 2024 - 23rd August 2024

Agenda Item:	8.5.2

Source:	OPPO

Title:	Discussion on On-Demand SSB

Document for:	Discussion, Decision

- Introduction

This is to discuss Objective-1 of R19 NES.

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4]

  - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling)

  - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum.

- Discussion

Based on latest R1 conclusion

Agreement

- For a cell supporting on-demand SSB SCell operation,

  - Support RRC based signaling to indicate on-demand SSB transmission on the cell.

  - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell.

  - FFS: Whether to support DCI based signaling to indicate on-demand SSB transmission on the cell.

    - This DCI signaling does not provide SCell activation/deactivation.

    - If supported, details on DCI including UE-specific or group-common DCI, DCI contents, etc.

  - FFS: Scenarios where the above signalings are applicable

I.e., although the applicable scenario is still FFS, R1 has concluded the support of RRC and MAC-CE based signaling.

Consider MAC CE can be used to dynamically (de)activate the OD-SSB, the usage of RRC approach seems more for the initial (de)activation status setting, as we adopted for the duplication, and DTX/DRX setting.

- RRC based signaling is used to set the initial (de)activation status of OD-SSB, and MAC-CE is used (on top of the initial status set by RRC) to dynamically (de)activate the OD-SSB.

Firstly, for RRC signaling, R1’s progress is as follows

Agreement

- For a cell supporting on-demand SSB SCell operation, at least the following for on-demand SSB via higher layer RRC signaling is supported.

  - Frequency of the on-demand SSB

  - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst

  - Periodicity of the on-demand SSB

  - FFS: Whether more than one on-demand SSB configurations can be configured for the cell to UE

  - FFS: Whether the RRC is newly introduced or existing RRC is reused

Agreement

- For a cell supporting on-demand SSB SCell operation, at least the followings for on-demand SSB are known to UE.

  - Sub-carrier spacing of the on-demand SSB

  - Physical Cell ID of the on-demand SSB

  - Location of on-demand SSB burst

  - Downlink transmit power of on-demand SSB

  - FFS: Other parameters

  - FFS: Whether each of above parameters is configured/indicated explicitly or not

Agreement

For SSB burst(s) indicated by on-demand SSB SCell operation via MAC CE, UE expects that on-demand SSB burst(s) is transmitted from time instance A which is determined as follows.

- Alt 3-1: Time instance A is [the slot boundary of] the first SSB time domain position [of actually transmitted on-demand SSB burst] which is T [slots or symbols] after the [slot or symbol] where UE receives a signalling from gNB to indicate on-demand SSB transmission

  - The SSB time domain positions of on-demand SSB burst are configured by gNB.

- FFS: Details of the value of T (≥ 0) including possibility of T comprising of multiple components

- Note: The value of T is not less than existing timeline required for UE’s MAC CE processing for SCell activation

- FFS: Whether the value of T is predefined or indicated/configured by gNB

Although it is obvious the existing fields are sufficient to configure the OD-SSB (except T), considering the other issues which are still FFS and also it is related to case#1/2 (in case#2, the existing configuration may be used for the always-on SSB rather than the OD-SSB), R2 can only wait for further R1 conclusion.

ServingCellConfigCommon ::=         SEQUENCE {

physCellId                          PhysCellId                                                          OPTIONAL,   -- Cond HOAndServCellAdd,

downlinkConfigCommon                DownlinkConfigCommon                                                OPTIONAL,   -- Cond HOAndServCellAdd

[…]

ssb-PositionsInBurst                CHOICE {

shortBitmap                         BIT STRING (SIZE (4)),

mediumBitmap                        BIT STRING (SIZE (8)),

longBitmap                          BIT STRING (SIZE (64))

}                                                                                                       OPTIONAL, -- Cond AbsFreqSSB

ssb-periodicityServingCell          ENUMERATED { ms5, ms10, ms20, ms40, ms80, ms160, spare2, spare1 }   OPTIONAL, -- Need S

[…]

ssbSubcarrierSpacing                SubcarrierSpacing                                                   OPTIONAL, -- Cond HOAndServCellWithSSB

tdd-UL-DL-ConfigurationCommon       TDD-UL-DL-ConfigCommon                                              OPTIONAL, -- Cond TDD

ss-PBCH-BlockPower                  INTEGER (-60..50),

[…]

}

FrequencyInfoDL ::=                 SEQUENCE {

absoluteFrequencySSB                ARFCN-ValueNR                                                   OPTIONAL,   -- Cond SpCellAdd

frequencyBandList                   MultiFrequencyBandListNR,

absoluteFrequencyPointA             ARFCN-ValueNR,

scs-SpecificCarrierList             SEQUENCE (SIZE (1..maxSCSs)) OF SCS-SpecificCarrier,

...,

[[

referenceCell-r18                   ServCellIndex                                                   OPTIONAL    -- Cond SSBlessSCell

]]

}

- R2 wait for further R1 conclusion before deciding whether to rely on legacy fields for OD-SSB configuration.

For the solution, R1 reached the following conclusion.

Agreement

For a cell supporting on-demand SSB SCell operation, further study the following options.

- Option 1: Separate signaling between legacy/existing signaling (e.g., RRC, MAC CE) providing SCell activation/deactivation and signaling providing On-demand SSB transmission indication.

- Option 2: A single signaling in which both SCell activation/deactivation and On-demand SSB transmission indication are provided.

  - FFS: Details of the signaling

- Other options are not precluded.

- FFS: Details on On-demand SSB transmission indication

Firstly, for Scenario-2A, i.e., OD-SSB indication upon SCell activation/deactivation, there seems no need to design an additional MAC-CE but can reuse the legacy SCell A/D MAC-CE. And in order for UE to know that the OD-SSB would be triggered together with SCell A/D, RRC configuration can be used to indicate that in advance. And how RRC configuration indicate that depends on the further down-selection between the following option-1/1A/2/3/4.

Secondly, for Scenario-2, a separate MAC-CE specifically for OD-SSB (de)activation is necessary anyway.  So the problem is whether to apply the separate MAC-CE to Scenario-2A as well.

- If apply the separate MAC-CE to Scenario-2A as well, network needs to send two MAC-CEs (SCell activation and OD-SSB activation) together. In case there is no DL grant big enough to carry the two MAC-CEs, the OD-SSB activation would be delayed

- If not apply the separate MAC-CE to Scenario-2A, at least there would be a problem for OD-SSB deactivation. Since one may not want to keep the OD-SSB activated till SCell being deactivation.

So it seems more reasonable to rely on legacy SCell activation MAC-CE to activation OD-SSB (whether it can be used is up to RRC configuration), and rely on the separated OD-SSB MAC-CE for the OD-SSB (de)activation after SCell being activated.

- The OD-SSB (de)activation MAC-CE applies at least for OD-SSB Scenario#2 (i.e., OD-SSB (de)activation after SCell activation).

- For OD-SSB scenario#2A (i.e., OD-SSB (de)activation upon SCell activation), rely on legacy SCell Activation MAC-CE as indication of both SCell activation and OD-SSB activation, which can be enabled by RRC configuration.

During R1#116bis meeting, it was concluded that

Agreement

- For a cell supporting on-demand SSB SCell operation,

  - L1 and/or L3 measurement based on on-demand SSB is supported for the cell.

    - FFS further details on L1 and/or L3 measurement

And further at R1#117

Agreement

- At least support L1 measurement based on on-demand SSB

  - For L1 measurement based on on-demand SSB, periodic, semi-persistent, [and aperiodic] L1 measurement reports based on existing CSI framework are supported.

    - FFS on potential enhancements of CSI report configuration and/or triggering/activation mechanisms for L1 measurement based on on-demand SSB

I.e., R1 only concluded on the support of L1 measurement based on OD-SSB, but not for the support of L3 measurement based on the OD-SSB yet.

For the potential R2 impact due to the L3 measurement, e.g., the usage of one additional SMTC window (besides the one for always-on SSB), the motivation is not clear, since

- SMTC window was used to avoid UE measurement / searching across all time domain resources, so that by limiting the measurement into a time window, it helps UE power saving. But there is no such concern for OD-SSB, since the time location of OD-SSB would be indicated by DL signal (e.g., MAC-CE) explicitly. Furthermore, note that given the OD-SSB was introduced for power saving reason, it would be contradictory if OD-SSB was abused as long-term SSB transmission.

- The usage of SMTC window was feasible considering legacy always-on SSB is periodical, and thus a time window, which is aligned with the SSB time domain pattern, can be configured. But OD-SSB transmission is occasional, it is hard to predict the time domain pattern and thus decide on the SMTC configuration. Even if one believes it can be feasible, one can make use of the legacy SMTC window (i.e., to cover both always-on SSB and OD-SSB)

- R2 wait for further R1 conclusion before discussing any related issue for L3 measurement based on OD-SSB.

Based on the latest R1 conclusion

Agreement

For the identified scenarios and cases (as per RAN1#116 agreement), on-demand SSB can be triggered by gNB at least for the following scenarios/cases:

- Scenario #2 and Case #1

- Scenario #2 and Case #2

- Scenario #2A and Case #1

- Scenario #2A and Case #2

- FFS: Scenario #3A and Case #1

- FFS: Scenario #3A and Case #2

- FFS: Scenario #3B and Case #1

- FFS: Scenario #3B and Case #2

I.e., R1 has concluded to support OD (On-Demand) SSB before and upon SCell activation but leave the OD-SSB after SCell activation (completion) to be FFS. RAN2 can leave the decision to R1 for this issue.

- For on-demand SSB SCell operation, R2 waits for R1 decision for scenario-3A/B (for case-1/2).

Another issue is CD-SSB vs NCD-SSB.

Agreement

- For a cell supporting on-demand SSB SCell operation,

  - Note: It is up to gNB implementation whether always-on SSB (if transmitted) on the cell is cell-defining SSB or not.

  - For on-demand SSB on the cell, downselect between the following alternatives

    - Alt-1: It is up to gNB implementation whether on-demand SSB is cell-defining SSB or not.

    - Alt-2: On-demand SSB is limited to non-cell-defining SSB.

      - FFS: Further limitations to on-demand SSB

Considering objective-1 is limited to SCell case, if extended to CD-SSB, it may lead to issues for either RRC_IDLE/RRC_INACTIVE or PCell case, i.e., out of WID scope. So, before R1 extend the scenario further, it is safer for R2 to focus on NCD-SSB.

- For on-demand SSB SCell operation, R2 focus on NCD-SSB before further R1 conclusion.

The other issue is about UL/DL triggering.

For non-collocated scenario, there would be different levels/types of backhaul connection, e.g.,

Table 1 Categorization of Backhaul Latency

| Backhaul Technology | Latency (One way) | Throughput |
| --- | --- | --- |
| Fiber Access 1 | 10-30ms | 10M-10Gbps |
| Fiber Access 2 | 5-10ms | 100-1000Mbps |
| Fiber Access 3 | 2-5ms | 50M-10Gbps |
| Fiber Access 4 (NOTE 1) | less than 2.5 us (NOTE2) | Up to 10Gbps |
| DSL Access | 15-60ms | 10-100 Mbps |
| Cable | 25-35ms | 10-100 Mbps |
| Wireless Backhaul | 5-35ms | 10Mbps – 100Mbps typical, maybe up to Gbps range |

Apparently, the latency due to non-ideal backhaul would alleviate the gain from on-demand SSB, i.e., it would slow down the overall procedure of SCell activation, and also would waste SSB transmission (the SSBs from SCell being aware of it, till UE being notified by PCell).

On the contrary, a UL-based triggering would be helpful to adapt with this kind of backhaul.

- For on-demand SSB SCell operation, R2 confirm to include the support of UL based triggering.

Conclusion

Proposal 1	RRC based signaling is used to set the initial (de)activation status of OD-SSB, and MAC-CE is used (on top of the initial status set by RRC) to dynamically (de)activate the OD-SSB.

Proposal 2	R2 wait for further R1 conclusion before deciding whether to rely on legacy fields for OD-SSB configuration.

Proposal 3	The OD-SSB (de)activation MAC-CE applies at least for OD-SSB Scenario#2 (i.e., OD-SSB (de)activation after SCell activation).

Proposal 4	For OD-SSB scenario#2A (i.e., OD-SSB (de)activation upon SCell activation), rely on legacy SCell Activation MAC-CE as indication of both SCell activation and OD-SSB activation, which can be enabled by RRC configuration.

Proposal 5	R2 wait for further R1 conclusion before discussing any related issue for L3 measurement based on OD-SSB.

Proposal 6	For on-demand SSB SCell operation, R2 waits for R1 decision for scenario-3A/B (for case-1/2).

Proposal 7	For on-demand SSB SCell operation, R2 focus on NCD-SSB before further R1 conclusion.

Proposal 8	For on-demand SSB SCell operation, R2 confirm to include the support of UL based triggering.