3GPP TSG-RAN WG2 Meeting #127                                                      R2-2406267

Maastricht, Netherlands, August 19th – 23th, 2024

Source:	CAICT

Title:	Discussion on MBS service in NTN system

Agenda Item:	8.8.4

Document for:	Discussion and Decision

Introduction

As outlined in [1], the WID for Phase 3 of NR NTN includes an aim to enhance the MBS service user experience through several approaches:

Specify signaling of the intended service area of a broadcast service (e.g. MBS broadcast) via NR NTN [RAN2, RAN3]

- Specify SIB signaling to indicate the intended service area in case the satellite footprint covers a larger area. [RAN2]

- Specify the necessary signaling between CN and NG-RAN. [RAN3]

In this document, we aim to elaborate on our perspectives regarding the feasibility of limiting broadcast transmission strictly to designated service areas. This discussion builds upon the agreements reached in the previous RAN2 meeting and presents our observations and proposals on the matter.

Discussion

Support of Diverse MBS Services/Contents in Large-Area Cells

It remains an open issue whether broadcast transmission can be confined solely to the intended service area. If this is feasible, different services might be needed for various sub-areas within a single cell. Distinct MBS sessions should be associated with specific geographical areas within a cell. This information should be embedded in the System Information Block (SIB) to help User Equipment (UE) identify available MBS services and their broadcasting areas within the cell it is connected to.

Proposal 1: RAN2 agrees to associate different MBS sessions with distinct geographical areas, indicating this in the SIB.

an MBS area comprises a list of cells or TAIs. Each cell only distributes one specific content from a given MBS session. Therefore, it is reasonable for the MBS configuration to exclude the area session ID information. In scenarios where a cell covers a vast area, it is possible to distribute multiple content pieces for the same MBS session across different parts of the cell. Each content’s area session ID and its associated geographical range should be specified in the MBS configuration information.

Proposal 2: For location-dependent broadcast services,  RAN2 agrees to include the area session ID of each distinct MBS session content and its associated area range in the MBS configuration information.

Support of MBS Services/Contents Across Multiple NTN Cells

Based on our understanding, there are two scenarios when MBS services or content are accessible in multiple NTN cells:

When MBS services are available across multi-cells.

MBS services are available in at least one complete cell and only parts of other cells.

When MBS services cover entire cells, there is no need to specify the targeted broadcast area to the User Equipment (UE) within those cells, as they can receive the services throughout the entire cell. Conversely, if the services are only available in certain parts of a cell, additional enhancements should be considered, as noted in the previous section. We should consider both scenarios, where MBS services are entirely available in one cell or partially in another, by making any enhancements for indicating targeted MBS service areas within NTN cells optional.

Proposal 3: it is suggested that RAN2 support MBS services/contents in cells area of more than one NTN cells.

Conclusion

According to the discussion, we get below proposals:

Proposal 1: RAN2 agrees to associate different MBS sessions with distinct geographical areas, indicating this in the SIB.

Proposal 2: For location-dependent broadcast services,  RAN2 agrees to include the area session ID of each distinct MBS session content and its associated area range in the MBS configuration information.

Proposal 3: it is suggested that RAN2 support MBS services/contents in cells area of more than one NTN cells.

Reference

[1] RP-234078, Non-Terrestrial Networks (NTN) for NR Phase 3, 3GPP RAN#102.

[2] TS 38.300 V18.0.0, NR; NR and NG-RAN Overall Description, Stage 2, RAN

[3] TS 23.247, V18.4.0, Architectural enhancements for 5G multicast-broadcast services, Stage 2, SA.