3GPP TSG-RAN WG2 Meeting #127                                                    R2-2406268

Maastricht, Netherlands, August 19th – 23th, 2024

Source:	CAICT

Title:	Discussion on NTN regenerative payload

Agenda Item:	8.8.5

Document for:	Discussion and Decision

Introduction

For support of regenerative payload topic, the WID content is as follows:

Support of regenerative payload [RAN3, RAN2, RAN4] - Specify the support of gNB on board in TS 38.300 - Specify, if needed, any necessary enhancements related to the intra and inter-gNB mobility, especially for Xn interface over feeder link or over ISL. [RAN3] - Note: if any additional necessary stage-3 specifications impact for e.g. NGAP is identified, RAN3 will handle it.

In this contribution, we will discuss the potential issues on support of regenerative payload topic, our views will be given as well.

Discussion

For regenerative payload architecture, cross-satellite synchronization allows gNBs on different satellites to maintain synchronization without ground network intervention. This feature is critical to ensure seamless communication and reduce latency during satellite switchovers. It also minimizes the risk of data loss and ensures continuity of service.  Implementing cross-satellite synchronization requires precise timing and coordination mechanisms. This can be achieved through advanced inter-satellite links (ISLs) that provide low-latency, high-reliability communication channels. The synchronization mechanism ensures that both old and new satellites can simultaneously support active connections without disruption. The cross-satellite synchronization feature may be supported in regenerative payload architecture by implementing advanced ISL technologies to facilitate real-time data transfer and synchronization.

Proposal 1：It is recommended to further investigate and develop cross-satellite synchronization mechanisms to enhance the performance of regenerative payload architecture.

In the context of regenerative payload, power optimization for User Equipment (UE) is crucial. The RA-SDT procedure can be adjusted to minimize power consumption by delaying PDCCH monitoring for further RRC messages after sending HARQ feedback. This allows UEs to enter low-power states, reducing unnecessary energy expenditure while maintaining robust communication.

Proposal 2：For power optimization in RA-SDT procedure, delay PDCCH monitoring for further RRC messages to allow UEs to enter low-power states after sending HARQ feedback.

In regenerative payload scenarios, frequent gNB switches can impact network performance, especially in dynamic environments with moving cells. This can lead to increased latency and potential service disruptions. A comprehensive study on the impact of frequent gNB switches is necessary to develop mitigation strategies and ensure stable network performance.

Proposal 3：Conduct a study to assess the impact of frequent gNB switches on network performance and develop strategies to mitigate any negative effects.

Conclusion

According to the discussion, we get below proposals:

Proposal 1：It is recommended to further investigate and develop cross-satellite synchronization mechanisms to enhance the performance of regenerative payload architecture.

Proposal 2：For power optimization in RA-SDT procedure, delay PDCCH monitoring for further RRC messages to allow UEs to enter low-power states after sending HARQ feedback.

Proposal 3：Conduct a study to assess the impact of frequent gNB switches on network performance and develop strategies to mitigate any negative effects.

Reference

[1] RP-240775, “Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3”, March 18-22, 2024.

[2] TS 38.300 V18.0.0, NR; NR and NG-RAN Overall Description, Stage 2, RAN