3GPP TSG-RAN WG2 Meeting #127	R2-2406269

Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda Item:	8.7.4

Source: 	OPPO

Title:  	Discussion on scheduling enhancements for XR

Document for:	Discussion and Decision

# Introduction

In Rel-19 XR WID [1], the detailed objectives on enhancement for Scheduling are captured as follows.

- Specify Enhancements for Scheduling, as follows: - For the UL, Study and if justified, Specify enhancements using delay/deadline information, for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs. [RAN2]. - Note: LCP implementation complexity need to be taken into account when evaluating solutions. - Note: Check in RAN#105

This contribution would like to provide our considerations on delay-aware enhancement for scheduling, including LCP and DSR.

# Discussion

# Delay-aware LCP enhancement

In RAN2#125bis, RAN2 had a initial discussion on delay-aware LCP enhancement and the following agreements were made, where two alternatives to enable priority transmission of delay-critical data are listed in the agreement for further study.

RAN2 will study whether/how to resolve the issue of data with low remaining time being delayed due to other data from LCHs with higher LCH priority when using the existing LCP procedure. At least the following alternatives will be studied: - Alternative 1: Enhance LCP restrictions/LCH selection. - Alternative 2: Enhance LCH prioritization. RAN2 should consider potential impact on traffic from SRBs.

In RAN2#126, as shown in the following agreements, it is confirmed that delay-aware LCP enhancement is supported to resolve the issue of data with low remaining time being delayed due to data from other LCHs with no delay critical data. Regarding two alternatives, i.e., LCH prioritization and LCH restrictions, some further conclusions were made.

LCP enhancements – LCH prioritization Delay-aware LCP enhancement to resolve the issue of data with low remaining time being delayed due to data from other LCHs with no delay critical data is supported in Rel-19 XR. The solution should consider impact on UE complexity (as already indicated in SI objective description) For delay-aware LCP enhancement, RAN2 considers the following option to override/adjust the priority of LCH based on delay/deadline information as a baseline: - - Use additional priority configured to LCHs in case of these LCHs with delay-critical data FFS whether the priority only applies to delay-critical data within the LCH or for the whole LCH LCP enhancements – LCH restrictions We try to avoid RAN1 impacts. RAN2 assumes no dynamic indications are needed for triggering the delay-aware LCP mechanism. RAN2 assumes this mechanism is configured in a semi-static way. For LCP restrictions based solutions, RAN2 will not discuss solutions requiring RAN1 work. FFS whether other LCP restrictions based approaches are needed/beneficial The solutions should not disallow non-delay critical data from using an UL grant. LCP enhancements – granularity LCP prioritization within a logical channel will not be considered in RAN2 discussions FFS whether a separate remaining time threshold can be configured for delay aware LCP (i.e. different from the one used for DSR).

Here we further discuss the leftover issues on two alternatives as highlighted above.

For LCH prioritization-based LCP enhancement, it is agreed to use additional priority configured to LCHs in case of these LCHs with delay-critical data, but it is FFS whether the priority only applies to delay-critical data within the LCH or for the whole LCH. In our understanding, RAN2 has already made conclusion to not consider LCP prioritization within a logical channel in the last meeting, therefore it is straightforward that the priority should apply to the whole LCH. It is still good to further confirm this.

- For LCH prioritization-based LCP enhancement, RAN2 to confirm that the additional priority applies for the whole LCH with delay-critical data, i.e., not only for delay-critical data within LCH.

For LCH restriction-based LCP enhancement, the solution requiring RAN1 work is excluded, and RAN2 solution should not disallow non-delay critical data from using an UL grant. The candidate solutions are narrowed down into a small range, but it is still open to consider other approaches. For example, for LCHs with delay-critical data which cannot be selected for the UL grant due to not fulfil the legacy LCP restrictions, we may enhance to break or relax the current LCP restrictions to allow them to be selected and transmitted. However, in our understanding, network may know whether there is delay-critical data in LCHs, e.g., based on delay-aware enhancement for DSR, therefore network could schedule a UL grant by appropriate implementation to avoid LCHs with delay-critical data to be excluded. Besides, considering we have already made conclusions on LCH prioritization-based solution, so it is not necessary to have LCP restriction-based solution too unless its benefit could be justified clearly.

- Based on delay-aware enhancement for DSR, network could schedule a UL grant by appropriate implementation to avoid LCHs with delay-critical data to be excluded due to LCP restriction.

- RAN2 not to further consider LCP restriction-based solution.

Another leftover issue is that whether a separate remaining time threshold different from the one used for DSR can be configured for delay aware LCP enhancement. In our understanding, a separate threshold has benefit on decoupling DSR and delay-aware LCP enhancement, which could provide more flexibility to NW configuration. Thus, separate threshold is slightly preferred.

- Separate remaining time threshold is introduced for delay-aware LCP enhancement (i.e., different from the one used for DSR).

# Delay-aware enhancement for DSR

In 5G NR, NW can configure the DSR mechanism for UE. When the UE meets the DSR reporting conditions, it will report delay status information of LCGs to serving gNB, which includes remaining time, i.e., the smallest remaining value of the PDCP discardTimers among SDUs buffered for the LCG, and the total amount of delay-critical UL data for the LCG. The reporting of the DSR information can help the NW understand the remaining time of the data to assist the serving gNB in scheduling. Whether to trigger DSR is determined based on whether the remaining value of the PDCP discard timer is less than the threshold.

In RAN2#125bis and RAN2#126, the following agreement was made on the enhancement of existing DSR with additional information.

RAN2 will study enhancing existing DSR with additional information, e.g. multiple pairs of remaining time/buffer information, importance. FFS whether this only includes more information on delay-critical data or also information about non-delay critical data.

DSR enhancements – multiple thresholds and grouping of data Enhance DSR to report with multiple pairs of remaining time and buffer size for the LCG. FFS whether DSR triggering is impacted FFS whether PDU set importance needs to be included

Additional information on delay-critical data

For the information on delay-critical data, it is agreed to enhance DSR to report with multiple pairs of remaining time and buffer size for the LCG. It is FFS on whether DSR triggering is impacted. Besides, whether PDU set importance needs to be included is also FFS.

Regarding the DSR triggering, it is proposed by companies that multiple DSR reporting thresholds could be configured for the LCG. In our understanding, it is not necessary to configure multiple DSR reporting thresholds. Even based on only one existing DSR triggering, it also supports the UE to report finer information, i.e., more than one pair of remaining time and buffer size. From UE implementation perspective, multiple DSR triggering would introduce additional consumption.

- For enhancement of DSR to report with multiple pairs of remaining time and buffer size for the LCG, not introduce multiple thresholds for DSR triggering, i.e., rely on existing DSR triggering.

Regarding the PDU set importance (PSI), it is proposed by companies to differentiate importance level of delay-critical data based on PSI in DSR MAC CE to enable that network can schedule for the important PDU sets firstly. In our understanding, PSI is only used for congestion, i.e., network allows to discard PDU sets with low importance according to PSI in case of congestion. PSI only differentiates the importance level of PDU sets from congestion perspective, and it does not mean the PDU sets with different PSI have different delay requirements for scheduling. Therefore, it is not necessary to include PSI-based importance information in DSR.

- PSI only differentiates the importance level of PDU sets from congestion perspective rather than from scheduling perspective.

- No need to include importance information in DSR.

Information about non-delay-critical data

For the information about non-delay critical data, whether to include it in DSR or not is still FFS. In the legacy DSR mechanism, the requirement of priority transmission for delay-critical data is not fully met. In detail, the existing DSR reporting is the total amount of delay-critical data for the LCG. In some cases, there may be data of LCH1 and LCH2 in an LCG. The priority of LCH1 is higher than the priority of LCH2, but the data in LCH1 is not delay-critical. In this way, If network schedules the uplink resource based on DSR, UE would prioritize to transmit the data of LCH1 instead of the potential delay-critical data in LCH2, resulting in the delay-critical data in LCH2 not being transmitted in time. To avoid this issue, besides the information on delay-critical data reported by legacy DSR procedure, gNB needs to know more comprehensive information about non-delay critical data at the same time, so as to schedule uplink resources for delay-critical data better. Thus, for enhancing existing DSR with additional information, we suggest RAN2 to consider including both information about delay-critical data and non-delay critical data.

- Besides the information on delay-critical data reported based on DSR, serving gNB needs to know more comprehensive information about non-delay critical data at the same time, so as to schedule uplink resources for delay-critical data better.

- The information of both delay-critical data and non-delay critical data is included in the enhanced DSR.

Regarding how to provide the information of non-delay critical data, the most simple way is to enhance DSR MAC CE to indicate the presence of non-delay critical data for LCGs. One bit per LCG is sufficient for this way. If going to include more information, we may consider enhancing DSR MAC CE to capture the data volume of non-delay critical data, which provides more detailed information for serving gNB.

- For the information of non-delay critical data, RAN2 discusses the following options:

- Option 1: Enhanced DSR MAC CE to indicate the presence/absence of non-delay-critical data

- Option 2: Enhanced DSR MAC CE to indicate the data volume of non-delay-critical data

# 3. Conclusion

Based on the discussion above, we made the following observations and proposals:

Delay-aware LCP enhancement

- For LCH prioritization-based LCP enhancement, RAN2 to confirm that the additional priority applies for the whole LCH with delay-critical data, i.e., not only for delay-critical data within LCH.

- Based on delay-aware enhancement for DSR, network could schedule a UL grant by appropriate implementation to avoid LCHs with delay-critical data to be excluded due to LCP restriction.

- RAN2 not to further consider LCP restriction-based solution.

- Separate remaining time threshold is introduced for delay-aware LCP enhancement (i.e., different from the one used for DSR).

Delay-aware DSR enhancement: Additional information on delay-critical data

- For enhancement of DSR to report with multiple pairs of remaining time and buffer size for the LCG, not introduce multiple thresholds for DSR triggering, i.e., rely on existing DSR triggering.

- PSI only differentiates the importance level of PDU sets from congestion perspective rather than from scheduling perspective.

- No need to include importance information in DSR.

Delay-aware DSR enhancement: Information about non-delay-critical data

- Besides the information on delay-critical data reported based on DSR, serving gNB needs to know more comprehensive information about non-delay critical data at the same time, so as to schedule uplink resources for delay-critical data better.

- The information of both delay-critical data and non-delay critical data is included in the enhanced DSR.

- For the information of non-delay critical data, RAN2 discusses the following options:

- Option 1: Enhanced DSR MAC CE to indicate the presence/absence of non-delay-critical data

- Option 2: Enhanced DSR MAC CE to indicate the data volume of non-delay-critical data

# 4. Reference

[1] RP-234057, New WID: XR (eXtended Reality) for NR Phase 3, MediaTek