3GPP TSG-RAN WG2 Meeting #127								R2-2406270

Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda Item:	8.5.4

Source: 	OPPO, Samsung, ZTE, Huawei, HiSilicon, Qualcomm

Title:  	Discussion on PO Confinement Options

Document for:	Discussion and Decision

# Introduction

In R2#125bis meeting, the following agreements on paging adaptation were made, where two PO confinement options were put on the table for further discussion.

For adaptation of paging occasions in time domain, RAN2 to study a) bundle paging frames and b) extend the values of N to have increased interval between PFs (e.g. T/64, T/128 ...) and compensating decrease in number of PFs by increasing POs per PF.

This contribution would like to only focus on the discussion of these two PO confinement options.

# Discussion

In legacy, the PF and PO for paging are determined by the following formulae:

SFN for the PF is determined by: (SFN + PF_offset) mod T = (T div N)*(UE_ID mod N), Index (i_s), indicating the index of the PO is determined by: i_s = floor (UE_ID/N) mod Ns,

where T denotes DRX cycle of the UE, N denotes the number of total paging frames in T, Ns denotes the number of paging occasions for a PF, PF_offset is the offset used for PF determination, and UE_ID is derived based on 5G-S-TMSI.

As stated above, in R2#125bis meeting, two PO confinement options are put on the table for further discussion, i.e.,

- Option a), revise the formula to bundle PFs,

- Option b), extend the values of N to have increased interval between PFs (e.g. T/64, T/128 ...) and compensating decrease in number of PFs by increasing Ns (i.e., POs per PF), without formula change.

Both options could confine the PF/PO in the time domain. From the spec impact perspective,

- For Option a), it would introduce larger impact since it has to modify the PF formula, where the impact is not only from specification effect perspective, but also for 5G UE and network implementation;

- While the spec impact on Option b) is limited to the adjustment of parameters N/Ns.

- Option a) has to modify the PF formula which has larger impact, not only from specification effect perspective, but also to 5G UE and network implementation.

In our understanding, there is no real difference between Option a) and Option b). The only difference between them is that for Option a) POs are distributed in multiple PFs, while for Option b) POs are associated with the only PF (but without restriction to the PO distribution in time domain). It is POs where UE actually needs to wake up on for paging reception, i.e., the distribution of PF does not make a real difference to UE reception and network transmission behaviour.

As stated in 38304, PF is used to define the logical starting point of POs, but it does not matter whether POs associated with a PF are involved in single or multiple PFs.

## 7.1 Discontinuous Reception for paging NOTE 1: A PO associated with a PF may start in the PF or after the PF. NOTE 2: The PDCCH monitoring occasions for a PO can span multiple radio frames. When SearchSpaceId other than 0 is configured for paging-SearchSpace the PDCCH monitoring occasions for a PO can span multiple periods of the paging search space.

- From paging transmission/reception operation perspective, there is no essential difference between Option a) and Option b).

Besides, for Option b), thanks to PO offset as shown below, associating multiple POs with one PF would not have impact on other channels, i.e., the offset can be used to leave some space to the scheduling of other channels.

PCCH-Config ::=             SEQUENCE {

[…]

ns                                  ENUMERATED {four, two, one},

firstPDCCH-MonitoringOccasionOfPO   CHOICE {

sCS15KHZoneT                                                                SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..139),

sCS30KHZoneT-SCS15KHZhalfT                                                  SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..279),

sCS60KHZoneT-SCS30KHZhalfT-SCS15KHZquarterT                                 SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..559),

sCS120KHZoneT-SCS60KHZhalfT-SCS30KHZquarterT-SCS15KHZoneEighthT             SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..1119),

sCS120KHZhalfT-SCS60KHZquarterT-SCS30KHZoneEighthT-SCS15KHZoneSixteenthT    SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..2239),

sCS480KHZoneT-SCS120KHZquarterT-SCS60KHZoneEighthT-SCS30KHZoneSixteenthT    SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..4479),

sCS480KHZhalfT-SCS120KHZoneEighthT-SCS60KHZoneSixteenthT                    SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..8959),

sCS480KHZquarterT-SCS120KHZoneSixteenthT                                    SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..17919)

}      OPTIONAL,           -- Need R

[…]

firstPDCCH-MonitoringOccasionOfPO-v1710  CHOICE {

sCS480KHZoneEighthT        SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..35839),

sCS480KHZoneSixteenthT     SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..71679)

}                                                                               OPTIONAL  -- Need R

}

- For Option b), confining many POs in one PF has no impact on other channels thanks to PO offset.

Therefore, based on the observations above, Option b) is preferred from our perspective, i.e., relying on legacy PF/PO determination mechanism to confine the PF/PO in the time domain based on the adjustment of parameters N/Ns.

- For adaptation of paging, Option b) is adopted, i.e., extend the values of N to have increased interval between PFs (e.g. T/64, T/128 ...) and compensating decrease in number of PFs by extending the values of Ns to increase POs per PF.

# 3. Conclusion

Based on the discussion above, we made the following observations and proposals:

- Option a) has to modify the PF formula which has larger impact, not only from specification effect perspective, but also to 5G UE and network implementation.

- From paging transmission/reception operation perspective, there is no essential difference between Option a) and Option b).

- For Option b), confining many POs in one PF has no impact on other channels thanks to PO offset.

- For adaptation of paging, Option b) is adopted, i.e., extend the values of N to have increased interval between PFs (e.g. T/64, T/128 ...) and compensating decrease in number of PFs by extending the values of Ns to increase POs per PF.