3GPP TSG-RAN WG2 Meeting #127	R2-2406277

Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda Item:	7.0.2.2

Source:	Huawei, HiSilicon, Intel Corporation

Title:	Discussion on NCR capability

Document for: Discussion

1	Introduction

RAN2 received an LS [1] from RAN4 with the feature list [2] of NCR-MT enclosed after RAN2#125bis. Specifically, the following was described in the LS:

In addition, RAN4 discussed the applicable FGs for Rel-18 NCR MT and agreed that following FGs in TR38.822 are applicable to NCR MT:

-	Rel-15 FGs: details refer to R4-2402517

-	Rel-16 and Rel-17 FGs: All of R16 and R17 features are optional for NCR-MT except for CA, DC related feature, unlicensed band and, HPUE Duty cycle, MPR related feature which is not applicable for NCR-MT.

Based on the LS, a new table (Table 4.2.23.1-3) was added in 38.306 to capture the RF/RRM capabilities of NCR-MT. However, it seems not entirely aligned with what RAN4 indicated in the feature list. In this contribution, we will analyse divide the capabilities into different categories based on the LS and propose to capture them in the specs accordingly.

2	Discussion

Since the general guidance for Rel-16 and Rel-17 FGs has already been captured properly in the spec, the discussion here mainly focuses on the Rel-15 FGs.

To make it clear for the Rel-15 FGs, an extra column was added in the feature list based on the legacy table in 38.822 to indicate whether the existing UE features are mandatory, optional or not applicable to NCR-MT. Basically the Rel-15 FGs can be categorized into the following three groups for NCR-MT:

  - FGs which are mandatory without capability signalling for NCR-MT

According to the feature list, the following FGs are mandatory without capability signalling for NCR-MT:

| Features | Index | Feature group | Components | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note | NCR-MT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1. System parameter | 1-3 | 64QAM for PUSCH | 64QAM for PUSCH | No | No | Capability can be discussed in future, e.g. when low cost device (e.g. IoT) and/or higher frequency band in FR2 are introduced | mandatory without capability signalling |
| 2. UE RF | 2-10 | Multiple frequency band indication | Multiple frequency band indication | No | No | Per UE capability | mandatory without capability signalling |

These two FGs should be kept in Table 4.2.23.1-3 of TS 38.306. Other FGs should be removed from Table 4.2.23.1-3.

Proposal 1: Remove the FGs from Table 4.2.23.1-3 of TS 38.306, except for FG1-3 and FG2-10.

  - FGs which are not applicable to NCR-MT

According to the feature list, the following FGs are not applicable for NCR-MT:

| Features | Index | Feature group | Components | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note | NCR-MT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1. System parameter | 1-9 | Support of EN-DC with LTE-NR coexistence in UL sharing from UE perspective | 1) LTE and NR UL Transmission in the shared carrier via TDM only 2) LTE and NR UL Transmission in the shared carrier via FDM only 3) LTE and NR UL transmission in the shared carrier via FDM or TDM | No | Applicable only to FR1 |  | not applicable |
|  | 1-10 | Switching time between LTE UL and NR UL for EN-DC with LTE-NR coexistence in UL sharing from UE perspective | Support of switching type between LTE UL and NR UL for EN-DC with LTE-NR coexistence in UL sharing from UE perspective. Type 1: <0.5us Type 2: <20us | No | Applicable only to FR1 | This feature is the switching time between LTE UL and NR UL in the same carrier Per band combination signalling UE Capability signalling elements. 1: <0.5us switching type. 2: <20us switching type. | not applicable |
|  | 1-11 | 7.5kHz UL raster shift | 7.5kHz UL raster shift | No | No |  | not applicable |
| 2. UE RF | 2-2 | Simultaneous reception or transmission with same or different numerologies in CA | Support of simultaneous reception or transmission with same or different numerologies in CA | No | No | From RAN4 perspective UE shall be able to signal the supported SCS per CC for each band combination Same numerology for intra-band NR CA including both continuous and non-continuous is mandatory support for Rel15 The capability of supporting SCS within the single carrier in the CA configuration will be signalled separately, i.e., there is no need to mandatory UE to support mixed numerologies in CA case If a UE supports inter-band NR CA including both FR1 band(s) and FR2 band(s), the UE shall support two mixed numerologies between FR1 band(s) and FR2 band(s) in DL and UL with capability signalling. | Not applicable |
|  | 2-3 | Non-contiguous intra-band CA frequency separation class for FR2 | 1) Support of frequency separation classes to handle the total frequency span for DL for intra-band non-contiguous CA 2) Support of frequency separation classes to handle the total frequency span for UL for intra-band non-contiguous CA | No | Applicable only to FR2 | UE signals the supported Frequency separation classes with per band granularity (Type 1) based on R4-1803363 Separate Frequency separation classes can be signalled for DL and UL | Not applicable |
|  | 2-4 | Simultaneous reception and transmission for inter-band EN-DC (TDD-TDD or TDD-FDD) | Simultaneous reception and transmission for inter-band EN-DC (TDD-TDD or TDD-FDD) | No | No | For TDD-FDD and TDD-TDD band combinations for which simultaneous RxTx capability is agreed to be supported, corresponding capability indication must be set to "supported". Band combinations for which simultaneous RxTx capability is mandatory EN-DC combinations (Both FR1 LTE – FR1 NR and FR1 LTE- FR2 NR) are captured in TS 38.101-3. | Not applicable |
|  | 2-5 | Simultaneous reception and transmission for inter band CA (TDD-TDD or TDD-FDD) | Simultaneous reception and transmission for inter band CA (TDD-TDD or TDD-FDD) | No | No | For TDD-FDD and TDD-TDD band combinations for which simultaneous RxTx capability is agreed to be supported, corresponding capability indication must be set to "supported". Band combinations for which simultaneous RxTx capability is mandatory are captured in TS 38.101-1, TS 38.101-2 and TS 38.101-3. | not applicable |
|  | 2-6 | Asynchronous FDD-FDD intra-band EN-DC DC | Asynchronous FDD-FDD intra-band EN-DC | Applicable only to FDD | Applicable only to FR1 |  | Not applicable |
|  | 2-8 | UE power class | 1) Support of FR1 UE power class 2) Support of FR2 UE power class 3) Support of FR1 UE power class for EN-DC 4) Support of FR1 UE power class for NR-CA | No | No | Capability signalling - FR1 UE power class (per band) - FR2 UE power class (per band) - FR1 UE power class for EN-DC (per band combination) - FR1 UE power class for NR CA (per band combination) Default power class for each component is indicated in TS38.101-1/2/3. If the default power class is not indicated, UE shall report supported power class. The component 2) is also used as power class for intra-band NR-CA in FR2 | not applicable |
|  | 2-9 | Simultaneous reception and transmission for SA SUL band combinations | Simultaneous reception and transmission for SA SUL band combinations | No | No |  | Not applicable |
|  | 2-11 | Modified MPR behaviour | Modified MPR behaviour | No | No | Per band capability | not applicable |
|  | 2-13 | Maximum uplink duty cycle for FR1 power class 2 UE | Maximum percentage of uplink symbols can be scheduled within a certain evaluation period provided by regulatory bodies. The value range is {60%, 70%, 80%, 90%, 100%}. If the field is absent, 50% shall be applied. | No | Applicable only to FR1 | Per band capability. If this capability is absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than 50%, or this capability is not absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than this capability, apply all requirements for the default power class. The evaluation period is up to UE implementation, no less than one radio frame. UE do not need to do UL duty cycle calculation when it's transmit power is below 23dBm and all the UL/DL configurations can be scheduled. | Not applicable |
|  | 2-15 | Maximum uplink duty cycle for FR2 | 1) Maximum percentage of uplink transmission time that can be scheduled within 1s time window in order to ensure compliance with applicable electromagnetic power density exposure requirements provided by regulatory bodies. The value range is {15%, 20%, 25%, 30%, 40%, 50%, 60%, 70%, 80%, 90%, 100%}. | No | Applicable only to FR2 | Per band capability. If the field of UE capability is present and the percentage of uplink symbols transmitted within any 1 s evaluation period is larger than this capability, the UE follows the uplink scheduling and can apply P-MPR as in TS38.101-2. If the field of UE capability is absent, the compliance to electromagnetic power density exposure requirements are ensured by means of scaling down the power density or by other means. This capability is applicable for all power classes in FR2 | Not applicable |
|  | 2-16 | PA architectures for intra-band EN-DC | Support of dual PA | No | No | Per band per band combination capability Single PA is default architecture The following requirements are involved by this capability - A-MPR/MPR and MSD values for dual uplink. Whether two sets of requirements will be introduced in RAN4 can be further discussed for each specific band combination - Switching time between LTE UL and NR UL in single switched UL operation mode for intra-band EN-DC | Not applicable |
|  | 2-17 | PA architectures for intra-band UL CA | Support of dual PA | No | No | Per band per band combination capability Single PA is default architecture The following requirements are involved by this capability - A-MPR/MPR and MSD values for dual uplink. Whether two sets of requirements will be introduced in RAN4 can be further discussed for each specific band combination | Not applicable |

According to the description in Section 4.2.23.1 of TS 38.306, “All other feature groups or components of the feature groups as captured in TR 38.822 [24] as well as capabilities specified in this specification are optional for an NCR-MT, unless indicated otherwise”. In order to avoid potential misunderstanding, the above lists FGs should be clarified in TS 38.306 to be not applicable for NCR-MT. Otherwise, they may be deemed as optional. Among them, for different capabilities, the handling should be different as follows:

- For FG1-9, FG1-10, FG2-2, FG2-3, FG2-4, FG2-5, FG2-6, FG2-9, FG2-11, FG2-13, FG2-15 and FG2-16, FG2-17, as they are DC/CA/HPUE Duty cycle/MPR related capabilities and it is already specified in Section 4.2.23.1 of TS38306 that “CA, MR-DC, handover (e.g. CHO, DAPS, CPAC, etc), unlicensed band, HPUE Duty cycle, MPR related UE features and corresponding capabilities are not supported by an NCR-MT”, no further clarification is needed for these FGs.

- For FG1-11 and FG2-8 (highlighted in green), it should be clarified that they are not applicable to NCR-MT. For FG2-8, as there is existing capability signalling in TS38306, it can be clarified in the field description. For FG1-11, as there is no existing capability signalling (mandatory without signalling for normal UE), it is better to clarify in Clause 4.2.23 as part of the general description.

Proposal 2: Clarify in TS 38.306 that FG2-8 and FG1-11 are not applicable to NCR-MT.

  - FGs which are optional to NCR-MT

According to the feature list, the following FGs are optional for NCR-MT:

| Features | Index | Feature group | Components | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note | NCR-MT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1. System parameter | 1-1 | 60kHz of subcarrier spacing for FR1 | 60kHz subcarrier spacing for data channel in FR1 | No | Applicable only to FR1 |  | Optional with capability signalling |
|  | 1-2 | 64QAM modulation for FR2 PDSCH | 64QAM modulation for FR2 PDSCH | No | Applicable only to FR2 | Capability can be discussed in future, e.g. when low cost device (e.g. IoT) and/or higher frequency band in FR2 are introduced | Optional with capability signalling |
|  | 1-4 | 256QAM for PDSCH | 256QAM for PDSCH | No | Yes | For FR1, it can be revisited in the future whether the 256QAM is mandated in all UE types or categories | Optional with capability signalling for FR1 |
|  |  |  |  |  |  | For FR2, RAN4 agreed that no BS and UE requirements will be introduced in Rel.15. | Optional with capability signalling for FR2 |
|  | 1-5 | 256QAM for PUSCH | 256QAM for PUSCH | No | Yes | For FR1, RAN4 can further discuss to mandate 256QAM for PUSCH for FR1 in future release. For FR2, RAN4 agreed that no BS and UE requirements will be introduced in Rel.15. | Optional with capability signalling (for both FR1 and FR2) |
|  | 1-6 | pi/2-BPSK for PUSCH | pi/2-BPSK for PUSCH | No | Yes | RAN4 will define the same minimum requirements for pulse-shaped pi/2 BPSK and non-pulse shaped pi/2 BPSK for FR2. | Optional with capability signalling for FR1 Mandatory with capability signalling for FR2 |
|  | 1-7 | pi/2-BPSK for PUCCH format 3/4 | pi/2-BPSK for PUCCH format 3/4 | No | Yes |  | Optional with capability signalling for FR1 Mandatory with capability signalling for FR2 |
|  | 1-8 | Active BWP switching delay | Support of active BWP switching delay specified in TS38.133, candidate values set: {type1, type2} | No | No | For this feature, RAN4 also sent another LS (R4-1803283). Network cannot configure the shorter delay for certain UE type. | Optional with capability signalling |
| 2. UE RF | 2-1 | Maximum channel bandwidth supported in each band for DL and UL separately and for each SCS that UE supports within a single CC | 1) FR1 channel bandwidths in TS38.101-1 Table 5.3.5-1 2) FR2 channel bandwidths in TS38.101-2 Table 5.3.5-1 | No | No | UE capability signalling shall follow RP-172832 (Per-band capability signalling, separately for DL and UL and for each SCS) Whether a bandwidth newly introduced in future is mandatory for UE shall be discussed case by case. | All BW should be optional instead of mandatory for largest channel bandwidth per-band |
|  | 2-7 | Almost contiguous UL CP-OFDM | Support of almost contiguous UL CP-OFDM transmissions | No | Yes | RAN4 had defined the requirements for "Almost contiguous UL CP-OFDM" in Rel-15. | Optional with capability signalling |
|  | 2-12 | Multiple NS/P-Max | Multiple NS/P-Max | No | No | Per UE capability | Optional |
|  | 2-14 | Power boosting for Pi/2 BPSK for power class 3 UE | Power boosting for Pi/2 BPSK for power class 3 UE in TDD bands n40, n77, n78 and n79 with duty cycle less than 40% | Applicable only to TDD | Applicable only to FR1 | Per band capability | Optional with capability signalling |
| 3. Baseband | 3-1 | Independent measurement gap configurations for FR1 and FR2 | Measurement gaps for FR1 and FR2 are configured independently. | No | No |  | Optional |
|  | 3-2 | Simultaneous reception of data and SS block with different numerologies when UE conducts the serving cell measurement or intra-frequency measurement | Simultaneous reception of data and SS block with different numerologies when UE conducts the serving cell measurement or intra-frequency measurement | No | Yes |  | Optional |
|  | 3-3 | Short measurement gap | Measurement gap patterns with short MGL (gap pattern#2, 3, 6, 7, 8, 10) are supported for E-UTRAN measurement. Gap patterns #6, 7, 8, 10 only apply to E-UTRAN measurement when MO includes both E-UTRAN and NR. | No | No | Per UE capability This capability is signalled as a part of supportedGapPattern in TS38.306. | Optional |
|  | 3-4 | SU-MIMO Interference Mitigation advanced receiver | 1) R-ML (reduced complexity ML) receivers with enhanced inter-stream interference suppression for SU-MIMO transmissions with rank 2 with 2 RX antennas. 2) R-ML (reduced complexity ML) receivers with enhanced inter-stream interference suppression for SU-MIMO transmissions with rank 2, 3, and 4 with 4 RX antennas. | No | No | UE supporting the feature is required to meet the Enhanced Receiver Type requirements in TS 38.101-4 | Optional |

For the above listed FGs, as they are optional with capability signalling, the corresponding optional capability IEs should be defined in TS 38.331 and TS 38.306. Among them, for different capabilities, the handling should be different as follows:

- For FG1-1, FG1-5, FG1-6, FG1-7, FG2-7, FG2-14, FG3-1, FG3-2, FG3-3 and FG3-4, as they are also optional for the normal UEs, the existing optional capability IEs can be reused for NCR-MT.

- For FG1-4 and FG1-8, as there are existing fields, they can be reused with some clarification in the field descriptions.

- For FG1-2 and FG2-1 (highlighted in green), as there are no existing fields for them, new optional capability IEs should be introduced in TS 38.331 and TS 38.306.

- Specially, for FG2-12 (highlighted in red), it is marked as optional in the feature list, but we understand it should be not applicable to the NCR-MT. There was a CR agreed for this [3] and it has already been captured in TS 38.331 that the NCR-MT should ignore the p-Max and nr-NS-PmaxList fields in SIB and only follow the output power and emissions requirements specified in TS 38.106. So there is no need for NCR-MT to report this capability. For this misalignment, it is better to send an LS to RAN4 to confirm.

Proposal 3a: Clarify in TS 38.306 that FG1-4 and FG1-8 are optional for NCR-MT.

Proposal 3b: Introduce new optional capability fields corresponding to FG1-2 and FG2-1 in TS 38.331 and TS 38.306 for NCR-MT.

Proposal 4: Send an LS to RAN4 to confirm the understanding that the multiple NS/Pmax (FG2-12) is not applicable to NCR-MT.

Proposal 5: Agree the 38331 CR in R2-2406278 and the 38306 CR in R2-2406279.

# 3	Conclusion

This contribution provides discussions of RF/RRM capabilities for NCR-MT. Based on the discussion, the following are proposed:

Proposal 1: Remove the FGs from Table 4.2.23.1-3 of TS 38.306, except for FG1-3 and FG2-10.

Proposal 2: Clarify in TS 38.306 that FG2-8 and FG1-11 are not applicable to NCR-MT.

Proposal 3a: Clarify in TS 38.306 that FG1-4 and FG1-8 are optional for NCR-MT.

Proposal 3b: Introduce new optional capability fields corresponding to FG1-2 and FG2-1 in TS 38.331 and TS 38.306 for NCR-MT.

Proposal 4: Send an LS to RAN4 to confirm the understanding that the multiple NS/Pmax (FG2-12) is not applicable to NCR-MT.

Proposal 5: Agree the 38331 CR in R2-2406278 and the 38306 CR in R2-2406279.

# 4	References

- R2-2402132, LS on RAN4 UE feature list for Rel-18 (version 3), CMCC, RAN2#125bis

- R4-2402517, RAN4 feature list for NCR-MT, ZTE Corporation, RAN4#110.

- R2-2403445, Correction to P-Max and NS value usage for NCR-MT	Nokia