3GPP TSG-RAN WG2 Meeting #127	R2-2406283

Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda Item:	8.9.2

Source:	Huawei, HiSilicon, Turkcell

Title:	RAN2 aspects of the Store and Forward satellite operation

Document for: Discussion and Decision

# 1	Introduction

In the last meeting, the following agreements were achieved for the Store & Forward (S&F) satellite operation [1]:

Agreements: 1. For the uplink/downlink messages transmission for MO, from RAN2 perspective the following steps are taken as baseline for S&F satellite operation (in case only eNB is on the satellite): 1) The UE sends uplink data signalling to eNB when service link is available and the eNB stores it. 2) When feeder link is available, the eNB sends the uplink data/NAS signalling to the CN. 3) The eNB (same or different) receives the downlink data/NAS signalling from the CN and stores it when feeder link is available (and service link is not available). 4) The eNB (same or different) sends the downlink data/signalling to the UE when service link is available again 2. S&F indication can be provided by SIB (FFS on the details). RAN2 assumes that no NAS indication is needed 3. RAN2 understands legacy UEs may be barred by legacy cellBarred and cellBarredNTN

In this contribution, we further discuss the Access control and UE’s RRC state when the service link is not available in S&F satellite operation.

# 2	Discussion

2.1	Access control for S&F

It was agreed in the last meeting that the legacy UEs “may” be barred from a S&F cell by reusing the legacy barring bits. There were different opinions whether legacy UEs can be served by the S&F network. Based on the latest SA2 conclusions [2], it seems not possible for the legacy UEs to access the S&F network as there are enhancements needed from the UEs to support S&F operation.

Firstly, UE needs to indicate the support of S&F mode to the MME following existing NAS capability as specified (highlighted in green as below). Otherwise, it may be rejected. Secondly, UE needs to receive and recognize the new information introduced in the Attach or TAU Reject message (highlighted in yellow as below) to initiate the re-attempt.

---Start of excerpt from TR 23.700-29--- 2) When UE initiates Attach or TAU procedure, it indicates support for S&F mode to the MME following existing NAS capability, the MME sends Attach or TAU Reject message to the UE if these procedures cannot be completed due to S&F operation. The Attach or TAU Reject message includes: a) A new information indicating the UE that attach or TAU procedure cannot be completed because of the S&F operation and that the UE can re-attempt the attach or TAU in this PLMN in a next satellite pass. This indicates to the UE that the information contained in the Attach or TAU Request message is stored by the MME and the network will be available to the UE after interaction with ground network. b) Wait timer: Indicates to the UE the time it should wait before re-attempting the Attach/TAU procedure in the current or another satellite of the same PLMN. c) Optionally, The list of Satellite IDs over which the UE may re-attempt the Attach/TAU procedure, after wait timer expires. The Satellite IDs are based on the SIB information broadcasted by eNB. ---End of excerpt from TR 23.700-29---

Therefore, it is observed and proposed:

Observation 1: To be served by the S&F network, UE needs to indicate support to the CN and re-attempt based on the newly introduced assistance information received from the CN.

Proposal 1: Legacy UEs should be barred by the legacy barring bits of cellBarred and cellBarredNTN.

For similar reasons, the Rel-19(+) UEs not supporting S&F should also be barred from the S&F cell. As these UEs can recognize the S&F indication, they can just consider the S&F cells as barred.

Proposal 2: UEs not supporting S&F considers a cell performing S&F satellite operation as barred.

For the S&F UEs, as the capacity of the NW is limited. If the NW cannot accommodate more UEs at some time or if the S&F cell is reserved for some purpose, the NW should be able to bar the S&F UEs. This principle is similar to what we did for NTN, NES and NPN. Therefore, a new barring bit can be introduced in the SIB, e.g., cellBarredSF.

In this case, this new barring bit can also be used implicitly as the S&F indication, and no additional explicit S&F indication is needed. Specifically, the presence/absence for the IE indicates whether the cell is a S&F cell. If the IE is present, the barred/not-barred value indicates whether the S&F UEs are barred.

Proposal 3: For access control of the S&F capable UEs, a new S&F barring indication with values {barred, notBarred} is introduced in the SIB. This indication can be used as the S&F indication.

2.2	UE’s RRC state when service link is not available

Another issue that needs to be discussed is UE’s RRC state when the service link is not available. There are the following candidates:

1) UE is released to RRC_IDLE

2) UE is released to RRC_INACTIVE

3) UE stays in RRC_CONNECTED with the AS operation suspended.

For 2) and 3), since the multiple satellite scenario is supported, we need to consider whether it is always possible to transfer the UE context between the satellites. The may depend on RAN3 discussion. For now, we RAN2 can agree on 1) at least.

Proposal 4: UE can be released to RRC_IDLE when service link is not available. FFS on RRC_INACTIVE and RRC_CONNECTED (pending RAN3 discussion).

# 3	Conclusion

This contribution discusses the store and forward operation for IoT NTN and we have the following proposals:

Access control for S&F

Observation 1: To be served by the S&F network, UE needs to indicate support to the CN and re-attempt based on the newly introduced assistance information received from the CN.

Proposal 1: Legacy UEs should be barred by the legacy barring bits of cellBarred and cellBarredNTN.

Proposal 2: UEs not supporting S&F considers a cell performing S&F satellite operation as barred.

Proposal 3: For access control of the S&F capable UEs, a new S&F barring indication with values {barred, notBarred} is introduced in the SIB. This indication can be used as the S&F indication.

UE’s RRC state when service link is not available

Proposal 4: UE can be released to RRC_IDLE when service link is not available. FFS on RRC_INACTIVE and RRC_CONNECTED (pending RAN3 discussion).

4	References

- R2-2405703, Report from Break-out session on NR-NTN and IoT-NTN, Session Chair (ZTE Corporation), RAN2 126.

- TR 23.700-29, Study on integration of satellite components in the 5G architecture; Phase 3 (Release 19)