3GPP TSG-RAN WG2 Meeting #127	R2-2406285

Maastricht, Netherlands, Aug 19th – 23th, 2024

Agenda item:	8.4.3

Source:	Huawei, HiSilicon

Title:	RRM measurement relaxation and offloading in RRC_IDLE or RRC_INACTIVE

Document for:	Discussion and Decision

- Introduction

In RAN2#126 meeting [1], the following agreements are reached on RRM measurement relaxation and offloading.

- For serving cell measurement offloading (i.e., serving cell measurement fully offloaded to LR and no serving cell measurement via MR is required), RAN2 should focus on specifying the offloading criterion for serving cell for UEs supporting LP-WUS, and assume that RAN4 will define the measurement offloading requirements for serving cell. - RAN2 understand that the RRM measurement of the neighboring cell can only be performed by MR. Can discuss again if RAN1 inform us otherwise. - RAN2 will further discuss the neighbor cell measurement relaxation criteria (if the UE is using LR to measure the serving cell), e.g., considering reuse Rel-16 criteria for ‘not at cell edge’ and ‘low mobility’.

In this contribution, we further discuss RRM measurement relaxation and offloading for IDLE/INACTIVE modes.

- Discussion

General principle

During the online discussion in RAN2#126 meeting, whether the RRM measurement relaxation and offloading is coupled with LP-WUS or can be independent was discussed but not concluded. In our understanding, if a UE is not operating with LP-WUR, then this UE is just like a legacy UE and legacy behavior is applicable. For example, if a UE is not capable of LP-WUR or is capable of LP-WUR but not operating with LP-WUR, the UE operates with MR and the NR legacy behaviors for RRM measurement (including legacy RRM measurement relaxation) should be performed. Regarding the RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE discussed in Rel-19 LP-WUS/WUR WID, it should only apply to UEs operating with LP-WUR.

Proposal 1: If a UE is not capable of LP-WUR or capable of LP-WUR but not operating with LP-WUR, NR legacy behaviors for RRM measurement are performed.

RRM measurement when UE is operating with LP-WUR

Serving/neighbor cell RRM measurement relaxation and offloading

- Serving cell RRM measurement by LP-WUR

If a UE is operating with LP-WUR, UE should perform serving cell measurement by LP-WUR as this is necessary in order to determine at least the fulfillment of exit condition of LP-WUS monitoring.

Proposal 2: If UE is operating with LP-WUR, i.e., the entry condition of LP-WUS monitoring is fulfilled, UE should perform serving cell measurement by LP-WUR.

- Neighbor cell RRM measurement relaxation for MR

If a UE is operating with LP-WUR, the number of transitions from LP-WUR to MR should be minimized to obtain the power saving gain. As stated in TR 38.869 [2], using the MR for RRM measurements has a significant impact on UE energy consumption and reduces the power saving gain. Thus, the neighbor cell RRM measurements by MR should be relaxed once the entry condition of LP-WUS monitoring is fulfilled. In other words, the entry condition of LP-WUS monitoring can be regarded as the criteria for neighbor cell RRM measurement relaxation by MR.

Proposal 3: If UE is operating with LP-WUR, the relaxed neighbor cell RRM measurement should be performed by MR. The entry condition of LP-WUS monitoring can be regarded as the criteria for neighbor cell RRM measurement relaxation by MR.

According to the existing specification, UE may not perform neighbor cell measurement if the criteria specified as below is fulfilled:

- For intra-frequency measurements:

- If the serving cell fulfils Srxlev > SIntraSearchP and Squal > SIntraSearchQ: - If distanceThresh and referenceLocation are broadcasted in SIB19, and if UE supports location-based measurement initiation and has obtained its location information: - If the distance between UE and the serving cell reference location referenceLocation is shorter than distanceThresh, the UE may not perform intra-frequency measurements; - Else, the UE shall perform intra-frequency measurements; - Else, the UE may not perform intra-frequency measurements; - Else, the UE shall perform intra-frequency measurements.

- For NR inter-frequency with an equal or lower priority and inter-RAT frequency with lower reselection priority:

- If the serving cell fulfils Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ: - If distanceThresh and referenceLocation are broadcasted in SIB19, and if UE supports location-based measurement initiation and has obtained its UE location information: - If the distance between UE and the serving cell reference location referenceLocation is shorter than distanceThresh, the UE may choose not to perform measurements of NR inter-frequency cells of equal or lower priority, or inter-RAT frequency cells of lower priority; - Else, the UE shall perform measurements of NR inter-frequency cells of equal or lower priority, or inter-RAT frequency cells of lower priority according to TS 38.133 [8]; - Else, the UE may choose not to perform measurements of NR inter-frequency cells of equal or lower priority, or inter-RAT frequency cells of lower priority; Else, the UE shall perform measurements of NR inter-frequency cells of equal or lower priority, or inter-RAT frequency cells of lower priority according to TS 38.133 [8].

If the UE is near the centre of the cell, UE is not likely to perform cell re-selection to a neighbor cell of equal or lower priority (including intra-frequency, NR inter-frequency and inter-RAT frequency). This principle should be applied when UE is operating with LP-WUR, i.e., if the criteria for no neighbor cell measurement is fulfilled, UE is not required to perform the neighbor cell measurement of equal or lower priority. In this case, the criteria is based on LP-WUR measurement result, which is up to other WGs to further discuss.

Proposal 4: If UE is operating with LP-WUR, UE is not required to perform neighbor cell RRM measurement of equal or lower priority, if the criteria for no neighbor cell RRM measurement is fulfilled. The criteria is based on the LP-WUR measurement result.

Besides, measurement relaxation for higher priority frequency needs to be further considered. Higher priority frequency may be still needed even when UE is in the centre of the cell, and basically the measurement for higher priority frequency, when UE is in the centre of the cell, is already relaxed according to existing RRM requirement, and may be further relaxed which is up to RAN4 discussion. If UE only performs higher priority frequency measurement, without performing neighbor cell measurement of equal or lower priority, UE can only rely on the measurement results of higher priority frequency to perform cell reselection.

Proposal 5: Measurement relaxation for higher priority frequency is FFS.

- Serving cell RRM measurement relaxation for MR

For similar reasons as neighbor cell RRM measurement (to minimize transitioning from LP-WUR to MR), if UE is operating with LP-WUR, either relaxed or no serving cell RRM measurements should be performed by MR.

When the UE is at the edge of the cell and has started to perform neighbor cell measurement of equal or lower priority by MR, UE can also perform serving cell measurement by MR without much additional effort, since anyway the intra-frequency measurement should be performed. So the criteria for neighbor cell RRM measurement relaxation, i.e., the entry condition of LP-WUS monitoring, can be reused as the criteria for serving cell RRM measurement relaxation by MR.

Proposal 6: If UE is operating with LP-WUR, the relaxed serving cell RRM measurement should be performed by MR. The criteria for neighbor cell RRM measurement relaxation, i.e., the entry condition of LP-WUS monitoring, can be reused as the criteria for serving cell RRM measurement relaxation by MR.

If the UE is operating with LP-WUR, and the criteria for no neighbor cell measurement of equal or lower priority by MR is met, it is beneficial that UE does not perform serving cell measurement by MR either. Because serving cell measurement relying on the measurement by LP-WUR is sufficient to determine whether to start neighbor cell measurement of equal or lower priority based on measurement result by LP-WUR. There is reduction on power gain to perform serving cell measurement by MR in this case.

Proposal 7: If UE is operating with LP-WUR, UE performs no serving cell measurement by MR, if the criteria for no neighbor cell RRM measurement is fulfilled.

Details about how LP-WUR or MR performs RRM measurement are illustrated in the figure and table below.

Fig. 1. Examples of UE behaviors for RRM measurement relaxation in different scenarios

Table. 1. Summary of RRM measurement criteria and UE RRM measurements behavior

| RRM measurement criteria |  | RRM measurements behavior |  |
| --- | --- | --- | --- |
|  |  | Serving cell | Neighbor cell |
| Entry condition for LP-WUS monitoring is fulfilled | Srxlev > SIntraSearchP and Squal > SIntraSearchQ; and Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ FFS low mobility criteria | Serving cell measurement offloading (i.e. serving cell measurement by LP-WUR and no serving cell measurement by MR) | No neighbor cell measurement of equal or lower priority by MR FFS higher priority frequency |
|  | Srxlev ≦SIntraSearchP and Squal ≦ SIntraSearchQ; or Srxlev≦SnonIntraSearchP and Squal ≦ SnonIntraSearchQ FFS low mobility criteria | Serving cell measurement by LP-WUR and relaxed serving cell measurement by MR | Relaxed neighbor cell measurement of equal or lower priority by MR FFS higher priority frequency |
| Exit condition for LP-WUS monitoring is fulfilled |  | NR legacy behaviors for RRM measurement |  |

Other aspects on the criteria

Based on Proposal 3 above, the entry condition of LP-WUS monitoring can be regarded as serving/neighbor cell measurement relaxation by MR. Thus, additional neighbor cell measurement relaxation criterion is not needed. And basically the “not at cell edge” has been considered for entry/exit condition of LP-WUS monitoring or criteria for performing neighbor cell measurement. In addition to the criteria for not being at cell edge, it can be further discussed whether the criteria for low mobility is needed or how to combine it if needed.

Proposal 8: The introduction of low mobility criteria for measurement relaxation of MR can be further studied.

Furthermore, UE obtains the serving and neighbor cell measurement results to evaluate whether the RRM measurement related criteria or cell reselection criteria is fulfilled or not. Based on Proposal 4 above, since there is no serving cell measurement by MR, UE relies on serving cell measurement by LP-WUR to determine whether the criteria for starting neighbor cell measurement relaxation of equal or lower priority is fulfilled or not. However, for cell reselection criteria, in some cases there is comparison between serving cell measurement results and neighbor cell measurement results, thus, both serving and neighbor cell measurement results by MR can be used. RAN2 can further discuss the criteria based on more inputs from RAN1/4.

Proposal 9: Whether the criteria evaluation for cell reselection is based on measurement by LP-WUR (LP-SS or PSS/SSS) or by MR can be further studied based on RAN1/4 progress.

- Conclusion

In this paper, we discuss RRM measurement relaxation and offloading for IDLE/INACTIVE modes, and have the following proposals.

General principle

Proposal 1: If a UE is not capable of LP-WUR or capable of LP-WUR but not operating with LP-WUR, NR legacy behaviors for RRM measurement are performed.

Serving cell RRM measurement by LP-WUR

Proposal 2: If UE is operating with LP-WUR, i.e., the entry condition of LP-WUS monitoring is fulfilled, UE should perform serving cell measurement by LP-WUR.

Neighbor cell RRM measurement relaxation for MR

Proposal 3: If UE is operating with LP-WUR, the relaxed neighbor cell RRM measurement should be performed by MR. The entry condition of LP-WUS monitoring can be regarded as the criteria for neighbor cell RRM measurement relaxation by MR.

Proposal 4: If UE is operating with LP-WUR, UE is not required to perform neighbor cell RRM measurement of equal or lower priority, if the criteria for no neighbor cell RRM measurement is fulfilled. The criteria is based on the LP-WUR measurement result.

Proposal 5: Measurement relaxation for higher priority frequency is FFS.

Serving cell RRM measurement relaxation for MR

Proposal 6: If UE is operating with LP-WUR, the relaxed serving cell RRM measurement should be performed by MR. The criteria for neighbor cell RRM measurement relaxation, i.e., the entry condition of LP-WUS monitoring, can be reused as the criteria for serving cell RRM measurement relaxation by MR.

Proposal 7: If UE is operating with LP-WUR, UE performs no serving cell measurement by MR, if the criteria for no neighbor cell RRM measurement is fulfilled.

Other aspects on the criteria

Proposal 8: The introduction of low mobility criteria for measurement relaxation of MR can be further studied.

Proposal 9: Whether the criteria evaluation for cell reselection is based on measurement by LP-WUR (LP-SS or PSS/SSS) or by MR can be further studied based on RAN1/4 progress.

- References

- R2-2405702, Report from session on R18 MIMOevo, R18 MUSIM, and R19 LP-WUS.

- TR 38.869, Study on low-power wake up signal and receiver for NR.