3GPP TSG-RAN WG2 Meeting #127	R2-2406286

Maastricht, Netherlands, 19th– 23rd August, 2024

Agenda Item:	 8.6.3

Source:	 Huawei, HiSilicon

Title:	 Discussion on measurement event evaluation

Document for:  Discussion and Decision

1	Introduction

LTM (L1/L2 Triggered Mobility) was introduced in Rel-18. The network receives L1 measurement reports to determine the LTM target cell and target beam. There are periodic, aperiodic and semi-persistent L1 reports. Objectives outlined in the WID for Rel-19 mobility are to define the required improvements for enabling event-triggered L1 measurements.

RP-240299, Revised Work Item: NR mobility enhancements Phase 4 - Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - RAN1 and RAN2 to progress independently on the event triggered measurements objectives of their respective MIMO and Mobility enhancement WIs. Review progress at RAN#105 to see if any modification of objectives is required to avoid/manage any overlap in the work - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management, and/or other necessary physical layer operations on candidate cells before LTM [RAN1]

In this contribution, we present some view for the event-triggered L1 measurement evaluation, including the beams of the serving cell and neighboring cells used for event evaluation, and the necessity for cell-level measurement results in the context of event evaluation.

2	Discussion

2.1	Support of LTM Events

The following agreements were made in RAN2#126 about the newly defined LTM events, while whether to support Event LTM1 is for FFS.

Agreements on measurement enhancements for LTM:

3.	Support the following LTM events based on beam specific quality of serving cell and candidate cells as the L1 LTM measurement events.

-	Event LTM2: Beam of serving cell becomes worse than absolute threshold;

-	Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell;

-	Event LTM4: Beam of candidate cell becomes better than absolute threshold;

-	Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2.

FFS on what beam(s) of the serving cell and neighboring cell is used for event evaluation. FFS on the need of Event LTM1.

We show the event configuration for inter-frequency measurement in the following figure. At the begining, the network configures event A2 to see if the quality of the cell is good enough. While if event A2 is triggered (the quality of the serving cell is below a certain threshold) and cell under different carriers are available for handover in the cell relation list, the network configures event A1 and event A5 (coverage-based handover). Measurement gap can also be configured in this case to allow for inter-frequency measurement. Then, if the event A1 is triggered (the quality fo the serving cell is above a certain threshold), the network can remove the event configuration for A5 and MG, to avoid inte-frequency measurement.

Figure1: Example of RRC signalling for legacy L3 inter-frequency measurement

We think the same is also applicable for LTM events, since we have already agreed on Event LTM A4/5. With Event LTM1, when the UE is performing measurement for the beam of candidate cell, meeting the Event LTM1 condition means that the beam of serving cell is better than an absolute threshold. Consequently, the UE can cease the measurement of beams of LTM candidate cells, as the serving cell beams quality is good enough and LTM cell switch is not needed. Then, the gNB can indicate to the UE to stop performing inter-frequency measurements for events LTM4/LTM5 and, if configured, remove measurement gap configuration. Unnecessary measurements and interruption of data transmission can be avoided.

Proposal1: Support Event LTM1 (Beam of serving cell becomes better than absolute threshold).

In legacy L3 measurement, Event A6 was defined as that a neighbour cell is offset better than the SCell. This event is important for the modification or addition of SCells and assists the network in determining which candidate cell can be configured as an SCell. Thus, we also think that the utility of Event A6 is evident for LTM: Event LTM 6 is essential for enabling dynamic adjustments to SCell change based on LTM measurements.

Proposal2: Support Event LTM 6 (Beam of candidate cell becomes amount of offset better than beam of SCell).

2.2	Triggering quantity

In the RAN2#126 agreements for the definition of the LTM events, "beam of serving cell" or "beam of the neighbor cell" is mentioned. However, it is unclear what is the meaning of the " beam of serving cell" or "beam of the neighbor cell".

2.2.1	Beam of serving cell

For the serving cell, the unified TCI state (DL or joint TCI state) is obtained by the legacy beam management procedure. When evaluating LTM events with beam -level measurement as the triggering quantity, we think that the currently used beam should be representative of the quality of the current cell. Thus, we think the currently used beam should be the beam of the serving cell.

Proposal3: For LTM events with beam-level measurement as triggering quantity, the applicable beam of the serving cell is the currently used beam (DL or joint TCI state).

2.2.2	Beam of the candidate cell

In the legacy, the term "applicable cell" refers to the cells that are applicable for event evaluation. For example, for event A1/2, only the serving cells are applicable. We think for LTM, since the scope of cells for measurement are already determined before the LTM cell switch execution, i.e., LTM candidate cell. It is obvious that for the events requiring the neighbouring cell quality as the triggering quantity, only LTM candidate cells are applicable.

Proposal4: For LTM events involving beam of the neighboring cell as the triggering quantity (e.g., Event LTM A3/5), only LTM candidate cells are applicable.

For certain events, such as Event LTM 3, in addition to the beam of the serving cell, the beam of the candidate cells also needs to be involved in the event evaluation. As for the "applicable beam of the candidate cell", the straightforward understanding is the best beam of a candidate cell. L1 event triggered reports should let the network timely know if there are good candidate cells and beams, which can be achieved by triggering the report if the best beam of a candidate cell meets the criteria.

Proposal5: For LTM events with beam-level measurement of candidate cell as triggering quantity, the applicable beam of the candidate cell is the best beam of that cell.

2.2.3	Cell-level measurement as LTM event triggering quantity

Agreements on measurement enhancements for LTM:

2. 	For event triggered L1 measurement, use of beam level measurement result for event evaluation is baseline. FFS for the cell level measurement.

InRAN2#126, using beam level measurements as LTM event triggering quantity was agreed, but whether cell level measurement can also be the triggering quantity is FFS.

Figure 2: General framework for L3 mobility measurement

In the legacy L3 measurements, as shown in Fig.2, the cell level measurement result is obtained from the beam level measurement results by beam consolidation/selection. This together with L3 filtering results in more reliable and stable measurement report triggering as compared to relying solely on the current best beam, in order to avoid ping-pong.

For intra-CU LTM, if RACH-less LTM cell switch is used and RLC is not re-established (intra-DU case), only HARQ operation is interrupted and if RLC is re-established, PDCP data recovery is triggered by the unchanged CU.

At inter-CU LTM cell switch, the MN key changes, PDCP is re-established and the RAN-CN connection is moved, which requires more time compared to intra-CU LTM and extra network signalling. Therefore, a more stable triggering mechanism would be desirable to reduce ping-pong at inter-CU LTM cell switch. Using beam consolidation in order to trigger reporting can lead to less frequent inter-CU LTM cell switch.

For how to obtain the cell-level measurement of serving cell and candidate cell, we think the beam consolidation/selection procedure in the legacy L3 measurement is enough and no further change is required.

Proposal6: Support cell-level measurement as LTM event triggering quantity. Cell-level measurement is obtained from the beam-level measurement by beam consolidation as in legacy.

2.3	Spec modelling of event-triggered L1/2 reporting

At RAN1#117, in Rel-19 MIMO discussions, the following has been agreed for the event evaluation: MIMO measurement report is triggered when the condition is satisfied more than M times during a time window.

Agreement Regarding the triggering event determination for Event 2: - If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs. - Note: Event-2 instance for a new beam is determined if the L1-RSRP of the new beam becomes a threshold value better than the current beam Above feature is subject to UE capability. - Basic feature: Once the L1-RSRP of the new beam becomes a threshold value better than the current beam, UE initiated beam report occurs FFS: Whether the above is captured in RAN1 or RAN2 specification.

The MIMO agreement to trigger a report include considerations of time windows and of a number of occurrences. Such mechanisms are currently not specified in L1 specifications e.g., there are similar to the mechanisms used to declare BF specified in TS 38.321 or RLF specified in 38.331.

Observation 3: The event triggering evaluation agreed by RAN1 for MIMO involves time windows and counters, (MAC/RRC).

2.3.1	Evaluation of the entering and leaving conditions

The event-triggering procedures for LTM, we see two possible specification approaches for evaluating the entry/leaving condition:

-	Option 1: L1 checks the entry/leaving condition for the events and indicates to MAC or RRC when a condition is satisfied. MAC or RRC determine whether to trigger a report:

-	Option 2: L1 report results to upper layer, MAC or RRC evaluates the entering and leaving conditions for the events and determine whether to trigger a report.

For Option1, if the comparison occurs in the PHY layer, it sends the instance to the higher layers. Alternatively for Option2, if conducted in the higher layers, the PHY layer sends the beam quality metrics, such as L1-RSRP, to the higher layers, allowing them to perform the comparison based on beam quality.

Between these two approaches, we think that the it is more suitable for the L1 to check whether the entering/leaving condition is satisfied as the best beam or the currently used beam is known to the PHY, but transparent to the MAC layer.

Proposal7: The entering/leaving conditions for events with beam-level measurement as triggering quantity are evaluated in L1. After evaluation, L1 indicates to higher layer whether the condition is satisfied.

2.3.2	Evaluation of the event triggering

Then, the next question is which layer should perform the evaluation for the event triggering, between MAC layer and RRC layer. The current RRC procedure for L3 event triggered report is not totally applicable for L1 report and many of the procedures are redundant to the L1/2 report. Also, two L1/2 report formats, i.e., UCI or MAC CE, are under discussion and it is more suitable for the MAC layer to trigger the lower layer report from spec modelling point of view.

Proposal8: The evaluation of the event triggering is performed in the MAC layer.

2.4	Configuration framework

2.4.1	Framework for event triggered L1/2 report

In R18 LTM, we use the following Fig. 3 to illustrate the structure of configuration for measurement report

Figure 3: General framework for LTM resource and report configuration

As can be seen: (a) LTM-CSI-ReportConfig is per serving cell and is similar to the CSI-ReportConfig that exists since Rel-15; (b) LTM-Config is directly inRRCReconfiguration and LTM resource config (SSB configuration) is under LTM-Config, which is different from Rel-15; (c) The semi-persistent and aperiodic trigger state are the Rel-15 ones

As discussed above, if the event evaluation is performed per cell group, the configuration of the event, i.e., the report configuration should also be in at the cell group configuration level rather than under CSI-MeasConfig, which is per serving cell.

Proposal9: The configuration of event-triggered L1/2 reports should be per cell group.

2.4.2	Fast event configuration

As mentioned in proposal 1 for the discussion on Event LTM2, for inter-frequency measurements, in order to reduce impacts to data transmission, there is a sequential usage of event LTM2, and then, of events LTM1/LTM5. If we follow the legacy configuration approach, for event triggered LTM, the signalling flow can be illustrated in Fig. 4.

Figure4, LTM event configuration by RRCReconfiguration message

It can be seen that if we follow the current RRC configuration approach for LTM, RRCReconfiguration needs to be used in Step 4 and Step 7 to configure or release the LTM 5 configurations, which is quite in-efficient. The main benefits of LTM-based cell switch/HO is the latency reduction compared with L3 approach. However, the legacy event configuration mechanism may not be sufficient in this functionality. We should discuss how to be able to quickly configure LTM events.

In the legacy R17 discussion, pre-configured MG configuration is proposed under two topics MG enhancement, and positioning enhancements.

- For positioning, the pre-configured measurement gap procedure is used by the network to provide measurement gap for NR DL-PRS measurements. The serving gNB may activate/deactivate the pre-configured measurement gap upon receiving the request from a UE or LMF.

- For MG enhancements, there are two approaches (a) network-based approach where the network configures the activation/deactivation state explicitly; (b) UE-based approach where the UE autonomously determines the MG activation/deactivation when switching to a certain BWP

For event configuration for LTM, we think similar mechanism should be investigated to reduce the signalling/measurement latency for the UE. Possible solution can be that the event configuration/MG configuration is preconfigured and can be quickly activated/deactivated for measurement.

Proposal10: Support pre-configured event configuration/MG configuration and its activation/deactivation for event-triggered L1/2 report.

# 3	Conclusion

In this paper, we propose the following under various issues:

Support of LTM events

Proposal1: Support Event LTM1 (Beam of serving cell becomes better than absolute threshold).

Proposal2: Support Event LTM 6 (Beam of candidate cell becomes amount of offset better than beam of SCell).

Triggering quantity

Proposal3: For LTM events with beam-level measurement as triggering quantity, the applicable beam of the serving cell is the currently used beam (DL or joint TCI state).

Proposal4: For LTM events involving beam of the neighboring cell as the triggering quantity (e.g., Event LTM A3/5), only LTM candidate cells are applicable.

Proposal5: For LTM events with beam-level measurement of candidate cell as triggering quantity, the applicable beam of the candidate cell is the best beam of that cell.

Proposal6: Support cell-level measurement as LTM event triggering quantity. Cell-level measurement is obtained from the beam-level measurement by beam consolidation as in legacy.

Spec modelling

Proposal7: The entering/leaving conditions for events with beam-level measurement as triggering quantity are evaluated in L1. After evaluation, L1 indicates to higher layer whether the condition is satisfied.

Proposal8: The evaluation of the event triggering is performed in the MAC layer.

Configuration framework

Proposal9: The configuration of event-triggered L1/2 reports should be per cell group.

Proposal10: Support pre-configured event configuration/MG configuration and its activation/deactivation for event-triggered L1/2 report.

# 4	Reference

- RP-240299, Revised Work Item: NR mobility enhancements Phase 4;

- RAN1 #117 chairman notes;

- TS 3GPP 38.300 V18.1.0