3GPP TSG-RAN WG2 Meeting #127	R2-2406287

Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda Item:	 8.6.4

Source:	 Huawei, HiSilicon

Title:	 Discussion on event triggered measurement report

Document for: Discussion and Decision

1	Introduction

LTM (L1/L2 Triggered Mobility) was introduced in Rel-18. The network receives L1 measurement report to determine the LTM target cell and target beam. Three types of report are defined, namely, periodic, aperiodic and semi-persistent L1 report.

For R19 mobility enhancements, goals outlined in the WID is to define the required improvements for enabling event-triggered L1 measurements.

RP-240299, Revised Work Item: NR mobility enhancements Phase 4 - Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - RAN1 and RAN2 to progress independently on the event triggered measurements objectives of their respective MIMO and Mobility enhancement WIs. Review progress at RAN#105 to see if any modification of objectives is required to avoid/manage any overlap in the work - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management, and/or other necessary physical layer operations on candidate cells before LTM [RAN1]

In this contribution, we present a detailed analysis of event-triggered L1 measurement reports. This includes the granularity of the report, the content within the report, and the structure of the report container, single/periodic report, report on leave, etc.

2	Discussion

2.1	Measurement report quantity

The following agreements were made during RAN2#126: it was mentioned that LTM events triggering evaluation are based on beam-level measurement results, and FFS on cell-level measurement results.

Agreements on measurement enhancements for LTM:

2. 	For event triggered L1 measurement, use of beam level measurement result for event evaluation is baseline. FFS for the cell level measurement.

Regarding the measurement result report, RAN2 can also discuss the granularity, such as beam level, cell level, or both.

In the legacy LTM cell switch procedure, the network instructs the UE to perform the switch by sending the LTM Cell Switch Command MAC CE. This procedure is executed after the network receives the L1 measurement report, based on which LTM handover decision is performed. This MAC CE contains the TCI state ID for the LTM target cell, which means that UE can switch to the specific beam of the target cell after the LTM handover procedure. To facilitate this procedure, the network must be aware of the quality of different beams for the candidate cells. A beam-level report from UE can provide detailed information about the quality of each beam to the network. For event-triggered L1/2 report and inter-CU LTM, we think that the same rationale holds, that the beam-level report quantity is beneficial for the LTM performance.

Proposal1: Support reporting beam-level measurement result.

RACH-based LTM is supported in LTM handovers. In this procedure of RACH-based LTM, the UE does not directly select the beam of the target cell. Instead, it chooses the target beam through the RACH procedure. Hence that the UE performs the handover based on cell-level criteria rather than beam-level. Following the LTM cell switch, the UE selects the target beam, rather than choosing the beam directly during the handover process. Therefore, only cell-level results need to be provided to the network, as the handover is determined by cell-level measurements.

Observation: Support RACH-based LTM, where SSB is selected by the UE during the RACH procedure while not indicated by the cell switch command.

Furthermore, the use of cell-level L1 reports can be considered for other specific cases. For instance, in a cell operating within a low-frequency range like FR1, which has excellent coverage capabilities, indicating that a single TCI state can cover the entire cell. Therefore, this scenario is suitable for cell-level reporting because there is no need for beam level results.

Observation: For LTM in FR1, beam level measurement may not be relevant.

Proposal2: Support reporting cell-level measurement result.

In RAN1 spec TS 38.214, the table below presents the legacy L1/L2-triggered mobility report in UCI format, outlining the mapping order of CSI fields for one report concerning SSBRI/RSRP, which are the quality metrics corresponding to each CSI-SSB index.

In legacy LTM measurement reports, such as periodic reports on PUCCH, the network determines the association of SSBs with candidate cells through PUCCH resources, where each PUCCH resource corresponds to a set of SSBs linked to candidate cells. This method allows the network to identify the specific candidate cell to which an SSB belongs based on the PUCCH resources used.

However, in Rel-19 LTM, since the network does not know the exact triggering timing of the event, it is not suitable allocating periodic PUCCH resources for the report. Therefore, the existing association method between PUCCH resources and candidate cells/SSBs may no longer be appropriate. In this case, it is necessary for the report content to include additional information to specify the candidate cell ID. The UCI format used in R18 is inadequate for this purpose.

Observation: The UCI content for R18 LTM may not be adequate for R19 event-triggered LTM, it is necessary for the report content to include the candidate cell ID.

Furthermore, it is necessary for the L1 measurement report to specify the triggering event, and the applicable RS-Type utilized in the report configuration both for SSB and CSI-RS to be introduced in R19 LTM.

Proposal3: The report content should be able to indicate the reference signal ID or candidate cell ID for the report quantity (e.g., RSRP, RSRQ).

2.2	Measurement report format

For legacy L3 measurements, multiple quantities may be reported as mentioned above. Also, any cell on the MO frequency can be reported for the same event. In Rel-18, L1 reports can even mix LTM candidate on different frequencies, so perhaps results of all candidate could be reported together, if they meet the reporting conditions at the same time. Also, L3 report can include all beams above a threshold. If similar choices are made for L1 reports in Rel-19, the report size and contents could be highly variable. If, on the contrary, the number beams of candidate cell to be reported is fixed, the report size and contents may be rather fixed.

Observation: From the point of view for flexibility in the report content, MAC CE is better than UCI.

Besides, in the previous RAN1#127 meeting, the MIMO has agreed that event-triggering report is sent by UCI, and the UCI can be transferred either in PUCCH or PUSCH.

Working Assumption On beam report transmission procedure for UE-initiated/event-driven beam reporting - For mode-A, at least support one-bit indication in the first PUCCH channel to request a resource for a second UL channel to carry beam report. - In such case, a periodic PUCCH resource (with PUCCH format 0/1) is configured by dedicated RRC signalling. - For mode-B, at least support one-bit indication in the first PUCCH channel to notify a second UL channel to carry beam report. - In such case, a periodic PUCCH resource (with PUCCH format 0/1) is configured by dedicated RRC signalling. - FFS: Whether/how to support multi-bit indication in the first PUCCH for mode-A and mode-B, e.g., when multi-event(s) are approved. - FFS: details on the dedicated RRC signalling.

However, in this agreement, resource may be pre-allocated to the UE. One feature of event triggered report is that the UE only sends the reports when a particular event is triggered. Thus, if the event is not triggered, the allocated resources remain unused, leading to potential inefficiencies. A similar outcome is observed when reusing the legacy Rel-18 LTM L1 report, which is transmitted via UCI for periodic reporting.

Also, if the size of the report is largely variable, it is rather difficult to have pre-allocated resources. If a MAC CE is used, the UE can send the MAC CE when there is an available UL grant and there is flexibly sharing of the grant size between UL transmission for RBs and for the MAC CE, which is more efficient From UL resource perspective than using UCI.

Observation: From UL resource efficiency perspective, using MAC CE is better than UCI in pre-allocated resources.

It is evident that using MAC CE offers flexibility in terms of its size and does not require any pre-allocated resources. As such, UCI is not suitable for the event-triggering measurement process.

Proposal4: RAN2 acknowledges the benefits and feasibility of using MAC CE for event-triggered L1/2 report. LS should be sent to RAN1 for final confirmation.

2.3	Report after event triggering

In legacy L3 measurement report, the parameter reportAmount is used to indicate the number of reports that the UE sends to the network when the event is triggered. The network may instruct the UE perform a handover after receiving an event-triggered report with reportAmount set to 1. The network may also require multiple reports from the UE in order to make a more informed decision. After the network has received a first report, it may not make the handover decision immediately and wait for further reports over a period of time before initiating a handover, in order to avoid handover too early.

For event-triggered L1 reports for LTM cell switch, if the same mechanism should be supported, the network could similarly decide to make the LTM cell switch decision immediately upon receiving a report or after receiving several reports. Therefore, the UE should be allowed to report multiple times after an event is triggered.

In addition, event-triggered periodic report is similar to the legacy periodic report in R18 intra-CU LTM, except that the report is triggered by the UE and sent periodically after the event is triggered.

Proposal5: Support both the event-triggered periodic report and event-triggered one-shot report.

In L3 measurements, when the event is triggered for a specific cell, this cell is added to the cellsTriggeredList defined within the VarMeasReportList and a report is sent. Then, no new report is triggered for the cells already in the cellsTriggeredList and the network assumes that the quality of the cell remains unchanged. If the cell satisfies the leaving condition, the UE removes it from the cellsTriggeredList and it could trigger another report.

For L1 event-triggered reports, RAN2 already agreed to have an entering condition and a leaving condition, Therefore, it also makes sense that when the L1 event is triggered for a certain cell i.e., entry condition satisfied during the TTT, no new report is triggered for the cell until the leaving condition is satisfied during TTT. This avoids unnecessary signaling, which is the motivation of introduction of event-triggered LTM.

Proposal6: When event is triggered by a certain cell, it shouldn't be triggered again until the leaving condition is satisfied and the entering condition is met again.

Another question is whether to trigger a report when the leaving condition is satisfied during the TTT after a report was triggered for the entering condition i.e., report on leave.

In the L3 measurement, the parameter reportOnLeave indicates whether the UE should trigger the measurement reporting procedure when the leaving condition is satisfied for a candidate cell after meeting the entering condition. In the agreements of last meeting, the legacy mechanism for event triggering of entering and leaving condition is agreed to be reused. Without report set by reportOnLeave, the network remains unaware of the any change in cell quality, so it might instruct the UE to perform a handover to a low-quality cell, which is likely to fail. Therefore, when the UE meets the leaving condition, it is essential to send a report to inform the network timely.

Observation: If measurement report is not sent to the network when the leaving condition is met, the network may assume that the cell remains unchanged and make wrong handover decisions.

As can be seen above, in the legacy L3 measurement, the support of report on leave goes hand-in-hand with the above-mentioned UE variable cellTriggeredList: the UE will not trigger the measurement report for the cells in the cellsTriggeredList for satisfying entry condition to save power consumption.

In LTM, it is also crucial for the network to get the updated cell quality when the previously reported cell has met the leaving condition if we think that the event should not be repeatedly triggered by entering condition.

Proposal7: Report on leave is supported for event-triggered L1 measurement reporting.

# 3	Conclusion

In this paper, we have addressed the objective regarding improvements in event-triggering reports for LTM. There are the following proposals:

Measurement Report Quantity

Proposal1: Support reporting beam-level measurement result.

Proposal2: Support reporting cell-level measurement result.

Proposal3: The report content should be able to indicate the reference signal ID or candidate cell ID for the report quantity (e.g., RSRP, RSRQ).

Measurement Report Format

Proposal4: RAN2 acknowledges the benefits and feasibility of using MAC CE for event-triggered L1/2 report. LS should be sent to RAN1 for final confirmation.

Report after triggering

Proposal5: Support both the event-triggered periodic report and event-triggered one-shot report.

Proposal6: When event is triggered by a certain cell, it shouldn't be triggered again until the leaving condition is satisfied and the entering condition is met again.

Proposal7: Report on leave is supported for event-triggered L1 measurement reporting.

# 4	Reference

- RP-240299, Revised Work Item: NR mobility enhancements Phase 4;

- RAN2 #126 chairman notes;

- TS 3GPP 38.331 V18.1.0

- TS 3GPP 38.212 V18.3.0