3GPP TSG-RAN2 Meeting # 127 	R2-2406293

# Masstricht, Netherlands, 19 – 23 August, 2024

Title:		Remaining issues on MAC spec for R18 positioning

Source:	Huawei, HiSilicon

Agenda item:	7.2.6

Document for:	Discussion and Decision

- Background

This paper discusses the remaining RAN2 issues in the MAC spec for R18 positioning

2	Discussions

2.1	SP SRS activation/deactivation MAC CE

The following has been agreed in RAN1 according to the LS R1-2401708

Furthermore, the following has been agreed for the activation/deactivation of SRS resources within SRS resource sets across different carriers.

One question remains on how to indicate spatial relation for different SRS resources. As can be seen by the NOTE above, the MAC CR should indicate the single spatial relation for the SRS resources aggregated from different resource sets.

Option1

One possible solution is to restrict on the number of SRS resources across different aggregated resource sets to be the same, and then the SRS resources with the same indices are aggregated together. As shown by the example below, 3 resource sets on 3 CCs are configured and each resource set is configured with 4 SRS resources. Then, the SRS resource with the same index across SRS resource sets are aggregated together and spatial relation is indicated for each aggregated resources.

Option2

Another possible solution is to aggregate the SRS by the rule and then indicate spatial relation for each of the aggregated resources by the ordering of SRS resources with aggregation on a certain CC. During previous RAN1 meeting, the following has been agreed in RAN1 that the aggregated SRS resources need to have the same periodicityAndOffset and SlotOffset and it is assumed that for aggregated resources, the condition is satisfied.

Then, based on this rule, one example can be shown in the following figure. The following two linkage is shown, Then, the network needs to indicate spatial relation for each linkage

- SRS resource#1 in SRS resource set #1 on CC1+ SRS resource#2 in SRS resource set #3+ on CC2+ SRS resource#3 in SRS resource set #2 pm CC3

- SRS resource#1 in SRS resource set #1 on CC1+ SRS resource#2 in SRS resource set #3+ on CC2+ SRS resource#3 in SRS resource set #2 pm CC3

2.2	Proposals

Based on the discussion above, we summarize the above discussion into two options below:

- Option1: (Aggregation by SRS resource index) Configure the same number of SRS resources for the aggregated SRS resource sets from different carriers and indicate the same number of spatial relations as the number of SRS resource in each SRS resource set.

- Option2: (Aggregation by the rule) The SRS resources are aggregated by the rule specified in the RAN1 spec and indicate the spatial relation for each such aggregation.

Within the above two options, we think there might be no reason to restrict the number of SRS resource set within the SRS resource sets of each aggregated CC. The interpretation can be as follows: The Spatial Relation for Resource ID1 indicates the first aggregated resource in resource set#1 on CC1 and its aggregated resources on CC2 and CC3; Spatial Relation for Resource ID2 indicates the second aggregated resource in resource set#1 on CC1 and its aggregated resources on CC2 and CC3, and so on.

Proposal1: Clarify that Spatial Relation for Resource IDi indicates the spatial relation of the ith SRS resources within the first CC that is aggregated with the other SRS resource in the other CCs. Adopt the TP in Annex A.

3	Conclusions

Proposal1: Clarify that Spatial Relation for Resource IDi indicates the spatial relation of the ith SRS resources within the first CC that is aggregated with the other SRS resource in the other CCs. Adopt the TP in Annex A.

4	Annex A: Text proposal for TS 38.321

=================================TP BEGINS=======================================

#### 6.1.3.83	Aggregated SP Positioning SRS Activation/Deactivation MAC CE

The Aggregated SP Positioning SRS Activation/Deactivation MAC CE is identified by a MAC subheader with eLCID as specified in Table 6.2.1-1b. It has a variable size with following fields:

-	Positioning SRS Aggregation ID: This field indicates one of the combinations of linked Positioning SRS Resource Sets corresponding to SRS-PosResourceSetAggBW-CombinationList or SRS-PosRRC-AggBW-InactiveConfigList specified in TS 38.331 [5]. Value 0 corresponds to the first entry within the list SRS-PosResourceSetAggBW-CombinationList or SRS-PosRRC-AggBW-InactiveConfigList; value 1 corresponds to the second entry within the list SRS-PosResourceSetAggBW-CombinationList or SRS-PosRRC-AggBW-InactiveConfigList and so on;

-	C1, C2, C3: These fields indicate the activation/deactivation status of each Positioning SRS Resource Set that is linked for SRS for positioning bandwidth aggregation configured in SRS-PosResourceSetAggBW-CombinationList or SRS-PosRRC-AggBW-InactiveConfigList specified in TS 38.331 [5]. C1 corresponds to the first entry in SRS-PosResourceSetAggBW-CombinationList or SRS-PosRRC-AggBW-InactiveConfigList, C2 corresponds to the second one and so on. The Ci field is set to 1 to indicate that the Positioning SRS Resource Set corresponding to Ci shall be activated. The Ci field is set to 0 to indicate that the Positioning SRS Resource Set corresponding to Ci shall be de-activated;

-	S: This field indicates whether the fields Spatial Relation for Resource IDi are present. If the field is set to 1, the fields Spatial Relation for Resource IDi are present; otherwise, they are absent;

-	Spatial Relation for Resource IDi: The field Spatial Relation for Resource IDi is only present if MAC CE is used for activation, i.e. at least one of the C1, C2, and C3 is set to 1. There can be as many as 16 entries of Spatial Relation for Resource IDi depending on the RRC configuration. There are 4 types of Spatial Relation for Resource IDi, which is indicated by the F (F0 and F1) field within, defined as in Figure 6.1.3.36-2 to 6.1.3.36-5 in clause 6.1.3.36. Spatial Relation for Resource IDi corresponds to the spatial relation of the ith SRS resource of SRS resources within the first carrier (i.e., the first entry configured by the field SRS-PosResourceSetAggBW-CombinationList or SRS-PosRRC-AggBW-InactiveConfigList) that is aggregated with the SRS resources from the other carriers ;

-	C: This field indicates whether the octets containing Resource Serving Cell ID field(s) and Resource BWP ID field(s) within the field Spatial Relation for Resource ID i are present, except for Spatial Relation Resource IDi with DL-PRS or SSB. If the combination of the linked SRS resource sets is configured by the field SRS-PosResourceSetAggBW-CombinationList and this field is set to 1, the octets containing Resource Serving Cell ID field(s) and Resource BWP ID field(s) in the field Spatial Relation for Resource IDi are present, otherwise if this field is set to 0, they are not present; if the combination of the linked SRS resource sets is configured by the field SRS-PosRRC-AggBW-InactiveConfigList, this field shall be set to 0 and the octets containing Resource Serving Cell ID field(s) and Resource BWP ID field(s) in the field Spatial Relation for Resource IDi are not present;

-	R: Reserved bit, set to 0.

Figure 6.1.3.83-1: Aggregated SP Positioning SRS Activation/Deactivation MAC CE

===============================TP ENDS==========================================