3GPP TSG-RAN2 Meeting # 127 	R2-2406294

# Masstricht, Netherlands, 19 – 23 August, 2024

Title:		Discussion on the remaining issues for SLPP

Source:	Huawei, HiSilicon

Agenda item:	7.2.3

Document for:	Discussion and Decision

- Background

This paper discusses the remaining RAN2 issues in the SLPP for R18 positioning

2	Discussions

2.1	Discussions on the RAN4 LS R4-2410352

From R4-2410352, the following request has been sent from RAN4 on the use of synchronization source in sidelink positioning:

During the discussion on RRM core requirements for SL positioning, RAN4 reached the following agreement. - Agreement - It is a RAN4 understanding that UE performing measurements may not be aware on the synchronization source change at the Tx UE. - For synchronization reference source change occurring at Tx side, measurement accuracy requirements do not apply and no specific UE behaviour is defined. - Note: the agreement can be revisited if a RAN1/2 solution is introduced to inform the UE performing measurements on the synchronization source change at the Tx UE. RAN4 would like to check whether RAN1 and RAN2 have introduced or are working on any solutions to inform a UE performing an SL positioning measurement (e.g., SL RSTD, SL Rx-Tx, and SL RTOA) about synchronization reference source change at a UE which is transmitting SL-PRS for the measurement. 2. Actions: To RAN1, RAN2: ACTION: RAN4 kindly requests RAN1 and RAN2 to clarify whether they have introduced or are working on any solutions to inform an SL UE performing an SL positioning measurement (e.g., SL RSTD, SL Rx-Tx, and SL RTOA) about synchronization reference source change at a UE which is transmitting SL-PRS for the measurement.

For the question asked by RAN4, we think that this can be done based on the UE’s implementation, that whenever the synchronization sources of the TX UE changes, it sends a SLPP message for informing the RX side of the update in synchronization sources. For this, it is already supported by the current spec and no further changes are required.

Proposal1: Confirm on the understanding that when the synchronization source changes, it is up to the UE’s implementation the Tx side of the SL-PRS sends another provideAssistanceInformation SLPP message to information the Rx side of the update. No spec change required

Another issue we see that has been mentioned by RAN4 is the role of synchronization source in SL Rx-Tx measurement. The issue is that there is no synchronization sources indication in the current SLPP message for SL-RTT. We think that the synchronization source should be added as assistance data for SL-RTT. Hence, we propose the following:

Proposal2: Add synchronization sources as assistance data in SL-RTT-ProvideAsssitanceData for SL-RTT.

2.2 SL UE Rx-Tx time difference measurements with different ARPs

In Section 8.15.2 of TS 38.305, it is stated that a UE may report (a) multiple SL Rx-Tx time difference measurements for the same SL-PRS transmission and up to 4 different SL-PRS receptions, or; (b) report multiple SL Rx-Tx time difference measurements for the same SL-PRS reception and up to 4 different SL-PRS transmissions; or (c) both. The request for the report of multiple SL Rx-Tx time difference measurements for different pairs of SL-PRS transmissions or SL-PRS receptions is made with the field multipleSL-PRS-RxTxTimeDiffRequest within the IE SL-RTT-RequestLocationInformation as captured in TS 38.355. As stated in Section 8.4.4 in TS 38.214, a UE shall use the same ARP for both the transmission and reception of sidelink positioning reference signals while performing an SL Rx-Tx time difference measurement.

### 8.4.4 SL PRS reception procedure The UE may be configured to measure and report one or more of the SL RSTD, SL Rx-Tx time difference, SL RTOA, SL PRS-RSRPP, for the first detected path and up to 8 additional detected paths, and SL PRS-RSRP measurements. The UE may be configured to measure and report one or more of the SL AoA, SL PRS-RSRPP for the first path and up to 2 additional detected paths, and SL PRS-RSRP measurement. The UE may report an ARP ID associated with the reported measurements. The UE may provide the ARP location information via sl-ARP-LocationInfoPerTxUE. The UE uses the same ARP for both the transmission and reception of sidelink positioning reference signals while performing an SL Rx-Tx time difference measurement. =====TEXT OMITTED====

In case a UE is equipped with multiple panels as shown in Figure 1, a UE may not always be able to use the same ARP (antenna reference point) for both the transmission of a SL-PRS and reception of another SL-PRS.

Figure 1. SL-PRS transmission and reception with different panels

For a SL UE Rx-Tx time difference, if the UE is not able to use the same ARP for both the transmission of the SL-PRS and for the reception of the SL-PRS, the UE should not report a requested SL Rx-Tx time difference measurement. However, this is not captured in the request of multiple SL Rx-Tx time difference measurements in TS 38.355. As such we make the following proposal.

Proposal3: Capture in the field description of SL-RTT-ProvideLocationInformation SLPP message that when the ARP for SL-PRS transmission and reception are different, the UE shall not report SL Rx-TX time difference measurement.

2.3	ARP-related discussions related to RAN1 LS R1-2403622

In R1-2401949, the following has been mentioned regarding the ARP info for provideLocationInformation.

For the following question from RAN2 RAN2 do not have consensus on the scenario where the SL-PRS Rx UE reports measurements for multiple Rx ARP-IDs in a single measurement report. Current signalling structure cannot support this scenario, and it will be changed to accommodate it if RAN1 want to support the scenario. RAN1 would like to support the scenario that the SL-PRS Rx UE reports measurements for multiple Rx ARP-IDs for the same resource or different resource(s) from the same Tx UE in a single measurement report.

Then, after the LS, reply LS R1-2403622 was received

RAN1 thanks RAN2 for the LS on decisions on SLPP and the opportunity to feedback. RAN1 discussed the indicated agreements from RAN2 and would like to feedback the following information. For the following agreement from RAN2 The association information between ARP-ID and the already transmitted SL PRS resource(s) is placed inside the CommonSL-PRS-MethodsIEsRequestLocationInformation/CommonSL-PRS-MethodsIEsProvideLocationInformation, based on the corresponding TP of P3 from R2-2401244. To do this, the SL-PRS Tx UE can send the CommonSL-PRS-MethodsIEsProvideLocationInformation without providing any measurements. Notify RAN1 by LS. RAN1 would like to point out that from RAN1 perspective, for location calculations for UE-based SL positioning, it should be possible that the Rx UE can be provided the information about association between Tx ARP-ID and already transmitted SL PRS. It is unclear whether current signaling design from RAN2 can support this scenario. For the following question from RAN2 RAN2 do not have consensus on the scenario where the SL-PRS Rx UE reports measurements for multiple Rx ARP-IDs in a single measurement report. Current signalling structure cannot support this scenario, and it will be changed to accommodate it if RAN1 want to support the scenario. RAN1 would like to support the scenario that the SL-PRS Rx UE reports measurements for multiple Rx ARP-IDs for the same resource or different resource(s) from the same Tx UE in a single measurement report.

2.3.1	ARP info in provide location information

In the RAN1 LS, the following has been mentioned

For the following question from RAN2 RAN2 do not have consensus on the scenario where the SL-PRS Rx UE reports measurements for multiple Rx ARP-IDs in a single measurement report. Current signalling structure cannot support this scenario, and it will be changed to accommodate it if RAN1 want to support the scenario. RAN1 would like to support the scenario that the SL-PRS Rx UE reports measurements for multiple Rx ARP-IDs for the same resource or different resource(s) from the same Tx UE in a single measurement report.

Hence, we propose the following

Proposal4: Support multiple per ARP measurement results for the same SL-PRS in a single report. Adopt the TP in Annex C.

2.3.2	ARP info in provide assistance data

In R1-2401949, the following has been mentioned regarding the ARP info for provideAssistanceData.

For the following agreement from RAN2 The association information between ARP-ID and the already transmitted SL PRS resource(s) is placed inside the CommonSL-PRS-MethodsIEsRequestLocationInformation/CommonSL-PRS-MethodsIEsProvideLocationInformation, based on the corresponding TP of P3 from R2-2401244. To do this, the SL-PRS Tx UE can send the CommonSL-PRS-MethodsIEsProvideLocationInformation without providing any measurements. Notify RAN1 by LS. RAN1 would like to point out that from RAN1 perspective, for location calculations for UE-based SL positioning, it should be possible that the Rx UE can be provided the information about association between Tx ARP-ID and already transmitted SL PRS. It is unclear whether current signaling design from RAN2 can support this scenario.

While then in the RAN1 LS, the following concern has been raised by RAN1

For the following agreement from RAN2 The association information between ARP-ID and the already transmitted SL PRS resource(s) is placed inside the CommonSL-PRS-MethodsIEsRequestLocationInformation/CommonSL-PRS-MethodsIEsProvideLocationInformation, based on the corresponding TP of P3 from R2-2401244. To do this, the SL-PRS Tx UE can send the CommonSL-PRS-MethodsIEsProvideLocationInformation without providing any measurements. Notify RAN1 by LS. RAN1 would like to point out that from RAN1 perspective, for location calculations for UE-based SL positioning, it should be possible that the Rx UE can be provided the information about association between Tx ARP-ID and already transmitted SL PRS. It is unclear whether current signaling design from RAN2 can support this scenario.

From RAN2's perspective, the issue raised by RAN1 is not an issue. The SLPP message provideLocationInformation can be provided without requestLocationInformation, in an un-solicited way. Hence, no change for this is needed.

Proposal5: Confirm that provideLocationInfomation can be provided in un-solicited manner and the concern raised from RAN1 is not an issue.

3	Conclusions

RAN4 LS R4-2410352

Proposal1: Confirm on the understanding that when the synchronization source changes, it is up to the UE’s implementation the Tx side of the SL-PRS sends another provideAssistanceInformation SLPP message to information the Rx side of the update. No spec change required.

SL-RTT synch source

Proposal2: Add synchronization sources as assistance data in SL-RTT-ProvideAsssitanceData for SL-RTT. Adopt the TP in Annex A.

Measurement per ARP

Proposal3: Capture in the field description of SL-RTT-ProvideLocationInformation SLPP message that when the ARP for SL-PRS transmission and reception are different, the UE shall not report SL Rx-TX time difference measurement. Adopt the TP in the Annex B.

Proposal4: Support multiple per ARP measurement results for the same SL-PRS in a single report. Adopt the TP in Annex C.

Proposal5: Confirm that provideLocationInfomation can be provided in un-solicited manner and the concern raised from RAN1 is not an issue.

4	Reply LS to RAN4

3GPP TSG-RAN2 Meeting # 127 R2-240xxxx # Masstricht, Netherlands, 19 – 23 August, 2024 Title: Reply LS on synchronization source change at the transmitting anchor UE in SL positioning Release: Rel-18 Work Item: NR_pos_enh2-Core Source: 3GPP TSG-RAN WG2 To: 3GPP TSG-RAN WG4, 3GPP TSG-RAN WG1 Contact Person: Name: Yinghao Guo E-mail Address: yinghaoguo@huawei.com Attachments: None - Overall Description: RAN2 would like to thank RAN4 for informing the related issues and discussions related to SLPP spec. Regarding the following question, the answer is as follows: Question: RAN4 would like to check whether RAN1 and RAN2 have introduced or are working on any solutions to inform a UE performing an SL positioning measurement (e.g., SL RSTD, SL Rx-Tx, and SL RTOA) about synchronization reference source change at a UE which is transmitting SL-PRS for the measurement. Answer: RAN2 thinks that the current spec can already enable the Tx side UE to inform the Rx side UE the update in the synchronization source update when there is a change in the synchronization source in the Tx Side. No spec change is required. Furthermore, RAN2 notices that the synchronization source indication in the assistance data for SL-RTT is currently missing and RAN2 has already agreed to add it in the SL-RTT-ProvideAssistanceData in TS 38.355. 2. Actions: To RAN4: ACTION: RAN2 respectfully asks RAN4 to take the above answer into account. 3. Date of Next TSG-RAN WG2 Meetings: TSG-RAN WG4 Meeting #112-bis Hefei, China October 14 – October 18, 2024

5	Annex: SLPP Text proposal for RAN4 LS

=================================TP BEGINS=======================================

#### –	SL-RTT-ProvideAssistanceData

-- ASN1START

-- TAG-SL-RTT-PROVIDEASSISTANCEDATA-START

SL-RTT-ProvideAssistanceData ::= SEQUENCE {

...,

[[

syncSourceType-r18              ENUMERATED { gnss, gNB-eNB, ue}    OPTIONAL

]]

}

-- TAG-SL-RTT-PROVIDEASSISTANCEDATA-STOP

-- ASN1STOP

===============================TP ENDS==========================================

6	Annex: SLPP Text proposal for SL-RTT

===================================TP BEGINS=====================================

#### –	SL-RTT-ProvideLocationInformation

-- ASN1START

-- TAG-SL-RTT-PROVIDELOCATIONINFORMATION-START

SL-RTT-ProvideLocationInformation ::= SEQUENCE {

sl-RTT-SignalMeasurementInformation   SL-RTT-SignalMeasurementInformation    OPTIONAL,

sl-RTT-Error                          SL-RTT-LocationInformationError        OPTIONAL,

...

}

SL-RTT-MeasElementPerARP-ID-Rx ::= SEQUENCE (SIZE(1..4)) OF SL-RTT-MeasElement

SL-RTT-SignalMeasurementInformation ::= SEQUENCE {

sl-RTT-MeasList                         SEQUENCE (SIZE(1..maxNrOfUEs)) OF SL-RTT-MeasElementPerARP-ID-Rx,

...

}

SL-RTT-MeasElement ::= SEQUENCE {

applicationLayerID                    OCTET STRING              OPTIONAL,  -- Cond FirstElement

los-NLOS-Indicator                    LOS-NLOS-Indicator        OPTIONAL,  -- sl-losNlosIndicator

sl-POS-ARP-ID-Rx                      INTEGER (1..4)            OPTIONAL,  -- sl-pos-arpID-Rx

sl-PRS-RxTxTimeDiffMeasResult         SL-PRS-RxTxTimeDiffMeasResult,

...

}

SL-PRS-RxTxTimeDiffMeasResult ::= CHOICE {

single-SL-PRS-RxTxTimeDiff        SL-PRS-RxTxTimeDiffResult,

multiple-SL-PRS-RxTxTimeDiff          SEQUENCE {

sameSL-PRS-TxAndDiffSL-PRS-Rx         SEQUENCE (SIZE (2..4)) OF SL-PRS-RxTxTimeDiffResult    OPTIONAL,

sameSL-PRS-RxAndDiffSL-PRS-Tx         SEQUENCE (SIZE (2..4)) OF SL-PRS-RxTxTimeDiffResult    OPTIONAL

},

...

}

SL-PRS-RxTxTimeDiffResult ::= SEQUENCE {

sl-PRS-ResourceId             INTEGER (0..16)               OPTIONAL,  -- sl-PRS-ResourceId

sl-PRS-RxTxTimeDiffResult         CHOICE {

k0                                INTEGER (0..1970049),

k1                                INTEGER (0..985025),

k2                                INTEGER (0..492513),

k3                                INTEGER (0..246257),

k4                                INTEGER (0..123129),

k5                                INTEGER (0..61565)

}                                                           OPTIONAL,  -- sl-PRS-RxTxTimeDiff

sl-PRS-RSRP-Result            INTEGER (0..126)              OPTIONAL,  -- sl-PRS-RSRP

sl-PRS-RSRPP-Result           INTEGER (0..126)              OPTIONAL,  -- sl-PRS-RSRPP

sl-RTT-AdditionalPathList     SL-RTT-AdditionalPathList     OPTIONAL,

sl-TimeStamp                  SL-TimeStamp                  OPTIONAL,  -- sl-Timestamp

sl-TimingQuality              SL-TimingQuality              OPTIONAL,  -- sl-TimingQuality

tx-TimeInfo                   SL-TimeStamp                  OPTIONAL,  -- tx-Time-Info

...

}

SL-RTT-AdditionalPathList ::= SEQUENCE (SIZE(1..8)) OF SL-RTT-AdditionalPath

SL-RTT-AdditionalPath  ::= SEQUENCE {

sl-PRS-RxTxTimeDiffAdditionalPathResult    CHOICE {

k0                                         INTEGER (0..8191),

k1                                         INTEGER (0..4095),

k2                                         INTEGER (0..2047),

k3                                         INTEGER (0..1023),

k4                                         INTEGER (0..511),

k5                                         INTEGER (0..255)

}                                                                   OPTIONAL,  -- additionalPath-SL-PRS-Rx-Tx-TimeDiff

sl-PRS-AdditionalPathRSRPP-Result          INTEGER (0..126)         OPTIONAL,  -- additionalPath-SL-PRS-RSRPP

sl-TimingQuality                           SL-TimingQuality         OPTIONAL,  -- sl-TimingQuality

...

}

SL-RTT-LocationInformationError ::= ENUMERATED { undefined, assistanceDataMissing, notAllRequestedMeasurementsPossible, ... }

-- TAG-SL-RTT-PROVIDELOCATIONINFORMATION-STOP

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| FirstElement | The field is mandatory present in the first SL-RTT-MeasElement in IE SL-RTT-MeasElementPerARP-ID-Rx. Otherwise, it is not present. |

SL-RTT-ProvideLocationInformation field descriptions

los-NLOS-Indicator This field specifies the UE's best estimate of the LOS or NLOS of the UE measurements (including RSTD, RTOA, RSRP, RSRPP, AoA and UE Rx-Tx time difference).

sl-POS-ARP-ID-Rx This field indicates ARP ID of an ARP used for reception for per-ARP measurement reporting. The ARP ID is used to uniquely identify an ARP associated with a UE. For a requested SL Rx-Tx time difference measurement, if the ARP used for a SL-PRS transmission is different to the ARP used for a SL-PRS reception, the UE shall not report the SL Rx-Tx time difference measurement.

sl-PRS-ResourceId This field specifies the PRS resourde ID used for SL positioning measurements.

sl-PRS-RxTxTimeDiffResult This field specifies SL Rx-Tx time difference measurement based on first path of arrival. The mapping of the field is defined in TS 38.133 [13].

sl-PRS-RSRP-Result This field specifies the sidelink PRS reference signal received power (RSRP) measurement.

sl-PRS-RSRPP-Result This field specifies the SL-RSRPP measurement based on first path of arrival.

sl-RTT-AdditionalPathList This field specifies the sidelink PRS measurements based on additional path of arrival.

sl-TimeStamp This field specifies the time instance at which the SL Rx-Tx time difference and SL-PRS RSRP (if included) measurement is performed.

============================TP ENDS==============================================

7	Annex: SLPP text proposal for ARP

=================================TP BEGINS=======================================

#### –	SL-AoA-ProvideLocationInformation

-- ASN1START

-- TAG-SL-AOA-PROVIDELOCATIONINFORMATION-START

SL-AoA-ProvideLocationInformation ::= SEQUENCE {

sl-AoA-SignalMeasurementInformation   SL-AoA-SignalMeasurementInformation    OPTIONAL,

sl-AoA-Error                          SL-AoA-LocationInformationError        OPTIONAL,

...

}

SL-AoA-SignalMeasurementInformation ::= SEQUENCE {

dummy                         SEQUENCE (SIZE(1.. maxNrOfUEs)) OF SL-AoA-MeasElementPerARP-ID-Rx,

...,

[[

sl-AOA-MeasList-vxy00                        SEQUENCE (SIZE(1.. maxNrOfUEs)) OF SL-AoA-MeasElementPerARP-ID-Rx

]]

}

SL-AoA-MeasElementPerARP-ID-Rx-vxy00 ::= SEQUENCE (SIZE(1..4)) OF SL-AoA-MeasElement-vxy00

SL-AoA-MeasElement-vxy00 ::= SEQUENCE {

applicationLayerID                    OCTET STRING              OPTIONAL,  -- Cond FirstElement

sl-PRS-ResourceId                     INTEGER (0..16)           OPTIONAL,  -- sl-PRS-ResourceId

sl-LCS-GCS-Translation                LCS-GCS-Translation       OPTIONAL,  -- Cond FirstElement

sl-AoA-MeasElementPerARP-List             SEQUENCE (SIZE (1..4)) OF SL-AOA-MeasElementPerARP    OPTIONAL,

...

}

SL-AOA-MeasElementPerARP ::= SEQUENCE {

los-NLOS-Indicator                    LOS-NLOS-Indicator        OPTIONAL,  -- sl-losNlosIndicator

sl-AngleQuality                       MeasurementAngleQuality   OPTIONAL,  -- sl-AngleQuality

sl-AoA-AdditionalPathList             SL-AoA-AdditionalPathList OPTIONAL,

sl-AzimuthAoA-Result                  INTEGER (0..3599)         OPTIONAL,  -- sl-PRS-AoA

sl-POS-ARP-ID-Rx                      INTEGER (1..4)            OPTIONAL,  -- sl-pos-arpID-Rx

sl-PRS-RSRP-Result                    INTEGER (0..126)          OPTIONAL,  -- sl-PRS-RSRP

sl-PRS-RSRPP-Result                   INTEGER (0..126)          OPTIONAL,  -- sl-PRS-RSRPP

sl-TimeStamp                          SL-TimeStamp              OPTIONAL,  -- sl-Timestamp

sl-ZenithAoA-Result                   INTEGER (0..1799)         OPTIONAL,  -- sl-PRS-AoA

...

}

SL-AoA-MeasElement ::= SEQUENCE {

applicationLayerID                    OCTET STRING              OPTIONAL,  -- Cond FirstElement

sl-LCS-GCS-Translation                LCS-GCS-Translation       OPTIONAL,  -- Cond FirstElement

los-NLOS-Indicator                    LOS-NLOS-Indicator        OPTIONAL,  -- sl-losNlosIndicator

sl-AngleQuality                       MeasurementAngleQuality   OPTIONAL,  -- sl-AngleQuality

sl-AoA-AdditionalPathList             SL-AoA-AdditionalPathList OPTIONAL,

sl-AzimuthAoA-Result                  INTEGER (0..3599)         OPTIONAL,  -- sl-PRS-AoA

sl-POS-ARP-ID-Rx                      INTEGER (1..4)            OPTIONAL,  -- sl-pos-arpID-Rx

sl-PRS-ResourceId                     INTEGER (0..16)           OPTIONAL,  -- sl-PRS-ResourceId

sl-PRS-RSRP-Result                    INTEGER (0..126)          OPTIONAL,  -- sl-PRS-RSRP

sl-PRS-RSRPP-Result                   INTEGER (0..126)          OPTIONAL,  -- sl-PRS-RSRPP

sl-TimeStamp                          SL-TimeStamp              OPTIONAL,  -- sl-Timestamp

sl-ElevationAoA-Result                INTEGER (0..1800)         OPTIONAL,  -- sl-PRS-AoA

...

}

SL-AoA-AdditionalPathList ::= SEQUENCE (SIZE(1..2)) OF SL-AoA-AdditionalPath

SL-AoA-AdditionalPath  ::= SEQUENCE {

sl-AngleQuality                            MeasurementAngleQuality   OPTIONAL,  -- sl-AngleQuality

sl-AzimuthAoA-AdditionalPathResult         INTEGER (0..3599)         OPTIONAL,  -- additionalPath-SL-PRS-AoA

sl-ElevationAoA-AdditionalPathResult       INTEGER (0..1800)         OPTIONAL,  -- additionalPath-SL-PRS-AoA

sl-PRS-AdditionalPathRSRPP-Result          INTEGER (0..126)          OPTIONAL,  -- additionalPath-SL-PRS-RSRPP

...

}

MeasurementAngleQuality ::= SEQUENCE {

azimuthQuality              INTEGER (0..255),

elevationQuality            INTEGER (0..255)        OPTIONAL

}

SL-AoA-LocationInformationError ::= ENUMERATED { undefined, assistanceDataMissing, notAllRequestedMeasurementsPossible, ... }

-- TAG-SL-AOA-PROVIDELOCATIONINFORMATION-STOP

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| FirstElement | The field is mandatory present in the first SL-AoA-MeasElement in IE SL-AoA-MeasElementPerARP-ID-Rx. Otherwise, it is not present. |

SL-AoA-ProvideLocationInformation field descriptions

los-NLOS-Indicator This field specifies the UE's best estimate of the LOS or NLOS of the UE measurements (including RSTD, RTOA, RSRP, RSRPP, AoA and UE Rx-Tx time difference).

sl-AngleQuality This field specifies the angle quality for measurement results reported. Scale factor 0.1 degree.

sl-AoA-AdditionalPathList This field specifies the sidelink PRS measurements based on additional path of arrival.

sl-AzimuthAoA-LCS-GCS-Translation This field provides the angles α (bearing angle), β (downtilt angle) and γ (slant angle) for the translation of a Local Coordinate System (LCS) to a Global Coordinate System (GCS) as defined in TR 38.901 [8].

sl-AzimuthAoA-Result This field specifies the first path result of azimuth angle of arrival. Scale factor 0.1 degree; range 0 to 359.9 degrees.

sl-ElevationAoA-LCS-GCS-Translation This field provides the angles α (bearing angle), β (downtilt angle) and γ (slant angle) for the translation of a Local Coordinate System (LCS) to a Global Coordinate System (GCS) as defined in TR 38.901 [8].

sl-ElevationAoA-Result This field specifies the first path result of elevation angle of arrival. Scale factor 0.1 degree; range 0 to 180 degrees.

sl-POS-ARP-ID-Rx This field indicates ARP ID of an ARP used for reception for per-ARP measurement reporting. The ARP ID is used to uniquely identify an ARP associated with a UE.

sl-PRS-ResourceId This field specifies the PRS resource ID used for SL positioning measurements.

sl-PRS-RSRP-Result This field specifies the sidelink PRS reference signal received power (RSRP) measurement.

sl-PRS-RSRPP-Result This field specifies the SL-RSRPP measurement based on first path of arrival.

sl-TimeStamp This field specifies the time instance at which the SL-PRS RSRP (if included) measurement is performed.

============================TP ENDS==============================================