3GPP TSG-RAN2 Meeting # 127 	R2-2406298

# Masstricht, Netherlands,  19 – 23 August, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 1879 | rev | - | Current version: | 17.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on PPW for MAC spec |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSIlicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh-Core |  |  |  |  |  | Date: |  |  | 19-08-2024 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For PPW, priority and type are defined: - priority indicates the prioiry between PRS and the other DL channels and signals; - type indicates whether the transmission on the other chanenls on the same cc, on the other cc within FR or band are affected. In the description of the PPW and its affected symbols, the current spec only considers prioirty while might not well considered the type in the PPW configuration/UE procedure. For the explanation on the priority and type, the RAN1 spec TS 38.214 has the following explanation already: About priority The UE is expected to measure the DL PRS outside the measurement gap, subject to UE capability, if the DL PRS is inside the active DL BWP and has the same numerology as the active DL BWP and is within the DL PRS processing window indicated by higher layer parameter DL-PPW-PreConfig. The UE is not expected to measure the DL PRS outside the measurement gap if the expected received timing difference between the DL PRS from the non-serving cell and that from the serving cell, determined by the higher layer parameters nr-DL-PRS-ExpectedRSTD and nr-DL-PRS-ExpectedRSTD-Uncertainty, is larger than maximum Rx timing difference provided by UE capability. For receiving the DL PRS outside the measurement gap and within the DL PRS processing window, the priority between DL PRS and SSB is defined in [11, TS 38.133] and the UE determines the DL PRS priority as indicated by higher layer parameter priority subject to UE capability or as implied by UE capability, except for SSB: - with value 'st1' where the DL PRS is higher priority than all the DL signals and channels, or - with value 'st2' where the DL PRS is lower priority than PDCCH and the PDSCH scheduled by DCI formats 1_1 or 1_2 or 4_2 with the priority indicator field in the corresponding DCI format set to 1, and is higher priority than other DL signals and channels, or - with value 'st3' where the DL PRS is lower priority than all the DL signals and channels.. About type When the UE is expected to measure the DL PRS outside the measurement gap in a configured DL PRS processing window with type1A and if the DL PRS is determined to be higher priority than the DL signals and channels inside the DL PRS processing window, those DL signals and channels are not expected to be measured by the UE. When the UE is expected to measure the DL PRS outside the measurement gap in a configured DL PRS processing window with type1B and if the DL PRS is determined to be higher priority than the DL signals and channels inside the DL PRS processing window, those DL signals and channels in the same band as the DL PRS are not expected to be measured by the UE. When the UE is expected to measure the DL PRS outside the measurement gap in a configured DL PRS processing window with type2 if the DL PRS is determined to be higher priority than the DL signals and channels inside the DL PRS processing window, those DL signals and channels from the impacted serving cells are not expected to be measured by the UE on the overlapped symbols with the DL PRS, where impacted serving cells refer to the serving cell on which the DL-PPW-PreConfig is configured for a frequency range 1 band, and all the serving cells in the same band as the DL PRS for a frequency range 2 band. When the UE is expected to measure the DL PRS outside the measurement gap in a configured DL PRS processing window with type1B or type2, and if the DL PRS is determined to be higher priority than the DL signals and channels inside the DL PRS processing window, the UE behavior is described in [11, TS 38.133] for inter-band case for frequency range 2 for the DL signals/channels from a different frequency range 2 band than the frequency range 2 band of the DL PRS. ===omitted spec==. For this, we think that it is fine to leave this to RAN1 and remove the wording related to priority in MAC spec, such that the spec is consistent in the description of the priority and type |
| Summary of change: |  | 1/ Remove the part "and PRS has higher priority than DL channels and signals" |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 1/ it is not clear why there is RAN2 description for priority, but not for type. With the PRS has higher priority than other channels, the desctiption is duplicated with the wording "for the affected symbols" Inter-Operability analysis: Impacted functionality: PPW Inter-operability: There is no inter-operability issue for the CR. The correct UE behavior can be inferred from the other parts of the spec and other specs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.24 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

===================================CHANGE BEGINS====================================

## 5.24	Handling of PRS Processing Window

When PPW is activated, for the affected symbols within the PPW according to clause 5.1.6.5 in TS 38.214 [7], the MAC entity shall:

1>	if the ra-ResponseWindow or the ra-ContentionResolutionTimer or the msgB-ResponseWindow is running:

2>	monitor the PDCCH as specified in clauses 5.1.4 and 5.1.5.

1>	else:

2>	not receive DL-SCH;

2>	not receive PDCCH.

====================================CHANGE ENDS====================================