3GPP TSG-RAN WG2 Meeting #127	R2-2406301

Maastricht, Netherlands, 19th– 23rd August, 2024

Agenda item:	7.5.3

Source: 	Huawei, HiSilicon

Title: 	Remaining issues for DRX operation for XR

Document for:	Discussion and Decision

1. Introduction

During the last RAN2 meeting, the initialization procedure of the DRX_SFN_COUNTER was discussed and clarified. But a remaining issue about the handover case was raised. In this contribution, we will discuss the issue and give our view.

2. DRX initialization in HO

RAN2 has agreed that the DRX_SFN_COUNTER is initialized when the UE applies the DRX configurations. The corresponding procedural text in TS 38.321 is excerpted.

1> if the drx-NonIntegerLongCycleStartOffset is configured: 2> increment DRX_SFN_COUNTER by 1 in the first symbol of a slot in which SFN changes to 0; 2> if DRX is (re-)configured by RRC: 3> if drx-TimeReferenceSFN is included in the RRC (re-)configuration which is received during the first half of a hyper frame (i.e., SFN is between 0 and 511): 4> set DRX_SFN_COUNTER to 1. 3> else: 4> set DRX_SFN_COUNTER to 0.

Depending on when the DRX configuration is received, the UE initializes the DRX_SFN_COUNTER to 0 or 1 accordingly to keep aligning between the UE and the gNB. To achieve the alignment, there is a precondition that assumes the time gap between the gNB sending the DRX configuration and the UE receiving the DRX configuration is shorter than half of a hyper frame, so that it is sure that the SFN can at most wrap around for one time.

In case of handover, the above precondition may be not met. Especially for LTM and CHO, the DRX configuration is provided by the target gNB in advance, while the handover may be executed after a long period of time, which means that the SFN may have wrapped around for a number of times when the UE initializes the DRX_SFN_COUNTER. In this case, the DRX_SFN_COUNTER maintained by the UE and the gNB will be misaligned. The issue was illustrated in Fig.1. This will impact the UE power saving gain and the network capacity. The target gNB may try to schedule the UE while the UE is not monitoring PDCCH. If this happens, the scheduled resource will be wasted. On the contrary, when the UE monitors PDCCH, the target gNB may not schedule it, making the consumed power wasted.

Fig. 1 The issue for DRX initialization in case of HO.

However, the issue can be resolved by the NW implementation. For example, the target gNB can re-configure the UE with non-integer periodicity DRX after the handover finishes, so that the DRX_SFN_COUNTER will not be impacted by the handover procedure. As another alternative, the target gNB may determine the value of the DRX_SFN_COUNTER after receiving the RRC reconfiguration complete message from the UE.  The solutions are simple and effective. There is no need to specify any enhancement.

Proposal 1: The issue of DRX initialization in case of handover can be resolved by NW implementation. There is no need for enhancement.

3. Conclusion

In this contribution, the following proposals were made:

Proposal 1: The issue of DRX initialization in case of handover can be resolved by NW implementation. There is no need for enhancement.

4. Reference

[1]. 3GPP TS 38.321: "NR; Medium Access Control (MAC) protocol specification".