3GPP TSG-RAN WG2 Meeting #127	R2-2406302

Maastricht, Netherlands, 19th– 23rd August, 2024

Agenda item:	8.7.2

Source: 	Huawei, HiSilicon

Title: 	Discussion on multi-modal XR

Document for:	Discussion and Decision

1. Introduction

During the last meeting, RAN2 has sent an LS to SA2 for multi-modal information awareness. Before getting the reply, RAN2 can also study some potential multi-modal enhancements within RAN2 scope simultaneously. In this contribution, we will give our views.

Besides, we will also have some discussion on the other potential aspects related to multi-modal XR, including enhancements for haptic scheduling and DRX.

2. Discussion

2.1	RAN awareness

During RAN2#126 meeting, RAN2 discussed the potential benefits of RAN being aware of the multi-modal association between traffic flows.  According to the discussion, the multi-modal association information knowledge at RAN  can be used to improve the user experience and the network capacity from the following potential aspects.

- Joint admission control

If the MMSID which identifies the multiple flows associated to the same multi-modal service from the CN can be provided to RAN during the PDU session setup/modification procedures, it can be useful for admission control. In this way, multiple flows needed by the same multi-modal service can be accepted together to guarantee the service. On the other hand, if there is no enough resource for the service, the gNB can reject all the associated flows to avoid unnecessary resource waste.

Besides PDU session setup, the association information is also useful during NG-RAN handover. Both the MMSID from the CN and the information provided by the UE can be used for the handover scenario. For example, the source gNB can choose a proper destination for the multi-modal XR UE by considering potential results of admission control at the target gNB. With this association information, the target gNB can perform joint admission control with the purpose to guarantee the multi-modal service experience during UE mobility.

- QoS flow level synchronization

According to the previous study in SA2/SA4, there are some synchronization requirements between different data associated with a multi-modal service. Some detailed use cases and synchronization thresholds can be found in TR 22.847 [2] and TS 22.261 [3]. To satisfy the E2E synchronization requirements, some enhancements may be considered to guarantee the synchronized transmission at RAN.

One alternative is to perform synchronization at the QoS flow level. Specifically, the CN or the UE may provide the synchronization thresholds to RAN together with the QoS flow association information. Then RAN can consider such thresholds when determining the scheduling strategy or DRB mapping rules to avoid too large delay difference between the inter-dependent data. For example, RAN can try to guarantee the same delay budget for the associated QoS flows to ensure small delay difference. It can be left to the NW implementation on how to perform the synchronization using the association and thresholds information.

- Packet level synchronization

In the above alternative, RAN is not aware of the dynamic inter-dependency among packets from different flows. Synchronization is only considered based on the QoS flow level. If the dynamic inter-dependency among packets can be identified, RAN may be able to further perform more precise scheduling to guarantee that the transmission of each packet satisfies the synchronization threshold.

More specifically, for DL, the packet level dependency information may be provided by the CN, e.g. via GTP-U. How the gNB perform DL scheduling considering the dependency can be left to NW implementation. For UL, the packet level dependency may be identified by the UE. However, since the air interface is the first hop for uplink transmission, it may not be necessary to ensure the synchronization at RAN. Furthermore, it is also very complex to specify such packet level synchronisation for both UL and DL and thus we believe this alternative should be excluded at the moment.

Due to the advantages mentioned above, RAN2 decided to support Multi-Modality awareness in RAN in Rel-19 for both UL and DL. It can be seen that the multi-modal association information can at least be used for joint admission control with very little specifications impact, since how to use the multi-modal association information at RAN can be left to gNB implementation. With the enhancement, the user experience and network capacity may be improved. Moreover, if the synchronization requirements and the inter-dependency can be provided, RAN may further perform QoS flow level synchronization based on implementation.

Proposal 1: The multi-modal information is useful on the following aspects: (a) The multi-modal association information (i.e. MMSID) can be used for joint admission control at least. (b) If the synchronization thresholds are provided in addition, synchronization on QoS flow level or packet level may be performed based on NW implementation.

During the last meeting, RAN2 has sent the LS [1] to SA2 to ask whether/what multi-modal information can be provided to RAN. Multi-Modal Service ID (MMSID) and synchronization thresholds are given as some examples in the LS. Although RAN2 did not receive the reply from SA2, RAN2 can continue to study the potential enhancements for multi-modal XR within RAN2 scope.

For the purpose of RAN awareness for multi-modal XR, it is also possible to obtain the multi-modal association information from the UE side. The UE may obtain the multi-modal association information from the APP client directly based on UE implementation. Then it can report the association information to RAN, e.g., via UAI, similar to R18 report for PDU set identification, jitter, etc.

More specifically, the association information provided by the UE can be similar to the MMSID defined by SA2, i.e. the UE may indicate which flows are associated to the same multi-modal service so that RAN can take it into account when handling the QoS flows.

Apart from the association information, RAN2 can discuss whether the other multi-modal information can also be provided to RAN. For example, for some applications, it may be also possible for the UE to get the synchronization thresholds between different flows.

Proposal 2: The multi-modal information (at least the association information similar to MMSID) can also be sent from the UE to the gNB.

2.2 Scheduling enhancements for haptic

Haptic service is an important component for multi-modal XR. It provides the real-world-like experience to users in immersive applications such as VR games and metaverse services. While according to TS 22.261 [3], there are quite strict KPI requirements for haptic data, as excerpted below. It can be seen that the haptic traffic asks for very stringent PDB and high reliability.

## 7.11 KPIs for tactile and multi-modal communication service The 5G system shall support tactile and multi-modal communication services with the following KPIs. Table 7.11-1: Multi-modal communication service performance requirements Use Cases; Characteristic parameter (KPI); Influence quantity; Remarks. Max allowed end-to-end latency; Service bit rate: user-experienced data rate; Reliability; Message size (byte); UE Speed; Service Area. Immersive multi-modal VR (UL: device application sever); 5 ms (note 2); 16 kbit/s -2 Mbit/s (without haptic compression encoding); 0.8 - 200 kbit/s (with haptic compression encoding); 99.9% (without haptic compression encoding) 99.999% (with haptic compression encoding) [40]; 1 DoF: 2-8 3 DoFs: 6-24 6 DoFs: 12-48 More DoFs can be supported by the haptic device; Stationary or Pedestrian; typically < 100 km2 (note 5); Haptic feedback.

Meanwhile, the burst size and the periodicity of a haptic stream can be unpredictable and irregular. According to TR 22.847 [2], the sampling rate of haptic devices may reach 1000 times per second, and there can be up to 60 sensors, which is quite frequent and can lead to large amount of data at the same time. The sampled data may be compressed by IEEE 1918.1 [4]. With the compression, it is uncertain whether the data would be transmitted at each sampling point. As illustrated in Fig.1, a haptic sensor may sample the haptic signal every millisecond. But with the compression, the sample data would not be transmitted unless the difference between its value and the last transmitted value is larger than a threshold. If considering multiple sensors, it is possible that there is a large amount of haptic data to be transmitted at the same time (e.g., all the sensors want to convey the sampled data), while there is few haptic data at some other time (e.g., only few sensors want to convey the sampled data).

Fig.1 Haptic traffic model with compression.

With the above characteristics, the existing scheduling mechanisms are not suitable for transmitting UL haptic data. On the one hand, DG may be unable to meet the short PDB (i.e. 5ms) due to the procedure of BSR and SR. On the other hand, it is difficult to accommodate the varying data volume using CG. To ensure the in-time transmission, the CG resource size should be sufficient to accommodate the largest possible burst size. But this will lead to a waste of resource most of the time since the actual haptic data volume will usually be smaller. The R18 multi-PUSCH CG may not resolve the issue as well since it would be difficult for the gNB to reallocate the unused CG resources considering the short periodicity and PDB.

Observation 1: Legacy DG/CG may be not efficient for the transmission of haptic data which have stringent PDB, unpredictable burst size and irregular periodicity.

Besides, the frequent and irregular haptic data burst may disturb the transmission of video/audio data and make the network inefficient. For example, if the haptic data and the video data are transmitted within the same MAC PDU, to guarantee the high reliability for the haptic data, the network would have to allocate more reliable resources. However, the video data do not ask for such a high reliability, and the allocated resource is unnecessary for video and somehow wasted. If the video data and the haptic data are restricted to be transmitted within different MAC PDUs, considering the frequent arrival of haptic data, there may be little chance for video transmission, which leads to unexpected delay.

Observation 2: The transmission of haptic data may impact the transmission of video and audio, and decrease the resource efficiency.

Fig.2 provides a simulation result comparing the network capacity when the XR services are enabled with or without haptic traffic. The video and haptic traffic are modelled according to TR 38.838, TR 22.847 and TS 22.261. It can be seen that the network capacity for XR would be decreased from 7.5 UEs to 2.8 UEs (60% off) if haptic is supported without any scheduling enhancements.

Video modal: 30Mbps, 60fps, 99%@10ms; [TR 38.838 & TS 22.261] Haptic modal: 60 sensors, up to 200kbps per sensor, 2kHz with compression, 99.999%@5ms; [TR 22.847 & TS 22.261] The network capacity is evaluated using the metric that 90% UEs are satisfied.

Fig.2 Simulation on network capacity comparing haptic traffic included or not.

Observation 3: Under the existing scheduling mechanism, the network capacity may be decreased if there is haptic traffic.

To avoid the capacity loss caused by haptic data, RAN may need to investigate some scheduling enhancements. One potential direction is to enhance the CG mechanism. For example, apart from the CG configured for each UE specifically, RAN can additionally configure some CG resource which can be shared by multiple UEs. When the specific CG resource is not large enough, the UE can use the sharing resource and indicate this to the gNB, e.g. via UCI. Since the data of the multiple UEs arrive randomly, from the statistic point of view, the sharing resource would be always used by some of the UEs and it will not cause huge waste. It can provide additional resource to guarantee the delay for the UE when the haptic size is large.

Considering RAN1 is more familiar with the traffic model and the physical resource allocation, RAN2 can also ask RAN1 to evaluate the existing scheduling mechanism to see if it can well support the multi-modal XR services with haptic data. So that RAN can decide whether any enhancement is needed for haptic.

Proposal 3: Scheduling enhancements are needed to support multi-modal XR services with haptic data from the haptic KPI and the network capacity point of view.

2.3 Multiple DRX configurations

In R19 study, simultaneously active multiple DRX configurations has been proposed as a candidate solution for XR enhancement.

The intention is to match the multiple DRX on-durations with different flows with different periodicities by configuring multiple DRX corresponding to multiple flows. However, the idea was not pursued in R18 because it is believed that the network can configure a proper DRX configuration by comprehensively considering the characteristics of multiple flows. For example, as shown in the following figure, if there is a video flow with periodicity 16.67ms and PDB 10ms, and an audio flow with periodicity 10ms and PDB 30ms, at the same time. The DRX cycle can be configured to 16.67 ms, and the audio traffic will experience at most 13.33ms delay, which is tolerable.

Fig. 3 Single DRX configuration is able to deal with multiple flows.

Observation 4: A proper single DRX configuration can satisfy the requirements of multiple flows.

Some simulation evaluations were provided in R2-2404512 [5]. Some of the results are excerpted below.

Table 1 Results for 8 UEs/cell and VR multi-stream traffic: DL video (60 fps, 30 Mbps, ±4 ms jitter, 10 ms PDB), DL audio (10 ms periodicity, 30 ms PDB), UL pose (4 ms periodicity, 10 ms PDB) Power Saving Scheme; DRX cycle (ms); drx-onDurationTimer (ms); drx-InactivityTimer (ms); % of satisfied UE; Mean PSG of all UEs; Mean PSG of satisfied UEs. Always On; -; -; -; 92.1%; -; -. R15/16 Long DRX; 10; 8; 4; 85.5%; 5.7%; 5.8%. R15/16 Short DRX; 4; 2; 4; 82.9%; 6.7%; 6.8%. Rel-18 non-integer DRX; 16.66; 10; 4; 89.6%; 8.1%; 8.2%. Multiple active DRX; 16.66; 10; 4; 91.1%; 6.0%; 6.0%. 10; 2; 4. Table 2 Results for 8 UEs/cell and VR multi-stream traffic: DL video (60 fps, 30 Mbps, ±4 ms jitter, 10 ms PDB), DL audio (10 ms periodicity, 10 ms PDB), UL pose (4 ms periodicity, 10 ms PDB) Power Saving Scheme; DRX cycle (ms); drx-onDurationTimer (ms); drx-InactivityTimer (ms); % of satisfied UE; Mean PSG of all UEs; Mean PSG of satisfied UEs. Always On; -; -; -; 92.1%; -; -. R15/16 Long DRX; 10; 8; 4; 85.5%; 5.7%; 5.8%. R15/16 Short DRX; 4; 2; 4; 82.9%; 6.7%; 6.7%. Rel-18 non-integer DRX; 16.66; 10; 4; 63.5%; 8.1%; 14.0%. Multiple active DRX; 16.66; 10; 4; 91.1%; 6.0%; 6.0%. 10; 2; 4. Table 3 Results for 5 UEs/cell and VR multi-stream traffic: DL video (30 fps, 30 Mbps, ±4 ms jitter, 10 ms PDB), DL audio (10 ms periodicity, 10 ms PDB), UL pose (4 ms periodicity, 10 ms PDB) Power Saving Scheme; DRX cycle (ms); drx-onDurationTimer (ms); drx-InactivityTimer (ms); % of satisfied UE; Mean PSG of all UEs; Mean PSG of satisfied UEs. Always On; -; -; -; 94.3%; -; -. R15/16 Long DRX; 10; 8; 4; 89.0%; 6.3%; 6.3%. R15/16 Short DRX; 4; 2; 4; 87.4%; 8.1%; 8.1%. Rel-18 non-integer DRX; 33.33; 10; 4; 0%; 19.6%; -. Multiple active DRX; 33.33; 10; 4; 93.4%; 14.1%; 14.3%. 10; 2; 0.

Although in the excerpted Table 2 and Table 3, the multiple active DRX shows large benefit comparing to the R18 DRX enhancements, however, it is not convincing since the simulation assumption is not set for proper comparison. In Table 2 and Table 3, the PDB for DL audio was assumed as 10ms, while the R18 non-integer DRX cycle was configured as 16.66ms and 33.33ms, which does not allow for fair comparison. To perform the simulation, the single DRX cycle should be configured considering both the requirements of video and audio. With the assumption in Table 2 and Table 3, the R18 DRX should be configured with DRX cycle 10ms.

The simulation in the excerpted Table 1 seems much more reasonable. It has the same assumption as Fig.3. From the results, it can be seen that comparing to R18 non-integer DRX, the multiple active DRX solution even decreases the power saving gain from 8.1% to 6.0%. Meanwhile, the capacity gain obtained by multiple active DRX is quite limited. Comparing to R18 DRX enhancements, the capacity is only improved from 89.6% to 91.1%, with only 1.7% improvement. However, with two active DRX configured, there is a larger energy consumption for the UE and cost does not merit the benefits.

Observation 5: Multiple active DRX will decrease the power saving gain while bring limited capacity improvement.

Based on the above analysis, we have the following proposal.

Proposal 4: RAN2 not to consider multiple DRX enhancements for multi-modal XR.

3. Conclusion

In this contribution, we analyzed the potential aspects for enhancements related to multi-modal XR. The following observations and proposals were made:

RAN awareness:

Proposal 1: The multi-modal information is useful on the following aspects: (a) The multi-modal association information (i.e. MMSID) can be used for joint admission control at least. (b) If the synchronization thresholds are provided in addition, synchronization on QoS flow level or packet level may be performed based on NW implementation.

Proposal 2: The multi-modal information (at least the association information similar to MMSID) can also be sent from the UE to the gNB.

Scheduling enhancements for haptic:

Proposal 3: Scheduling enhancements are needed to support multi-modal XR services with haptic data from the haptic KPI and the network capacity point of view.

Multiple DRX configurations:

Proposal 4: RAN2 not to consider multiple DRX enhancements for multi-modal XR.

4. Reference

[1]. R2-2405782, Nokia (Rapporteur), LS on multi-modality awareness at RAN.

[2]. 3GPP TR 22.847: "Study on supporting tactile and multi-modality communication services; Stage 1".

[3]. 3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

[4]. IEEE 1918.1 “Tactile Internet” Standards Working Group and its Standards.

[5]. R2-2404512, Ericsson, Discussion on multi-modality.