3GPP TSG-RAN WG2 Meeting #127	R2-2406303

Maastricht, Netherlands, 19th– 23rd August, 2024

Agenda item:	8.7.1.1

Source: 	Huawei, HiSilicon

Title: 	Discussion on incoming LSs for R19 XR

Document for:	Discussion and Decision

1. Introduction

RAN2 has received some LSs from SA2 and RAN3. In this contribution, we will analyse the questions and give our view on the potential replies.

2. Discussion

2.1 Discussion on S2-2407351

In S2-2407351 [1], SA2 has asked some question about the dynamic information that may be provided to RAN. Question 1 asks for feedback on dynamic indication of the periodicity. The question is excerpted as below.

- Question1 [for SA4, RAN2 and RAN3]: - SA2 discusses indicating periodicity via in-band signaling (i.e. in GTP-U) for dynamic changes of the periodicity and kindly asks RAN2 and RAN3 to feedback on that approach. - To SA4: is it possible for application server to provide the periodicity to the PSA UPF in RTP header extension?

In our view, the question can be analysed considering the following two cases.

- Case 1: The periodicity changes slowly.

In R18, it was assumed that the XR traffic periodicity would not change frequently. If the assumption still holds for R19, then there is no need to indicate the periodicity information dynamically. The existing TSCAI mechanism is enough to provide the periodicity information to RAN and the value can be updated when the periodicity changes.

Proposal 1: TSCAI is enough to provide the periodicity information if it doesn’t change frequently.

- Case 2: The periodicity changes quickly.

If the periodicity can change frequently in R19, the situation will be somehow different. In this case, the updating of TSCAI via CP signalling is not so efficient to provide the latest periodicity information to RAN. However, we doubt whether it is proper to define the periodicity for such case. If the traffic periodicity changes quickly, it seems more proper to regard the traffic as aperiodic.

We noticed that SA2 also discussed about dynamically indicating the “time to next burst” to RAN, as explained in question 2 in the LS.

- Question2 [for SA4 and RAN2]: There is some discussion about time to next burst. - To SA4: is it possible that the application server provides the time to next burst (i.e. the time interval between the current burst and the next burst) in the 1st packet of the burst via N6? - Does RAN2 think the time to next burst is useful for RAN resource scheduling, especially with regard to the possible jitter between the NG-RAN and the Application Server?

In our view, such information has the same intention as the dynamic periodicity indication. We do not see the necessity to support multiple solutions for the same issue. Considering the definition of periodicity, it seems more appropriate to use the time to next burst if the traffic is aperiodic or the periodicity changes quickly.

The information is beneficial for RAN resource scheduling and UE power saving. From the resource scheduling point of view, given the time to the next burst, RAN can perform resource scheduling for the next burst in advance, so that a short delay can be guaranteed, which is meaningful for the delay-critical XR services. If DRX is configured, RAN may also consider the information to avoid large transmission delay. For example, if the next burst will arrive after a short period of time, RAN can proactively extend the DRX inactivity timer by sending some DL packets. Then the next burst can be transmitted in time, instead of waiting for the next DRX on-duration. If the information shows that the traffic is aperiodic, the gNB can decide not to use the periodic scheduling methods such as SPS, to avoid delay or resource waste.

From UE power saving point of view, RAN can indicate the UE to go to sleep before the next burst arrival, e.g. via dynamic PDCCH skipping. If DRX is configured, WUS can be used to skip some of the DRX on-durations if the burst arrival interval is larger than the DRX periodicity. In this way, the power saving gain can be obtained. Since the XR devices usually have limited batteries, improving the power saving gain is important.

Proposal 2: The time to next burst is useful for RAN resource scheduling and UE power saving in case the traffic is aperiodic or the periodicity changes frequently.

2.2 Discussion on R3-243957

In R3-243957 [2], RAN3 asks RAN2 about the UE behaviour for PSI based discard in case of DC, as following.

# 1. Overall Description: RAN3 is currently discussing XR DC signaling enhancements for various bearer types. In case of NR-DC split bearer, MN DU and SN DU congestion is local to each node. Based on congestion situation in MN DU and/or SN DU, each of the nodes independently send PSI-Based SDU Discard Activation/Deactivation MAC CE to UE for activation/deactivation of UL PSI based PDU set discarding of the split bearer. when UE receives MAC CE from both nodes or from a single node, it is not clear how UE will activate/deactivate UL PSI based PDU set discarding when split bearer is configured. Thus, RAN3 kindly asks RAN2 to clarify the UE behavior for handling uplink PSI based PDU discarding when MN and/or SN node sends the PSI-Based SDU Discard Activation/Deactivation MAC CE. # 2. Actions: # To RAN2: ACTION: RAN3 kindly asks RAN2 to provide feedback to the above question.

According to TS 38.321 [3], when the UE receives the PSI-Based SDU Discard Activation/Deactivation MAC CE, it will activate/deactivate the PSI-based discard for the DRBs configured with PSI-based discard and with RLC entity(ies) associated with the MAC entity. The specification is excerpted as below.

6.1.3.73 PSI-Based SDU Discard Activation/Deactivation MAC CE The PSI-Based SDU Discard Activation/Deactivation MAC CE is identified by MAC subheader with an one-octet eLCID as specified in Table 6.2.1-1b. It has a fixed size and consists of one octet defined as follows (Figure 6.1.3.73-1): - Di: This field indicates the activation/deactivation status of the PSI-based SDU discard of DRB i, where i is the ascending order of the DRB ID among the DRBs configured with discardTimerForLowImportance and with RLC entity(ies) associated with this MAC entity. The Di field set to 1 indicates that the PSI-based SDU discard shall be activated for DRB i. The Di field set to 0 indicates that the PSI-based SDU discard shall be deactivated for DRB i. Figure 6.1.3.73-1: PSI-Based SDU Discard Activation/Deactivation MAC CE

In case of split DRB, the DRB is associated to both the RLC entity at the MN side and the RLC entity at the SN side, so that no matter whether the UE receives the MAC CE from the MN or the SN, it will perform the activation/deactivation accordingly.

Observation1: According to the current RAN2 specification, for a certain DRB, the PSI-based SDU discard can be activated/deactivated by the MAC CE received from either the MN or SN.

Proposal 3: RAN2 to reply to RAN3 that, for a certain DRB, the PSI-based SDU discard functionality can be activated/deactivated by the MAC CE received from either the MN or SN.

3. Conclusion

In this contribution, we analyzed the question asked by SA2 in the incoming LSs. The following proposals were made:

Discussion on S2-2407351:

Proposal 1: TSCAI is enough to provide the periodicity information if it doesn’t change frequently.

Proposal 2: The time to next burst is useful for RAN resource scheduling and UE power saving in case the traffic is aperiodic or the periodicity changes frequently.

Discussion on R3-243957:

Proposal 3: RAN2 to reply to RAN3 that, for a certain DRB, the PSI-based SDU discard functionality can be activated/deactivated by the MAC CE received from either the MN or SN.

4. Reference

[1]. S2-2407351, vivo, LS on FS_XRM Ph2.

[2]. R3-243957, Qualcomm, LS on UL PSI based PDU discarding in NR-DC.

[3]. 3GPP TS 38.321: "NR; Medium Access Control (MAC) protocol specification".

[4]. 3GPP TS 38.323: "NR; Packet Data Convergence Protocol (PDCP) specification".