3GPP TSG RAN WG2 #127                                                                                                  R2-2406305

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Source:	CATT

Title:	Discussion on inter-CU LTM

Agenda Item:    8.6.2

Document for:  Discussion and Decision

Introduction

This paper further discusses the remaining issues related to inter-CU LTM. More details, the following aspects are covered:

- RRC signaling design

- Early synchronization

- LTM cell switch execution

- Inter-CU LTM without security key change

- Inter-CU LTM with DC

Section 2 gives the detailed discussion of this paper, and our proposals are summarized in Section 3.

Discussion

RRC signaling design

Reference configuration

Reference configuration is introduced in Rel-18 intra-CU LTM for the purpose of signaling overhead reduction. With same motivation, there is no doubt to support it in inter-CU LTM. For inter-CU LTM, a issue was raised on the commonalities between candidate configurations belonging to different CUs and it was argued on whether only one reference configuration for all candidates is sufficient in previous meeting. The options on the tables are as follows,

- Option 1: One reference configuration (i.e., reusing R18 design)

- Option 2: Multiple reference configurations (e.g., one separate reference configuration per CU)

The following Table provides a comparison between one reference configuration and multiple reference configurations.

Table 1

|  | Option 1:One reference configuration | Option2:Multiple reference configurations |
| --- | --- | --- |
| Signaling overhead reduction | Can reduce the signaling overhead effectively as that in R18 LTM | More signaling overhead is introduced due to more reference configurations to be transferred over Uu, which defeats the purpose of using reference configuration(i.e., reducing signaling overhead) |
| Spec impact | no spec impact as Rel-18 signaling structure and UE handling can be reused | Extra spec impacts are needed as new signaling design and UE handling is needed. |
| Commonalities | It can be up to network to decide whether to use reference configuration or not when it prepares an inter-CU LTM configuration, e.g., if there are enough commonalities between candidate configurations, then reference configuration is used. Otherwise, the complete configuration is used. | There may be more commonalities among the candidate cells in one CU, but it is questionable the reduced overhead for candidate configuration, is more than the added overhead due to using multiple reference configurations itself, it is obviously the more reference configurations are used, the lower benefit on signaling overhead. . |

Based on above comparison, the benefit on multiple reference configurations is suspicious. And the case is similar with inter-SN subsequent CPAC which is only one reference configuration too. Taking all above into consideration, it is obvious that one reference configuration should be used for inter-CU LTM.

Therefore, it is proposed that,

Proposal 1: Only one reference configuration is configured for all candidates no matter whether they belong to different CUs or the same CU.

CSI resource configuration

In Rel-18 intra-CU LTM, it is the gNB-CU who generates the common CSI resource configuration and configures it to UE as a common list. The signaling design is as follows,

LTM-Config-r18 ::= SEQUENCE { ltm-ReferenceConfiguration-r18 SetupRelease {ReferenceConfiguration-r18} OPTIONAL, -- Need M ltm-CandidateToReleaseList-r18 SEQUENCE (SIZE (1..maxNrofLTM-Configs-r18)) OF LTM-CandidateId-r18 OPTIONAL, -- Need N ltm-CandidateToAddModList-r18 SEQUENCE (SIZE (1..maxNrofLTM-Configs-r18)) OF LTM-Candidate-r18 OPTIONAL, -- Need N ltm-ServingCellNoResetID-r18 INTEGER (1..maxNrofLTM-Configs-plus1-r18) OPTIONAL, -- Need N ltm-CSI-ResourceConfigToAddModList-r18 SEQUENCE (SIZE (1..maxNrofLTM-CSI-ResourceConfigurations-r18)) OF LTM-CSI-ResourceConfig-r18 OPTIONAL, -- Need N ltm-CSI-ResourceConfigToReleaseList-r18 SEQUENCE (SIZE (1..maxNrofLTM-CSI-ResourceConfigurations-r18)) OF LTM-CSI-ResourceConfigId-r18 ... }

For inter-CU LTM, it was agreed in RAN2 last meeting that the source gNB-CU will generates the common CSI resource configuration,

Stage-2 signaling flows and procedures 3. The preparation of inter-CU LTM configuration is initiated by the source gNB-CU. 4. For each candidate cell, the preparation of lower layer configuration is initiated by the candidate gNB-CU, based on the LTM request from the source gNB-CU. RAN2 assumes the interaction between the candidate gNB-CU and candidate gNB-DU follows the same signaling procedure for intra-CU LTM. 5. The source gNB-CU is responsible to collect the configurations and information of candidate cells from multiple candidate gNB-CUs and generates the common CSI resource configuration for L1 measurement on candidate cells.

Based on the agreement above, it is straightforward that the list of CSI resource configurations are common for all the LTM candidate cells, i.e., R18 signalling design for CSI resource configuration is reused. We see no motivation to further enhance it，e.g., there is no need to configure LTM CSI resource configuration to UE on a per CU basis.

Therefore, it is proposed that,

Proposal 2: For inter-CU LTM, the LTM-CSI-ResourceConfig-r18 is reused to provide the CSI resource configuration no matter the candidate configuration is inter-CU or intra-CU.

Early synchronization

Early DL sync

For inter-CU LTM, the early DL sync had been agreed to be also supported,

Early DL and UL sync is also supported for inter-CU LTM. Inform RAN3 of this. Early DL sync using CSI-RS should be considered, pending RAN1 approval.

In Rel-18 intra-CU LTM, early DL sync to candidate cell is achieved by the Candidate TCI State Activation/Deactivation, as indicated in TS 38.300,

When configured by the network, it is possible to activate TCI states of one or multiple cells that are different from the current serving cell. For instance, the TCI states of the LTM candidate cells can be activated in advance before any of those cells become the serving cell. This allows the UE to be DL synchronized with those cells, thereby facilitating a faster cell switch to one of those cells when cell switch is triggered.

To perform early DL sync in inter-CU LTM, we see no issues to reuse the R18 candidate TCI State activation/deactivation design (including MAC CE and related UE handling) ,

Therefore it is suggested that,

Proposal 3: For inter-CU LTM, the R18 candidate TCI State activation/deactivation design (including MAC CE and related UE handling) is reused.

LTM cell switch execution

Mixture of inter-CU LTM and intra-CU LTM

In RAN2#125-bis meeting, it was agreed to support the mixture of subsequent inter-CU LTM and subsequent intra-CU LTM after an inter-CU or intra-CU LTM switch.

Agreements on scenarios:

- RAN2 first focus on inter-CU LTM in NR standalone scenario and use it as baseline for supporting inter-CU LTM in NR-DC scenarios.

- Rel-19 inter-CU LTM also supports mixture of subsequent inter-CU LTM and subsequent intra-CU LTM after an inter-CU or intra-CU LTM switch.

- UE can be configured with a mixture of intra-CU and inter-CU candidate LTM cells and irrespective of how the UE is configured with this mixture, UE measurement and reporting procedures will be the same for both intra-CU and inter-CU candidate LTM cells.

UE behaviors are different for intra-CU LTM execution or inter-CU LTM execution, as in the Table below,

Table 2

|  | Intra-CU case | Inter-CU case |
| --- | --- | --- |
| L2 handling | - MAC reset - RLC re-establishment (inter-DU case) - PDCP data recovery (inter-DU case) | - MAC reset - RLC re-establishment - PDCP re-establishment |
| Security update | No | Yes |

So there should be a way to let UE know the type (inter-CU or intra-CU) of the LTM execution.

Similar as the Rel-18 mobility design for differentiating the LTM candidate cells from different DUs (i.e., set ID is used), a new set ID can be introduced to differentiate LTM candidate cells belonging to different CUs. Each CU is associated to a unique set ID and the set ID of each LTM candidate cell and serving cell are indicated to UE. With this, UE can determine what kind of behaviors to perform by comparing the set ID of serving cell and set ID of target cell upon LTM execution.

Proposal 4: New set ID is introduced in RRC configuration to inform UE whether to perform inter-CU LTM execution or intra-CU LTM execution.

Procedural Order between triggering cell switch to UE and informing target gNB-DU of the cell switch

For the interactions between source and target upon inter-CU LTM execution, RAN3 has agreements as follows,

Cell Switch Notification from source DU to target DU (in different gNB from source) for LTM execution.

Based on above, we need further consider the procedural order between triggering cell switch execution to UE and informing target DU of the cell switch.

In Rel-18 LTM, the procedure is that the serving gNB-DU triggers the execution to the UE and then informs the gNB-CU of the serving cell switch. The gNB-CU then informs the target gNB-DU of the serving cell switch. For inter-CU LTM, it seems straightforward to reuse this procedural order. However, one may think that the Xn interface interaction (i.e., additional interaction delay comparing to intra-CU LTM) may delay the UE access to the target cell.

Typically the delay for one way communication over Xn interface is around 10ms and over F1 interface is around 5ms. As the cell switch notification procedure over Xn/F1 interface are performed in parallel with UE processing handling, the overall Xn/F1 delay (i.e., 20ms which includes 10 ms over Xn and 5 ms*2 over F1) should be no more than the UE processing time (i.e., 20~40ms). Hence, there is no additional delay caused by involving interaction over Xn interface.

Observation 1: The latency of inter-CU LTM execution is not increased due to the Xn interaction.

Therefore, it is proposed that,

Proposal 5: Reuse R18 procedure upon inter-CU LTM execution, i.e., the serving gNB-DU first triggers the cell switch execution to the UE and then informs the cell switch to the target gNB-DU (i.e. transferred via CUs). Inform this agreement to RAN3.

Inter-CU LTM without security key change

In previous discussion, it was proposed by some companies that although CU is changed, same PDCP anchor can still be used, and then PDCP re-establishment and security key update are not needed.

From our view, since anyway we will design the security key update mechanism for inter-CU LTM, and the candidate solutions had been sent to SA3 for final decision, we see no motivation to design such parallel scheme without security update. Besides, the suggested solution should be firstly discussed by RAN3 due to the significant impact on the gNB structure. Even RAN3 agrees to support this architecture, from UE perspective it is equivalent to intra-CU LTM, i.e., the UE doesn’t care the PDCP PDU is directly transferred from the anchor CU or transferred from the anchor CU via another CU.

Therefore, it is proposed that,

Proposal 6: It’s up to RAN3 on whether to support inter-CU LTM without change of PDCP anchor. RAN2 will not discuss it further.

Inter-CU LTM with DC

Inter-CU SCG LTM with MCG unchanged

Scenarios

Co-existence of intra-CU MCG/SCG LTM and inter-CU SCG LTM

In last meeting, it had been clarified that inter-CU MCG LTM and inter- CU SCG LTM cannot be configured simultaneously,

An LTM configuration with inter-CU LTM candidate cells can be configured either by the MCG or SCG (but not for both simultaneously) and it is up to the network to handle this (further details up to RAN3, if any). No restriction for intra-CU LTM candidate cells.

However, it is still not clear whether the intra-CU MCG/SCG LTM can be configured simultaneously with the inter-CU SCG LTM.

For the co-existence between intra-CU MCG and inter-CU SCG LTM, we see no motivation to not allow the NW to configure them to UE simultaneously. The MN node does not change due to the intra-CU MCG LTM execution, so the candidate configurations for inter-CU SCG LTM are still valid after an intra-CU MCG LTM execution.

Proposal 7a: Intra-CU MCG and inter-CU SCG LTM can be configured to UE simultaneously.

For the co-existence between intra-CU SCG LTM and inter-CU SCG LTM, it should be reasonable to configure UE with a mixture of intra-CU SCG and inter-CU SCG candidate LTM cells, similar as what we have agreement for inter-CU MCG LTM.

Proposal 7b: UE can be configured with a mixture of intra-CU SCG and inter-CU SCG candidate LTM cells.

High level design of inter-CU SCG LTM

Preparation phase

In legacy inter-SN CPAC or inter-SN SCPAC, both MN-initiated and SN-initiated cases are supported. However, MN initiated case is only used for load balance but not for mobility case as MN does not have the measure results of serving PSCell, so it seems not suitable for the LTM purpose(i.e., for mobility).

Therefore, it is suggested that,

Proposal 8: Inter-CU SCG LTM preparation can be initiated by source SN. FFS whether it can be initiated by source MN.

Since MN will be involved for inter-CU SCG LTM, the candidate configuration for inter-CU SCG LTM should be the MN format RRCReconfiguration message which includes both the MCG part configuration and SCG part configuration. The detailed signaling can be further discussed later, for example, whether to reuse the Rel-18 LTM configuration to include the candidate configuration for inter-CU SCG LTM.

Proposal 9: The inter-CU SCG LTM configuration is provided to UE in MN format.

Execution phase

For SN-initiated inter-CU SCG LTM, since it is mainly used for mobility use case, it should be source SN to acquire the L1 measurement results of candidate cells and the serving PSCell. This way allows source SN to determine whether to change the PSCell with consideration of the quality for both source PSCell and candidate cells. Therefore, it should be up to source SN to decide whether to trigger the inter-CU SCG LTM cell switch.

Proposal 10: For inter-CU SCG LTM, the LTM cell switch command MAC CE is sent by source SN.

Security key update

To support subsequent inter-CU SCG LTM, we need to further consider the security key update mechanism upon cell switch.

In SCPAC, UE is preconfigured with a sk-counter list per SN and UE chooses one to derive the security key used for SN upon inter-SN PSCell change execution. It seems feasible to use the similar method for subsequent inter-CU SCG LTM.

Therefore, it is suggested that,

Proposal 11: For the security key update of inter-CU SCG LTM, SCPAC security key update mechanism is taken as baseline.

Inter-CU MCG LTM with SCG kept or released

How to keep or release SCG

For inter-CU MCG LTM with NR-DC, from the NW side, it is reasonable to leave the candidate MN to determine whether the SCG is kept or released. If so, the general procedure can take the procedure of Inter-Master Node handover with/without Secondary Node change (10.7.2 of 37.340) and Master Node to eNB/gNB Change (10.8.2 of 37.340) for reference.

From UE perspective, it needs to perform PSCell addition/release/reconfiguration based on the applied candidate configuration upon the execution of inter-CU MCG LTM. It means the execution of MCG LTM will not only involve MCG but also involve SCG. However, currently the LTM execution procedure is within one cell group, which should be enhanced to support the inter-CU MCG LTM with SCG kept/released.

Proposal 12: It’s up to NW implementation to configure inter-CU MCG LTM with SCG kept or released. The LTM execution procedure at UE side should take both MCG and SCG handling into consideration.

SCG security key update for inter-CU MCG LTM with SCG kept

If UE performs inter-CU MCG LTM with SCG kept, the security key used for MCG will be changed. Thus, the SCG security key should be updated accordingly. Regarding on how to update the SCG security key, the legacy way works well, i.e., UE calculates the SCG security key based on the updated MCG security key and the sk-counter provided by the candidate configuration. Thus, there is no need to make additional design on the SCG security key update for inter-CU MCG LTM with SCG kept.

Proposal 13: If SCG is kept during inter-CU MCG LTM execution, the SCG security key should be updated as legacy. FFS the detailed spec impact.

Conclusion

Based on the previous analysis in Section 2, our observations and proposals are summarized as follows:

Observation 1: The latency of inter-CU LTM execution is not increased due to the Xn interaction.

RRC signaling design

Proposal 1: Only one reference configuration is configured for all candidates no matter whether they belong to different CUs or the same CU.

Proposal 2: For inter-CU LTM, the LTM-CSI-ResourceConfig-r18 is reused to provide the CSI resource configuration no matter the candidate configuration is inter-CU or intra-CU.

Early synchronization

Proposal 3: For inter-CU LTM, the R18 candidate TCI State activation/deactivation design (including MAC CE and related UE handling) is reused.

LTM cell switch execution

Proposal 4: New set ID is introduced in RRC configuration to inform UE whether to perform inter-CU LTM execution or intra-CU LTM execution.

Proposal 5: Reuse R18 procedure upon inter-CU LTM execution, i.e., the serving gNB-DU first triggers the cell switch execution to the UE and then informs the cell switch to the target gNB-DU (i.e. transferred via CUs). Inform this agreement to RAN3.

Inter-CU LTM without security key change

Proposal 6: It’s up to RAN3 on whether to support inter-CU LTM without change of PDCP anchor. RAN2 will not discuss it further.

Inter-CU LTM with DC

Proposal 7a: Intra-CU MCG and inter-CU SCG LTM can be configured to UE simultaneously.

Proposal 7b: UE can be configured with a mixture of intra-CU SCG and inter-CU SCG candidate LTM cells.

Proposal 8: Inter-CU SCG LTM preparation can be initiated by source SN. FFS whether it can be initiated by source MN.

Proposal 9: The inter-CU SCG LTM configuration is provided to UE in MN format.

Proposal 10: For inter-CU SCG LTM, the LTM cell switch command MAC CE is sent by source SN.

Proposal 11: For the security key update of inter-CU SCG LTM, SCPAC security key update mechanism is taken as baseline.

Proposal 12: It’s up to NW implementation to configure inter-CU MCG LTM with SCG kept or released. The LTM execution procedure at UE side should take both MCG and SCG handling into consideration.

Proposal 13: If SCG is kept during inter-CU MCG LTM execution, the SCG security key should be updated as legacy. FFS the detailed spec impact.

Reference

[1] 3GPP TS 38.300: "NR; NR and NG-RAN Overall Description" v18.2.0

[2] R2-2309251, Reply LS on Approaches during execution for inter-DU LTM