3GPP TSG-RAN WG2 Meeting #127                                                                   R2-2406306

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Source:              CATT

Title:	             Measurement Event Evaluation

Agenda Item:	8.6.3

Document for:	Discussion and Decision

Introduction

In this contribution, we further discuss the open issues on event evaluation.

The contribution is organized as the following. In section 2, we discuss the following aspects,

- Event definition

- Beams for event evaluation

- Node to generate the event configuration

- Configuration signalling aspects

- Which layer to perform the event evaluation

Our proposals are summarized in Section 3.

Discussion

Event definition

Last meeting it was agreed to support Event LTM2, LTM3, LTM4, LTM5 but whether to support event LTM1 is kept FFS. For event LTM 1(i.e., Beam of serving cell becomes better than absolute threshold), the motivation to support it is not clear as it seems irrelevant to the agreed LTM purposes of Event triggered L1 measurement:

- Select the candidate beam/cell to trigger early synchronization.

- Select the target beam/cell and trigger LTM cell switch procedure.

So it is preferred to not support it.

Proposal 1: Event LTM1 (i.e., Beam of serving cell becomes better than absolute threshold) is not supported.

Beams for event evaluation

Beam of the serving cell used for event evaluation

For the agreed event LTM2,LTM3,LTM5, beam of serving cell are involved in the event evaluation. A further issue is which beam of the serving cell is used for evaluation.This issue was discussed last meeting and companies’ views are splitted. The options on the table are as follows,

- Option 1:current active beam of serving cell

- Option 2:best beam of serving cell

Even though RAN1 has agreed to used the current beam of serving cell for R19 MIMO,it is not suitable to simply reused the same principle in LTM. The purpose of event triggering in R19 LTM is for cell switch whereas that in R19 MIMO is for beam switching.

For beam management, the intention of the event is to select a better beam than the current active beam, so taking the current active beam as the serving beam is logical. But different from beam management, the event evaluation for LTM is to dertermine one better cell.So the best beam of serving cell should be used for the event evaluation in LTM. Because if even the current serving beam of the current serving cell is not good enough but the other beams of the current serving cell is still good, it doesn’t need to perform LTM cell switch as beam switching within the serving cell is enough.

Proposal 2a: For event LTM2/LTM3/LTM5, the “beam of serving cell” refers to the best beam (i.e., the best one among all the configured RS) of the serving cell for event evaluation.

It is possible that the best beam of serving cell may change during the event evaluation.It is necessary to consider the UE behavior for such case.i.e, whether UE should reset the TTT timer.

If TTT timer is reset when the best beam of serving cell changes,it will increase the difficulty of triggerring the event and NW may not be able to get the L1 measure result timely. As a result, NW can not trigger the LTM cell switch when it is necessary. Moreover,the intention of event triggering is to send the measurement results to NW when the beam quality of serving cell become bad and/or the beam quality of candiate cell is better than the beam quality of serving cell. It does not matter which beam is the best beam of serving cell. Hence,there is no need to reset the TTT timer when the best beam of serving cell changes from one to another. Anyway the consequence of keeping the evaluation upon best beam changes is that more/early L1 measurement results report may be triggerred, it doesn’t bring negative impact on the NW, and it could provide more infomration to the NW, so we suggest to continue the TTT upon best beam of serving changes.

Proposal 2b: For event LTM2/LTM3/LTM5, TTT timer is not reset when the best beam of serving cell changes.

Beam(s) of candidate cell used for event evaluation

This issue is related to the association between the event configuration and measure resource configuration. However, regardless how the association will be, each beam corresponding to each RS is supposed to be associated to an event configuration eventually. Since we have agreed that use of beam level measurement result for event evaluation is baseline, the event evaluation should be performed separately to each beam of candidate cell.

Proposal 3: The “beam of candidate cell” is each RS which is associated to an event configuration. Event evaluation is performed separately to each beam of candidate cell.

Whenever the measure result of any beam meets the associated event criteria during the TTT, UE should initiate the measurement reporting.

Proposal 4: UE initiates measurement reporting when the event is fulfilled for any associated beam of candidate cell.

Node to generate the event configuration

In legacy L3 measurement, the serving cell determines the event configuration due to the serving cell has more knowledge of its neighbor cells. The same principle can be reused for LTM event triggered measurement. To support subsequent LTM, each candidate cell should provide the event configuration which is to be used when the candidate becomes the new serving cell.

Proposal 5a: Current serving cell determines the event configuration of the candidates for the initial LTM cell switch.

Proposal 5b: Each candidate cell determines the event configuration for other candidates which is used when the candidate becomes the new serving cell.

Configuration signalling aspects

Event configuration

Different RS/beams may share the same event configuration, so it may reduce the signalling overhead if event configurations are provided to UE as a common pool, i.e., similar as ReportConfigToAddModList in legacy L3 measurement. It can be further discussed whether to provide the list per candidate configuration or per LTM configuration.

Proposal 6: The event configurations are provided to UE as a list(s). FFS whether it is provided per candidate configuration or per LTM configuration.

Association between resource configuration and event configuration

In previous meeting, it was agreed that L1 LTM measurement event configuration is associated with L1 measurement resource configuration provided in LTM configuration via RRC signaling. Besides we have agreement that beam level measurement result for event evaluation is baseline. One RS/beam needs to be associated to one event configuration. There are several options for such association,

| Option | Description | Cons/pros Analysis |
| --- | --- | --- |
| Option 1 | per measure resource configuration(i.e., one event configuration is associated to one LTM-CSI-ResourceConfig-r18) | 1. It has Less spec impacts by reusing the R18 signalling structure LTM-CSI-ResourceConfig-r18. 2. It is flexible to associate one event configuration to beams of one cell or beams of multiple candidate cells. In R18 LTM, each LTM-CSI-ResourceConfig is defined as a group of SSB resources for one or more LTM candidate configurations. NW can choose to include SSB resources of one candidate or multiple candidate cells in one resource configuration |
| Option 2 | per RS/beam | 1. More spec impacts as new resource configuration needs to be defined on a per RS/beam basis. 2. signalling overhead is increased |
| Option 3 | per cell(one event configuration is associated to one LTM candidate cell, i.e., all beams of the cell share the same event configuration) | 1. More spec impacts as new resource configuration needs to be defined on a per cell basis. |
| Option 4 | per frequency(i.e., similar as L3 measurement) | 1. More spec impacts as new resource configuration needs to be defined on a per frequency basis. |

As analysed in above table, option 1 is preferred with the assumption that R18 LTM CSI resource configuration (i.e., LTM-CSI-ResourceConfig-r18) can be reused.

Proposal 7: RAN2 assumes R18 LTM CSI resource configuration (i.e., LTM-CSI-ResourceConfig-r18) is reused as the measurement resource configuration of event triggered L1 measurement reporting. It can be revisited if critical issue is found.

Proposal 8: One event configuration can be associated to one measure resource configuration (i.e., all the RS/beams in one resource configuration share the same event configuration).

In legacy L3 measurement, any measurement object can be linked to any reporting configuration of the same RAT type. It would be straightforward that the similar principle is taken in LTM event triggering measurement.

Proposal 9: Any measurement resource configuration can be associated to any event configuration.

How to associate  one event configuration to one measure resource configuration

The next question is how to associate one event configuration to a measure resource configuration. There are following possible options,

- Option 1: The index of an event configuration is included in LTM CSI Report Config(i.e., reusing LTM-CSI-ReportConfig-r18).

If to reuse the LTM CSI Report Config (LTM-CSI-ReportConfig) for event triggering reporting, the index of an event configuration can be included in it, which will associate the measurement resource configuration and the event configuration. When the index of an event configuration is present, UE will trigger the measurement report based on the event.

- Option 2: A new id is introduced to associate the event configuration and a measure resource configuration.

Similar as in legacy L3 measurement, a new id can be introduced and each id is mapped to an index of an event configuration and an index of a resource configuration. UE performed event triggered measurement report based on the configured id.

Proposal 10: To associate one event configuration to one measure resource configuration, the following signaling options are considered,

- Option 1: The index of an event configuration is included in LTM CSI Report Config(i.e., reusing LTM-CSI-ReportConfig-r18).

- Option 2: A new id is introduced to associate one event configuration(s) to one measure resource configuration.

Which layer to perform the event evaluation

Regarding which layer to perform the event evaluation, there are several options (e.g., PHY, MAC,). In our understanding it is more suitable to perform the event evaluation in MAC layer. PHY layer is typically used for dynamic scheduling/measurement in time, so it is not reasonable to do it in PHY layer. MAC layer is already responsible for the control of the LTM triggering by handling the LTM cell switch command MAC CE. It will also facilitate the design for conditional LTM, i.e., it would be simpler to perform both the triggering of conditional LTM execution and event evaluation in MAC layer. Furthermore, we can take the BFD as baseline, it is the PHY to measure each instance for BFD, but it is the MAC entity to evaluate whether BF occurs.

Proposal 11: MAC layer performs the event evaluation for event-triggered L1 measurement reporting.

Conclusion

Based on the previous analysis in section 2, our proposals are summarized as follows:

Event definition

Proposal 1: Event LTM1 (i.e., Beam of serving cell becomes better than absolute threshold) is not supported.

Beams for event evaluation

Proposal 2a: For event LTM2/LTM3/LTM5, the “beam of serving cell” refers to the best beam (i.e., the best one among all the configured RS) of the serving cell for event evaluation.

Proposal 2b: For event LTM2/LTM3/LTM5, TTT timer is not reset when the best beam of serving cell changes.

Proposal 3: The “beam of candidate cell” is each RS which is associated to an event configuration. Event evaluation is performed separately to each beam of candidate cell.

Proposal 4: UE initiates measurement reporting when the event is fulfilled for any associated beam of candidate cell.

Node to generate the event configuration

Proposal 5a: Current serving cell determines the event configuration of the candidates for the initial LTM cell switch.

Proposal 5b: Each candidate cell determines the event configuration for other candidates which is used when the candidate becomes the new serving cell.

Configuration signalling aspects

Proposal 6: The event configurations are provided to UE as a list(s). FFS whether it is provided per candidate configuration or per LTM configuration.

Proposal 7: RAN2 assumes R18 LTM CSI resource configuration (i.e., LTM-CSI-ResourceConfig-r18) is reused as the measurement resource configuration of event triggered L1 measurement reporting. It can be revisited if critical issue is found.

Proposal 8: One event configuration can be associated to one measure resource configuration (i.e., all the RS/beams in one resource configuration share the same event configuration).

Proposal 9: Any measurement resource configuration can be associated to any event configuration.

Proposal 10: To associate one event configuration to one measure resource configuration, the following signaling options are considered,

- Option 1: The index of an event configuration is included in LTM CSI Report Config(i.e., reusing LTM-CSI-ReportConfig-r18).

- Option 2: A new id is introduced to associate one event configuration(s) to one measure resource configuration.

Which layer to perform the L1 measurement evaluation

Proposal 11: MAC layer performs the event evaluation for event-triggered L1 measurement reporting.

Reference

- RP-241515, Revised Work Item: NR mobility enhancements Phase 4

- R2-2405701,Report from session on V2X/SL, R19 NES and MOB

- TS 38.331, NR; Radio Resource Control (RRC) protocol specification,18.2.0