3GPP TSG-RAN WG2 Meeting #127                                                                   R2-2406307

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Source:              CATT

Title:	             Measurement Reporting

Agenda Item:	8.6.4

Document for:	Discussion and Decision

Introduction

In this contribution, we discuss the issues related to the measurement reporting.

The contribution is organized as the following. In section 2, we discuss the following aspects,

- Contents of the measurement report

- Message to carry the measurement report

- Measurement reporting when the leaving condition is met

- Periodical measurement reporting for the triggered event

Our proposals are summarized in Section 3.

Discussion

Contents of the measurement report

Maximum number of the reported beams

When the measurement report is triggered, the UE may have multiple measurement results available for multiple beams of multiple cells, but it does not necessarily to report all of them as some of them may be not useful for the network. In L3 measurement the maximum numbers of neighboring cells/beams that can be included in the measurement results are configurable. Similar limitation can be used for LTM event triggered L1 measurement reporting.

Proposal 1: The max number of candidate beams to include in the measurement report is configured by gNB.

What beams to be reported

In L3 measurement, the cells which have triggered the event condition are maintained in cellsTriggeredList for a measure id. The measure results of the cells in cellsTriggeredList can be included in one measurement report according to the limitation on the max number of non-serving cells. Same mechanism can be adopted for LTM event triggering measurement reporting. More specifically, for beams which are associated to the same event configuration, UE maintain a list of all the beams which have fulfilled the event condition. When the measurement reporting is triggered, some or all of them are included in the measurement report according to limitation on the max number of candidate beams configured by gNB.

Therefore, it is proposed that,

Proposal 2: Candidate beams (i.e., beams associated to the same event configuration) which have fulfilled the event condition are included in the measurement report according to limitation on the max number of candidate beams configured by gNB.

In L3 measurement, measure results of serving cells are always reported together with the measure results of the triggered cells. NW could make handover decision based on both the serving cell measurement results and neighbor cell measurement results. But for LTM event triggering measurement, it seems no need to make the serving cell reporting mandatory as the NW could get the L1 measurement results of beams of serving cell based on CSI-ReportConfig as in legacy L1 measurement. Hence, it can be up to network to configure whether to include it in the measurement report.

Proposal 3: Whether to include the beam of the serving cell in the measurement report is configured by gNB.

Contents for a reported beam

When receiving a measurement report, NW needs to know some basic information of a reported beam, e.g., the beam measure result, the identity of the beam and which candidate cell it belongs to, and the identity of the triggered event. Some of the information may be reported explicitly or implicitly depending on the RRC signalling design for the event configuration and resource configuration, e.g., UE can just report the index which is used to associate an event configuration to a measure resource configuration, the NW will know associated event configuration.

Hence, the information of a reported beam in a measurement report at least includes,

‐	The beam Measure results

‐	The beam information (e.g., SSB index, candidate configuration id)

‐	The triggered event information

Proposal 4: For a reported beam in the measurement report, the information at least includes (explicitly or implicitly),

‐	Beam measure results

‐	Beam information (e.g., SSB index, candidate configuration id)

‐	The triggered event information

Message to carry the measurement report

Another key issue is which message is used to carry the event triggered L1 measurement report. RRC signalling should be excluded firstly as LTM decision is made by DU but RRC signalling is handled by CU. So the possible options could be MAC CE or UCI. Both MAC CE and UCI are feasible and either way has pros and cons as compared in table below.

|  | Option 1:MAC | Option 2:UCI |
| --- | --- | --- |
| Design Complexity | New design is needed comparing to UCI as it is a different solution than R19 MIMO | Less design is expected. As it has been decided in R19 MIMO to use UCI to report the event triggered L1 measurement results, the design in R19 MIMO may be reused as baseline for LTM |
| Flexibility of message content | It is flexible to include information of variable length in the MAC CE | Less flexible on the content. The fixed size of UCI is agreed in R19 MIMO, the information that can be included is less flexible. Variable size is under discussion. |
| Dynamic of UL resource scheduling | UL resource scheduling for MAC CE based on UE request and NW schedule as in legacy | For R19 MIMO, 2 modes are supported in RAN1 for UL resource scheduling, i.e., mode A(dynamically scheduling UCI),mode B(UCI in pre-configured resource). |

Table 1

Based on the comparison in the table above, MAC CE is preferred as it is more important to use a flexible message format and it can support dynamic UL resource scheduling by simply reusing legacy mechanism.

Proposal 5: MAC CE is used to carry the measurement report.

Measurement reporting when the leaving condition is met

In the legacy L3 measurement, UE can be configured to send the measurement report when the leaving condition is met for a cell which was reported previously. It is useful for NW to know when a cell is not good enough anymore as it will enable the NW to exclude this cell when determine a target cell for handover, similar intention as legacy L3 measurement. In L3 measurement, the report on leaving condition fulfilled is configurable. For L1 event measurement report, the same mechanism can be used.

Therefore, it is proposed that,

Proposal 6: UE can send the measurement report when the leaving condition is met. It can be enabled/disabled by NW.

Periodical measurement reporting for the triggered event

In the legacy L3 measurement, reporting interval and reporting amount can be configured for event triggered measurement. UE can send the measurement results periodically according to the configured reporting amount after the event has been triggered. Similar mechanism can be supported in LTM as it is beneficial for the network to get the latest quality status/quality changing trend of the reported cells.

Therefore, it is proposed that,

Proposal 7: UE can send the measurement report periodically after the event has been triggered, i.e., reportAmount and reportInterval can be configured optionally to UE.

Conclusion

Based on the previous analysis in section 2, our proposals are summarized as follows:

Contents of the measurement reporting

Proposal 1: The max number of candidate beams to include in the measurement report is configured by gNB.

Proposal 2: Candidate beams (i.e., beams associated to the same event configuration) which have fulfilled the event condition are included in the measurement report according to limitation on the max number of candidate beams configured by gNB.

Proposal 3: Whether to include the beam of the serving cell in the measurement report is configured by gNB.

Proposal 4: For a reported beam in the measurement report, the information at least includes (explicitly or implicitly),

‐	Beam measure results

‐	Beam information (e.g., SSB index, candidate configuration id)

‐	The triggered event information

Message to carry the measurement reporting

Proposal 5: MAC CE is used to carry the measurement report.

Measurement reporting when the leaving condition is met

Proposal 6: UE can send the measurement report when the leaving condition is met. It can be enabled/disabled by NW.

Periodical measurement reporting for the triggered event

Proposal 7: UE can send the measurement report periodically after the event has been triggered, i.e., reportAmount and reportInterval can be configured optionally to UE.

Reference

- RP-241515, Revised Work Item: NR mobility enhancements Phase 4