3GPP TSG-RAN WG2 #127	R2-2406310

Maastricht Netherlands, August. 19th – 23rd, 2024

Agenda Item:	8.3.2.1

Source:	OPPO

Title:	Discussion on simulation result of RRM measurement

Document for:	Discussion, Decision

Introduction

In RAN2#126, the following cases are identified for RRM measurement prediction with 3 cases of high priority for evaluation.

| Case number | Prioritization | Evaluation scenario combination | target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal | TBD |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal | TBD |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1st goal | Intra-cell |

This contribution intends to show our evaluations of those RRM measurement prediction cases that are of high priority.

Discussion

Based on the assumptions we have agreed upon so far, we would like to share our evaluation results for RRM measurements.

FR1 to FR1 inter-frequency (frequency domain)

As in RAN2#126 agreements, for inter-frequency inter-cell prediction, the measurement on cell for measurement should not be reduced in both temporal and spatial domains. As shown in Figure 1, the sampling period Tper is 40ms for FR1. X time instances of L3-RSRP samples of frequency#1 are used as model input to derive the corresponding L3-RSRPs of frequency#2. The input and output cells are co-located.

Figure 1. Example for inter-frequency prediction

The simulation assumptions and information of datasets and the AI/ML model are listed in Table 1.

Table 1: FR1 to FR1 inter-frequency prediction assumptions

| Report parameters |  | OPPO | FR1 to FR1 inter-frequency |
| --- | --- | --- | --- |
| Reported simulation assumptions | UE trajectory option (option 1,2,3 in[4]) | Option 1 |  |
|  | UE trajectory boundary processing option (option 1,2,3 in[4]) | Option 3 |  |
|  | UE speed (30,60,90,120 Km/h) | 30 if not explicitly stated |  |
|  | Inter-frequency correlation assumption in general (yes or no)(Note 1) | yes |  |
|  | inter-frequency shadow fading correction (e.g. full, partial, no)(Note 1) | Full |  |
|  | Whether LOSsoft is modeled or not | No |  |
|  | spatial consistency option (A or B) | A |  |
|  | Number of TX beams | 1 |  |
|  | Number of RX beams | 1 |  |
| Data Size (Number of Samples) | Training/validity | ~0.82M/~0.20M |  |
|  | Testing | ~0.26M |  |
| AI/ML model input/output | Model input (Note 5) | L3-RSRP of Cell A |  |
|  | Model output(Note 6) | L3-RSRP of Cell B |  |
| AI/ML model description | Model type (e.g., LSTM, CNN, transformer …) | 2 LSTM layer + 1 dense layer |  |
|  | Model complexity in a number of parameters(M) | ~0.18M |  |
|  | Model complexity in model size (e.g. Mbyte) | ~0.69MB |  |
|  | Computational complexity [FLOPs] | ~0.40M |  |

We have tested the prediction accuracy of three different datasets:

- Given cell: the training and testing data of frequency#1 come from a randomly chosen cell, which can be a serving cell or non-serving cell during UE’s movement. For example, all measured RSRPs come from cell#1 at frequency#1. From time t1 to t2, cell#1 would be the serving cell of UE. At the rest of the time, cell#1 is a non-serving cell whose RSRPs might be pretty low as UE gets far away from the cell.

- Give cell with post-process (N dBm): the training and testing data of frequency#1 come from a randomly chosen cell, but only the time instances whose RSRPs are higher than N dBm are used for calculating prediction accuracy. The data used for calculating is thus a subset of the above given cell testing datasets. Figure 2 illustrates how we get the dataset for calculation.

- Serving cell: the training and testing data of frequency#1 come from the serving cell whose RSRPs are always high.

Figure 2. Example of post-process for a given cell

Table 2 shows the prediction results of these three datasets. We use 2GHz to predict the measurement results of 4GHz.

Table 2. Inter-frequency prediction results for a given cell and serving cell

| 2GHz->4GHz | Given cell | Given cell with post-process (-120dBm) | Given cell with post-process (-110dBm) | Serving cell |
| --- | --- | --- | --- | --- |
| Average L3 cell-level RSRP difference (dB) | 0.12 | 0.3 | 0.33 | 0.33 |

From Table 2, it can be seen that the prediction for the serving cell case has a larger RSRP difference than the given cell case. As the post-process N dBm gets higher, the average RSRP difference of the given cell gradually approaches the performance of the serving cell. It indicates the predictions of a cell near to UE’s position (i.e., with high-level RSRPs) are different from a cell away from the UE (whose RSRPs are relatively low).

Observation 1: In FR1 to FR1 inter-frequency prediction, the datasets used for training and testing would impact the performance of RSRP difference. The prediction error for cells with high RSRPs (0.33 dB error) is larger than cells with low RSRPs (0.12 dB error).

One possible reason is the RSRP difference variation. For a non-serving cell, the signal strength can be rather weak as UE moves far away from it. The RSRP difference between 2GHz and 4GHz can be flat in the time domain. Therefore, it is easier for AI/ML to predict. For the serving cell or a cell that is near to the UE (i.e., UE can get high RSRP), the RSRP difference between 2GHz and 4GHz varies when UE’s position changes and is thus harder to predict accurately. When the RSRP difference between 2GHz and 4GHz is flat in the time domain (e.g., a constant), the sample-and-hold method can already achieve good performance. There is no need to introduce AI/ML that is more complicated but with fewer gains. Besides, cells with higher RSRPs are of more interest in implementation, especially in determining target/candidate cells. Given these, it is proposed that

Proposal 1: RAN2 to focus on predicting cells with high signal strength, e.g., the serving cell and/or cells with high RSRP values.

Following our proposal, we use the serving cell dataset to compare the inter-frequency prediction results at different UE speeds as shown in Table 3.

Table 3. Inter-frequency prediction results at different UE speeds

| Average L3 cell-level RSRP difference (dB) | 30km/h | 60km/h |
| --- | --- | --- |
| 2GHz->4GHz | 0.33 | 0.33 |
| 4GHz->2GHz | 0.39 | 0.38 |

Observation 2: UE speed has less impact on the prediction accuracy of FR1 to FR1 inter-frequency prediction.

Observation 3: Both prediction from 2GHz to 4GHz and prediction from 4GHz to 2GHz have good prediction accuracy (within 0.39 dB). The former works slightly better.

Given that a UE can handover from a low-frequency cell to a high-frequency cell to get better channel quality, and switch from a high-frequency cell to a low-frequency cell to have better coverage, it is proposed that

Proposal 2: Inter-frequency prediction considers both prediction from low-frequency cell to high-frequency cell and prediction from high-frequency cell to low-frequency cell. Only one UE speed is considered for inter-frequency prediction in simulation, e.g., 30km/h.

FR1 to FR1 intra-frequency temporal domain case B

The aim of temporal domain case B is to reduce measurement overhead and use those predicted results to reflect the wireless channel changes to make mobility decisions. There seem to be different understandings towards case B, for example, if we adopt a measurement reduction rate (MMR) of 50%:

- Understanding 1: Using current and historical measured RSRPs to predict the RSRPs of time instances in the future (e.g., using RSRPs at time instances 1, 3, 5, 7 to predict RSRP at time instance 8). The prediction needs to be done every other Tper, i.e., AI/ML will do the next prediction at time instance 3 by using RSRPs at time instance 3, 5, 7, 9 to predict RSRP at time instance 10.

- Understanding 2: Using current and historical measured RSRPs to predict the RSRPs of time instance in history and in the future (e.g., using RSRPs at time instances 1, 3, 5, 7 to predict RSRPs at time instance 2, 4, 6, 8). The prediction can be done every X times of Tper (8 Tper as shown in Figure 3).

- Understanding 3: Using current and historical measured RSRPs to predict the RSRPs of time instances in the future but do this every X times of Tper (e.g., using RSRPs at time instances 1, 2, 3, 4 to predict RSRPs at time instances 5, 6, 7, 8, and then using RSRPs at time instances 9, 10, 11, 12 to predict that at 13, 14, 15, 16.)

Figure 3. Example for intra-frequency temporal domain case B

There could be other variations of the way to achieve 50% MMR. It would be hard for companies to align them considering the huge time it may consume and the little benefit it can bring. A similar thing also happens to the inter-frequency prediction case, e.g., understanding of how many instances to be model input and model output can be different. For example, companies can use the RSRP at time instance 1 to predict the corresponding RSRP at another frequency or can use the 2GHz RSRPs at time instances 1, 2, 3 to predict the corresponding 4GHz RSRPs at time instances 1, 2, 3. In RAN2#126, we have agreed that

“Intra-frequency intra-cell spatial domain prediction is done by measuring sub set of configured SSB as input to the model to predict L3 cell level measurements for every instance of the same cell. It is only evaluated for FR2 intra-frequency scenario and RRM sub case 1 and 3. Several measurement reduction rates should be aligned among company without defining detail pattern.”

The detailed pattern of beams and time instances chosen for prediction does not need to be aligned for spatial domain prediction. We can adopt similar agreements for intra-frequency temporal domain case B.

Proposal 3: For intra-frequency temporal domain case B prediction, no need to align detailed patterns and time instances chosen for AI/ML model input and output as long as the basic assumptions, e.g., measurement reduction rate, are aligned.

Our following simulation results are based on understanding 2, i.e., FR1 to FR1 intra-frequency temporal domain case B uses AI/ML to reduce measurement overhead by interpolation. For interpolation, we do not need to predict a long sequence as multiple short sequences can achieve better prediction accuracy with a less complex model. Therefore, we adopt an L3-RSRP prediction with a maximum length of 8 time instances (with a 40ms time interval). A measurement reduction rate (MMR) of 50% is used to analyze the performance for sub case 1, 2, and 3 of a given cell.

- Case 1: We used L1 beam level measurement results at time instances [k, k+2, k+4, k+6] to predict L1 beam level measurement results at time instances [k+1, k+3, k+5, k+7], and then used them all to obtain L3 cell level measurement results from time instant k to k+7.

- Case 2: We used L3 cell level measurement results at time instances [k, k+2, k+4, k+6] to predict L3 cell level measurement results at time instances [k+1, k+3, k+5, k+7]

- Case 3: We used L1 beam level measurement results at time instances [k, k+2, k+4, k+6] to predict L3 cell level measurement results from time instant k to k+7.

Table 4. FR1 to FR1 intra-frequency temporal domain case B prediction assumptions

| Report parameters |  | OPPO |  | FR1 to FR1 intra-frequency temporal domain case B |  |
| --- | --- | --- | --- | --- | --- |
| Reported simulation assumptions | UE trajectory option (option 1,2,3 in[4]) | Option 1 |  |  |  |
|  | UE trajectory boundary processing option (option 1,2,3 in[4]) | Option 3 |  |  |  |
|  | UE speed (30,60,90,120 Km/h) | 30 if not explicitly stated |  |  |  |
|  | Whether LOSsoft is modeled or not | No |  |  |  |
|  | spatial consistency option (A or B) | A |  |  |  |
|  | Number of TX beams | 1 |  |  |  |
|  | Number of RX beams | 1 |  |  |  |
|  | Measurement reduction rate | 50% |  |  |  |
| Data Size (Number of Samples) | Training/validity | ~0.82M/~0.20M |  |  |  |
|  | Testing | ~0.26M |  |  |  |
| AI/ML model input/output | Model input (Note 5) | L1-RSRP of cell A without being filtered | L3-RSRP of cell A |  | L1-RSRP of cell A without being filtered |
|  | Model output(Note 6) | L1-RSRP of cell A without being filtered | L3-RSRP of cell A |  | L3-RSRP of cell A |
| AI/ML model description | Model type (e.g., LSTM, CNN, transformer …) | 2 LSTM layer + 1 dense layer |  |  |  |
|  | Model complexity in a number of parameters(M) | ~0.18M |  |  |  |
|  | Model complexity in model size (e.g. Mbyte) | ~0.69MB |  |  |  |
|  | Computational complexity [FLOPs] | ~0.40M |  |  |  |

Similar to the inter-frequency prediction case, we evaluate the performance with different datasets as shown in Table 5. The last row reveals again that the prediction for cells with high RSRPs is harder than for cells with low RSRPs, which proves the necessity of proposal 1. Besides, we can observe that

Observation 4: AI/ML prediction can achieve high accuracy (maximum 1.06 dB error) for all sub cases 1, 2, and 3 in FR1 to FR1 intra-frequency temporal domain case B.

The high accuracy lies in that the time difference between two sampling time instants is small, thus the predicted RSRP between two sampling time instances would not deviate much from the RSRPs at the two sampling instances. This becomes more evident in sub case 2 where the model input and output are both filtered. The more flat RSRP of sub case 2 (compared to sub cases 1 and 3) contributed to a smaller RSRP difference observed. Although sub case 2 outperforms sub cases 1 and 3 in terms of average L3 cell-level RSRP difference, it can not reduce any L1 measurement as we stated in [1].

Table 5. temporal domain case B results for sub cases 1, 2, and 3

| 4GHz | Given cell | Given cell with post-process (-120dBm) | Given cell with post-process (-110dBm) | Serving cell |
| --- | --- | --- | --- | --- |
| Sub case 1 | 0.38 | / | / | / |
| Sub case 2 | 0.12 | 0.19 | 0.26 | / |
| Sub case 3 | 0.32 | 0.64 | 0.82 | 1.06 |

We also stimulated the RSRP difference under different UE speeds. The results use serving cell datasets. It can be seen from Table 6 that

Observation 5: The average RSRP difference of UE under 60km/h (2.26 dB) is more than 1dB larger than 30km/h (1.06 dB) in FR1 to FR1 intra-frequency temporal domain case B in sub case 3. Higher UE speed leads to larger prediction errors.

Table 6. Temporal domain case B results at two different UE speeds

| Average L3 cell-level RSRP difference (dB) | 30km/h | 60km/h |
| --- | --- | --- |
| Sub case 3 | 1.06 | 2.26 |

FR2 to FR2 intra-frequency temporal domain case A

Figure 4. Example for intra-frequency temporal domain case A

In intra-frequency temporal domain case A, the AI/ML model can use L1-RSRP measured in the past time window (e.g., 400ms) to predict the L3-RSRP values in the future time sequence. The prediction window size should be relatively long to leave enough time for the network to take action. Table 7 shows the prediction assumptions of different observation/prediction window sizes of a serving cell.

Table 7. FR2 to FR2 intra-frequency temporal domain case A prediction assumption

| Report parameters |  | OPPO |  | FR2 to FR2 intra-frequency temporal domain case A |  |
| --- | --- | --- | --- | --- | --- |
| Reported simulation assumptions | UE trajectory option (option 1,2,3 in[4]) | Option 1 |  |  |  |
|  | UE trajectory boundary processing option (option 1,2,3 in[4]) | Option 3 |  |  |  |
|  | UE speed (30,60,90,120 Km/h) | 60, 90, and 120 |  |  |  |
|  | Whether LOSsoft is modeled or not | No |  |  |  |
|  | spatial consistency option (A or B) | A |  |  |  |
|  | Number of TX beams | 32 |  |  |  |
|  | Number of RX beams | 4 |  |  |  |
|  | Observation window | 200ms | 400ms | 200ms | 400ms |
|  | Prediction window | 200ms | 400ms | 200ms | 400ms |
|  | Any other parameters | Serving cell dataset is adopted |  |  |  |
| Data Size (Number of Samples) | Training/validity | ~0.25M/~0.06M |  |  |  |
|  | Testing | 0.09M |  |  |  |
| AI/ML model input/output | Model input (Note 5) | L1-RSRP of cell A without being filtered |  | L3-RSRP of cell A |  |
|  | Model output(Note 6) | L3-RSRP of cell A |  | L3-RSRP of cell A |  |
| AI/ML model description | Model type (e.g., LSTM, CNN, transformer …) | 2 LSTM layer + 1 dense layer |  |  |  |
|  | Model complexity in a number of parameters(M) | ~0.19M |  |  |  |
|  | Model complexity in model size (e.g. Mbyte) | ~0.72MB |  |  |  |
|  | Computational complexity [FLOPs] | ~0.42M |  |  |  |

Figure 5 shows the RSRP difference results under different UE speeds and sub cases. The corresponding values for each dot are recorded in the Annex.

Figure 5. FR2 to FR2 temporal domain case A results for sub cases 2, and 3 under different speeds

As prediction window increase, the RSRP difference goes higher for both sub cases 2 and 3.

Observation 6: Prediction error with 400ms observation/prediction window is more than 0.3 dB larger than that with 200ms window. A longer observation/prediction window results in a larger RSRP difference in FR2 to FR2 temporal domain case A.

Although deviating a little from each other, the evaluation results of sub case 2 under different prediction window sizes at different UE speed all control their average L3-RSRP difference within 1 dB, which is a relatively high accuracy that can be used to predict a measurement event. As the UE speed increases, it can be seen that

Observation 7: Average RSRP difference increases by up to 6.9 dB when UE speed increases from 60km/h to 120km/h in FR2 to FR2 intra-frequency temporal domain case A.

It aligns with our instinct that the trajectory of high-speed UE is hard to capture. For sub case 3, things are not as attractive as sub case 2. UEs with different speeds all have very large prediction errors, making it unfavourable for implementation. When UE speed is higher than 90km/h, the over 6 dB error shows that the AI/ML model could not learn the mobility behavior well and is unable to correctly predict future RSRPs.

Observation 8: For RRM  sub case 2, AI/ML model can predict the RRM measurement in the next 400ms with relatively high prediction accuracy (within 0.86 dB).

Observation 9: RRM Sub case 3 with up to 9.6 dB error is not a good candidate for FR2 to FR2 intra-frequency temporal domain case A prediction.

The huge difference between sub cases 2 and 3 is because L1-RSRP has more volatility than L3-RSRP. In another word, non-filtered L1-RSRP has more noise that would interfere the prediction. The same issue would also occur for sub case 1. Besides, with a similar AI/ML model structure, the larger the number of parameters the input has, the bigger the error could be. The prediction for sub case 3 needs a more complicated AI/ML model than sub case 2 to get a close result.

Spreadsheets for collection of simulation assumptions and results

In TR 38.843, all the simulation assumptions and results from different companies are collected into case-specific spreadsheets to reflect companies’ efforts and for comparison. It would be better if we could do this work from the star. Therefore, it is proposed that

Proposal 4: Companies to use case-specific spreadsheets to collect and report simulation assumptions and results as what TR 38.843 presented.

We have attached 3 spreadsheets to accompany this Tdoc. It could be a template for further improvement.

Conclusion

In this contribution, we have shared our simulation results on RRM measurement prediction and have the following observations:

Observation 1: In FR1 to FR1 inter-frequency prediction, the datasets used for training and testing would impact the performance of RSRP difference. The prediction error for cells with high RSRPs (0.33 dB error) is larger than cells with low RSRPs (0.12 dB error).

Observation 2: UE speed has less impact on the prediction accuracy of FR1 to FR1 inter-frequency prediction.

Observation 3: Both prediction from 2GHz to 4GHz and prediction from 4GHz to 2GHz have good prediction accuracy (within 0.39 dB). The former works slightly better.

Observation 4: AI/ML prediction can achieve high accuracy (maximum 1.06 dB error) for all sub cases 1, 2, and 3 in FR1 to FR1 intra-frequency temporal domain case B.

Observation 5: The average RSRP difference of UE under 60km/h (2.26 dB) is more than 1dB larger than 30km/h (1.06 dB) in FR1 to FR1 intra-frequency temporal domain case B in sub case 3. Higher UE speed leads to larger prediction errors.

Observation 6: Prediction error with 400ms observation/prediction window is more than 0.3 dB larger than that with 200ms window. A longer observation/prediction window results in a larger RSRP difference in FR2 to FR2 temporal domain case A.

Observation 7: Average RSRP difference increases by up to 6.9 dB when UE speed increases from 60km/h to 120km/h in FR2 to FR2 intra-frequency temporal domain case A.

Observation 8: For RRM  sub case 2, AI/ML model can predict the RRM measurement in the next 400ms with relatively high prediction accuracy (within 0.86 dB).

Observation 9: RRM Sub case 3 with up to 9.6 dB error is not a good candidate for FR2 to FR2 intra-frequency temporal domain case A prediction.

Based on these observations, it is proposed that

Proposal 1: RAN2 to focus on predicting cells with high signal strength, e.g., the serving cell and/or cells with high RSRP values.

Proposal 2: Inter-frequency prediction considers both prediction from low-frequency cell to high-frequency cell and prediction from high-frequency cell to low-frequency cell. Only one UE speed is considered for inter-frequency prediction in simulation, e.g., 30km/h.

Proposal 3: For intra-frequency temporal domain case B prediction, no need to align detailed patterns and time instances chosen for AI/ML model input and output as long as the basic assumptions, e.g., measurement reduction rate, are aligned.

Proposal 4: Companies to use case-specific spreadsheets to collect and report simulation assumptions and results as what TR 38.843 presented.

Reference

R2-2406311, Discussion on open issue of RRM measurement use cases, OPPO

Annex A

Recorded prediction results

Table 8. Recorded results for Figure 5.

| Average L3 cell-level RSRP difference (dB) | Observation/prediction window | 60km/h | 90km/h | 120km/h |
| --- | --- | --- | --- | --- |
| Sub case 2 | 200ms | 0.26 | 0.36 | 0.71 |
|  | 400ms | 0.74 | 0.67 | 0.86 |
| Sub case 3 | 200ms | 2.26 | 6.79 | 8.73 |
|  | 400ms | 2.70 | 8.18 | 9.60 |