3GPP TSG-RAN WG2 #127	R2-2406311

Maastricht France August 19th – 23th, 2024

Agenda Item:	8.3.2.2

Source:	OPPO

Title:	Discussion on open issues of RRM measurement

Document for:	Discussion, Decision

Introduction

At RAN2#126 meeting, general simulation assumptions, RRC parameters for RRM measurement and priority of evaluation scenarios etc. are concluded. Still there few open issues left for RRM measurement. This paper intends to address those open issues.

Discussion

L3 Cell level prediction

Further clarification on sub use cases

RAN2 agreed following use cases in RAN2#125bis:

For cell level measurement prediction model, at least consider the following cases:

Case 1: To predict beam level results, then generate cell level results based on the predicted beam results;

Case 2: To directly predict cell level results based on cell level results.

Case 3: To directly predict cell level results based on beam level results

In current measurement model, there is L1 cell level measurement and L3 cell level measurement after L3 filtering. So, it is not crystal clear which one it is. From company contributions, offline and online discussion, we think it should be L3 filtered cell level measurement result at least for output of the model. In addition, following agreements in RAN2#126 also imply it is L3 filtered cell level measurement:

Methodology of Intra_F_C_S: Intra-frequency intra-cell spatial domain prediction is done by measuring sub set of configured SSB as input to the model to predict L3 cell level measurements for every instance of the same cell

Agreements

1	For the cell level measurement prediction, start with consider a fixed value for L3 filtering in simulation.   FFS which fixed value

2	For cell level prediction, RSRP difference to the actual measurement is calculated based on L3 filtered measurement result

Proposal 1: In the definition of RRM sub cases, RAN2 confirms the output cell level result refers to L3 filtered cell level measurement result

For the input cell level result in sub use case 2, it could be L3 filtered cell level measurement result. During offline discussion some company think it could be also L1 cell level measurement result. L1 cell level measurement result keeps some L1 information with less numbers of input parameters compared to L1 beam level measurement results. The reduction of input parameter may help to improve model’s performance with same level of complexity. So, it is worthy to check the applicability of L1 cell level measurement result as input of the model.

At RAN2#126 meeting, RAN2 agreed that sub case 2 is not applicable for intra-frequency spatial domain. This simply because input measurement result supposes to be an actual L3 cell level measurement result, which should contain the measurement result of all beams. The same reason still stands for L1 cell level result because actual L1 cell level result should be consolidation result of all beams.

For FR1 to FR1 inter-frequency prediction and intra-frequency temporal domain case A, sub use case 2 with L1 cell level result as input is still applicable because no measurement is reduced in both temporal and spatial domain for the cell to be actually measured.

But we find case B need more discussion. For sub use case 2 with L3 cell level result as input assuming some L3 RSRP results are to be predicted and hence skipped (with grey colour in Figure 2.1-1 and 2.1-2), no L1 measurement samples are saved at all because of L1/L3 filtering scheme itself.

Figure 2.1-1: L1/L3 filtering option 1

Figure 2.1-2: L1/L3 filtering option 2

Now assuming the input is L1 cell level result i.e., without L3 filtering, the L1 measurement can be saved. For L1/L3 filtering option 1, it works as long as raw L1 sample is used for consolidation for L1 cell level result. For option 2, it works when either raw L1 samples or L1 filtered samples are used for consolidation for L1 cell level result.

Proposal 2: For RRM sub use case2, both L1 and L3 cell level measurement result can be input of the model

Proposal 3: When L1 cell level measurement result is input, sub use case 2 is applicable for temporal and frequency domain prediction but not spatial domain prediction

Proposal 4: For RRM sub use case 2, L1 instead of L3 cell level measurement result should be applied for intra-frequency temporal domain case B as model input.

Clarification on temporal domain prediction

During RAN2#126 meeting for intra-frequency temporal domain, during offline discussion there is some ambiguity whether the case illustrated in Figure 2.1-3 is intra-frequency temporal case A or case B:

Figure 2.1-3: one intra-frequency temporal domain case B

In Figure 2.1-3 UE perform actual measurement in observation window e.g. between t0 and t1 or between t2 and t3. The measurement results within prediction window e.g. between t1 and t2 are always predicted and hence skipped. The intention is clear i.e. to save the measurement within prediction window. At RAN2#126 when few evaluation scenarios are prioritized, the target goal of intra-frequency temporal domain case A is 2nd goal while temporal domain case B is 1st goal. RAN2 agreed at RAN2#126 meeting that “For the evaluation exercise for 2nd study goal, RAN2 should assume that there is no reduction in measurement overhead”. Obviously, the case illustrated in Figure 2.1-3 belongs to intra-frequency temporal domain case B. For case A, in order to predict whenever a new input measurement result is available, a sliding window scheme should be applied for both observation window and prediction window and eventually no measurement is reduced as illustrated in Figure 2.1-4.

Figure 2.1-4: intra-frequency temporal domain case A

Proposal 6: RAN2 confirm Intra-frequency intra-cell temporal domain case B prediction is evaluated for 1st study goal by predicting a sub set of measurement instances in temporal domain of the same cell. The detail TDM pattern e.g. as illustrate in Figure 2.1-1 is up to implementation.

Clarification on inter-frequency prediction

For FR1 to FR1 inter-frequency, the intention is to save measurement gap, which supposes to be used for measuring non-serving frequency. For baseline case it means UE needs do the measurement in TDM way. But it may not be the case for evaluation, inference and monitoring procedure.

Figure 2.1-5 Baseline methodology

Figure 2.1-6 evaluation methodology

Figure 2.1-7 inference methodology

Figure 2.1-8 monitoring methodology

Figure 2.1.5~8 illustrate our understanding of the FR1 to FR1 inter-frequency prediction. For evaluation purpose, in order to calculate prediction accuracy, the time instances to be measured on serving frequency and to be predicted on non-serving frequency should be align with the time instance, when the corresponding label is measured as indicated in Figure 2.1.6. During inference, more measurement results can be predicted assuming more measurement is performed on serving frequency. It reduces the measurement period equivalently compared to baseline case and hence also improve the measurement performance. Upon monitoring Time instance of measurement on serving frequency and non-serving frequency is performed in TDM way just like baseline. But the time instance to be predicted should be aligned with serving frequency in order to align with evaluation case by reusing the same model.

Observation 2: For FR1 to FR1 inter-frequency prediction, the methodology could be different among baseline, evaluation, inference and monitoring procedures.

Proposal 7: For FR1 To FR1 inter-frequency prediction, same model should be applied for evaluation, inference and monitoring procedure.

L3 beam level measurement prediction

The index of reference signal along with its L3 beam level measurement results can be reported in measurement report. The L3 beam measurement result can be used e.g., to select right PRACH resource. It is sort of optional information i.e. not critical for mobility procedure. We think L3 beam level measurement prediction can be put on the table. But RAN2 should focus on evaluation cases with cell level measurement.

Proposal 8: L3 beam level measurement prediction can be addressed after L3 cell level measurement use case

The L3 beam level measurement is firstly L3 filtered based on L1 beam level measurement of the same beam and then subset of them is picked based on either absThreshSS-BlocksConsolidation or absThreshCSI-RS-Consolidation. Technically all the agreed evaluation scenarios are also applicable for L3 beam level measurement. As for sub cases, we can also have 3 sub cases:

- L1 beam level measurement to L1 beam level measurement and then L3 beam level measurement

- L3 beam level measurement to L3 beam level measurement

- L1 beam level measurement to L3 beam level measurement

In section 2.1.1 RRM sub case 2 with L3 cell level result as input is excluded for intra-frequency temporal domain case B. We think the reason behind is also applicable for beam level RRM sub case 2.

Proposal 9: Agreed evaluation scenarios for cell level RRM measurement are applicable for L3 beam level measurement use case

Proposal 10: Beam level RRM sub case 2 is not applicable for intra-frequency temporal domain case B and intra-frequency spatial domain

The definition of prediction accuracy i.e., average difference of L3 cell level RSRP can modified to be average difference of L3 beam level RSRP. As for measurement reduction rate, MRRS is not applicable because the measurement granularity per beam, while MRRT is still applicable.

Proposal 11: MRRT is applicable for L3 beam level measurement while MRRS is not.

Proposal 12: The prediction accuracy of L3 beam level measurement is defined as average difference of L3 beam level RSRP

Cluster approach

At RAN2#126 meeting, RAN2 agreed that “To define cluster approach at least based on the number of input cells, number of output cells and their relationship in temporal domain, spatial domain and frequency domain”. For prediction in any domain, it is not feasible if UE is out of the coverage of concerned cells. And within one frequency layer, usually UE is in the coverage of up to 3 cells of one frequency layer regardless whether they are co-located or not as illustrated in Figure 2.3-1. It means the maximum number of input cells and output cells should be <=3. For FR1 to FR1 inter-frequency prediction, because the input cells and output cells are from different frequency and hence the maximum number of input cells and output cells should be also <=3. Note this view is mainly from inference perspective.

Figure 2.3-1

Proposal 13: For cluster approach the maximum number of input cells or output cells of one frequency layer should be <=3

For inter-frequency prediction, it is straight way that input cells and output cells are located in different frequency layer. For intra-frequency prediction output cells could cover at least input cells. The question is whether cell(s) in addition to input cell(s) can be also predicted. We intend to think it is difficult for UE sided model and for generalization study.

For both inter-frequency and intra-frequency prediction, it may not be necessary to limited either input and/or output cell within same site i.e. co-location is not necessary the condition for prediction.

Proposal 14: For cluster approach of inter-frequency prediction, input cell(s) and output cells are located in different frequency layer and can be co-located or non-co-located.

Proposal 15: For cluster approach of intra-frequency prediction, output cell(s) can be the same set of input cell(s).

Conclusion

Observation 1: there is mainly two ways to do L1/L3 filtering as illustrated in Figure 2.1-1 and Figure 2.1-2

Proposal 1: In the definition of RRM sub cases, RAN2 confirms the output cell level result refers to L3 filtered cell level measurement result

Proposal 2: For RRM sub use case2, both L1 and L3 cell level measurement result can be input of the model

Proposal 3: When L1 cell level measurement result is input, sub use case 2 is applicable for temporal and frequency domain prediction but not spatial domain prediction

Proposal 4: For RRM sub use case 2, L1 instead of L3 cell level measurement result should be applied for intra-frequency temporal domain case B as model input.

Proposal 6: RAN2 confirm Intra-frequency intra-cell temporal domain case B prediction is evaluated for 1st study goal by predicting a sub set of measurement instances in temporal domain of the same cell. The detail TDM pattern e.g. as illustrate in Figure 2.1-1 is up to implementation.

Observation 2: For FR1 to FR1 inter-frequency prediction, the methodology could be different among baseline, evaluation, inference and monitoring procedures.

Proposal 7: For FR1 To FR1 inter-frequency prediction, same model should be applied for evaluation, inference and monitoring procedure.

Proposal 9: Agreed evaluation scenarios for cell level RRM measurement are applicable for L3 beam level measurement use case

Proposal 10: Beam level RRM sub case 2 is not applicable for intra-frequency temporal domain case B and intra-frequency spatial domain

Proposal 11: MRRT is applicable for L3 beam level measurement while MRRS is not.

Proposal 12: The prediction accuracy of L3 beam level measurement is defined as average difference of L3 beam level RSRP

Proposal 13: For cluster approach the maximum number of input cells or output cells of one frequency layer should be <=3

Proposal 14: For cluster approach of inter-frequency prediction, input cell(s) and output cells are located in different frequency layer and can be co-located or non-co-located.

Proposal 15: For cluster approach of intra-frequency prediction, output cell(s) can be the same set of input cell(s).

Reference

[1] R2-2404713 Discussion on simulation assumptions of RRM measurement OPPO