3GPP TSG-RAN WG2 Meeting #127	R2-2406322

Maastricht, Netherlands, August 19th – 23rd, 2024

Source:	CATT

Title:	Further discussion on regenerative payload

Agenda Item:	8.8.5

Document for:	Discussion and Decision

Introduction

In this document, we continue to discuss the remaining issues for regenerative payload with our proposals provided.

Discussion

Common TA and Kmac

In RAN2#126 meeting [1], the setting of common TA and Kmac for regenerative payload with full gNB on board was agreed as follow:

- RAN2 assumes that the typical setting for common TA and kmac would be zero in case of regenerative mode with full gNB on board. FFS whether we capture in the spec that common TA and Kmac shall/should be set to zero.

For regenerative payload with full gNB on board, companies failed to reach consensus on where RP is located and whether/how this should be specified. Some companies commented that RP can be located flexibly according to the network deployment. However, this will definitely bring complexity on NW implementation. Take common TA as the example. The common TA can be set to zero when RP is located at satellite. If RP is located at another place than the satellite, there would be the following technical problems:

- If it is assumed that the RP is one point on the service link, it means that the full TA for the pre-compensation should be the service link TA minus the common TA. However, this may not be supported by the TA computation mechanism in the current Spec, as this requires a configuration for a negative common TA value that may not be able to be supported by the current common TA parameters;

- Alternatively, if it is assumed that the RP is still a point on the feeder link, it would result in the full TA larger than the UE-gNB RTT and it is unclear what the use case for this actually is. Also, it is questionable how the Kmac should be set in this case and whether the existing definition on the UE-gNB RTT in the current Spec [2] can still be reused:

UE-gNB RTT: For non-terrestrial networks, the sum of the UE's Timing Advance value (see TS 38.211 [8] clause 4.3.1) and kmac.

As a result, if the RP not located on the satellite is considered, there is the risk that companies may re-open discussion on common TA and Kmac configurations. As we only take the satellite with full gNB on board as the regenerative payload deployment to provide service for UE directly in this Release, in order to reduce the spec impacts and ease the discussion, it is suggested that RAN2 confirms the support of "RP-on-Satellite" as follows:

Proposal 1: RAN2 confirms RP is located on the satellite in the case of regenerative payload with full gNB on board.

If Proposal 1 is agreed, it is straightforward that common TA should be set zero.

Proposal 2: RAN2 confirms Common TA equals zero in the case of regenerative payload with full gNB on board.

For Kmac, it is approximately equal to the RTT between the RP and the gNB as specified in current TS 38.300. If RP is located on satellite, the RTT between the RP and the gNB is zero, and this is similar to the case that the RP is located at gNB in of the transparent payload. Hence, Kmac is approximately equal to zero in the case of regenerative payload with full gNB on board.

Proposal 3: RAN2 confirms Kmac is approximately equal to zero in the case of regenerative payload with full gNB on board.

If Proposal 1-Proposal 3 are agreed, the Spec impacts in TS 38.300 are proposed as follows:

Proposal 4: For regenerative payload with full gNB on board, the following should be captured in 38.300 as the implementation option:

- the RP is located on the satellite;

- common TA is set to zero;

- Kmac is approximately equal to zero.

RACH-less handover

The following agreement was made at the last meeting: [1]

- RAN2 aims at supporting RACH-less handover for regenerative architecture. Can further discuss whether any optimization is needed

RAN2 aims at supporting RACH-less handover for regenerative payload architecture. In the case of regenerative payload with full gNB on board, the main difference mainly lies in timing, compared with transparent NTN payload. N_TA with value identical to the source cell or zero is indicated by the source cell which is configured by the target cell.

In RAN3#123bis meeting, RAN3 has discussed the following scenarios for regenerative payload: [3]

- Intra/inter-gNB mobility scenarios: Scenario #1: intra-gNB(intra satellite) mobility; Scenario #2: inter-gNB mobility(between on board gNBs via ISL) Scenario #3: inter-gNB mobility(over feeder link)

For intra-gNB(intra satellite) mobility, the gNB/satellite is not changed. Therefore, the UE shares the same N_TA between source cell and target cell.

For inter-gNB mobility (between on board gNBs via ISL or over feeder link switch), both the gNB and the satellite are changed. With the help of ephemeris of the target satellite, the UE can synchronize to the target cell, assuming that gNB can provide valid assistance information of the target cell to UE. Note that RAN2 confirmed RACH-less handover for the inter-satellite inter-gNB scenario can be supported in Uu for the transparent payload in Rel-18 NR NTN [4], and also note that the main difference of regenerative payload in Uu, compared with transparent payload, is no more than a special setting of the common TA related and Kmac configurations. If the Rel-18 RACH-less handover design in Uu is already sufficient to support inter-satellite inter-gNB scenario in transparent payload, we find no showstopper to reuse it directly into the regenerative payload, especially considering that it is more accurate on TA estimation in regenerative payload than in transparent mode with only service link pre-compensation needed. There may be some optimizations needed considering topology between the source cell and target cell changes for regenerative payload with gNB on board. But they are in RAN3 scope.

Hence, we think there is no need for discussion/consideration on RACH-less HO enhancement in regenerative payload with full gNB on board in Uu from RAN2 perspective, unless more inputs from RAN3 (e.g. impact by RAN architecture change) are received. This is proposed as follows:

Proposal 5: From RAN2 perspective, no discussion/consideration for RACH-less HO in regenerative payload with full gNB on board is needed in Uu, until more inputs from RAN3 are received.

- 3.	Conclusion

In this document, we analyse the potential impacts brought by regenerative mode of gNB on board, and find the following proposals:

Proposal 1: RAN2 confirms RP is located on the satellite in the case of regenerative payload with full gNB on board.

Proposal 2: RAN2 confirms Common TA equals zero in the case of regenerative payload with full gNB on board.

Proposal 3: RAN2 confirms Kmac is approximately equal to zero in the case of regenerative payload with full gNB on board.

If Proposal 1-Proposal 3 are agreed, the Spec impacts in TS 38.300 are proposed as follows:

Proposal 4: For regenerative payload with full gNB on board, the following should be captured in 38.300 as the implementation option:

- the RP is located on the satellite;

- common TA is set to zero;

- Kmac is approximately equal to zero.

Proposal 5: From RAN2 perspective, no discussion/consideration for RACH-less HO in regenerative payload with full gNB on board is needed in Uu, until more inputs from RAN3 are received.

- 4.	Reference

- RAN2#126 chairman notes;

- TS 38.321, V18.2.0, Medium Access Control (MAC) protocol specification, Release 18;

- R3-243002, RAN3 #123-bis Meeting Report.

- R2-2306553, Report of 3GPP TSG RAN WG2 meeting #121bis-e, Online