3GPP TSG-RAN WG2 Meeting #127	R2-2406323

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda Item:	8.8.4

Source: 	CATT

Title: 	Discussion on support of broadcast service in NR NTN

Document for: 	Discussion and Decision

Introduction

In RAN2#126 [1], the following agreements were achieved:

Agreements:

- For MBS broadcast service, both EFC and EMC are supported.

- RAN2 will not define means for the NW to prevent the reception of the content of the service outside of the intended service area.

- MBS broadcast intended service area is provided via system information

- For MBS broadcast RAN2 considers the following possibilities for including the service area information: SIB20/ SIB21/ MBSBroadcastConfiguration. FFS for ETWS

- When intended service area (e.g., geographical area/TN coverage) is provided for MBS broadcast service, it needs to be associated with MBS session (FFS on the details)

In this contribution, the signalling design of service area and the UE behaviour of receiving MBS service in NR NTN will be discussed.

Discussion

Signalling design of service area

Issue#1: Geographical area format to model service area

The service area is conceptually with similarity compared with the TN area supported in Rel-18. In Rel-18, we had a long discussion on how to model the TN area, and finally RAN2 decided to use a circle-like area configuration to describe the TN coverage area. Signalling-wise, the same mechanism can be reused here for the MBS service area, i.e. adopting the configuration of a circular area with a reference location and a distance parameter. Considering that the MBS service area may be irregular shape or discrete, a set of circles on the ground could be used to describe the service areas of a broadcast service in this case. Each circle is signalled by a reference location and a radius which could reuse the signalling format defined for TN area in Rel-18 NR NTN.

Proposal 1: The intended service area of a MBS session is modelled as a list of circles, with each circle signalled with a reference location and a radius. The signalling format defined for TN area in Rel-18 NR NTN is reused.

Issue#2: How to provide the service area configuration to UE

In RAN2#126, it was agreed that RAN2 considers the following possible signalling for including the service area information: SIB20/ SIB21/ MBSBroadcastConfiguration (in MCCH).

Currently the SIB20 contains the information required to acquire the MCCH/MTCH configuration, whereas SIB21 contains the mapping between the current and/or neighbouring carrier frequencies and MBS FSAI. If the service area were to be added in SIB20/SIB21, the MBS session ID would also need to be added in the related SIB to indicate the mapping between the service area and MBS session. Adding both the service area configuration and MBS session ID will bring significant signalling overhead for the system information.

If the service area is contained in the MBSBroadcastConfiguration message in MCCH, the MBS session ID has already existed in the message and it can then be reused to map the service area to the intended MBS service, avoiding duplication of the MBS session ID information in different messages.

Observation 1: Compared with MBSBroadcastConfiguration, if SIB20/SIB21 is used to contain the service area, it will bring significant signalling overhead by adding both the MBS session ID and service area configuration into system information.

Additionally, for the location depend MBS service [2], in legacy mechanism, only MBS session ID is associated with the scheduling info with one cell associated to one Area Session ID. UE does not need to distinguish the area session ID from the AS signalling. For NTN, it is possible that an NTN cell provides services of more than one area sessions for the same MBS session. In this case, the UE should be able to distinguish the scheduling info of each area sessions of this MBS session, so that it could apply the scheduling info corresponding to its location.

### 6.2.3 Location dependent MBS service A location dependent MBS is identified by MBS Session ID, and provided in several MBS service areas. The location dependent MBS service enables distribution of different content data to different MBS service areas. The same MBS Session ID is used but a different Area Session ID is used for each MBS service area. The Area Session ID is used, in combination with MBS Session ID, to uniquely identify the service area specific part of the content data of the MBS service within 5GS. The network supports the location dependent content distribution for the location dependent MBS services, while UEs are only aware of the MBS Session ID (i.e. UEs are not required to be aware of the Area Session IDs). When UE moves to a new MBS service area, content data from the new MBS service area shall be delivered to the UE, and the network ceases to deliver the content data from the old MBS service areas to the UE.

This issue can be solved without additional ASN.1 impact, if the service area is contained in the message MBSBroadcastConfiguration on the MCCH. In detail, the service area is introduced for each entry of the MBS-SessionInfoList. For location depend MBS service, there may be multiple entries of MBS-SessionInfo having the same mbs-SessionId but each associated with different service areas corresponding to different area sessions. For the same mbs-SessionId that is contained in more than one entry of MBS-SessionInfoList, the UE can determine the configuration in which entry to apply, by comparing its location and the service area included in each entry. By this means, explicit indication of area session ID is not needed, which is aligned with the green highlight text shown as above in legacy.

MBS-SessionInfoList-r17 ::= SEQUENCE (SIZE (1..maxNrofMBS-Session-r17)) OF MBS-SessionInfo-r17 MBS-SessionInfo-r17 ::= SEQUENCE { mbs-SessionId-r17 TMGI-r17, g-RNTI-r17 RNTI-Value, mrb-ListBroadcast-r17 MRB-ListBroadcast-r17, mtch-SchedulingInfo-r17 DRX-ConfigPTM-Index-r17 OPTIONAL, -- Need S mtch-NeighbourCell-r17 BIT STRING (SIZE(maxNeighCellMBS-r17)) OPTIONAL, -- Need S pdsch-ConfigIndex-r17 PDSCH-ConfigIndex-r17 OPTIONAL, -- Need S mtch-SSB-MappingWindowIndex-r17 MTCH-SSB-MappingWindowIndex-r17 OPTIONAL -- Cond MTCH-Mapping }

Observation 2: If the service area information is added in the message MBSBroadcastConfiguration on the MCCH, the location dependent MBS service can be support by associating different service areas (each corresponding to one area session) with multiple entries including the same mbs-SessionId value. This would lead to no additional ASN.1 impact specific to location dependent service.

As per above analysis, we suggest to contain the service area in the message MBSBroadcastConfiguration on the MCCH. The service area is associated with each entry of the MBS-SessionInfoList.

Proposal 2: The service area is provided via MBSBroadcastConfiguration message on the MCCH, and the service area is associated with each entry of the MBS-SessionInfoList.

UE behaviour of receiving MBS service

In RAN2#126, the UE behaviour of receiving MBS service when UE is not within the service area was discussed.

Come back next meeting to check whether a UE may not (or should not) establish MRB(s) associated with a service limited to an intended service area when it is not located within the area (up to UE implementation how the UE detects it’s not in the intended service area) Come back next meeting to check whether a UE may (or should) release its established MRB(s) associated with a service limited to an intended service area when exiting the area (up to UE implementation how the UE detects it’s not in the intended service area)

Note that here it is the trigger of MRB establishment/release procedure that is under discussion. After checking the procedure texts for legacy MRB configuration [3] (see citation in the box below), it can be seen that the MRB establishment and MRB release procedures are not enforced for UE, when related conditions are fulfilled, i.e., using “may” instead of “should” for the trigger of UE behaviour of MRB establishment/release for receiving MBS service. This needs to be followed also for the trigger of MRB establishment/release procedures, when MBS is supported in NR NTN, taking also the associated service area into consideration.

#### 5.9.3.2 Initiation The UE applies the broadcast MRB establishment procedure to start receiving an MBS session of an MBS broadcast service it is interested in. The procedure may be initiated e.g. upon start of the MBS session, upon entering a cell providing an MBS broadcast service the UE is interested in, upon becoming interested in the ongoing MBS broadcast service, upon removal of the UE capability limitations inhibiting reception of the ongoing MBS broadcast service UE is interested in. The UE applies the broadcast MRB release procedure to stop receiving a session of an MBS broadcast service. The procedure may be initiated e.g. upon stop of the MBS session, upon leaving the cell broadcasting the MBS service the UE is interested in, upon losing interest in the MBS service, when capability limitations start inhibiting reception of the concerned service.

Proposal 3a: UE may initiate establishment procedure of MRB(s) associated with a service limited to an intended service area, upon entering the area.

Proposal 3b: UE may initiate release procedure of its established MRB(s) associated with a service limited to an intended service area, upon exiting the area.

Conclusion

In this contribution, supportive of broadcast service via NR NTN is discussed with giving corresponding proposals:

Observation 1: Compared with MBSBroadcastConfiguration, if SIB20/SIB21 is used to contain the service area, it will bring significant signalling overhead by adding both the MBS session ID and service area configuration into system information.

Observation 2: If the service area information is added in the message MBSBroadcastConfiguration on the MCCH, the location dependent MBS service can be support by associating different service areas (each corresponding to one area session) with multiple entries including the same mbs-SessionId value. This would lead to no additional ASN.1 impact specific to location dependent service.

Proposal 1: The intended service area of a MBS session is modelled as a list of circles, with each circle signalled with a reference location and a radius. The signalling format defined for TN area in Rel-18 NR NTN is reused.

Proposal 2: The service area is provided via MBSBroadcastConfiguration message on the MCCH, and the service area is associated with each entry of the MBS-SessionInfoList.

Proposal 3a: UE may initiate establishment procedure of MRB(s) associated with a service limited to an intended service area, upon entering the area.

Proposal 3b: UE may initiate release procedure of its established MRB(s) associated with a service limited to an intended service area, upon exiting the area.

Reference

- R2-2405703	Report from Break-out session on NR-NTN and IoT-NTN.

- 3GPP TS 23.247: "Architectural enhancements for 5G multicast-broadcast services".

- 3GPP TS 38.331: "Radio Resource Control (RRC) protocol specification".