3GPP TSG-RAN WG2 Meeting #127	R2-2406324

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda Item:	8.8.2

Source: 	CATT

Title: 	Discussion on Downlink Coverage Enhancements

Document for: 	Discussion and Decision

Introduction

In RAN2#126, RAN2 sent LS [1] to RAN1 asking the following questions:

Question1 : Can RAN1 provide the information on their progress on whether the existing SSB pattern for an NR cell (e.g. SSB position in burst, SSB index number, etc.) is changed in Rel-19 NR NTN, and whether the SSB periodicity is extended compared with existing TN values? Question2 : Can RAN1 provide the information on whether/how the solution RAN1 is investigating is expected to impact common control signalling for UEs in RRC idle / RRC inactive? Question3 : Can RAN1 provide the feedback on whether UL beam hopping is also being studied in RAN1 (and whether this is separate from DL beam hopping)? Question4 : RAN2 would like to remind RAN1 that satellite beams are currently not visible to UEs and any decision about different beam status (i.e. "off", "common messages only" and "active traffic") will likely have to relate to beams visible to the UE (e.g. SSB beams). RAN2 would also like to know whether RAN1 intends to define beam status for beams not visible to the UE or to define new beam status for beams visible to the UE. Question5 : Can RAN1 provide the feedback on whether the beam status in different beams (visible to the UE) of one cell are the same or can be different in any given time, i.e.. the beam status is cell specific or beam specific?

This contribution will further analyze RAN2 impacts on this feature.

Discussion

For the system-level enhancement, RAN1 is working on the evaluation of the SSB coverage with different SSB periodicities and this is related to one of the key issues asked by RAN2 in LS [1] to RAN1 (see Question 1 in the LS). Some latest progress and findings in RAN1 are cited as follow [2]:

Observation Based on the results of DL coverage ratio evaluation at system level collected from 7 sources for all the three LEO600km satellite parameter sets where the beam footprint diameter is 50 km: - For Set 1-1/1-3, the coverage ratio can be improved from 10% to 100% if the SSB periodicity is increased from 20ms to 80ms and beam hopping is applied - For Set 1-2, the coverage ratio can be improved from 1.5% to 96.8% if the SSB periodicity is increased from 20ms to 320ms and beam hopping is applied. - Note: coverage ratio is N2+N3/ total beam footprints - Note: the baseline assumes no beam hopping. TDM between SIB1 and SIB19 is assumed in those results, following current specs.

Based on RAN1’s evaluation, it can be seen that, to improve the coverage ratio, the SSB periodicity needs to be enlarged and beam hopping needs to be applied. However, no consensus has ever been reached in RAN1 on whether and how to extend the SSB periodicity. If the SSB periodicity is not extended, using an NES-like cell DTX/DRX mechanism does not help to improve the DL coverage in NTN, as it cannot concentrate the beam energy to raise the EIRP of each beam as the goal of the WID [2]. Only if the SSB periodicity is extended, does it make sense to apply NES-like cell DTX/DRX mechanism to provide different status of one cell/beam, e.g. with the inactive/active state of DTX/DRX. Also note that RAN1 has not made sufficient conclusions on the beam pattern of the UE-visible beams, i.e. NR beams, which are the beams based on which any further discussion/enhancements should be carried out.

Hence, from our perspective, whether RAN2 needs to support NES-like cell/beam level DTX/DRX mechanism should be decided after receiving RAN1 replies.

Proposal 1: RAN2 waits for RAN1 LS response, in order to decide whether to support NES-like DTX/DRX mechanism in NR NTN.

At the last meeting, another key issue discussed is how to deal with the legacy Rel-17/18 NR NTN UEs that cannot support DL coverage enhancement adopted in Rel-19 NR NTN, if the SSB periodicity is extended. With the extended SSB periodicity, for the UEs which do not support this feature, e.g., legacy UE, there may be some negative impacts on UEs access to the NW. For the initial access, legacy UEs in RRC_IDLE/INACTIVE will search cells still with the assumption of 20 ms default SSB periodicity, and may thus fail in initial access due to missing the SSB detected. Even if the legacy UE happens to detect the SSB, gets the SIB1 and camps on a cell successfully, it may not work as normal either. For example, if the SSB periodicity of the serving cell is extended and configured as 320 ms, legacy UE can only perform initial access according to the existing value of ssb-PeriodicityServingCell (See below citation), which is smaller than the actual extended SSB periodicity applied by the NW. That says, there could be a mismatch between the understanding of SSB periodicity by the legacy UE (e.g. 160 ms per ssb-PeriodicityServingCell), and the SSB periodicity really used by the NW (e.g. 320ms). Whether the UE could perform initial access successfully needs to be discussed. As for a legacy UE whether the extended default SSB periodicity is acceptable (e.g. impacts to the RRM requirements) also needs to be evaluated by RAN4.

ssb-PeriodicityServingCell          ENUMERATED {ms5, ms10, ms20, ms40, ms80, ms160},

Observation 1: If the default SSB periodicity is extended to be larger than 160ms, there may be some impacts on UE not supporting this feature, e.g. initial cell search, access and measurement.

As a result, it is proposed that RAN2 discusses whether to allow the UEs not supporting extended SSB periodicity to get access to the NR NTN with extended SSB periodicity.

Proposal 2: Discuss whether the UE that does not support the extended SSB periodicity could access the NTN cell with extended SSB periodicity (if agreed to be supported by RAN1).

Conclusion

In this contribution, DL coverage enhancement is discussed with the corresponding proposals listed below:

Observation 1: If the default SSB periodicity is extended to be larger than 160ms, there may be some impacts on UE not supporting this feature, e.g. initial cell search, access and measurement.

Proposal 1: RAN2 waits for RAN1 LS response, in order to decide whether to support NES-like DTX/DRX mechanism in NR NTN.

Proposal 2: Discuss whether the UE that does not support the extended SSB periodicity could access the NTN cell with extended SSB periodicity (if agreed to be supported by RAN1).

Reference

- R2-2405767	LS on DL coverage enhancements.

- RAN1 #117 meeting minutes.

- 3GPP TS 38.331: "Radio Resource Control (RRC) protocol specification".